- Applications under consideration

The following candidate applications are considered:

- Terrestrial broadcasting

- Mobile broadband

- Mobile multimedia downlink (MMD)

- Satellite-enabled Services

- Programme Making and Special Events (PMSE)

- Public Protection and Disaster Relief (PPDR)

- Broadband Direct-Air-to-Ground Communications (BDA2GC)

- Regulatory framework options for 1452-1492 MHz

The EU regulatory framework indicates that regulatory measures should be to the maximum extent possible technology and service neutral, while recognising the need for application selection by administrations in specific cases, as stated in the European Union Directive 2009/140/EC.

“Flexibility in spectrum management and access to spectrum should be increased through technology and service-neutral authorizations to allow spectrum users to choose the best technologies and services to apply in frequency bands declared available for electronic communications services in the relevant national frequency allocation plans in accordance with Community law (the ‘principles of technology and service neutrality’). The administrative determination of technologies and services should apply when general interest objectives are at stake and should be clearly justified and subject to regular periodic review.”

In this context, service neutrality means that an electronic communications service (ECS) may be provided in any band over any type of Electronic Communications Network (ECN). Electronic Communications Service (ECS) means a service normally provided for remuneration which consists wholly or mainly in the conveyance of signals on Electronic Communications Networks (ECN), including telecommunications services and transmission services in networks used for broadcasting, but exclude services providing, or exercising editorial control over content transmitted using electronic communications networks and services.

On the other hand, “restrictions on the principle of technology neutrality should be appropriate and justified by the need to avoid harmful interference, for example by imposing emission masks and power levels, to ensure the protection of public health by limiting public exposure to electromagnetic fields, to ensure the proper functioning of services through an adequate level of technical quality of service, while not necessarily precluding the possibility of using more than one service in the same frequency band, to ensure proper sharing of spectrum, in particular where its use is only subject to general authorisations, to safeguard efficient use of spectrum, or to fulfil a general interest objective in conformity with Community law.”

In the context of the 1452-1492 MHz band, restrictions to the principles of technology and service neutrality, such as for instance the selection of one or more specific applications or the allocation of specific sub-bands for specific applications, could only be justified if there are substantial benefits (e.g. access to certain content of general interest, technical quality or an appropriate spectrum sharing) that cannot be achieved without such restrictions. This could for instance be the case in the following situations:

- Benefits related to widest pan-European take-up of the application, eg, economies of scale and/or coverage, in order to reduce the risk of fragmentation. Selection of one or more applications compatible with a regulatory framework based on LRTCs can improve potential for economies of scale while allowing flexibility at national or regional level for other applications. Identification and recommendation (or reconfirmation), for instance on a preferred or non-binding basis, of one or more sub-bands at a pan-European level could further improve the chances of achieving the expected benefits;

- Requirement for full pan-European geographical coverage, where flexibility at national or regional level would not be possible. Typically, Broadband Direct-Air-to-Ground Communications (BDA2GC) would require identification and binding adoption of an exclusive sub-band at European level.

It should be noted that only WAPECS bands have been considered for LRTC so far, which, in accordance with RSPSPG05-102final, could limit the approach to 1,452 – 1,479.5 MHz in the band under consideration.

- Least Restrictive Technical Conditions - Principles

A technology and service neutral regulatory framework is implemented through the definition of Least Restrictive Technical Conditions (LRTCs) applicable to the band. The definition of LRTC allows to harmonise the technical conditions applicable to the band (and potentially enables equipment to benefit from economies of scale) while allowing flexibility [in licensing] at national level, as it is being done for instance for the 800 MHz spectrum (Digital Dividend).

The definition of LRTCs is based on the principle that they solely address the technical compatibility issues (in particular, co-frequency and adjacent frequency interference, compliance with, where necessary, existing international and/or regional agreements).

This option enables authorised users of the band to deploy any application, provided such deployment respects the technical conditions set out in the LRTC. This kind of regulatory regime provides clear and transparent conditions of co-existence for other users of frequency blocks in the same band as well as users in adjacent frequency bands. The emissions allowed at block/band edges are stable and well known over time. Operators and regulators also benefit from the maximum possible flexibility to determine the use of the band.

Least restrictive licence conditions may still be incompatible with some applications or combinations of applications, however such incompatibility would be solely related to co-existence issues resulting in the impossibility for this (these) application(s) to respect the LRTCs.

  - ECC experience with LRTCs

The ECC has a significant experience in the development of LRTCs. LRTCs have been developed for several applications in several bands.

The CEPT Report 19 [Editorial, add reference] studied the applicability of LRTCs and developed LRTCs applicable to frequency bands addressed in the context of WAPECS in a number of bands (in particular 3.4-3.8 GHz and 2500-2690 MHz). The report notes, inter alia, that coexistence between two WAPECS blocks using the same frequency in geographically separated areas can for instance be determined by the use of field strength, and that a frequency separation of 5 MHz is needed between an FDD uplink block or unsynchronised TDD block with another TDD block operating in the same geographical area.

The CEPT Report 30 [Editorial, add reference] developed LRTCs for both Electronic Communication networks (ECNs), PMSE applications and low power applications in the band 790-862 MHz. It should be noted that the CEPT Report 30 [Editorial, add reference] relied on studies and conclusions from the CEPT Report 23 [Editorial, add reference] and CEPT Report 29 [Editorial, add reference] which were studying the general coexistence issues in the band 470-862 MHz.

The CEPT Report 39 [Editorial, add reference] developed LRTCs for Electronic Communication networks (ECNs) and low power applications in the 2 GHz bands (i.e. in the bands 1920-1980 MHz paired with 2110-2170 MHz as well as in the bands 1900-1920 MHz and 2010-2025 MHz).

The ECC Report 131 [Editorial, add reference] studied the coexistence issues between terminals deployed under LRTCs in the band 2500-2690 MHz.

The main conclusions of this body of work with respect to implementing LRTCs are that:

- Block Edge Masks (BEMs) combined with a harmonised band plan are the preferred option to control adjacent frequency interference (both within the band and at the edge of the band);

- The aggregate PFD model or field strength is suitable to address scenarios related to co-frequency compatibility in geographically different service areas;

- Establishing LRTCs relies on the definition of a reference system (network scenario, receiver performance, etc). However, LRTCs can be developed for high level groups gathering a large number of applications. This is detailed in the following section.

It is not the purpose of this Report to establish the technical detail of appropriate LRTCs, however a general LRTC framework can be derived from past CEPT experience.

- Possible sets of LRTC to be developed

An initial split between systems with and without national flexibility needs to be considered due to the very different characteristics of the systems. Broadband Direct-Air-to-Ground Communications (BDA2GC) requires a binding specific allocation of a sub-band at pan-European level for the application to be commercially and technically feasible, which limits the benefits of an LRTC approach, as only a single application is deployed Europe-wide within such sub-band. In such a case, national flexibility is non-existent within that specific sub-band. As mentioned above, certain applications (such as for instance mobile broadband, mobile multimedia downlink, as well as the various satellite-enabled services) may benefit from the identification and recommendation (or reconfirmation), for instance on a preferred or non-binding basis, of one or more sub-bands at a pan-European level, but this does not affect their eligibility for consideration of a potential regulatory implementation using LRTC.

3.1	Systems allowing national flexibility

The past CEPT studies on LRTCs have identified a number of cases where the development of LRTCs may or may not require specific measures (e.g. restricted blocks, stringent BEMs). Such cases are in particular related to the co-existence (adjacent frequency and/or co-frequency interference) of high-power and low-power networks, and to the adjacent frequency operation of FDD and TDD networks within the same geographical area.

This has led to the following classification of applications:

- ECN’s Downlink (DL) to receivers with high directionality and/or high gain (eg, fixed). For such applications, the corresponding target received field strength is low. None of the applications studied in this report are targeting only this kind of deployment scenario.

- ECN’s DL to mobile receivers with low directionality and/or low gain (eg, handheld devices for on-the-move, nomadic or stationary use). For such applications, the corresponding target received field strength is high enough to enable reception at ground level to receivers with low gain antennas. This category includes terrestrial broadcasting, downlink of Mobile broadband, Mobile multimedia downlink (MMD), satellite-enabled services (satellite component and downlink of the terrestrial component, if applicable) and downlink of Public Protection and Disaster Relief (PPDR).

- ECN’s Uplink (UL) for low power applications (eg, handheld devices for on-the-move, nomadic or stationary use). This category includes uplink of mobile broadband (if applicable), Programme Making and Special Events (PMSE), uplink of the terrestrial component of satellite-enabled services (if applicable) and uplink of Public Protection and Disaster Relief (PPDR).

Figure: ECN’s DL to fixed receivers and 	ECN’s DL to mobile receivers result in different LRTCs.

Figure: In case of duplex communications, two sets of LRTCs are required. One for DL and one for UL.

## Reference systems

In order to establish compatibility criteria for systems operating in the band, it may be necessary to make some assumptions about likely systems in the band.. The expression of minimum technical restrictions will be linked to a given set of assumptions generally identified through a market analysis.

For each band (or sub-band) one or more reference systems can be identified, based on the market analysis of most likely systems for that band (typically through research and consultation with interested parties).

As noted in CEPT Report 019, the technical analysis is valid under the assumption derived from the market analysis, based on the information available at the time. That means that if an application or use develops and becomes significantly different from the assumed application/technology, then the initially determined technical rules to limit interference may not be appropriate for that application/technology. In other words, least restrictive usage conditions are suitable for a given band and a given set of applications. As an example, Commission Decision of 6 May 2010 on harmonised technical conditions of use in the 790-862 MHz frequency band for terrestrial systems capable of providing electronic communications services in the European Union specifies that “harmonised technical conditions will facilitate economies of scale without requiring any type of particular technology to be used, based on optimised parameters for the most likely use of the band” in line with CEPT Reports 29, 30, 31 and 32.

Furthermore, as confirmed for instance in RSPG102, matching market demand to service delineation has always been a challenge to spectrum managers. And, as stated in CEPT Report 019, “In a fast changing environment where new technologies continue to develop and evolve, the most beneficial use may well not be the same as that which has been decided by the regulator as any regulator may have limited and incomplete information on which to base a decision.”

LRTC should therefore attempt to capture as many applications as reasonably possible within a common framework, even if certain applications may appear to be less likely at the time of establishing the conditions. This will contribute to the robustness and longevity of such LRTCs, and their relevance to industry and national administrations.

LRTC usually consist of the combination of:

- A harmonised band plan,

- Generic Block Edge Masks for adjacent frequency use (both within the band and at the band edges), enabling co-existence within a group of applications deployed in the same service area.

- Aggregate PFD or field strength measures enabling co-frequency co-existence within groups of applications deployed in geographically different service areas

- Restricted Block Edge Masks for co-existence between groups of applications.

The coexistence between high power broadcasting networks and low power ECNs has been studied in particular in the CEPT Report 23, 29 and 30 [Editorial, add reference]. The main conclusions are:

- The most difficult coexistence scenario is between low power ECNs UL and low power applications on one hand and high power broadcast to fixed receivers on the other hand.

- Coexistence between low power ECNs DL and high power broadcast networks is mainly governed by the reception mode of the broadcast transmission (see CEPT Report 23 [Editorial, add reference]).