|  |  |  | FM50(12)024 |
| --- | --- | --- | --- |
| FM50 – M4 |  |  |  |
| Mainz, 06-09 March 2012 |  |  |  |
| Date issued: | 28th February 2012 |  |  |
| Source: | Germany |  |  |
| Subject: | Section 4 Compatibility with the current regulatory framework - the Radio Regulations |  |  |
| N Password protection required? (Y/N) |  |  |  |
| Summary: |  |  |  |
| The Project Team shall through comparison of the possible regulatory options assess which future use(s) of the 1452-1492 MHz band would be the most appropriate for CEPT. The comparison of the possible regulatory options shall be based on the five agreed criteria. One of these criteria is ”Compatibility with the current regulatory framework”. The introductory part of working document for section 4 gives a concrete form to this criterion: To evaluate if the application(s) under study is(are) compatible: - with the Radio Regulations (article 5), - with MA02CO07 within 1452-1479.5 MHz. - with ECC DEC(03)02 within 1479.5-1492 MHz. Format of answer on compatibilities: Yes / Yes with conditions / No, If compatible justification needed Need to change/adapt the Maastricht plan? [insert a table] A drafting activity regarding the second bullet point “compatibility of the candidate applications with MA02revCO07” was conducted within the ECO Forum. It is proposed to sum up this issue in a table. The idea is to have a similar table for each application describing the different levels of compatibility with the special arrangement and to import these tables in the relevant sections of the report. The proposal retains the three levels of compatibility with the special arrangement that were identified at the last meeting. A similar situation is given with regard to the compatibility of the candidate applications with the Radio Regulations. |  |  |  |
| Proposal: |  |  |  |
| FM 50 is invited to follow the same nuanced approach like under consideration at the second bullet point (“compatibility with MA02CO07 within 1452-1479.5 MHz”) with regard to the first bullet point (“compatibility with the Radio Regulations (article 5)”): to have similar tables for each application describing the different levels of compatibility with the Radio Regulations and to import these tables in the relevant sections of the report. |  |  |  |
| Background: |  |  |  |

Compatibility with the RR is understood as statement, whether there would be the need to change the provision concerned in case the candidate application would be the future use of the band. The follow up question what kind of changes might be needed (or whether a realisation is possible without changes in the first instance), has to be answered in the future section 6.

Compatibility with the RR is not understood in terms of sharing as the “possibility to combine/share with other applications” is a separate criterion.

As in section 6 the impacts of the different regulatory options will be compared, information about the compatibility with the RR as far as Region 1, 2 and 3 are concerned is relevant; at least under two aspects:

a) to assess in section 6 the “easiness” to realise a change, and

b) to verify the findings of the subsection dealing with the criteria “Potential for economies of scale (need and potential for harmonisation within and outside CEPT)” as this indicates the probability of the claimed “potential for harmonisation … outside CEPT”.

The tables below show that under the current Art. 5 RR for none of the candidate applications full harmonisation would be possible

- neither globally,

- nor CEPT wide:

Terrestrial Broadcast

| Allocations | Changes necessary? |
| --- | --- |
| FIXED |  |
| MOBILE |  |
| Region 1: except aeronautical mobile |  |
| Region 2: 5.343 |  |
| BROADCASTING 5.345 | yes, because 5.345 limits the use to audio, but the candidate application as described in section 3 covers audio and video |
| BROADCASTING-SATELLITE 5.208B 5.345 |  |
| 5.341 |  |
| Region 1: 5.342 | no, but in Armenia, Azerbaijan, Belarus, Bulgaria, the Russian Federation, Uzbekistan, Kyrgystan and Ukraine, the band 1 429-1 535 MHz is also allocated to the aeronautical mobile service on a primary basis exclusively for the purposes of aeronautical telemetry |
| Region 2 & 3 : 5.344 | no, but in the United States, the band 1 452-1 525 MHz is allocated to the fixed and mobile services on a primary basis and whereas in Region 2, the use of the band 1 435-1 535 MHz by the aeronautical mobile service for telemetry has priority over other uses by the mobile service |

Mobile Broadband

| Allocations | Changes necessary? |
| --- | --- |
| FIXED |  |
| MOBILE |  |
| Region 1: except aeronautical mobile |  |
| Region 2: 5.343 | no, but the use of the band 1 435-1 535 MHz by the aeronautical mobile service for telemetry would have priority over other uses by the mobile service |
| BROADCASTING 5.345 |  |
| BROADCASTING-SATELLITE 5.208B 5.345 |  |
| 5.341 |  |
| Region 1: 5.342 | no, but in Armenia, Azerbaijan, Belarus, Bulgaria, the Russian Federation, Uzbekistan, Kyrgystan and Ukraine, the band 1 429-1 535 MHz is also allocated to the aeronautical mobile service on a primary basis exclusively for the purposes of aeronautical telemetry |
| Region 2 & 3 : 5.344 | no, but in the United States, the band 1 452-1 525 MHz is allocated to the fixed and mobile services on a primary basis and whereas in Region 2, the use of the band 1 435-1 535 MHz by the aeronautical mobile service for telemetry has priority over other uses by the mobile service |

MMD

| Allocations | Changes necessary? |
| --- | --- |
| FIXED |  |
| MOBILE |  |
| Region 1: except aeronautical mobile |  |
| Region 2: 5.343 | no, the use of the band 1 435-1 535 MHz no, but the use of the band 1 435-1 535 MHz by the aeronautical mobile service for telemetry would have priority over other uses by the mobile service |
| BROADCASTING 5.345 |  |
| BROADCASTING-SATELLITE 5.208B 5.345 |  |
| 5.341 |  |
| Region 1: 5.342 | no, but in Armenia, Azerbaijan, Belarus, Bulgaria, the Russian Federation, Uzbekistan, Kyrgystan and Ukraine, the band 1 429-1 535 MHz is also allocated to the aeronautical mobile service on a primary basis exclusively for the purposes of aeronautical telemetry |
| Region 2 & 3 : 5.344 | no, but in the United States, the band 1 452-1 525 MHz is allocated to the fixed and mobile services on a primary basis and whereas in Region 2, the use of the band 1 435-1 535 MHz by the aeronautical mobile service for telemetry has priority over other uses by the mobile service |

Satellite-enabled Services

| Allocations | Changes necessary? |
| --- | --- |
| FIXED |  |
| MOBILE | yes, an additional allocation to the mobile-satellite service would be needed, see Art. 1.25 RR |
| Region 1: except aeronautical mobile |  |
| Region 2: 5.343 |  |
| BROADCASTING 5.345 |  |
| BROADCASTING-SATELLITE 5.208B 5.345 | yes, as far as not only broadcast audio is planed, because 5.345 limits the allocation to audio |
| 5.341 |  |
| Region 1: 5.342 | no, but in Armenia, Azerbaijan, Belarus, Bulgaria, the Russian Federation, Uzbekistan, Kyrgystan and Ukraine, the band 1 429-1 535 MHz is also allocated to the aeronautical mobile service on a primary basis exclusively for the purposes of aeronautical telemetry |
| Region 2 & 3 : 5.344 | no, but in the United States, the band 1 452-1 525 MHz is allocated to the fixed and mobile services on a primary basis and whereas in Region 2, the use of the band 1 435-1 535 MHz by the aeronautical mobile service for telemetry has priority over other uses by the mobile service |

PMSE

| Allocations | Changes necessary? |
| --- | --- |
| FIXED |  |
| MOBILE |  |
| Region 1: except aeronautical mobile |  |
| Region 2: 5.343 | no, although the use of the band 1 435-1 535 MHz by the aeronautical mobile service for telemetry would have priority over other uses by the mobile service harmonisation could be achieved, because PMSE would be a secondary service, see RR Art. 5.28-5.30 |
| BROADCASTING 5.345 |  |
| BROADCASTING-SATELLITE 5.208B 5.345 |  |
| 5.341 |  |
| Region 1: 5.342 | no, but in Armenia, Azerbaijan, Belarus, Bulgaria, the Russian Federation, Uzbekistan, Kyrgystan and Ukraine, the band 1 429-1 535 MHz is also allocated to the aeronautical mobile service on a primary basis exclusively for the purposes of aeronautical telemetry |
| Region 2 & 3 : 5.344 | no, although in the United States, the band 1 452-1 525 MHz is allocated to the fixed and mobile services on a primary basis and whereas in Region 2, the use of the band 1 435-1 535 MHz by the aeronautical mobile service for telemetry has priority over other uses by the mobile service, harmonisation could be achieved, because PMSE would be a secondary service, see RR Art. 5.28-5.30 |

PPDR

| Allocations | Changes necessary? |
| --- | --- |
| FIXED |  |
| MOBILE |  |
| Region 1: except aeronautical mobile |  |
| Region 2: 5.343 | no, but the use of the band 1 435-1 535 MHz by the aeronautical mobile service for telemetry would have priority over other uses by the mobile service |
| BROADCASTING 5.345 |  |
| BROADCASTING-SATELLITE 5.208B 5.345 |  |
| 5.341 |  |
| Region 1: 5.342 | no, but in Armenia, Azerbaijan, Belarus, Bulgaria, the Russian Federation, Uzbekistan, Kyrgystan and Ukraine, the band 1 429-1 535 MHz is also allocated to the aeronautical mobile service on a primary basis exclusively for the purposes of aeronautical telemetry |
| Region 2 & 3 : 5.344 | no, but in the United States, the band 1 452-1 525 MHz is allocated to the fixed and mobile services on a primary basis and whereas in Region 2, the use of the band 1 435-1 535 MHz by the aeronautical mobile service for telemetry has priority over other uses by the mobile service |

BDA2GC

| Allocations | Changes necessary? |
| --- | --- |
| FIXED |  |
| MOBILE |  |
| Region 1: except aeronautical mobile | yes, but than conformity with Region 2 & 3 would be achived |
| Region 2: 5.343 | no, but the use of the band 1 435-1 535 MHz by the aeronautical mobile service for telemetry would have priority over other uses by the mobile service |
| BROADCASTING 5.345 |  |
| BROADCASTING-SATELLITE 5.208B 5.345 |  |
| 5.341 |  |
| Region 1: 5.342 | no, but in Armenia, Azerbaijan, Belarus, Bulgaria, the Russian Federation, Uzbekistan, Kyrgystan and Ukraine, the band 1 429-1 535 MHz is also allocated to the aeronautical mobile service on a primary basis exclusively for the purposes of aeronautical telemetry |
| Region 2 & 3 : 5.344 | no, but in the United States, the band 1 452-1 525 MHz is allocated to the fixed and mobile services on a primary basis and whereas in Region 2, the use of the band 1 435-1 535 MHz by the aeronautical mobile service for telemetry has priority over other uses by the mobile service |

ANNEX I

Extract of the Radio Regulations Article 5 for 1452-1492 MHz

| Allocation to services |  |  |  |
| --- | --- | --- | --- |
| Region 1 | Region 2 |  | Region 3 |
| 1 452-1 492 FIXED MOBILE except aeronautical mobile BROADCASTING 5.345 BROADCASTING-SATELLITE 5.208B 5.345 5.341 5.342 |  | 1 452-1 492 FIXED MOBILE 5.343 BROADCASTING 5.345 BROADCASTING-SATELLITE 5.208B 5.345 5.341 5.344 |  |

5.208B*	In the bands:

137-138 MHz,
	387-390 MHz,
	400.15-401 MHz,
	1 452-1 492 MHz,
	1 525-1 610 MHz,
	1 613.8-1 626.5 MHz,
	2 655-2 690 MHz,
	21.4-22 GHz,

Resolution 739 (Rev.WRC-07) applies.     (WRC-07)

5.341	In the bands 1 400-1 727 MHz, 101-120 GHz and 197-220 GHz, passive research is being conducted by some countries in a programme for the search for intentional emissions of extraterrestrial origin.

5.342	Additional allocation:  in Armenia, Azerbaijan, Belarus, Bulgaria, the Russian Federation, Uzbekistan, Kyrgystan and Ukraine, the band 1 429-1 535 MHz is also allocated to the aeronautical mobile service on a primary basis exclusively for the purposes of aeronautical telemetry within the national territory. As of 1 April 2007, the use of the band 1 452-1 492 MHz is subject to agreement between the administrations concerned. (WRC-2000)

5.343	In Region 2, the use of the band 1 435-1 535 MHz by the aeronautical mobile service for telemetry has priority over other uses by the mobile service.

5.344	Alternative allocation:  in the United States, the band 1 452-1 525 MHz is allocated to the fixed and mobile services on a primary basis (see also No. 5.343).

5.345	Use of the band 1 452-1 492 MHz by the broadcasting-satellite service, and by the broadcasting service, is limited to digital audio broadcasting and is subject to the provisions of Resolution 528 (WARC-92) *.

See FM50(11)041 Annex 4 ”Agreed outline of the IA”.

FM50(11)078 Annex 8

1.25		mobile-satellite service:  A radiocommunication service:

–	between mobile earth stations and one or more space stations, or between space stations used by this service; or

–	between mobile earth stations by means of one or more space stations.

This service may also include feeder links necessary for its operation.

5.28		3)	Stations of a secondary service:

5.29	a)	shall not cause harmful interference to stations of primary services to which frequencies are already assigned or to which frequencies may be assigned at a later date;

5.30	b)	cannot claim protection from harmful interference from stations of a primary service to which frequencies are already assigned or may be assigned at a later date;

* This provision was previously numbered as No. 5.347A. It was renumbered to preserve the sequential order.

* 	Note by the Secretariat:  This Resolution was revised by WRC-03.