Criteria 4 summary

|  | Terrestrial broadcasting |  |
| --- | --- | --- |
| Band designation | Existing now MA02revCO07 |  |
| Technical conditions | Existing now MA02revCO07 |  |
| Licensing | Licensing procedure could take up to 12 month, depending on national circumstances, in some countries the band is already licensed for Terrestrial broadcasting |  |
| Standardisation | Existing now (SDR family of standards and DAB/DAB+/DMB family of standards) Status of DAB/DAB+/DMB The most important ETSI specifications are: ETSI EN300401: Radio Broadcasting Systems; Digital Audio Broadcasting (DAB) to mobile, portable and fixed receivers ETSI TS102428: Digital Audio Broadcasting (DAB); DMB video service; User Application Specification ETSI TS102563: Digital Audio Broadcasting (DAB); Transport of Advanced Audio Coding (AAC) audio ETSI’s SDR (Satellite Digital Radio) standard stands among the possible technologies that may be used for the deployment of subscription radio services in the L band. As designed, the SDR standard enables the deployment of autonomous terrestrial radio networks, with a view to their possible extension to reach national coverage through companion satellite components. It also enables the broadcast, within a same 1.7 MHz terrestrial multiplex, of different categories of independent broadcast services, e.g. enabling a mix of linear and non-linear audio services aimed to compose an offer. This standard is described in the following documentation: ETSI - TR 102 525: Functionalities, architecture and technologies ETSI - TS 102 550: Outer Physical Layer of the Radio Interface ETSI - TS 102 551: Inner Physical Layer of the Radio Interface Part 1 (TS 102 551-1): Single carrier transmission Part 2 (TS 102 551-2): Multiple carrier transmission A number of European manufacturers have already developed broadcast and receiver equipment implementing the SDR standard, which are therefore ready for large scale production and deployment. These would include a series of digital radio receiver chipsets, as well as a range of digital radio broadcast tower transmitters. No further standardisation work is required before implementing terrestrial broadcasting services. |
| Equipment | Existing now A number of receiver models have been developed and are potentially available on any market where the services are to be implemented. The receivers normally incorporate both the band III and the L-band. ] |
| Operational deployment | Depends on commercial opportunities, may vary from country to country, few existing operational deployments even though the regulatory framework is available |

|  | Mobile broadband No plans to provide further information at this stage |
| --- | --- |
| Band designation | End of 2012 |
| Technical conditions | LRTC by the end of 2013 |
| Licensing | X month after band designation, depending on national circumstances |
| Standardisation |  |
| Equipment |  |
| Operational deployment |  |

|  | Mobile SDL |  |  |
| --- | --- | --- | --- |
| Band designation | End of 2012 |  |  |
| Technical conditions | LRTC by the end of 2013 This requires a Decision by the ECC defining the LRTC for the band 1452-1492 MHz, based on a harmonised band plan of 8 blocks of 5 MHz each with associated Block Edge Mask (BEM). |  |  |
| Licensing | Licensing procedure could take up to 24 months after band designation, depending on national circumstances (taking into account existing licenses) Taking into account that the band is already available in a number of countries, administrations can start licensing the band for mobile use as soon as the ECC Decision on LRTC is adopted and subject to theirnational market demand. |
| Standardisation | 3 GPP standardization for the band is expected to be completed by the end of 2013. The 3GPP specifications already contain almost all the capability that would be required to combine operation on the current European mobile paired FDD band with a SDL at 1452-1492 MHz. The key things those specifications would need to cover are the capability to aggregate downlink channels across bands (to create greater bandwidth). This is already supported in standards for UMTS from Release 9 (HSPA+) and for LTE from Release 10 (LTE-Advanced). Each specific band combination would be defined in 3GPP. Likewise channel management, power control, quality management and handover are all within the capability of the existing standards specifications. The Work Items (WI) for similar arrangements in the 700 MHz band in the USA have already been approved in the 3GPP (see RP-110435). The work on this WI is already ongoing and is planned to be completed in September 2012. A new Work tem in 3GPP would be required for a 1452 – 1492 MHz SDL. However the current AT&T will cover the technical issues that will need to be addressed. Once a decision on the harmonised use of the band is taken by the CEPT/ECC end of 2012, a request can be made to 3GPP to undertake the necessary work to include the 1452 – 1492 MHz SDL in the specifications for HSPA+ from Release 9, as well as for LTE from Release 10, i.e. the first LTE release which supports multiple carrier aggregation capabilities. This work will be undertaken in parallel to the development of the harmonized LRTC by the ECC similar to previous work undertaken (e.g. UMTS900, 3.4 GHz). Based on experience with progressing specifications in 3GPP for AT&T’s WI, the 1452 – 1492 MHz WI could be completed by the end of 2013 in 3GPP subject to the approval of the harmonised LRTC for the band by the ECC. |
| Equipment | Chipsets are planned to become available in 2013 to enable commercial devices in 2014. Base stations are planned to be available in 2014. HSPA+ Release 9 (capable of supporting dual band and dual carrier) and LTE Release 10 (capable of supporting carrier aggregation) chipset solutions are already in the planning stage within the mobile broadband industry. Those chipsets are planned to become available in 2013 to enable commercial devices in 2014. This development would happen in parallel with the work in 3GPP. Adding requirements for an SDL in the band 1452 – 1492 MHz does not impact the timeline for availability of those Release 9 HSPA+ and Release 10 LTE capable chipsets since a solution of the former entirely leverages the architecture of the latter. RF band support is a Release-independent feature in 3GPP, hence the 1452 – 1492 MHz band can be supported by Release 9 HSPA+ and Release 10 LTE devices although the band 1452 – 1492 MHz numerology and conformance test specifications will be part of a later 3GPP release. In summary, the timeline for availability of chipsets with a 1452 – 1492 MHz SDL capability will depend primarily on the timeline for adoption by the ECC of the LRTC in the band. This would enable commercial devices in 2014, subject to the adoption by the ECC of the harmonized LRTC in the band in 2013. Manufacturers will design, test and build Base Stations and other network equipment supporting the 1452 – 1492 MHz band. It is expected that this process will take 12 months leading to availability of equipment in 2014, subject to the adoption by the ECC of the harmonized LRTC in the band in 2013. |
| Operational deployment | Operational deployment can start in 2014 - The timeline shown below highlights the various interrelated regulatory and market steps for the deployment of Mobile SDL in the 1452-1492 MHz band starting in 2014 in Europe: |  |

|  | Satellite audio broadcasting |  |
| --- | --- | --- |
| Band designation | Existing for satellite audio broadcasting within 1479.5-1492 MHz, ECC DEC(03)02 Alternatively within the option of using the whole 1467-1492 MHz band the timeframe depends on the establishment of a possible new ECC Decision including a new regulatory framework for satellite audio broadcasting. It is possible that this process will be completed by the end of 2012; |
| Technical conditions | Assuming that S-DAB would be deployed under the Existing framework there is no need for new technical conditions, If SDAB want to use the whole 1467-1492 MHz band then the associated technical specifications and regulations would be available by the end of 2013. |
| Licensing | [Already licensed in some CEPT countries] |  |
| Standardisation | [Existing There is no further standardisation work needed before possible implementation of Satellite Broadcasting;] |  |
| Equipment | [Terminal equipment is existing ? A number of European manufacturers have already developed broadcast and receiver equipment for this application, which are therefore ready for large scale production and deployment, including a series of digital radio receiver chipsets. As demonstrated by the digitization of TV services, digital satellite can play an important role enabling the deployment of new digital technologies and services. Equipment supplies would have to be confirmed and ramped up. It is expected that this process will take at least 12 – 18 months, suggesting potential operational deployment in 2013/14;] |  |
| Operational deployment | [depending on the availability of a new satellite A new satellite supporting the 1.4 GHz band can be deployed and in commercial operation within approx. 28 – 32 months from order, ie in the second or third quarter of 2015 assuming a decision towards the end of 2012. Potential operational deployment in 2013/14 (existing satellite or temporarily terrestrial-only ahead of satellite launch during 2015)] |  |

|  | PMSE |  |
| --- | --- | --- |
| Band designation | End of 2012 |  |
| Technical conditions | LRTC by the end of 2013 |  |
| Licensing | Up to 6 months after technical conditions are available, depending on national circumstances (some administrations have allocated PMSE on a secondary basis in parts of the 1452-1492 MHz band according to their national frequency allocation table) |
| Standardisation | The ETSI Standard EN 300 422 for wireless microphones was published in 2008.:. For applications not included in EN 300 422 such as audio conference systems (see section 3.5) standardization work would be required and could take approximately one year. For the existing standard to comply with the new LRTC, standardization work could be required and could take approximately one year after the completion of the new LRTC. |  |
| Equipment | PMSE wireless microphones devices for operation in the L-Band will be available 6 month after band licensing (if EN 300 422 does not need to be revised to take into account new LRTC). . Research and development was done for the 1.8GHz spectrum and devices are on the market. PMSE manufacturers have invested into the development of equipment noting that in some countries parts of the 1452-1492 MHz band have been authorized for PMSE on a secondary basis according to their national frequency allocation table. There is an existing SRdoc since Jan 2007 (TR 102546) requesting spectrum for PMSE. |
| Operational deployment | Operational deployment can start in 2014 |  |

|  | PPDR |  |
| --- | --- | --- |
| Band designation | End of 2012 |  |
| Technical conditions | LRTC by the end of 2013 |  |
| Licensing | Up to six month after LRTC are available, depending on national circumstances |  |
| Standardisation | Approximately by the end of 2013 Usually, if broadly supported by stakeholders, standardisation can take 24 to 36 month. Under the assumption that LTE technology will be used by PPDR for temporary and local use the basic technology is available already today. The on-going standardisation activities would be limited to the additional development of the PPDR safety requirements. This includes the adoption of already existing requirements (e.g. from TETRA standard) and its implementation into the existing LTE standard. Assuming an overall support by the parties concerned, this will take only a few months. 3GPP Group SA (Service Aspects) is currently discussing a proposal issued by the US and Canada to consider the implementation of PPDR requirements into a LTE standard. There seems consensus on the possibility but the focus of the discussions is on the direct mode and its impact to the public availability of mass market TE. The usage needs to be limited to PPDR forces only, since otherwise public TE, equipped with the same chipset, would be able to listen to the DMO transmissions in the related local area. |
| Equipment | Would be available shortly after standardization process. Once standardisation activities are completed, network and user equipment will have to be developed and deployed. The needed time will vary on the intention of the Administrations, their considerations on urgency and available money. A dedicated estimation is therefore hardly possible. The time until related terminal equipment (TE) and infrastructure is available, depends on the willingness of CEPT Administrations and industry to agree on an appropriate standardisation measure (e.g. within ETSI) and the time needed by stakeholders and industry to adopt the related measure. The choice of standardised equipment will enhance its availability from every manufacturer throughout Europe and minimise therefore time for deployment and provision. However, as of today, there is no agreement between CEPT administrations and in the industry on a standardization measure, thus it is not possible to estimate when related equipment could be made available. |
| Operational deployment | Operational deployment can start in 2014. The operational availability of broadband ad hoc/temporary networks depends on national needs and public finance constraints. |

|  | BDA2GC |  |
| --- | --- | --- |
| Band designation | After the conclusion of FM50 work and a decision by the ECC about the most appropriate future use of the band 1452-1492 MHz which is expected to happen in October 2012, at least one WRC (WRC 15) would be needed to change the Radio Regulations (suppression of “except aeronautical mobile”). |
| Technical conditions | LRTC by the end of 2013 |  |
| Licensing | Licensing procedure could take up to 24 month after band designation taking into account that a pan-European authorization is needed. |  |
| Standardisation | based on 3GPP LTE standard: - GS corresponds to eNodeB in 3GPP LTE specifications. - AS corresponds to UE in 3GPP LTE specifications, with BDA2GC specific enhancements (increased power, Doppler shift compensation, etc.). - No need to develop a harmonized standard. The access network of the BDA2GC system consists of a terrestrial mobile radio access network. The baseline access technology is the 3GPP evolved packet system (E-UTRAN+EPC) that is already commercially available (Release 8, known as LTE), and higher releases, i.e. the ground station equipment is in conformity with 3GPP specifications. The RF unit would need to be adapted to the operational frequency. BDA2GC specific enhancements required correspond mainly to operational aspects, e.g. antenna up-tilt at the Ground Station (Base Station), and network management issues. These modifications of ground stations do not require standardisation efforts. The rollout of BDA2GC will start with about 200 to 250 ground stations, covering of the European area with higher density air traffic, followed by a successively development to enlarge coverage and density in very high traffic areas (see Figure). In the final state, the ground network covering Europe will encompass 400 to 500 ground stations. The addressable market in Europe for the onboard unit is about 4500 commercial aircraft, and about the same number of business aircraft. For this number of aircraft stations no 3GPP standardisation activities are expected to take place. |
| Equipment | Adaptations of the ground components are expected to be completed ten months after spectrum licensing. The aircraft station acts as a mobile station (MS) in the BDA2GC system. Compared to a MS used in a usual mobile network, increased output power is required due to the higher path loss (larger cell sizes) in the BDA2GC network. Also improved synchronization features, e.g. to compensate the high Doppler shift, will be implemented. Time necessary for manufacturing the specific aircraft installation kits and for the corresponding type approval at aviation authorities is estimated to be about one and a half year. Aircraft assembly will take place during maintenance or overnight stop. |  |
| Operational deployment | Operational deployment can start in 2018 [subject to the result the next WRC]. The build-up of the ground network is assumed to last about one year, but backhaul and site preparations could be started in parallel with the adaptations of the ground components. The service rollout with first aircraft can be expected about 18 months after spectrum licensing. The estimated timeline for equipment availability for a LTE-based BDA2GC system is illustrated in the Figure below: |

A complete set of relevant specifications is available from ETSI as well as from the WorldDMB (http://www.worlddab.org/introduction_to_digital_broadcasting/standards_specs)