	[image: image1.wmf]ECC

Electronic Communications Committee

CEPT


	
	FM50(12)057

	7th Meeting of FM50

	22-25 May 2012, London, United Kingdom
	

	Date issued:
16th May 2012
Source: France
Subject: French position on the use of the 1452-1492 MHz band

	Password protected:
yes
	
	
No
	x


	Summary

France supports the regulatory Option 2 of section 5 of the draft ECC Report developed by FM50 and informs FM50 that it intends to use, at the national level, part of the 1452-1492 MHz for terrestrial digital radio and the other part for Mobile Supplemental Downlink (SDL).
Also, France proposes some initial elements for a future ECC Decision on the 1452-1492 MHz band so as to implement the regulatory Option 2.


	Proposal

FM PT 50 is invited to note the French position with regard to the future use of the 1452-1492 MHz band and to take due account of the proposed elements of a future ECC Decision on the harmonised used of the 1452-1492 MHz band.


	Background



1. French position about the European harmonised future use of the 1452-1492 MHz band:
France supports the Regulatory Option 2 as described in section 5 of the draft ECC Report (doc. FM50(12)044).

As already described in doc FM50(11)026, a process of authorization of terrestrial digital radio in the 1452-1492 MHz band has started before FM PT50 set up and is currently under way. It is expected that the process will lead to the granting of authorisation to one digital radio distributor in 2012 with frequencies based on the existing allotments of the MA02revCO07 Plan. In that perspective, based on CEPT Regulatory Option 2, France intends to use part of the 1452-1492 MHz for terrestrial digital radio and the other part for Mobile Supplemental Downlink (SDL).
In addition, France supports the Regulatory Option 2 with the following understandings:
· the ECC should develop and adopt an ECC Decision designating the band for MFCN and defining the Least Restrictive Technical Conditions with a harmonised band plan for the 1452-1492 MHz band, based on 8 blocks of 5 MHz with associated generic BEM and restricted BEM (for coexistence between uplink and downlink blocks) allowing other uses of the band to meet national requirements;
· the MA02revCO07 should be retained for cross-border coordination bilateral agreements in the sub-band 1452-1479.5 MHz, between Terrestrial Broadcasting (for countries wishing to deploy terrestrial broadcasting in part of the band) and mobile, similarly as the framework developed by CEPT for the 800 MHz band with GE06 and ECC DEC/(09)03;

· an administration wishing to implement terrestrial digital sound broadcasting networks in part of the 1452-1492 MHz may choose to do so using the MA02revCO07 Plan and its possible modifications;

· an ECC Recommendation should be developed with regard to mobile to mobile cross-border coordination in the band 1452 – 1492 MHz as for other mobile tot mobile cross-border coordination in other bands;
· the ECC/DEC/(03)02 Decision will need to be suppressed;
· an agenda item to introduce pfd limits to be included in Article 21 of the RR should be submitted at the next WRC. The objective is to protect MFCN use within CEPT from harmful interference from satellite use outside CEPT.
It should be noted that:

· MFCN is proposed instead of TRA-ECS as this latter is too vague to ensure a proper harmonisation at the CEPT level and for consistency with other ECC Decisions designating bands for harmonised use by mobile in CEPT;

· It is proposed to introduce the pfd limits in RR Article 21 at the WRC immediately following WRC-15 so as to have enough hindsight on the SDL deployment. In the meantime, Administrations will have the possibility (and should be encouraged) to register terrestrial stations of the SDL mobile service in the MIFR in order to ensure protection of MFCN from satellite interference through the ITU coordination procedure.
2. Elements for an ECC Decision for the harmonised use of the 1452-1492 MHz band:
France considers that the following elements, based on the approach adopted in ECC/DEC/(09)03, should be included in a future ECC Decision for the harmonised use of the 1452-1492 MHz band:
“CONSIDERING

x) that administrations wishing to implement terrestrial digital sound broadcasting networks in part of the 1452-1492 MHz may choose to do so using the MA02revCO07 Plan and its possible modifications, based on a 1.7 MHz channel bandwidth;
y) that the Maastricht Arrangement MA02revCO07 provides the necessary regulatory procedures for cross-border coordination between administrations having to coordinate terrestrial digital sound broadcasting networks in one country and mobile service in another country;”

and:

“DECIDES

1. that the frequency band 1452-1492 MHz is designated to mobile/fixed supplemental downlink communications networks;

2. that the preferred harmonised frequency arrangement for the 1452-1492 MHz is given in Annex 1;
[Editorial note: include in Annex 1 a band plan formed of 8 blocks, 5 MHz each, downlink (base station to mobile terminals)];
3. that administrations implementing mobile/fixed supplemental downlink communications networks, in accordance with Decides 1 and 2, shall adopt the common and minimal (least restrictive) technical conditions specified in Annex 2 to this Decision;”
