| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 1A/TEMP/113(edited) Subject: Question ITU-R 221/1, Report ITU-R SM.2158 | Annex 2 to Document 1A/311-E |
|  | 12 July 2010 |
|  | English only |
| Annex 2 to Working Party 1A Chairman’s Report |  |
| Working Document towards a PRELIMINARY Draft Modification of Report ITU-R SM.2158 Impact of power line telecommunication systems on radiocommunication systems operating in the LF, MF, HF and VHF bands below 80 MHz |  |

Working document towards a preliminary draft modification of Section 2 of Report ITU-R SM.2158, as follows:

# 2	Characteristics of radio frequency emission from PLT Systems

## 2.1	Radiation sources in a PLT system

Household power lines consist of two or three conducting wires, that is, live, neutral, and earth wires, where AC electric power is carried by the live and neutral wires. Similarly, in a PLT system in domestic use, signal power is fed into the live and neutral wires by PLT equipment (modem), and the HF signal current in each wire is intended to be equal in magnitude and opposite in the directions. In most cases, however, the currents in two wires have components flowing in the same direction. Those in-phase components behave like so-called antenna currents that become primary sources of the unwanted radiation from the PLT system.

Similarly, in distribution networks, in-phase HF current components in power line conductors can be regarded as primary radiation sources, if the separation distance between the conductors is much less than the wavelength of the PLT signals.

### 2.1.1	Differential-mode and common-mode currents

In general, PLT signal currents in two power line conductors are intended to be equal in magnitude and flow in the opposite directions to each other. This fundamental current mode is referred to as various technical terms in the transmission line theory, for example, differential-mode, symmetric-mode, balanced-mode, and transverse-mode. However, if the signal source, power lines, or load are not electrically balanced with respect to the ground and nearby objects or power line wires are geometrically unparallel, the HF currents in the line conductors have components flowing in the same direction. This in-phase current mode is called common-mode, asymmetric-mode, or longitudinal-mode. Thus, the PLT signal current in each conductor can be expressed as a vector sum of differential- and common-mode components, i.e. Id and Ic, as shown in Fig. 2-1 (a). These two mode currents propagate independently along the power lines if they are balanced. However they are coupled at unbalanced elements on the power line network.

Since differential-mode PLT currents on two closely-aligned conductors flow in opposite directions, generated electromagnetic fields can be cancelled out, resulting in no significant field at positions distant from the power lines. In contrast, the common-mode PLT currents may form loop currents as depicted in Fig. 2-1 (a), producing electromagnetic fields, especially in the MF/HF ranges. At HF and much higher frequency ranges, they may radiate electromagnetic waves in a similar way to monopole antennas or folded dipole antennas. Thus, the common-mode currents are considered to be the primary radiation sources in the PLT system. Therefore it is very important to fully describe the physical generation mechanisms of the common-mode currents on the power line network.

The international standard CISPR 22 ed. 6.0 (2008) therefore requires to limit the differential-mode and common-mode currents flowing on the power lines by the limits for the terminal voltages of the mains port of an IT equipment under the specified load conditions (i.e., an artificial mains network (AMN), an asymmetric artificial network (AAN) or an impedance stabilization network (ISN)).

### 2.1.2	Generation of the common-mode PLT current

PLT signal currents in the differential mode (DM) may be transformed into common-mode (CM) currents by two different mechanisms. One is caused by the imbalance in the electromotive force and impedance of a PLT modem. The other is caused by the imbalance in the power lines. The imbalance in the power line network includes (i) the unbalanced load connected to an outlet, (ii) the switch branch which may consist of ceiling lamp(s) and single-pole wall switch, and (iii) singly grounded service wire in some countries, as shown in Fig. 2-2.

### 2.1.3	Common-mode current flowing on power lines

Since the lengths of power lines are comparable with a wavelength in the HF band, the differential-mode and common-mode currents must be treated by means of the distributed constant circuit or the transmission line theory. However, an equivalent circuit illustrated in Fig. 2-1 (b) can be applicable at position x and it yields the following expression for the common-mode current:

,	(2-1)

where the differential-mode and common-mode impedances of the PLT system are:

and  ,	(2-2)

respectively, with .

From these equations, it is found that the common-mode currents are induced from the differential-mode signal currents because of imbalance in the PLT system: imbalance in the power lines, imbalance in the PLT modem (electromotive force,  and , and source impedances,  and ), and imbalance in the connected loads,  and .

Figure 2-1 (b) may hold in general in any position along the power-line. According to the transmission line theory, Z1, Z2, and Z3 in equation (2-1) periodically change with x. Hence, the voltage and current in each mode vary with the observation point resulting in standing-wave patterns as illustrated in Figure 2-3. The standing-wave pattern widely varies depending on the characteristics of power lines and their layouts, and the characteristics of the connected PLT modems and loads.

FIGURE 2-1

Transmission line model of a PLT system and its equivalent circuit

ZS1(x)

ZS2(x)

e1(x)

e2(x)

+

+

ZL3(x)

ZL1(x)

ZL2(x)

ZS3(x)

(b) Equivalent circuit model

ZS3

ZL3

ZL1

ZL2

ZS1

ZS2

Earth wire, ground, and nearby objects

PLT

modem

x=0

x=L

Load

x=0

e1

e2

+

+

Power line

(a) Transmission line model

Differential mode

Commonl mode

FIGURE 2-2

Common-mode currents on the power line network

FIGURE 2-3

Example of standing-wave patterns of the voltage and
current in each mode along a power-line

### 2.1.4	Electrical characteristics of in-house power lines

As described in the previous paragraph, the voltage and current in each mode change with the observation point along the power-line. In addition, they are easily affected by the characteristics of power lines and their layouts, and by the characteristics of the connected PLT modems and loads. Accordingly, the common- mode and differential-mode impedances and the longitudinal conversion loss measured at wall sockets may not represent the electrical characteristics and potential emissions of the entire in-house power lines.  However, if a lot of measurement results is collected at many outlets in various houses for various frequencies and treated in a statistical way, it may provide useful information on the characteristics of the in-house power lines.

As explained in previous paragraphs, unwanted radiation from PLT systems is usually caused by common-mode currents that are transformed from signal currents (differential mode) in power lines. Thus, the characteristics of the power lines, such as common-/differential-mode impedances and electrical balance, are key factors for analysing the PLT radiation. A large number of measurements were therefore made at wall sockets in various houses including wooden houses and reinforced concrete flats in Japan.

### 2.1.4.1	Impedances of in-house power lines measured at outlets1

As implied by equation (2-2), differential-mode and common-mode impedances of actual power lines vary widely with the measurement frequency and time as well as position. In addition, they are seriously affected by household appliances and other electrical/electronic equipment connected to the power lines. Therefore, the impedance characteristics have to be treated on a statistical basis.

Figure 2-4 shows the differential-mode impedance of power lines measured at various wall sockets in various houses. From this figure, it is found that, in many cases, the differential-mode impedances of power lines are around 100 . This measurement result agrees well with the CISPR 16-1-2 ed. 1.2 (2006) specifications for the load (i.e. artificial mains network) used in equipment compliance tests.

Figure 2-5 also gives the common-mode impedance measured at many wall sockets. It is evident that the common-mode impedances are usually greater than 100 . However, CISPR 16-1-2 specifies the common-mode impedance of the test load to be equal to 25 , because such low impedance can emphasize the imbalance characteristics of equipment under test (EUT) as deduced from equation (2-1).

### 2.1.4.2	Imbalance of in-house power lines measured at outlets1

Figure 2-6 shows data on LCL values measured at a number of wall sockets of various houses in Japan. The LCL (longitudinal conversion loss) is a parameter representing imbalance of a parallel line system, defined by the ratio of the applied common-mode voltage to the differential-mode voltage induced at a multi-terminal port. Well-balanced lines like a telephone unshielded pair cable usually have an LCL greater than 50 dB. The LCL depends on the differential-mode and common-mode impedances seen from the port. Since those impedances of power lines greatly change with time, frequency, and position, actual LCL values also change in a very wide range from 20 dB to 60 dB, as shown in Fig. 2-6.

FIGURE 2-4

Differential mode impedance measured at wall sockets in dwelling houses

FIGURE 2-5

Common-mode impedance measured at wall sockets in dwelling houses

FIGURE 2-6

LCL measured at wall sockets in dwelling houses

### 2.1.5	Folded-dipole antenna effect of the switch branch

There are many branch circuits connected in parallel with backbone power lines in houses and buildings. At some specific frequencies where the length of a branch approaches half a wavelength, the branch circuit behaves like a folded dipole antenna as illustrated in Fig. 2-7. Then, the resonant branch radiates electromagnetic waves. The magnitude of the common-mode currents in the branch depends on the length and loads of the branch, the location of the connection points, and the impedances of the backbone lines presented at the connection points. These factors vary from branch to branch, and there are as many switch branches as the number of rooms in a house. If the connecting point is located at the center of a branch with the length of half a wavelength, the maximum antenna current in the folded dipole, Ic, approaches twice as large as a portion of the differential-mode current, Id, that can penetrate into the branch as illustrated in Figure 2-7.

This is close to the worst case scenario for a single switch branch. However since there are many such branches in a house, the aggregated emission from many switch branches must be considered and there is no reason to assume all of them are far away from the worst case scenario. Note that the antenna current in the folded-dipole formed by the switch branch is invisible from the backbone lines and the outlets, and consequently that the LCL measured at the outlet does not include the folded-dipole antenna effect of the switch branches. Therefore the outlet LCL is not a barometer of the antenna currents generated in branch lines.

FIGURE 2-7

Folded dipole antenna formed by a switch branch

Switch

ZL1

Lamp

ZL2

-

DM

CM

DM

CM

DM

CM

DM

### 2.1.6	Leakage from the in-house power line to the service wires outside the house

In-house PLT systems raise serious concern about interference problems caused by leakage of the PLT signals from houses. Since the service wires outside the houses are unshielded, may extend several tens metres in length at the height of nearly ten metres above ground, the common-mode current on the service wires has stronger potential of causing interferences to the radio services in MF and HF bands. Furthermore the service wires are singly grounded at the service transformers, are highly unbalanced and may convert the differential-mode current into the common-mode current quite efficiently in some countries. Therefore the leakage of both the common-mode and the differential-mode currents from the power line network inside the house to the service wires outside the house must be carefully investigated. Since there are diverged data as shown below, further studies would be required.

At the interface between access and in-house power lines, there are power meters, circuit breakers, and distribution circuits that may attenuate the PLT signals. Therefore, a number of measurements were carried out on the differential-mode voltages inside and outside houses to evaluate the insertion loss provided by power network equipment such as distribution circuits1. The results shown in Figure 2-8 demonstrate that such network equipment can suppress the PLT differential-mode signal by more than 20 dB in almost all cases.

FIGURE 2-8

Median
Value

Another example in Figure 2-9 indicates that (i) the differential-mode current measured on the service wires just outside the house is 0 to 30 dB smaller than that measured at the output of the breaker inside the house, (ii) the common-mode current measured on the service wires just outside the house is very close to the differential-mode current measured at the same point, and (iii) the common-mode current measured on the service wires just outside the house is 10 to 30 dB larger than the common-mode current measured at the output of the breaker inside the house. The second observation reflects the fact that the service wires are singly grounded at the service transformer in Japan and the differential-mode current is 100% converted into the common-mode current. The third observation suggests that the radiated emission from the service wires may be 20 to 47 dB stronger than that from the power line inside the house if the shielding effects of the house in Table 2-1 are applied.

FIGURE 2-9

Differential-mode and common-mode currents inside and outside a house

## 2.2	Radiation from a PLT system

### 2.2.1	Equations for the electromagnetic radiation

As explained in 2.1.1, differential-mode PLT currents on two closely-aligned conductors do not produce significant electromagnetic fields at positions distant from the power lines. However, the common-mode PLT currents may generate electromagnetic fields, especially in the MF/HF ranges.

The electromagnetic field radiated from a common-mode current Ic in free space can be evaluated from the vector potential A given by

(2-3)

where Ic is the vector of a segment of the common-mode current flowing on the power lines. The vector R is the vector from the point of the current segment to an observation point in the field and R is its magnitude. Using the vector potential, the electromagnetic field (E, H) radiated from the power lines can be calculated from

(2-4)

(2-5)

The vector potential given by equation (2-3) can apply only in the case of free-space environment. Therefore, for evaluating the leakage field from a house equipped with PLT systems, the shielding effects of the building materials and structures should be taken into consideration.

### 2.2.2	Shielding effectiveness of the exterior walls of a house

Electromagnetic fields radiated from power lines may be shielded to some extent by the exterior walls and ceiling of a house. Hence, numerical analysis using an FI (Finite Integration) code was carried out to investigate the electromagnetic fields of a PLT system leaked from various housings, such as a wooden house and a reinforced concrete one. In this analysis, the shielding effectiveness was defined by the ratio of the maximum field strength at 10 m apart from the power lines not enclosed by a house to that with the power lines enclosed by the house. The results considerably vary with the structure of house, power line layout, and frequency. The mean values of the derived shielding effectiveness are listed in Table 2-1. However these values have not been verified by measurements.

Table 2-1

Shielding effectiveness of the exterior wall of a house

|  | Wooden house | Reinforced concrete house |
| --- | --- | --- |
| 2-10 MHz | 17 dB | 27 dB |
| 10-30 MHz | 10 dB | 27 dB |

Median
Value

____________________

Attachment 1 of “Technical Report on the High Data Rate PLT” issued in Japanese by the Information and Communications Council to the Ministry of Internal Affairs and Communication (MIC), Japan, 2006.

“Measurements of the Radiated Electric Field and the Common Mode Current from the In-house Broadband Power Line Communications in Residential Environment III,” by Mr. Kitagawa and Mr. Ohishi, IEICE Tech. Rep., vol. 107, no. 533, EMCJ2007-117, pp. 1-6, March 2008.

“Effect of structure and materials of building on electromagnetic field s generated by indoor power line communication systems”, by S. Ishigami, K. Gotoh, and Y. Matsumoto, in Proceedings on EMC Europe Workshop 2007, June 2007.