Doc. MARFG(12)INFO 03

WG FM –Maritime Forum Group

2nd. meeting:

Hamburg, 7-8 March 2012.

Date issued:	20.February 2012

Source: 	 Ireland.

Information document

Subject: 	Ireland would like to draw the attention of the Maritime Forum Group members to a man over board type device brought to the attention of the Irish Administration.

Ireland provides this information and asks for any information that other administrations may have on this device.  The device operates outside maritime mobile service frequencies in a telemetry band close to 173MHz.

Details of the device are available on line to the general public, but are alsoprovided at Annex 1 and 2. Below:

_______________________

Annex 1.

SEASAFE MAN-OVERBOARD & LOST-DIVER TRACKING & LOCATION SYSTEM

Technical Specifications

System complies with AS 4268.2 as amended by LIPD (Low Interference Potential Device) 24

August 2005. In particular table ZZ1

Seasafe Transmitter Model: SSTX/01

Frequency 173.4875 MHz

Dimensions 80mm H x 78mm W x 32mm D

Weight (with battery) +/- 250 g

Temperature tolerance -10º C to +55º C

Frequency Stability 15 PPM

Transmitter Power 65 Milliwatts nominal at antenna terminal

Radiated Power Approx 8 Milliwatts

Modulation FM +/- 3 KHz

Tone Frequencies 171 HZ, 195 HZ, 227 HZ

Tone Frequency Interval 125 Milliseconds

Spurious Radiation < 0.1 uW (-40 DBM)

Power requirement 3V Lithium cell

Seasafe Receiver Model: SSRX/01

Frequency 173.4875 MHz

Frequency Stability 15 PPM

Dimensions 256mm L x 130mm W x 60mm D

Weight (with batteries) +/- 850 g

Temperature Tolerance -10º C to +55º C

Decode Sensitivity - 128 DBM nominal

Adjacent channel selectivity 80 DB

Image and Spurious responses -80DB

Spurious Radiation < 57 DBM

Power Requirement 6 VDC or 5 x 1.2V NiMH AA rechargeable batteries

System ‘C-Tick’ Compliance

‘C-Tick’ compliance registration number: N15546

CE Compliance Certification

Annex 2.

SEASAFE MODEL SSRX/01

MAN-OVERBOARD AND LOST DIVER ALARM & TRACKING

RECEIVER

INSTALLATION & OPERATION MANUAL

www.sea-safe.com.au

1

2

Seasafe SSRX/01 Receiver User Manual

Contents Page

1. Technical support 3

2. Warranty 3

3. Regulatory compliance 3

4. Box contents 3/4

5. Receiver Layout and Controls 5

6. Equipment introduction 6

7. Equipment specification 6

8. Installation 6

8.a Receiver 6

8.b Power/auxiliary output cable 7

8.c Omni-directional antenna 8

8.d Collapsible directional-antenna 8

9. System operation 9/10

10. Signal tracking procedure 11

11. Accessories 12

12. Copyright 12

13. Warranty registration 13

1. Technical Support

For any technical questions or for support, please contact your local Seasafe distributor or visit:

Internet: www.sea-safe.com.au

Email: info@sea-safe.com.au

Telephone: +61 7 3390 2287

2. Warranty

Seasafe warrants this product to be free from defects in materials and workmanship for 12 months after the

date of purchase. If your Seasafe Receiver is found to be defective within that time, we will promptly repair or

replace it. This warranty does not cover accidental damage, wear and tear, or consequential or incidental loss.

Warranty registration at www.sea-safe.com.au

3. Regulatory Compliance

This device complies with AS 4268.2 as amended by LIPD (Low Interference Potential Device) 24 August

2005. In particular table ZZ1.

Operation is subject to the following two conditions: (1) This device may not cause harmful

This device must accept any interference received, including any interference that may cause undesired

operation.

a.

b. 1999/5/EC ( ETSI EN 300 220-2, ETSI EN 300 220-1, ETSI EN 301 489

ETSI EN 301 489-3, EN 61000-3-2, EN 6100-3-3)

c. AS 4268.2

4. Box Contents

Included in the purchase of the Seasafe SSRX/01 Alarm & Tracking Receiver are the following components:

¨ Seasafe Model: SSRX/01 Alarm & Tracking Receiver

¨ Mounting cradle

¨ 6 x NiMH rechargeable batteries (installed by dealer upon purchase)

¨ Audio headset

¨ Power cable & connector

¨ Signal Attenuator module

¨ Marine omni-directional antenna (not in box)

¨ Portable collapsible directional antenna (not in box)

¨ Instruction manual

N15546

3

over interference, and (2)

ference 489-1,

4

Seasafe SSRX/01 Receiver Box Contents

Omni- Directional and Directional Antennae are not supplied in the box but are supplied as part of the kit

Audio Headset

Mounting Cradle Batteries

(Installed by agent)

Receiver

Omni-Directional Antenna

Power Cable / Plug

Directional Antenna folded

for easy stowage

Directional Antenna unfolded

for use

5

5. Seasafe Model: SSRX/01 Receiver Lay-out & Controls

1…………………………………………….….. Directional Antenna Input / Plug

2…………………………………………...Omni-Directional Antenna Input/Plug

3………………………………………….………...Power Plug / Auxiliary Output

4………………….………………………………..………… Audio Headset Input

5………………….…………………….………...Visual Signal Strength Indicator

6……………….…...……………………………..….…..’Search’ Selector Switch

7……………....……………………………………….…..’Alarm’ Selector Switch

8………………………………….………………….…..……..’Volume Up’ Switch

9………………………………….………….…….………..’Volume Down’ Switch

10……………………..…………….…………………………….’Power On’ Switch

11………………….….…………….…………………………….’Power Off’ Switch

12………………….…..…..……….’Battery Charge On’ / Vessel Power Indicator

13………………………..………….………….’Signal Gain Adjustment Switches’

14………………………..………….….……….……….Receiver Mounting Cradle

5

6

7

8

9

10 11 12

13

13

13

13

13

13

14

13

1 2 3 4

6

6. Equipment Introduction

Congratulations on your purchase of the Seasafe SSRX/01 Alarm and Tracking Receiver. Used in

conjunction with the Seasafe SSTX/01 or SSTX/DV Personal Transmitters, the Seasafe system

represents a major breakthrough in personal tracking and location technology.

Please remember that the Seasafe system is an Aid to rescue only and DOES NOT guarantee your

safety. This at all times remains the responsibility of the individual.

Before using the Seasafe system is important that you ensure the equipment complies with any or all

standards and regulations which may pertain to the prospective country/s in which you intend to use the

system.

The Seasafe system is a stand-alone and self-contained Alarm, Tracking & Location system. It operates

independently of traditional EPIRB satellite and International Emergency Frequency based systems.

Seasafe allows non-critical and critical situations to be responded to immediately at the time and place at

which they occur. The system is not reliant on a response from National or International Search & Rescue

Authorities.

The Seasafe system may be used in any marine or land-based pursuit whereby a person may become

lost, disorientated or in need of assistance.

Upon purchase, please be sure to complete the warranty registration form at the back of this manual or

the on-line Product Warranty Registration at www.sea-safe.com.au

7. Equipment Specification

Frequency…………………..……………………..…….……………......….…….173 - 174 MHz

Frequency Stability……..….………………………..…………………….......…………..15 PPM

Dimensions……………………………………...….........…....256mm L x 130mm W x 60mm D

Weight (with batteries)…………………………......……....………………...………..…+/- 850 g

Temperature Tolerance………………………….........….……………...…..….-10º C to +55º C

Decode Sensitivity…………………………………..…….......……………..- 128 DBM nominal

Adjacent channel selectivity………….……………..…….…......……………………..…..80 DB

Image and Spurious responses………………………..….….......……………………..…-80DB

Spurious Radiation………………………………………...…….......………………….< 57 DBM

Power……….......................12 VDC via 6 x internal1.2V NiMH AA rechargeable batteries

‘C-Tick’ compliance registration number………………….......………………...……… N15546

Australia / New Zealand Compliance………..........................................................AS 4268.2

European CE conformance……………………………...……………..…………......1999/5/EC

8. Installation (To be carried out by a qualified Marine or

Automotive Electrician only)

a. Receiver

The Seasafe Receiver may be operated either in-hand (portably) or via the dash-mount cradle. If using the

cradle, you should select a position on the vessel where the receiver will be clearly visible to the skipper,

protected from direct sunlight (if possible) and protected from direct exposure to moisture (rain, spray,

breaking waves etc). The cradle should be attached to the vessel dash-board or bulk head via two self

tapping screws (preferably stainless steel). Two appropriate sized holes should be drilled and the cradle

attached by the use of the two self-tapping screws. Once the cradle has been attached to the vessel, the

receiver can be slid into position. The receiver can be removed from the cradle at any time and either

operated in-hand or even removed from the vessel for portable operation. If the receiver is to be used

outside of the cradle, ensure that the omni-directional antenna is plugged into the receiver in order for the

device to receive emergency transmitter signals. The only time the omni-directional antenna should be

unplugged is when an alarm has been received and the directional-antenna has been plugged into the unit

in order to “search’ for the source of the alarm. When a search has been completed, the receiver should

7

be returned to stand-by mode and the omni-directional antenna should be plugged into the unit once

more. For portable operation, the receiver batteries will need to be fully charged. The batteries are

automatically charged when the receiver is connected to the vessel 12 VDC power supply. Batteries are

installed by the Seasafe dealer upon purchase of the equipment and are not replaceable by the user.

Whilst the receiver is connected to the vessel’s power it will run off this power source at the same time the

internal batteries will be charged. When disconnected from the vessel’s power source, the receiver will

operate off the internal battery supply.

b. Power / Auxiliary Output Cable

The receiver is supplied with a connection cable and plug. The cable consists of three wires.

A red ‘positive’ power connection wire, a black ‘negative’ connection wire and a white ‘auxiliary’ output

connection wire. Connection of the Seasafe Receiver Power and Auxiliary Output cable should only be

performed by a qualified marine or automotive electrician. The red wire is supplied with an in-line fuse

which will protect the receiver in the event of a short-circuit or electrical system overload. The red wire

must be connected to the positive 12 VDC power supply on the vessel. The black wire must be connected

to the negative or ground power supply of the vessel. If a vessel has only 24 Volt DC power available, a

24 volt to 12 volt DC converter should be used.

The white wire is an auxiliary or optional output from the receiver. In the event of an ‘alarm’ signal being

received by the receiver, an output is automatically transmitted through this white wire. This output signal

may be used to activate an ‘engine cut-out’, a flashing light, an external alarm klaxon or may even be

interfaced to an external MOB input on certain marine electronic GPS and navigation systems. If utilising

this auxiliary output signal, it MUST be connected to any peripheral equipment via an external relay. This

must be performed by a qualified marine or automotive electrician. Once the power cable has been

connected to the vessel power (vessel power should be switched on), the receiver is now ready for

operation.

Receiver Mounting Cradle Receiver Mounting Cradle

Un-assembled Assembled

Receiver Cradle

mounting holes

(use appropriate hardware

for mounting surface)

Receiver mounted in cradle

8

Typical ‘Alarm Output’ connection to peripheral device via external relay

c. Omni-Directional Antenna

The omni-directional-antenna is a marine grade antenna which has the capability of receiving a Seasafe

Transmitter signal within 360 degrees, provided the Transmitter is within signal receiving range. The

higher this antenna is mounted on the vessel, the greater the receiving range of the system. Therefore

the omni-directional antenna should be mounted at the highest practical point on the vessel. The antenna

should be mounted using the antenna mounting bracket (supplied) and stainless steel fasteners (not

supplied). The coaxial antenna cable should be run from the antenna to the Receiver ensuring that the

cable is not exposed or in a position to be damaged in any way. The coaxial cable plug must be

connected to the receiver input connector marked “omni-directional antenna”.

d. Collapsible Directional-Antenna

The directional or Yagi antenna is a portable hand-held powder coated marine grade aluminium antenna.

It should be safely stowed in the vessel folded up where it is easily accessible in case of an emergency. In

the event that it is required to use this antenna, it should be removed from its plastic sleeve and the folded

elements should be folded out for use. The coaxial cable attached to the antenna must be connected to

the input marked “directional-antenna” on the receiver. The “directional-antenna” may be mounted on the

vessel in a semi-permanent position utilising suitable ‘U’ bolt and nuts.

“directional-antenna” unfolded for use “directional-antenna” folded for easy stowage when not in use

Connects to White wire from

receiver power plug.

(Pin 2 on plug). Alarm output

signal.

Connects to battery 12

Volt positive.

Do not connect to AC

Voltage supply

Appropriate external relay.

Minimum coil resistance of 100 ohms

Connection to external

device / engine cut-out

etc.

9

9. System Operation

Once the receiver and omni-directional antenna have been correctly installed, the Seasafe Receiver is

ready for operation. To switch the system on, depress the “on” key. Once the “on” key has been

depressed, the “alarm”, “on” and signal gain button “6” will be illuminated. If the receiver is connected to

the vessel power, the “BATT CHG ON” indicator will also be illuminated. This indicates that the internal

batteries are being charged. Each time the receiver is switched on, the device will default to these

settings. If the “BATT CHG ON” indicator fails to illuminate when connected to the vessel power, please

first check that power is being supplied to the receiver and also check the condition of the in-line fuse.

Provided there is power to the device and the fuse is operational, if the receiver “BATT CHG ON” still fails

to illuminate then return the receiver to your closest Seasafe dealer for inspection and possible battery

replacement.

If the receiver is removed from the vessel power source, the “BATT CHG ON” indicator will no longer be

illuminated. However, continued use of the receiver is possible as the receiver will be powered by the

internal batteries. The length of time the receiver is able to operate in this mode is dependent on whether

the internal batteries are fully charged. The receiver may continue to operate for up to 3 hours on a full

charge. It is not recommended to operate the receiver in portable mode for extended lengths of time. As

the internal battery power decreases, the range and accuracy capabilities of the receiver are reduced

substantially and normal operation becomes unreliable. The receiver should only be used in the portable

mode (disconnected from vessel power) in absolute emergencies only.

The receiver is now ready to receive any emergency signal transmitted by a Seasafe transmitter

(SSTX/01or SSTX/DV) which is within range.

If a Seasafe transmitter is activated within range of the receiver, the receiver’s in-built alarm siren will emit

a high pitched audible alarm tone. The volume level of this alarm is factory set and cannot be adjusted. At

the same time, a number of “signal strength indicator” lights will be illuminated. The more lights which are

illuminated on the “signal strength indication”, the stronger the signal being received. At this point the

operator or skipper should depress the “SEARCH” button. This button will become illuminated and the

“ALARM” button will no longer be illuminated. The omni-directional antenna must now be unplugged from

the receiver and the collapsible directional-antenna must be unfolded and plugged into the receiver socket

marked “directional antenna”. (The omni-directional antenna need only be unplugged if the receiver is to

be used portably. The receiver automatically switches between omni and directional antennae when the

receiver is switched from “Alarm’ to ‘Search’ mode). The receiver will begin to emit a warbling audible

tone. This is an audible signal strength indication. The volume level of this audible signal strength indicator

is adjustable either up or down via the volume “UP” or “DOWN” buttons. The directional antenna is held in

the hand in the vertical plane (see picture over page) and the operator should very slowly rotate the

antenna through 360 degrees. If the directional antenna is permanently fixed to the vessel then the vessel

should be slowly manoeuvred through 360 degrees. The visual and audible signal strength indicators will

vary as the antenna is moved. The point within 360 degrees where the most signal indicator panel lights

are illuminated and where the audible tone is most highly pitched, will indicate the approximate direction

from which the transmitted signal is being received. If maximum signal strength is received through an arc

(say 90 degrees) then the vessel should proceed directly towards the middle point of this arc. See diagram

over page. The vessel must now move in this direction whilst the operator continues to scan with the

directional antenna. As the vessel gets closer to the transmitter signal source, the visual and audible

signal strengths will increase. In order to narrow the search angle or arc, the operator should now start to

systematically depress the “SIGNAL GAIN” switches down from number “6”. The signal strength

indications will reduce in intensity. Again, if the strongest signal is received through an arc then the boat

should proceed towards the centre position of this arc. If the vessel has an on-board compass, then a

compass bearing can be used to obtain the direction of the centre of this arc. The operator should

randomly continue to rotate the antenna through 360 degrees to check for any other transmitted signals as

there may be more than one activated transmitter at a time. By continuing to reduce the signal gain and to

follow the direction of the strongest received signal, the vessel should eventually come in visual contact

with the victim and they can be safely retrieved. If on ‘GAIN POSITION 1’ maximum signal strength is

being received through 360 degrees and the victim is not visual, the directional antenna may be

unplugged from its socket and plugged into the ‘Omni-Directional’ antenna socket. This will allow the

victim to be tracked even closer. If the victim is still not visible, the BNC attenuator module can be plugged

into the ‘OMNI-ANTENNA’ socket and the ‘DIRECTIONAL ANTENNA’ plugged into the attenuator. This

should provide a location accuracy within 5 to 10 metres. The attenuator module only need be used if the

victim is not visible already. The attenuator module MUST be removed when tracking is complete. Once a

10

victim has been retrieved, their Seasafe transmitter should be deactivated. Assuming there were no other

victims in the water at the same time, the receiver audible and visual signal strength indications should

cease. If the receiver continues to receive a signal, the same search procedure should be followed and

any other victims located and retrieved.

Once all victims have been retrieved, the receiver should be switched “OFF” and then “ON” to return it to

its default stand-by mode. If the system has been used in the portable mode, the directional-antenna must

be unplugged from the receiver, folded up and stowed safely for future use and the Omni-directional

antenna must now be immediately plugged into the receiver to scan for any future alarms.

Attenuator Module Receiver with attenuator and

Tracking antenna plugged into

Omni-Directional Antenna Port

Tracking a Seasafe Personal Transmitter signal using Seasafe Receiver and Directional Antenna

(note: the antenna is held in the vertical plane). Receiver is being used in the portable mode with the

optional rubber protection holster and neck-strap

SIGNAL TRACKING PROCEDURE

REPEAT PROCEDURE ON NEXT GAIN POSITION (5) ETC

KEEP REPEATING THIS PROCESS AND REDUCING THE ‘GAIN’ UNTIL TARGET IS LOCATED.

MAXIMUM SIGNAL STRENGTH MAXIMUM SIGNAL STRENGTH

RECEIVER SET ON

MOVE BOAT TOWARDS CENTRE OF MAXIMUM RECEIVED

SIGNAL STRENGTH ARC

MOVE BOAT TOWARDS CENTRE OF MAXIMUM RECEIVED SIGNAL

STRENGTH ARC

(THIS ARC GETS SMALLER AS THE GAIN IS REDUCED AND AS THE BOAT

GETS CLOSER TO THE TARGET)

MAXIMUM SIGNAL STRENGTH MAXIMUM SIGNAL STRENGTH

11

‘GAIN’ POSITION 6

12

The Seasafe receiver may receive transmitter signals up to the following ranges:

¨ Up to 5 km – 10 km from water to vessel (antenna at 2m above sea level)

¨ Up to 10 km – 20 km from water to vessel (antenna at 5m above sea level)

¨ Up to 20 km – 30 km from water to base station (with 15m high antenna mast)

¨ Up to 100 km from water to aircraft at 1500 m altitude

These ranges are not guaranteed and signal transmission ranges may be affected by a number of factors

including but not limited to, prevailing weather conditions, sea state, wave height, cloud cover, proximity to

land masses and line of sight obstructions.

10. Accessories

a. Rubber Protection-Holster & Neck Strap (Optional)

The protection holster and neck strap allow the receiver to be comfortably used in a portable mode. The

rubber holster provides additional protection against unintentional physical abuse.

The receiver can be hung around the neck and quite easily operated in conjunction with using the

directional antenna.

b. Audio Headset (Standard)

The Seasafe receiver is supplied complete with a lightweight audio headset. Under conditions of extreme

ambient noise or in cases where the skipper is unable to watch the ‘visual’ signal strength meter, the

headset can be plugged into the device and used to track the received audio signal.

The volume level of the audible signal strength indicator is adjustable up or down via the volume “UP” or

“DOWN” buttons.

11. Copyright

Copyright Seasafe Pty Ltd 2010

International Patents Pending

Specification subject to change without notice

E & OE

Simply fit the receiver into the

protection holster for portable use.

Can be hung around the neck for

WARRANTY REGISTRATION CERTIFICATE

Date of purchase………………………………………………………………..

Place of Purchase……………………………………………………………….

Authorised Dealer Name……………………………………………………….

Authorised Dealer Address……………………………………………………

Authorised Dealer Contact …………………………………………………….

Model Number of Equipment…………………………………………………..

Serial Number of Equipment…………………………………………………..

Name of Purchaser…………………………………………………………

Address of Purchaser…………………………………………………………….

Authorised Dealer Stamp

I have read and understand the terms of this warranty.

Signed by Purchaser

13

………………………………………………………………..

……………………………………………………………….

……………………………………………………….