Consideration on the alternatives of the Frequency Table in Appendix 17 of the Radio Regulations for the introduction of new digital technologies for the maritime mobile serviceto identify a possible solution of agenda item 1.9 (Resolution 351 (WRC-07))

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: DocumentsCPM11-2/121, 5B/713 PT46(11)INFO 057 | Document 5B-3/TEMP/005-E |
|  | 20 June 2011 |
|  | English only |
| Working Group 5B -3 Maritime |  |
| preliminary Draft New Report ITU-R M [Channel compare for App 17] |  |

This documentprovides a comparison of the alternative channelling plans for Appendix 17 was revised and is proposed to be promoted to a Preliminary Draft New Report.

A TEMP document is proposed be presented to WP-5B for attachment to the 5B Report.

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: DocumentsCPM11-2/121, 5B/713 | Document 5B/TEMP/xxx-E |
|  | 20 June 2011 |
|  | English only |
| Working Party 5B (WG 5B-3 Maritime) |  |

preliminary Draft New Report ITU-R M

[Channel compare for App 17]

Consideration on the alternatives of the Frequency Table in Appendix 17 of the Radio Regulations for the introduction of new digital technologies for the maritime mobile serviceto identify a possible solution of agenda item 1.9 (Resolution 351 (WRC-07))

This document contains in its Attachment a report which seeks to provide a comparison of the alternative channelling plans for Appendix 17 as a possible solution to issues addressed in agenda item 1.9 (WRC-12)

[ Administrations are invited to review the proposed revisions, with a view of upgrading this document to a draft status and subsequently sending it to Study Group 5 during the November 2011 meeting of Working Party 5B. ]

Attachment:	1

preliminary draft new report

Consideration on improvement of the frequency table in Appendix 17 of the Radio Regulations for the introduction of new digital technologies for the maritime mobile service

Objective

In response to the studies invited in Resolution 351 (Rev.WRC-07), this Report considers and summarizes features of possible methods to improve the frequency table in Appendix 17 of the Radio Regulations in relation to the introduction of new digital technologies for the maritime mobile service. It is not intended to pre-judge the results of future Conferences.

# 1	Introduction

Through the ITU-R study under Resolution 351 (Rev.WRC-07), two possible methods have been identified for improvement of the frequency table in Appendix 17 of the RR for the introduction of new digital technologies for the maritime mobile service(i.e. Method A1 and Method A2, as identified in Agenda item 1.9 in Chapter 1 of the CPM Report to WRC-12). Further consideration is required to finally decide which method is more appropriate to satisfy WRC-12 agenda item 1.19 towards revision of frequencies and channelling of Appendix 17.

In this Report, the detailed information of the current AP17, Method A1, and Method A2 are providedin a spread sheet for comparison and consideration by the Conference.

# 2	Discussion

Appendix 17 to the Radio Regulations includes two parts; PART A – specifies subdivided bands and Part B – specifies channel numberings. The modification of Appendix 17 has been based on the following concepts:

- reduce the current frequencies identified for NBDP use to a core band, which willinclude the GMDSS distress and safety requirement (see RR Appendix 15), with theaddition of some channels, in order to support current usage and to preclude the usageof other technologies in these core bands;

- release, after the transition period, the NBDP frequencies not included in the corebands for new exchange technologies (e.g. see Recommendation ITU-R M.1798) whileallowing administrations that choose to continue to use those bands for NBDP, to do sowithout claiming protection or causing interference;

- release the frequency bands designated for facsimile, wideband telegraphy and Morsetelegraphy A1A/A1B for digitally modulated emissions while allowing administrationsthat choose to continue to use them for facsimile, wideband telegraphy and Morsetelegraphy A1A/A1B, without claiming protection or causing interference;

- keep the frequency bands designated for duplex radiotelephony (linked withRR Appendix 25). However some administrations may allow stations to use digitallymodulated data emissions in the radiotelephony bands, in accordance with theRR Appendix 25 allotment Plan, without claiming protection from other stations andwithout causing any interference to other stations in the MMS using radiotelephony;

- establish a transition period, during which the authorized usage of NBDP isunchanged and administrations that introduce digital communications are encouraged tocoordinate with the affected administrations;

- cease NBDP transmission outside the core band at the end of the transition period.However administrations may continue to use NBDP technology without claimingprotection from or causing interference to stations in the MMS using digitallymodulated emissions;

- set a date for the cessation of NBDP transmission outside thecore band (end of the transition period);

- allow the use of digitally modulated emissions without a transition period in thefrequency bands designated for wideband telegraphy, facsimile and Morse telegraphyA1A/A1B;

- encourage administrations making assignments to stations using digitally modulatedemissions to effect coordination with potentially affected administrations from1 January 2017;

- do not change the bands dedicated for the radiotelephony simplex operations but makeprovision for the use of digitally modulated emissions on the basis of not claimingprotection from other stations and not causing any interference to other stations in theMMS using radiotelephony;

- do not modify RR Appendix 25 but allow the use of digitally modulated emissions inradiotelephony bands by administrations in accordance with the RR Appendix 25allotment Plan subject to not causing harmful interference to nor claiming protectionfrom other stations in the MMS using radiotelephony;

- provide some flexibility to administrations to introduce new simplex radiotelephonychannels (analogue or digital) in a portion of the bands 4, 6, 8 MHz in accordance withprovision RR No. 52.177, subject to not claiming protection from other stations in theMMS using digital modulated emissions

Both Method A1 and Method A2 (as identified in Agenda item 1.9 in Chapter 1 of the CPM Report to WRC-12) are based on the above concepts.

ANNEX

Graphical analysis/indication of current Appendix 17 (2008 edition), Method A1 and Method A2 for introduction of new digital technologies

This ANNEX graphically shows the situation of current Appendix 17 (2008 edition), Method A1 and Method A2 (as identified in Agenda item 1.9 in Chapter 1 of the CPM Report to WRC-12) in the attached spread sheets for easy comparisons and understandings.

In the spread sheets, the left row indicates the current Appendix 17 situation, the middle row indicates the CPM Report Method A1 and the right row indicates the CPM Report Method A2. The light green cells indicate the boundary frequencies of subdivided bands, the light blue cells indicated the bands for HF data communications and the values in parentheses in yellow cells indicate frequency gaps.

Tables 1A-1D indicate the channel numberings for new digital technologies specified in the CPM Report Method A2.To see the spread sheets, open the attached “Excel” file (“AP17 Detail.xlsx”)below.

Notes

The notes in spread sheets are shown below with proposed modifications for both CPM Report Method A1 and Method A2.

- See Part B, Section I.

b)	Until 1 January 2017,see Part B, Section III. After this date Section III will no longer apply and has to be deleted by a future competent WRC.

c)	The frequency bands may also be used by buoy stations for oceanographic data transmission and by stations interrogating these buoys.

d)	See Part B, Section II.

h)	(SUP – WRC-07)

i)	For the use of the carrier frequencies 4 125kHz, 6 215kHz, 8 291kHz, 12 290kHz and 16 420kHz in these sub-bands by ship and coast stations for distress and safety purposes, by single-sideband radiotelephony, see Article 31.     (WRC-07)

j)	For the use of the frequencies 4 177.5 kHz, 6 268 kHz, 8 376.5 kHz, 12 520 kHz and 16 695 kHz in these sub-bands by ship and coast stations for distress and safety purposes, by NBDP telegraphy, see Article 31.

k)	For the use of the frequencies 4 207.5 kHz, 6 312 kHz, 8 414.5 kHz, 12 577 kHz and 16 804.5 kHz in these sub-bands by ship and coast stations for distress and safety purposes, by digital selective calling, see Article 31.

l)	The following paired frequencies (for ship/coast stations) 4 208/4 219.5 kHz, 6 312.5/6 331 kHz, 8 415/ 8 436.5 kHz, 12 577.5/12 657 kHz, 16 805/16 903 kHz, 18 898.5/19 703.5 kHz, 22 374.5/22 444 kHz and 25 208.5/26 121 kHz are the first choice international frequencies for digital selective calling (see Article 54).

m)	Frequencies from these frequency bands may also be used for A1A or A1B Morse telegraphy subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions.

n)	The frequencies 4 210 kHz, 6 314 kHz, 8 416.5 kHz, 12 579 kHz, 16 806.5 kHz, 19 680.5 kHz, 22 376 kHz and 26 100.5 kHz are the exclusive international frequencies for the transmission of maritime safety information (MSI) (see Articles 31 and 33).

o)	The frequency 4 209.5 kHz is an exclusive international frequency for the transmission of NAVTEX type information (see Articles 31 and 33).

p)	These sub-bands, except the frequencies referred to in Notes i), j), n) and o), may be used for digitally modulated emissions for maritime mobile service (e.g. as described in Recommendation ITU-R M.1798). The provisions of No. 15.8 apply.

aa)	Until 1 January 2017, these bands may be used by narrow-band direct-printing applications. Before 1 January 2017, administrations who introduce digitally modulated emissionsfor radiocommunications are urged to take all practicable steps to prevent interference to the narrow-band direct-printing applications in the band.

bb)	From 1 January 2017 these bands, except the frequencies referred to in Notes n) and o), may be used by narrow-band direct-printing applications by administrations, subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions.

cc)	From 1 January 2017 administrations who will make assignments to stations using digitally modulated emissions are encouraged to effect coordination with potentially affected administrations.

dd)	These bands may be used by narrow-band direct-printing applications by the administrations, subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions.

ee)	Frequencies from these bands may be used for wideband telegraphy, facsimile, A1A/A1B Morse telegraphy and special data transmission on the condition that interference is not caused to and protection is not claimed from stations in the maritime mobile service using digitally modulated emissions.

ff)	The bands 4 345-4 351 kHz, 6 495-6 501 kHz, 8 701-8 707 kHz may be used for simplex (single-sideband) telephone operation (regularly spaced by 3 kHz), in accordance with provision No. 52.177, subject to not claiming protection from other stations in the maritime mobile service using digitally modulated emissions.

gg)	When assigning frequencies in the bands 4 202.25-4207.25 kHz, 6 300.25-6 311.75 kHz, 8 396.25-8 414.25 kHz, 12 559.75-12 576.75 kHz and 16 784.75-16 804.25 kHz administrations shall take all necessary precautions in order not to cause interference on the DSC distress frequencies 4207.5 kHz, 6312 kHz, 8 414.5 kHz, 12 577 kHz and 16 804.5 kHz.

hh) 	The bands 4 066.4-4 150.4 kHz, 4 352.4-4 436.4 kHz, 6 201.4-6 231.4 kHz, 6 502.4-6 523.4 kHz, 8 196.4-8 298.4 kHz, 8 708.4-8 813.4 kHz, 12 231.4-12 366.4 kHz, 13 078.4-13 198.4 kHz, 16 361.4-16 574.4 kHz, 17 243.4-17 408.4 kHz, 18 781.4-18 844.4 kHz, 19 756.4-19 798.4 kHz, 22 001.4-22 178.4 kHz, 22 697.4-22 853.4 kHz, 25 071.4-25 119.4 kHz, 26 146.4-26 173.4 kHz may be used, in accordance with the Appendix 25 allotment Plan, for digitally modulated emissions as described in Recommendation ITU-R M.1798 on condition that it shall not cause harmful interference to, or claimprotection from other stations in the maritime mobile service using radiotelephony operations. The digitally modulated emissions may be used provided that their occupied bandwidth does not exceed 2 800 Hz, it is situated wholly within one frequency channel and the peak envelope power of coast stations does not exceed 10 kW and the peak envelope power of ship stations does not exceed 1.5 kW per channel.

TABLE 1a

Channel numberings for new digital technologies for CPM Report Method A2

Note 1) 	the green cells indicate for non-paired (simplex) operation only.

Note 2)	the orange cells indicate to be assignable for wideband operation using multiple of 3 kHz bandwidth.

TABLE 1b

Channel numberings for new digital technologies for CPM Report Method A2 (continued)

TABLE 1c

Channel numberings for new digital technologies for CPM Report Method A2 (continued)

TABLE 1d

Channel numberings for new digital technologies for CPM Report Method A2 (continued)

________________

____________________