|  |  | SE43(11)47 |
| --- | --- | --- |
| #### 11th meeting of SE43 Lisbon, 19-21 September 2011 |  |  |
| #### Date issued: |  |  |
| #### Source: | #### INdT |  |
| #### Subject: | #### Further analysis on EIRP limits for WSDs |  |
| #### Summary: | #### This contribution is a revision of document SE43(11)32 based on the discussion in the previous SE43 meeting and some further analysis. Considerations on some issues are given, e.g. the adopted protection criteria and strategy to define WSD power limits, multiple interferers, and interference modelling. New WSD power limits are provided. |  |
| #### Proposal: | #### It is proposed that a new Annex to the working document is created about reference geometries, where Sections 2.1 and 3.3 are included. A second Annex formed by Sections 2.2 and 2.3 is proposed to be created to provide more details on the strategy to define WSD power limits as originally proposed in document SE43(11)12. |  |
| #### Background: | #### CEPT SE 43 group addressed the identification of a common set of the parameters to calculate location specific WSD Power as a topic that requires immediate further studies (issue B1 of section 11 of ECC Report 159). #### In the previous meeting, document SE43(11)32 has addressed the modeling and the methodology presented in document SE43(11)12 to the calculation of WSD power limits. The attachment is an update of the attachment in document SE43(11)32 taking into account discussion in the previous meeting. |  |

| Password protection required? (Y/N) | N |
| --- | --- |

ATTACHMENT

FURTHER ANALYSIS ON EIRP LIMITS FOR WSDs

(Considerations on the SE43(11)12 contribution)

- INTRODUCTION

SE43(11)12 contribution to ECC Report 159 proposes a set of parameters for the calculation of location specific WSD power levels, as well as maximum WSD power values for fixed and portable outdoor WSDs to protect broadcast services.

Assumptions and methodology in SE43(11)12 are revisited in this contribution. Considerations are made on the composition of scenarios and on the adopted criterion to protect the primary service and strategy to define WSD power limits. New WSD power limits are provided.

- BACKGROUND

Five interference scenarios combining mobile or fixed WSD transmission with mobile or fixed DTT reception are considered in SE43(11)12. The study is developed considering:

- A minimum separation distance between the WSD transmitter and the DTT receiver;

- The worst angular scenarios between WSD transmit antenna and DTT receive antenna;

- 3 equivalent WSDs contributing to the interference to the DTT receiver.

Mobile WSD transmit antenna and mobile DTT receive antenna are omnidirectional, and present 0 dBi and 2.15 dBi gain, respectively. Fixed DTT receive antenna gain is 9.15 dBi. Due to the short distances between WSD and DTT receiver, the free space propagation model is adopted, i.e.,

PLFS=32.45+20log10fMHz+20log10dkm [dB],

where PLFS is the free space path loss as a function of the frequency fMHz in MHz and of the transmitter-receiver distance dkm in kilometers. A fixed carrier frequency is considered in calculations, fMHz=650 MHz.

The five scenarios are described below.

  - Scenarios considered in SE43(11)12 contribution

    - Scenario 1: Mobile WSD transmission (1.5 m agl) and fixed roof-top DTT reception (10 m agl)

Figure 1 illustrates the reference geometry of Scenario 1.  In this scenario, 22 m separate a mobile WSD at 1.5 m above ground level (agl) from the base of a rooftop DTT receive antenna at 10 m agl. The distance between the two antennas is then 23.58 m, and the corresponding path loss is 56.15 dB. A vertical DTT receive antenna pattern is considered, where the discrimination from the WSD to the DTT antenna for this reference geometry is 0.45 dB, and the front-to-back ratio is 3 dB.

Figure 1 - Reference geometry of Scenario 1.

    - Scenario 2: Mobile WSD transmission (10 m agl) and fixed roof-top DTT reception (10 m agl)

Figure 2 illustrates the reference geometry of Scenario 2. In this scenario, 20 m separate a mobile WSD at 10 m agl, supposed in a story window, from a rooftop DTT receive antenna at 10 m agl. The corresponding path loss is 54.72 dB. There is no receive antenna or polarization discrimination. The front-to-back ratio is 16 dB.

P

IB

Fixed rooftop DTT

reception

TV

-

54.72 dB

Portable WSD

from nth story window

10 m

-

16 dB

+9.15

dBi

20m

Figure 2 - Reference geometry of Scenario 2.

    - Scenario 3: Mobile WSD transmission (1.5 m agl) and mobile DTT reception (1.5 m agl)

Figure 3 illustrates the reference geometry of Scenario 3.  In this scenario, 2 m separate a mobile WSD at 1.5 m agl from a mobile DTT receive antenna also at 1.5 m agl. The corresponding path loss is 34.72 dB. There is no receive antenna or polarization discrimination.

Figure 3 - Reference geometry of Scenario 3.

2 m

1.5 m

1.5 m

Mobile DTT reception at 1.5 m agl

WSD

DTT

-34.72 dB

    - Scenario 4: Fixed WSD transmission (10 m agl) and fixed roof-top DTT reception (10 m agl)

Figure 4 illustrates the reference geometry of Scenario 4.  In this scenario, 20 m separate a fixed WSD at 10 m agl from a rooftop DTT receive antenna at 10 m agl. The corresponding path loss is 54.72 dB. There is no receive antenna discrimination, the polarization discrimination is 3 dB, and the front-to-back ratio is 16 dB.

Figure 4 - Reference geometry of Scenario 4.

P

IB

Fixed rooftop DTT

reception

TV

-

54.72 dB

Fixed WSD

10 m

-

16 dB

+9.15

dBi

H-polar

Slant-polar

20m

    - Scenario 5: Fixed WSD transmission (10 m agl) and mobile DTT reception (1.5 m agl)

Figure 5 illustrates the reference geometry of Scenario 5. In this scenario, 20 m separate the base of a fixed WSD at 10 m agl from a mobile DTT receiver with antenna at 1.5 m agl. The distance between the two antennas is then 21.73 m, and the corresponding path loss is 55.45 dB. A vertical WSD transmit antenna pattern is considered, where the discrimination from the WSD to the DTT antenna for this reference geometry is 10 dB. There is no receive antenna or polarization discrimination. Therefore, the total loss is 55.45 + 10 = 65.45 dB.

Mobile DTT reception

P

IB

-

- 55.45 dB

Fixed WSD

1.5 m

20m

10 m

Figure 5 - Reference geometry of Scenario 5.

  - Protection criteria considered in SE43(11)12 contribution

SE43(11)12 contribution proposes the use of two simultaneous criteria to protect the primary service from WSD interference: the limitation of the location probability degradation (ΔLP) and the limitation of interference.

Location probability (LP) is a measure commonly used in DTT planning to represent the availability of the DTT signal in a given area. An area is considered covered if at least 95% of the locations within the area can receive the minimum field strength Eminwmed_ref [dBμV/m]. Limiting the degradation on LP is then a measure to protect the DTT service. SE43(11)12 proposes as protection criterion maximum ΔLP=0.1%.

Limitation of interference into the broadcast service is considered by defining maximum I/N thresholds. ITU-R BT 1895 recommends maximum I/N= -20 dB for all radiations and emissions without a corresponding frequency allocation in the Radio Regulations. SE43(11)12 proposes as protection criterion maximum I/N= -3 dB.

The combination of the two protection criteria divides the DTT coverage area into two regions, as illustrated in Figure 6. Region I corresponds to the layer externally delimited by the coverage edge, where the median field strength Ewmed [dBμV/m] is such that the location probability (LP) is 95%. The limit between Region I and Region II is determined by the edge where the field strength is 10 dB above the one at the coverage edge, and the area internal to this edge composes the Region II.

Figure 6 - Protection criteria inside the DTT coverage area - SE43(11)12.

In Region I, the maximum WSD EIRP is location specific and satisfies ΔLP=0.1%. Differently, the maximum WSD EIRP in Region II assumes a fixed value, defined at the external edge of the region, where it satisfies ΔLP=0.1%, and corresponds to I/N= -3 dB. Therefore, inside Region II the limiting protection criterion is the maximum interference-to-noise ratio.

  - Study methodology in SE43(11)12 contribution

Methodology adopted in SE43(11)12 was suggested in ECC Report 159 (Section 4.3.4. and Annex 6). It is based on Monte Carlo simulations, where the wanted DTT signal and the interfering signal (WSD) are modeled as statistical variables as follows:

- Ewmed [dBμV/m] ~ NEwmed, σw=5.5 is the field strength of the wanted signal at the DTT receiver antenna input, with mean Ewmed and standard deviation σw;

- Eimed [dBμV/m] ~ NEimed, σI=3.5 is the field strength of the interference at the DTT receiver antenna input, with mean Eimed and standard deviation σI.

After a large number of trials, one calculates:

- Location probability (LP)

LP=PrEwmed>Eminwmed_ref,

where Eminwmed_ref is the minimum field strength for DTT reception.

- Location probability under WSD interference (LPWSD)

LPWSD=PrEwmed>Eminwmed_ref+Eimed.

- Location probability degradation (ΔLP)

ΔLP=LP-LPWSD.

This methodology permits the calculation of the maximum interference median field strength Eimed at the DTT receiver antenna for a given ΔLP. From Eimed, the maximum WSD EIRP can be calculated for different scenarios according to specific path loss values and antenna configurations.

    - Understanding the calculation of the maximum interference median field strength Eimed at the DTT receiver antenna input for a given ΔLP

By definition, the location probability at the DTT service coverage edge is LP=95%. From ECC Report 159, the minimum median field strength for fixed outdoor DTT reception is Ewmed_ref=56.21 dBμV/m. Figure 7 illustrates the distribution of the wanted signal field strength at the DTT receiver antenna input for LP=95%.

EwmedLP=95%

0.95=PrEwmed>56.21=Q56.21-Ewmed5.5→Ewmed=65.26 dBμV/m

LP=95%Ewmed=65.26 dBμV/m-68.20 dBmLP=95%

Figure 7 - Median field strength of the wanted signal for LP=95%.

0.95=PrEw>Emin=QEmin -56.215.5→Emin=47.16 dBμV/m.

The minimum field strength at the coverage edge, Emin=47.16 dBμV/m, corresponds to -86.29 dBm.

Location probability (LP) can be directly calculated from the median field strength of the wanted signal at the DTT receiver, Ewmed, as expressed below:

LP=PrEwmed>Eminwmed_ref=QEminwmed_ref-Ewmedσw.

The maximum interference median field strength Eimed at the DTT receiver antenna for a given ΔLP is obtained by Monte Carlo simulations. For instance, for ΔLP=0.1% at the coverage edge, where LP=95%, LPWSD=94.9%. Then, the interference median field strength Eimed to be found by simulations is such that:

LPWSD=PrEwmed>Eminwmed_ref+Eimed=0.949,

where the field strengths are summed in linear scale, and Ewmed and Eimed are defined as the statistical variables below:

Ewmed [dBμV/m] ~ NEwmed=56.2165.26 dBμV/m, σw=5.5

Eimed [dBμV/m] ~ NEimed, σI=3.5.

Simulations were carried out over 1,000 trials and 1,000 random variable samples per trial. Table 1 gives the theoretical and the simulated values of LP for different wanted median field strengths Ewmed from the coverage edge until the location where the median field strength is 10 dB higher than Ewmed at the edge. The corresponding values of maximum interference median field strength for ΔLP=0.1% are also presented in Table 1. These values are calculated for a fixed DTT reception co-channel protection ratio PR0=21 dB (ECC Report 159 and SE43(11)12 contribution).

Table 1– Relationship between Ewmed, LP, and EimedE'imed for fixed outdoor DTT reception and ΔLP=0.1%.

| Location | Ewmed [dBμV/m] | LP [%] (calculated) | LP [%] (simulated) | EimedE'imed[dBμV/m] |
| --- | --- | --- | --- | --- |
| Coverage edge (Ewmed) | 56.21 | 95.01 | 95.00 | 5.60 |
| Ewmed+1 dB | 57.21 | 96.62 | 96.61 | 7.20 |
| Ewmed+2 dB | 58.21 | 97.77 | 97.78 | 8.60 |
| Ewmed+3 dB | 59.21 | 98.58 | 98.57 | 10.30 |
| Ewmed+4 dB | 60.21 | 99.12 | 99.10 | 11.90 |
| Ewmed+5 dB | 61.21 | 99.47 | 99.47 | 13.60 |
| Ewmed+6 dB | 62.21 | 99.69 | 99.69 | 15.40 |
| Ewmed+7 dB | 63.21 | 99.82 | 99.82 | 17.40 |
| Ewmed+8 dB | 64.21 | 99.90 | 99.90 | 19.20 |
| Ewmed+9 dB | 65.21 | 99.95 | 99.95 | 21.10 |
| Ewmed+10 dB | 66.21 | 99.97 | 99.97 | 22.90 |

The same procedure described above can be used to obtain the maximum permitted interference median field strength for portable outdoor DTT reception. According to ECC Report 159, the minimum median field strength of the wanted signal at the DTT receiver input in this case is Ewmed_ref=61.21 dBμV/m. Furthermore, the co-channel protection ratio is given by PR0=19 dBGa=2.15 dBi. Table 2 shows the obtained values of LP and maximum interference median field strength for ΔLP=0.1% for different wanted median field strengths Ewmed.

Table 2– Relationship between Ewmed, LP, and EimedE'imed for portable outdoor DTT reception and ΔLP=0.1%.

| Location | Ewmed [dBμV/m] | LP [%] (calculated) | LP [%] (simulated) | EimedE'imed[dBμV/m] |
| --- | --- | --- | --- | --- |
| Coverage edge (Ewmed) | 61.21 | 95.01 | 95.00 | 12.40 |
| Ewmed+1 dB | 62.21 | 96.62 | 96.60 | 14.00 |
| Ewmed+2 dB | 63.21 | 97.77 | 97.76 | 15.50 |
| Ewmed+3 dB | 64.21 | 98.58 | 98.58 | 17.30 |
| Ewmed+4 dB | 65.21 | 99.12 | 99.14 | 18.80 |
| Ewmed+5 dB | 66.21 | 99.47 | 99.47 | 20.70 |
| Ewmed+6 dB | 67.21 | 99.69 | 99.69 | 22.60 |
| Ewmed+7 dB | 68.21 | 99.82 | 99.83 | 24.20 |
| Ewmed+8 dB | 69.21 | 99.90 | 99.90 | 26.10 |
| Ewmed+9 dB | 70.21 | 99.95 | 99.95 | 28.10 |
| Ewmed+10 dB | 71.21 | 99.97 | 99.97 | 29.60 |

- SOME CONSIDERATIONS ON THE STUDY DEVELOPED IN SE43(11)12 CONTRIBUTION

ΔLP=0.1%

ΔLP=0.1%7 dB9.15 dBi2.15 dBi

7 dB

Results shown in Table 1 and Table 2, obtained through the described procedure to calculate the maximum interference median field strength at the DTT receiver antenna input agree with those presented in document SE43(11)12 for fixed and portable outdoor DTT reception, ΔLP=0.1%, and 1 WSD interferer (SE43(11)12 – Table 2 and Table 4).

Other aspects are discussed in the following.

  - Criteria for primary service protection and definition of WSD EIRP limits

As mentioned in Section 2.2., SE43(11)12 contribution proposes the combination of two criteria to protect the primary DTT service and define WSD power limits.I/N WSD EIRPs inside the DTT coverage area must simultaneously respect a maximum level of location probability degradation (ΔLP) and a maximum interference-to-noise ratio (I/N) at the DTT receiver. According to this protection strategy, in the layer near the DTT coverage edge WSD emissions are limited by the maximum ΔLP, and are thus location specific. On the other hand, in locations more internal to the DTT coverage area, where the DTT signal is stronger, WSD EIRPs are limited to a fixed level that satisfies I/N= -3 dB.

It is important to emphasize that with this strategy the desirable flexibility on the definition of WSD power limits according to the location is restricted to a tight layer of the coverage area, where the quality of the DTT signal is poor (Region I in Figure 6). Because of the I/N constraint, this strategy does not explore the higher potential of secondary use of vacant spectrum on the large area where the DTT signal is stronger.

Limitation of interference to broadcasting systems is recommended in ITU-R BT.1895. It recommends:

I/N

“that the total interference at the receiver from all radiations and emissions without a corresponding frequency allocation in the Radio Regulations should not exceed 1 per cent of the total receiving system noise power.”

However, it is also recommended:

“that the values in recommends 2 and 3 be used as guidelines, above which compatibility studies on the effect of radiations and emissions from other applications and services into the broadcasting service should be undertaken.”

Studies related to the impact of interference on systems operating in the broadcasting service have considered as interference sources non-broadcasting radiocommunication devices like ultra-wideband (UWB) devices and short-range FM modulators. Further compatibility studies on the effects of WSD emissions on the DTT service are thus required. These studies may determine if a protection criterion based on maximum I/N values is necessary in addition to protection criteria based on the planning of DTT services and on the capabilities of DTT receivers in dealing with interference.

Planning criteria for DTT services consider reference values of minimum field strength and protection ratio for acceptable DTT service quality (Recommendation ITU-R BT.1368-3). Specific studies on the impact of interference from mobile service (especially from LTE) on DVB-T receivers (ECC Report 148) define reference values of overloading threshold.

Protection of the primary DTT service should be defined according to specific studies on the impact of interference on DTT receiver and on DTT reception quality. The protection criterion can jointly consider the following three elements: location probability, protection ratio and overloading threshold. Location specific emissions of WSDs can be limited by a determined level of location probability degradation, where a reference protection ratio is considered and the overloading threshold is respected.

  - Multiple WSD interferers

Simultaneous multi-interferers are considered in SE43(11)12 studies. In adjacent channels, 3 equivalent WSDs are assumed to contribute to the interference to the DTT receiver. This assumption decreases the permitted interference levels and the WSD EIRPs in about 4-5 dB.

Considering that:

- the interference limits are calculated for the worst case separation distance between the WSD transmitter and the DTT receiver and the worst case angle from the WSD transmitter to the direction of main response of the DTT receive antenna;

- t

- this deterministic reference spatial scenario represents a specific situation that may present a very low probability of occurrence, resulting in unnecessary protection to the primary service and sub-utilization of the available spectrum,

then, the  issue of cumulative interference deserves further investigation, and the adoption of deterministic reference scenarios for the calculation of WSD EIRP limits may not be adequate in the sense of efficient utilization of vacant spectrum.

The spatial composition of 3 WSDs interferers in the worst case interference configuration is improbable, for instance, in the case of fixed outdoor WSDs and DTT reception (Scenario 4). Figure 8 illustrates the situation.

Figure 8 - Discrimination of DTT receiver antenna (ITU-R BT.419-3) and the spatial composition of the reference configuration with 3 WSDs interferers.

The main response of DTT receive antenna comprises -20o≤θ≤+20o, where there is no signal attenuation. Then, for causing the worst case interference to a DTT receiver at 20 m, 3 WSDs should be placed in front of the DTT receive antenna over an arc of less than 14 m extension. This is an improbable spatial configuration for fixed outdoor WSDs. The probability of occurrence of different geometries is further investigated in document “Reference spatial geometries between WSDs and DTT-RXs for the calculation of the maximum permitted WSD EIRP”, available as input document of the 11th SE43 meeting.

100 m×100 m

  - Composition of new reference scenarios for WSD operation

Scenarios considered in SE43(11)12 correspond to reference geometries for the calculation of WSD EIRP limits.

Fixed outdoor WSD transmit antenna is considered in Scenarios 4 and 5 at 10 m agl. Fixed outdoor WSD transmit antenna at 30 m agl. is foreseen in ECC Report 159 (Section 3.2.4) in a scenario that resembles the traditional fixed service point-to-point usage, where the WSD receive antenna is also fixed outdoor above roof-top. This WSD transmit antenna type must also be considered. Then, two new scenarios are described below.

The definition of reference scenarios where a difference of height between WSD transmitter and DTT receiver exists is strongly influenced by the vertical radiation pattern of antennas, since the reference distance between WSD and DTT receiver corresponding to the lowest total loss between them will depend on the attenuation/discrimination of both antennas.

For illustrative purpose, reference radiation patterns recommended by ITU are used to model the vertical antenna attenuation/discrimination. The fixed outdoor WSD transmitter antenna follows the model given in ITU-R F.1336-2, which gives the antenna pattern shown in Figure 11. The fixed outdoor DTT receiver antenna follows the model given in ITU-R BT.419-3 and shown in Figure 8. No discrimination is considered for portable DTT receiver antenna.

Figure 11 - Vertical attenuation of WSD transmitter antenna (ITU-R F.1336-2).

    - Scenario 6: Fixed WSD transmission (30 m agl) and fixed roof-top DTT reception (10 m agl)

Figure 12 illustrates the reference geometry of Scenario 6.  In this scenario, 50 m separate the base of a fixed WSD at 30 m agl from the base of a rooftop DTT receive antenna at 10 m agl. The corresponding path loss is 63.33 dB. From Figure 11, the vertical WSD transmit antenna attenuation at 45 21.8o is  14 dB. 45oDiscrimination at this angle for the DTT reception is  0.72 dB. Therefore, the total loss gives 57.74+25.12+15.61=98.47 dB 63.33+14+0.72=78.05 dB.

Figure 12 - Reference geometry of Scenario 6.

    - Scenario 7: Fixed WSD transmission (30 m agl) and mobile DTT reception (1.5 m agl)

Figure 13 illustrates the reference geometry of Scenario 7. In this scenario, 47 m separate the base of a fixed WSD at 30 m agl from a mobile DTT receiver with antenna at 1.5 m agl. The corresponding path loss is  63.51 dB. From Figure 11, the vertical WSD transmit antenna attenuation at 55 31.2o is  13.99 dB. There is no receive antenna or polarization discrimination. Therefore, the total loss is59.54+39.06=98.60 dB 63.51+13.99=77.5 dB.

31.2°

Figure 13 - Reference geometry of Scenario 7.

Because of the vertical antenna radiation pattern of both WSD transmitter and DTT receiver, the total loss between them in Scenario 6 is 98.47 78.05-57.72= 40.75 20.33 dB higher than in Scenario 4. In Scenario 7, the total loss is 98.60 77.5-65.45=33.15 12.05 dB higher than in Scenario 5. Therefore, the WSD EIRP limits in Scenarios 6 and 7 for fixed outdoor WSD transmission at 30 m agl. exceed the ones determined for fixed outdoor WSD transmission at 10 m agl. in Scenarios 4 and 5 respectively by 40.75 20.33 dB and 39.06 12.05 dB. Scenarios 6 and 7 account for the promising WSD operation in rural areas.

  - Modeling of interference

σI=3.5 dBσI=1 dB

In the methodology suggested in ECC Report 159 (Section 4.3.4. and Annex 6) and adopted in document SE43(11)12 to calculate WSD EIRP limits, the wanted DTT signal and the interfering WSD signal at the DTT receiver antenna input are modeled as log-normal random variables with specific mean and standard deviation values.

This model is in accordance with fundamental propagation models where a log-normal random variable is added (in dB) to a mean path loss which depends on the distance between transmitter and receiver. The random variable accounts for uncertainties due to reflections and obstructions (shadowing).

The value of standard deviation adopted to the wanted DTT signal is σw=5.5 dB. This is a common value in broadcasting, where the distance between DTT transmitter and receiver is long. On the other hand, the distance between WSD transmitter and DTT receiver may be short, usually less than 100 m, and the adopted standard deviation to the interference WSD signal is σi=3.5 dB.

In spite of the common use of σi=3.5 dB for low transmitter-receiver separation distances, e.g. in SEAMCAT, a detailed OFCOM technical report based on measurement campaigns shows that 3.5 dB standard deviation of log-normal shadowing is excessive for transmitter-receiver distances lower than 100 m. The adopted 1 dB standard deviation is shown to fit well the two-ray path gain model and the propagation measurements for short range distances when associated to the free space model or to the extended Hata model. Therefore, the interference standard deviation σi=1 dB is considered in the following simulations.

- WSD EIRP LIMITS

We present now the maximum interference field strengths and the corresponding WSD EIRP limits for each scenario (including the proposed Scenarios 6 and 7), according to the considerations made in Section 3:

- Protection criterion is based on the limitation of the location probability degradation, i.e., ΔLP=0.1%;

- Protection ratio is the same adopted in SE43(11)12: (co-channel) PR(0)=21 dB for fixed DTT reception and 19 dB for mobile DTT reception; (1st adjacent channel) PR(Δf)1st=-30 dB; and (2nd adjacent channel) PR(Δf)2nd=-40 dB;

- Overloading threshold values are given in Tables 3 and 4 (10th percentile values are adopted);

Table 3 - DVB-T overloading thresholds in the presence of a time constant LTE BS interfering signal in a Gaussian channel environment (adapted from Table 5b - ECC Report 148).

| DVB-T Oth for 64-QAM 2/3 DVB-T signal (LTE UE TPC off) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel edge separation (MHz) | Oth (dBm) |  |  |  |  |  |
|  | 90th |  | 50th |  | 10th |  |
|  | Fixed DTT | Portable DTT | Fixed DTT | Portable DTT | Fixed DTT | Portable DTT |
| 1 | -2 | -3 | -8 | -17 | -13 | -26 |
| 9 | 4 | 0 | -1 | -13 | -7 | -22 |

Table 4 - DVB-T overloading thresholds in the presence of a time constant LTE UE interfering signal in a Gaussian channel environment (adapted from Table 7b - ECC Report 148).

| DVB-T Oth for 64-QAM 2/3 DVB-T signal (LTE UE TPC off) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel edge separation (MHz) | Oth (dBm) |  |  |  |  |  |
|  | 90th |  | 50th |  | 10th |  |
|  | Fixed DTT | Portable DTT | Fixed DTT | Portable DTT | Fixed DTT | Portable DTT |
| 1.5 | -14… -9 | -3 | -16… -16 | -15 | -23 … -17 | -27 |
| 9.5 | -10 … 9 | -13 | -28 … 2 | -30 | -46 … -5 | -47 |

- DTT fixed receivers are considered to use silicon-type tuners implemented in set-top boxes and/or integrated TV (iDTV);

- DTT portable receivers are considered to use silicon-type tuners implemented in small USB-type devices;

- Only 1 WSD interferer is considered;

- Interference is modeled as a Gaussian random variable with σI=1 dB;

- Other configuration parameters are identical to the ones in SE43(11)12 simulation.

Tables 5 and 6 show the obtained values of LP and maximum interference field strength for ΔLP=0.1% for different wanted median field strengths Ewmed. Results for fixed outdoor DTT reception are in Table 5 and for mobile outdoor DTT reception in Table 6. Results for the location where the wanted field strength is Ewmed+20 dB are also shown.

Table 5– Relationship between Ewmed, LP, and EimedE'imed for fixed outdoor DTT reception and ΔLP=0.1%.

| Location | Ewmed [dBμV/m] | LP [%] (calculated) | LP [%] (simulated) | EimedE'imed[dBμV/m] |
| --- | --- | --- | --- | --- |
| Coverage edge (Ewmed) | 56.21 | 95.00 | 94.98 | 7.00 |
| Ewmed+1 dB | 57.21 | 96.61 | 96.63 | 8.30 |
| Ewmed+2 dB | 58.21 | 97.77 | 97.77 | 9.90 |
| Ewmed+3 dB | 59.21 | 98.57 | 98.58 | 11.50 |
| Ewmed+4 dB | 60.21 | 99.12 | 99.12 | 13.20 |
| Ewmed+5 dB | 61.21 | 99.47 | 99.47 | 15.20 |
| Ewmed+6 dB | 62.21 | 99.69 | 99.69 | 17.00 |
| Ewmed+7 dB | 63.21 | 99.82 | 99.82 | 19.00 |
| Ewmed+8 dB | 64.21 | 99.90 | 99.91 | 21.00 |
| Ewmed+9 dB | 65.21 | 99.95 | 99.95 | 23.00 |
| Ewmed+10 dB | 66.21 | 99.97 | 99.97 | 24.90 |
| Ewmed+20 dB | 76.21 | 100.0 | 100.0 | 37.70 |

Maximum interference field strengths in Table 5 are about 1.5-2 dB above the ones in Table 1. The difference is a consequence of the lower standard deviation σI in the last simulation.

Table 6– Relationship between Ewmed, LP, and EimedE'imed for portable outdoor DTT reception and ΔLP=0.1%.

| Location | Ewmed [dBμV/m] | LP [%] (calculated) | LP [%] (simulated) | EimedE'imed[dBμV/m] |
| --- | --- | --- | --- | --- |
| Coverage edge (Ewmed) | 61.21 | 95.00 | 94.99 | 13.70 |
| Ewmed+1 dB | 62.21 | 96.61 | 96.63 | 15.00 |
| Ewmed+2 dB | 63.21 | 97.77 | 97.76 | 16.80 |
| Ewmed+3 dB | 64.21 | 98.57 98.58 | 98.58 | 18.40 |
| Ewmed+4 dB | 65.21 | 99.12 | 99.12 | 20.10 |
| Ewmed+5 dB | 66.21 | 99.47 | 99.47 | 22.00 |
| Ewmed+6 dB | 67.21 | 99.69 | 99.69 | 23.90 |
| Ewmed+7 dB | 68.21 | 99.82 | 99.82 | 26.10 |
| Ewmed+8 dB | 69.21 | 99.90 | 99.91 | 27.90 |
| Ewmed+9 dB | 70.21 | 99.95 | 99.95 | 29.90 |
| Ewmed+10 dB | 71.21 | 99.97 | 99.97 | 31.70 |
| Ewmed+20 dB | 81.21 | 100.0 | 100.0 | 44.60 |

Maximum interference field strengths in Table 6 are about 1-2.5 dB above the ones in Table 2. The difference is a consequence of the lower standard deviation σI in the last simulation.

Next, the maximum permitted interference field strengths at the DTT receive antenna input EimedE'imed shown in Tables 5 and 6 for fixed and portable outdoor DTT reception are translated to interference power levels IdBm at the receiver input through:

PIdBm=EdBμV/m -20log10fMHz-77.2+Ga+PR0-PRΔf|Δf≠0,

where  Ga is the DTT receiver antenna gain (including down lead loss) and the last term accounts for the protection ration in adjacent channels.

Using the referenced values of protection ratio, the maximum permitted interference powers at the receiver input for 1st and 2nd adjacent channels are organized in Table 7. Overloading thresholds for each combination of WSD transmitter and DTT receiver types are also given. Observe that except in the case of portable WSD transmission and portable DTT reception, when the overloading threshold for the 2nd adjacent channel is met at Ewmed+7 dB 5 dB location, the overloading threshold is far from be achieved in Ewmed+10 dB20 dB location. For fixed DTT reception, one approximates the overloading threshold only at Ewmed+20 dB location, with portable WSD transmission.

Table 7 –Maximum permitted interference I[dBm] at the receiver input and overloading thresholds Oth[dBm].

| Location | Fixed outdoor DTT receiver |  | Portable outdoor DTT receiver |  |
| --- | --- | --- | --- | --- |
|  | 1st adjacent channel | 2nd adjacent channel | 1st adjacent channel | 2nd adjacent channel |
| Coverage edge (Ewmed) | -66.31 | -56.31 | -68.61 | -58.61 |
| Ewmed+1 dB | -65.01 | -55.01 | -67.31 | -57.31 |
| Ewmed+2 dB | -63.41 | -53.41 | -65.51 | -55.51 |
| Ewmed+3 dB | -61.81 | -51.81 | -63.91 | -53.91 |
| Ewmed+4 dB | -60.11 | -50.11 | -62.21 | -52.21 |
| Ewmed+5 dB | -58.11 | -48.11 | -60.31 | -50.31 |
| Ewmed+6 dB | -56.31 | -46.31 | -58.41 | -48.41 |
| Ewmed+7 dB | -54.31 | -44.31 | -56.21 | -46.21 |
| Ewmed+8 dB | -52.31 | -42.31 | -54.41 | -44.41 |
| Ewmed+9 dB | -50.31 | -40.31 | -52.41 | -42.41 |
| Ewmed+10 dB | -48.41 | -38.41 | -50.61 | -40.61 |
| Ewmed+20 dB | -35.61 | -25.61 | -37.71 | -27.71 |
| Oth [dBm] – LTE BS transmitter | -13 | -7 | -26 | -22 |
| Oth [dBm] – LTE UE transmitter | -20 | -25.5 | -27 | -47 |

Now, from the maximum permitted interference at the DTT receiver input given in Table 7, one calculates the maximum permitted WSD EIRP for each scenario. Table 8 shows these values for Scenarios 1, 2, 3, 4 and 5, while the results for Scenarios 6 and 7 are given in Table 9. WSD EIRP limits already consider the overloading thresholds.

Scenarios in yellow present the most restrictive WSD power limits according to the type of WSD transmitter (fixed or portable). WSD EIRPs that were limited by the overloading threshold are also indicated.

Because of the low value of overloading threshold for portable DTT reception (see Table 7), WSD emissions for portable WSD transmission / portable DTT reception are limited by the overloading threshold in locations where the wanted field strength is equal to or higher than Ewmed+7 dB5 dB. For fixed WSD transmission, the EIRP at Ewmed+20 dB location is near the overloading threshold, which limits the EIRP at 43.45 dBm.

Table 8 - Maximum permitted WSD EIRP [dBm] (Scenarios 1-5).

| Location | Portable WSD transmitter |  | Portable WSD transmitter |  | Portable WSD transmitter |  | Fixed WSD transmitter |  | Fixed WSD transmitter |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fixed DTT receiver |  | Fixed DTT receiver |  | Portable DTT receiver |  | Fixed DTT receiver |  | Portable DTT receiver |  |
|  | Scenario 1 |  | Scenario 2 |  | Scenario 3 |  | Scenario 4 |  | Scenario 5 |  |
|  | 1st adj | 2nd adj | 1st adj | 2nd adj | 1st adj | 2nd adj | 1st adj | 2nd adj | 1st adj | 2nd adj |
| Coverage edge (Ewmed) | -18.9 | -8.9 | -20.7 | -10.7 | -36.0 | -26.0 | -17.7 | -7.7 | -5.2 | 4.8 |
| Ewmed+1 dB | -17.6 | -7.6 | -19.4 | -9.4 | -34.7 | -24.7 | -16.4 | -6.4 | -3.9 | 6.1 |
| Ewmed+2 dB | -16.0 | -6.0 | -17.8 | -7.8 | -32.9 | -22.9 | -14.8 | -4.8 | -2.1 | 7.9 |
| Ewmed+3 dB | -14.4 | -4.4 | -16.2 | -6.2 | -31.3 | -21.3 | -13.2 | -3.2 | -0.5 | 9.5 |
| Ewmed+4 dB | -12.7 | -2.7 | -14.5 | -4.5 | -29.6 | -19.6 | -11.5 | -1.5 | 1.2 | 11.2 |
| Ewmed+5 dB |  | -10.7 | -0.7 |  | -12.5 |  | -2.5 |  |  | -27.7 | -17.7 |  | -9.5 |  | 0.5 |  | 3.1 |  | 13.1 |  |
| Ewmed+6 dB |  | -8.9 | 1.1 |  | -10.7 |  | -0.7 |  |  | -25.8 | -15.8 |  | -7.7 |  | 2.3 |  | 5.0 |  | 15.0 |  |
| Ewmed+7 dB | -6.9 | 3.1 | -8.7 | 1.3 | -23.6 | -14.4 | -5.7 | 4.3 | 7.2 | 17.2 |
| Ewmed+8 dB | -4.9 | 5.1 | -6.7 | 3.3 | -21.8 | -14.4 | -3.7 | 6.3 | 9.0 | 19.0 |
| Ewmed+9 dB | -2.9 | 7.1 | -4.7 | 5.3 | -19.8 | -14.4 | -1.7 | 8.3 | 11.0 | 21.0 |
| Ewmed+10 dB | -1.0 | 9.0 | -2.8 | 7.2 | -18.0 | -14.4 | 0.2 | 10.2 | 12.8 | 22.8 |
| Ewmed+20 dB | 11.8 | 21.8 | 10.0 | 20.0 | -5.1 | -14.4 | 13.0 | 23.0 | 25.7 | 35.7 |

Table 9 - Maximum permitted WSD EIRP [dBm] (Scenarios 6 and 7).

| Layer | Fixed WSD transmitter |  | Fixed WSD transmitter |  |
| --- | --- | --- | --- | --- |
|  | Fixed DTT receiver |  | Portable DTT receiver |  |
|  | Scenario 6 |  | Scenario 7 |  |
|  | 1st adj | 2nd adj | 1st adj | 2nd adj |
| Ewmed_ref | 2.6 | 12.6 | 6.7 | 16.7 |
| Ewmed_ref + 1 dB | 3.9 | 13.9 | 8.0 | 18.0 |
| Ewmed_ref + 2 dB | 5.5 | 15.5 | 9.8 | 19.8 |
| Ewmed_ref + 3 dB | 7.1 | 17.1 | 11.4 | 21.4 |
| Ewmed_ref + 4 dB | 8.8 | 18.8 | 13.1 | 23.1 |
| Ewmed_ref + 5 dB | 10.8 | 20.8 | 15.0 | 25.0 |
| Ewmed_ref + 6 dB | 12.6 | 22.6 | 16.9 | 26.9 |
| Ewmed_ref + 7 dB | 14.6 | 24.6 | 19.1 | 29.1 |
| Ewmed_ref + 8 dB | 16.6 | 26.6 | 20.9 | 30.9 |
| Ewmed_ref + 9 dB | 18.6 | 28.6 | 22.9 | 32.9 |
| Ewmed_ref + 10 dB | 20.5 | 30.5 | 24.7 | 34.7 |
| Ewmed_ref + 20 dB | 33.3 | 43.3 | 37.6 | 47.6 |

- SUMMARY AND CONCLUSIONS

The appropriate methodology of calculation of maximum WSD power levels with the protection of the primary DTT service must consider the effective impact of WSD interference on the DTT receiver and on the DTT reception quality.

Based on the studies presented above, location specific emissions of WSDs can be limited by determined levels of location probability degradation, where  reference protection ratios are considered and overloading thresholds are respected. It is needed further investigation on:

- Suitable levels of location probability degradation, possibly variable according to the location;

- Reference values of protection ratio;

- Reference values of overloading threshold for DTT receivers.

Absolute values of WSD maximum power are provided for different scenarios (Table 8 and Table 9), including two scenarios that resemble the traditional fixed service point-to-point usage, where both the WSD transmit and the WSD receive antenna are fixed outdoor above roof-top, or represent the WSD operation in rural areas.

The reference scenarios, spatial compositions of one WSD transmitter and one DTT receiver, are situations with possibly low probability of occurrence. Considering simultaneous multiple WSD interferers over the reference scenarios is a worst case over worst case assumption. For this reason, it is suggested that the calculation of WSD power limits takes into account statistics of reference scenarios, issue addressed in other contribution.

INdT (Nokia Institute of Technology) is a non-profit research and development institute founded by Nokia in Brazil.

EwmedLP=95%Ewmed_refEwmed1 dBEwmedEwmed+10 dB

Minimum value of the signal-to-interference ratio required to obtain a specified reception quality under specified conditions at the receiver input.

Interfering signal level above which the receiver begins to lose its ability to discriminate against interfering signals at frequencies differing from that of the wanted signal.

100 m×100 m

OFCOM, “Technical analysis of interference from mobile network base stations in the 800 MHz band to digital terrestrial television,” 10 June 2011.