|  |  | SE43(11)55 |
| --- | --- | --- |
| #### 11th meeting of SE43 Lisbon, 19 – 21 September 2011 |  |  |
| #### Date issued: | #### 8 September 2011 |  |
| #### Source: | #### BT |  |
| #### Subject: | #### Parameters for TVWS devices |  |

- INTRODUCTION

The CEPT SE43 working document (Annex 1 to SE43(11)40) identifies in Sections 4 & 5 typical parameters for different use cases of TVWS devices.  These include the idea of using TVWS for mobile broadband coverage.  However there are also trials currently ongoing in the UK investigating TVWS being used to provide fixed broadband connections.  Therefore we believe that this use case should also be included in the SE43 working document.

- PARAMETERS FOR FIXED BROADBAND USING TVWS

The following example parameters should be included in Sections 4 & 5 (as appropriate) of the CEPT SE 43 working document, to address the possible use of TVWS to provide fixed broadband connections.  This is obviously an emerging technology, and we acknowledge that other parameter values may be used in any future deployments, particularly if other example systems are used.

|  | Downlink (Base Station transmit) | Uplink (Base Station receive) |
| --- | --- | --- |
| Data rate | Up to 10 Mbit/s (per user) | Up to 2 Mbit/s (per user) |
| Transmission bandwidth | 5 MHz | 5 MHz |
| EIRP | 36 dBm (subject to authorisation based on the local radio environment) | 27 dBm (subject to authorisation based on the local radio environment) |
| Receiver noise figure | 5 dB | 4 dB |
| Transmit antenna gain | 7 dBi | 14 dBi |
| Receive antenna gain | 14 dBi | 7 dBi |
| Maximum range | Up to 10 km | Up to 10 km |

END