|  |  | Document SE43(11)90 |
| --- | --- | --- |
| 12th Meeting of SE43 Cambridge, 14-16 December 2011 |  |  |
| Date issued: 13 December 2011 Source: Russian Federation Subject: Proposed modifications to section 7 of ECC Report 159 |  |  |

Proposed modifications to ECC Report 159

Summary: This Document proposes to review ECC Report 159 and to make modifications regarding protection of ARNS systems operating in the frequency band 645 – 790 MHz from emissions of cognitive radio devices operating in “white spaces” (WSD). In particular it is proposed: - to include additional ARNS characteristics to Section 7.1 of the Report in accordance with characteristics contained in JTG 5-6 output documents (Annex 5 to Document 5-6/180) and CPM Report; -to include the study results presented in this document to Section 7 of the Report -to bring the conclusions given in Section 12 in line with the presented studies.

Proposal:

WG SE 43 is invited to review ECC Report 159 and make the appropriate modifications presented in Annex 1.

Background:

ECC Report 159 developed in January regarding protection of ARNS systems operating in the frequency band 645 – 790 MHz from WSD emissions contains ARNS system characteristics operating in the frequency range 645-862 MHz and preliminary proposals concerning possible protection measures of ARNS system only. Additionally the need of carrying out additional studies was noted in the Report. With this respect this Report was analyzed and the results showed that characteristics of the considered ARNS systems are incomplete and its more detailed description can be found in JTG 5-6 output documents (Annex 5 to Document 5-6/180) and CPM Report. Using these characteristics possible measures for providing protection of ARNS systems from WSD emissions were studied taking into account possible interference caused by broadcasting service allotments. Calculation results and proposals for modifications of ECC Report 159 are given in Annex 1.

Annex 1

Working Document towards Preliminary Draft revision of ECC REPORT 159

Technical and operational requirements

for the possible operation of cognitive radio systems

in the ‘white spaces’ of the frequency band 470-790 MHz

[Cardiff, January 2011]

…

# Protection of aeronautical radionavigation (ARNS) in the band 645-790 MHz from emissions of cognitive radio devices operating in ‘white spaces’

## ARNS system characteristics and protection criteria

In accordance with No.5.312 RR the frequency band 645-862 MHz is allocated to aeronautical radionavigation service (ARNS) in several countries on a primary basis. Several types of the radionavigation systems are used in this service including:

- radio system of short-range navigation (RSBN);

- air traffic control secondary radars, including terrestrial radar and airborne transmitter;

- airfield and route primary radars for ATC

The indicated systems are used to support navigation and air traffic control.

The European Common Allocation includes the note EU13.

EU13 CEPT Administrations are urged to take all practical steps to clear the band 645-960 MHz of the

assignments to the aeronautical radionavigation service.

The main characteristics of different types of the aeronautical radionavigation stations operating in the frequency band 645-862 MHz are in JTG 5-6 output documents (Annex 5 to Document 5-6/180) and CPM Report and presented in Table 12

| Type of station Characteristics | RSBN | RLS 2 (Type 1) |  | RLS 2 (Type 2) |  | RLS 1 (Type 1) | RLS 1 (Type 2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Application | “Air-to-Ground” | Secondary radars – Type 1 (air traffic control) |  | Secondary radars – Type 2 |  | Primary radars – Type 1 | Primary radars – Type 2 |  |
| Transmitter characteristics |  |  |  |  |  |  |  |  |
| Station name | Aircraft transmitter | Ground radar transmitter | Aircraft transponder transmitter | Ground radar transmitter | Aircraft transponder transmitter | Ground radar transmitter | Ground radar transmitter |
| Place of station | Aircraft | Airfields | Aircraft | Airfields | Aircraft | Airfields | Airfields |
| Maximum effective radiated pulse power (e.r.p.), dBW | 30.5 | 48 | 35 | 69,5 | 34,5 | 82 | 82 |
| Pulse power, dBW | 27 | 31 | 32 | 40 | 31 | 52,5 | 52,5 |
| Mean power, dBW | 0.5 | 1 | 14 | 10,5 | 10,5 | 19,5 | 19,5 |
| Pulse repetition cycle, ms | 2.3 | 1,3 | 0,6 | 1,8 | 1,8 | 1,8 | 1,8 |
| Pulse length, μs | 5.1 | 1,3 | 8,7 | 2 | 16 | 0,9-2 | 0,9-2 |
| Necessary emission bandwidth, MHz | 3/0,7 | 4 | 4 | 3 | 3 | 3 | 3 |
| Antenna height, m | 10 000 | 10 | 10 000 | 10 | 10 000 | 10 | 10 |
| Maximum antenna gain (dBi) | 3,5 | 17 | 3 | 29,5 | 3,5 | 29,5 | 29,5 |
| Antenna pattern | ND | 3 dB beamwidth: vert. pl. = 28° hor. pl. = 4° | ND | 3 dB beamwidth: vert. pl. = 45° hor. pl. = 3-5° | ND | 3 dB beamwidth: vert. pl. = 45° hor. pl. = 4° | 3 dB beamwidth: vert. pl. = 45° hor. pl. = 4° |
| Direction of the antenna main beam | Lower hemisphere | Azimuth: 0-360° Scan rate: 6 min-1 | Lower hemisphere | Azimuth: 0-360° Scan rate: 10 min-1 | Lower hemisphere | Azimuth: 0-360° Scan rate: 6/10 min-1 | Azimuth: 0-360° Scan rate: 6/10 min-1 |

| Type of station Characteristics | RSBN | RLS 2 (Type 1) (air traffic control) |  | RLS 2 (Type 2) |  | RLS 1 (Type 1) | RLS 1 (Type 2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Receiver characteristics |  |  |  |  |  |  |  |
| Station name | Ground radar receiver | Aircraft responder of ground radar | Ground radar receiver | Aircraft responder of ground radar | Ground radar receiver | Ground radar receiver | Ground radar receiver |
| Antenna height, m | 10 | 0–10 000 | 10 | 0–10 000 | 10 | 10 | 10 |
| Polarization | Linear, horizontal | Linear, vertical | Linear, vertical | Linear, horizontal | Linear, horizontal | Linear, horizontal | Linear, horizontal |
| Maximum antenna gain (dBi) | 22 | 3 | 17 | 3 | 28,4 | 29,5 | 29.5 |
| Antenna pattern | 3 dB beamwidth: vert. pl. = 50° hor. pl. = 360° | ND | 3 dB beamwidth: vert. pl. = 28° hor. pl. = 4° | ND | 3 dB beamwidth: vert. pl. = 45° hor. pl. = 3-5° | 3 dB beamwidth: vert. pl. = 45° hor. pl. = 3-5° | 3 dB beamwidth: vert. pl. = 45° hor. pl. = 3-5° |
| Direction of antenna main beam | Azimuth: 0-360° Revolution speed 100 rev/min | Lower hemisphere | Azimuth: 0-360° Scan rate: 6 min-1 | Lower hemisphere | Azimuth: 0-360° Scan rate: 10 min-1 | Azimuth: 0-360° Scan rate: 6/10 min-1 | Azimuth: 0-360° Scan rate: 6/10 min-1 |
| Permissible aggregate co-channel interference field strength provided for the necessary emission bandwidth (from all services), E, dB(μV/m)* | 42 | 52 / 59 | 291 / 332 | 73 | 241 / 282 | 13 | 13 |

*The values given in the Table are the permissible values of the aggregate co-channel interference field strength presented for the required emission bandwidth (from all services). There are two values that can be used for sharing studies and these values shall be updated after detailed consideration of study results and should not contradict GE06 Agreement.

Table 12: Technical characteristics of ARNS systems operating in the 645-862 MHz frequency band

The analysis of the presented data showed that in the considered frequency band mainly 3 types of ARNS systems are used:

- radio system of short-range navigation (RSBN) (radiolink aircraft – ground);

- Secondary radar Type 1 (radiolinks ground– aircraft, aircraft – ground);

- Secondary radar Type 2 (radiolink aircraft – ground).

The indicated systems are considered below in the studies of sharing feasibility of WSD with ARNS systems.

## Possible interference scenario from WSD to ARNS systems

## The analysis of WSD deployment scenario given in Section 3 and the analysis of ARNS characteristics operating in the considered frequency band showed that the following interference scenario can be possible:

1) WSD causes interference to ARNS ground station;

2) WSD causes interference to ARNS airborne station.

Taking into account possible usage of WSD presented in Section 3 it can be assumed that WSD will be deployed mainly in the mobile service and will have characteristics similar to characteristics of MS which are planned to be used in the frequency band 790 – 862 MHz .

The studies of the required protection measures of ARNS from mobile service planned to be used in the frequency band 790 – 862 MHz were carried out in the framework of ITU-R JTG 5-6.This group developed methodologies for defining the required coordination distances (see Doc. 5-6/180).The indicated methodologies can be used for defining the required protection distances for ARNS systems in the considered frequency band from WDS network systems.

The frequency band 645–790 MHz subject to GE-06 Agreement. This Agreement includes allotment plan for broadcasting service. This Plan was developed in 2006 based on the assumption that only ARNS and broadcasting service systems will operate in the indicated frequency band. It was developed taking into account no any margins for interference caused by other systems to ARNS systems excluding broadcasting service allotments/assignments. While planning the usage of the frequency band 645–790 MHz by WSD it should be taken into account.

In case interference caused by broadcasting service allotments/assignments to ARNS are exceeded or meet ARNS protection criteria without margins then operation of WSD is practically impossible. In this case distance between WSD and ARNS station can be not less 1000 km (in accordance with Recommendation ITU-R P.1546-4). This case is mostly common since when broadcasting plan was developed administrations did their utmost to receive maximum broadcasting allotment parameters.

If interference caused by broadcasting allotments/assignments to ARNS do not exceede ARNS protection criteria and there is a margin for interference then in this case minimum protection distance for ARNS systems can be defined. It is obvious that this protection distance will be defined by margin values, MS antenna height, its deployment scenario and etc. The example of estimation of the required separation distances between ARNS stations and WSD is given below.

## Assumptions and methodologies used in the estimations

For defining the protection distances it was assumed that interference margin caused by systems excluding broadcasting systems is minimum and do not exceed 0.1 dB. This interference margin was used for defining the permissible interference field strength values from WSD given in Table 12a.

| ARNS systems | Protected field strength dB (µV/m) (under Rec. ITU-R M.1830) | Permissible interference field strength from WSD, dB (µV/m) |
| --- | --- | --- |
| RSBN ground-based receiver | 42 | 25.6 |
| RLS 2 Type 1 ground-based receiver | 29 | 12.6 |
| RLS 2 Type 2 ground-based receiver | 24 | 7.6 |
| RLS 2 Type 1 airborne receiver | 52 | 35.6 |

Table 12а. Permissible interference field strength values caused by WSD.

Taking into account uncertainty with respect to WSD types and characteristics methodology based on statistical approach was used for defining separation distances between WSD and ground-based ARNS stations. This methodology was developed in the framework of ITU-R JTG 5-6 and given in detail in Doc. 5-6/180.

In estimation it was assumed that WSD antenna heights are 40-60 (set randomly) and EIRP of WSD is 55 dBm (comply with typical EIRP of MS base stations).It was also assumed that WSD stations form a network with the following deployment densities of stations:

- 0.008 per km2 for rural area;

- 0.13 per km2 for suburban area;

- 2.05 per km2 for urban area.

## Estimation results of WSD emission influence to ARNS airborne and ground-based receivers

In defining protection distances two following options are considered:

- Interference caused by WSD stations, located in rural area;

- Interference caused by WSD stations, located in accordance with urban-suburban-rural scenario considered by JTG 5-6 (Fig.1)

Fig. 1 Urban-suburban-rural scenario

For both options land path (100 % land) and mixed (5 % land, 95 % sea) path were considered.

The results for defining protection distances are given in tables A and B. Table A presents estimation results of the required protection distances for WSD in rural area. Table B presents estimation results of the required protection distances for WSD in accordance with urban-suburban-rural scenario.

Table А

| System | Transmitter antenna height, m | Distance up to the first line of transmitters, km |  |
| --- | --- | --- | --- |
|  |  | 100% land | 5% land , 95% sea |
| RSBN | 40 … 60 | 83 | 162 |
| RLS 2 Type 1 | 40 … 60 | 225 | 278 |
| RLS 2 Type 2 | 40 … 60 | 280 | 335 |

Table В

| System | Transmitter antenna height, m | Distance up to the first line of transmitters, km |  |
| --- | --- | --- | --- |
|  |  | 100% суша | 5% суша, 95% море |
| RSBN | 40 … 60 | 190 | 262 |
| RLS 2 Type 1 | 40 … 60 | 320 | 375 |
| RLS 2 Type 2 | 40 … 60 | 370 | 430 |

Table D

The analysis of the received data showed that in rural area the required protection distance for land path is not less 280 km and for the mixed path is not less 335 km. In conditions of urban-suburban-rural scenario the required protection distance for land path is not less 370 km and for the mixed path is not less 430 km.

Except the above-mentioned cases influence from WSD to ARNS airborne receivers was considered. The received results showed that in this case the required protection distance shall be not less than line-of-sight distance (444 km) for all ARNS type systems.

## Conclusions

The presented studies showed:

In case interference caused by broadcasting allotments/assignments to ARNS are exceeded or meet ARNS protection criteria without margins then operation of WSD is impossible in radius of 1000 km with respect to ARNS stations.

- If interference caused by broadcasting allotments/assignments to ARNS do not exceeded ARNS protection criteria and there is a margin for interference then for protection of ARNS systems stringent limitations for WSD station deployment shall be specified. For providing protection of ground stations of ARNS systems separation distance of typical WSD from ARNS stations shall be not less 370 km for land path and 430 km for mixed path.

- For protection of ARNS airborne stations the separation distance of typical WSD from ARNS stations exceeds the line-of-sight distance ad shall be not less than 432 km.

Taking into account that in many cases there are no margins for interference the usage of WSD shall be avoided in the frequency bands of ARNS systems operating in countries indicated in RR No.5.312.

…

# conclusion

…

- Protection of aeronautical radionavigation (ARNS) in the 645-790 MHz band

Preliminary considerations have been provided on the relevance of the sensing and geo-location techniques for the protection of ARNS.

The presented studies showed:

- In case interference caused by broadcasting allotments/assignments to ARNS are exceeded or meet ARNS protection criteria without margins then operation of WSD is impossible in radius of 1000 km with respect to ARNS stations.

- If interference caused by broadcasting allotments/assignments to ARNS do not exceeded ARNS protection criteria and there is a margin for interference then for protection of ARNS systems stringent limitations for WSD station deployment shall be specified.

  - For providing protection of ground stations of ARNS systems separation distance of typical WSD from ARNS stations shall be not less 370 km for land path and 430 km for mixed path.

  - For protection of ARNS airborne stations the separation distance of typical WSD from ARNS stations exceeds the line-of-sight distance and shall be not less than 444 km.

Taking into account that in many cases there are no margins for interference the usage of WSD shall be avoided in the frequency bands of ARNS systems operating in countries indicated in RR No.5.312.

Further it can be required:

- to refine characteristics and usage scenario of WSD;

- to refine protection criteria for ARNS with respect to WSD;

-to carry out additional studies taking into account the refined protection criteria for ARNS and WSD characteristics including studies estimation studies of the required protection measures of ARNS ground stations from multiple interference caused by WSD.

…

Presented by RCC countries

Can be used with regard to several other countries mentioned in No. 5.312, except RCC countries