|  |  | SE43(12)10 |
| --- | --- | --- |
| #### 13th meeting of SE43 Paris, 20 – 22 March 2012 |  |  |
| #### Date issued: | #### 10 March 2012 |  |
| #### Source: | #### INdT |  |
| #### Subject: | #### Revisions and complementary study on Annex 5 of the draft complementary report “A2” on geo-location approach |  |
| #### Summary: | #### This contribution updates Annex 5 (A5.10 – A5.14) of the working document SE43(11)105A2(Rev.1). It extends the current analysis of WSD EIRP limits for the protection of fixed outdoor DTT reception to the cases of portable outdoor and portable indoor DTT reception. Location specific WSD EIRP limits with variable degradation of location probability are thus defined for the three DTT reception modes. |  |
| #### Proposal: | #### To consider the changes proposed in the Attachment as update of Annex A5.10 – A5.14 of the working document “Technical and operational requirements for the operation of white space devices under geo-location approach”. |  |
| #### Background: | In last meeting, the analysis presented in document SE43(11)83 (incorporated as Annex A5.10 – A5.14) for the definition of upper limits of WSD EIRP and location probability degradation for the protection of fixed DTT reception was required to be extended to portable outdoor and portable indoor DTT reception. |  |

| Password protection required? (Y/N) | N |
| --- | --- |

ATTACHMENT

  - [INdT editor’s note: Changes in the current text of Annex A5.10 – A5.14 maintain the essence of the original text. They just clarify the main aspects of the contribution.]

  - a5.10 Use of variable acceptable degradation of the coverage probability

The use of TV white spaces is conditioned to the protection of the primary DTT broadcasting service against WSD interference. This protection is represented by the respect of tolerable levels of DTT location probability degradation (ΔLP), i.e. tolerable degradations of the DTT coverage quality.  Low values of ΔLP are essentially tolerable, but they may be too restrictive to the operation of white space systems in vacant channels. On the other hand, the increase of ΔLP may lead to good conditions for white space systems operation, but at the cost of causing harmful interference to the DTT broadcasting system.

An optimum degradation level is not expected to be found, since the decision about the ΔLP to be adopted is influenced by several aspects, like the target DTT coverage quality, the specific characteristics of different regions (urban, suburban, rural areas), etc. In spite of this, upper limits to ΔLP can be derived by considering the technical limitations of the DTT receivers in dealing with interference. The methodology presented in this annex provides ΔLP upper limits and can be used to guide Administrations in their decisions about the location probability degradation levels to be adopted in different situations and scenarios. Since in some countries the protection of portable outdoor and portable indoor DTT reception is required in addition to the protection of fixed outdoor DTT reception, the three reception modes are considered.

The signal modeling and the calculation of location probability are in accordance with ECC Report 159, where DTT wanted and WSD interfering signals are considered log-normal random variables with specific standard deviation, and the location probability is obtained by Monte Carlo simulations. All scenarios presented in Annex 2 of complementary report A2 are considered in the determination of WSD EIRP limits according to the type of WSD and the DTT reception mode to be protected. Aggregate or cumulative interference due to simultaneous operation of multiple WSDs is not considered. Then, the EIRP limits provided in this section should be reduced by appropriate multiple interference margins (Annex 1 of complementary report A2).

a5.11 RELEVANT PARAMETERS for THE PROTECTION OF THE DTT RECEIVER AGAINST INTERFERENCE

WSD interference degrades two distinct aspects of DTT broadcasting service:

- The coverage quality of DTT service;

- The ability of the DTT receiver in discriminating the desired signal from interference signals.

From the perspective of the DTT receiver, two parameters are important to appropriate operation in the presence of interference:

- Protection ratio: the minimum value of the signal-to-interference ratio at the DTT receiver required to obtain a specified reception quality under specified conditions;

- Overloading threshold: the interference level above which the receiver begins to lose the ability to discriminate against interfering signals at frequencies differing from that of the wanted signal.

Conditions for the protection of the DTT receiver against interference taking into account the two aforementioned parameters provide interference limits for appropriate operation of the DTT receiver. The appropriate operation in this case means the respect of the protection ratio and the overloading threshold. Given these interference limits, it is possible to calculate the corresponding maximum permissible levels of ΔLP, and thus to assess the feasible levels of ΔLP, i.e. the levels of ΔLP for which the DTT receiver is able to handle interference.

The conditions based on the protection ratio and on the overloading threshold for the protection of the DTT receiver against interference are usual in ECC compatibility studies, as in ECC Report 138  (see Annex C) and ECC Report 148. They take into account statistical location variations in the wanted and interfering signals, and provide interference limits for the protection of the DTT receiver for a percentage of locations. For simplicity, interference of other DTT transmitters is ignored.

  - a5.12 satisfaction of Protection ratio

The usual protection condition of the DTT receiver against interference is related to the protection ratio. It is given by

Ewmed ≥ Eimed + PR(Δf) + µx% √(σ2w+ σ2i)

where:

Ewmed: wanted DTT median field strength at the DTT receiver;

σw: wanted DTT field standard deviation;

Eimed: WSD interference median field strength at the DTT receiver;

σi: WSD interference standard deviation;

PR(Δf)PR(Δf): protection ratio for a Δf frequency offset (co-channel or adjacent channels);

µx% √(σ2w+ σ2i)μX%√(σw2+σi2)μX%√(σw2+σi2): location correction factor for X% of locations within the small covered area (pixel).

Location correction factor takes into account the statistical location variations of both the wanted and the interfering signals.

Therefore, the limiting interference median field strength Eimed_maxEimed_maxEimed_max_PR for the satisfaction of the appropriate (co-channel or adjacent channels) protection ratio PR(Δf)PR(Δf) for X% of locations within the pixel is expressed as

Ewmed_max_PR = Ewmed - PR(Δf) - µx% √(σ2w+ σ2i)

for a given wanted median field strengthEwmedEwmed Ewmed and wanted and interfering standard deviations σw and σi.

  - a5.13 respect of the Overloading threshold

The protection of DTT receiver against overloading consists in satisfying:

IFE ≤ Oth - µx%×σi

where:

IFE IFEIFE: interference power at the front-end of the DTT receiver;

Oth OthOth: overloading threshold;

µx%×σi μX%σiμX%σi: location correction factor for X% of locations within the small covered area (pixel).

Location correction factor in this case takes into account the statistical location variations of the interfering signal only, since DTT receiver overload does not depend on the wanted DTT signal power.

The interference power at the front-end of the DTT receiver relates to the interference power at the DTT receiver input I as follows:

IFE = I – POL + Ga

where POL and Ga represent antenna polarization discrimination and antenna gain (including feeder losses). The relation between interference I and interference median field strength Eimed Eimed Eimedis given by

I = Eimed – 20.log10(fMHz) – 77.2

for the frequency of operation fMHzfMHz given in MHz.

Therefore, the limiting interference median field strength at the DTT receiver, Eimed_max_Oth, for the protection of the DTT receiver against overload for X% of locations within the pixel OthOthis given by

Eimed_max_Oth = OthOthOth - µx%×σi + POL – Ga + 20.log10(fMHz) + 77.2

It is noticeable that the receiver overload does not depend on the protection ratio.

  - a5.14 limits for location probability degradation and wsd eirp

The protection of the DTT receiver against interference for X% of locations inside the pixel is guaranteed by the most stringent interference upper limit for the satisfaction of the PR and the respect of the Oth for X% of locations. Then, the limiting interference field strength at the DTT receiver, Eimed_max, is given by

Eimed_max = min(Eimed_max_PR, Eimed_max_Oth)

where Eimed_max_PR and Eimed_max_Oth are calculated as previously described.

The methodology proposed in the ECC Report 159 is thus used to obtain by Monte Carlo simulations both the original LP (without WSD interference) and the resulting LP with the limiting WSD interference Eimed_max. The corresponding ΔLP, i.e. the difference between the original and the resulting LP, is considered the ΔLP upper limit for the protection of the DTT receiver for X% of locations inside the pixel.

The WSD EIRP upper limit is finally calculated from Eimed_max (or from its corresponding interference power Imax=Eimed_max-20log10(fMHz)-77.2) according to the specific WSD transmitter / DTT receiver scenario (reference geometry, as in Annex 2 of complementary report A2). This leads to the following general relationship:

EIRPmax= Imax+LOSS+Aatt+Adisc

where EIRPmax is the WSD EIRP upper limit for the protection of the DTT receiver at the reference geometry for X% of locations inside the pixel, Imax is the interference limit at the DTT receiver, LOSS is the propagation loss between the WSD and the DTT in the reference geometry, Aatt is the WSD antenna attenuation, and Adisc is the DTT antenna discrimination.

  - [INdT editor’s note: The current Sections A5.13 and A5.14 about scenarios, parameters, and WSD EIRP results are deleted and replaced by new sections on the same subject. The purposes of the changes are 1) be more precise on the definition of relevant parameters and scenarios; 2) extend the analysis on WSD EIRP limits previously carried out for fixed outdoor DTT reception to portable DTT reception modes; 3) improve the presentation/discussion of results by adding curves of location probability, location probability degradation, and maximum WSD EIRPs as a function of the DTT wanted median field strength.]

Ewmed_refEwmed_ref

|  |  |  | PRΔf1PRΔf1 | PRΔf2PRΔf2 | OthOth |
| --- | --- | --- | --- | --- | --- |

EwmedEwmed

Ei [dBμV/m] ~ NEimed, σi=3.5Ei[dBμV/m]~NEimed,σi=3.5EimedEimed

| LP= Pr Ew≥ Emin⊕(Ei+PR(Δf)) |  |
| --- | --- |

⊕LPEimedEimedEwmedEwmedLPLPPR0-PR(Δfi)

EwmedEwmed= Ewmed_ref=56.21 dBμV/mEwmedEwmed=Ewmed_ref+30 dBEimed_max [dBμV/m] Eimed_maxEimed_max [dBμV/m] Eimed_maxPR(Δf1)PR(Δf2)Eimed_maxEimed_max

OthOthEimed_maxEwmedEimed_max=96.49 dBμV/mEimed_max=99.16 dBμV/mEimed_max=101.55 dBμV/m

|  | Ewmed | Eimed_max [dBμV/m] | Eimed_max [dBμV/m] |  |  | ΔLP |  | LP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PR(Δf1) | PR(Δf2) |  | Δf1 | Δf2 | Δf1 | Δf2 |
|  | Ewmed_ref |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+5 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+10 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+15 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+20 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+25 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+30 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+5 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+10 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+15 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+20 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+25 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+30 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+5 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+10 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+15 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+20 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+25 dB |  |  |  |  |  |  |  |  |
|  | Ewmed_ref+30 dB |  |  |  |  |  |  |  |  |

ΔLPLP

ΔLPΔLPLPEwmedEwmed=Ewmed_ref+25 dBΔLP=0.11%LP=99.89%OthEimed_max=96.49 dBμV/mΔLP=1.01%LP=98.99%OthEimed_max=99.16 dBμV/mΔLP

ΔLPsΔLPs

ΔLP=0.1%ΔLP=0.1%Ewmed_refEwmed=Ewmed_ref+30 dBEwmed=Ewmed_ref+20 dB

LP≥98.93%Ewmed≥Ewmed_ref+5 dB

OthLP=95%

LP≥97.39%Ewmed≥Ewmed_ref+5 dB

ΔLPΔLPOthLP=95%ΔLP>1%ΔLP>1%LPLP

ΔLPΔLP

  - a5.15 scenarios and relevant parameters

It is important to define upper limits for the location probability degradation and the WSD EIRP according to the type of WSD and the DTT reception mode to be protected. There are three possible DTT reception modes: fixed outdoor, portable outdoor and portable indoor DTT reception. The considered WSD types are the ones present in Scenarios 1 to 8, in Annex 2 of complementary report A2: portable WSD (Scenarios 1, 2, and 3), fixed WSD at 10 m agl. (Scenarios 4, 5, and 6), and fixed WSD at 30 m agl. (Scenarios 7 and 8). Simulation parameters presented in Annex A5.8 of complementary report A2 for each scenario are adopted. Then, for each WSD type, limits for the location probability degradation and the WSD EIRP are presented for each DTT reception mode.

Table 1 presents relevant parameters for the three DTT reception modes: the wanted DTT median field strength at the DTT receiver in the coverage edge (LP = 95%), Ewmed_ref, and reasonable values of protection ratio for co-channel, first, and second adjacent channels. Fixed outdoor DTT reception becomes possible with 56.21 dBμV/m wanted median field strength at 10 m agl. For portable outdoor DTT reception, it is needed a wanted median field strength at 1.5 m agl. of at least 61.21 dBμV/m, which corresponds to Ewmed = 78.21 dBμV/m at 10 m agl., with 17 dB height loss. For portable indoor DTT reception, 62.95 dBμV/m median field strength is needed at the DTT receiver (1.5 m agl.). In this case, the corresponding median field strength at 10 m agl. is 87.95 dBμV/m, where in addition to 17 dB height loss, 8 dB wall loss is considered.

Table 1: Wanted DTT median field strength at the coverage edge (LP = 95%) and protection ratio.

| DTT reception mode | Ewmed_ref [dBμV/m] | Co-channel protection ratio PR(0) [dB] | 1st adj. channel protection ratio PRΔf1 [dB] | 2nd adj. channel protection ratio PRΔf2 [dB] |
| --- | --- | --- | --- | --- |
| Fixed outdoor | 56.21 | 21 | -30 | -40 |
| Portable outdoor | 61.21 (78.21 at 10 m agl.) | 19 | -30 | -40 |
| Portable indoor | 62.95 (87.95 at 10 m agl.) | 17 | -30 | -40 |

The measurements of LTE interference into DVB-T receivers reported in ECC Report 148 and considered in ECC Report 159 are used to define the overloading thresholds according to the types of (LTE-like) WSD transmitter and DTT receiver, as shown in Table 2. Values obtained for the silicon USB antenna type are adopted as overloading thresholds for portable DTT receivers. For fixed DTT receivers, can and silicon set-top box and/or integrated TV antenna types are considered. Measurements of overloading threshold for portable WSD – fixed DTT receiver indicate a wide range of possible values, from which -20 dBm is adopted for both first and second adjacent channels.

Table 2: Overloading threshold.

|  | Fixed DTT |  | Portable DTT |  |
| --- | --- | --- | --- | --- |
|  | 1st adjacent channel overloading threshold OthΔf1 [dBm] | 2nd adjacent channel overloading threshold OthΔf2 [dBm] | 1st adjacent channel overloading threshold OthΔf1 [dBm] | 2nd adjacent channel overloading threshold OthΔf2 [dBm] |
| Fixed WSD | -13 | -8 | -26 | -22 |
| Portable WSD | -20 | -20 | -27 | -47 |

Protection ratio and overloading threshold have strong influence on the upper limits of permissible WSD interference at the DTT receiver, and consequently on the limits of location probability degradation and WSD EIRP. The methodology presented in this annex is applicable to any other values for these parameters, which can arise with the development of the actual WSDs.

  - a5.16 main results

The upper limits of ΔLP and WSD EIRP, and the resulting LP are presented below for the parameters given in Table 1 and Table 2. The reference geometries are given in Annex 2 or derived from it and A5.8 (case of portable indoor DTT reception scenarios).

The first set of results is shown in Figure 1 for fixed outdoor WSD at 10 m agl., where the protection of a DTT receiver at the reference geometry is guaranteed for X = 99.9% of locations inside a pixel (quasi perfect DTT receiver operation with respect to interference). For fixed outdoor DTT reception (blue curves), the WSD EIRP upper limits for the second adjacent channel vary from about -2.6 dBm at the coverage edge to 29.75 dBm at locations where Ewmed≥91.21 dBμV/m (wanted median field strength at least 40 dB higher than at the coverage edge), when the maximum permissible interference is limited by the overloading threshold. The limiting scenario in this case is Scenario 4 in Annex 2. The corresponding ΔLP upper limit decays from 0.91% at the coverage edge to about 0.1% in the region where Ewmed≥91.21 dBμV/m (ΔLP is almost negligible for Ewmed≥126.21 dBμV/m). The curve of DTT LP which results from the WSD interference indicates that already in locations where the wanted median field strength is only 5 dB above the one at the coverage edge, the resulting LP is around 99%.

If the protection of portable outdoor and portable indoor DTT reception modes is considered, red and black curves in Figure 1 indicate the upper limits of ΔLP and WSD EIRP, and the resulting LP. For portable outdoor DTT reception (red curves), the behavior of ΔLP upper limits and resulting LP is essentially the same as for fixed outdoor DTT reception, with similar values but for different coverage areas. The limiting scenario is Scenario 5 in Annex 2. As expected, the protection of this DTT reception mode imposes more restrictive upper limits for the EIRP of fixed outdoor WSD at 10 m agl. The protection of portable indoor DTT reception (Scenario 6 in Annex 2) for locations where Ewmed at 10 m agl. is at least 87.95 dBμV/m also imposes new restrictions in this region: the WSD EIRP upper limit is about 11.5 dBm at the coverage edge and increases until 29.2 dBm for Ewmed≥108 dBμV/m (wanted median field strength 20 dB higher than at the coverage edge). The corresponding ΔLP upper limits for portable indoor DTT reception are lower than for the other reception modes, going from 0.25% at the coverage edge to 0.1% with the increase of Ewmed.

Figure 1: Fixed outdoor WSD transmission at 10 m agl. – Upper limits for WSD EIRP at the 2nd adjacent channel and the ΔLP, and the resulting LP for the protection of a DTT receiver at the reference geometry for X = 99.9% of locations inside the pixel.

The EIRP upper limits for fixed WSD at 10 m agl. in the first adjacent channel are shown in Figure 2. Similar overall behavior, but more restrictive EIRP values are observed.

Figure 2: Fixed outdoor WSD transmission at 10 m agl. – Upper limits for WSD EIRP at the 1st adjacent channel for the protection of a DTT receiver at the reference geometry for X = 99.9% of locations inside the pixel.

The set of results for fixed outdoor WSD at 30 m agl. is shown in Figure 3. Once more it is considered the protection of a DTT receiver at the reference geometry for X = 99.9% of locations inside a pixel (quasi perfect DTT receiver operation with respect to interference). The ΔLP upper limits and the resulting LP are very similar to the ones obtained for fixed outdoor WSD at 10 m agl., but the WSD EIRP upper limits are higher because of the elevated coupling loss between WSD transmitter and DTT receiver. For the protection of fixed outdoor DTT reception (Scenario 7 in Annex 2), the WSD EIRP upper limits in the second adjacent channel increase from 22 dBm at the coverage edge (Ewmed=56.21 dBμV/m) to 51.45 dBm for locations where Ewmed≥86.21 dBμV/m. More restricted upper limits are obtained if portable DTT reception is taken into account. In the case of portable outdoor DTT reception (Scenario 8 in Annex 2), at locations where Ewmed=78.21 dBμV/m at 10 m agl., WSD EIRP is limited to 26.3 dBm, and increases with Ewmed until Ewmed=98.21 dBμV/m, when it is limited by the overloading threshold to 43.7 dBm. Similar behavior is observed if the protection of portable indoor DTT reception is considered, with WSD EIRP limited to 24.81 dBm at the coverage edge (Ewmed=87.95 dBμV/m at 10 m agl.), and 42.4 dBm for Ewmed≥108 dBμV/m.

From the results presented above, EIRP “masks” can be derived for fixed outdoor WSD. Figures 4 and 5 show the EIRP masks for the second adjacent channel for WSDs at 10 m and 30 m agl., respectively. If only fixed outdoor DTT reception is to be protected, the EIRP “masks” are given by the blue curves only. Depending on the values of relevant parameters (protection ratios, overloading thresholds, level of protection for the DTT receiver) the EIRP upper limits can change.

Figure 3: Fixed outdoor WSD transmission at 30 m agl. – Upper limits for WSD EIRP at the 2nd adjacent channel and the ΔLP, and the resulting LP for the protection of a DTT receiver at the reference geometry for X = 99.9% of locations inside the pixel.

Figure 4: EIRP “mask” for fixed outdoor WSD transmission at 10 m agl. (2nd adjacent channel).

Figure 5: EIRP “mask” for fixed outdoor WSD transmission at 30 m agl. (2nd adjacent channel).

Now the results for portable WSD are shown in Figure 6, where the upper limits of ΔLP and WSD EIRP (second adjacent channel), and the resulting LP are calculated for the protection of the DTT receiver for X = 99.9% of locations inside the pixel. The considered overloading thresholds are given in Table 2.

To protect the fixed outdoor DTT reception (Scenario 2 in Annex 2, blue curves in Figure 6), the portable WSD EIRP is limited to -2.6 dBm at the coverage edge (Ewmed=56.21 dBμV/m) and increases linearly with Ewmed until Ewmed=76.21 dBμV/m, from where the WSD EIRP is limited by the overloading threshold to 14.75 dBm. The corresponding ΔLP upper limits vary from about 0.91% to 0.1% when entering the DTT coverage area, and achieve negligible values for Ewmed≥126.21 dBμV/m. As in the case of fixed outdoor WSD, the LP which results with portable WSD interference is above 99% for locations where Ewmed is 5 dB above Ewmed at the coverage edge.

Because of the very low overloading threshold of portable DTT receiver with respect to portable WSD interference, -47 dBm, as indicated in Table 2 (from ECC Report 148), the protection of portable DTT reception restricts the portable WSD EIRP to a very low value, -25.2 dBm, for any location inside portable DTT coverage area. The relaxation of the protection criterion from X = 99.9% of locations to X = 99% or X = 90% of locations where the DTT receiver at the reference geometry (2 m distant from the WSD) is protected leads to still low and constant WSD EIRP limits like -22.5 dBm and -18.9 dBm, respectively.

To illustrate the strong influence of overloading threshold values on the WSD EIRP upper limits, Figure 7 shows the EIRP upper limits for the operation of portable WSD in the first adjacent channel, whose overloading threshold is -27 dBm (from ECC Report 148). It is thus observed that the behaviour of WSD EIRP upper limits in this case is similar to the fixed WSD case. Therefore, according to the restrictions imposed by the overloading threshold, an EIRP “mask” for portable WSD transmission would follow: 1) the shape of WSD EIRP curves in Figure 6, with a constant EIRP value in the region of portable DTT coverage; or 2) the shape shown in Figure 7.

Figure 6: Portable WSD transmission – Upper limits for WSD EIRP at the 2nd adjacent channel and the ΔLP, and the resulting LP for the protection of a DTT receiver at the reference geometry for X = 99.9% of locations inside the pixel.

Figure 7: Portable WSD transmission – Upper limits for WSD EIRP at the 1st adjacent channel for the protection of a DTT receiver at the reference geometry for X = 99.9% of locations inside the pixel.

  - [INdT editor’s note: The text of the concluding section of Annex 5, if it is needed, is proposed below.]

  - a5.17 Conclusions

The two approaches presented in this Annex provide location specific maximum EIRP limits for different types of WSDs and DTT reception modes. This content serves as guidance for Administrations, since it discusses first the adoption of a fixed level of degradation of the DTT coverage quality as protection criterion (A5.2 – A5.9), and then the adoption of variable levels of degradation of the DTT coverage quality according to the capabilities of the DTT receivers in handling interference (A5.10 – A5.16).

INdT (Nokia Institute of Technology) is a non-profit research and development institute founded by Nokia in Brazil.