|  |  | SE43(12)CallConf03 |
| --- | --- | --- |
| #### SE43 Web meeting # 2 #### 23 May 2012, 9h30-18h00 CET |  |  |
| #### Date issued: | #### 14 March 2012 |  |
| #### Source: | #### Portugal, EBU, INdT, Sony |  |
| #### Subject: | #### Revisions to Section 5 and annexes of the working document towards Report A2 (Geo-location) |  |

| #### Summary: | #### This contribution was prepared in order to: #### - revise section 5; #### - revise the annexes of the working document, to increase the rediability of the document. The revisions have considered: #### - deletion of annex 4 (EBU is analysing this proposal); #### - deletion of annexes 6 and 7 (merged to annex 3) #### - merge of section from annex 5 (A5.2, A5.4 and A5.5) in new annex 4. |
| --- | --- |
| #### Proposal: | #### To consider by SE43 group the proposed revisions. |
| #### Background: | #### During last SE43 meeting the group was able to agree the text of the working document, except section 5 and annexes which were not considered. This contribution aims to revise the remain sections in order to increase the rediability of the document. |

| Password protection required? (Y/N) | N |
| --- | --- |

Annex 2 to
SE43(12)39

ECC Report <No>

Technical and operational requirements for the operation of white space devices under geo-location approach

Month YYYY (Arial 9pt bold)

(last updated: Month YYYY) (Arial 9pt)

# Executive summary

Body text (style: ECC Paragraph)

(advice: the Executive Summary should provide a short and concise explanation on the purpose of the respective ECC Report and should clearly indicate the covered subjects to which it applies. In addition, it should clearly explain the application of the document.)

TABLE OF CONTENTS

0	Executive summary	2

1	Introduction	6

2	Principles and general considerations	7

2.1	Considerations on locations	7

3	Requirements for WSD	8

3.1	Technical information to be communicated to the geo-location database	8

3.1.1	Considerations on the device elevation	8

3.1.2	WSDs requiring less than the maximum allowed power	8

3.2	Technical information to be communicated to the WSD	8

3.3	Operational requirements	8

4	Technical aspects of the geo-location database management	9

4.1	Considerations on the implementation of the geo-location process for the protection of pmse	10

5	Translation process for the protection of different services	11

5.1	Protection of the broadcasting service in the band 470-790 mhz	11

5.1.1	Time and location probability	11

5.1.1.1	Time probability	11

5.1.1.2	Location probability	13

5.1.2	Methodologies to calculate local-specific wsd power levels	13

5.1.2.1	Monte Carlo simulation	14

5.1.2.2	Analytical calculations	15

5.1.2.3	WSD out-off-block emission limit	18

5.1.3	Alternative methods to specify the local-specific output power level of wsds	18

5.1.4	Harmonized parameters to calculate location specific wsd power levels	21

5.1.5	Example of a translation process for the protection of dtt	24

5.1.5.1	DB Data Storage	25

5.1.5.2	Data for Calculation	25

5.1.5.3	Data for WSD characteristics	27

5.1.5.4	Storage of authorised WSD	28

5.1.5.5	Interference Management Methodology	30

5.1.5.6	Calculation of the interfering fields	34

5.2	Protection of PMSE in the band 470-790 MHz	37

5.2.1	Methodology for protection of PMSE	37

5.2.1.1	Prevention of co-channel interference	38

5.2.1.2	Prevention of Adjacent channel interference	39

5.3	Protection of RAS in the band 608-614 MHz	40

5.4	Protection of ARNS in the band 645-790 mhz	41

5.5	Protection of services in the bands adjacent to 470-790 MHz	41

5.5.1	General	41

5.5.2	Protection of mobile service in the band 790-862 MHz	41

6	master/slave concept	43

6.1	Definitions	43

6.2	General principles of master/slave concept	43

6.3	Requirements for master WSDs	43

6.3.1	Technical information to be communicated from a master WSD to the geo-location database	43

6.3.2	Technical information to be received by a master WSD from the geo-location database	43

6.3.3	Operational and security requirements	43

6.4	Requirements for slave WSDs	43

6.4.1	Technical information to be communicated from a slave WSD to a master WSD	43

6.4.2	Technical information to be received by a slave WSD from a master WSD	44

6.4.3	Operational requirements	44

7	Combined sensing and geo-location	45

7.1	Methodology	45

7.2	Algorithm	45

8	Conclusions	46

ANNEX 1: MULTIPLE-INTERFERENCE	47

ANNEX 2: Interference scenarios	54

APPENDIX 1 OF ANNEX A2	65

APPENDIX 2 OF ANNEX A2: VERTICAL RADIATION PATTERN OF THE WSD ANTENNA FOR FIXED WSD TRANSMISSION AT 30 M	76

ANNEX 3: Number of WSD and Nuisance Power	77

ANNEX 4: DTT COVERAGE AREA AND RECEPTION MODE CONSIDERATIONS	88

ANNEX 5: CONSIDERATIONS OF DEGRADATION OF COVERAGE LOCATION PROBABILITY FOR THE DETERMINATION OF MAXIMUM WSD EIRP LIMITS	103

ANNEX 6:	143

ANNEX 7:	150

ANNEX 8: PMSE Reference geometries	152

ANNEX 9: applications EXAMPLE(s) of master/slave concept	153

ANNEX 10: Trade-off between ‘false-vacancy-detection’ and ‘false-occupancy-detection’ as a function of increasing detection thresholds	154

ANNEX 11: Preliminary results on combination of geo-location database and sensing techniques in a real scenario (from Doc. 96)	165

ANNEX 12: Methodology used to construct a geolocation database for WSD deployment in the UK	172

Appendix to ANNEX A12 Derivation of the Maximum Protected Field Strength 	182

ANNEX 13: List of reference	185

Note on the Table of Contents (delete after reading)

This is automatically styled and compiled from the headings, subheadings and page numbers from the document that follows. To update the Table of Contents move cursor within the table and press F9.

LIST OF ABBREVIATIONS

| Abbreviation | Explanation |
| --- | --- |
| CEPT | European Conference of Postal and Telecommunications Administrations |
| ECC | Electronic Communications Committee |
| EBU | European Broadcasting Union |
| ACLR | Adjacent Channel Leakage Ratio |
| ACS | Adjacent Channel Selectivity |
| BPL | Building Penetration Loss |
| BS | Broadcasting Service |
| CDF | Cumulative distribution function |
| CRS | Cognitive Radio System |
| DB | Data base |
| DTT | Digital Terrestrial Television |
| DTV | Digital Television |
| DVB-T | Digital Video Broadcasting – Terrestrial |
| DVB-H | Digital Video Broadcasting- Handheld |
| e.i.r.p. | Equivalent isotropically radiated power |
| ENG/OB | Electronic News Gathering outside broadcast |
| ETSI | European Telecommunications Standards Institute |
| FCC | Federal Communications Commission |
| IEEE | Institute of Electrical and Electronics Engineers |
| IMD | Inter-Modulation Distortion |
| IM | Interference margin |
| LIM | Limiting nuisance power |
| LP | Location Probability |
| M-to-M | Machine-to-Machine communication |
| MI | Multiple interference margin |
| PR | Protection Ratio |
| PM | Propagation model |
| PMSE | Program Making and Special Event |
| POL | Polarisation discrimination |
| PWMS | Professional wireless microphone systems |
| RAS | Radio astronomy service |
| RFSENS | Reference Sensitivity |
| Oth | Overloading threshold |
| OOB | Out-of-Band |
| QAM | Quadrature amplitude modulation |
| PWMS | Professional Wireless Microphone system |
| SEAMCAT | Spectrum Engineering Advanced Monte Carlo Tool |
| SINR | Signal to interference plus noise ratio |
| TVWS | TV White Spaces |
| WSD | White Space Device |

# Introduction

# Principles and general considerations

## Considerations on locations

Figure 1: Illustrative example of geo-location database pixel granularity

# Requirements for WSD

## Technical information to be communicated to the geo-location database

Figure 2: Relation of database pixels to cognitive device location accuracy. “Location” of the WSD is always an area and may comprise of one or more database pixels

### Considerations on the device elevation

### WSDs requiring less than the maximum allowed power

## Technical information to be communicated to the WSD

## Operational requirements

# Technical aspects of the geo-location database management

There are several issues related to the geo-location database management:

- Technical information on services/systems to be protected

This information, that should be loaded into the database, could either be a set of transmitter parameters (including location, height, transmit power, etc.) or a grid of pixels associated with characteristics of the received signals of incumbent services to be protected (see § 5.1.4) or some combination of the two.

- Database update delay

Database update delay is the period within which the database should be updated once protected services/systems provide a notice of a change in their assignments. If the assignment updates are provided electronically over the Internet the update delay may be very short. The assignments may be provided to each database directly, or distributed from one central database depending on the requirements set by the National Authority. In the latter case the update delay defines the delay through the whole link of interconnected databases.

- Database update frequency

Database update frequency is the periodicity with which the database should be updated so that the information it contains remains valid. This may be especially valid in case there is a master database and a number of query databases, which the WSD’s will consult, and which have to have identical data related to incumbent use.

The appropriate update frequency will depend on the rate at which the assignments of the protected services/systems change and the notice provided. In general, protected services may need a rapid update as this will provide them with flexibility to make rapid changes to their assignments; this holds especially for PMSE and for DTT used in cases of events or field trials.

The type of protected services/systems and the speed with which they change their use of the spectrum may vary from country to country.

- Translation of the information provided to the database into the basic elements in the database

The database would have to convert the incumbent information provided to the database into a list of allowed frequencies and associated transmit powers to WSDs. Hence, a translation must be performed between these two.

It is clearly critical that this translation is performed appropriately. If it is not then there is a risk either of interference occurring to the protected services or of the WSDs’ access to the spectrum being limited unnecessarily.

The translation mechanism depends on the type of a protected service and its coverage information already available. The cases when a WSD in one country could potentially interfere with a protected service in a neighboring country should be taken into account in the translation process.

The database will provide WSDs with a maximum power level that it can use in a given location and for a particular frequency range. In arriving at these data, the algorithms employed need to ensure that a device in that location – plus a certain area of location uncertainty – transmitting with the given power level will not cause harmful interference to a protected service.

The algorithms need to know the potential location of receivers to be protected, the level of interfering signal they can tolerate before the interference becomes harmful and the propagation loss between the WSD and the receiver. If all these are known then the WSD transmit power can readily be determined.

An example of the translation process is provided in § 5.1.5. The algorithms used in the database may also take into account the frequencies and powers used by other WSDs.

## Considerations on the implementation of the geo-location process for the protection of pmse

For PMSE, there are two categories of concerns: administrative and technical. On the administrative side, it is crucial that PMSE users who need interference protection can conveniently register their locations in the database. The registration process must be straightforward and easy to complete so that it can be done quickly. This will be especially important for users whose plans must change suddenly or which operate systems under a nationwide (general) license or under a licence-exempt regime.

There are several technical challenges that are of interest to PMSE users. One is the way the location of the PMSE equipment is to be registered. If the equipment works in a certain location, a street address is usually known, but not necessarily the coordinates. Another issue is the determination of an operational area if the PMSE equipment is not in a certain “fixed” location. A third issue is that the PMSE equipment location may change in a short time.

The question of how frequently the database should be updated, and how often WSD should re-consult it is of great interest to PMSE users. This is in particular true for PMSE applications TV-productions and Electronic News Gathering. It could be beneficial, if the PMSE registration would also contain an indication of the time duration that the PMSE equipment is to stay operational in the current location.

The protection zone approach could be used in connection with any registered PMSE. This zone is drawn around the PMSE receiving antennas. When an administration is employing safe-harbour channels to accommodate PMSE needs (see § 5.6 of ECC Report 159 for the discussion on the safe-harbour), the geo-location database will simply exclude these channels from the list of candidate channels for WSDs.

# Translation process for the protection of different services

As stated in Section 4, through the translation process the geo-location database will determine a list of allowed frequencies for WSDs and associated transmit powers. In particular, in-block and out-of-block emission levels will need to be set, which are defined as follows:

- In-block emissions are emissions corresponding to those segments of a radiated signal’s frequency spectrum which carry information intended for a receiver. The width of the in-block segment of the frequency spectrum is the nominal bandwidth of the signal.

- Out-of-block emissions are emissions corresponding to those segments of a radiated signal’s frequency spectrum (outside the in-block segment) which correspond to unintended radiations.

## Protection of the broadcasting service in the band 470-790 mhz

Broadcasting is a primary service in the 470-790 MHz band. Normally reception is protected with a 95% location probability at the DTT coverage edge. Broadcasting caters to various reception modes (e.g. fixed, portable,...) in various environments (e.g. rural, urban, …).

The minimum median field strength values at the DTT coverage edge that correspond to some of these reception situations are provided in Table 5.1.

Table X: DTT reception modes and corresponding minimum medium field strengths

|  | Fixed rural | Fixed urban | Fixed HDTV (DVB-T2) urban | Portable indoor urban | Portable outdoor / Mobile urban | Mobile TV (DVB-H) Handheld Indoor urban | Mobile TV (DVB-H) Handheld outdoor |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rx height (m) | 10 | 10 | 10 | 1.5 | 1.5 | 1.5 | 1.5 |
| Field strength (dBµV/m) | 56.21 | 56.21 | 56.21 | 70.95 | 61.21 | 77.49 | 56* |
| * Evaluated for 90% location probability (all others evaluated at 95% location probability) |  |  |  |  |  |  |  |

### Time and location probability

Location probability criterion for protecting the wanted DTT signal is usually combined with time probability: 95% of location probability for 99% of the time (i.e. using the 1% time propagation curves for the interfering sources). Protecting against the 1%-time interfering signals ensures that, when the interference does occur, it is limited to a very small duration in time (i.e. 1% of the time).

This is done when protecting broadcast against broadcast interference, and the interference is coming from outside the victim DTT coverage area.

#### Time probability

In the presence of noise only, digital television transmissions provide a very stable reception quality when the wanted signals are ‘sufficiently’ strong.

The parameter which quantifies the word ‘sufficiently’ is the carrier to noise ratio, [C/N]. Laboratory measurements to determine the required [C/N]ref value are usually carried out using ‘constant’ wanted signal strength and constant noise level.

For the wanted signal, these temporally ‘constant’ conditions usually prevail within any DTT coverage area in the following sense:

- In Figure 1, a ITU Rec. P. 1546 propagation curves are indicated for 1% and 50% of the time. The X% curve gives median (location) field strength values, Emed, which are exceeded X% of the time.

- The curve which would give median (location) field strength values which are exceeded for 99% of the time, E99%, is virtually identical to the 50% time curve. Thus choosing [Emed/N] ≥ [C/N]ref is essentially the same as choosing [E99%/N] ≥ [C/N]ref, in other words the [C/N]ref limit is met or exceeded for 99% time, i.e . ‘always’ or ‘constant’.

- The 1% time median (location) signal, however, behaves very much differently than the 50% time median (location) signal. There is a difference between the 1% time curves and the 50% time curves, and this difference increases with propagation path distance. For small propagation distances (< 3 km), the difference is small (< 1 dB), growing larger (2 dB to 10 dB or more) as the propagation distance increases (5 km to 100 km, or more).

Figure 3: Comparison between 1% and 50% time curves

When planning broadcasting networks (e.g. at RRC06) the protection criteria are based on a ‘constant’ (50% time) median (for 50% locations) wanted signal, and a 1% time median (for 50% locations) interfering field strength.

As seen in Figure 3, for short propagation distances, less than 1 km or 2 km, the difference between the 1%-time (‘rare’) interference level and the 50%-time interference level is practically nil. This means that WSD interference sources inside a DTT coverage area, arriving over short distances, is nearly constant. In other words, the interference effects (e.g. DTT dropout when it occurs) are essentially continuous (i.e. ‘all the time’).

For this reason, when dealing with short range WSD interference to DTT reception (i.e. with distances less than 1 or 2 km), where the interference is effectively constant in time, extremely stringent location probability restrictions must be imposed. In the past, when planning for analogue television, an additional 10 dB or so, was added to the relevant protection ratios when the interference was continuous (i.e. for 50% time instead of 1% time).

#### Location probability

##### Theoretical planning and protection aspects

DTT location probability (LP) is defined as the probability with which DTT reception would be acceptable at any site within a ‘small’ area; i.e., it is the probability with which the wanted signal level is appropriately greater than a minimum required value at any site within the small area.

Location probability is widely used in the planning of DTT networks in terms of protection of the broadcast coverage edge against other (usually distant) broadcast networks. If a minimum agreed reference LP (e.g. 95%) is reached everywhere on the DTT coverage edge for 1% of the time, the coverage area is considered covered in the sense that the sites interior to the DTT coverage area have an even higher LP.

In order to quantify the quality of coverage throughout a DTT service area, LP is typically calculated for every 100 m  100 m pixel across the country. The existence of any additional interference naturally results in a reduction of the planned DTT location probability. Such a reduction in LP is therefore a highly suitable metric for specifying regulatory emission limits for WSDs operating in DTT frequencies.

In analytical terms, we can discuss LP as follows. Consider a pixel where the DTT location probability is q1 in the presence of noise and interference. Then it can be writen (in the linear domain):

(5.1-1)

where:

- Prob{A} is the probability of event A;

- PS is the received power of the wanted DTT signal;

- PS,min is the DTT receiver’s (noise-limited) reference sensitivity level;

- PU,i is the (1%-time) received power of the ith unwanted signal;

- rU,i is the protection ratio for the ith interferer.

In the planning of DTT networks, PS(dBm) and each individual PU,i(dBm) are modelled as Gaussian random variables. U represents the random variable which is the power sum of the individual interference sources.

Note that in Equation (5.1-1), the (statistically varying) powers are summed in the linear domain. For this reason, the most accurate way of calculating the probability q1 is to use a Monte Carlo simulation where a large number of trials are performed with values for each variable generated according to their Gaussian distribution. Such a Monte Carlo approach is described in section 5.1.2.1. In addition, an approximate analytic approach is described in section 5.1.2.2.

### Methodologies to calculate local-specific wsd power levels

If WSDs are to be used on a non-interference basis, strict rules for determining operating WSD eirp levels must be developed to ensure that DTT reception is caused minimal interference.

The calculation of regulatory emission limits for an autonomous WSD for operation in DTT bands has to be based on worst-case geometries (see Annex 2 for reference protection scenarios) between the interfering WSD and the victim DTT receiver. Consequently, adequate protection of the DTT service can result in very stringent (i.e. low) regulatory emission limits for the WSD.

The distance between the WSD and the DTT receiver, dWSD_DTT, defines the allowed minimum operational distance separating a WSD from a DTT receiving antenna. This distance, together with the specified protection criterion (e.g. permitted degradation of DTT location probability), allows its emission limits to be calculated.

WSDs can cause interference to DTT reception in two different ways:

  - WSDs operating inside of a DTT coverage area (see Annex 5)

Because of the extremely high interference potential of co-channel usage, the WSDs will be limited to using frequencies adjacent to that (or those) of the DTT service(s) in the area in question. Consideration of ‘short distance’ WSD interference provides one of the main constraints on WSD eirp limits, in particular because of its ‘continuous’ effect. In particular, several ‘reference interference scenarios’ have been developed (see Annex 2) to calculate the ‘nearby’ interference effects. Monte Carlo techniques as well as approximate analytical formula can be used to determine WSD eirp limits for each scenario.

  - WSDs operating outside of a DTT coverage area (see Annex 6)

In case a), because of the extremely high interference potential of co-channel usage, the WSDs will be limited to using frequencies adjacent to that (or those) of the DTT service(s) in the area in question. Consideration of ‘short distance’ WSD interference provides one of the main constraints on WSD eirp limits, in particular because of its ‘continuous’ effect. In particular, several ‘reference interference scenarios’ have been developed (see Annex 2) to calculate the ‘nearby’ interference effects. Monte Carlo techniques as well as approximate analytical formula can be used to determine WSD eirp limits for each scenario.

Due to greater separation distances between WSD interferer and DTT receiver, potential co-channel interference will be the most significant, but care must also be taken with respect to adjacent channel usage. Consideration of ‘longer distance’ WSD interference provides another of the main constraints on WSD eirp limits (especially for high power WSD base stations). Monte Carlo techniques as well as approximate analytical formula can be used to determine WSD eirp limits for each scenario.

In accordance with Equation 5.1-1 specifying the calculation of the relevant DTT LP, broadcast transmissions are protected according to a set of restrictive parameters. For example, specific protection ratios must be maintained, minimum location probabilities must be achieved or exceeded, minimum time probabilities for acceptable reception must be achieved or exceeded, etc.

This calculation can be carried out using Monte Carlo simulations (the most accurate determination) or analytic approximations. These approaches are discussed in the following two subsections.

#### Monte Carlo simulation

Monte Carlo simulations can be used to determine location probability in situations where a statistically varying wanted DTT signal is present and also noise and one or more statistically varying interfering sources are present. In other words, Monte Carlo simulations can be used to evaluate equation 5.1-1 in an exact manner.

To briefly sketch how a Monte Carlo simulation is effected, we first list the relevant parameters that would be included and indicate how they are used:

- EW_med, σW: median wanted DTT field strength and standard deviation;

- PRi: the appropriate protection ratio(s)  corresponding to the relevant frequency offset(s) f;

- EWSD_med_J, σWSD_J: median WSD interfering field strength and standard deviation for J interferers;

- POLJ, ATTJ, DISCJ: polarization discrimination(s), transmit antenna attenuation(s), receive antenna discrimination(s).

In a Monte Carlo simulation, a large number of ‘trials’ are made in which the statistical variables are used; and on the basis of the statistics of the trials, probabilities (in our case, location probabilities) can be calculated.

For each trial the following calculations are carried out:

- a random wanted DTT field strength is calculated using

EW = EW_med + random (Gaussian, σW) variation

- random interfering WSD field strengths, for each interferer J, are calculated using

EWSD_J = EWSD_med_J + random (Gaussian, σWSDJ) variation;

- The corresponding nuisance fields, NUWSD_J, are calculated using the EWSD_J with the relevant protection ratios, POL, DIR, etc;

- The power sums for the NUWSD_J and the noise nuisance field NUN are carried out for each trial, leading to a ‘total’ nuisance field NUsum, which is compared to the trial value of Ew.

- The ratio of the number of trials where EW ≥ NUsum to the total number of trials gives the location probability, LP, in the presence of the interfering WSDs and the noise.

In cases where the degradation in LP is to be calculated when WSD interference is introduced, it is necessary to carry out two complete Monte Carlo simulations, one for the initial situation without WSD interference (giving LPbefore), and the second with the WSD interference included (giving LPafter). The degradation in LP, LP, is calculated as LP = LPbefore – LPafter.

#### Analytical calculations

To simplify the mathematical notation in Equation 5.1-1, we introduce a few new terms.

(5.1-2)

PS,min is the minimum wanted signal power with which the receiver can operate correctly in a noise-limited environment. It can be expressed as the sum of the noise power, PN, and the minimum [C/N] ratio. In linear terms, we can write PS,min = rNPN, where rN = 10[C/N]/10. To unify the terminology we write PU,0 = PN and rU,0 = rN. Then Equation 5.1-1 can be re-expressed as follows:

(5.1-3)

q1 expresses the location probability that the wanted DTT power is stronger that the total (power summed) interfering power.

If an additional received interfering power, Pwsd_rec with protection ratio rwsd, is introduced, the location probability will be reduced to q2

(5.1-4)

Pwsd_rec can be expressed, linearly, in terms of the WSD transmit eirp, Pwsd_tx, as

Pwsd_rec = GwsdPwsd_tx 	(5.1-5)

where the coupling gain, Gwsd, includes the path loss, receiver antenna gain, as well as receiver antenna discrimination and polarization discrimination. The coupling gain, Gwsd, expressed in dB, is a log-normal random variable with a median value, mG_dB (dB), and a standard deviation G_dB (dB). The WSD transmitter eirp, Pwsd_tx, is a constant, i.e. it is not variable.

Then Equation 5.1-4 can be expressed as:

(5.1-6)

A new random variable, Z, can be introduced by defining

(5.1-7)

Z, expressed in dB, has a median value, mZ_dBm, and a standard deviation, Z_dB; mZ_dBm and Z_dB can be estimated using numerical techniques such as the Schwartz-Yeh algorithm, the method of moments, or Monte Carlo simulations.

Using these new random variables, Equation 5.1-6 can be reformulated as

(5.1-8)

In dB units, this becomes

(5.1-9)

Lei Shi, et al, have shown what an approximation of Equation 5.1-9 is:

(5.1-10)

But the conditional probability is given by:

,

and

.

So, from Equation 5.1-10 it follows that

(5.1-11)

A further new variable, Z´, is defined as

Z´ (dBm) can be approximated by a log-normal random variable, , which has mean  and standard deviation . These parameters can also be estimated by the method of moments.

Then equation 5.1-11 can be re-written as

(5.1-12)

and the maximum WSD eirp is

(5.1-13)

A margin, IMdB, to account for multiple interfering WSD sources or as a ‘safety factor’, could be added to this expression, if desired.

Equation 5.1-13 summarizes the following information:

- An existing interference situation (represented by a median interference power , a normal distribution with standard deviation ) leads to q1% LP for DTT reception,

- A single additional allowed (e.g., WSD) interferer (represented by a (maximum) transmitted eirp,  , with a median coupling gain, , a log-normal distribution with standard deviation G_dB,  and protection ratio rwsd_dB) leads to q2% LP for DTT reception, where q2 < q1.

- The resulting degradation in LP is LP = q1 – q2.

- Looking at it the other way around, if a desired LP is required, choosing q2 = q1 – LP in equation 12 will provide the maximum single-entry WSD eirp, , which leads to an LP degradation not exceeding the desired LP.

#### WSD out-off-block emission limit

Equation 5.1-13 implicitly specifies the maximum permitted WSD out-of-block eirp through the use of WSD-to-DTT protection ratios.

This is because the protection ratio is a function of both the spectral leakage of the WSD transmitter and the spectral selectivity of the DTT receiver. Specifically, the protection ratio r∆f(dB) is given (in the linear domain) by:

r∆f=PS*PAC*=PS*PI*.PS*PI*=r01ACIR∆f=r0ACLRCR-1∆f+ACSDTT-1∆f 		(5.1-14)

where:

- *: denotes the value at the point of receiver failure;

- PI: is the interference power;

- PAC: is the power of the adjacent channel interferer;

- ACIR: is the adjacent-channel interference ratio;

- ACLRCR: is the adjacent-channel leakage ratio of the WSD transmitter;

- ACSDTT: is the adjacent-channel selectivity of the DTT receiver.

By definition, the maximum permitted WSD out-of-block emission level is given (in the logarithmic domain) as

POOBCRΔf=POOBCR-ACLRCRΔf 		(5.1-15)

#### Alternative methods to specify the local-specific output power level of WSDs

Equation 5.1-13 provides an approximate upper limit for a single WSD interferer eirp. The introduction of the WSD interferer would decrease the DTT LP from q1 to q2. If the allowed degradation of the location probability is specified as LP, then q2 = q1 – LP and Pwsd_tx_dBm is derived from Equation 5.1-13. However, a DTT installation would not be protected if subjected to the interference of two or more WSDs, each interfering at the single-entry eirp limit.

Consider Equation 5.1-13 again, this time with an additional ‘multiple interference margin’, IMdB:

(5.1-16)

It should be noted that the relative frequency offsets between wanted DTT and interfering WSDs are reflected in the protection ratio term, . This means that, for example, three different WSD powers may have the same individual interference effect, because they are perhaps working in different channels, co-channel, 1st adjacent, 2nd adjacent:

Pwsd0 = -50 dBm, Pwsd1 = 0 dBm, Pwsd2 = 10 dBm.

Nevertheless, the same IMdB factor would apply to each of them when attempting to compensate for aggregate interference.

If a set of WSD base stations (or UEs) lies outside of a DTT coverage area, the cumulative interference of those interferers at the DTT coverage edge will force the eirp values of the individual base stations to be reduced compared to the values they could have if they were ‘acting alone’. One way to deal with this situation is to introduce a fixed ‘multiple interference margin’, IM, in the ‘single entry’ protection criteria (as done in Equation 5.1-16 above), in order to make an estimated correction to the total cumulative effect. This could be done on the basis of an assumed number of interferers, e.g. IMdB = 10 log N, if there are N assumed WSD interferers. If the assumed number, N, is larger than the actual number of WSDs, the restriction on the eirp of the WSD base stations would be more severe than necessary. If the assumed number, N, is smaller than the actual number of WSDs, the interference at the DTT coverage edge would exceed the allowed limit.

The following three methods, in the subsections below, could be used to determine the appropriate IMdB value for a multitude of base stations or UEs in a WSD network in order to maintain the required protection of the incumbent service. The first two methods are based on the assumption that N interfering powers add, and a power reduction factor 10 log N dB should provide an adequate compensation.

Fixed/Predetermined IM value

This approach is based on the potential maximum number of interferers in each operational frequency in a given area at the same time which is defined as follows:

(5.1-17)

It would be up to the geo-location data base to determine in each area what the potential ‘maximum number’ of WSD interferers might be. The disadvantage here is that the ‘maximum number’ of WSDs might not be all working at the same time. This situation would impose an additional, unnecessary restriction on WSD usage.

Flexible IM value

This approach is based on the maximum number of active/actual interferers, in a given area operating at the same time, which is defined as follows:

(5.1-18)

It would be up to the geo-location data base to determine in each area what the actual ‘maximum number’ of active WSD interferers is at any given time. The disadvantage here is that some of the WSDs might be working far below the maximum allowed eirp. Thus, the required reduction may be more than actually needed, again imposing an unnecessary restriction on WSD usage, although less than the ‘fixed/predetermined’ approach.

Flexible Minimized IM value

This approach is based on the actual characteristics of each active interferer in each operational frequency of WSD in a given area at the same time.

The flexible minimized margin is ‘fitted’ to correspond to the number of active WSD interferers, and their relative interfering nuisance powers.

The method is carried out in the following steps:

- The desired value of acceptable degradation to LP is selected, e.g., LP = 0.1% (see Annex 5);

- Test pixels, L = 1,…,T, are chosen (e.g., along the DTT coverage edge) for the interference calculation. Within each pixelL, the median wanted DTT power in the receiver is calculated to be Pw_med_L;

- The existing LP, q1L, is calculated for each pixelL taking account of all interference except WSD interference;

For each WSDJ, J = 1, … , K, the single entry WSD transmitter eirp ,

is calculated for each pixeli, according to Equation (5.1-16), using q1, q2 = q1 – LP ,

- and the other known WSDJ parameters.

- For each WSDJ, a ‘trial’ eirp is selected as the minimum of the  over all of the pixels, L=1,…,T,

(5.1-19)

- In the Lth pixelL, the Jth WSDJ single entry nuisance power, , at the DTT receiver is calculated using

(5.1-20)

where GLJ is the coupling gain for the Jth WSDJ with respect to the Lth pixelL.

- For q2% LP, the following inequality holds for each pixelL and WSDJ nuisance power:

(5.1-21)

- Thus, for each pixelL, we can define a maximum allowed nuisance power

(5.1-22)

- The power sum of the single entry nuisance powers of each WSDJ, J = 1,...,K, is calculated at each test pixelL, yielding the ‘overall’ nuisance power at each test pixelL, L = 1,...,T,

(5.1-23)

- A ‘flexible minimized margin’, , is defined as the minimum difference of:

(5.1-24)

for all the test pixels,

α=minL=1,…,TPnuis_LMAX-Pnuis_LOVERALL	(5.1-25)

where  represents a flexible term which is to minimize IMdB every time a new interference situation arises (for example, whenever a ‘new’ WSD starts operation, or an ‘old’ one stops operation).

- The maximum allowed WSDJ eirps, , J = 1, ... , K, are chosen such that:

(5.1-26)

### Harmonized parameters to calculate location specific wsd power levels

A) For adjacent channel interference arising from the use of WSDs inside a DTT coverage area

- Use the reference interference scenarios detailed in Annex 2

- Calculations are made according to the methodology described in section 5.1.3 above;

- For the relevant scenario, the relevant interfering eirp, eirpscen is calculated, according to the prescription given below, so that the chosen LP is not exceeded.

    - UE WSDs:

The allowed interfering, eirpallow, is derived from eirpscen by subtracting IM dB to account for multiple interfering UE WSD sources (e.g. 3 equivalent interferers, assumed identical, gives 5 dB):

eirpallow = eirpscen – IM

    - Fixed WSDs:

Operating with a single adjacent channel frequency: The allowed interfering, eirpallow, is derived from eirpscen by subtracting IM (dB) to account for multiple interfering fixed WSD sources:

eirpallow = eirpscen – IM

Operating with N adjacent channel frequencies: The allowed interfering, eirpallow, is derived from eirpscen by subtracting 10 log N dB to account for the multiple frequencies on the fixed WSD antenna, and by subtracting an additional IM (dB) to account for multiple interfering fixed WSD sources:

eirpallow = eirpscen – 10 log N – IM

- The WSD eirp must be restricted to an absolute maximum level defined by the DTT overloading threshold corresponding to each DTT reception mode and channel adjacency. Limiting WSD eirps are discussed in Annex 5.

Using the procedure described in Annex 3 the total allowable median nuisance field at the DTT coverage boundary is determined. For example, based on LP = 0.1%, a total median aggregate nuisance field limit NUISTOT = 24.545 dBµV/m is calculated as the limiting nuisance field at the DTT coverage edge. Using this median aggregate nuisance field value, the required protection for DTT reception can be based on a simple power sum of the individual median nuisance fields produced within the pixels in question. If necessary, other values of the total median aggregate nuisance field limit NUISTOT can be determined from different required values of LP by using the same procedure.

- Using the procedure described in section 5.1.2 protection can also be afforded DTT reception, although with more calculation and more restrictions on WSD eirp levels.
(The hoghlighet text should BE CHECKED FURTHER BY EBU)

B) The three modes of DTT reception, fixed roof top, portable outdoor and portable indoor should be protected, progressively, from the DTT coverage edge moving inwards toward the DTT transmitter. The switch to each mode is made as function of the wanted field strength level by referring to the thresholds of field strength above which a given mode of reception is possible (going from fixed roof top to portable outdoor to portable indoor). Refer to Annex 4 to more detailed calculations;

C) For the DTTB receive antenna (for fixed reception scenarios), Recommendation ITU-R F.419 has been used in studies to define the reference DTTB receive antenna pattern (see Figure below), for both horizontal and vertical radiation pattern:

Figure 4: Fixed DTT reference receive antenna pattern

In order to determine maximum eirp levels for new (‘high tower’- see Scenarios 7 and 8 of Annex 2) it is necessary to have suitable reference vertical antenna patterns for the WSD transmit antenna.

For compatibility studies, the base station WSD antenna vertical and horizontal patterns derived from Recommendation ITU-R F.1336-2 have been suggested. The vertical pattern is shown in Figure bellow. The largest attenuation for any vertical angle is 22.5 dB.

Figure 5: Vertical WSD base station antenna pattern

D) In relation to the degradation of the location probability several proposals were discussed:

(i) by defining of fixed value of LP for acceptable degradation to the location probability of about LP = 0.1% throughout the DTT coverage area, respecting the most stringent DTT reception modes, i.e., fixed roof top, portable outdoor, portable indoor, according to the ambient DTT field strength

(ii)  - by defining upper limits for the acceptable degradation to the location probability throughout the DTT coverage area, respecting the most stringent DTT reception modes, i.e., fixed roof top, portable outdoor, portable indoor, according to the ambient DTT field strength.

Administrations should consider the appropriate level for the degradation, balancing the usage of white spaces and guaranteeing the protection for the broadcasting service. Annex 5 presents the basis of both approaches as well the implications of the choice of LP level.

F) Location probability depends on the predicted level of the DTT signal and consequently on the propagation model used. The maximum eirp of the WSD will then be derived from the predicted median field strength. If this predicted field strength is over-estimated, due to the imprecision of the field strength prediction model, or the inaccuracy of the transmitter characteristics, the resulting power assigned to the WSD may be too high and consequently may cause interference. The prediction error can be as large as 20 dB.

The practical aspects of DTT viewer loss/inconvenience resulting from the use of calculated values of location probability as a measure of the LP margin which can be allocated for the implementation of WSD services is discussed in section 5.1.1.2.2 and section 3.1.1 of the draft SE43 Report on WSD operation.

As a general conclusion, it is felt that administrations should take these considerations on board when performing the WSD eirp limit calculations.

E) The compliance of a WSD with an unwanted emission level of -65 dBm/8 MHz defined in the ECC Decision(09)03 has an impact on the protection ratio that will be experienced by TV receivers. New studies and measurement campaigns on protection ratios and overloading thresholds (foreseen) for WSD unwanted emission levels are required..

### Example of a translation process for the protection of DTT

For a given geographic pixel, the database must examine all relevant co-channel and adjacent-channel interference scenarios with respect to the victim DTT channels.

For each WSD-to-DTT frequency offset maximum permitted WSD eirp emission levels required for a tolerable level of interference to the DTT services.

The following calculations are be performed for any given pixel where the WSD operates, and for all frequency separations between the WSD’s candidate channel and the DTT channels:

- In order to reduce the computacional time, the geo-location database could contain a set of informations, namely the frequencies, median mS(dBm)ms (dBm) and standard deviation S(dB)σS(dB) of the received DTT signal power/field strength, the median mU(dBm)mU (dBm) and standard deviation U(dB)σU(dB) of the DTT interferer powers/field strengths, as well as the resulting DTT location probability q1 in every geographic pixel. The above parameters can be provided by the national DTT network planning model. In the absence of such a model, the above parameters can be calculated explicitly based on the technical characteristics and locations of the DTT transmitters, as described in Equation 5.1-2;

- The geo-location database translation engine will need tocalculate the median and standard deviation of the coupling loss between the WSD interferer and victim DTT receiver. This requires the use of appropriate propagation models and interferer-victim geometries. The selection of such interferer victim geometries could be assisted by information provided by WSD in a database query (e.g. antenna pointing direction, type of antenna used, etc). For victim DTT channels that are used by the DTT service in the same pixel as the WSD, the coupling gain must be based on a reference coexistence geometry that are deemed suitable in the context of protecting the DTT platform. Such reference geometry is necessary because the precise spatial separation between the WSD and a victim DTT receiver within the given pixel cannot be known by the database. For victim DTT channels that are not used by the DTT service in the same pixel as the WSD, the coupling gain can be based on the actual spatial separation between the pixel where the WSD operates and the pixel where the DTT channel is used by the DTT service.

- The geo-location database will need to assume a certain number of parameters:

  - tolerable degradation q= q1 – q2 Δq=q1-q2in the DTT location probability of pixels where the DTT services are used.

  - the appropriate protection ratios for the WSD.Δfr(Δf)

- The database can readily compute the maximum permitted WSD eirps by the Monte Carlo methodology or the analytical approximations described in sections above.

PIBCRdBm≤mZdBm-mGdB-r∆fdB-μq2σZdB2+σGdB2-IMdB

POOBCRΔf=POOBCR-ACLRCRΔf

#### Database (DB) Data Storage

The following tables give a schematic representation of the data that the data base should contain in order to facilitate the required calculations.

#### Data for Calculation

For each pixel, the DTT frequencies received with a sufficient level (corresponding to the planned reception mode: fixed, portable outdoor or portable indoor) in the pixel are stored.

For each frequency, the following data is stored:

- the median wanted field strength received in the pixel;

- the location percentage in the pixel, taking into account noise and interference from the DTT network;

- the total nuisance field allowed in the pixel on the basis of an acceptable degradation in the location percentage of that pixel (see section below and Annex 3 for explanation on the calculation of the total nuisance field).

Table X Data stored for each frequency

| DTT Pixels |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pixel | DTT frequencies | Wanted FS level | Loc % | Total Nuis. allowed |
| 1 | F1 | Ewm1 | LP1 | NT1 |
|  | F2 | Ewm2 | LP2 | NT2 |
|  | F3 | Ewm3 | LP3 | NT3 |
|  | … | … | … | … |
|  | Fi | Ewmi | LPi | NTi |
| 2 | F1 | Ewm1 | LP1 | NT1 |
|  | F2 | Ewm2 | LP2 | NT2 |
|  | F3 | Ewm3 | LP3 | NT3 |
|  | … | … | … | … |
|  | Fi | Ewmi | LPi | NTi |
| J | F1 | Ewm1 | LP1 | NT1 |
|  | F2 | Ewm2 | LP2 | NT2 |
|  | F3 | Ewm3 | LP3 | NT3 |
|  | … | … | … | … |
|  | Fi | Ewmi | LPi | NTi |

#### Data for WSD characteristics

The Database (DB) should store the characteristics of the WSD which have been authorized. These WSDs must confirm the frequency and the eirp that they are using. This feedback from the WSD to the DB should be made mandatory in order to construct the DB as described here.

The Category and the Tx-Height of the WSD are used to select the reference geometry for the local interference (related to the same pixel and in the surrounding 8 pixels, see explanation below).

The DB stores the following eirp levels:

- Eirp-cat-max: the maximum eirp of the WSD category;

- Eirp-auth-max: the authorized maximum eirp;

- Eirp-eff: the eirp effectively used by the WSD and confirmed to the DB.

The information on the antenna transmit pattern and the polarisation are used in the calculation of the nuisance filed strength of the WSD, when relevant.

For each WSD the DB contains also the protection ratios that should be used for the calculation of the nuisance fields, for all the offsets corresponding to the relevant frequencies (i.e from co-channel to N±9)

The following Table and Figure gives a schematic representation of the WSD characteristics that the DB will need to perform particular calculations.

(EBU to check the consistency between this table, the content of section 3 and the figure below this table)

|  |  | WSD1 |  | WSD2 |  | WSD3 |  | ... |  | WSDk |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category |  | Portable _master |  | Fixed_10m _master |  | Fixed_30 _master |  |  |  | Portable _slave |  |
| Frequency or channel |  |  |  |  |  |  |  |  |  |  |  |
| Tx-Height (Device elevation) |  | Clutter height |  | 10 m |  | 30 m |  |  |  | 1.5 m |  |
| Eirp-max (dBm) |  | 0 |  | 20 |  | 30 |  |  |  | -10 |  |
| Eirp-eff (dBm) |  | 0 |  | 18 |  | 25 |  |  |  | -13 |  |
| Protection Ratios (dB) |  |  |  |  |  |  |  |  |  |  |  |
| WSD type |  |  |  |  |  |  |  |  |  |  |  |
| Location and its accuracy |  |  |  |  |  |  |  |  |  |  |  |

Table X

|  | WSD1 | WSD2 | WSD3 | … | WSDk |
| --- | --- | --- | --- | --- | --- |
| Category | Portable_master | Fixed_10m_master | Fixed_30_master |  | Portable_slave |
| Frequency or channel |  |  |  |  |  |
| Tx-Height (Device elevation) | Clutter height | 10 m | 30 m |  | 1.5 m |
| Eirp-max (dBm) |  |  |  |  |  |
| Eirp-eff (dBm) |  |  |  |  |  |
| Protection Ratios (dB) |  |  |  |  |  |
| WSD type |  |  |  |  |  |
| Location and its accuracy |  |  |  |  |  |

…

third upper channel

second upper channel

first upper channel

co-channel

…

first lower channel

second lower channel

third lower channel

…

WSD PR

WSD Types

WSD
Device class

WSD
EIRP max

WSD
Device technol id

WSD
Protection Ratios

WSD
Device model/ID

WSD
Ant Character’s

WSD
Location

WSD
Location accuracy

WSD
Expected Oper area

WSD
Device elevation

WSD
Type

WSD
Information

Figure 6: WSD information stored in DB

#### Storage of authorised WSD

For each WSD registered in the DB (i.e. authorised at a given frequency and with a given eirp), its nuisance field relevant for a given frequency in a given pixel is stored with a link to the concerned frequency and the concerned pixel. The criteria to decide when the nuisance field is relevant is explained in Section bellow - Step 4.

Table X: Authorised WSD

| Nuis. FS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Pixel | WSD1 | WSD2 | WSD3 | ... | WSDk |
| 1 | N1F1_1 | N1F1_2 |  | ... | N1F1_k |
|  | N1F2_1 |  | N1F2_3 | ... | N1F2_k |
|  |  | N1F3_2 | N1F3_3 | ... | N1F3_k |
|  |  |  |  | ... |  |
|  | N1Fi_1 |  |  | ... | N1Fi_k |
| 2 |  | N2F1_2 | N2F1_3 |  |  |
|  |  | N2F2_2 |  |  |  |
|  |  | N2F3_2 |  |  |  |
|  |  |  | N2Fi_3 |  |  |
| J |  | NJF1_2 |  |  | NJF1_k |
|  |  | NJF2_2 |  |  |  |
|  |  | NJF3_2 |  |  | NJF3_k |
|  |  |  |  |  | NJFi_k |

#### Interference Management Methodology [ - Method ‘Explicit’]

The proposed DB methodology is based on a global approach to interference effects. This means that all ‘significant’ WSD interferers are taken into account in each pixel affected. In order to do this, each pixel must be considered individually.

A WSD with a given transmit antenna height, htx is to be introduced within a given pixel (designated as pixel P0) – WSD0.

The DB is to assign the maximum eirp to the WSD0, eirpauth_max, according to the WSD’s ‘category’, which satisfies all DTT protection criteria.

There is a set, S0, of n available frequencies for the pixel P0: S0 = {f1, f2, … , fn}.

The following steps are to be carried out for each frequency, in the set S0, or until a frequency and eirp have been assigned to WSD0.

- The WSD0 is located in the pixel, P0, with the red star.

- The 8 surrounding pixels are shaded grey.

- The 9 shaded pixels constitute the ‘local’ interference zone

- The radius Rmax is determined by the maximum distance within which a ‘relevant’ interference might be caused.

- The pixels within the circle other than the 9 in the ‘local’ zone are in the ‘extended’ interference zone

Rmax

J

Figure 7:???

Step 1: Determination of pixels to be examined

It is assumed that a maximum WSD eirpauth-max and htx_max have been established and known by the DB. A maximum ‘relevant’ interference distance, Rmax, is calculated as follows:

- Using the agreed propagation model the distance, Rmax, is calculated such that the reference field strength, Eref, is reached (here the reference field strength is 56.21 dBµV/m, which provides DTT coverage in the presence of noise, with a 95% location probability (LP),

E(eirpmax + 20 htx_max, Rmax) = 56.21 dBµV/m.

- The value eirpmax + 20 is used as the eirp level in the field strength prediction when determining Rmax in order to ensure that cumulative interference effects (from faraway interferers from which received power is not strong however large numbers can contribute to the cumulative effect – EBU?) can be fully accounted for.

- All pixels within a distance Rmax of the WSD0 to be introduced are to be ‘examined’.

- A pixel grid is shown in Figure 6. The red star represents the WSD0, located in the pixel P0. The 8 surrounding pixels are designated as pixels P1 to P8.

Step 2: Examination of individual pixels

- Each pixel within the red circle (Rmax) is examined. As an example we look at pixel J, also indicated in the Figure 6.

- The WSD0 is assumed to have eirpstart = eirpcat_max, as appropriate.

- According to the DB, pixel J is served by a set of m frequencies SJ = {F1, F2, …, Fm} . Some of these frequencies are either adjacent to, or co-channel to f1 (i.e. Fi = f1, f1  1, f1  2, … , f1  9). Those Fi which satisfy this condition are labeled ‘relevant’ frequencies.

- If there are no ‘relevant’ frequencies, the pixel is not affected, and the process begins again with a new pixel.

- Each ‘relevant’ frequency is considered in turn.

Step 2.1 Suppose F1 is the first ‘relevant’ frequency

- The DB has stored all relevant nuisance fields for F1 in pixel J: NJF1_1, NJF1_2, NJF1_3, …, NJF1_k.

- It has also stored the maximum allowed cumulative nuisance field, NTF1, for F1 in pixel J (see Table X);

- NTF1 is the nuisance field which must not be exceeded by the cumulative effects of all of the interfering WSDs (see Annex 3 which describes the concept of total nuisance field).

- The new nuisance field, NF1, produced by WSD0 in pixel J is calculated for ‘relevant’ frequency F1.

The power sum of the new nuisance field NF1 and the existing nuisance fields, NJF1_1, NJF1_2, NJF1_3, … NJF1_k, is carried out ,

, and a comparison with NTF1 is made.

- If NTOT ≤ NTF1, eirpWSD0 can be maintained equal to eirpstart. Otherwise, eirpWSD0t will have to be reduced, and a new value of NF1 and NTOT calculated until NTOT ≤ NTF1.

- The result of this step is eirpWSD0 which may be smaller than to begin with (i.e. eirpWSD0 ≤ eirpstart)

Step 2.2 Step 2.1 is repeated for each ‘relevant’ frequency using the WSD0 eirp resulting at the end of Step 2.1; the consideration of each new ‘relevant’ frequency may reduce the allowed maximum WSD0 eirp.

Step 3 Priority of WSDs

The steps above describe the procedure for the first-come first-served approach.

That is an existing DTT and WSD ‘structure’ exists and a newcomer wishes entry. A search is made to find available channels and suitable/adequate eirp values. The newcomer must be fitted into the existing situation – if this is not possible, because of the DTT protection criteria, the new comer must wait until some better time.

A different approach might use some type of ‘equitable’ treatment to fit the newcomer in by reducing the eirps of the existing WSDs, e.g. so that:

- The relevant eirps are reduced by the same amount, or

- the total allowed nuisance field is shared equally amongst the newcomer and the existing WSDs which contribute to the high nuisance level.

These possibilities are described in the following steps.

Step 3.1 Equal reduction of eirps

- Once again each available frequency is treated in its own turn.

- The above Step 2.1 is carried out for every pixel within the red circle, but the eirp is not reduced pixel by pixel. Instead, for each interfered-with frequency, the pixels are ‘rated’ from ‘worst’ to ‘best’. The ‘worst’ pixel, Pworst, is the one where we have the largest diff given by:

diff = NTOT – NTFi >  0

- The eirp of each of the WSDs contributing interference to the relevant frequency could be reduced by the same amount, diff, which would reduce NTOT to the required limit (NTFi).

- Once the ‘worst’ pixel is ‘fixed’, the nuissance field contribution to the other pixels must be taken into account.

- Each eirp which has been reduced means a reduction to the nuisance fields in many other pixels. These reductions must be taken into account in reevaluating the NTOT for each pixel.

- This continues until there are no more pixels with diff = NTOT – NTFi >  0, for any ‘relevant’ pixel frequency.

- The same procedure can be carried out for each available frequency for WSD0. Some selection process can be selected to determine which of the available frequencies will be used by WSD0 and at which maximum eirp level.

Step 3.2 Equalization of the nuisance fields

- As in Step 3.1, we must order the pixels from ‘worst’ to ‘best’. And we treat the ‘worst’ first.

- For a given WSD0 frequency, and a specific interfered-with frequency in the specific pixel, suppose there are N contributing WSD interference sources. If the relevant maximum cumulative nuisance field is NTFi and

diff = NTOT – NTFi >  0

- Then NTFi is to be partitioned ‘equally’ in the following sense. The limiting ‘allowed equitable’ nuisance field for each WSD interferer in the specific pixel would be Nlim =NTFi – 10 log N.

- All of the nuisance fields contributing to the interference which exceed Nlim would be reduced to Nlim (with the corresponding reduction in the relevant WSD eirp). Those WSDs whose nuisance field does not exceed the Nlim value would not have their eirp values touched.

- Once the ‘worst’ pixel is ‘fixed’, the ‘perturbations’ to the other pixels must be taken into account. Each eirp which has been reduced means a reduction to the nuisance fields in many other pixels. These reductions must be taken into account in reevaluating the NTOT for each pixel. Some of the NTOT values will be reduced, perhaps some not. In any case a reevaluation of the ‘worst’ pixel can be made, and the same procedure as just described carried out.

- This continues until there are no more pixels with diff = NTOT – NTFi >  0, for any ‘relevant’ pixel frequency.

- The same procedure can be carried out for each available frequency for WSD0. Some selection process can be selected to determine which of the available frequencies will be used by WSD0 and at which maximum eirp level.

Step 4 Update of the DB

Once a frequency, Fx,  and an eirpX have been determined by the process above, and the WSD0 is to be implemented, the DB must be updated accordingly.

This means that the nuisance field produced by WSD0 in each pixel within the red circle must be calculated, for every relevant frequency (i.e. for every co- and adjacent channel to Fx). The level of the nuisance field is stored if it exceeds a threshold calculated using the Total nuisance field of the affected frequency in the affected pixel. For example for frequency F1 in pixel J, the threshold is calculated as follows:

Threshold = NTF1 – X dB , X is to be determined, values of 20 or 30 dB for X could be used.

In particular, the information for the pixel P0 must be updated. The WSD0 is situated in P0 so there is a new nuisance field for Fx and all the adjacent channels for P0 itself.

If one of the more complex procedures has been followed, there will also be readjustments of the eirps of some (or perhaps all) of the existing WSDs which had to be ‘adjusted’ to fit the newcomer in; then the DB will have to be updated with respect to each of those changes (eirp reduction).

In some sense, this is not a ‘huge’ undertaking’. X dB reduction in eirp will be reflected in an X dB reduction in the relevant nuisance fields listed in the DB.

#### 5.1.5.5.1	Calculation of the interfering fields

In order to implement the methodology described above it is necessary to calculate the interfering fields generated by the WSDs. This interference is calculated either

- within the pixel, P0, in which WSD0 is situated and also within its immediate 8 neighboring pixels (i.e. within the ‘local’ interference zone), or

- between the WSD0 and the pixels in the ‘extended’ zone which are more distant.

The designations are made clear in the Figure 6.

The following 2 subsections describe how these calculations are carried out.

‘Local’ interference zone

The WSD0 is located in pixel P0. The relevant nuisance field fo P0 is calculated according to the corresponding reference scenario with the associated parameters, e.g. fixed WSD at 10 m height interfering with fixed DTT reception at 10 m, etc.

The free space propagation model is used over these short distances.

However, to reflect the possibility that the wanted field, Ew_med, might be changing over the extent of the pixel and its direct neighbors, the relevant nuisance fields for the surrounding contiguous pixels, P1 to P8, are calculated as if the WSD0 were located in those pixels.

Note that the median wanted field, Ew_med, may change from pixel to pixel, amongst the pixels P0 to P8. It might occur that fixed reception is to be protected in one pixel (because of the value of Ew_med) and portable outdoor reception in its immediate neighbor (because of a sufficiently higher value of Ew_med).

‘Extended’ interference zone

For the pixels in the ‘extended’ zone, the procedure to calculate the interfering field is as follows.

The propagation path of the interfering signal is calculated between a specific calculation point in the pixel P0 and a specific calculation test point in each pixel in the ‘extended’ zone.

The calculation location of WSD0 within P0 is taken to be its actual physical site, if is a fixed installation. Otherwise, for a UE WSD the calculation location of WSD0 is taken to be the center of pixel P0.

The calculation test point of the ‘extended’ pixel is taken to be the center of that ‘extended’ pixel.

The height of the WSD0 is that corresponding to its use (e.g. 10 m, 30 m etc for fixed, 1.5 m for UE).

The height of the test point of the ‘extended’ pixel is that corresponding to the relevant DTT reception mode (e.g. 10 m for fixed, 1.5 m for PO, etc).

The agreed propagation is then used to calculate the resulting field over the calculated path.

#### Interference Management Methodology [: Method ‘Simplistic’]

Place holder for OFCOM contributions on the DB translation methodology

## Protection of PMSE in the band 470-790 MHz

PMSE (Programme Making and Special Events) is a term covering many different wireless production systems operating in a number of frequency bands. For this report we focus on wireless microphone systems using the band 470-862 MHz, also referred to as professional wireless microphone systems (PWMS).

PWMS includes wireless microphones (typically hand-held or body-worn devices), In-Ear Monitoring (IEMs) and other audio systems including fixed point to point links for programme contribution feeds.

There is no single scenario which describes the diverse usage of PMSE. Compatibility and sharing studies must consider various possible scenarios. The parameters required for a geo-location approach will also be dependent upon the PMSE application and usage scenario. For example, an outdoor event, using receive antennas at elevated height, will require a larger exclusion zone than an indoor application.

Typical use cases for PMSE include indoor and outdoor applications at antenna heights ranging from 1.5m to 10, or even 30m in some cases. Receive and transmit antennas may be fixed or mobile

From the geo-location point of view the two main scenarios for PMSE are

Stationary site, such as a theatre, studio or a concert hall/stadium: in this category PMSE could be used either outdoors or indoors, but typically the locations would cover a building or a few buildings or a limited area. Typically these sites stay in the same location and PMSE is used daily or frequently.

Temporary sites such as an exhibition, sports event, interview at a location related to TV Programme making, etc. In this case the PMSE could also be used either indoors or outdoors, in fixed or mobile manner. The nature of this use is temporary.

The stationary sites could relatively easily be registered in a database, but there may be changes in the use of microphones within the site that may need to be registered. A simple approach to protect the site would be to define a protection zone around the sites. The protection zone could cover all use of WSDs (use of all channels be forbidden in the protection zone) or be defined only for the channels that are actually in use at the stationary site. The exact location of the stationary sites can also be defined relatively easily, coordinates could be used, or an address.

Registration of the temporary sites would be a frequent task, thus an easy registration procedure should be employed. As part of the registration process, the microphones may get instructions on what channels to use in their location.

A range of PMSE reference geometries are provided in Annex 6 showing illustrative coupling losses between a WSD and a PMSE receiver. Using these scenarios and the receiver protection ratios (given in ECC Report X1), the WSD e.i.r.p. limits can be calculated.

### Methodology for protection of PMSE

In order to protect PMSE, appropriate e.i.r.p. restrictions must be placed on WSDs operating in the vicinity of the event. The suggested approach is to limit the interference at the PMSE receiver such that the sensitivity of the equipment is degraded by no more than an acceptable amount.

To achieve this, the interference from the WSD, weighted by the receiver ACS value should be positioned below the noise floor of the receiver to minimise loss of sensitivity. The precise value of I/N, ,will determine the loss in sensitivity,, and is given by the following equation:

The behaviour of this function is shown in Figure 8:

Figure 8: Degradation in receiver sensitivity as function of I/N

Protection against co-channel and adjacent channel interference require slightly different strategies.

#### Prevention of co-channel interference

To prevent co-channel interference, an exclusion zone will be necessary to protect a PMSE assignment. This will define the distance required before the frequency assigned to PMSE can be released for reuse by another white space application, either WSD or another PMSE application.

PMSE
 Rx

PTX,

PMSE Operation Zone

WSD Exclusion Zone

DX

PWSD

Figure 9: Definition of a WSD exclusion zone to protect a PMSE event

The radius of the exclusion zone should be chosen such that the interference power density from the WSD will fall below the noise floor of the PMSE receiver.

where:

is the PMSE receiver noise figure

is the propagation loss between the unwanted WSD at distance

is the e.i.r.p.  of the WSD

is the bandwidth occupied by the WSD signal

is the noise temperature of the UHF channel, taken to be 290K
 is Boltzmann’s constant

is the I/N margin, defining the increase in noise level at the PMSE receiver

#### Prevention of Adjacent channel interference

To prevent adjacent channel interference, the selectivity characteristic of the PMSE receiver together with a reference geometry for the class of PMSE event to be protected must be considered.

The selectivity characteristics of the receiver will be expressed in terms of protection ratios that define the minimum ratio of the wanted PMSE signal to unwanted WSD interference for an acceptable impairment in performance. The protection ratios depend upon the frequency offset between the PMSE and WSD signals and also the saturation characteristics of the PMSE receiver. The saturation characteristics can be modelled in detail or simplified by considering an overload threshold.

The reference geometry defines a worst case separation between the WSD and the PMSE receiver for a given class of PMSE application. This geometry together with an appropriate propagation model can be used to estimate the coupling loss, and hence the maximum WSD interference level expected at the PMSE receiver. This data, together with the receiver protection performance can be used to set appropriate limits on the WSD e.i.r.p. to ensure impairment free operation of the PMSE equipment.

This approach can be explored further considering a typical scenario as shown in Figure 10.

hWSD

DWSD

PMSE
 Rx

PWSD

hRX

PTX,

hTX

DPMSE

Figure 10: ???

Consider a PMSE transmitter radiating a signal e.i.r.p. PTX at height hTX, operating in the vicinity of a WSD radiating an e.i.r.p. PWSD at height hWSD.

The path loss between the PMSE transmitter and its associated receiver will be given by:

where  is a propagation model taking account of the height of the frequency of operation, the height of the receiver, the separation and the receiver from the wanted transmitter.

Similarly, the path loss between the interfering WSD and the victim PMSE receiver antenna will be given by:

To prevent adjacent channel interference the power of the interfering WSD signals must be constrained such that

where:

is the coupling gain between the th WSD and the victim DTT receiver.

is the e.i.r.p. from the th WSD at frequency

is the frequency of operation of the PMSE receiver

is the noise figure of the PMSE receiver

is the PMSE receiver bandwidth

is the Boltzmann constant

the noise temperature of the receiver, taken as 290K at UHF

is the chosen value of I/N expressed in linear units to protect the PMSE receiver

## Protection of RAS in the band 608-614 MHz

In the frequency range 608-614 MHz (TV Channel 38) there is a secondary allocation to the radio astronomy service (RAS) used for observations in a number of European countries. The use of this band for RAS is also addressed in footnote 5.149 of the ITU RR.

The sharing studies conducted in ECC Report 159 for both the co-channel (WSD in the 608-614 MHz band) and adjacent channel cases (WSD in the TV channels 37 and 39) have shown that the separation distances between WSDs and RAS depend largely on the type of RAS observations and on the radiated power of the WSDs. For WSDs which have access to a geo-location database, exclusion zones around RAS sites should be defined in the database for TV channels 37, 38 and 39. The size of the exclusion zones can be defined on the basis of the results of sharing studies given in ECC Report 159[2].

Regarding the usage of TV Channel 38 it is noted that the separation distances can go beyond the national borders of some European countries and thus cannot be regulated by an administration concerned. Therefore, the use of TV Channel 38 by WSDs controlled by a geo-location database will be subject to one of the two following options:

- Should the database management be performed centralized on a European level, the exclusion zones can be defined for each RAS site on the basis of the results of sharing studies given in ECC Report 159[2];

- Should the database management be performed on a national level, the use of TV channel 38 is subject to bilateral/multilateral agreements.

## Protection of ARNS in the band 645-790 mhz

(relevant parts from Issue E1, no inputs so far)

## Protection of services in the bands adjacent to 470-790 MHz

### General

From the analysis on permissible WSD EIRP limits for protection of LTE in the adjacent bands to 470-790 MHz (provided in ECC Report [A1]) it is clear that some limitations towards WSDs are necessary in order to limit the interference towards LTE to acceptable levels.

In principle, such limits could be solely calculated on a case-by-case basis by the geo-location database, e.g. by using the calculation method and scenario geometries provided in ECC Report [A1] and by using context-specific information eventually provided by the WSD (e.g. antenna height). This would have the advantage of maximum flexibility for administrations to decide about the details of how this calculation should be done and which parameters and scenarios the calculation should be based on. Note that this approach would lead to only device-specific emission limits (as opposed to the location and device dependent limits for protection of TV broadcasting and PMSE) for protection of services in adjacent bands, which would have to be reconciled with the limits for protection of the incumbent services within the TV band in order to identify the most limiting interference scenario.

This approach would, for example, be appropriate for those cases where “intermediate” EIRP limits can be allowed, which means that the results are inconclusive as to whether a reasonable WSD operation under those limits is possible or not.

Nevertheless the analysis has also shown that some cornerstones seem to be rather independent from scenario and selected parameters (e.g. see § 6.3.5 of ECC Report [A1]). It might thus be beneficial to regulate those elements in a generic way in order to provide certainty and optimization potential to the WSD industry and reduce the computational complexity of a geo-location database.

### Protection of mobile service in the band 790-862 MHz

Calculations of WSD power limits for protection of the mobile services in the band 790-862 MHz need to be conducted as part of the translation process within the geo-location database. Though appropriate calculations methods and scenarios may still need to be developed, those elaborated in § 6.3 of ECC Report [A1] could be used as a starting point.

For the I/N threshold and baseline level assumed in the assessment of WSD interference into the LTE service in the band 790-862 MHz, it is concluded that:

- WSD EIRP limits required to keep interference to LTE UEs below the considered I/N thresholds seem to indicate that WSD operation on channel 60 is in general not advisable for all scenarios. For portable devices the required EIRP limits on channel 59 seem to be too low to allow practical operation as well.

- For all scenarios operation on channels 59-57 (except channel 59 for portable devices) could be possible within certain limits that may or may not allow practical usage depends on the requirements of the WSD user.

- For channel 56 and below the required EIRP limits do no longer depend on frequency offset since it is assumed that the LTE UE selectivity does not increase further beyond the corresponding frequency offsets. The allowable WSD EIRP levels on these channels seem not to be constraining the respective use case scenarios significantly

However, there can be other WSD deployment scenarios as well as other national protection requirements with respect to the mobile service that may change the conclusions listed in the three bullet points above.

It is also recommended for the geo-location database to assume conservative parameters for a WSD to derive its power limits, when the database does not receive the information from the WSD necessary to categorize it for the purpose of translation calculations.

# master/slave concept

## Definitions

## General principles of master/slave concept

## Requirements for master WSDs

### Technical information to be communicated from a master WSD to the geo-location database

### Technical information to be received by a master WSD from the geo-location database

### Operational and security requirements

## Requirements for slave WSDs

### Technical information to be communicated from a slave WSD to a master WSD

The following information shall be communicated by the slave WSD to its serving master WSD:

[Ed. note: For some power classes WSDs might not be subject to submission of all the parameters related to the location. This needs to be further developed. The concept of non-geo-located slave is to be considered.]

- slave WSD antenna location (only for geo-located slaves),

- slave WSD antenna location accuracy (only for geo-located slaves),

- slave WSD class,

- slave WSD technology identifier,

- slave WSD model,

- antenna locations and antenna characteristics of the associated slave WSDs (provided such information is available).

The following information may be optionally communicated by a slave WSD to its serving master WSD:

- slave WSD antenna height,

- slave WSD antenna angular discrimination,

- slave WSD antenna polarisation,

- slave WSD ID.

### Technical information to be received by a slave WSD from a master WSD

The following information shall be received by the slave WSD from its serving master WSD:

- An indication of the lower and upper frequency boundaries within which the slave WSDis authorized to operate,

- a maximum permitted slave WSD EIRP spectral density between each lower frequency boundary and its corresponding upper frequency boundary,

- a validity time for the parameters communicated by the serving master WSD,

- a geographical area of validity for the parameters communicated by the serving master WSD,

- instructions for the slave WSD to cease transmissions immediately when requested by the master WSD (so called “kill switch”).

The following information may be optionally received by a slave WSD from its serving master WSD:

- a sensing level for the detection of DTT use of spectrum,

- a sensing level for the detection of PMSE use of spectrum,

### Operational requirements

A slave WSD may only transmit within the band 470-790 MHz in accordance with the relevant instructions and parameters provided by its serving master WSD as listed in § 6.4.2, and for a time period which does not exceed the time validity of those instructions and parameters.

A slave WSD shall cease transmission immediately when

- instructed so by its serving master WSD, or

- when no communication is established with the master WSD after the validity time for the parameters received previously, or

- when moving outside the area, for which the parameters communicated by the master WSD are valid.

- A slave WSD may communicate with another slave WSD provided that each is controlled via communication over the band 470-790 MHz by its serving master WSD.

# Combined sensing and geo-location

## Methodology

Table 1: Channel occupancy based on spectrum sensing only

Table 2: Channel occupancy based on geo-location only

Table 3: Channel occupancy based on combined detection

Figure 11: Decision chart for the combined sensing and geo-location approach

## Algorithm

Figure 12: Operational algorithm for the combined detection approach in a single WSD

Figure 13: Operational algorithm for the combined detection approach in Master/Slave WSDs

# Conclusions

Within this complementary report to ECC Report 159, the CEPT studied the principles and set up the requirements for the operation of WSDs under the geo-location approach. In particular, the report:

- lists the mandatory and optional information that needs to be exchanged between the WSD and the geo-location database;

- considers the technical aspects of the geo-location database management;

- provides translation algorithm examples for the protection of different incumbent radio services in the band 470-790 MHz and adjacent to it;

- discusses the master/slave operation on WSDs;

- develops the concept of the combines sensing and geo-location approach.

MULTIPLE-INTERFERENCE

Introduction

The information in this Annex is relevant for calculating maximum eirp levels for networks of WSDs lying outside of any given co-channel or adjacent channel DTT coverage area, taking into account the cumulative effects of multiple sources of interference. Calculation methods for treating this situation are presented in section 5.1.2.4.

Methods for dealing with maximum WSD eirp levels for WSDs lying inside of an adjacent channel DTT coverage area are presented in Section 5 and in Annex 5, using the parameters of the various reference interference scenarios of Annex 2.

Simulation methodology and its results

Figure 14 shows the simulation methodology. The simulation parameters based on ECC Report 159 are shown in Table 4.  The simulation procedure is as follows:

STEP 1. Set the incumbent service operation parameters

The parameters in the Table 4 of Section 4.1 of ECC Report 159 are adopted.

STEP 2. Calculate the protection area of the incumbent service operation and the protection contour

The protection contour is calculated based on the method in Section 4.1 of ECC Report 159.

STEP 3. Set the number of distributed master WSDs and its geolocation related parameters of the master WSDs.

The following parameters related to the geolocation information of the master WSD are considered in this simulation:

- Protection distance (D1 [km]).

This parameter is required in considering the interference effects from the slave WSD in each WSD network managed by each master WSD.

- Separation distance (D2 [km]) of each master WSD.

Each master WSD is located to a distributed point where is separation distance (D2 [km]) away from each other as shown in Figure 14.

STEP 4. Set the WSD network operation parameters managed by each master WSD

TRX configuration parameters such as antenna height and antenna gain are set to be here. The detail simulation parameters are shown in Table 4.

STEP 5. Calculate the location specific output power level of each master WSD

The detail simulation parameters are shown in Table 4.

Figure 14: Simulation methodology

Figure 15: Geolocation information related parameters

Table 4: Simulation parameters

| Parameter | Value |
| --- | --- |
| Frequency-related parameters |  |
| Frequency | 600 MHz |
| Number of channels | 1, 5 or 10 |
| Channel bandwidth | 7.6 MHz |
| Channel separation | 8 MHz |
| ACLR (=) v | Scenario #1 ACLR: 33 dB for the first adjacent channel, 36 dB for the second and later adjacent channels(from 3GPP TS36.101) ACS: 33 dB (from 3GPP TS36.101) Scenario #2 ACLR: 36 dB for all adjacent channels ACS: 61 dB |
| # ACS (=) |  |
| Geolocation-related parameters |  |
| Protection contour from incumbent transmitter | 59.2 km |
| Protection distance (=D1) | 20, 40 km |
| Separation distance (=D2) | 2 km |
| Reference point selection criteria for incumbent service protection | Regarding the calculation step in fixed/flexible method, the closest point in the protection contour of the incumbent service would be chosen as the reference point of each target master WSD. Regarding the calculation step in maximized method, the closest point in the protection contour of the incumbent service should be chosen for the reference point of each target master WSD in the first calculation step. After that, the calculation engine will try to find the most severe interfere-victim reference point of each WSD to adjust the output power of WSDs while considering in-block/out-block interference effects from multiple WSDs. |
| Number of active master WSDs | 300 |
| Range of angle where master WSD are distributed | 0 – 180 degrees |
| Propagation-related parameters |  |
| BS broadcaster antenna height | 200 m |
| BS receiver antenna height | 10 m |
| Master WSD antenna height | 20 m |
| Slave WSD antenna height | 10 m |
| Propagation model | Recommendation ITU-R P.1546[9] (Rural, Time percentage = 1%) |
| BS receiver antenna directivity discrimination | 0 Db (with respect to DTT transmitter), -16 Db (with respect to WSD Ddir) |
| TV receiver antenna gain (=) | 12 Db |
| TV receiver feeder loss (=) | 3 dB |
| BS receiver polarization discrimination with respect to the WSD signal () | 0 dB |
| Output power-related parameters |  |
| WSD location specific output power calculation methods | Fixed method (based on Eq.(2)), Flexible method (based on Eq.(3)), and Maximized method (based on Eq.(11)) |
| Limit of WSD output power | 36 dBm |
| TV broadcaster transmission power | 79.15 dBm/channel |
| Noise-related parameters |  |
| Noise density | -174 dBm/Hz |
| Noise figure | 7 dB for all nodes |
| Aggregated interference power level related parameters |  |
| Multiple interference margin: IMdB(Fixed method) | 10*log10 (Potential maximum number of active master WSDs in each available channel which depends on the simulation parameter setting in allocating active WSDs in available channels) |
| Multiple interference margin:IMdB(Flexible method) | 10*log10 (Maximum number of all the number of active master WSDs in each available channel) |
| Incumbent service operation parameters |  |
| Minimum incumbent service (BS) power @ receiver | 77.1 dBm |
| Protection ratio | 23.1 Db |
| Location probability (LP) without interference from WSD | 95 % |
| LP with interference from WSD | 94.9 % |
| Acceptable degradation to LP | 0.1 % |
| Standard deviation of BS power @ receiver | 5.5 dB |
| Standard deviation of coupling gain between BS receiver and WSD transmitter | 3.5 dB |

Comparison of the location specific output power level of WSD

The location specific output power level of WSD is shown here. The results of 5% and 50% CDFs of the location specific WSD e.i.r.p. [dBm] when the number of master WSDs is 300 are highlighted.

Table 5: Comparison of output power level of WSD (Number of TVWS channels = 1)

| Number of master WSDs |  | 300 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number of channels |  | 1 |  |  |  |  |  |
| Protection distance [km] |  | 20 |  |  | 40 |  |  |
| Output power calculation method |  | Fixed | Flexible | Flexible Minimized | Fixed | Flexible | Flexible Minimized |
| Output power level of WSD [dBm] | 5% CDF | 9.1 | 9.1 | 18.0 | 21.8 | 21.8 | 28.8 |
|  | 50% CDF | 10.9 | 10.9 | 19.9 | 22.6 | 22.6 | 29.6 |

Table 6: Comparison of output p8ower level of WSD (Number of TVWS channels = 5)

| Number of master WSDs |  | 300 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number of channels |  | 5 |  |  |  |  |  |
| Protection distance [km] |  | 20 |  |  | 40 |  |  |
| Output power calculation method |  | Fixed | Flexible | Flexible Minimized | Fixed | Flexible | Flexible Minimized |
| Output power level of WSD [dBm] | 5% CDF | 9.1 | 17.0 | 22.7(Scenario#1) 21.1(Scenario#2) | 21.8 | 29.8 | 36.0 |
|  | 50% CDF | 10.9 | 19.2 | 24.8 (Scenario#1) 23.1 (Scenario#2) | 22.6 | 30.6 | 36.0 |

Table 7: Comparison of output power level of WSD (Number of TVWS channels = 10)

| Number of master WSDs |  | 300 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number of channels |  | 10 |  |  |  |  |  |
| Protection distance [km] |  | 20 |  |  | 40 |  |  |
| Output power calculation method |  | Fixed | Flexible | Flexible Minimized | Fixed | Flexible | Flexible Minimized |
| Output power level of WSD [dBm] | 5% CDF | 9.1 | 19.6 | 24.4(Scenario#1) 23.1(Scenario#2) | 21.8 | 36.0 | 36.0 |
|  | 50% CDF | 10.9 | 21.5 | 26.6(Scenario#1) 25.1(Scenario#2) | 22.6 | 36.0 | 36.0 |

comparison among different Im calculation method and its analysis

The results in previous section show that the consideration of the number of active WSDs of each available channel in the calculation engine will bring us the highest communication opportunity of WSDs, according to the performance differences among fixedIMdB, flexible IMdBand flexible minimizedIMdBmethod. In this simulation, the potential maximum number of active WSDs in each available channel is fixed, so we can just see no performance difference between the fixed IMdBmethod and flexible IMdBmethod in a case where the number of available channel is in case of single available channel. However, in actual operation, the geolocation database does not know the potential maximum number of active WSDs in each available channel, so an excessive value should be adopted. Hence, we can see more performance differences between fixed method and flexibleIMdBmethod or flexible minimizedIMdBmethod in actual TVWS operation.

The flexible minimized IMdB method can show the highest performance in three methods, because there will be some redundancy in calculating output power level of WSDs in cases where the fixed and flexible margin based calculation methods are adopted. This may be due to the fact that the fixed and flexible margin based calculation methods cannot differentiate between the path loss conditions of a target WSD from one of the other potential interferers in calculating output power level of a target WSD.

Table 8 shows the comparison of different calculation methods from viewpoints of the calculation overhead, the system overhead and the WSD network capacity.One can see that the calculation overhead for the calculation engine and its system overhead will increase due to the consideration of the number of active master WSDs in each available channel, so some additional study on this issue will be necessary.

Table 8: Comparison of different calculation methods

|  | Calculation overhead | System overhead | WSD network capacity |
| --- | --- | --- | --- |
| Fixed margin based method | Low | Small | Low |
| Flexible margin based method | Moderate | Moderate | Moderate |
| Flexible minimized method | High | Large | High |

There will be several possible deployment scenarios for the calculation engine of location specific output power level. For example, the calculation engine may be a part of geolocation database controlled by a regulatory body, or a separate engine (namely advanced geolocation engine) from the geolocation database managed by regulatory body as shown in Figure 11. In a case where it is a separate engine, a third party should take a responsibility to protect the incumbent service receivers from an aggregated interference problems, and the operation should be kept under surveillance by regulatory body. The merit will be to enable the processing load of the geolocation database managed by regulatory body to offload to the third party engine. Such third party engines may be also provide other services, such as coexistence services (which will be standardized in specifications such as  ETSI TS102 908 Cognitive radio network coexistence for white space or IEEE P802.19.1 (TVWS network coexistence system) to the WSDs operating in the same area.

Figure 16: Network deployment scenario

Conclusions

This Annex introduces the WSD network capacity comparison and analysis between the location specific WSD output power level calculation method for database approach in ECC REPORT159 and its extension methods which consider the number of active master WSDs of each available channel in WSD master-slave operation. This result shows that the consideration of the number of active master WSDs of each available channel will have large impact for location specific output power level of WSDs while satisfying the incumbent service protection.

Further study will be necessary in SE43 and/or ETSI, specifically for the feasibility of the interface between the geolocation database and WSDs allowing for the number of active master WSDs to calculate the location specific WSD output power level in a geolocation database.

Interference scenarios

When extending the basic methodology of ECC Report 159 to calculate the WSD maximum power levels the following scenarios could be considered:

Portable outdoor WSD transmission

Mobile WSD transmission and fixed roof-top DTT reception (Scenario 1)

The model we use is shown below. A mobile WSD at 1.5 m above ground level (agl) radiates into a rooftop DTT antenna. Taking account of the vertical DTT receive antenna pattern, the maximum loss and receive antenna discrimination from the WSD to the DTT antenna is 56.15 + 0.45 dB - 9.15 = 47.45 dB.

Figure 17: Scenario 1: Reference geometry for mobile WSD at 1.5 m agl.

The path loss shown is valid for a carrier at 650 MHz.

Portable WSD transmission at 10 m agl and fixed roof-top DTT reception (Scenario 2)

The model we use is shown in below. A mobile WSD at 10 m agl (e.g. from a window in a multi-story house) radiates directly into a rooftop DTT antenna at 10 m agl. There is no receiving antenna or polarisation discrimination. The maximum loss from the WSD to the DTT antenna is 54.72 dB.

Figure 18: Scenario 2: Reference geometry for mobile WSD at 10 m agl.

The path loss shown is valid for a carrier at 650 MHz and is based on a distance (d) ranging from 6 m to 20 m. In practice this separation will vary across Europe, and a different value may be appropriate where the housing density is lower. The justification for the 6 m can be found at APPENDIX 1, which presents details of UK household separation analysis.

For the general case, the coupling loss G, between WSD and Fixed DTT, will be given by:

In order to take into account the varying distance across Europe, the distance D may be classified as a variable. Three selectable distance separations which determine the coupling loss, G, are included below in Table A2.1:

| Distance (m) | FSL @ 650MHz | DTT Antenna Gain () | Coupling Gain () |
| --- | --- | --- | --- |
| 6 | 44.32 | 9.15 | 35.17 |
| 10 | 48.76 | 9.15 | 39.61 |
| 20 | 54.79 | 9.15 | 45.63 |

Table A2.1: Variable distance (d) and resulting Coupling Loss (G)

Mobile WSD transmission at 1.5 m agl and mobile DTT reception at 1.5 m agl (Scenario 3a)

The model we use is shown below. A mobile WSD at 1.5 m agl radiates directly into a mobile DTT antenna at 1.5 m agl. There is no receiving antenna or polarisation discrimination. The maximum loss from the WSD to the DTT antenna is 34.7 dB.

Figure 19: Scenario 3a: Reference geometry for mobile WSD and mobile DTT at 1.5 m agl

The path loss shown is valid for a carrier at 650 MHz.

Mobile WSD transmission at 1.5 m agl and mobile DTT reception at 1.5 m agl (Scenario 3b)

The reference geometry for the protection of outdoor portable reception is shown in next Figure. The choice of separation between the WSD and the DTT antenna is a compromise. Short separations will imply the greatest restrictions on the WSD.

Figure Scenario 3b: Reference geometry for mobile WSD and portable outdoor DTT at 1.5 m agl

The coupling loss G between the WSD and the victim portable receiver at short range can be modelled as free space loss. Assuming a portable DTT antenna gain of 2.15 dBi, and no polarization discrimination and a separation of 4 m, the coupling loss G at 650MHz will be G = 40.8 -  2.15 = 38.65 dB. This path loss figure is valid for a WSD operating at 650 MHz.

Geometry for Protection of Indoor Portable DTT from mobile WSD UE (Scenario 3c)

The ECC 159 geometry for protection of portable is shown in Figure below.

Scenario 3c: Reference geometry for portable indoor reception

The coupling loss G between the WSD and the victim portable receiver at short range is modeled as free space loss. Assuming 8 dB wall loss, 40.8dB free space loss at 4m, 0dB polarization discrimination and 2.15dBi DTT antenna gain, the typical coupling loss G = 40.8 + 8 - 2.15 = 46.6dB at 650 MHz.

Fixed WSD transmission

Fixed WSD transmission and fixed DTT reception at 10 m agl (Scenario 4a)

The model we use is shown below. A fixed WSD at 10 m agl radiates directly into a fixed DTT antenna at 10 m agl. The distance between the antennas ranges from 6 m (densely inhabited area) to 20 m (less densely inhabited area). There is no receiving antenna discrimination. There would be 3 dB polarisation discrimination. The maximum loss from the WSD to the DTT antenna, including polarisation discrimination, ranges from  44.32 + 3 = 47.32 dB to 54.72 + 3 = 57.72dB.

PIB

Fixed rooftop DTT

reception

TV

FSL

Fixed WSD

10 m

-3 dB

+9.15 dBi

H-polar

Slant-polar

d

Figure 20. Scenario 4a: Reference geometry for fixed WSD and fixed DTT reception at 10 m agl

The path loss shown is valid for a carrier at 650 MHz.

Fixed WSD transmission and fixed DTT reception at 10 m agl  (Scenario 4b)

The model we use is shown above. A fixed WSD at 10 m agl radiates directly into a fixed DTT antenna at 10 m agl. The separation D, used for coupling loss calculations will depend upon the density of housing in a particular national administration. For the UK, studies of typical housing separations (see APPENDIX 1) suggest a distance of 6m should be used for the free space loss calculation.

Assuming a fixed DTT antenna gain of 9.15dBi, 6m separation with a free space loss of 44.3dB at 650MHz, and 16dB cross polar discrimination, the coupling gain G will be 51.2dB. For CPE installations where the CPE is operating on the same polarization as the fixed DTT receiver, the coupling G reduces to 35.2dB.

Figure 21: Reference geometry for fixed outdoor reception and fixed WSD CPE

Note that the 6 m separation between WSD antenna and DTT receive antenna may also occur in les densely inhabited areas if the two antennas are situated on the same roof.

For the general case, the coupling loss G will be given by:

For cross-polarisation between the WSD and the DTT antenna, and in order to take into account the varying distance across Europe, the distance D may be classified as a variable. Three selectable distance separations which determine the coupling loss, G, are included below in Table A2.1:

| Distance (m) | FSL @ 650MHz |  | DTT Antenna Gain () | Coupling Gain () |
| --- | --- | --- | --- | --- |
| 6 | 44.32 | 16 | 9.15 | 51.17 |
| 10 | 48.76 | 16 | 9.15 | 55.61 |
| 20 | 54.79 | 16 | 9.15 | 61.62 |

Table A2.2: Variable distance (D) and resulting Coupling Loss (G) for Cross-Polarisation

For non co-polarisation between the WSD and the DTT antenna, and in order to take into account the varying distance across Europe, the distance D may be classified as a variable. Three selectable distance separations which determine the coupling loss, G, are included below in Table A2.3:

| Distance (m) | FSL @ 650MHz |  | DTT Antenna Gain () | Coupling Gain () |
| --- | --- | --- | --- | --- |
| 6 | 44.32 | 0 | 9.15 | 35.17 |
| 10 | 48.76 | 0 | 9.15 | 39.61 |
| 20 | 54.79 | 0 | 9.15 | 45.63 |

Table A2.3: Variable distance (D) and resulting Coupling Loss (G) for Co-Polarisation

Fixed WSD transmission at 10 m agl and mobile DTT reception at 1.5 m agl (Scenario 5)

The model we use is shown below. A fixed WSD at 10 m agl radiates directly into a mobile DTT antenna at 1.5 m agl. There is no receiving antenna or polarisation discrimination. We assume a 0.5 dB transmit antenna attenuation in the downward direction. The maximum loss from the WSD to the DTT antenna, including transmit antenna attenuation, is 56 + 0.5 = 56.5 dB at 650 MHz.

Figure 22: Scenario 5: Reference geometry for fixed WSD at 10 m and mobile DTT at 1.5 m agl.

The path loss shown is valid for a carrier at 650 MHz.

FIXED WSD AT 10 m agl INTERFERING WITH PORTABLE INDOOR DTTB AT 1.5 magl  (Scenario 6)

The geometry used to calculate the interference from a fixed WSD transmit antenna at 10 m to a portable indoor DTT receiver at 1.5 m height is shown below.

The relevant parameters are the following:

Reference frequency: 650 MHz

Horizontal separation between the WSD and the DTTB: 20 m

Path loss: 56 dB

WSD transmit antenna discrimination: 0.5 dB

Polarization discrimination: 0 dB

Wall loss: 8 dB, standard deviation 5.5 dB

We assume a 0.5 dB transmit antenna attenuation in the downward direction. The maximum loss from the WSD to the DTT antenna, including transmit antenna attenuation, is 56 + 0.5 + 8 = 64.8 dB at 650 MHz.

Figure 23: Scenario 6: Reference geometry for fixed WSD at 10 m and mobile indoor DTT at 1.5 m agl.

Fixed WSD transmission at 30 m agl and fixed DTT reception at 10 m agl (Scenario 7)

3.0 dB

13.55 dB

62.87 dB

23.05°

47 m

LOSS = 62.87 + 13.55 + 3.0 = 79.42 dB

Figure 24: Scenario 7: Reference geometry for fixed WSD at 30 m and fixed DTT at 10 m agl.

If 30 m BS transmit antennas are foreseen, this type of usage should be restricted to rural areas. In urban environments, fixed DTTB receive antenna installations might also be foreseen at 30 m. In this case the WSD eirp restrictions would be the same as that calculated for Scenario #4.

Other assumptions for this scenario were proposed, referring to the results presented in APPENDIX 1, where both horizontal and vertical polarization of the WSD antenna is considered.

A fixed WSD at 30m agl, horizontally polarized, radiates into a fixed horizontally polarized DTT antenna at 10 m agl. Taking into account the vertical radiation pattern of the WSD antenna the path loss is 67 + 3.6 - 9.15 = 61.44dB. The vertical radiation pattern of the DTT antenna has been considered but gives no extra discrimination for less than 20 degrees.

A fixed WSD at 30m agl, vertically polarized, radiates into a fixed horizontally polarized DTT antenna at 10 m agl. Taking into account the vertical radiation pattern of the WSD antenna the path loss is (67 + 3.6 - 9.15) + 16 = 77.45dB.

Figure 52. a): Reference Geometry for 30m WSD BS and Fixed DTT receiver

Fixed WSD transmission at 30 m agl and mobile DTT reception at 1,5 m agl (Scenario 8a)

27 m

18.01 dB

60.59 dB

46.6°

LOSS = 60.59 + 18\.1 + .0 = 78.69 dB

Figure 25: Scenario 8 a): Reference geometry for fixed WSD at 30 m and fixed DTT at 10 m agl.

If 30 m BS transmit antennas are foreseen, this type of usage should be restricted to rural areas. In urban environments, when Portable Outdoor DTTB reception is to be protected, because of the portability of the mobile DTTB apparatus, such equipment can also be located at higher than 1.5 m (e.g. at 10 m or 20 m at the window of a high rise).

Fixed WSD transmission at 30 m agl and mobile DTT reception at 1,5 m agl (Scenario 8b)

Other assumptions for this scenario were proposed by one administration, referring to the antenna pattern presented in APPENDIX 2.

A fixed WSD at 30m agl, radiates into a mobile DTT antenna at 1.5 m agl. Assume co-polarisation of the WSD and DTT antennas. Taking into account the vertical radiation pattern of the WSD antenna the path loss is 66.45 + 3.45 = 69.9dB. The vertical radiation pattern of the DTT antenna has been considered but gives no extra discrimination for less than 20 degrees.

Figure 53.a): Reference Geometry for 30m WSD BS and Fixed DTT receiver

If 30 m BS transmit antennas are foreseen, this type of usage should be restricted to rural areas. In urban environments, when Portable Outdoor DTT reception is to be protected, because of the portability of the mobile DTT apparatus, such equipment can also be located at higher than 1.5 m (e.g. at 10 m or 20 m at the window of a high rise).

- APPENDIX 1 OF ANNEX A2

- INTRODUCTION

High quality address location data for the UK and Northern Ireland is available from the Ordnance Survey and Ordnance Survey Northern Ireland as commercially licensed products.  Equivalent data is also available from other sources for the Isle of Man and Channel Islands, but these jurisdictions are excluded from this analysis.

The address data is at a positional resolution of 10cm, in many cases determined by GPS positioning.  The physical point in the address selected for the location is determined by the OS product purchased, and varies from a point somewhere within the dwelling for the basic product through to the precise location of the postal delivery point, usually the letter box, in the premium product.

It is possible to process the positional data to determine the distance between address locations which is a first approximation to the separation between dwellings.  The data available to Digital UK is at the lowest order of accuracy, i.e. the address position represents a point somewhere within the dwelling, but not necessarily the Delivery Point.

The “MapInfo” GIS software package includes a tool to calculate separation distances and this can be used to find the “nearest neighbour” to selected addresses. The tool is computationally intensive and the computational resource available to Digital UK means that analysis must be restricted to relative small geographical areas.

MapInfo was initially used to assess the address separations in Uckfield, a typical small rural town in East Sussex.  The assessment area and the delivery points are shown in Figure 26.

- Assessment area boundary

Figure 26

This area contains 6,730 separation distances of 1metre or more. Separations of less than 1 metre are treated as co-located addresses and therefore have been ignored. Similarly, addresses in multi-occupancy dwellings have been ignored, the building being treated as having a single address, rather than multiple addresses. This will tend to skew the results towards an over-estimate, rather than an under-estimate of the separation distances for the affected premises.

Figure  shows the frequency of occurrence of the distances, to the nearest metre. Note that the curve has a long tail and has been truncated at 50m for clarity. The maximum separation distance in this area is 373m.

A closer view of the Newtown area, which is in the central part of the town to the south of the railway station, is given in Figure . Newtown comprises a mix of housing types and ages, typical of the town.  The map also shows the OS address points, the 100m tiles with address count, and the nearest-neighbour derivation.  The corresponding separation distance histogram is given in Figure .

Figure 54

Figure 55

By way of comaparison, a MapInfo assessment has also been made of a small sample area in Islington, London. The area selected is shown in Figure . As for Newtown, the map also shows the OS address points, the 100m tiles with address count, and the nearest-neighbour derivation. The corresponding separation distance histogram is given in Figure .

Figure 56

  - Conclusion

This limited analysis demonstrates that the typical Ordnance Survey address separation distance in both areas is around 5m.  In many cases this will be an underestimate because the Ordnance Survey position is located somewhere within the dwelling, whereas in many cases the address in the sample areas are terraced or semi-detached which means that parts of the adjacent dwellings are much closer to each other than 5m.

Figure 57: Address locations and nearest neighbours in Newtown, Uckfield

Figure 58: Address locations and nearest neighbours in Islington, N1

- WIDER ANALYSIS

Although the areas for the initial analysis were selected at random, it is not a given that they are representative of the wider UK situation.  While interesting, it is not necessary to identify and map the nearest neighbours to each address; rather, the separation distance alone provides sufficient information to make the necessary assessment.

To permit larger areas to be analysed within the available computing power, a “Nearest Neighbour” routine was developed in MySQL.  While still computationally intensive, this can calculate the separation distances for larger datasets than is possible using MapInfo.  Even so, the realistic upper limit is around 450,000 addresses, which still takes in excess of 12 hours to compute.

The computational limitations make it necessary to sub-sample the 26.1million UK addresses to allow calculation to be possible.  One way would be to select blocks of addresses based on the National Grid, but this would be hit-and-miss as to the number of addresses contained within each block.  An alternative, more convenient, method is to use UK post areas – identified by the initial letter or two letters of the address postcode – because post areas generally contain fewer than 450,000 addresses and generally (but not always) have boundaries in more sparsely populated areas.

It is acknowledged that any analysis area with artificial, rather than natural, boundaries (i.e. where the boundary is not uninhabited water!) will introduce a degree of error because the nearest neighbour to a perimeter address may be outside the boundary selected, but the proportion of the total number of distances so affected will be small, and the error will tend to over-estimate, rather than under-estimate, the minimum separation distance so the overall results will not be skewed to be lower than reality.

The post areas selected for the analysis provide a range of geographies. The areas are shown in Figure , and the details are set out in Table :

Table AP1.1: Post Areas analysed

| Post Area | Description | Addresses | Geography |
| --- | --- | --- | --- |
| BH | Bournemouth | 266,765 | Urban, sub-urban and rural |
| CF | Cardiff | 453,344 | Urban, sub-urban and rural |
| CV | Coventry | 368,537 | Urban, sub-urban and rural |
| DG | Dumfries & Galloway | 77,231 | Small towns and rural |
| HS | Western Isles | 15,475 | Sparsely populated, remote rural |
| IG | Ilford | 127,509 | Urban and sub-urban |
| KT | Kingston-upon-Thames | 230,976 | Urban and sub-urban |
| LS | Leeds | 364,705 | Urban and sub-urban |
| NW | North West London | 212,186 | Urban and sub-urban |
| PO | Portsmouth and Isle of Wight | 383,881 | Urban, sub-urban and rural |
| RH | Redhill | 234,140 | Urban, sub-urban and rural |
| SO | Southampton | 297,604 | Urban, sub-urban and rural |
| SR | Sunderland | 117,584 | Urban and sub-urban |
| SY | Shrewsbury and mid-Wales | 158,566 | Urban, sub-urban, rural and remote |
| TD | Galashiels | 60,127 | Small towns and rural |
| ZE | Shetland | 11,565 | Sparsely populated, remote rural |
|  | Total | 3,380,195 |  |

The normalised frequency of separation distance found in these areas is shown in Figure  and the CDF in Figure .

  - Conclusion

The wider analysis of 3,380,195 address locations demonstrates that, except for the remote rural areas represented by the HS and ZE post areas, the typical Ordnance Survey address separation distance is around 6 m. As for the initial analysis, this will tend be an overestimate because the Ordnance Survey position is located somewhere within the dwelling, whereas in many cases the address in the sample areas are terraced or semi-detached which means that parts of the adjacent dwellings are much closer to each other than 6 m.

The analysis also shows that, except in the sparsely populated remote rural areas, at least 50% of addresses have the nearest neighbour within 10m.

Figure 59 : Post Areas selected for analysis

Figure 60

Figure 61

- APPENDIX 2 OF ANNEX A2: VERTICAL RADIATION PATTERN OF THE WSD ANTENNA FOR FIXED WSD TRANSMISSION AT 30 M

To calculate the coupling loss, G, between the fixed WSD antenna mounted at a height of 30m with 0º beam tilt and a DTT receiver (sections A2.2.4 and A2.2.5), the attached vertical radiation pattern was used.

Figure 62:  Vertical Radiation Pattern of the WSD Antenna for Fixed WSD transmission at 30 m

Number of WSD and Nuisance Power

????

NUISANCE FIELDS AND POWER SUMMATION

The ‘nuisance field’ is a convenient concept when calculating interference effects. ‘Instantaneous’ nuisance fields are used in Monte Carlo simulations, and ‘median’ nuisance fields are used in analytical calculations, involving interference determination.

NOISE ONLY

For acceptable DTT reception at a given point, in the presence of noise only, the wanted DTT field strength Ew must exceed the noise equivalent field strength, N, by a minimum amount, [C/N], the required DTT carrier-to-noise ratio:

Ew ≥ N + [C/N].

The expression, N + [C/N], is called the nuisance field, NUN, for the noise:

NUN = N + [C/N].

Thus, the condition/requirement for acceptable DTT reception in the presence of noise only can be written simply as

Ew ≥ NUN.

ONE INTERFERER

If, at a given point, the wanted DTT field strength is Ew and a (single) interfering field strength is Ei1, then the DTT reception is ‘acceptable’ (in the absence of noise) if

Ew ≥ Ei1 + PR(f) – POL – DIR,

where PR(f) is the required protection ratio for a given frequency offset, f; POL is the polarization discrimination, if any; DIR is the receive antenna discrimination (vis-à-vis the interfering signal), if any.

‘Nuisance field’ is defined as NUi1, corresponding to the interfering field Ei1 to be

NUi1 = Ei1 + PR(f) – POL – DIR.

Thus, the condition/requirement for acceptable DTT reception in the presence of one interferer (ignoring the noise) can be written simply as

Ew ≥ NUi1

NOISE AND ONE INTERFERER

Taking into account the noise and a single interferer, the requirement for an acceptable reception is

Ew > NUi1  NUN,

where  represents the power sum of the nuisance fields, and NUi1  NUN is the ‘total summed nuisance field’, NUT,

NUT = NUi1  NUN.

Thus, the condition/requirement for acceptable DTT reception in the presence of one interferer and the noise can be written simply as

Ew ≥ NUT.

NOISE AND MULTIPLE INTERFERERS

If there are K interfering signals, Ei1, Ei2, ..., EiK, then the total summed nuisance field (including noise) is

NUT = NUi1  NUi2  ...  NUiK  NUN.

And for an acceptable DTT reception,

Ew ≥ NUT.

UTILISATION OF NUISANCE FIELDS

Nuisance fields can be used in Monte Carlo simulations (as ‘instantaneous’ values) for determining location probabilities and also in analytic expressions (as ‘median’ values) to determine maximum WSD eirp limits.

In Monte Carlo simulations, the statistical variations of the various fields are incorporated directly into the calculations, and the results faithfully reflect those statistics.

In analytical calculations, the statistical behaviour of the fields is only approximated. When multiple interfering WSD sources may be present, it is sometimes proposed to include an appropriate ‘margin’ to produce results which better reflect the more accurate Monte Carlo results.

If such a multiple interference margin should be included ‘IM’, the single-interferer nuisance field for a WSD interferer would be modified to incorporate the IM term.

NUWSD = EWSD + PR(f) – POL – DIR + IM.

The appropriate magnitude of IM would depend on the number of interfering WSDs to be expected.

The usual power summation would again be used to determine the total summed nuisance field.

Number of WSD and Nuisance Power

In order to assess whether broadcasting service protection is granted from the WSD operation the protection criteria of the broadcasting service together with the planning principles have to be applied, for example as laid down in GE06.

At a given location more than one WSD can operate and these devices can use different channels available in this location. Some channels have the same interference impact on the wanted channel received in the corresponding location (example: channels N+1, N-1, N+9 have similar Protection Ratios (PR) with regard to channel N, therefore if they are used simultaneously in the same location their interference on channel N will add up correspondingly). Figure 27 sketches the situation.

WSD channel N-1

PR = -30 dB

d = 20 m

WSD channel N+1

PR = -30 dB

WSD channel N+9

PR = -30 dB

DTT receiving location, channel N

Figure 27: Example of geographical configuration and channel combination

Based on this simple but realistic configuration, a margin of 5 dB (corresponding to the difference between one single WSD and 3 WSDs with equal impact) is proposed.

In order to further investigate this effect of cumulative interference from multiple WSDs, this section contains a detailed study showing that WSD adjacent channel usage, with a fixed antenna at 10 m height, within a DTT coverage area can lead to cumulative interference effects with respect to DTT fixed reception. The probability and the magnitude of the cumulative effect are shown to be functions of the density of WSDs used in a given area.

In order to properly take into account the impact of cumulative interference caused by several WSDs in the UHF band the following principles have to be considered:

- Cumulative interference can result from

- different WSDs using the same channel in different geographical locations or

- different WSDs using different channels in the same geographical locations.

- Two options exist:

  - Option 1: Single WSD treatment: The maximum permissible output powers on the channels potentially available for a single WSD have to be derived by applying a margin for each WSD (e.g. 5 dB for the adjacent channels configurations).

  - Option 2: Multiple WSDs treatment: The maximum permissible output powers on the channels potentially available for all relevant WSDs have to be derived in a calculation that takes into account the cumulative interference in a single step. The allocation of powers and the channels onto the WSDs, i.e. the underlying algorithm, rests with the data base management system. This Option 2 could imply continuous adjustment of the already allocated powers each time a new WSD comes into operation.

An alternative solution, yet to be proven implementable in practice, would consist in maintaining minimum separation distances between fixed WSD transmitters (using the same or different channels), in order to protect DTTB reception. .

For simplicity, as an approximation, it has been considered only the interfering median nuisance powers from the WSDs (i.e. noise is not taken into account) and their power sum. Because of the short distances involved, SEAMCAT Hata propagation model has been used.

It has been considered explicitly only the case of non-co-sited adjacent channel WSD base stations. Otherwise the multiple interference margin would have to be increased by at least 10 log N, where N is the number of co-sited WSD transmissions.

Nuisance power and its application

In Figure below is a graphical representation of the maximum fixed WSD EIRP limits to protect fixed DTT reception, using a location probability degradation of ΔLP=0.1% criterion.

Figure 28:: Maximum fixed WSD EIRP limits for fixed WSDs (LP = 0.1%)

It is seen that, at coverage edge (Emed = 56.21 dBµV/m):

- Ptx_lim = -23.5 dBm (channel N1, N + 9, PR = -30 dB),

- Ptx_lim = -13.5 dBm (channel N2, PR = -40 dB),

- Ptx_lim = -3.5 dBm (channels N3 and beyond, PR = -50 dB).

At 20 m distance from the fixed interfering WSD transmit antenna, the limiting median power present at the DTTB fixed receive antenna is Prx_lim = Ptx_lim – LFS(0.02) – POL, where LFS(0.02) = 54.73 dB is the free space loss at 20 m, and POL = 3 dB slant polarisation discrimination.

Thus, explicitly, at the DTTB coverage edge (56.21 dBµV/m) the limiting median received interfering powers are

- Prx_lim = -23.5 dBm – 57.73 dB = -81.23 dBm (channel N1, N + 9, PR = -30 dB),

- Prx_lim = -13.5 dBm – 57.73 dB = -71.23 dBm (channel N2, PR = -40 dB),

- Prx_lim = -3.5 dBm – 57.73 dB = -61.23 dBm (channels N3 and beyond, PR = -50 dB).

Notice that, in each case Prx_lim + PR = -111.23 dBm. The quantity “Prx_lim + PR” is called the ‘limiting median nuisance power’, Pnuis_lim, and it has a single value for all adjacent channel configurations. Thus, the limiting median nuisance power must not be exceeded at the DTT receive antenna in order to protect DTT reception to the required extent. This approach allows us to treat all relevant adjacent channels simultaneously, on an equal footing.

More generally, the quantity Pnuis = “Prx + PR – [POL, DIR]” is called the median ‘nuisance power’.

If there are 2 or more interfering WSDs (WSD1, WDS2, etc) each will produce a median nuisance power (Pnuis_1, Pnuis_2, etc). The power sum of these nuisance powers will give rise to a total median nuisance power Pnuis_tot (= Pnuis_1 + Pnuis_2 +...), using power summation. In particular, in order to protect DTT fixed reception, the total (summed) median nuisance power must not exceed the limiting median nuisance power: Pnuis_tot ≤ Pnuis_lim.

Propagation loss: determination and usage

SEAMCAT Hata propagation model has been used to determine the loss, LOSS(d), at a distance d between the WSD transmit antenna and the DTT fixed receive antenna, and thus the median receive power, Prx_med, at the DTT receive antenna (including the slant polarisation discrimination, if aplicable):

Ptx – LOSS(d) – POL = Prx_med.

Knowing the median receive power, Prx_med, and the relevant adjacent channel protection ratio, PRi, the corresponding median nuisance power can be determined Pnuis_med_i = Prx_med + PRi.

In order for DTT fixed reception to be protected from the WSD transmission, the median wanted power, Pw_med, at the DTT receive antenna must satisfy Pw_med ≥ Pnuis_med_i for each relevant adjacent channel Ni. In particular, at a 20 m separation distance (and more) Pnuis_lim ≥ Pnuis_med_i.

In the presence of two (or more) adjacent channel interferences, N+i and N+j (and N+k,...) say, the individual median nuisance powers (Pnuis_i, Pnuis_j, Pnuis_k, ...) must not only not exceed the limiting median nuisance power, Pnuis_lim, but also the total median nuisance power Pnuis_tot = Pnuis_i + Pnuis_j + Pnuis_k + ..., must not exceed Pnuis_lim.

WSD cumulative interference

In this section, it is considered the possibility of multiple interference due to WSD transmitters, assuming a WSD transmitter density, , of 1 to 10 WSD base stations per pixel (and also  = 17 to give a potential upper limit).

It is considered a constellation consisting of 9 pixels (100 m x 100 m) as shown in Figure 27. For simplicity of explanation, in Figure 27 it was considered  = 5 WDS base stations per pixel (the same pictorial representation could be made for any number of WSDs by increasing or decreasing the number of points to ).

A DTT receive site is located at the centre of the central pixel (indicated by the star). A WSD is placed at 20 m in front of the star to represent a single WSD interferer. It were choosed ( - 1 = 4) other WSD sites within the central pixel in a random manor. WSD sites are chosen only if they are separated by 20 m or more from the DTT receive site. To get ‘representative’ results, let’s consider 1 000 000 random distributions of the possible  - 1 WSD sites.

For a given Monte Carlo trial, a random distribution of  - 1 WSD transmitters is selected within the central pixel (see the small blue ‘circles’); the same relative distribution of the  WSDs is positioned within each of the other 8 pixels (see the small red ‘circles’). This gives an approximate WSD ‘network structure’ with which to do calculations. Interference calculations (power summing the individual contributions) are carried out for each  WSD distribution to determine the cumulative interference effect of the WSDs. The 8 pixels (and their WSDs) surrounding the central pixel are included to ensure that the full effects of cumulative WSD interference are reflected.

The eirps of the WSDs are chosen so that the limiting median nuisance power, Pnuis_lim, at 20 m distance is Pnuis_lim = -106.73 dBm (from section 3.1 above). Recall that Pnuis_lim = Prx_perm + PRi – [POL, DIR]. Prx_perm is the limiting permissible power occurring 20 m away from the WSD at a potential DTTB receive antenna , PRi is the relevant adjacent channel protection ratio.

20m

Figure 29: 9 pixels with  = 5 WSD transmitters per pixel

The DTT receive antennas are all assumed to point to the top of the diagram in above, the assumed direction of the ‘faraway’ DTT transmitter. The Recommendation ITU-R BT.419 [3] antenna pattern is used to determine the discrimination of the DTT receive antenna relative to the interfering WSDs, including a 3 dB slant-polarisation discrimination (see above).

Figure 30: ITU-R BTF.419 antenna pattern

1 000 000 Monte Carlo trials were carried out for each WSD density (i.e. 1 000 000 different random distributions of  = 1 to 10, and  = 17 WSDs, respectively, within the central pixel, with ‘clones’ placed in the surrounding 8 pixels). The distribution of the cumulative interfering nuisance powers occurring at the DTTB receiver site is shown in Figure 31, Figure 32,Figure 33, in increasing ‘close-ups’.

Figure 31: Distribution of the cumulative interfering nuisance powers

Figure 32: Close-up of Figure 57

Figure 33: Close-up of Figure 57

In Table 9 bellow are listed the ‘extremes’ of the distributions, according to the number of WSDs per pixel. It is seen that, as the WSD density increases:

- both the minimum and the maximum cumulative nuisance power increase,

- the percentage of incidents where the limiting nuisance power limit is exceeded increases,

- the allowable nuisance power limit is exceeded 0.02 dB to 7.73 dB depending on the density of the interfering WSD networks.

| WSD Density (# of WSDs/pixel) | Cumulative Nuisance Interference Power |  | % exceeding limit + X dB X = |  |  |  |  | Margin (dB) to ensure the single entry limit is observed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Minimum (dBm) | Maximum (dBm) |  |  |  |  |  |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 |  |
| 1 | -106.71 | -106.71 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.02 dB |
| 2 | -106.66 | -103.70 | 6.49% | 1.35% | 0.02% | 0.00% | 0.00% | 3.03 dB |
| 3 | -106.61 | -102.02 | 14.86% | 3.59% | 0.33% | 0.01% | 0.00% | 4.71 dB |
| 4 | -106.57 | -101.34 | 25.10% | 6.74% | 0.99% | 0.04% | 0.00% | 5.39 dB |
| 5 | -106.52 | -100.95 | 38.86% | 10.92% | 2.02% | 0.12% | 0.00% | 5.78 dB |
| 6 | -106.46 | -100.76 | 49.82% | 16.24% | 3.60% | 0.29% | 0.01% | 5.97 dB |
| 7 | -106.40 | -100.60 | 63.42% | 22.44% | 5.65% | 0.56% | 0.02% | 6.13 dB |
| 8 | -106.34 | -100.48 | 76.24% | 29.62% | 8.30% | 1.00% | 0.04% | 6.25 dB |
| 9 | -106.27 | -100.14 | 86.77% | 37.32% | 11.53% | 1.65% | 0.09% | 6.59 dB |
| 10 | -106.22 | -100.02 | 93.95% | 45.38% | 15.28% | 2.59% | 0.18% | 6.71 dB |
| 17 | -105.69 | -99.00 | 91.38% | 54.20% | 19.33% | 3.29% | 0.20% | 7.73 dB |

Table 9: Margin to ensure protection of DTT, considering different WSD densities

In the scenario with  = 1 WSD/pixel, the median nuisance power ‘limit’, -106.73 dBm, at the fixed DTTB receive antenna (at 10 m height) is exceeded by 0.02 dB by the cumulative interference compared to the single entry fixed WSD base station (at 10 m height).

It is seen from the Figures 57 to 59 and Table 9 that that ‘limit’ will be exceeded in the case of multiple WSD interferers by larger and larger percentages, as the density of WSDs/pixel increases. It is seen that the increase in the maximum cumulative nuisance power can be as much as 6 dB or more if the density is as large as  = 7 WSDs/pixel.

The final column in Table 9 indicate by how much the individual eirp limits would have to be reduced in order that the single base station cumulative interference effect is not exceeded. It is seen that with a WSD density of  = 3/pixel or larger, a margin between 4 dB and 6 dB would be appropriate.

All in all, it is seen that cumulative interference effects due to ‘equivalent’ WSD base stations is indeed ‘probable’, with cumulative effects amounting to up to about 6 dB. In order for cumulative effects to be ignored, a ‘rule’ would have to be established that would allow only one adjacent channel (i.e. 1 adjacent channel between N – 8 to N + 9) WSD network to be operated within a given channel-N DTTB coverage area, and only one of the network’s base stations within each pixel.

WSD EIRP LIMITS BASED ON TOTAL NUISANCE FIELD: NUISTOT

In addition to the possibility of having adjacent channel interference arising from the short-distance reference scenarios, there may also be situations where there will be long distance co-channel interference.

The following Figures show that the WSD interference distance can range up to 200 km for 30 m WSD antenna heights and high eirps (e.g. 60 dBm). The Figures have been obtained using the JTG 5-6 propagation model, and are based on protecting 56.21 dBµV/m at 10 m at a DTT coverage edge; this means that an interfering field cannot exceed 5.69 dBµV/m to avoid a degradation LP > 0.1%. Note that neither 3 dB polarisation discrimination not 16 dB receive antenna discrimination have been taken into account. This is because it is not generally known in which direction the DTT antennas will be pointing with respect to the WSD interferer.

With these large separation distances, it is possible that a relatively large number of co-channel WSDs will be contributing to the total interference. Theoretical studies based on dense uniform networks of interferers operating at large distances have shown that the cumulative interference effects can be much as 20 dB increase in compared to the ‘single-entry’ interference effect. This means that aggregated interference effects must be calculated.

Figure A6.:???

It is shown that, when using analytical methods to calculate eirp limits for large numbers of WSDs in a WSD network (either base stations or UEs) located outside a DTT coverage area, the task can be reduced to limiting the total (power-summed) median nuisance field, NUISTOT, arising at the DTT coverage edge. Note that the calculation to determine the total interference when nearby adjacent channel WSDs are also operating (for example, with the reference geometries) is more difficult and is under study.

In order to calculate the total nuisance field, it was considered the situations with 1, 10, 100, and 1000 WSDs. These situations will be divided into 2 sub-cases, one where all the powers are equal, and the other where the powers are not equal and are chosen randomly or non-randomly

The parameters are the following:

- Fw = 56.21 dBµV/m

- w = i = 5.5 dB

- NUISTOT is the total median nuisance field (power sum of the individual median nuisance fields)

- Number of WSDs for cases 1 to 4: N1 = 1, N2 = 10, N3 = 100, N4 = 1000.

- Equal individual median nuisance field for each WSDs for cases i = 1, 2, 3, 4: NUISi = NUISTOT – 10 log Ni.

In Figure A6.1 the degradation in location probability, LP, is plotted as a function aggregate median nuisance field, NUISTOT. NUISTOT ranges from 25 dBµV/m to 75 dBµV/m. The first simulations involved 100 000 trials (in steps of 1 dBµV/m).

It is seen that:

- the curves are ‘higher’ as the number of WSDs increases,

- the curves converge for sets of WSDs containing more than about 100 WSDs

- all the curves converge for total nuisance fields less than about 35 dBµV/m, i.e. for LP values less than about 1% or 2%.

Because the protection criterion for protecting DTTB reception is LP ≤ 0.1%, this convergence indicates that a simplification in WSD EIRP/nuisance field management might be possible.

Figure A6.1: LP vs. NUISTOT (equal partition of total nuisance field)

FURTHER CONSIDERATIONS

Since the curves seem to converge into a single curve at small values of LP, we consider this situation more closely. We look at the case of 100 WSDs and at a narrower aggregate nuisance field region: 15 dBµV/m to 35 dBµV/m.  This is the region encompassing LP = 0.1%. We consider a situation with 100 WSDs and 5 variations:

- V1 (‘equal’) consists of all WSDs having the same individual median nuisance field, NUISi (whose power sum is NUISTOT)

- V2 (‘random’) consists a uniform random distribution of individual median nuisance fields (whose power sum is NUISTOT),

- V3 (‘slant’) consists a ‘stepped’ (i.e. evenly spaced in the linear domain) distribution of individual median nuisance fields (whose power sum is NUISTOT);

- V4 (‘10 equal’) consists of 10 WSDs with equal median nuisance fields (whose power sum is NUISTOT), and 90 WSDs with no nuisance field (i.e. 0 mV/m in the linear domain)

- V5 (‘1 equal’) consists of 1 WSD with median nuisance field = NUISTOT, and 99 WSDs with no nuisance field (i.e. 0 mV/m in the linear domain).

In Figure A6.2 the degradation in location probability, LP, is plotted as a function of NUIStot. NUIStot ranges from 15 dBµV/m to 35 dBµV/m, for each of the 5 variations. The simulations again involved 100 000 trials (in steps of 0.5 dBµV/m).

Figure A6.2: LP vs. NUISTOT (100 WSDs, various partitions of total nuisance field)

Again it is seen that the curves are nearly identical in this region. It is felt that the small variations in the curves are due to the fineness of the statistical distribution in this very low LP region.

Notice that, for any given NUISTOT, the LP is the ‘same’ for whatever combination of 100 WSD nuisance fields which add up to NUISTOT.

A still closer view around LP = 0.1% is given in Figure A6.3. To obtain more precision, 30 000 000 trials were used at 0.01 dB spacing.

It is seen that for LP = 0.1%  0.0004%, NUISTOT = 24.545 dBµV/m  0.015 dB for all the curves.

Other things to note at the low end of NUISTOT are the following:

- - LP is higher for a smaller number of equal WSD contributors to the interference (see the dotted and dashed lines in Figure A6.3 for ‘1 equal’, ’10 equal’, and ‘100 equal’ WSDs); this is in contrast to Figure A6.1 where the ‘order’ is reversed,

- - in the case of 100 WSDs, it is seen that a ‘slant distribution’ (solid red line) and a ‘random distribution’ (solid yellow line) of 100 individual interference contributions gives higher values of  LP than 100 equal contributions (lowest dotted line), seeming to be close to the distribution for 10 WSDs (middle dot-and-dashed line) having equal interference contributions.

- - for the case of 1 WSD interferer only, NUISTOT  24.53 dBµV/m, for 100 equal WSD interferers, NUISTOT  24.56 dBµV/m; the difference between these two values is 0.03 dB. We propose NUISTOT = 24.545 dBµV/m as the single limit value which will protect against all WSD situations to 0.1%  0.0004%.

Thus, a single value of NUISTOT emerges to protect DTTB from all WSD configurations. This includes the possibility to take into account distant adjacent channel interference.

Figure A6.3: LP vs. NUISTOT (various partitions of total nuisance field)

CONCLUSIONS

It is proposed that a total aggregate nuisance field limit NUISTOT = 24.545 dBµV/m be used as the limiting value when considering all contributing co-channel (or adjacent channel) sources of WSD interference which are operating outside of the co-channel DTT coverage area.As mentioned above, the case where the reference scenarios are also contributing interference, the calculation is more difficult (and requires further study).

For any given WSD and any given DTTB reception site (usually located at the DTTB coverage edge) the individual nuisance field, NUISi, is calculated as

NUISi = FSi (d, Hti, 10 m, EIRPi) + PR – POL – DIR 					      (A6.1)

where FSi (d,Hti,10m,EIRPi) is

- the field strength at the distance d (between the WSD and the DTTB receive site), at the DTTB receive height 10 m, resulting from a WSD with EIRPi, and antenna height Hti,

- and PR is the protection ratio, POL is the polarization discrimination (if any) and DIR is the DTTB receive antenna discrimination.

- The aggregate median nuisance field NUISTOT is the power sum of all contributing individual median nuisance fields

DTT COVERAGE AREA AND RECEPTION MODE CONSIDERATIONS

In this section it is shown:

- various relationships between DTT wanted field strength, location probability, propagation distance, I/N;

- in particular, their relationship to fixed DTT reception as well as portable outdoor/indoor DTT reception modes is illuminated for a protection criterion related to a fixed permissible degradation, LP, of the DTT reception location probability;

- A comparison of maximum WSD eirp limits and corresponding I/N values (as a function of protection ratio) for various values of LP.

DTT coverage areas are often planned for a specific reception scenario, however other reception modes may also be available within the same area, or within sub-areas as shown in the next Table.

| Within planned reception area | Within sub areas of planned reception area |
| --- | --- |
| Fixed | Portable outdoor, portable indoor |
| Portable outdoor | Portable indoor |

Such additional reception possibilities must not be interfered with by non-licensed WSD services. This aspect of protection for DTT reception and the required protection criteria are treated in the next sections.

Wide area rural environment (planned for fixed DTT reception)

Figure 59 displays the median wanted field strength levels (solid blue curve) as a function of distance from the DTT transmitter. In this case, the coverage area has a 50 km radius serving a rural district. The transmit antenna is 300 m, radiating with an erp = 10.6 dBkW, providing a 56.21 dBµV/m field strength at 50 km distance (and 10 m reception height). The entire 50 km radius coverage area serves fixed (F) DTT antenna installations. In addition, portable outdoor (PO) reception at 1.5 m is possible within a 20 km radius of the DTT transmitter and portable indoor (PI) reception at 1.5 m is possible within a 10 km radius of the DTT transmitter.

Figure 34: Wide area rural environment (50 km radius, F, PO, PI reception)

Large area suburban environment (planned for fixed DTT reception)

Figure 60 displays the median wanted field strength levels (solid blue curve) as a function of distance from the DTT transmitter. In this case, the coverage area has a 25 km radius serving a suburban district. The transmit antenna is 150 m, radiating with an erp = 0.625 dBkW, providing a 56.21 dBµV/m field strength at 25 km distance (and 10 m reception height).

The entire 25 km serves fixed (F) DTT antenna installations; portable outdoor (PO) reception at 1.5 m is possible within a 7 km radius of the DTT transmitter; portable indoor (PI) reception at 1.5 m, inside, is possible within a 3.3 km radius of the DTT transmitter.

Figure60: Large area suburban environment (25 km radius, F, PO, PI reception)

Medium area suburban environment (planned for fixed DTT reception)

Figure 61 displays the median wanted field strength levels (solid blue curve) as a function of distance from the DTT transmitter. In this case, the coverage area has a 10 km radius serving a suburban district. The transmit antenna is 150 m, radiating with an erp = -16.05 dBkW, providing a 56.21 dBµV/m field strength at 10 km distance (and 10 m reception height).

The entire 10 km serves fixed (F) DTT antenna installations; portable outdoor (PO) reception at 1.5 m is possible within a 1.9 km radius of the DTT transmitter; portable indoor (PI) reception at 1.5 m, inside, is possible within a radius of 0.8 km from the DTT transmitter.

Figure 35: Medium area suburban environment (10 km radius, F, PO, PI reception)

small area urban environment (planned for PO DTT reception)

Figure 62 displays the median wanted field strength levels (solid blue curve) as a function of distance from the DTT transmitter. In this case, the coverage area has a 5 km radius serving an urban district with PO reception. The transmit antenna is 75 m, radiating with an erp = 0.68 dBkW, providing a 78.21 dBµV/m field strength at 5 km distance (and 10 m reception height; i.e. 61.21 dBµV/m at 1.5 m reception height).

The entire 5 km serves portable outdoor (PO) reception at 1.5 m; portable indoor (PI) reception at 1.5 m, inside, is possible within a 2.5 km radius. Needless to say that fixed reception at 10 m is possible everywhere.

Figure62: Small area urban environment (5 km radius, F, PO, PI reception)

Conclusions

It is seen in Figures 59 to 62 that significant sub-areas of any DTT coverage area is suitable for portable outdoor (PO) and portable indoor (PI) DTT reception.

Any limits to be set on WSD eirp values must provide protection not only for fixed DTT reception, but also protect PO and PI reception in those areas where this reception is possible.

For this reason, maximum WSD eirp limits must be calculated very carefully, taking into account each and every reception mode possible in the area, using the most restrictive limits in order to protect every relevant DTT reception mode.

RELATIONSHIPS BETWEEN DTT WANTED FIELD STRENGTH, LOCATION PROBABILITY, PROPAGATION DISTANCE, I/N

This section discusses the relationship between the DTT wanted field strength and various parameters:

- Wanted field strength vs. the corresponding propagation distance,

- Wanted field strength vs.  the reception location probability, LP,

- Wanted field strength vs. the resulting I/N to maintain LP = 0.1%.

This exposition will give an idea of the behavior of these parameters within a DTT coverage area and not just at the DTT coverage edge.

Fixed DTT reception areas

Within a DTT coverage area, the wanted field strength generally increases the closer the DTT receiver is to the DTT transmitter. At the same time, the location probability (LP) for acceptable DTT reception also increases.

Figure 63 displays the dependency of LP on wanted DTT field strength. Figure 64 displays two other related relationships, viz.

- the relationship between distance and wanted DTT field strength, and

- the relationship between I/N on wanted DTT field strength which result when maintaining LP = 0.1%.

In Figure 64, the horizontal axis represents the wanted field strength, ranging from 56.21 dBµV/m to 116.21 dBµV/m. The vertical axis represents either a propagation path distance (measured in km) or a value of I/N (measured in dB), as relevant.

LP vs. median wanted field strength

Figure 63 shows the relationship between the wanted field strength and the corresponding LP, in the presence of noise only. It is representative for an entire fixed reception DTT coverage area.

It is seen from the Figure 63 that, for example,

- the wanted field strength = 56.21 dBµV/m corresponds to a 95% LP (in the presence of noise only)

- the wanted field strength = 66.21 dBµV/m (i.e. a 10 dB increase) the LP raises to 99.97%, and

- the wanted field strength = 74.21 dBµV/m (i.e. with a further 8 dB increase), the LP rises to 100%.

Figure 63: Fixed DTT reception: LP vs. Wanted Field Strength (WFS)

Coverage distance vs. wanted field strength

In this example, for a land propagation path, the frequency is 600 MHz, the wanted transmitter erp is 10.5 dBkW = 11.2 kW, and the transmit antenna height is 300 m; Recommendation ITU-R 1546 is the propagation model used.

The dashed blue curve in Figure 64 shows the distance from a DTT transmitter as a function of the wanted field strength. The upper 2/3rds (0 km to 50 km) of the vertical axis in this case represents the propagation distance, ranging from 0 km to 50 km.

It is seen that, as the wanted field strength increases, the distance to the wanted transmitter decreases. In particular, in this example, the propagation distance 50 km corresponds to wanted field strength = 56.21 dBµV/m, and the propagation distance 0.9 km corresponds to wanted field strength = 116.21 dBµV/m.

Figure 64: Fixed DTT reception: I/N and Propagation Distance vs. Wanted Field Strength

I/N vs. WANTED FIELD STRENGTH

In order to calculate [I/N]med vs. median wanted field strength, the following model was used:

- within a given pixel having a given median wanted field strength, the LP (in the presence of noise only) was calculated using Monte Carlo simulation.

- a second Monte Carlo simulation to determine LP was carried out after introducing a median interfering field, I, with 3.5 dB standard deviation. Noise and interference were power summed in the simulations.

- [I/N]med values were calculated from median interfering and wanted field strength values leading to LP = 0.1%. One million (1 000 000) trials were used in each simulation.

The solid curve in Figure 64 shows the relationship between [I/N]med and wanted field strength for LP = 0.1%. The vertical axis in this case represents the values of [I/N]med ranging from -25 dB to 50 dB. The wanted field strength can increase by as much as 50 dB or more as the wanted transmitter is approached. Correspondingly, [I/N]med can increase up to 50 dB or more, if not limited.

This represents an [I/N]med which is 70 dB (or 60 dB) higher than [I/N]med = -20 dB (or -10 dB) often used as a protection criterion.

From the Figure F6, it is seen that, within a 50 km radius DTT coverage area, 35 km is the distance at which a wanted field strength = 66.21 dBµV/m (for LP  100%), 10 dB higher than at the coverage edge.

Thus, (35/50)2  70% of the coverage area has  100% LP. This means that, in the central coverage area (i.e. less than, equal 35 km distant from the DTT transmitter) [I/N]med ranges from [I/N]med = -3.5 dB at 35 km from the transmitter to [I/N]med = 48.9 dB at 1 km from the transmitter for LP = 0.1

Note in Figure F6 that excessive values of I/N are reached within DTT coverage areas if adjacent channel WSDs are not subject to eirp limits. Setting a fixed value of permissible ΔLP leads to increased values of I/N, potentially up to I/N = 30 dB, or more.

Such large I/N values imply correspondingly large interference powers which, unless restricted, can lead to DTT receiver overloading (the consequences of this are discussed in more detail in section A5.5.4).

PO and PI DTT reception areas

The behavior of LP, I/N and propagation distance vs. wanted field strength is similar for portable outdoor and portable indoor DTT reception areas is similar to that for fixed DTT reception, as shown in the following four Figures 65 to 68. Note that the wanted field strength values are calculated at 1.5 m DTT receive antenna height (outdoors).

The relationships between wanted field strength, LP, I/N and propagation distance are analogous to those discussed in section A4.21 above and will not be discussed further here.

Figure 65: Portable Outdoor reception: LP vs. Wanted field strength

Figure 66 Portable Outdoor reception: I/N and Distance vs. Wanted field strength

Figure 67 Portable Indoor reception: LP vs. Wanted field strength

Figure 68 Portable Indoor reception: I/N and Distance vs. Wanted field strength

STUDY PARAMETERS, FORMULAS AND EIRP CONSTRAINTS

General

- At the DTT antenna input, the median field strength and median (and instantaneous) received power are related by:

Pr_dBm = EdBµV/m – 20 log fMHz – 77.2 ; f = 650 MHz

Pt = Pr + LOSS(r) + POL + DISCr + ATTt

LOSS is the propagation loss, POL the polarization discrimination, if any, DISCr the receive antenna discrimination, and ATTt the transmit antenna attenuation.

Treating at the receive antenna input, we don’t need to take into account Ga (except when dealing with DTT receiver overload).

- Free space loss: LOSS(rkm) = 32.5 + 20 log fMHz + 20 log rkm

- Median wanted field strength: Ew_med ; standard deviation: w

- Required C/N ratio: [C/N]

- N effective noise at antenna input: N = Ew_med – [C/N] – µw

- Probability factor: µ = 1.645 for 95% LP, µ = 2.323 for 99% LP, µ = 3.090 for 99.9% LP

- Ei_med median interfering field at antenna input, wsd = 3.5 dB

- For non-interference at a DTT reception site: Ei + PR < Ew

- Height loss (10 m – 1.5 m): 17 dB

- Wall penetration loss: 8 dB, wall = 5.5 dB

- DTT receiver overload (dBm): Oth

- DTT receive antenna gain: Ga

- Pt < Oth – µx%wsd + [POL, DISCTV] + DISCWSD – Ga + LOSS(d) (+ wall loss if any)

WSD eirp to I/N CONVERSIONS

In this section, for information, the determination of maximum WSD eirp limits assuming I/N restrictions instead of degradation to location probability LP = 0.1% are imposed on WSD operation, to protect a given location probability, X%

WSD EIRPS LIMITED BY PROTECTION RATIO CONSTRAINTS

Let N represent the DTT noise power (thermal plus receiver noise), Pt the WSD eirp, and Nn the nuisance noise power:

Nn = N + [C/N]

and Imed the median WSD power in the DTT receiver

Imed = Pt – LOSS(r) - ATTt – POL – DISCr + Ga

Let In represent the median interference nuisance power

In = Imed + PR.

Then,

[In/Nn]med = Imed + PR – (N + [C/N] = [Imed/N] + PR – [C/N]

= Pt – LOSS(r) – ATTt – POL – DISCr + Ga + PR – [C/N]

WSD EIRP LIMITED BY PROTECTION RATIO AND [I/N] CONSTRAINTS

The ambient ‘effective noise’ field strength, Neff, and corresponding powers are calculated from the minimum median field strength, Ew_dBµV/m (assuming 95% location probability).

Neff = Ew_dBµV/m – [C/N] – µ95%w

The ‘effective noise’ power, Peff, is

Peff = Neff – 20 log fMHz – 77.2

The median nuisance noise power, Pn, is

Pn = Peff + [C/N]

For a given median field strength (wanted or interfering), Emed, the median ambient power, Pr, at the DTT receive antenna input is

Pr_ dBm = Emed – 20 log fMHz – 77.2

Taking account of the relevant protection ratio, PR, the median interfering nuisance power, Inuis, at the DTT receive antenna input, due to an ambient interfering field, Pr, is

Inuis = Pr + PR

[Inuis/Pn]med  is the median interfering nuisance field/power ‘divided’ by the nuisance noise field/power. Here we work with powers (dBm);

[Inuis/Pn]med = Pr + PR  –  Peff – [C/N] =

= Pt – LOSS(r) – ATTt + PR – Peff – [C/N]

If the median limiting [I/N] is set as [I/N]lim, then we need as the protection criteria, for protecting DTT reception for X% location probability,

Pt < [I/N]lim + LOSS(r) + ATTt – PR + Peff – µx% (w2 + wsd2)½

EXAMPLES:

We calculate examples for 3 reception modes.

Fixed reception:

- Ew = 56.21 dBµV/m, µ = 1.645 (95%),  = 5.5 dB, C/N = 21 dB

- Effective Nf = 26.16 dBµV/m  Peff = -107.30 dBm

- Noise nuisance power: Nf + C/N = Nf + 21

- LOSS = 54.77 dB, [POL, DISC] = 3 dB, DIR = 0, wall loss = 0 dB

- Pr = Pt – 54.77 – 3 – 0 – 0 = Pt – 57.77

- Pt < [I/N]lim + 57.77 – PR + 21 + (-107.3) + µx% (w2+wsd2)½ = [I/N]lim – PR – 28.5 + µx% (w2+wsd2)½

Portable Outdoor reception:

- Ew = 61.21 dBµV/m, µ = 1.645 (95%),  = 5.5 dB, C/N = 19 dB

- Effective No = 33.16 dBµV/m  Peff = -100.3 dBm

- Noise nuisance power: No + C/N = No + 19

- LOSS = 55.5 dB, [POL, DISC] = 0 dB, DIR = 10, wall loss = 0 dB

- Pr = Pt – 55.5 – 10 – 0 – 0 = Pt – 65.5

- Pt < [I/N]lim + 65.5 – PR + 19 + (-100.3) + µx% (w2+wsd2)½ = [I/N]lim – PR – 15.8 + µx% (w2+wsd2)½

Portable Indoor reception:

- Ew = 62.95 dBµV/m inside at 1.5 m, µ = 1.645 (95%),  = 7.78 dB, C/N = 17 dB

- Effective Ni = 33.16 dBµV/m  Peff = -100.3 dBm

- Noise nuisance power: Ni + C/N = Ni + 17

- LOSS = 55.5 dB, [POL, DISC] = 10 dB, DIR = 0, (inside at 1.5 m, so) wall loss = 8 dB

- Pr = Pt – 55.5 – 10 – 0 – 8 = Pt – 73.5

- Pt < [I/N]lim + 73.5 – PR + 17 + (-100.3) + µx% (w2+wsd2)½ = [I/N]lim – PR – 9.8 + µx% (w2+wsd2)½

WSD EIRP LIMITED BY OVERLOAD AND [I/N] CONSTRAINTS

In the case of overloading, [I/N]med is simply Oth/N, where N is the actual noise power in the receiver. In the preceding subsection, it has been the ‘effective noise’, Neff, which has been used. The effective noise at the DTT antenna input and the actual noise in the DTT receiver are related by Neff + Ga= N.

So at DTT overload, to protect at X% locations,

[I/N]med = Oth/N – µx%wsd =  {Pt – LOSS(r) – POL – DISCr – ATTt + Ga}/N

[I/N]med = Pt – LOSS(r) – POL – DISCr – ATTt + Ga – N.

If [I/N]med limits are set, [I/N]lim, this would imply that limits on Pt would also result, including DTT protection for X% location probability,

Pt < [I/N]lim + LOSS(r) + POL + DISCr + ATTt – Ga + N – µx%wsd

[THIS OLD ANNEX 4 IS TO BE ANALISED BY EBU ABOUT THE PROPOSAL FOR DELETION]

DTTB

20 m

d

d

‘‘Z’’

60°

‘‘Y’’

‘‘X’’

D

-

CONSIDERATIONS OF DEGRADATION OF COVERAGE LOCATION PROBABILITY FOR THE DETERMINATION OF MAXIMUM WSD EIRP LIMITS

Introduction

This annex is aimed to assist administrations in assessing an appropriate level for the degradation of the location probability to be used in the geo-location database calculations for the protection of the broadcasting services. In particular, considerations supported by calculation examples are provided (i) regarding the approach when the probability degradation is set to a fixed value and (ii) regarding the approach when the probability degradation varies across the coverage area of the broadcasting service.

As stated in the section 5.1.2 of the report, two options could be used when applying the methodology developed in the ECC Report 159 and presented in section 5.1.1 of this report.

This Annex provides the following information in various sections:

- Section A5.2 briefly summarize the parameters of the 8 reference WSD to DTT protection scenarios (see Annex 2) that are used in other sections.

- Section A5.4 details

- various relationships between DTT wanted field strength, location probability, propagation distance, I/N

- in particular, their relationship to fixed DTT reception as well as portable outdoor/indoor DTT reception modes is illuminated for a protection criterion related to a fixed permissible degradation, LP, of the DTT reception location probability.

- A comparison of maximum WSD eirp limits and corresponding I/N values (as a function of protection ratio) for various values of LP is provided.

- Section A5.3 considers the WSD eirp limits based on a fixed value for the acceptable degradation of the coverage probability, LP = 0.1%, providing

- a theoretical derivation of an analytical equation for calculating WSD eirp limits based on DTT receiver overload thresholds as well as providing justification using Monte Carlo simulation

- a derivation of absolute WSD eirp limits based on DTT receiver overload (for both fixed and UE WSDs)

- a derivation of relative WSD eirp limits based on a protection ratio, PR = 0 dB, which by translation provides WSD eirp limits for all protection ratio values (for both fixed and UE WSDs)

- a derivation of WSD eirp limits based on combined protection ratio and overload threshold (for both fixed and UE WSDs)

- consideration of WSD eirp limits based on I/N limits

- Section A5.4 describes the determination of limiting WSD eirp values near areas with differing DTT reception modes

- Section 5.5 considers the possibility of adopting variable acceptable degradation of the coverage probability, providing

- a theoretical derivation of interference upper limits at the DTT receiver according to the capability of the receiver in dealing with interference, i.e. by respecting protection ratios and receiver overloading thresholds;

- a derivation of WSD eirp limits based on the interference limits at the DTT receiver (for both fixed and UE WSDs)

- derivation of LP upper limits based on the derived interference limits at the DTT receiver (original and resulting LPs assessed through Monte Carlo simulations)

- discussion on the impact of LP levels higher than 0.1% on the DTT service

- Section A5.6 details possible ramifications when choosing the degradation of the location probability

- It should be noted that the protection considerations in this Annex only apply to WSD interference interior to a DTT coverage area, and as such, WSD co-channel usage will not be allowed.

- The treatment of co-channel (and adjacent channel) WSD interference arising from sources outside the DTT coverage area is treated in Annex 6

WSD INTERFERENCE SCENARIOS

Various reference scenarios are considered to determine the appropriate WSD eirp limits in most foreseen adjacent channel protection situations (see Annex 2) where the interfering WSDs are within a few dozen meters of the DTT antennas. In co-channel interference situations, the interfering WSDs and the DTT antennas may be separated by many dozens of km. Reference scenarios have not been developed for this situation. Instead, for example, the WSD interference potential is calculated between the WSD (with its given characteristics) and the DTT reception mode (e.g. fixed at the DTT coverage edge.

In Table T1 bellow, the relevant parameters are summarized (using the abbreviations of the preceding section). The values of Emed for DTT are given at the receive antenna heights indicated. For example, for DTT, F (10) means that the DTT receive antenna height is 10 m; F (1.5) means that the DTT receive antenna height is 1.5 m.

In order to determine the corresponding Emed at 1.5 m, or 10 m, respectively, it is necessary to take into account the field strength height loss between 10 m and 1.5 m:

Field strength Height loss (10 m – 1.5 m) = 17 dB

Emed (at 10 m) = Emed (at 1.5 m) + 17 dB

These parameter values can be used in the equations in sections A5.5 and A5.6 below to evaluate the relevant eirp limits, [I/N] values, etc, according to the relevant interference scenarios.

| Scenario |  | Emed (dBµV/m) | Pmed (dBm) | LOSS(d) (dB) | DISC/POL (dB) | DISCwsd (dB) | PR (dB) |  | Scenario Description |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | WSD |  | DTTB | d (m) |  |
| #1 | 56.21 |  | -77.25 | 56.15 | 0.45 | 0 | 21 | PO(1.5m) |  | F(10 m) | 22 |  |
| #2 | 56.21 |  | -77.25 | 54.72 | 0 | 0 | 21 | PO(10m) |  | F(10 m) | 20 |  |
| #3 | 61.21 |  | -72.25 | 34.72 | 0 | 0 | 19 | PO(1.5m) |  | PO(1.5m) | 2 |  |
| #4 | 56.21 |  | -77.25 | 54.72 | 3 | 0 | 21 | F(10m) |  | F(10m) | 20 |  |
| #5 | 61.21 |  | -72.25 | 55.45 | 0 | 10 | 19 | F(10m) |  | PO(1.5m) | 20 |  |
| #6 | 62.95* |  | -70.51 | 55.45** | 0 | 10 | 17 | F(10m) |  | PI(1.5m) | 20 |  |
| #7 | 56.21 |  | -77.25 | 62.87 | 3 | 13.55 | 21 | F(30m) |  | F(10m) | 47 |  |
| #8 | 61.21 |  | -72.25 | 60.59 | 0 | 18.01 | 19 | F(30m) |  | PO(1.5m) | 27 |  |
| Table T1: Scenario parameters for protection ratio and non-overload conditions Emed wanted median field strength; Pmed wanted median power; LOSS(d) propagation loss DISC/POL receive antenna /polarization discrimination; DISCwsd transmit antenna attenuation PR protection ratio; d separation distance between WSD transmit antenna and DTT receive antenna *62.95 dBµV/m inside at 1.5 m corresponds to 70.95 dBµV/m outside at 1.5 m and to 87.95 dBµV/m outside at 10 m ** there is an additional 8 dB wall penetration loss to take into account, with a 5.5 dB standard deviation |  |  |  |  |  |  |  |  |  |  |  |

COMPARISON LP VALUES

Although LP = 0.1% is a satisfactory (i.e. sufficiently relaxed) protection criterion in a secondary service ‘non-interference’ scenario, Figure F11 displays similar results (left vertical axis [I/N]med vs. wanted field strength) to Figure F6, this time for several values of LP = 0.1%, 0.2%, 0.5%, 1%, 2%, 5% relative to protecting a fixed DTT service. Similar graphs could also be drawn for portable outdoor and portable indoor DTT reception.

In addition, corresponding WSD eirp limits (restrictions) are indicated are indicated (right vertical axis) assuming a PR = -40 dB adjacent channel protection ratio. The -40 dB value is used for computation of the graph, based on a typical value for a noise-like WSD interferer operating in the second adjacent channel to the wanted DTT signal. A co-channel protection ratio of 20 dB is assumed, corresponding to the DVB-T 64-QAM 2/3 mode. The values of eirp are also a function of the DTT receiver performance characterised by the required [C/N] (protection ratio) for the WSD interference.

It is seen that [I/N]med is increased from about -21 dB to about -4 dB at the coverage edge (wanted field strength = 56.21 dBµV/m) and by about 9 dB at points interior to the coverage area, e.g. from about 28 dB to about 37 dB where the wanted field strength = 96.21 dBµV/m,  from about 48 dB to about 57 dB where the wanted field strength = 116.21 dBµV/m, etc.

Note that if the protection ratio were  dB greater than (less than) -40 dB, then the corresponding WSD eirps would be  dB less than (greater than) the corresponding eirp values depicted in Figure F11.

Obviously it is possible to apply LP = 1%, 2%, 5% (or even 50% and more) as a ‘protection criterion’ for WSD interfering with DTT reception.

However a corresponding loss in LP of 1%, 2%, 5%, etc is considered by broadcasters not to reflect the ‘non-interference’ stipulation for the use of secondary services, as this clearly represents a significant loss in coverage.

In addition, the concomitant acceptance of [I/N]med values significantly > 0 dB (e.g. 10 dB, 20 dB, 30 dB, …) would seem to pose a dangerous precedent for other primary services, which may also one day be called upon to ‘share’ with non-licensed secondary services.

For this reason, the LP = 0.1% curve in Figure F11 is considered in detail. It is seen that as the DTT wanted field increases (e.g. approaching the DTT transmitter) the permitted WSD eirp also increases. The DTT wanted field would nevertheless be protected for fixed DTT reception (i.e. with a 0.1% degradation in LP).

However, eventually the interfering field would be strong enough to cause DTT receiver overloading, irrespective of the strength of the wanted DTT field, and therefore a cut-off WSD eirp would be necessary.

Figure F11: Maximum WSD eirp, I/N vs. fixed reception DTT wanted field strength (parameter LP)

CONCLUSION

Often, to protect a primary service against another service a fixed value of I/N is foreseen (e.g. I/N = -20 dB, or -10 dB or -6 dB). Here, to offer more flexibility for implementing WSDs (a non-licensed secondary service), it is proposed to use the degradation to the LP, LP, caused by WSDs as the protection criterion. In particular, because secondary services are allowed to use spectrum on a ‘non-interfering’ basis, the value LP = 0.1% seems appropriate.

USE OF A FIXED VALUE FOR THE ACCEPTABLE DEGRADATION OF THE COVERAGE PROBABILITY

INTRODUCTION

The proposed approach in this section offers a flexible solution, by proposing a fixed value for the ΔLP, but considering nevertheless the real potential of portable outdoor and indoor DTT reception and its protection requirements.

By defining a priori a fixed value for the ΔLP, the interference will be limited, consistent with protection to be provided by a non-licensed service (WSD) for a primary service (DTT). The constant, fixed LP = 0.1% approach is shown to be flexible with respect to extended WSD usage nearer the DTT transmitter, in addition to preserving the DTT viewer’s flexibility in choosing his or her mode of viewing, fixed or portable, outdoor or indoor.

Based on these considerations, the proposed methodology and parameters are as follows:

- The calculation is made according to the methodology described in the ECC Report 159, and reference scenarios presented in Annex A2 (the relevant parameters are summarized in the section A5.3 above);

- Limiting the location probability degradation to 0.1% everywhere in the coverage area of the DTT transmitter.

- Protecting the three modes of reception progressively from the edge of the coverage inwards. The switch to each mode is made as function of the wanted field strength level by referring to the thresholds of field strength above which a given mode of reception is possible (going from fixed roof top to portable outdoor and to portable indoor);

- Limiting the eirp to a maximum level defined by the overloading threshold corresponding to each interference scenario and channel adjacency;

- Taking account of multiple interference from UE WSD by assuming that 3 equivalent UE WSDs are contributing to the interference to DTT coverage when adjacent channels are used (Annex A5 of the Report 159 presents the basis of this assumption);

- Taking account of multiple interference from fixed WSD, if N Fixed WSD transmissions are made from a common WSD transmit antenna, then the eirp limits indicated above must be reduced by a factor 10 log N.

LP = 0.1% AND THE INCREASE OF PERMISSIBLE MEDIAN WSD INTERFERING FIELD STRENGTH  AS A FUNCTION OF MEDIAN WANTED DTT FIELD STRENGTH

It has being proposed a LP = 0.1% criterion to limit the interference potential of WSDs to DTT reception.

This criterion is used with the location specific EIRP calculation taking into account the wanted signal level and allows the WSD EIRP to increase almost without limit as the WSD location is situated closer and closer to the DTTB transmitter.

Figure NN shows how the allowed interfering field of a WSD can increase as we pass from the DTT coverage edge (where the wanted field strength at 10 m receive antenna height for fixed reception is 56.21 dBµV/m) towards the DTTB transmitter (where the field strength may reach 100 dBµV/m or more), still limiting the degradation to location probability to 0.1% (ignoring the overload threshold, however).

Large increases in the interfering field will be due to high values of WSD transmit EIRPs which may lead to overloading effects in nearby DTT receive installations. To avoid this type of situation, WSD EIRP limits must be set.

Figure NN: Eimed vs. Ewmed to achieve LP = 0.1% in the co-channel (PR = 21 dB)

THEORETICAL ANALYSIS

We wish to calculate the suitable WSD EIRP limits on the basis of respecting the overload threshold for any given channel adjacency. In this section we use analytical calculations to get a feel for the magnitude to the problem.

We use a simple model in which a fixed WSD transmit antenna, situated at 10 m height, interferes with a fixed DTTB receiver with fixed antenna at 10 m height. The separation between the WSD and the DTTB is 20 m.

- The wanted signal strength is only taken into account as a reference – we are only interested in the DTT overload threshold and its relationship to Pt_wsd, the transmit EIRP of the WSD transmitter.

- Noise can be ignored compared to the field strength levels where DTT overloading occurs.

- The frequency considered is f = 650 MHz

- The interfering signal is assumed to have standard deviation wsd = 3.5 dB.

Referring to Figure F12, at the DTTB receive antenna level, the field strength (median or stochastic) and received power (median or stochastic) are related by:

Pr_dBm = EdBµV/m – 20 log fMHz – 77.2		(E1)

The (median and stochastic) power entering the receiver, after having passed through the antenna system, will be:

Prec = Pr_dBm – POL + Ga,		(E2)

where POL = 3 dB antenna polarisation discrimination, and Ga is the receive antenna gain (including feeder losses). Ga = 9.15 dBi for fixed DTTB reception, and Ga = 2.15 dBi for portable DTTB reception.

The WSD transmit EIRP, Pt, is related to the median Pr_dBm by

Pt – LOSS (d) = Pr_dBm 		(E3)

where ‘LOSS(d)’ is the median propagation loss over the distance dkm.

To protect against overloading the following must be satisfied:

- Prec < Oth for the stochastic power entering the receiver, and

- Prec < Oth – µx*wsd for the median power entering the receiver to protect at x% location probability.

µx is the probability factor for x% of the locations; e.g. for 95%, µx = 1.6448, for 99.9%, µx = 3.0902.

Pr = EdBµV/m – 20 log fMHz – 77.2

EdBµV/m

Ga (including feeder loss)

Prec = Pr + Ga

Giso = 0 dBi (isotropic) antenna)

Pt

Loss(d) = Pt-Pr
Loss between isotropic antennas

Figure F12: Explanatory diagram of the terminology used in the calculation

As mentioned, we ignore the effects of noise, and we make a simplifying assumption so that an analytical calculation can be carried out easily.

The assumption is explained in the following:

- Protection of DTTB to the extent that only a LP = 0.1% degradation to the location probability is permitted. The protection from overload is taken by using:

Prec < Oth – µxwsd,		(E4)

- We would like to determine the value of “x” which will give a good approximation to a LP = 0.1% degradation.

- We combine equations (E2), (E3) and (E4) as follows:

Pt = Pr_dBm + LOSS(d) =

Pt = Prec + POL – Ga + LOSS(d) =

Pt < Oth – µxwsd + POL – Ga + LOSS(d) 				(E5)

If there is DTT receive antenna discrimination, DISCTV, then the term ‘POL’ should be replaced by ‘[POL,DISCTV]’ which can be taken as max(POL,DISCTV) if ITU-R Rec. 419 is used for the receive antenna characteristic.

If there is WSD transmit antenna attenuation, DISCWSD, then an additional term ‘DISCWSD’ should be introduced into equation E5.

Then we can rewrite equation E5 as:

Pt < Oth – µxwsd + POL + DISCTV + DISCWSD – Ga + LOSS(d)		(E5´)

Knowing the overload threshold value for any given situation, we can evaluate the maximum permitted WSD eirp from equation E5´.

For the fixed DTTB reception case we have

Pt < Oth – µx*3.5 + 3 + 0 – 9.15 + 54.72 = Oth – 3.5µx + 48.57 dBm	(E6)

We can take an example from the overload threshold information provided in ECC Report 159; the Oth values can be as low as -19 dBm for 10th percentile DTTB receivers (-26 dBm for silicon USB receivers).

As an example we chose Oth = -20 dBm. Then equation E5 tells us that

Pt_max =  Oth – 3.5µx + 48.57 dBm = 28.57 – 3.5µx dBm

is the maximum fixed WSD power limit to avoid DTT overload. It should be noted that no cumulative WSD interference effects have been taken into account. For example, if we had 3 co-sited WSDs transmitting from a single site, then the aggregate interference would exceed the OTH, unless of course, the power of each transmission were reduced by 10 log 3 = 4.77 dB. In this case, Pt_max = 28.57 – 3.5µx dBm would be reduced to Pt_max = 23.8 – 3.5µx dBm.

In order to determine the value of “x” to be used in this analytic calculation of the WSD EIRP restrictions due to DTTB overload, we calculate exact results in the following section, using Monte Carlo simulation.

MONTE CARLO SIMULATION

The results in the previous section were based on ‘approximate’ analytic formulas. In this section we carry out the interference calculation using Monte Carlo simulations, with no approximations, in order to determine what value the parameter “x” should be given.

The Monte Carlo simulations are carried out on the following basis, with respect to protection ratio (giving protection information for the small to large WSD eirps) and also with respect to Oth (giving information for the largest WSD eirps). For simplicity, we carry out the calculation using powers (and not field strengths).

At the DTTB coverage edge the median wanted field strength at the DTTB fixed receive antenna (at 10 m height) for 95% location probability is E = 56.21 dBµV/m. We convert this to median receive power (at the receive antenna) using: Pw = E – 20log f – 77.2 = -77.245 dBm.

The calculations are carried out for wanted DTTB receive powers (at the receive antenna) starting at the median value -77.245 dBm and increasing by 1 dB, step by step. This represents the increase in wanted field strength as the receiver approaches the DTTB transmitter.

The Monte Carlo trial for compatibility consists of the usual steps with one additional condition: the interfering power inside the receiver (i.e. including the receive antenna gain and polarization loss) must be less than the DTT receiver overload threshold.

Fixed WSD (10 m) to Fixed DTTB (10 m) (1st adjacent channel interference)

The Monte Carlo simulation was carried out repeating the conditions of the analytical calculation presented in section A5.5.2. The results are shown in Figure F13. It is seen that, as the wanted power at the DTTB receive antenna increases, the WSD allowed transmit power also increases (while maintaining LP = 0.1%).

The curve which includes Oth in the protection criteria, however, increases roughly linearly, but eventually approaches 17.75 dBm asymptotically, and rises no further.

Figure F13: WSD EIRP RESTRICTIONS:  Fixed WSD (10 m) to Fixed DTTB (10 m) 1st Adjacent channel (PR = -30 dB), Oth = -20 dBm

Analytic approximation

We can substitute the previous result into equation E5´ to determine the corresponding value of “x”.

Inserting all the relevant parameters, we find Pt_max = 28.57 – 3.5µx dBm = 17.75 dBm, which means that 3.5µx = 10.82, or µx = 3.091. This value of µx corresponds to x = 99.9%. This means that if we are trying to protect LP at the level LP = 0.1%, we can use x = 99.9% in equation E5´, with no approximation.

- Fixed WSD (10 m) to Fixed DTTB (10 m) (2nd adjacent channel interference)

The same Monte Carlo simulation has been carried out again, this time for the 2nd adjacent channel (PR = -40 dB). Other than the protection ratio, no other parameter has been changed. The results are shown in Figure F14. It is seen that the maximum WSD EIRP is again 17.75 dBm in this case, even though the protection ratio has decreased from -30 dB to -40 dB. This shows, as could be expected, that the overload interference effect is independent of the protection ratio (at least as long as the protection ratio is negative).

Analytic approximation

As shown in the previous subsection, the same result could have been obtained analytically using equation E5´ and x = 99.9%.

Figure F14: WSD EIRP RESTRICTIONS:  Fixed WSD (10 m) to Fixed DTTB (10 m) 2nd Adjacent channel

(PR = -40 dB), Oth = -20 dBm

- Fixed WSD (10 m) to PO DTTB (1.5 m) (2nd adjacent channel interference)

We examine one more case using Monte Carlo simulation: Fixed WSD (10 m) to DTTB PO (1.5 m). We use 2nd adjacent channel interference, even though, as we have seen before, the result is independent of the value of the (negative) protection ratio. The results are shown in Figure F15. It is seen that the maximum WSD eirp is 32.48 dBm.

Figure F15: WSD EIRP RESTRICTIONS:  Fixed WSD (10 m) to PO DTTB (1.5 m) 2nd Adjacent channel (PR = -40 dB), Oth = -20 dBm

Analytic approximation

We now compare this result (Pt_max = 32.48 dBm) with the results of equation E5´ of Section A5.5.2:

Pt < Oth – µxwsd + POL + DISCTV + DISCWSD – Ga + LOSS(d),			(E5´)

using the appropriate parameters.

Based on our determination of the value of the parameter “x” for LP = 0.1%, we can evaluate equation E5´ as follows: Pt_max = -20 – 3.09*3.5 + 0 + 10 – 2.15 + 55.45 = 32.485 dBm. This is precisely the result delivered by the previous Monte Carlo simulation.

CONCLUSION

We conclude that the arguments given above for the applicability of Equation E5´, and its accuracy, are valid. In particular, x = 99.9% for LP = 0.1%. This allows a simplification in calculations determining permitted maximum eirps for WSDs. In other words, when determining absolute maximum WSD eirps related to DTT receiver overload, Monte Carlo simulation is not required – the analytical inequality in equation E5´ suffices.

WSD EIRP LIMITS DUE TO DTT RECEIVER OVERLOAD

Protection against DTT overloading provides an absolute upper limit to the WSD transmit power. This upper limit can be calculated simply, with no approximation (as demonstrated in sections A5.5.2, A5.5.3 above), using the following analytical expression:

Pt < Oth – µx%wsd + POL + DISCTV + DISCWSD – Ga + LOSS(d)		(E5´)

where

- Oth is the relevant overload threshold, dBm

- POL is the polarization discrimination, dB,

- Ga is the DTT receive antenna gain, dB,

- DISCTV is the DTT receive antenna discrimination, dB,

- DISCWSD is the WSD transmit antenna attenuation, dB,

- LOSS(d) = 32.45 + 20log fMHz + 20log dkm is the free space propagation loss over the distance d between the WSD transmit antenna and the DTT receive antenna, dB;

- wsd is the propagation loss standard deviation,

- µx*wsd is the statistical factor ensuring protection for X% location probability.

For example, the protection of fixed DTT reception against fixed WSD transmission, we can use equation E5´ to determine a range of limiting WSD eirp values as a function of Oth. This is shown in Table T2.

Note that these limits would be valid irrespective of ambient DTT wanted field strength level.

A range of limiting WSD e.i.r.p. values is given in Table 10 as a function of Oth. Note that these limits would be valid irrespective of ambient DTT field strength level.

Table 10: FIXED WSD Pt limits with respect to Oth levels

| Oth | LP = 0.1% (µL = 3.090) |
| --- | --- |
| 0 dBm | 38 dBm |
| -5 dBm | 33 dBm |
| -10 dBm | 28 dBm |
| -15 dBm | 23 dBm |
| -20 dBm | 18 dBm |
| -25 dBm | 13 dBm |
| -30 dBm | 8 dBm |
| -35 dBm | 3 dBm |
| -40 dBm | -2 dBm |
| -45 dBm | -7 dBm |
| -50 dBm | -12 dBm |

In the next two subsections we show the range of WSD eirp limits as a function of frequency offset. For a given frequency offset, in order to protect all 10th percentile DTT receivers, the lowest WSD Pt limit for all 10th percentile receiver types must be observed. In addition, in the case of a WSD with multiple DTT channel adjacencies, the minimum of the individual Pt limits must be observed.

The theoretical basis for these results was provided in the section A5.5.2 and the demonstration of their precision was provided in the section A5.5.3, using Monte Carlo simulations.

Fixed WSD limits

Table 11 is taken from ECC Report 159 for values of the overload threshold as a function of the interfering channel adjacency. These values of the overload threshold lead to fixed WSD e.i.r.p. limits as shown in Figure 36. The dependency of the WSD Pt limits on the channel adjacency (horizontal axis) is to be noted

Table 11: DVB-T Oth values in the presence of a time-constant LTE BS interfering signal in a Gaussian channel environment at the 10th percentile: can-tuners and silicon-tuners

| DVB-T Oth for 64-QAM 2/3 DVB-T signal (LTE BS, Constant Average Power) Gaussian channel |  |  |  |
| --- | --- | --- | --- |
| Channel edge separation (MHz) | Oth (dBm) |  |  |
|  | 10th percentile |  |  |
|  | Can STB/iDTV | Silicon STB/iDTV | Silicon USB |
| 1 | -12 | -13 | -25 |
| 9 | -8 | -7 | -22 |
| 17 | -9 | -6 | -18 |
| 25 | -10 | 3 | -14 |
| 33 | -7 | 3 | -14 |
| 41 | -7 | 2 | -14 |
| 49 | -6 | 1 | -14 |
| 57 | -7 | 0 | -13 |
| 65 | -3 | -5 | -17 |

In the 2nd adjacent channel, Oth = -8 dB for fixed DTT reception (CAN and Silicon), and Oth = -22 dB for mobile (PO and PI) USB DTT reception. These values of overload threshold correspond to maximum WSD eirp levels 29.8 dBm (F), 30.5 dBm (PO), and 29.2 dBm (PI).

There are three horizontal dashed lines corresponding to eirp = 29.8 dBm, 30.5 dBm, and 29.2 dBm indicated in Figure 38.

Note that although Figure 38 relates to an overload threshold, Oth = -8 dBm (-7 dBm is also possible) (fixed DTT) and Oth = -22 dBm (portable DTT), it can be also used for other values of Oth.

Figure 36: Fixed WSD eirp limited by Fixed (can and silicon tuners) and PO (USB) DTT receiver overload according to the frequency offset

It is seen from Figure 65 that fixed WSD eirps might be restricted to range from  25 dBm (1st adjacent channel) to  29 dBm (2nd adjacent channel) to  28 dBm (4th adjacent channel) to  34 dBm (9th adjacent channel).

As a specific example, in the 2nd adjacent channel, Oth = -8 dB for fixed DTT reception (CAN and Silicon), and Oth = -22 dB for mobile (PO and PI) USB DTT reception. These values of overload threshold correspond to maximum WSD eirp levels 29.8 dBm (F), 30.5 dBm (PO), and 29.2 dBm (PI). These values are used as WSD eirp cut-off limits in Figure F17 below.

UE WSD limits

Table 23 bellow is taken from ECC Report 159 for values of the overload threshold corresponding to UE WSD as a function of the interfering channel adjacency.

These values of the overload threshold lead to fixed WSD eirp limits as shown in Figure F17. The dependency of the WSD Pt limits on the channel adjacency (horizontal axis) is to be noted.

It is important to further note that the values in Table T4 are valid only for UE with TPC off. With TPC on, the values may be as much as 10 dB lower.

Table 12: DVB-T Oth values in the presence of a LTE UE interfering signal without TPC in a Gaussian channel environment at the 10th percentile: can-tuners and silicon-tuners

| DVB-T PR for 64-QAM 2/3 DVB-T signal (LTE UE TPC off) |  |  |  |
| --- | --- | --- | --- |
| Channel edge separation (MHz) | Oth (dBm) |  |  |
|  | 10th percentile |  |  |
|  | Can STB/iDTV | Silicon STB/iDTV | Silicon USB |
| 1.5 | -21 … -19 | -23 … -17 | -27 |
| 9.5 | -18 … -4 | -46 … -5 | -47 |
| 17.5 | -31 … -26 | -47 … -2 | -49 |
| 25.5 | -19 … -11 | -44 … -6 | -42 |
| 33.5 | -17 … -7 | -43 … -5 | -41 |
| 41.5 | -18 … -7 | -41 … -7 | -37 |
| 49.5 | -16 … -3 | -39 ... -5 | -37 |
| 57.5 | -16 … -3 | -35 … -7 | -39 |
| 65.5 | -9 … -3 | -32 … -10 | -40 |

Figure 37: UE WSD e.i.r.p. limited by Fixed (can and silicon tuners) & PO (silicon tuner) DTT receiver overload according to the frequency offset

It is seen from Figure F17 that UE WSD eirps might be restricted to range from  -25 dBm (1st  adjacent channel) to  -27 dBm (2nd adjacent channel) to  -15 dBm (5th and 6th adjacent channel) to  -20 dBm (9th adjacent channel).

As a specific example, in the 2nd adjacent channel, Oth = -31 dB for fixed DTT reception (CAN) , Oth = -47 dB for fixed DTT reception (SIL), and Oth = -49 dB for PO and PI mobile (USB) DTT reception. These values of overload threshold correspond to maximum WSD eirp levels 3.8 dBm (F-CAN), -12.2 dBm (F-SIL), -27.2 dBm (PO-USB), and -28.8 dBm (PI-USB). These values are used as WSD eirp cut-off limits in Figure F22 below.

Once again, it is important to note that the values in Figure F17 are valid only for UE with TPC off. With TPC on, the limiting WSD eirp values may be as much as 10 dB lower.

WSD EIRP LIMITS DUE TO DTT PROTECTON RATIO

This section deals with WSD eirp limits calculated on the basis of protection ratios. Once again we set the protection criterion to be an allowed 0.1% degradation to the location probability, LP. The reference location probability (LP) is that referred to noise only.

The ‘protection ratio case’ differs from the ‘overloading case’ because, as the wanted signal increases, the interfering signal can also be increased while maintaining the required protection ratio. In addition, the magnitude of the maximum WSD eirp also varies directly with the magnitude of the protection ratio. Because of this direct (linear) dependence, we consider ‘relative’ WSD eirp limits. This means that the results will be ‘scaled’ to a protection ratio, PR = 0 dB. Then, with a simple ‘translation’, the relevant maximum WSD eirp for the actual protection ratio can be obtained by a simple addition.

Fixed WSD interference

Figure F18 provides a global indication of, X, the relative maximum fixed WSD eirp. The curves were calculated to provide a LP = 0.1% degradation to the location probability as the protection criterion.

The curves have been calculated for three reference cases refer to fixed WSD at 10 m agl interfering with

- fixed DTT reception at 10 m, with a 20 m lateral separation

- portable outdoor DTT reception at 1.5 m, with 20 m lateral separation

- portable indoor DTT reception at 1.5 m, with 20 m lateral separation

Note that the three curves have different ‘starting points’:

- The ‘fixed DTT’ curve starts at 56.21 dBµV/m at 10 m agl, which is the point at which fixed DTT reception with 95% location probability is assured,

- The ‘PO DTT’ curve starts at 78.21 dBµV/m at 10 m agl, corresponding to 61.21 dBµV/m at 1.5 m agl, which is the point at which PO DTT reception with 95% location probability is assured,

- The ‘PI DTT’ curve starts at 87.95 dBµV/m at 10 m agl, corresponding to 70.95 dBµV/m at 1.5 m agl, which is the point at which PI DTT reception with 95% location probability is assured.

These curves have been calculated using Monte Carlo simulations. A reference protection ratio of PR = 0 dB was chosen so that the curves can be easily adjusted to correspond to any value of protection ratio.

Thus, the curves can be used in the following manner. The reference WSD eirp “X” value,

eirpPR=0 = X,

can be read for any given median wanted field strength. Then the actual WSD eirp for a protection PR = Y dB, eirpPR=Y, can be calculated as

eirpPR=Y = eirpPR=0 – Y.

As a first example, consider the wanted field strength, 66.21 dBµV/m, at 10 m agl. For fixed DTT reception, we see from Figure F18 that X = -32 dBm, so eirpPR=0 = -32 dBm. If we are actually dealing with 2nd adjacent channel WSD interference, with a protection ratio, PR = Y = -40 dB, then the maximum WSD eirp is

eirpPR=-40 = eirpPR=0 – Y = -32 – (-40) = 8 dB.

As a second example, consider the wanted field strength, 106.21 dBµV/m, at 10 m agl. For portable indoor DTT reception, this corresponds to 89.21dBµV/m at 10 m (outside), we see from Figure F18 that X = -10.5 dBm, so eirpPR=0 = -10.5 dBm. If we are actually dealing with 1st adjacent channel WSD interference, with a protection ratio, PR = Y = -30 dB, then the maximum WSD eirp is

eirpPR=-30 = eirpPR=0 – Y = -10.5 – (-30) = 19.5 dB.

Figure F18: Maximum fixed WSD eirp (relative to PR = 0 dB) vs. wanted field strength at 10 m agl

UE WSD interference

Figure F19 provides a global indication of, X, the relative maximum UE WSD eirp. The curves were calculated to provide a LP = 0.1% degradation to the location probability as the protection criterion.

The curves have been calculated for three reference cases refer to UE WSD at 1.5 m agl interfering with

- fixed DTT reception at 10 m, with a 20 m lateral separation

- portable outdoor DTT reception at 1.5 m, with 2 m lateral separation

- portable indoor DTT reception at 1.5 m, with 2 m lateral separation.

The curves in Figure F19 are to be interpreted as were those in Figure F18, except they refer to UE WSD interferers instead of fixed WSD interferers.

Figure F19: maximum UE WSD eirp (relative to PR = 0 dB) vs. wanted field strength at 10 m agl

OVERALL RESULTS: COMBINED PR AND Oth WSD EIRP LIMITS FOR WSD INTERFERENCE

Combined PR and Oth WSD eirp limits for Fixed WSD interference

The parameters for the fixed WSD e.i.r.p. study are given in the section A5.2. The reference geometries are giving in Annex 2.

It has been shown in section A5.5.4 that absolute eirp limits on WSD transmitters are imposed by DTT overload threshold values. Therefore the curves in Figures F18 and F19 will have to be ‘capped’ when these absolute eirp limits are reached

Figure 38 shows the results for the case of 2nd adjacent channel WSD interference with the protection ratio, PR = -40 dB. The protection criterion is the 0.1% degradation of the location probability: LP = 0.1%.

Note that for a different protection ratio, say PR = -50 dB, the three solid curves would be translated upward by (-40 – (-50) =) 10 dB. Nevertheless, the absolute maximum WSD eirp would still be capped at around 29 dBm, due to the (protection ratio independent) overload threshold.

Figure 38: Maximum Fixed WSD eirp limits for 2nd adjacent channel WSD interference

The horizontal axis of Figure 38 corresponds to the wanted field strength level at a 10 m DTT receive antenna height. It runs from 56.21 dBµV/m (the median field strength at the DTT coverage edge for fixed reception) to 125.21 dBµV/m (which corresponds to the field strength levels very near to the DTT transmitter).

The vertical axis of Figure 38 corresponds to the maximum fixed (at 10 m antenna height) WSD eirp. in order that the LP = 0.1% limit is not exceeded (at 10 m for fixed, and at 1.5 m for portable outdoor/indoor).

Combined PR and Oth WSD eirp limits for UE WSD interference

The parameters for the UE WSD eirp study are given in the section A5.7. The reference geometries are giving in Annex 2.

It has been shown in section A5.5.4 that absolute eirp limits on WSD transmitters are imposed by DTT overload threshold values. Therefore the curves in Figures F18 and F19 will have to be ‘capped’ when these absolute eirp limits are reached

Figure 39 shows the results for the case of 2nd adjacent channel WSD UE interference with the protection ratio, PR = -40 dB. The protection criterion is the 0.1% degradation of the location probability: LP = 0.1%.

It is seen that the maximum UE eirp is severely restricted due to the overload threshold (horizontal dashed lines: -12.2 dBm for fixed silicon receivers and -28.5 dBm for USB receivers; see Figure 66, second adjacent channel). As mentioned before, these restrictions may become 10 dB more severe if the DTT overload threshold values for UE TPC ‘on’ are used.

Figure 39: eirp limits for UE WSD for LP = 0.1% (PR = -40 dB)

CONCLUSIONS ON THE USE OF A FIXED LP = 0.1% DTT PROTECTION CRITERION

A very stringent approach to protecting DTT against WSD interference would be to set a fixed limit of I/N, as is required for other primary services (e.g. I/N = -20 dB, or I/N = -10 dB, or I/N = -6 dB).

The wanted field strength increases as the DTT transmitter is approached. This could allow a relaxation of a fixed I/N limit. One way to relax the fixed I/N limit and still to maintain the required protection for DTT reception is to allow a fixed degradation to the location probability, LP, in any given area.

A moderate value, LP = 0.1%, is felt to be suitable. This means that the interfering field strength can also be increased while maintaining the protection criterion LP= 0.1%.

Maintaining LP= 0.1% throughout the entire DTT coverage area, however, will lead to values of I/N in excess of 40 dB or more, and DTT receiver overload, if adjacent channel WSDs are not subject to eirp limits. Larger permitted values of LP > 0.1% would only exacerbate this excess.

A fixed protection criterion of LP= 0.1% corresponds to a flexible limit to I/N, allowing I/N values to increase up to I/N  +10 dB, a flexibility which gives more opportunity for WSD operation.

It should be noted that the higher LP reception margins available for the DTT receivers located closer to the DTT transmitter are to a large extent consumed by the fact that the receiver installations have lower performances in these areas. Furthermore, DTT viewers make use of this margin to receive programs with portable outdoor and portable indoor receivers. It is not sensible to consider that this reception margin can be consumed to allow higher interfering levels, to the detriment of the DTT viewer.

The constant, fixed LP = 0.1% approach is flexible with respect to extended WSD usage nearer the DTT transmitter, in addition to preserving the DTT viewer’s flexibility in choosing his or her mode of viewing, fixed or portable, outdoor or indoor.

Use a variable acceptable degradation of the coverage probability

The use of TV white spaces is conditioned to the protection of the primary DTT broadcasting service against WSD interference. This protection is represented by the respect of tolerable levels of DTT location probability degradation (ΔLP), i.e. tolerable degradations of the DTT coverage quality.  Low values of ΔLP are essentially tolerable, but they may be too restrictive to the operation of white space systems in vacant channels. On the other hand, the increase of ΔLP may lead to good conditions for white space systems operation, but at the cost of causing harmful interference to the DTT broadcasting system.

An optimum degradation level is not expected to be found, since the decision about the ΔLP to be adopted is influenced by several aspects, like the target DTT coverage quality, the specific characteristics of different regions (urban, suburban, rural areas), etc. In spite of this, upper limits to ΔLP can be derived by considering the technical limitations of the DTT receivers in dealing with interference. The methodology presented in this annex provides ΔLP upper limits and can be used to guide Administrations in their decisions about the location probability degradation levels to be adopted in different situations and scenarios. Since in some countries the protection of portable outdoor and portable indoor DTT reception is required in addition to the protection of fixed outdoor DTT reception, the three reception modes are considered.

The signal modeling and the calculation of location probability are in accordance with ECC Report 159, where DTT wanted and WSD interfering signals are considered log-normal random variables with specific standard deviation, and the location probability is obtained by Monte Carlo simulations. All scenarios presented in Annex 2 of complementary report A2 are considered in the determination of WSD EIRP limits according to the type of WSD and the DTT reception mode to be protected. Aggregate or cumulative interference due to simultaneous operation of multiple WSDs is not considered. Then, the EIRP limits provided in this section should be reduced by appropriate multiple interference margins (Annex 1 of complementary report A2).

RELEVANT PARAMETERS FOR THE PROTECTION OF THE DTT RECEIVER AGAINST INTERFERENCE

WSD interference degrades two distinct aspects of DTT broadcasting service:

- The coverage quality of DTT service;

- The ability of the DTT receiver in discriminating the desired signal from interference signals.

From the perspective of the DTT receiver, two parameters are important to appropriate operation in the presence of interference:

- Protection ratio: the minimum value of the signal-to-interference ratio at the DTT receiver required to obtain a specified reception quality under specified conditions;

- Overloading threshold: the interference level above which the receiver begins to lose the ability to discriminate against interfering signals at frequencies differing from that of the wanted signal.

Conditions for the protection of the DTT receiver against interference taking into account the two aforementioned parameters provide interference limits for appropriate operation of the DTT receiver. The appropriate operation in this case means the respect of the protection ratio and the overloading threshold. Given these interference limits, it is possible to calculate the corresponding maximum permissible levels of ΔLP, and thus to assess the feasible levels of ΔLP, i.e. the levels of ΔLP for which the DTT receiver is able to handle interference.

The conditions based on the protection ratio and on the overloading threshold for the protection of the DTT receiver against interference are usual in ECC compatibility studies, as in ECC Report 138  (see Annex C) and ECC Report 148. They take into account statistical location variations in the wanted and interfering signals, and provide interference limits for the protection of the DTT receiver for a percentage of locations. For simplicity, interference of other DTT transmitters is ignored.

SATISFACTION OF PROTECTION RATIO

The usual protection condition of the DTT receiver against interference is related to the protection ratio. It is given by

Ewmed ≥ Eimed + PR(Δf) + µx% √(σ2w+ σ2i)

where:

- Ewmed: wanted DTT median field strength at the DTT receiver;

- σw: wanted DTT field standard deviation;

- Eimed: WSD interference median field strength at the DTT receiver;

- σi: WSD interference standard deviation;

- PR(Δf)PR(Δf): protection ratio for a Δf frequency offset (co-channel or adjacent channels);

- µx% √(σ2w+ σ2i)μX%√(σw2+σi2)μX%√(σw2+σi2): location correction factor for X% of locations within the small covered area (pixel).

Location correction factor takes into account the statistical location variations of both the wanted and the interfering signals.

Therefore, the limiting interference median field strength Eimed_maxEimed_maxEimed_max_PR for the satisfaction of the appropriate (co-channel or adjacent channels) protection ratio PR(Δf)PR(Δf) for X% of locations within the pixel is expressed as

Ewmed_max_PR = Ewmed - PR(Δf) - µx% √(σ2w+ σ2i)

for a given wanted median field strengthEwmedEwmed Ewmed and wanted and interfering standard deviations σw and σi.

Respect of the Overloading threshold

The protection of DTT receiver against overloading consists in satisfying:

IFE ≤ Oth - µx%×σi

where:

- IFE IFEIFE: interference power at the front-end of the DTT receiver;

- Oth OthOth: overloading threshold;

- µx%×σi μX%σiμX%σi: location correction factor for X% of locations within the small covered area (pixel).

Location correction factor in this case takes into account the statistical location variations of the interfering signal only, since DTT receiver overload does not depend on the wanted DTT signal power.

The interference power at the front-end of the DTT receiver relates to the interference power at the DTT receiver input I as follows:

IFE = I – POL + Ga

where POL and Ga represent antenna polarization discrimination and antenna gain (including feeder losses). The relation between interference I and interference median field strength Eimed Eimed Eimedis given by

I = Eimed – 20.log10(fMHz) – 77.2

for the frequency of operation fMHzfMHz given in MHz.

Therefore, the limiting interference median field strength at the DTT receiver, Eimed_max_Oth, for the protection of the DTT receiver against overload for X% of locations within the pixel OthOthis given by

Eimed_max_Oth = OthOthOth - µx%×σi + POL – Ga + 20.log10(fMHz) + 77.2

It is noticeable that the receiver overload does not depend on the protection ratio.

Limits for location probability degradation and wsd eirp

The protection of the DTT receiver against interference for X% of locations inside the pixel is guaranteed by the most stringent interference upper limit for the satisfaction of the PR and the respect of the Oth for X% of locations. Then, the limiting interference field strength at the DTT receiver, Eimed_max, is given by

Eimed_max = min(Eimed_max_PR, Eimed_max_Oth)

where Eimed_max_PR and Eimed_max_Oth are calculated as previously described.

The methodology proposed in the ECC Report 159 is thus used to obtain by Monte Carlo simulations both the original LP (without WSD interference) and the resulting LP with the limiting WSD interference Eimed_max. The corresponding ΔLP, i.e. the difference between the original and the resulting LP, is considered the ΔLP upper limit for the protection of the DTT receiver for X% of locations inside the pixel.

The WSD EIRP upper limit is finally calculated from Eimed_max (or from its corresponding interference power Imax=Eimed_max-20log10(fMHz)-77.2) according to the specific WSD transmitter / DTT receiver scenario (reference geometry, as in Annex 2 of complementary report A2). This leads to the following general relationship:

EIRPmax= Imax+LOSS+Aatt+Adisc

where EIRPmax is the WSD EIRP upper limit for the protection of the DTT receiver at the reference geometry for X% of locations inside the pixel, Imax is the interference limit at the DTT receiver, LOSS is the propagation loss between the WSD and the DTT in the reference geometry, Aatt is the WSD antenna attenuation, and Adisc is the DTT antenna discrimination.

Scenarios and relevant parameters

It is important to define upper limits for the location probability degradation and the WSD EIRP according to the type of WSD and the DTT reception mode to be protected. There are three possible DTT reception modes: fixed outdoor, portable outdoor and portable indoor DTT reception. The considered WSD types are the ones present in Scenarios 1 to 8, in Annex 2 of complementary report A2: portable WSD (Scenarios 1, 2, and 3), fixed WSD at 10 m agl. (Scenarios 4, 5, and 6), and fixed WSD at 30 m agl. (Scenarios 7 and 8). Simulation parameters presented in Annex A5.8 of complementary report A2 for each scenario are adopted. Then, for each WSD type, limits for the location probability degradation and the WSD EIRP are presented for each DTT reception mode.

Table 1 presents relevant parameters for the three DTT reception modes: the wanted DTT median field strength at the DTT receiver in the coverage edge (LP = 95%), Ewmed_ref, and reasonable values of protection ratio for co-channel, first, and second adjacent channels. Fixed outdoor DTT reception becomes possible with 56.21 dBμV/m wanted median field strength at 10 m agl. For portable outdoor DTT reception, it is needed a wanted median field strength at 1.5 m agl. of at least 61.21 dBμV/m, which corresponds to Ewmed = 78.21 dBμV/m at 10 m agl., with 17 dB height loss. For portable indoor DTT reception, 62.95 dBμV/m median field strength is needed at the DTT receiver (1.5 m agl.). In this case, the corresponding median field strength at 10 m agl. is 87.95 dBμV/m, where in addition to 17 dB height loss, 8 dB wall loss is considered.

Table 1: Wanted DTT median field strength at the coverage edge (LP = 95%) and protection ratio.

| DTT reception mode | Ewmed_ref [dBμV/m] | Co-channel protection ratio PR(0) [dB] | 1st adj. channel protection ratio PRΔf1 [dB] | 2nd adj. channel protection ratio PRΔf2 [dB] |
| --- | --- | --- | --- | --- |
| Fixed outdoor | 56.21 | 21 | -30 | -40 |
| Portable outdoor | 61.21 (78.21 at 10 m agl.) | 19 | -30 | -40 |
| Portable indoor | 62.95 (87.95 at 10 m agl.) | 17 | -30 | -40 |

The measurements of LTE interference into DVB-T receivers reported in ECC Report 148 and considered in ECC Report 159 are used to define the overloading thresholds according to the types of (LTE-like) WSD transmitter and DTT receiver, as shown in Table 2. Values obtained for the silicon USB antenna type are adopted as overloading thresholds for portable DTT receivers. For fixed DTT receivers, can and silicon set-top box and/or integrated TV antenna types are considered. Measurements of overloading threshold for portable WSD – fixed DTT receiver indicate a wide range of possible values, from which -20 dBm is adopted for both first and second adjacent channels.

Table 2: Overloading threshold.

|  | Fixed DTT |  | Portable DTT |  |
| --- | --- | --- | --- | --- |
|  | 1st adjacent channel overloading threshold OthΔf1 [dBm] | 2nd adjacent channel overloading threshold OthΔf2 [dBm] | 1st adjacent channel overloading threshold OthΔf1 [dBm] | 2nd adjacent channel overloading threshold OthΔf2 [dBm] |
| Fixed WSD | -13 | -8 | -26 | -22 |
| Portable WSD | -20 | -20 | -27 | -47 |

Protection ratio and overloading threshold have strong influence on the upper limits of permissible WSD interference at the DTT receiver, and consequently on the limits of location probability degradation and WSD EIRP. The methodology presented in this annex is applicable to any other values for these parameters, which can arise with the development of the actual WSDs.

Main results

The upper limits of ΔLP and WSD EIRP, and the resulting LP are presented below for the parameters given in Table 1 and Table 2. The reference geometries are given in Annex 2 or derived from it and A5.8 (case of portable indoor DTT reception scenarios).

The first set of results is shown in Figure 1 for fixed outdoor WSD at 10 m agl., where the protection of a DTT receiver at the reference geometry is guaranteed for X = 99.9% of locations inside a pixel (quasi perfect DTT receiver operation with respect to interference). For fixed outdoor DTT reception (blue curves), the WSD EIRP upper limits for the second adjacent channel vary from about -2.6 dBm at the coverage edge to 29.75 dBm at locations where Ewmed≥91.21 dBμV/m (wanted median field strength at least 40 dB higher than at the coverage edge), when the maximum permissible interference is limited by the overloading threshold. The limiting scenario in this case is Scenario 4 in Annex 2. The corresponding ΔLP upper limit decays from 0.91% at the coverage edge to about 0.1% in the region where Ewmed≥91.21 dBμV/m (ΔLP is almost negligible for Ewmed≥126.21 dBμV/m). The curve of DTT LP which results from the WSD interference indicates that already in locations where the wanted median field strength is only 5 dB above the one at the coverage edge, the resulting LP is around 99%.

If the protection of portable outdoor and portable indoor DTT reception modes is considered, red and black curves in Figure 1 indicate the upper limits of ΔLP and WSD EIRP, and the resulting LP. For portable outdoor DTT reception (red curves), the behavior of ΔLP upper limits and resulting LP is essentially the same as for fixed outdoor DTT reception, with similar values but for different coverage areas. The limiting scenario is Scenario 5 in Annex 2. As expected, the protection of this DTT reception mode imposes more restrictive upper limits for the EIRP of fixed outdoor WSD at 10 m agl. The protection of portable indoor DTT reception (Scenario 6 in Annex 2) for locations where Ewmed at 10 m agl. is at least 87.95 dBμV/m also imposes new restrictions in this region: the WSD EIRP upper limit is about 11.5 dBm at the coverage edge and increases until 29.2 dBm for Ewmed≥108 dBμV/m (wanted median field strength 20 dB higher than at the coverage edge). The corresponding ΔLP upper limits for portable indoor DTT reception are lower than for the other reception modes, going from 0.25% at the coverage edge to 0.1% with the increase of Ewmed.

Figure 1: Fixed outdoor WSD transmission at 10 m agl. – Upper limits for WSD EIRP at the 2nd adjacent channel and the ΔLP, and the resulting LP for the protection of a DTT receiver at the reference geometry for X = 99.9% of locations inside the pixel.

The EIRP upper limits for fixed WSD at 10 m agl. in the first adjacent channel are shown in Figure 2. Similar overall behavior, but more restrictive EIRP values are observed.

Figure 2: Fixed outdoor WSD transmission at 10 m agl. – Upper limits for WSD EIRP at the 1st adjacent channel for the protection of a DTT receiver at the reference geometry for X = 99.9% of locations inside the pixel.

The set of results for fixed outdoor WSD at 30 m agl. is shown in Figure 3. Once more it is considered the protection of a DTT receiver at the reference geometry for X = 99.9% of locations inside a pixel (quasi perfect DTT receiver operation with respect to interference). The ΔLP upper limits and the resulting LP are very similar to the ones obtained for fixed outdoor WSD at 10 m agl., but the WSD EIRP upper limits are higher because of the elevated coupling loss between WSD transmitter and DTT receiver. For the protection of fixed outdoor DTT reception (Scenario 7 in Annex 2), the WSD EIRP upper limits in the second adjacent channel increase from 22 dBm at the coverage edge (Ewmed=56.21 dBμV/m) to 51.45 dBm for locations where Ewmed≥86.21 dBμV/m. More restricted upper limits are obtained if portable DTT reception is taken into account. In the case of portable outdoor DTT reception (Scenario 8 in Annex 2), at locations where Ewmed=78.21 dBμV/m at 10 m agl., WSD EIRP is limited to 26.3 dBm, and increases with Ewmed until Ewmed=98.21 dBμV/m, when it is limited by the overloading threshold to 43.7 dBm. Similar behavior is observed if the protection of portable indoor DTT reception is considered, with WSD EIRP limited to 24.81 dBm at the coverage edge (Ewmed=87.95 dBμV/m at 10 m agl.), and 42.4 dBm for Ewmed≥108 dBμV/m.

From the results presented above, EIRP “masks” can be derived for fixed outdoor WSD. Figures 4 and 5 show the EIRP masks for the second adjacent channel for WSDs at 10 m and 30 m agl., respectively. If only fixed outdoor DTT reception is to be protected, the EIRP “masks” are given by the blue curves only. Depending on the values of relevant parameters (protection ratios, overloading thresholds, level of protection for the DTT receiver) the EIRP upper limits can change.

Figure 3: Fixed outdoor WSD transmission at 30 m agl. – Upper limits for WSD EIRP at the 2nd adjacent channel and the ΔLP, and the resulting LP for the protection of a DTT receiver at the reference geometry for X = 99.9% of locations inside the pixel.

Figure 4: EIRP “mask” for fixed outdoor WSD transmission at 10 m agl. (2nd adjacent channel).

Figure 5: EIRP “mask” for fixed outdoor WSD transmission at 30 m agl. (2nd adjacent channel).

Now the results for portable WSD are shown in Figure 6, where the upper limits of ΔLP and WSD EIRP (second adjacent channel), and the resulting LP are calculated for the protection of the DTT receiver for X = 99.9% of locations inside the pixel. The considered overloading thresholds are given in Table 2.

To protect the fixed outdoor DTT reception (Scenario 2 in Annex 2, blue curves in Figure 6), the portable WSD EIRP is limited to -2.6 dBm at the coverage edge (Ewmed=56.21 dBμV/m) and increases linearly with Ewmed until Ewmed=76.21 dBμV/m, from where the WSD EIRP is limited by the overloading threshold to 14.75 dBm. The corresponding ΔLP upper limits vary from about 0.91% to 0.1% when entering the DTT coverage area, and achieve negligible values for Ewmed≥126.21 dBμV/m. As in the case of fixed outdoor WSD, the LP which results with portable WSD interference is above 99% for locations where Ewmed is 5 dB above Ewmed at the coverage edge.

Because of the very low overloading threshold of portable DTT receiver with respect to portable WSD interference, -47 dBm, as indicated in Table 2 (from ECC Report 148), the protection of portable DTT reception restricts the portable WSD EIRP to a very low value, -25.2 dBm, for any location inside portable DTT coverage area. The relaxation of the protection criterion from X = 99.9% of locations to X = 99% or X = 90% of locations where the DTT receiver at the reference geometry (2 m distant from the WSD) is protected leads to still low and constant WSD EIRP limits like -22.5 dBm and -18.9 dBm, respectively.

To illustrate the strong influence of overloading threshold values on the WSD EIRP upper limits, Figure 7 shows the EIRP upper limits for the operation of portable WSD in the first adjacent channel, whose overloading threshold is -27 dBm (from ECC Report 148). It is thus observed that the behaviour of WSD EIRP upper limits in this case is similar to the fixed WSD case. Therefore, according to the restrictions imposed by the overloading threshold, an EIRP “mask” for portable WSD transmission would follow:

1) the shape of WSD EIRP curves in Figure 6, with a constant EIRP value in the region of portable DTT coverage; or

2) the shape shown in Figure 7.

Figure 6: Portable WSD transmission – Upper limits for WSD EIRP at the 2nd adjacent channel and the ΔLP, and the resulting LP for the protection of a DTT receiver at the reference geometry for X = 99.9% of locations inside the pixel.

Figure 7: Portable WSD transmission – Upper limits for WSD EIRP at the 1st adjacent channel for the protection of a DTT receiver at the reference geometry for X = 99.9% of locations inside the pixel.

CONSIDERATIONS when choosing the degradation of the location probability

The two approaches presented in this Annex provide location specific maximum EIRP limits for different types of WSDs and DTT reception modes. This content serves as guidance for Administrations, since it discusses first the adoption of a fixed level of degradation of the DTT coverage quality as protection criterion (A5.2 – A5.9), and then the adoption of variable levels of degradation of the DTT coverage quality according to the capabilities of the DTT receivers in handling interference (A5.10 – A5.16).

When considering a particular value for the degradation in location probability, it should be remembered that, this degradation will result in a reduction in coverage and a loss in the number of viewers who can receive the broadcast service -  the larger the degradation, the larger the loss.

For each broadcasting service a required minimum location probability is defined that has to be reached in each pixel at the edge of the coverage. Typical values for these required minimum location probabilities at the edge are 99% (mobile reception), 95% (portable reception) or 95% (fixed reception). Location probability for pixels inside the coverage area is generally higher than those at the edge, and this represents a proportionally larger number of households that have acceptable DTT reception within those pixels.

The number of households losing DTT reception in a pixel can be estimated by multiplying the degradation in probability by the number of households per pixel. The total impact to the broadcast service can then be assessed by considering the summation of this quantity for all populated pixels in the DTT network.

[To be deleted: moved to Annex 3)

PMSE Reference geometries

Overview

Scenario I - Outdoor Events

Figure 40: Outdoor PMSE operation at a street fair, foot race, or similar event

Scenario II - Indoor Halls

Figure 41: Indoor PMSE operation in a concert hall or theatre

Scenario III - Indoor Meeting Halls

Figure 42: Indoor PMSE operation in a meeting room or conference center

applications EXAMPLE(s) of master/slave concept

M2M network deployment

The following example is provided to illustrate how a master/slave approach might work in practice. It is based on the deployment of a nationwide machine-to-machine network where there are many thousands of base stations across a country and potentially tens of thousands of machine terminals in each base station. This is the model being considered, for example, within the Weightless standard.

Each base station in such a network becomes a master device. Coverage maps for each base station at its preferred power level, antenna height, etc, are pre-generated using commercial coverage planning tools. Such base stations might have a preferred transmit power level of around 30dBm e.i.r.p. where regulation allows. Within the core network is a frequency assignment entity that is responsible for obtaining from the geo-location database a list of allowed frequencies at each base station and assigning them so that inter-base station interference is avoided as far as possible. This entity sends to the geo-location database an enquiry for each base station. Such an enquiry consists of one interrogation for the base station and multiple (often many thousand) interrogations for each pixel within the coverage area. These pixel interrogations would be for terminals that would be at a lower height and typically have a much lower transmit power than the base station (eg 16dBm e.i.r.p.)

The results for all base stations across the network are aggregated and a frequency plan assembled that uses the best available channels and minimizes interference. This process is repeated whenever a channel validity time expires.

Improvements could be envisaged where the geo-location database is informed of the selection of frequencies and monitors changes to licensed usage that will impact upon these. The network is then informed when such changes occur and hence only starts a re-planning process when necessary. However, this functionality would be outside of that mandated by the regulator and may be a value-added service provided by the database operator.

Trade-off between ‘false-vacancy-detection’ and ‘false-occupancy-detection’ as a function of increasing detection thresholds

General

The purpose of WSD sensing and of the geo-location Data Base (DB) is to ensure that incumbent services, including DTT transmissions, are not interfered with by WSD usage. To this end it may be necessary to be somewhat ‘overprotective’ because of the inherent limitations of the WSD sensor and of the DB information and calculation abilities.

DB determinations of the presence or absence of DTT signals in a given area can be based on ‘internal’ calculations or on additional ‘real time’ information obtained from external sources, for example based on the ‘actual’ (measured) field strength values prevailing within the given area.

If complete ‘real time’ knowledge of precise ‘actual’ DTT field strength values were available everywhere, then DB calculations for the wanted DTT field strength values would not be necessary. Neither would WSDs need to be equipped with a sensing capability.

However, a complete knowledge of the actual DTT coverage situation will probably not be available, so that reliance on DB calculations as well as on WSD sensory capabilities must be taken on board to ensure sufficient protection for DTT reception.

Because both DB facilities and WSD sensing are ‘imperfect’, there is the possibility, using either (or both) technique(s), of arriving at

- a ‘false-vacancy-detection’, i.e. the indication that the DTT channel is not being used when in fact it is occupied, or

- a ‘false-occupancy-detection’, i.e. the indication that the DTT channel is occupied when in fact it is not being used.

We are more concerned with preventing ‘false-vacancy-detections’ than with preventing ‘false-occupancy-detections’ because the purpose of DB and WSD sensors is to avoid interference to DTT reception, and ‘false-vacancy-detections’ will lead to DTT interference, whereas ‘false-occupancy-detections’ will not lead to DTT interference. Of course, ‘false-occupancy-detections’ should be avoided to the extent possible, but not at the cost of increasing the likelihood of ‘false-vacancy-detections’.

WSD sensing

WSD sensing is oriented towards the ‘actual’ values of the ambient field strength. That is, a DTT field strength is measured/detected (or not) but not calculated. The field strength value which the WSD measures/detects is that which is present at the WSD site and it is usually not that which is actually present at nearby DTT reception sites. For example, a WSD may measure/detect a DTT field at street level (say 1.5 m) where the field may be very weak, whereas the DTT field of interest is much stronger, being received at rooftop height (say 10 m) 20 or 30 m away. Because of this ‘disconnect’, the WSD field strength sensor must be very sensitive, that is it must be able to detect field strengths at levels much lower than would be usable by a DTT receiver. Also, because of this disconnect, the WSD sensing is not really reliable with respect to predicting what the actual DTT signal strength is that may be receivable nearby. In other words, a WSD might not detect an ambient DTT signal, indicating (incorrectly) that either there is no DTT signal present or else that the field at the nearby DTT reception site would be too low for satisfactory reception. The WSD would register a ‘false-vacancy-detection’, although the DB with complete knowledge would register a ‘true occupancy’. To reduce this possibility of ‘false-vacancy-detections’ the WSD sensing threshold level must be very low. Of course, the lowering of the sensing threshold may increase the likelihood of ‘false-occupancy-detections’.

DB calculations

DB calculations/determinations of the presence (or not) of a wanted DTT signal have yet to be defined. Such calculations/determinations could consist of some of the following ingredients.

Input:

- DTT transmitter characteristics (site, transmit antenna site and antenna pattern, e.r.p....);

- DTT Rx characteristics (site/area, Ewmed_ref, ...).

- DTT coverage contours can be calculated using this information, or else;

- DTT coverage contours can be specified/defined by external sources.

- In addition, the WSD location would need to be known.

One potential drawback of calculating or defining the DTT coverage contour is that often it is possible to have a satisfactory DTT reception ‘beyond’ the calculated coverage contour. In other words, the ‘true’ coverage ‘edge’ may be ‘outside’ the calculated/defined coverage contour. Either this is taken into account when the ‘reference’ coverage contour is calculated/defined or else areas of actual DTT reception may be ‘overlooked’ by the DB; this type of occurrence would constitute a possibility of concluding a ‘false-vacancy-detection’ for the presence of a DTT signal being reported by the DB. Of course, the opposite effect may also arise sometimes, where a ‘false-occupancy-detection’ would be indicated.

DB errors

DB calculated field strength values may sometimes be erroneous. For example, calculated wanted field strength values may be ‘too low’ (e.g. near the DTT coverage edge) compared to the actual values which, though low, may still be sufficiently high to be receivable, and thus the DB would register a ‘false-vacancy-detection’.

It may also happen that the DB calculated field strength values are ‘too high’ compared to the actual values which may not be high enough to provide satisfactory reception quality, and thus the DB would register a ‘false-occupancy-detection’.

Both these cases (‘false-vacancy-detection’ and ‘false-occupancy-detection’) would be most likely to occur where the DTT reception is near the coverage ‘edge’, i.e. where the wanted DTT signal strength is ‘marginal’. For example, in an area where the wanted field strength (measured or calculated) is very high, an error in the calculations of some dB, either way, will not change the conclusion that a DTT signal is present, i.e. it will usually not lead to a ‘false-vacancy-detection’ decision.

WSD errors

On the other hand, even when the DB calculates the wanted field strength values incorrectly, and indicates a ‘false-vacancy-detection’, the WSD might detect an ‘actual’ field strength level at or above the threshold which, taking into account the difference in respective receiving heights (WSD at 1.5 m vs. DTT at 10 m) and other possible positional and topological/structural effects, would indicate that a receivable DTT signal could be available nearby.

To ensure that the WSD sensor is sufficiently reliable, a 99.99% reliability factor should be built into the sensor in terms of a low sensitivity threshold.

The 99.99% reliability factor ensures that even those channels providing nearby DTT receivers with a ‘weak’ field strength, around Emin (i.e. near the coverage ‘edge’), are marked as being ‘occupied’ with a very high probability. Because of the low value of the threshold, areas which are well within the DTT coverage area will be less prone to false WSD evaluations (‘false-occupancy-detection’ and ‘false-vacancy-detection’). That is, once again, the possibility of a WSD providing a ‘false-vacancy-detection’ or a ‘false-occupancy-detection’ evaluation would most likely occur where the DTT reception is near the coverage ‘edge’.

WSD detection errors when DTT signal is present

WSD sensitivity threshold TH0.

Assuming that a DTT signal is present at various levels of field strength, Error! Reference source not found. (1a) indicates the number of detections that are made by a sensor having a threshold level TH0. Below TH0, signals may be present, but they are not detected because the threshold TH0 is not low enough: i.e. there may be some ‘false-vacancy-detections’.

- The events between TH0 and TH1 will not be detected if the threshold is raised from TH0 to TH1 And thus they constitute ‘false-vacancy-detections.

The events between TH1 and TH2 as well as those between TH0 and TH1 will not be detected if the threshold is raised from TH1 to TH2. That is, additional ‘false-vacancy-detections’ will be registered.

Error! Reference source not found. (1b) shows how the probability of ‘false-vacancy-detections’ will increase as the threshold is increased

Field strength level

# of events detected

Probability of false-vacancy-detections

TH0

TH1

TH2

Threshold level

Figure 43: (1a) As the threshold is increased, ambient DTT fields at low levels may, erroneously, miss being detected

Figure70: (1b) Increased probability of ‘false-vacancy-detections’ as the threshold increases

Conversely, the probability of ‘false-occupancy-detections’ would decrease with increasing WSD sensing threshold (e.g. if the threshold were very, very high, there would be no ’occupancy’ at all, ‘true’ or ‘false’).

WSD DETECTION WHEN DTT signal is not present

If no DTT signal is present (or it is too weak to be usable), there is nevertheless a probability that a signal is erroneously indicated by the WSD as being present – a ‘false-occupancy-detection’: green curve in Figure 44. The lower the WSD threshold level, the more probable that a ‘false-occupancy-detection’ will be indicated (the green curve increases with lowering sensing threshold). If the WSD threshold level is high the probability of ‘false-occupancy-detections’ becomes smaller: In Figure 44, TH2 > TH1, and P2 < P1.

But as seen from Error! Reference source not found., the ‘false-vacancy-detections’ will increase with increasing WSD threshold level, and it is most important that the ‘false-vacancy-detections be kept to a minimum, in order to keep interference to DTT reception to a minimum.

TH1

TH2

N

TH

% ‘false-occupancy-detections’

P1

P2

Figure 44: % ‘false-occupancy-detections’ when a DTT field is not present as a function of WSD sensor threshold

Demonstration with a practical case

We will base our discussion in part on the Figure 40 of Annex 3 (“Example spectrum sensor implementation and field test results”) of the complementary Report) in particular on the ‘measured’ and ‘simulated’ results (see a copy of “Figure 45” below).

Figure 45: Measured DVB-T probability of detection compared to simulated performance. Simulation utilizes ideal receiver (NF=0dB)

For the sensor being simulated/measured, it is apparent that the probability of detecting an ambient signal increases as the ambient signal becomes stronger. This implies, of course, that there is a probability of not detecting a signal which is present at a frequency in the measurement range. For the sake of simplicity, we propose the corresponding linear models in Error! Reference source not found. to approximate Figure 45. The two approximations are parallel with a 6 dB shift. The corresponding equations for the probability of (correct) detection (“PROB”) are given as Eqn. 1M (‘measured’).

-140

-130

-120

-110

100

0

Probability of Detection (%)

x

L

Probability of Detection: signal present

x = received power

Eqn. 1M

Figure 46: Linear approximation to Figure 45

False-vacancy-detections

Those equations can be modified to equations for the probability, Pfn = 1 – PROB, of incorrect ‘non-detection’, i.e. a false-vacancy-detection. These are depicted in Figure 44 with the corresponding equations  Eqn. 2M. (We use the notation “Pfn” for the probability of a ‘false-vacancy-detection’, and “Pfp” for the probability of a ‘false-occupancy-detection’.)

-140

-130

-120

-110

100

0

Probability of False-vacancy-detectionsw (%)

x

L

Probability of False-vacancy-detections

x = received power

Eqn. 2M

Figure 47: Linear approximation to Probability of False-vacancy-detections

WSD SENSITIVITY MODEL

It should be noted that the real sensitivity of the implemented detector is always a fixed value, in this example -117 dBm. The decision threshold in the implemented detector is by default fixed at -127 dBm, which is the level where the detector starts to ‘wake up’.

Therefore, our model will be based on indents 1 and 2 of the Introduction, explicitly:

- The correct distinction between ‘sensitivity’ and ‘detection threshold’ is made,

- Different sensitivities can be obtained by a horizontal translation, the curves remain parallel,

- The chosen sensitivity and the corresponding decision threshold differ by 10 dB; e.g. in Figure 1,

  - simulated sensitivity is -123 dBm, and decision threshold is -133 dBm

  - measured sensitivity is -117 dBm, and decision threshold is -127 dBm.

On this basis, we generalize the Figure 3 to encompass 5 WSD sensors with the following

- sensitivities: 		-140 dBm, -130 dBm, -120 dBm, -110 dBm, -100 dBm,

- decision thresholds:	-150 dBm, -140 dBm, -130 dBm, -120 dBm, -110 dBm.

The results are shown in Figure 4x.

Figure 4x

The equations describing the ‘curves’ are the following (PDTT is the wanted DTT signal power, PROBFV is the probability of ‘false vacancy’):

| WSD sensitivity | PROBFV = 95% | 0% < PROBFV < 95% | PROBFV = 0% |
| --- | --- | --- | --- |
| -140 dBm | PDTT ≤ -150 dBm | PROBFV = -9.5 PDTT – 1330 dBm | -140dBm ≤ PDTT |
| -130 dBm | PDTT ≤ -140 dBm | PROBFV = -9.5 PDTT – 1235 dBm | -130dBm ≤ PDTT |
| -120 dBm | PDTT ≤ -130 dBm | PROBFV = -9.5 PDTT – 1140 dBm | -120dBm ≤ PDTT |
| -110 dBm | PDTT ≤ -120 dBm | PROBFV = -9.5 PDTT – 1045 dBm | -110dBm ≤ PDTT |
| -100 dBm | PDTT ≤ -110 dBm | PROBFV = -9.5 PDTT – 950 dBm | -100dBm ≤ PDTT |
| Table 1x: Model parameters |  |  |  |

PROBABILITY OF ‘FALSE VACANCY’ AS A FUNCTION OF WSD SENSITIVITY

In Figure 78 we see how the probability of detection of an existing DTT signal varies with the DTT signal strength.

In the present section, the overall ‘efficiency’ of WSD detecting some existing signal is examined. It will be seen that this ‘efficiency’ decreases as the WSD sensitivity is reduced.

We use the parameters of the model given in the preceding section.

We derive the behaviour of the probability of ‘false vacancy’ as a function of WSD sensitivity as follows:

- We assume there is a uniform, random distribution of DTT signals with powers, PDTT, between -140 dBm and -100 dBm. The strength of any given signal is not known. The probability of detecting them is to be calculated.

- The overall probability for detecting all of those signals using the WSDs with the sensitivity given for each sensitivity level given in the model is calculated.

- Those individual overall probabilities can be calculated by integrating the probability functions assumed in the model; the relevant probability functions are those shown in Figure 4x, and which are described mathematically in Table 1x. We are assuming that the probability of ‘false vacancy’, PROBFV, never exceeds 95% (which seems to be shown in Figures 78 & 4x). If PROBFV were assumed to reach 100% for the lower values of PDTT, then the results given below would be even more pessimistic (i.e. the overall probability of ‘false vacancy’ detection would increase).

- Since the region -140 dBm to -100 dBm is of interest, all integrations are performed over this region. The results for these 5 sensitivity levels are given in Table 2x.

| WSD sensitivity | PROBFV_OVERALL |
| --- | --- |
| -140 dBm | 0.00% |
| -130 dBm | 11.875% |
| -120 dBm | 35.625% |
| -110 dBm | 59.375% |
| -100 dBm | 83.125% |
| Table 2x |  |

The results for any WSD sensitivity between -140 dBm and -100 dBm can be given, as follows:

- For sensitivity SEN, -140 dBm ≤ SEN ≤ -130 dBm: PROBFV_OVERALL = 19(140 + SEN)/16

- For sensitivity SEN, -130 dBm ≤ SEN ≤ -100 dBm: PROBFV_OVERALL = 2.375SEN + 320.6233

The results for all WSD sensitivity values between -140 dBm and -100 dBm are plotted in Figure 5x.

Figure 5x

PROBABILITY OF ‘FALSE VACANCY’ AS A FUNCTION OF WSD SENSITIVITY

It should be noted that the detection of a DTT signal is purely based on the cyclic features of the DTT signal.

This means that in order to make a ‘false occupancy detection’, that is to make a detection of a signal which isn’t present, a well defined “cyclic feature of the DVB-T signal” would have to somehow be registered by the WSD sensor. This detection of a ‘cyclic feature’ of a non-existent signal should not be very probable: it would seem that a 50% probability for such a ‘detection’ would be rather high.

Although it has been claimed that the probability of ‘false occupancy detection’ decreases as the WSD sensitivity is increased, unfortunately, no measurement information has as been provided as to the actual behaviour of the probability of ‘false occupancy’ as a function of WSD sensitivity.

Nevertheless, in order to provide some theoretical feeling for what might be happening, the following is assumed:

- A set of 5 WSD sensors: WSD1, WSD2, WSD3, WSD4, WSD5 ranging in sensitivity from -140 dBm, -130 dBm, -120 dBm, -110 dBm, to -100 dBm, respectively – that is, from very sensitive to progressively less sensitive sensors.

- Because of the lack of information, we don’t know what the actual probability of ‘false occupancy detection’ would be, so we take a range of possibilities: 50%, 40%, 30%, 20%, 10% for the most sensitive sensor, WSD1. Presumably this range would cover the realistic range of possible ‘false occupancy detection’ probabilities.

- WSD1, with sensitivity -140 dBm is assumed to have the ‘worst’ set of probabilities for the ‘false occupancy detection’ ranging from 50%, 40%, 30%, 20%, 10%.

- WSD5, with sensitivity -100 dBm is assumed to have the ‘best’ set of probabilities for the ‘false occupancy detection’ ranging from 5%, 4%, 3%, 2%, 1%.

- WSD2, WSD3, WSD4 are assumed to have an increasingly improved set of probabilities for ‘false occupancy detection’, which are defined as linear interpolations between WSD1 and WSD5.

Using these assumptions, we arrive at the following Figure 6x which is representative of how the ‘improvement’ in the ‘false occupancy detection’ might look in practice, depending on the starting probability and the degree of improvement with increasing WSD sensing capability.

Figure 6x

Conclusions

Figures 5x and 6x are combined in Figure 7x to give a better overview of the complete situation.

Figure 7

It is seen that the overall probability of ‘false vacancy detection rises significantly faster than the probability of ‘false occupation detection’ decreases, irrespective of the ‘starting’ probability of ‘false occupation’.

DB calculations/determinations of ambient DTT signals may produce ‘false’ decisions, either ‘false-occupancy-detections’ or ‘false-vacancy-detections’. ‘False-vacancy-detections’ are more serious than ‘false-occupancy-detections’ because they can lead to interference to DTT reception.

WSD sensors may also produce ‘false’ decisions.

Increasing the WSD sensing threshold can lead to an increased number of ‘false-vacancy-detections’, which would lead to an increased number of interference situations for DTT reception. Conversely, the probability of ‘false-occupancy-detections’ would decrease with increasing WSD sensing threshold.

Using the practical implementation described in Annex 1, the increase in the average probability for false-vacancy-detections for receive signals in the lower power region (in our example, in the interval [-140 dBm,-100 dBm  dBm]) is greater than the decrease in probability for false-occupancy-detections.

It is most important that the ‘false-vacancy-detections be kept to a minimum, in order to keep interference to DTT reception to a minimum. Therefore the WSD sensor threshold level should be kept as minimum as possible.

Portable outdoor WSD sensing levels should be very stringent. Because their location is ‘variable’ they must be able to detect ambient DTT signals under the most severe receiving conditions.

Preliminary results on combination of geo-location database and sensing techniques in a real scenario (from Doc. 96)

Ed. Note: the need for this annex needs to be further assessed. In particular, this annex should be kept only if it discusses the possibility to relax the detection threshold on the basis of measurements. It is expected that this will be addressed in input contributions to the March 2012 SE43 meeting. Otherwise this annex will need to be deleted. It needs to be also remarked that some text from the main body of the report appears in this annex – unnessesary redundancy should be avoided]

Introduction

Chapter 11 of ECC Report no. 159 identifies a list of areas requiring further studies among which it is mentioned the elaboration of the approach(-es) combining the geo-location database and spectrum sensing.

This document describes an application methodology to combine geo-location database and spectrum sensing techniques for WSDs operation in the band 470-790 MHz. The proposed methodology is implemented in a real scenario in order to compare geo-location and sensing approaches and possibly identify the benefits of sensing techniques when used in combination with the geo-location database.  Simulation results of real DVB-T transmitters coverage areas have been obtained using a proprietary software tool while measurements of the received signal levels have been performed by means of a spectrum analyzer. Both simulations and measurements are used to assess DVB-T channel occupancy, respectively based on the geo-location database approach and on sensing techniques.

The methodology

In this document we apply the methodology described in Section 2.1 in the Working Document, on the combined sensing and geo-location approach. Channel occupancy based on the combined detection can be determined according to what presented in Error! Reference source not found. of the working document, recalled here below. The corresponding flow char of decision is also recalled, where the required sensing threshold represents a keypoint.

The methodology is applied in a real scenario in Italy, in the province of Bologna.

Table 13: Channel occupancy based on combined detection

| Geo-location flag | Sensing flag | Conclusion |
| --- | --- | --- |
| DG = 1 | DS(T) = 1 | The channel is occupied. |
| DG = 1 | DS(T) = 0 | The channel is occupied. |
| DG = 0 | DS(T) = 1 | The channel is occupied. |
| DG = 0 | DS(T) = 0 | The channel is vacant. |

Figure 48: Channel occupancy decision lind, based on combined detection

Application of the methodology in a real scenario

Geo-location database

The protection of the DVB-T service can be guaranteed based on the Reference Planning Configurations (RPCs) specified in Error! Reference source not found..  In Table 14 the minimum field strength levels of the GEO-06 RPC1 (fixed reception) at 650 MHz are shown for different location probability values.

Table 14: DVB-T Reference Planning Configurations (Fixed Reception)

| Frequency 650 MHz |  |
| --- | --- |
| Location Probability | Fk,min dBV/m at 10 m |
| 99% | 60 |
| 95 % | 56 |
| 50% | 48 |
| 1% | 34 |

In the considered scenario field strength levels for DVB-T channels from 21 to 60 are calculated over square pixels (400 m x 400 m), using the Recommendation ITU-R P.526[5] propagation model, which takes into account also diffraction phenomena. For each pixel and for each channel from 21 to 60, the software evaluates the maximum field strength level Erx (dBV/m) considering all possible DVB-T transmitters located both in the considered area and in neighbouring areas:

Erx (dBV/m) = Max(Etx (dBV/m) –Atot(dB))

where Erx (dBV/m) is a function of each considered DVB-T transmitter effective radiated power ERPTx (dBW) and Atot is the path loss.

The simulation results are then compared with the proper threshold (e.g. values listed in Table 14) in order to identify the DVB-T protection area, where the usage of the selected channel is prevented, and the paired zone outside the coverage area where the channel is potentially available for WSD. In particular, if for a given threshold Trx if :

| Erx (dBV/m) >= Trx | DG = 1 | The pixel is within the protected service contour. Hence the channel is occupied |
| --- | --- | --- |
| Erx (dBV/m) < Trx | DG = 0 | The pixel is outside the protected service contour. Hence the channel is vacant |

In 0 a schematic example of the distribution of the DVB-T coverage areas is shown assuming different DVB-T protection levels which correspond to different Location Probabilities (LP) values. As it can be expected the DVB-T coverage area increases as LP decreases. The lower the LP (e.g. LP = 1%), the stronger the protection level, as only a small percentage of users receive adequate DVB-T signal strength especially near the border of the protected area.

In 0 simulated field strength levels (dBV/m) are shown for channel 21 and 59. The colour scale reflects thresholds listed in Table 14. It can be noticed that for the considered channels most of the pixels of the province of Bologna are occupied, as only some hilly and mountainous areas quite far from Bologna are not reached by the DVB-T signal. Similar results have been computed with the employed simulation tool for all the DVB-T channels in the 470-790 MHz band (Channels 21-60). Based on this analysis the geo-location database can be populated: for each pixel and for each channel the proper value of the DG parameter (1 or 0) is stored.

Figure 49: DVB-T protected areas as a function of the considered Location Probability

Figure 50: Example of received DVB-T signal strength in the province of Bologna

1

2

3

4

In 0 and in 0 the geo-location database results obtained for channels 21-60 are reported only for the six locations (Villa Griffone, Ristorante Joli, Giardino Sasso Marconi, Val di Setta, Piccolo Paradiso, Mongardino), where also measurements are available. The selected location are those shown in 0.

Figure 51

1

2

3

4

5

6

: Geo-location database: comparison for 6 different locations (Villa Griffone, Ristorante Joli, Giardino Sasso Marconi, Val di Setta, Piccolo Paradiso, Mongardino) for the selected field strength threshold (1: 34 dBV/m, 2: 48 dBV/m, 3: 56 dBV/m, 4: 60 dBV/m)

Figure 52: Geo-location database: comparison in each location (1: Villa Griffone, 2: RistoranteJoli, 3: Giardino Sasso Marconi, 4: Val di Setta, 5: Piccolo Paradiso, 6: Mongardino) for different field strength threshold (34 dBV/m, 48 dBV/m, 56 dBV/m, 60 dBV/m).

Four different thresholds of 34 dBV/m, 48 dBV/m, 56 dBV/m, 60 dBV/m have been addressed (see Table 14) in order to identify if the channel is occupied (red) or free (green). Channel occupancy variation has been investigated comparing the different locations by fixing the threshold level (Figure 3) or investigating the effect of the selected threshold assuming the same position (Figure 4). From Figure 3 and Figure 4, it is evident that most of the DVB-T channels are occupied in the considered area. The selected minimum field strength level has an almost negligible impact on channel occupancy distribution.

Measurement for spectrum sensing

Further information on channel occupancy can be achieved by means of sensing techniques. To this aim several measurements have been realised in a number of locations of the considered area (province of Bologna).

In this work, a portable spectrum analyzer, the Narda SRM 3000[6], has been adopted for spectrum sensing. The instrument has been equipped either with a tri-axial isotropic antenna (0), which can operate in the frequency range 75 MHz – 3 GHz, or with a log periodic dipole array antenna, which can operate in the frequency range 200 MHz – 2.75 GHz. The two different antennas were chosen in order to have different sensitivity thresholds for the measurement set up: sensitivity threshold is as high as -80 dBm with the isotropic antenna and falls to -105 dBm with the logperiodic antenna.

Measurements have been performed using the “channel power” mode, in order to evaluate the total amount of power received in the DVB-T channel bandwidth of 8 MHz. A resolution bandwidth (RBW) of 30 kHz has been adopted and for each selected frequency range (e.g. Fmin = 590 MHz - Fmax = 598 MHz) the Integration Bandwidth (IBW) function has been used.

Measurements have been performed in six different locations listed in Error! Reference source not found. and shown Figure 54.

Figure 53: Narda[6] SRM 3000 equipment

Table 15: Location of the measurement locations

| Measurement point | X-UTM (ED50) | Y-UTM (ED50) | Height (m) above ground level |
| --- | --- | --- | --- |
| Villa Griffone | 680492 | 4922261 | 1.5 |
| Ristorante Joli | 679948 | 4920824 | 1.5 |
| Sasso Marconi | 679010 | 4917863 | 1.5 |
| Val di Setta | 679329 | 4917863 | 1.5 |
| Piccolo Paradiso | 679384 | 4915586 | 1.5 |
| Mongardino | 676364 | 4920300 | 1.5 |

Figure 54: Locations where measurements have been performed: Villa Griffone, Ristorante Joli, Giardino Sasso Marconi, Val di Setta, Piccolo Paradiso, Mongardino.

In Figure 54 measurements results obtained for channels 21-60 are reported in the six locations (Villa Griffone, Ristorante Joli, Giardino Sasso Marconi, Val di Setta, Piccolo Paradiso, Mongardino), for the two different probes: omnidirectional and log-periodic antenna. It is worth noting that for omnidirectional equipment results are characterized by received power always above a floor of -80 dBm, while with the log-periodic antenna the floor falls down to -105 dBm. Possible measured un-occupied channels (e.g. 34) correspond to field strength level below the instrument floor (-80 dBm for omnidirectional antenna and -105 dBm for log-periodic antenna). As it can be expected, as the value of the floor level decreases the number of un-occupied channel decreases.

1

2

Figure 55: Measured data: comparison for 6 different locations (Villa Griffone, Ristorante Joli, Giardino Sasso Marconi, Val di Setta, Piccolo Paradiso, Mongardino) for the two different instrument floor level (1: -80 dBm, 2:-105 dBm)

In Figure 55 measured data are shown for each location as a function of the two different floor levels. From Figure 55 it can be noticed that there are locations (e.g. location 5: Piccolo Paradiso) where the effect of the different threshold is negligible, while in other cases the number of un-occupied channels varies with the floor value.

1

2

4

5

6

3

Figure 56: Measured data: comparison in each location (1: Villa Griffone, 2: RistoranteJoli, 3: Giardino Sasso Marconi, 4: Val di Setta, 5: Piccolo Paradiso, 6: Mongardino) for the two different instrument floor level (1: -80 dBm, 2:-105 dBm)

Combined approach

In order to assess the effect of the combined approach, we considered both results from the geo-location database and measurements.

In Table 16 we report the percentage of channels where both geo-location database and measurements identify un-occupied channels (double green).

Table 16: Double Green Percentage

| Different field strength levels for DVB-T reference planning configurations (fixed reception) (dBV/m) | different instrument threshold (dBm) |  |
| --- | --- | --- |
|  | -80 | -105 |
| <=34 | 20.08 | 18.38 |
| <=48 | 38.89 | 34.19 |
| <=56 | 46.15 | 38.89 |
| <=60 | 50.43 | 41.88 |

As expected, the smaller percentage of double green channels is related to higher protection of the DVB-T service and to lower values of the equipment floor.

The double green percentage is in general more significantly dependent on the DVB-T protection threshold used to populate geolocation database, than on the sensitivity of the measurement equipment.

Conclusions

In this contribution we present an operative methodology to apply the geol-location database approach in combination with sensing measurements in real scenarios.

DVB-T coverage simulations have been performed in a real area of the province of the Bologna in order to identify occupied and vacant DVB-T channels. Subsequently channel occupancy of several specific locations has been investigated through experimental measurements using a portable SRM 3000 spectrum analyzer, equipped with different probes, in order to investigate the effect of the sensing threshold. Both simulations and measurements confirm that the WSD 470-790 MHz band is densely occupied in Italy.

Therefore in this scenario the geo-location database approach and the spectrum sensing technique should lead to similar conclusions on channel occupancy distribution.

Methodology used to construct a geolocation database for WSD deployment in the UK

Introduction

Ofcom have published a consultation paper [11], where they outline the idea of using a geolocation database for regulating ERP of White Space Devices (WSDs) which will reuse the broadcasting spectrum. The idea is that before transmitting, a WSD will interrogate a database, called the geolocation database, providing its location and its characteristics, and it will receive a list of channels it may use, as well as a limit on its maximum permissible ERP for each of these channels.

In Appendices A4 and A5 of Ofcom's consultation paper, some ideas on the generation of this geolocation database are presented. A more thorough analysis is presented in [12], accompanied with a wide range of examples.  JPP has also worked on this subject, and has proposed its own views on the issue. In this document we take into account both Ofcom's and JPP's proposals and focus on their implementation.

We present a methodology to perform a full computation of the data required to populate the geolocation database and use them in order to get some statistical data. We expand the procedures outlined in [12] and illustrate how they can be used in conjunction with the coverage data provided by UKPM.  An effort was made to align our implementation with the latest discussions that take place in the TVWS Technical Working Group, therefore we have adjusted some assumptions made in [12]:

  - The WSD-DTT coupling geometries proposed in [12], have been replaced by the reference and non-reference geometries discussed in [16], and the propagation model was adjusted accordingly.

  - We introduced the idea of DPSA layers, in order to decide which channels should be protected at each pixel

  - We only protect populated pixels, although our implementation is efficient enough to consider all land pixels of the UK.

In this work we also introduce is a semi-analytical approach to compute the statistics of the maximum protected WSD Field strength that can be tolerated at each pixel, without the use of computationally expensive numerical techniques (see [12] Section II.B). This allows the computation of the entire geolocation database data, with a good accuracy, in a reasonable amount of time (a few hours).

Finally we divided the methodology presented in [12] in two parts; this separation allows the caching and reuse of much of the data that are computationally expensive to derive.

The data we derived aim to protect outdoor DTT coverage, assuming a fixed directional antenna at roof top height (10m nominal). We do not take into account, for the moment, coverage protection for Portable Reception or PMSE. Issues like the presentation of the geolocation data to WSDs are also beyond the scope of this work.

Methodology

Our implementation suggestion is focused on a two phase approach. In the first phase we compute what we call the Protected Pixel-Channel Interference (PPCI) value, which is the maximum protected field strength of the interference that can be tolerated in any pixel-channel combination. A value higher that this will degrade TV reception, in that pixel and that channel.

Once the PPCI for every pixel-channel is computed, then for every pixel in the country, we compute the maximum ERP that will not result in excessive PPCI to ANY of the adjacent pixels and channels. Adjacent channel interference is taken into account at this stage. It is this maximum ERP that will be presented to the WSD.

We decided to split the process for two main reasons:

  - The computation of PPCI is largely independent from the WSD type. Since it is the protected field strength that is computed and cached, it is independent on the maximum ERP of the WSD or the WSD-DTT protection ratios. It is also unrelated to many of the model’s parameters, the propagation model to be used, or the assumptions on the link budget.

  - The computation of the PPCI takes significant time to compute. We developed a semi analytical method to derive it which is much faster than the Monte Carlo approach suggested in [11], but it is still relatively slow.

The two stage approach allows us to pre-calculate the PPCI values once and cache them, and then explore a number of strategies for computing the maximum ERP at a reasonable amount of time.

Computation of the PPCI cache

The PPCI cache can be seen as a collection of 2 dimensional layers, each layer representing a single channel. Each pixel of a layer corresponds to a 100 by 100m pixel of the UK and contains the maximum protected field strength that can be tolerated at that pixel. The protected field strength is defined as the actual field strength, adjusted by the appropriate WSD-DTT protection ratio. In this work we decided to protect all the UK pixels, not only the populated ones.

Figure 57 Derivation of the PPCI

The first step in the process is to decide which transmitter to protect at each pixel. In this work we follow the JPP proposal which suggests the use of the DPSA layers. A description of the DPSA concept is presented in [13].

JPP has generated a number of DPSA layers. Each layer defines, for each pixel in the UK, a single transmitter which must be protected. All channels of this transmitter are protected. For example, the 3PSB layer may state that Transmitter A will be protected while the PSBCOM layer may indicate transmitter B. Since we do not know which transmitter viewers at this pixel may opt for, we should protect all the channels of every transmitter that is listed in any of the DPSA layers, in this case all channels of transmitters A and B.

For each of the protected channels, we look up the UKPM results for the corresponding transmitter-channel, and retrieve four values:

  - the mean and standard deviation of the C/(I+N) named  and .

  - the mean and standard deviation of the wanted field strength called and .

These values allow us to derive the PPCI value as described in the following section.

Computation of the PPCI at a pixel in a channel

The computation of the PPCI is largely based on the analysis presented in [12]. Since UKPM coverage output is expressed in dBuV/m, we initially rewrite Eqns (1) to (4) of [12] in terms of Field Strength rather than received power.

Following the notation in [12], in the absence of interference from systems other than DTT, the location probability  at a channel is given by:

Where:

  - is the received power of the wanted signal

  - is the minimum received power and

  - and  are the received power of each interfering signal and the corresponding protection ratio.

The quantities in Eqn  are expressed in linear units, however it is more convenient to express them in dB. Then Eqn  becomes:

Eqn   can be rewritten in terms of Field Strength as:

Where:

  - is the wanted Field Strength at the receiver’s aerial.

  - is the minimum required field strength to get an acceptable reception.

  - is the protected field strength of each DTT interferer.

  - is the angle at which the receiver’s aerial ‘sees’ the corresponding interferer and is the corresponding azimuth gain reduction .

If we set:

then Eqn  becomes:

When expressed in dB,  can be modelled as a Gaussian random variable, with mean  and standard deviation . The same applies for the field strength of each interfering signal . Then, according to [15],  can also be approximated by a Gaussian, with mean value  and standard deviation .  Then , which is the protected C/(I+N) of the system, will also be a Gaussian, with mean value  and standard deviation :

and  can be extracted from the Coverage Assessment tools and are known at each covered pixel and  channel of  the country.

The presence of a WSD interferer will reduce the location probability to a value . Following the notation in [12]:

Where:

  - : is the WSD to DTT protection ratio.

  - : is the in-band  transmit power of WSD and

  - : is the WSD – DTT receiver coupling gain.

Expressing the values of Eqn  in dB gives:

If a WSD with ERP  and coupling gain  induces a field strength  at the receiver location, then Eqn  can be written in terms of field strength as:

We assume the worst case scenario where the receiver aerial points to the WSD, and the two antennas are co-polar, therefore there is no reduction in the aerial’s forward gain. If we set:

then  is the protected field strength of the interference generated by the WSD and Eqn  can be rewritten as

with operant  representing addition of and when expressed in linear units.  can be modelled as a Gaussian random variable with mean  and standard deviation . In this work we assume that:

is expressed in dBuV/m and is the mean value of the protected  WSD field strength. If we decide that the maximum deterioration in location probability is  then should be such that:

In other words  is the mean value of the protected field strength of the WSD interference that will result in a location probability reduction by. As long as the mean field strength of the interference from a WSD is kept below  then the location probability reduction will be less than . Therefore is the maximum permissible mean value for the WSD protected field strength, consequently, it is the value that should be cached in the PPCI.

As suggested in [12],  can be derived via Monte Carlo analysis; however we have developed a semi analytical technique, presented in the Appendix of this Annex, that allows it to be computed in a reasonable amount of time.

The above procedure is only valid when we deal with MFNs. We have devised a number of approximations so that it can be applied to SFNs, but more work is required in this direction.

Computation of the maximum permissible ERP

Once the PPCI for every pixel and channel has been computed, the computation of the maximum permissible ERP of a WSD, for every pixel in the UK, and every DTT channel, can take place. In order to find the maximum permissible power of a WSD at pixel, we define a circular area of interest around it, as shown in Fig 2:

Figure 58 :Definition of co and adjacent pixels

The radius of this area is a function of the class of the WSD. Following the extended Hata Urban propagation model, as suggested in [11] para A4.26, for WSD powers between 1-5W, a radius between 30 km and 50 km should be sufficient. If we want to consider more powerful devices, the radius should increase accordingly. The pixels outside the circle are assumed to be unaffected from the WSD.

When we consider adjacent channel interference, the radius will be significantly smaller. Given that the difference in the protection ratios can be more than 50 dB, following the extended Hata model, the radius should be 10-15 times smaller.

If the DTT receiver and the WSD are in the same pixel, we have no information on their separation, so the co-pixel computation needs to be based on a reference geometry. The same however applies for the 8 first-tier adjacent pixels, as the DTT receiver and WSD location uncertainty may result in minimal separation.

In order to find the maximum permissible ERP of the WSD, we search within the ‘volume’ of interest (co & adjacent pixels, co & adjacent channels) to find the pixel-channel combination that imposes the strictest restriction. We have to look at each such combination individually, compute the ERP that it permits, and select the combination that permits the lowest power. This is the ERP that can be allowed for the WSD.

Computation of the permissible ERP of a Pixel-Channel combination

Following the discussion above, we can identify 4 different scenarios:

|  | Co-channel | Adjacent channel |
| --- | --- | --- |
| Co-Pixel | Scenario A: Complete Ban | Scenario B : Use of reference Geometry |
| Adjacent Pixel | Scenario C: Apply a propagation Model & use co-channel protection Ratios | Scenario D: Apply a propagation Model & use adjacent-channel protection Ratios |

Table 17 : WSD to DTT receiver coupling possibilities

Scenario A: co-pixel co-channel

In this case the potential of a WSD to interfere with a DTT receiver is so great, that its transmit power should be too small for any practical use. Instead of stating a power which will be in the nW range, we opted for a complete ban.

Scenario B: co-Pixel adjacent channel

The difference between the co-channel and adjacent channel protection ratios can be large enough to allow the WSD to transmit at a useable power. In this report we only consider the ‘mobile - WSD’ reference geometry as shown in Fig 2 of [16].

If  is the ERP of the WSD, then the resulting Field Strength at a distance d from a dipole is given by [14] p.39 :

or:

In the remaining of this work we will assume that powers are expressed in dBm, field strengths are expressed in dBuV/m and distances in m. Then:

Since  is the mean value of , from Eqns  and  we get:

is the ERP of a White Space Device that results in a location probability degradation of .

Eqn  refers to the case where the DTT receiver aerial points directly to the WSD. If the WSDs are randomly located and randomly polarised, there will be some aerial directivity discrimination:

depends the azimuth radiation pattern of the DTT receiver aerial. ITU specify a reference pattern, which is used in the UKPM coverage calculations. In practice however, we can never be sure of the viewers’ aerials characteristics. Some viewers may use good quality and correctly installed models, that achieve the ITU gain and directivity, but in the same pixel, other viewers may opt for cheaper models, with unpredictable side lobes. The latter case is more common in areas with good coverage, where viewers may opt to reduce the cost of their installation. In order to cater for both scenaria, in this report we assume that there is no angular discrimination protection for WSD interference, i.e. 0. Introducing such discrimination requires a review of the commonly used aerial models and a derivation of a ‘typical installation’ reference pattern; both tasks are beyond the scope of this work.  Since both WSD and DTT aerial are assumed to be at 10m above ground, no vertical radiation pattern reduction is considered. Finally, the geometries presented in [16] suggest no cross polar discrimination. Under these assumptions, with d=20m, Eqn  becomes:

If the interference generated by the WSD is noise-like, we can use the protection margins suggested in [11] paragraph A4.17.

Scenario C: Adjacent-Pixel and co –channel

In order to compute the WSD ERP that will result at a specific field strength in a neighbouring pixel, we need to perform a link budget analysis between the two pixels. The propagation model that we assumed in this work is the extended Hata Urban model as suggested in [11] para A4.26. The path loss  is a function of the WSD – DTT receiver separation and is given by:

In the above formula, the distanceis expressed in metres. Following [16], table 2:

We set dB which is a good weighted average between large and small cities.

If the ERP of a WSD is  then the received power from a TV aerial at a distance  is

The term 2.15 is added because in this work we express  in dBd.

If the field strength of a WSD transmission at a location is , then the received power will be given by:

Where  is the frequency in MHz. The combination of the above equations gives:

or:

Since this scenario refers to the co-channel case,  therefore from Eqns  and  we get:

Solving for  gives:

Distance  is distance between the centres of the pixel where WSD transmits and that of the victim DTT receiver. According to the geometry in Fig.2, the minimum distance between ‘adjacent’ pixels is 200m. In practice however the WSD –DTT receiver separation can be smaller, since they can be at the edges of their pixels. It is obvious however that the actual distance can never be less than 100 m. Similarly, the distance uncertainty is always less that  m. Therefore in order to cope with the worst case scenario, we propose to use a modified value  in Eqn, defined as:

This approach has the advantage that, since , it does not violate the Tx-Rx separation restriction of the extended Hata model.

Scenario D: Adjacent Pixel, Adjacent Channel

This scenario is the extension of Scenario C with Eqn  written as:

Putting together all the possibilities presented in Table 1, the maximum ERP of a WSD device can be computed as:

Implementation

In order to get an estimate of the spectrum that is available for WSDs, we developed a set of utilities that implement the described procedures. We made the following assumptions:

  - The standard deviation of the WSD signal is 3.5dB. This is not based on measurements; it is an estimate that takes into account the relatively short WSD-receiver separation, the random polarisation of WSD signals, and the diversity of the viewer’s aerial installations.

  - We chose a fixed  1%. There is still a debate on the issue, but this is a good starting point, and the algorithms can be adapted to cater for a varying .

  - Out-of-band emissions of the WSD were ignored. They can be taken into account however by modifying the protection ratios as described in [11] para A5.19, but this is currently beyond the scope of this work.

  - No receiver overloading was taken into account.

  - We did not provide any extra margin for multiple WSDs, errors in the UKPM, errors in the Hata model, location uncertainty etc. Such a margin can be introduced in the calculations at a later stage.

We used the C 20.2 plan for the transmitter network, and used the final state of the network (end of Continental Clearance). The analysis was based on coverage results that take tropospheric interference into account, so the values for , and  are those predicted for 1% time interference.

Regarding the DPSA layers we used:

- The PSB layer

- The PSBCOM layer

- The Nations layer

- The dAPSA layer and

- The Marginal layer

We decided to protect services at Marginal level. This means that for each pixel, we protect every transmitter that provides at least 70% location probability, at 50% time. The coverage data used in this analysis are based on 1% time interference therefore, for some pixel – channel combinations, location probability may drop below the 70% acceptable threshold. We decided to continue protecting such pixel-channels, down to 50% location probability. We made that decision in order to protect viewers at locations with marginal coverage. Tropospheric interference will only affect these viewers a few days a year, but if we allow WSD interference, its effects can be continuous.

The current DPSA layers protect only the UK services. At some locations in Northern Ireland, WSDs may cause interference across the border, to the services of the Irish Republic. Furthermore, viewers in the UK may wish to receive these services, so services from the Irish Republic may also have to be protected at UK pixels. One way of doing this, is by introducing an extra DPSA layer, but more work is required on this issue. We also ignored the planned N.I. multiplexes.

We restricted the maximum ERP of WSDs to 1W, which is a reasonable limit for unlicensed consumer devices. The procedure described in this work can be easily adapted to cater for higher power devices. With this limit in mind, and , Eqn  implies that the radius of the area of interest (See Fig.2) for the co-channel case should be about 30km.

Conclusions - Future work

We presented a method of computing an upper limit for maximum ERP of white space devices in the UK, so as to contain their impact to DTT coverage. We focused on low power consumer devices, although the methodology can be easily adapted to cater for higher power, fixed, outdoor units. The proposed method is flexible and it can form the basis of computing the data that will populate the geolocation database.

The methodology is still under development and Ofcom have asked the BBC to make the database available to the TVWS industry. Some DPSA layers were not used, the number of the protected multiplexes in N.I. needs rethinking and there are no restrictions to protect PMSEs or portable coverage. We also need to decide on issues like the extra protection margins to compensate for errors in the prediction algorithms, or the presence of multiple WSDs in a pixel.

Our next plan is to address the above issues, and provide a more comprehensive set of availability maps and diagrams that will help decision making.

- Appendix to ANNEX A12
Derivation of the Maximum Protected Field Strength

Eqn  can be written as:

  - , when expressed in dB is a Gaussian random variable (R.V.) that represents the field strength of the wanted signal, and has a mean value  and standard deviation .

  - U is also Gaussian when expressed in dB, and represents the combined protected field strength of the DTT interferers. It has mean  and standard deviation.

  - Finally, when is expressed in dB, it is Gaussian as well, and represents the protected field strength of the WSD device, with mean  and standard deviation. is the value we want to compute, and  is set to a fixed value, in this work we set it at 3.5dB.

  - Operant refers to linear addition.

The combined protected field strength of the  DTT and WSD interference, , is then a Gaussian [15], with mean value  and standard deviation  . Therefore, the C/(I+N)  in the presence of the WSD, written as  ,  is also a Gaussian R.V with mean  and standard deviation . Then:

Eqn  then becomes:

If we knew , then:

And since :

is the location probability in the absence of WSD interference and is computed from:

and  are taken from the UKPM results (see Fig.1).

If we can estimate , then it will be possible to compute . The procedure is non-trivial; the first problem is that, although we know  we do not know . The paper of Schwartz and Yeh [15] shows a method to compute , but it requires - the exact value we want to estimate. One way to address this issue is through iterations. Initially we set  ( is the standard deviation of C/I+N in the absence of WSD). We use this initial value of  to compute  (through Eqn ), then we use  to compute , and finally we use  for a more accurate estimate of . Our implementation has shown that the values stabilise in less than 5 iterations.

Let’s have a look at the details:

If we set  then  can be derived from Eqn . We name the degradation of the C/(I+N) that is caused by the WSD and it can be computed through :

is the C/(I+N) value in the absence of WSD, and is a value that we get from the UKPM results (see Fig.1).  From Eqns  and  we have:

Using Eqn  the above equation can be written as:

or:

If we set, then  is also Gaussian (since it is expressed in dB) with mean value  and standard deviation :

Then, following Eqn 29 of [14]:

and combining it with Eqn  gives:

Since  is known through Eqn , and  from Eqns , we can use the above equation to compute  and then . To do that however we need to derive the inverse of (for a given ), which we believe is impractical. We chose instead to compute  numerically, and fill a lookup table for every practical value of  and  . Once  is estimated,  can be derived from:

is the mean value of the wanted field strength,  and is one of the values that we get from the UKPM (see Fig.1). Once a first estimate of  is computed (it is an estimate because we assumed , we can go back and compute :

Following Eqn (30) of [15] :

And since  ( Eqn  ):

If we repeat the process a few times, the value of  will stabilise and from that we can obtain  which is the protected field strength we seek.

is the standard deviation of the wanted signal (expressed in dB). In MFN networks, the wanted signal comes from one transmitter, so dB. In SFNs though the value can be different, As a result we have to read  form the UKPM results rather than setting it to a fixed value.

List of reference

- CEPT Report 24

ECC Report 159

Recommendation ITU-R BT.419

Recommendation ITU-R F.1336-2

Recommendation ITU-R P.526-11: “Propagation by diffraction”, Ottobre 2009

Narda (www.narda.com)

ECC Report 148

ECC Report 138

Recommendation ITU-R P.1546-2??

(See doc. SE43(11)11)

Ofcom, IMPLEMENTING GEOLOCATION http://stakeholders.ofcom.org.uk/binaries/consultations/geolocation/summary/geolocation.pdf

H.R.Karimi, Geolocation databases for white space devices in the UHF TV bands: Specification of maximum permitted emission levels. 2001 IEEE International Symposium on Dynamic Spectrum Access Networks (DySPAN) pp.443-453

JPP, Definition of the Digital Preferred Service Area Post DSO, Doc No JPP 667, v 1.04 28 June 2011

M.P.M. Hall et al Propagation of Radiowaves ISBN 0-85296-819-1

S.C.Schwartz , Y.S.Yeh On the distribution Function and Moments of Power Sums With Log-Normal Components. Bell System Technical Journal, Vol 61, No7, September 1982.

H.R.Karimi, S.Y.Tan, Interferer-victim geometries for use by white space databases in the context of DTT protection TVWS Discussion Document 29-11-2011, Updated 16-12-2011

The reference sensitivity level of a receiver is the minimum wanted signal power for which the receiver can operate correctly in a noise-limited environment. It corresponds to the DTT minimum median field strength.

Monte Carlo simulations provide the most accurate results because the statistical variations of the signals are taken into account in a comprehensive manner. Because of the ‘complete’ incorporation of the statistics, a computer must be used for the simulations

‘Nuisance field’ is defined, and its use described, in Annex 3.

Gwsd = -LOSSpath – POL – DISCrx + Ga, where POL is the polarization discrimination, DISCrx is the receive antenna discrimination, Ga the receive antenna gain, including losses.

S. Schwartz and Y. S. Yeh, “On the distribution function and moments of power sums with log-normal components”, Bell Syst. Tech. J., vol. 61, pp. 1441–1462, Sept. 1982

K. W. Sung, M. Tercero, and J. Zander, “Aggregate interference in secondary access with interference protection,” Communications Letters, IEEE, vol. 15, no. 6, pp. 629 –631, June 2011.

ECC Report 159 provides an incorrect approximation to Pwsd_tx_dBm based on equation 5.1-9. It should be noted that this wrong approximation can lead to a WSD eirp which is about 10 dB to large, which would have a significant detrimental impact on DTT coverage/reception.

Note that the antenna patterns given by Recommendation ITU-R F.1336-2 are intended to be used for coordination studies and for interference assessment, .e.g., between 2 primary services.

5.306	Additional allocation:  in Region 1, except in the African Broadcasting Area (see Nos. 5.10 to 5.13), and in Region 3, the band 608-614 MHz is also allocated to the radio astronomy service on a secondary basis.

According to 5.149, “administrations are urged to take all practicable steps to protect the radio astronomy service operating in the band 608-614 MHz from harmful interference. Emissions from space borne or airborne stations can be particularly serious sources of interference to the radio astronomy service (see Nos. 4.5 and 4.6 and Article 29).”

The WSD network capacity is compared here based on the simulation results on the location specific output power level when different types of IMdBcalculation method are used.

Sometimes the nuisance field for the noise is called the ‘minimum field’, Emin.

The power sum of two fields, A and B, is calculated as A  B = 10 log (10A/10 + 10B/10).

Note: this is a slightly unusual way of representing this relationship. Usually the field strength is plotted as a function of distance. The present representation is convenient for the purposes of this contribution.

Note: we have extrapolated Recommendation ITU R P.1546 below 1 km to 0.9 km in order to ‘finish’ the curve.

Presumably X% would be a percentage higher than 95% (which usually is the minimum percentage considered acceptable).

If wall loss is to be considered, then

[In/Nn]med = [Imed/N] + PR – [C/N]= Pt – LOSS(r) – Wloss – ATTt – POL – DISCr + Ga +PR – [C/N.]

If wall loss is to be considered Pt < [I/N]lim + LOSS(r) + Wloss + ATTt – PR + Pn – µx% (w2 + wsd2 + wl2)½

If wall loss is to be considered Pt < [I/N]lim + LOSS(r) + Wloss + POL + DISCr + ATTt – Ga + N – µx% (wsd2 + wl2)½

Note: the ‘stochastic’ values referred to represent the statistical fluctuations of the log-normal variable involved.

Note that w does not appear in this expression, because there is no dependence on the wanted power when considering DTTB receiver overloading.

We use free space loss for 20 m separation distance: LOSS(.02) = 32.45 + 20log f + 20log .02 = 54.72 dB.

Oth = -20 dBm, Ga = 9.15 dB, LOSS (.02) = 54.7 dB, POL = 3 dB, DISCTV = DISCWSD =0 dB.

Oth = -20 dBm, Ga = 2.15 dB, LOSS (.0217) = 55.45 dB, POL = 0 dB, DISCTV = 0 dB, DISCWSD = 10 dB.

Ga = 9.15 dB, POL = 3 dB, LOSS (.02) = 54.7 dB, µ99.9% = 3.09, WSD = 3.5 dB

See http://www.semtech.com/images/promo/Semtech_ACS_Rad_Pwr_Field_Strength.pdf

In the S&Y paper [14]  and  are expressed in Neper-Bell, while in this article we work with dB. An appropriate scaling has to be applied.  and  of this article correspond to  and of [14].