	[image: image40.wmf]117

127

115

.

11

095

.

117

0

127

140

95

.

-

£

£

-

-

-

=

-

>

=

-

<

<

-

=

x

if

x

P

dBm

x

if

P

dBm

x

if

P

measured

fn

fn

fn


	SE43(11)29


	9th meeting of SE43

Bologna, 5 – 7 July 2011
	

	Date issued:
	30 June 2011

	Source:
	EBU

	Subject:
	Proposed modifications to the SE43 working document


	Summary:
	This contribution proposes modifications to the working document contained in Annex 4 to SE43(11)18.

They concern mainly section 2 dealing with Combined sensing and geolocation, and propose additional text to the related Annex A2. The proposed modifications aim to further clarify the consequences of increasing the sensing threshold on the probabilities of “false negatives” and “false positives” using for this the practical example of sensing device presented in the working document.


	Proposal:
	The modifications are submitted for discussion and possible approval by the SE43 group


	Background:
	The SE43 group has identified the following issues to be addressed immediately: 

· Identification of a common set of the parameters defined in the methodology described in draft ECC Report 159 to calculate location specific WSD power levels; 

· Further studies on the impact from WSD on services in the bands adjacent to the 470-790 MHz band, e.g. on mobile service below 470 MHz and above 790 MHz; 

· Further elaboration of the approach(es) combining the geo-location database and spectrum sensing. The development of a working document on this was already initiated at the 8th SE43 meeting issue following the comments received during Public Consultation.


Annex 4 to 
Document SE43(11)18

working document on further definition of technical and 
operational requirements for the operation of white space devices in the band 470-790 MHz
1 Feasibility of autonomous operation of WSDs using sensing 
(in response to Item A2)
Autonomous operation of WSDs using sensing has been studied by theoretical work, simulations and by practical implementations. Trials have been performed to verify the performance also in the field. 

All this work has shown that a reasonable -120 dBm performance can be achieved at the sensing receiver input at ideal conditions taking into account specific DTT-transmission characteristics. The key challenge is antenna and sensor integration into the device and it is obvious that the strictest sensing requirements cannot be met using a single device and especially single shot decisions. However by collecting more samples, position information and combining the results from several devices better results could be expected.

Single device single snapshot detection is too unreliable due antenna gain minima, interference, and fading to protect incumbent users. Simply tightening the sensitivity requirement does not help, because IM products cause desensitization, masking, and false alarms, thus reducing available capacity for white space devices. Methods like geo location databases have to be used for reliable operation. Collaborative sensing reduces the antenna gain and radio propagation problem. However trade-offs between sensitivity in sensor linearity requirement and sensitivity has to be taken into account in algorithm development.

The details of practical implementation and field tests are presented in Annex A1. 

2 Combined sensing and geo-location 
(in response to Item G4)
The spectrum sensing can be used to support the detection of incumbent radio services conducted using the geo-location database. Such a combined technique to compute the white space spectrum has the following advantages:

· It reduces the risk of interference compared to cases when either sensing only or geo-location only techniques are used;

· It allows detection of services/systems  that are not registered in the database;

· It allows using sensing devices  which do not meet the required standalone sensitivity requirements because of practical implementation reasons. .


2.1 Methodology

The conclusion on the channel occupancy as derived on the basis of spectrum sensing only (DS(T)) can be presented as given in Table 9.1.

	Flag
	Conclusion

	DS(T) = 1
	The detected power is equal or higher than the detection threshold T. Hence the channel is occupied.

	DS(T) = 0
	The detected power is below the detection threshold T. Hence the channel is vacant.


Table 2.1: Channel occupancy based on spectrum sensing only
Similarly, the conclusion on the location of a WSD relative to the protected service contour as derived on the basis of geo-location only (DG) can be presented as given in Table 9.2.  

	Flag
	Conclusion

	DG = 1
	The WSD is within the protected service contour. Hence the channel is occupied.

	DG = 0
	The WSD is outside the protected service contour. Hence the channel is vacant.


Table 2.2: Channel occupancy based on geo-location only
When combining geo-location information with sensing results, the conclusion on the channel occupancy can be presented as given in Table 2.3.

	Geo-location flag
	Sensing flag
	Conclusion

	DG = 1
	DS(T) = 1
	The channel is occupied.

	DG = 1
	DS(T) = 0
	The channel is occupied.

	DG = 0
	DS(T) = 1
	The channel is occupied.

	DG = 0
	DS(T) = 0
	The channel is vacant.


Table 2.3: Channel occupancy based on combined detection

Figure X below shows the flow chart of decision in line with table 2.3 and introduces the notion of  Required sensing Threshold

[image: image2]
Because both DB facilities and WSD sensing are ‘imperfect’, there is the possibility, using either (or both) technique(s), of arriving at

i. a ‘false negative’, i.e. the indication that the DTT channel is not being used when in fact it is occupied, or

ii. a ‘false positive’, i.e. the indication that the DTT channel is occupied when in fact it is not being used.

Both DB and WSD sensing decisions will have more likelihood of error in areas where the DTT field strength is low, and DTT reception is most likely to be degraded. Thus it is necessary to use both DB and WSD sensing capabilities to their fullest extent.

The following section discusses the consequences of using an imperfect sensing device in terms of false negative and false positive decisions. 
We define two entities related to sensing:

1- The actual sensitivity of the device: This expresses the physical capability of the sensor to recognize the wanted signals with low levels. It can be defined in terms of a signal level associated with a probability of good detection (see for example ”Figure 7” in Annex A1).

2- The sensing threshold: This is a level which is arbitrarily set to be used as a criterion to decide whether the signal is considered present or not.

How to specify the required sensitivity (SN)?

The required sensitivity should ideally be equal to the lowest level that corresponds to the presence of a useable signal in the configuration being considered. Example, for a portable indoor sensing device that has to detect a DTT signal received with a roof top antenna, its sensitivity should ideally be as low as -140 dBm (see Rep159, page 22, Table 3, second column).

Practical implementation of sensors with such a low sensitivity is very challenging, if not impossible for the time being, as is shown in section 1 above.  Referring to ”Figure 7” of Annex A1, the actual measured sensitivity of a practical sensor is -117dBm with 100% probability of detection and -127 dBm with 5% probability of detection (the simulated sensitivity is -122dBm with 100%probability of detection and -133 dBm with 5% probability of detection).

How to specify the sensing threshold (ST)?
Using the practical example of implementation described in section 1 and in Annex A1, it is demonstrated in Annex A2 that:

· Increasing the WSD sensing threshold can lead to an increased number of ‘false negative’ detections (i.e. the indications that the DTT channel is not being used when in fact it is occupied), which would lead to an increased number of interference situations for DTT reception;
· Conversely, the probability of ‘false positive’ detections (i.e. the indications that the DTT channel is occupied when in fact it is not being used) would decrease with increasing WSD sensing threshold, which would lead to more spectrum available for the WSDs.
The increase in the average probability for false negatives for receive signals in the lower power region (in our example, in the interval [-140 dBm, -115 dBm]) is greater than the decrease in probability for false positives. It is most important that the ‘false negatives' be kept to a minimum, in order to keep interference to DTT reception to a minimum.
For the practical sensor considered and the range of signals to be detected in the example, the sensing threshold can be set at -127dBm (instead of -140dBm) as this corresponds to the point where the probability of “false negatives"  starts to increase rapidly.  
Such a sensing threshold level would not be usable in an autonomous operation as it has a 70% chance to give a false negative detection (see Figure XX in Annex A2). However, if used in combination with geo-location and using the decision algorithm described above it offers an additional check, giving an overall probability of false negatives lower than the individual probabilities of false negatives of each method (geo-location or sensing) used autonomously.
· 
· 
Based on the above there may be some scope for administrations to increase the detection thresholds with respect to their values derived for WSDs with spectrum sensing only capabilities in order to take into account the practical implementation constraints.. 
However, a relaxation in the detection threshold, if imposed for practical implementation reasons, would only apply for incumbent services/systems that are registered in the database. For incumbent services/systems that might operate without being registered in the database the sensitivity level for WSD standalone operation will need to be preserved. As already suggested in § 9.3.4 of ECC Report 159, the geo-location database may require the WSD to sense in conjunction with the geo-location at a given frequency. In this case, the database may also provide the WSD with information on the type of services/systems to be sensed as well as with sensitivity levels required for this purpose.

Further investigations for the possibility of relaxation as well as an amount of this relaxation and exact conditions are needed.

2.2 Algorithm

The combination of sensing with geo-location can be used in the following operational architectures:

· Single WSD. The device has access to a geo-location database and is equipped with spectrum sensing capabilities. 

· Master/Slave WSDs. The master WSD has access to a geo-location database and is not necessary equipped with spectrum sensing capabilities. The slave WSD is equipped with spectrum sensing capabilities and does not access the database itself.  

The algorithm of information exchange with the geo-location database under the combined detection approach for both single WSDs and Master/Slave WSDs is presented in Figure 9.1 and Figure 9.2, respectively.

	[image: image3.png]Remarks
Request for vacant channels Information is sent from WSD to DB
>l (see§9.3.3)
Vacant channels, powers, etc Information is sent from DB to WSD
(see §9.34)

Request for sensing

Spectrum Sensing with a given threshold
sensing

Computing Decision on channel occupancy
of vacant (see Table 9.3)

channel(s)

Transmission on a vacant channel.

Transmission General requirements to WSD apply
(see §9.3.2)






	Figure 2.1 Operational algorithm for the combined detection approach in a single WSD.


	[image: image4.png]Slave Master

Y

Request for vacant channels

Request
for sensing

Sensing

Request for sensing

results

Vacant
channel

Computing
of vacant
channel(s;

Transmission

Vacantchannels, powers, etc

ﬂ Remarks

Information is sent from WSD to DB
(see§9.3.3)

Informationiis sent from DB to WSD
(see §9.3.4)

Requestto the Slave to sense for
incumbent users on specific channels

Sensing with a given threshold

Return of sensing results to the Master

Decision on channel occupancy
(seeTable 9.3)

Information to the Slave on a channel to
be used

Transmission between the Master
and the Slave on a specified channel





	Figure 2.2 Operational algorithm for the combined detection approach in Master/Slave WSDs.


3 technical considerations on the protection of the broadcasting service 
(in response to Item B)
A proposal (Document SE43(11)12) was submitted to extend the methodology described in ECC Report 159 and to define a common set of parameters for the calculation of the location specific WSD power levels. This allowed to derive a set of maximum figures for power levels for fixed WSD and for portable outdoor WSD.

It consists in:

· Applying the criteria of 0.1% degradation for the location probability from the edge of the coverage until a distance inside the coverage area corresponding to a wanted field strength which is 10 dB higher than the minimum field strength. The calculation is made according to the methodology described in Report 159;

· Limiting the interference level for the remaining part of the coverage area to the highest interference level calculated above, corresponding to I/N ( -3 dB;

· Assuming that 3 equivalent WSDs are contributing to the interference to DTT coverage when adjacent channels are used;

· Assuming a minimum separation distance of 20-22 m between the interfering WSD (either fixed or portable) and a fixed roof top antenna of the wanted DTT;

· Assuming a minimum separation distance of 2 m between the interfering WSD (portable) and a portable antenna of the wanted DTT. 

With this set of assumptions, the maximum EIRP for WSD as function of the wanted field strength and the protection ratio can be derived. 

For example:

·  For the protection of DTT fixed reception, a fixed WSD (respectively portable outdoor WSD) using a channel with a frequency offset corresponding to a protection ratio of -50 dB (probably the third adjacent channel and beyond), can be operated with an EIRP in the range of -4 to +13 dBm (respectively -7 to +11 dBm) depending on its location within the DTT coverage area; 

· For the protection of DTT portable outdoor reception, a portable WSD using a channel with a frequency offset corresponding to a protection ratio of -50 dB (probably the third adjacent channel and beyond), can be operated with an EIRP in the range of -23 to -4 dBm depending on its location within the DTT coverage area.
Another contribution (Document SE43(11)11) noted that the methodology of EIRP calculation of WSD in Report 159 is based on the prediction of location probability of DTT coverage. The location probability depends on the predicted level of the DTT signal and consequently on the propagation model used. The maximum EIRP of the WSD will then be related to the predicted median field strength. If this latter is over-estimated, due to the imprecision of the field strength prediction model, or the inaccuracy of the transmitter characteristics, the resulting power assigned to the WSD may cause interference.
Annex A1: Example spectrum sensor implementation and field test results
A1.1 Spectrum sensor implementation

A1.1.1 Spectrum sensor embedded to a mobile device

In order to conduct field tests using a device with realistic form factor a spectrum sensor was embedded into a Nokia N900 mobile computer with all functionalities. The choice caused some extra challenges since the N900 has not been designed for a mobile TV receiver. Spectrum sensor hardware has been designed on a separate printed circuit board (PCB) and it has been equipped with hardware which enables to receive desired frequency bands and realize all spectrum sensor functionality, see Figure 1. Figure 2 shows the two complete signal paths that have been implemented on the PCB from an antenna element to a FPGA. Two separate RF frontend chips were required: one for UHF frequencies and one for IEEE802.11a/b/g (2.4/5.8 GHz). The used RF receivers are commercial RFIC and they are controlled by the FPGA. The analogue baseband data is digitized for the FPGA using two dual 10 bit AD converters operating at maximum rate of 80 MHz, depending on the system under detection.  Feature detector algorithms for spectrum sensing have been implemented on the FPGA.

[image: image5.jpg]



Figure 1. The spectrum sensor detector board inside N900 mobile phone.

[image: image6.png]FPGA:

Digital

A4

WLAN RF

— = control

Baseband

Control
Unit
(NIOSII)

CPUO
(NIOSII)

Result
Buffer
(FIFO)

—> =data

UART




Figure 2. Blocks on the detector board.

Communication between sensor board and the mobile device is done using a universal asynchronous receiver/transmitter (UART). The data rate between the sensor board and mobile device is 1 Mbit/s.

The spectrum sensor board is located inside the display slider case of the device. A custom plastic riser, see Figure 3 was required between the display and the bottom of the case to allow sufficient space for the board. Sensor board is located just behind of the display and on top of the slider mechanic. The slider mechanic is made of metal, as is the background of the display element. To ensure sufficient antenna efficiency both antennas had to be placed to the fin of the plastic riser that is outside the metal frame. It should however be noted that this is only due to the fact that the device has not been designed for spectrum sensor use. 

 Antenna design, especially at UHF band, is the utmost challenge in a spectrum sensor design. Best efficiency can be achieved with external antennas but for consumer devices embedded antennas have become de facto solutions. Relative bandwidths of the both antennas, UHF and WLAN, are reasonably high. Size and the location of the antennas inside the mobile device limit their efficiency and matching as well as the surrounding mechanics. Sizes of the antennas has been tried to keep as small as possible without losing performance too much. Antenna miniaturization in a mobile device scale is more problematic for an UHF antenna due to its longer electrical (and physical) length compared to a WLAN antenna.

[image: image7.png]Plastic riser

©—— original N900




Figure 3. Spectrum sensor prototype implementation on N900 mobile phone.
A1.1.2 System requirements related to spectrum sensing

Two very different kinds of target systems were addressed: DVB-T as an example of rather static TV primary system and 802.11a/g as an example of system having very dynamic traffic characteristics. Goal was to implement sensing strategy to measure both temporal and spectral characteristics of target systems. Another aspect was to measure spatial channel utilization in the field. We ended up in this phase to traditional channel numbering instead of generalized notation for cognitive radios in order to simplify control.

TV primary sensing requirement by FCC is -114 dBm sensitivity level averaged over a 6 MHz channel. This corresponds to -112.7 dBm averaged over a 8 MHz DVB-T channel. In order to measure UHF channel utilization, selected strategy is to make single detection per channel at each studied location. This requires quite low false alarm rate e.g. 1% and high probability of detection e.g. 99%. Excluding antenna losses, the sensor prototype presented in this work could reach these requirements with a sensing time of approximately 115 ms. However, for the longest detection time, i.e. 460ms, the headroom for antenna losses is only about 5dB.

In order to understand practical limitations of the platform and analyze filed test properly the prototype and its core entities were characterized both separately and as a complete system.

A1.1.3 Antenna

Two antennas were implemented inside the presented mobile spectrum sensing device. For UHF frequencies a commercial antenna based on planar technology has been used. Dimensions of the antenna are 45 mm x 5 mm and it has been designed to work at frequency range from 470 to 750 MHz (DVB-H EU). Antenna for 802.11a/b/g has been realized as a wideband structure which covers frequency range from 2 to 6 GHz. It has been implemented directly to the same PCB than the spectrum sensor. It requires slightly more area than the UHF antenna (32 mm x 8 mm). 

Both antennas were measured with and without the device mechanics to understand differences compared to conventional stand-alone antenna testing, and to evaluate actual performance in the field. Measurement results for the UHF antenna are presented in Figure 4 (left) and wideband antenna in Figure 4 (right). Deterioration of the efficiency of the UHF antenna due to mechanics is significant (6-8 dB) at low frequencies. The wideband antenna behaves better and its efficiency deterioration due to mechanics is only 1-2 dB over the whole band. The efficiencies of the antennas are -18-(-7)/-3/-6-(-4) dBs at UHF/2.4/5 GHz bands, respectively, depending on the specific channel. The results clearly indicate the issue of antenna performance at UHF band in small devices.

[image: image1.wmf]ECC

Electronic Communications Committee

CEPT

[image: image8.emf]
Figure 4.  Efficiency of the UHF antenna (left) and for comparison the WLAN-antenna (right).

A1.1.4 RF-parts

The used RF receivers are commercial RFIC and they are based on a direct-conversion architecture. Baseband filters are adjustable and they support several bandwidths used in different standards. Block diagrams of the receivers are presented in Figure 5. Typical noise figure (NF) of all receivers without front-end filter is around 4 dB depending on the band. Typical insertion-losses of front-end filter are 1.8 dB at UHF/2.4 GHz bands and 1.4 dB at 5 GHz band.  

[image: image9.png](a)

(b)

LX)

470-862 MHz T~ >
—($) ~ 3
Y4
_w—v:‘_. 0/90)
e
] > N
™
o PLL—@
2.4-2.5GHz
0/90)
4.9-5.8 GHz ~ >
)l Py 74
/'\/
0—0/90
g =] >
{ [ Ve 5
v
()

PLL /\\//

LX)




Figure 5. Block diagram of the UHF (a) and 802.11 a/b/g (b) receiver.

A1.1.5 Detector 

Detector core on the FPGA is developed from the FFT-based cyclostationary feature detector formerly presented by the authors in [1]. The structure of the detector is shown in Figure 6. The fixed-size-FFT implementation utilizes decimation after autocorrelation to control the detection time. Test for the presence of cyclostationary at given cyclic frequency (α) is performed from the FFT of the decimated autocorrelation signal.

[image: image10.wmf]
Figure 6. Structure of the implemented cyclostationary feature detector.

In this implementation, the range of selectable decimation ratios is extended up to M=2048 to support longer detection times. Similarly, the maximum autocorrelation delay (τmax) is increased to 8192. The modifications were required to enable detection of very long OFDM symbols used in DVB-T signals. The detector implementation utilizes 16k logic elements, 406k memory bits and 84 9-bit multiplier elements. The figures are 10.2%, 13.7% and 14.6% of all available resources on the FPGA, accordingly.

Detector sensitivity was measured for a WLAN signal at 2.4 GHz ISM band and for a DVB-T signal at the UHF band. Parameters related to modulation, signal bandwidth and transmit frequency of both systems are summarized in Table 1. During the measurements, the antennas were bypassed and the signal generator was connected directly to the RF receiver inputs, therefore the results exclude any antenna effects. The RF receivers operate at maximum gain. Detection times for WLAN and DVB-T were set to 0.8 ms and 460ms, accordingly. False alarm rate is 5%.

Table 1. Specifications of the primary signals used in detector performance measurements.

	
	WLAN
	DVB-T

	Modulation:
	OFDM
	OFDM

	 FFT-size (NFFT)
	64
	8192

	 Length of cyclic prefix (NCP)
	16
	1024

	 No. of non-zero subcarriers
	52
	6817

	 Subcarrier modulation
	16-QAM
	16-QAM

	Transmit frequency
	2437 MHz
	670 MHz

	Bandwidth
	20 MHz
	8 MHz


The measured sensitivities are presented in Figure 7 (left) for DVB-T and in Figure 7 (right) for WLAN signal. DVB-T detection reaches 95% probability of detection when the received power is about -117 dBm, while for the WLAN detection received power of -102 dBm is required. Both figures are below the thermal noise floor. Figure 7also show ideal simulation results for the same signals. The differences between simulated and measured probability of detections almost entirely match and are accounted by the non-zero noise figures of the RF receivers. The primary reason that DVB-T detection outperforms WLAN detection by such a large margin is the longer detection time that can be utilized in DVB-T detection. WLAN detection time is limited on the other hand by implementation, where larger FFT would be required to keep the cyclic frequency under the Nyquist frequency for larger decimation ratios, and on the other hand by duration of WLAN signal bursts which is already on the same scale with the detection time.

[image: image30.wmf]ò

ò

-

-

-

-

>=

<

115

140

115

140

dx

dx

P

P

fn

fn

[image: image11.wmf]
Figure 7.  Measured DVB-T (left) and WLAN (right) probability of detection compared to simulated performance. Simulation utilizes ideal receiver (NF=0dB).

A1.1.6 Platform Performance

Overall performance for the spectrum sensor hardware has been determined in the laboratory measurements. A 5 dB NF for UHF receiver path was measured at 660 MHz and it is only 1 dB more than NF of the pure UHF receiver. For dual-band 802.11a/b/g receiver, 5 dB and 6.5 dB NF at 2.427 and 5.130 GHz were measured, respectively. IIP3 of –10 dBm, -1 dBm and -1 dBm were measured for UHF, 2.4 and 5 GHz bands, respectively. 

When combined with antenna results the overall sensitivity of the signal detection for DVB-T signals at UHF band will be from –100 to –108 dBm depending on the channel of interest. This is significantly higher than FCC requirements but shows feasible values for small devices if the integration time of the detection is kept reasonable. IIP3 of the UHF receiver with antenna corresponds 8 - (-3) dBm compared to 0 dB antenna in the field tests, At some channels platform noise caused by processors and other noisy components in the device will further deteriorate the performance. However, those could be mostly avoided with proper design when UHF band requirements will be taken into account initially in the design of the device and its mechanics. For WLAN OFDM signal detection, the sensitivities using parameters given earlier in this paper will be –101 and –98 dBm (2.4 /5 GHz)  including the antenna.

A1.2 Field measurements
Two sets of field tests were carried out in capital area in Finland. First measurement set was done mostly outdoors in urban Ruoholahti area in Helsinki. Two sensors were used, both using internal antennas. The measurement set consists of spectral samples from 37 different locations, shown in Figure 8. One spectral sample includes detection time, GPS location, band, channel, received signal strength in dBm and DVB-T detection statistics from UHF channels 34 to 60 (578 – 784 MHz). Detection time was set to 460 ms and detection statistics positive detection threshold to produce constant false alarm rate of 1%. Measured signal strengths are shown in Figure 9. Corresponding estimated probabilities of DVB-T detections on different channels are shown in Figure 10. There is DVB-H repeater in the area, detected on channel 35. Espoo TV transmitter station is transmitting on channels 32, 35, 44, 46, and 53.

Table 2. DVB-T transmitter parameters.

	DVB-T transmitters
	Espoo
	Tallinn

	Latitude:
	60.1778
	59.4713

	Lognitude:
	24.6403
	24.8875

	Mast heigth:
	313 m
	272 m

	Transmission power:
	47 dBm
	42 dBm

	Occupied channels
	32, 35, 44, 46, 53
	45, 59, 64


TV transmissions on measurement range are detected with high probability. Channel 59 is occupied by Tallinn TV transmitter on average 78 km away. Open source Splat! [2] radio propagation calculation tool, using Longley-Rice Irregular Terrain Model [3] and NASA SRTM-3 Version 2 Elevation Models [4], was used for field strength estimation. Used transmitter parameters are shown in Table 2. , receiver was assumed to be 3m above sea surface. Estimated field strength, shown in Figure 11. in Ruoholahti area is 20-60 dBµV/m. Field strength has large variation within 1 km radius in urban area. With measured prototype antenna efficiency of -7.5 dB, it corresponds -123 – (-83) dBm signal input power at the receiver. Taking measured detector sensitivity into account we end up 0.6 to 1 detection probability of Tallinn TV transmitter in Ruoholahti, Helsinki. Tallinn transmission on channel 45, adjacent to much stronger Espoo TV transmitter on channel 44 and 46, is masked and it cannot be detected. One must remember that transmissions from Tallinn are out of the reach for typical TV reception setups in Helsinki households.

[image: image12.png]



Figure 8. Measurements results on UHF channel 44 (658 MHz) in Ruoholahti, Helsinki.
[image: image13.png]a0

0

00

33 3% ¥ 3w

3

o 4w e

= RSS10.1), ruoholahti

3 a1 a5 46 47 48

= RSSI(0.5), ruoholaht

50 51 52 53 54 55 56 57 55 59 60

s 1 E

R55(0.95) ruoholahi




Figure 9. Received signal strength (RSSI[dBm]) upper limit for 10%, 50% and 95% of measured samples in Ruoholahti.
[image: image14.png]3132 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 SO S1 52 53 54 S5 56 57 S8 59 60

= dett,ruoholahti





Figure 10. Measured probability of DVB-T detection, n = 37 per UHF channel, average distance to Espoo transmitter 15 km and 78 km to Tallinn transmitter.
[image: image15.png]68 dBEVim

78 dBpVim

88 dBpVim





Figure 11. Simulated field strengths on UHF channel 59 (778 MHz) from Tallinn TV tower, distance to Helsinki 77 km.
[image: image16.png]



Figure 12. Results on UHF channel 44 (658 MHz) in Espoo, average distance to Espoo TV tower is 8 km.
Second set was measured outdoors in suburban Espoo and target was to evaluate performance of the spectrum sensor in the vicinity of strong TV transmitter. Measurements were done using two sensors one with internal antenna and another with external reference dipole. Measurement locations and results for occupied channel 44 (658 MHz) are shown in Figure 12. Measured signal power on occupied channels was from -65 dBm up to -32 dBm, when using external reference dipole, as shown in Figure 13. The RSSI[dBm] limits tell how many percent of measurement samples have smaller RSSI value than presented. Basically this presents values of observed cumulative distribution function with 10%, 50% and 95% probabilities. TV signal strength is from 10 to 30 dB more than in the Helsinki measurement set. Main difference between results measured using internal and external antennas is that antenna efficiency of internal antenna is on average 7.5 dB lower than external reference dipole. Results using internal antenna are shown in Figure 14. Difference in antenna efficiency means additional noise figure of 7.5 dB, which decreases sensitivity and increases linearity of receiver. In addition there is also device induced noise in internal antenna measurements. 

Figure 15 shows the detection results over all channels with external and internal antennas. The antenna performance difference is very clear. Overall it can be seen that a high number of false detections happen due to the IM-products. There is also clear tradeoff between sensitivity and linearity. This is evident with the lower false alarm rate of the internal (lower gain, less signal power) antenna.

[image: image17.png]w
o
.
o
w | |||I|I|||II|||I | |||||||| III II

MASSI0.1) extantenna  # RSSO, extantenna  WASSI0.95),ext antenna




Figure 13. Received signal strength (RSSI[dBm]) with external antenna, upper limit for 10%, 50% and 95% of measured samples in Espoo.
[image: image18.png]u

5 a6 @

a S1os2 5154 s ss s7 s S9 60

WASSI0.1) intantenna W RSSI0S) intantenna  WRSSI(0.95) int antenna




Figure 14. Received signal strength (RSSI[dBm]) with internal antenna, upper limit for 10%, 50% and 95% of measured samples in Espoo.
[image: image19.png]100
050
080

o (R10111]8 TR

335 35 37 38 39 40 41 42 43 44 45 46 47 48 49 SO S1 52 53 54 S5 56 57 S8 59 60

= dett, external m det, internal





Figure 15. Measured probability of DVB detection, number of detections is 26 per channel, average distance to Espoo transmitter 9km and 78 km to Tallinn transmitter.

Annex A2: Trade-off between ‘false negative’ and ‘false positive’ detections as a function of increasing detection thresholds
A2.1 General

The purpose of WSD sensing and of the geo-location Data Base (DB) is to ensure that incumbent services, including DTT transmissions, are not interfered with by WSD usage. To this end it may be necessary to be somewhat ‘overprotective’ because of the inherent limitations of the WSD sensor and of the DB information and calculation abilities.

DB determinations of the presence or absence of DTT signals in a given area can be based on ‘internal’ calculations or on additional ‘real time’ information obtained from external sources, for example based on the ‘actual’ (measured) field strength values prevailing within the given area.

If complete ‘real time’ knowledge of precise ‘actual’ DTT field strength values were available everywhere, then DB calculations for the wanted DTT field strength values would not be necessary. Neither would WSDs need to be equipped with a sensing capability.

However, a complete knowledge of the actual DTT coverage situation will probably not be available, so that reliance on DB calculations as well as on WSD sensory capabilities must be taken on board to ensure sufficient protection for DTT reception.

Because both DB facilities and WSD sensing are ‘imperfect’, there is the possibility, using either (or both) technique(s), of arriving at

i. a ‘false negative’, i.e. the indication that the DTT channel is not being used when in fact it is occupied, or

ii. a ‘false positive’, i.e. the indication that the DTT channel is occupied when in fact it is not being used.

We are more concerned with preventing ‘false negatives’ than with preventing ‘false positives’ because the purpose of DB and WSD sensors is to avoid interference to DTT reception, and ‘false negatives’ will lead to DTT interference, whereas ‘false positives’ will not lead to DTT interference. Of course, ‘false positives’ should be avoided to the extent possible, but not at the cost of increasing the likelihood of ‘false negatives’.
A2.2 WSD sensing

WSD sensing is oriented towards the ‘actual’ values of the ambient field strength. That is, a DTT field strength is measured/detected (or not) but not calculated. The field strength value which the WSD measures/detects is that which is present at the WSD site, and it is usually not that which is actually present at nearby DTT reception sites. For example, a WSD may measure/detect a DTT field at street level (say 1.5 m) where the field may be very weak, whereas the DTT field of interest is much stronger, being received at rooftop height (say 10 m) 20 or 30 m away. Because of this ‘disconnect’, the WSD field strength sensor must be very sensitive, that is it must be able to detect field strengths at levels much lower than would be usable by a DTT receiver. Also, because of this disconnect, the WSD sensing is not really reliable with respect to predicting what the actual DTT signal strength is that may be receivable nearby. In other words, a WSD might not detect an ambient DTT signal, indicating (incorrectly) that either there is no DTT signal present or else that the field at the nearby DTT reception site would be too low for satisfactory reception. The WSD would register a ‘false negative’, although the DB with complete knowledge would register a ‘true positive’. To reduce this possibility of ‘false negatives’ the WSD sensing threshold level must be very low. Of course, the lowering of the sensing threshold may increase the likelihood of ‘false positives’.
A2.3 DB calculations

DB calculations/determinations of the presence (or not) of a wanted DTT signal have yet to be defined. Such calculations/determinations could consist of some of the following ingredients.

Input:

- DTT transmitter characteristics (site, transmit antenna site and antenna pattern, ERP, ...);

- DTT Rx characteristics (site/area, Ewmed_ref, ...).

DTT coverage contours can be calculated using this information, or else;

DTT coverage contours can be specified/defined by external sources.

In addition, the WSD location would need to be known.

One potential drawback of calculating or defining the DTT coverage contour is that often it is possible to have a satisfactory DTT reception ‘beyond’ the calculated coverage contour. In other words, the ‘true’ coverage ‘edge’ may be ‘outside’ the calculated/defined coverage contour. Either this is taken into account when the ‘reference’ coverage contour is calculated/defined or else areas of actual DTT reception may be ‘overlooked’ by the DB; this type of occurrence would constitute a possibility of concluding a ‘false negative’ for the presence of a DTT signal being reported by the DB. Of course, the opposite effect may also arise sometimes, where a ‘false positive’ would be indicated.
A2.4 DB errors

DB calculated field strength values may sometimes be erroneous (See doc. SE43(11)11). For example, calculated wanted field strength values may be ‘too low’ (e.g. near the DTT coverage edge) compared to the actual values which, though low, may still be sufficiently high to be receivable, and thus the DB would register a ‘false negative’.

It may also happen that the DB calculated field strength values are ‘too high’ compared to the actual values which may not be high enough to provide satisfactory reception quality, and thus the DB would register a ‘false positive’.

Both these cases (‘false negative’ and ‘false positive’) would be most likely to occur where the DTT reception is near the coverage ‘edge’, i.e. where the wanted DTT signal strength is ‘marginal’. For example, in an area where the wanted field strength (measured or calculated) is very high, an error in the calculations of some dB, either way, will not change the conclusion that a DTT signal is present, i.e. it will usually not lead to a ‘false negative’ decision.
A2.5 WSD errors

On the other hand, even when the DB calculates the wanted field strength values incorrectly, and indicates a ‘false negative’, the WSD might detect an ‘actual’ field strength level at or above the threshold which, taking into account the difference in respective receiving heights (WSD at 1.5 m vs. DTT at 10 m) and other possible positional and topological/structural effects, would indicate that a receivable DTT signal could be available nearby. 

To ensure that the WSD sensor is sufficiently reliable, a 99.99% reliability factor is built into the sensor in terms of a low sensitivity threshold.

The 99.99% reliability factor ensures that even those channels providing nearby DTT receivers with a ‘weak’ field strength, around Emin (i.e. near the coverage ‘edge’), are marked as being ‘occupied’ with a very high probability. Because of the low value of the threshold, areas which are well within the DTT coverage area will be less prone to false WSD evaluations (‘false positive’ and ‘false negative’). That is, once again, the possibility of a WSD providing a ‘false negative’ or a ‘false positive’ evaluation would most likely occur where the DTT reception is near the coverage ‘edge’.
A2.6 WSD detection errors when DTT signal is present

WSD sensitivity threshold TH0.

Assuming that a DTT signal is present at various levels of field strength, Figure 1a indicates the number of detections that are made by a sensor having a threshold level TH0. Below TH0, signals may be present, but they are not detected because the threshold TH0 is not low enough: i.e. there may be some ‘false negatives’.

The events between TH0 and TH1 will not be detected if the threshold is raised from TH0 to TH1. Those events will not be detected, i.e. they constitute ‘false negatives’

The events between TH1 and TH2 as well as those between TH0 and TH1 will not be detected if the threshold is raised from TH1 to TH2. That is, additional ‘false negatives’ will be registered.

Figure 1b shows how the probability of ‘false negatives’ will increase as the threshold is increased


[image: image20]
Conversely, the probability of ‘false positives’ would decrease with increasing WSD sensing threshold (e.g. if the threshold were very, very high, there would be no ‘positives’ at all, ‘true’ or ‘false’).

A2.7 DTT signal is not present

If no DTT signal is present (or it is too weak to be usable), there is nevertheless a probability that a signal is erroneously indicated by the WSD as being present – a ‘false positive’: green curve in Figure 2. The lower the WSD threshold level, the more probable that a ‘false positive’ will be indicated (the green curve increases with lowering sensing threshold). If the WSD threshold level is high the probability of ‘false positives’ becomes smaller: In Figure 2, TH2 > TH1, and P2 < P1.

But as seen from Figure 1b, the ‘false negatives’ will increase with increasing WSD threshold level, and it is most important that the ‘false negatives be kept to a minimum, in order to keep interference to DTT reception to a minimum.


[image: image21]
A2.8 Demonstration with a practical case
We will base our discussion in part on the Figure 7 of Annex A1, in particular on the ‘measured’ and ‘simulated’ results (see a copy of “Figure 7” below). 
[image: image22.wmf]
“Figure 7.  Measured DVB-T probability of detection compared to simulated performance. Simulation utilizes ideal receiver (NF=0dB).”
For the sensor being simulated/measured, it is apparent that the probability of detecting an ambient signal increases as the ambient signal becomes stronger. This implies, of course, that there is a probability of not detecting a signal which is present at a frequency in the measurement range. For the sake of simplicity, we propose the corresponding linear models in Figure 1 to approximate “Figure 7”. The two approximations are parallel with a 6 dB shift. The corresponding equations for the probability of (correct) detection (“PROB”) are given as Eqn. 1S (‘simulated’) and Eqn. 1M (‘measured’).


[image: image23]
Figure 1: Linear approximation to “Figure 7”

A2.9 FALSE NEGATIVES

Those equations can be modified to equations for the probability, Pfn = 1 – PROB, of incorrect ‘non-detection’, i.e. a false negative. These are depicted in Figure 2, with the corresponding equations Eqn. 2S and Eqn. 2M. (We use the notation “Pfn” for the probability of a ‘false negative’, and “Pfp” for the probability of a ‘false positive’.)


[image: image24]
Figure 2: Linear approximation to Probability of False Negatives

We calculate the probability of false negatives as the sensitivity level is raised from -140 dBm up to -115 dBm for both the ‘simulated’ and the ‘measured’ cases.
Simulated case:

If we have a wanted signal receive power somewhere in the interval [-140 dBm, -115 dBm] we can calculate the average probability of error (false negative) by integrating Pfn over [-140, ‑115]. We find the average probability of error <Pfn> = 45.6%. In other words, if the signal power lies somewhere in the interval [‑140 dBm, -115 dBm], there is a 45.6% probability that a false negative is registered.

We now raise the sensitivity threshold level from -140 dBm to L (≤ -115 dBm). This means that the ‘curve’ in Figure 1 gets ‘cut-off’ at L; in other words, for received power levels below L, the probability of detection becomes 0, and Pfn becomes 100% in Figure 2.
Then the equation for the probability, Pfn, of a false negative for a signal with a receive power occurring in the interval [-140 dBm, -115 dBm] can be written as Eqn. 3S as a function of L (note: there is a ‘jump’ in Pfn at L).


[image: image25.wmf]dBm

x

if

P

dBm

L

x

if

P

dBm

L

power

received

x

dBm

x

if

P

dBm

x

L

if

x

P

dBm

L

x

if

P

dBm

L

power

received

x

dBm

x

if

P

dBm

x

if

x

P

dBm

x

L

if

P

dBm

L

x

if

P

dBm

L

power

received

x

fn

fn

fn

fn

fn

fn

fn

fn

fn

123

0

1

115

123

;

123

0

123

685

.

11

095

.

1

123

133

;

123

0

123

133

685

.

11

095

.

133

95

.

1

133

;

-

>

=

£

=

-

<

<

-

º

-

>

=

-

£

£

-

-

=

£

=

-

<

<

-

º

-

>

=

-

£

£

-

-

-

=

-

£

<

=

£

=

-

£

º


(3S)
Integrating Eqn. 3S over the interval [-140, ‑115], we find the average
 probability of error, <Pfn>: 


<Pfn> = (.05 L + 18.4)/25



for -140 ≤ L ≤ -133 dBm
(4S1)


<Pfn> = (.0475 L2 + 12.685 L + 858.6275)/25
for -133 ≤ L ≤ -123 dBm
(4S2)


<Pfn> = (L + 140)/25



for -123 ≤ L ≤ -115 dBm
(4S3)

This is the average probability of error (i.e. of false negatives) for a signal with power somewhere in the interval [‑140 dBm, ‑115 dBm], if the sensing threshold is raised to L. This relationship is plotted in Figure 3 (‘false negative (simulated)’).

Measured case:

If we have a wanted signal receive power somewhere in the interval [-140 dBm, -115 dBm] we can calculate the average probability of error (false negative) by integrating Pfn over [-140, ‑115]. We find the average probability of error <Pfn> = 68.4%. In other words, if the signal power lies somewhere in the interval [‑140 dBm, -115dBm], there is a 68.4% probability that a false negative is registered.

We now raise the sensitivity threshold level from -140 dBm to L (< -115 dBm). This means that the ‘curve’ in Figure 1 gets ‘cut-off’ at L; in other words, for received power levels below L, the probability of detection becomes 0, and Pfn becomes 100%.
Then the equation for the probability, Pfn, of a false negative for a signal with a receive power occurring in the interval [-140 dBm, -115 dBm] can be written as Eqn. 3M (note: there is a ‘jump’ in Pfn at L).


[image: image26.wmf]dBm

L

x

if

P

dBm

L

x

if

P

dBm

L

power

received

x

dBm

x

if

P

dBm

x

L

if

x

P

dBm

L

x

if

P

dBm

L

power

received

x

dBm

x

if

P

dBm

x

if

x

P

dBm

x

L

if

P

dBm

L

x

if

P

dBm

L

power

received

x

fn

fn

fn

fn

fn

fn

fn

fn

fn

-

>

=

£

=

-

<

<

-

º

-

>

=

-

£

£

-

-

=

£

=

-

<

<

-

º

-

>

=

-

£

£

-

-

-

=

-

£

<

=

£

=

-

£

º

0

1

117

115

;

117

0

117

115

.

11

095

.

1

117

127

;

117

0

117

127

115

.

11

095

.

127

95

.

1

127

;


(3M)
Integrating Eqn. 3M over the interval [-140, ‑115], we find the average probability of error, <Pfn>: 


<Pfn> = (.05 L + 24.1)/25



for -140 ≤ L ≤ -133 dBm
(4M1)


<Pfn> = (.0475 L2 + 12.115 L + 790.2275)/25
for -133 ≤ L ≤ -123 dBm
(4M2)


<Pfn> = (L + 140)/25




for -123 ≤ L ≤ -115 dBm
(4M3)

This is the average probability of error (i.e. of false negatives) for a signal with power somewhere in the interval [‑140 dBm, ‑115 dBm], if the sensing threshold is raised to L. This relationship is plotted in Figure 3 (‘false negative (measured)’).

A2.10 FALSE POSITIVES

Unfortunately, no information is given about the sensor’s reliability in terms of not ‘detecting’ a signal which is not present (i.e. an estimate of Pfp, the probability of indicating a false positive). To cover this deficiency, we will have to make some assumptions which may or may not be correct. We make assumptions which appear to be plausible.

We assume that the WSD sensor operating in its most sensitive mode will have a probability Pfp to falsely ‘detect’ a signal which is not present (i.e. a false positive). The probability Pfp cannot be too large, because otherwise the sensor would be ‘useless’. 

We wish to give an approximate value to the Pfp which is ‘plausible’. We give the following plausibility argument (“L” represents the sensitivity threshold level, which will be varied between ‑140 dBm and -115 dBm):

> If Pfp is the probability of registering the presence of a signal, when no signal is present, then the probability of registering the presence of a signal must increase when a signal is present, Prp, must increase. (“Prp” is the probability of detecting a signal.)

> Thus Prp > Pfp.

> The probability Prp is about 5% for a receive power down to about -133 dBm for simulated signals and about -127 dBm for measured signals(see “Figure 7”).

> Thus, Pfp < 5%. To be the ‘most pessimistic’, we assume Pfp = 5% down to -133 dBm, -127 dBm, respectively for simulated and measured signals.

> It has been claimed that the probability for a false positive decreases when the sensor sensitivity threshold level is relaxed.

> We assume that the probability of a false positive decreases from 5% to 0% as the sensitivity threshold level is raised by 10 dB from L = -133 dBm (-127 dBm) to L = -123 dbBm (-117 dBm) for the simulated (measured) signal case.

> With this same 10 dB increase of the sensitivity threshold level, the probability for detection of a signal increases from 5% (at -133 dBm, -127 dBm, respectively) to 100% (at -123 dBm, ‑117 dBm, respectively) (see “Figure 7”).

So, on the basis of “Figure 7” above, we assume that Pfp = 5% when the sensor is working at its most sensitive level (-140 dBm) up to -133 dBm for simulated signals (-127 dBm for measured signals) and that Pfp = 0% when the sensing threshold level is relaxed to -123 dBm for simulated signals (L = -117 dBm for measured signals). We write simple equations (Eqn. 4S and 4M) for this relationship:


[image: image27.wmf]123

133

615

.

005

.

115

123

0

133

140

05

.

;

"

"

:

:

-

£

£

-

-

-

=

-

<

<

-

=

-

<

<

-

=

º

L

if

L

P

dBm

L

if

P

dBm

L

if

P

level

y

sensitivit

L

simulated

absent

signal

ction

dete

false

of

y

probabilit

P

fp

fp

fp

fp



(4S)



[image: image28.wmf]117

127

585

.

005

.

115

117

0

127

140

05

.

;

"

"

:

:

-

£

£

-

-

-

=

-

<

<

-

=

-

<

<

-

=

º

L

if

L

P

dBm

L

if

P

dBm

L

if

P

level

y

sensitivit

L

measured

absent

signal

ction

dete

false

of

y

probabilit

P

fp

fp

fp

fp


(4M)
This relationship for Pfp is compared graphically with the relationship for <Pfn> in Figure 3.
[image: image29.png]Average probability (%)

100

90

80

70

60

50

40

30

20

10

Average probability
E For false negatives and false positives (%)
For a signal somewhere in [-140 dBm, -115 dBm]

1 as a function of sensitivity threshold level
probability of false negative
(simulated)
probability of false negative
(measured)
probability of false positive
(simulated)
probability of false positive
(measured)

—
-140 -135 -130 -125 -120 -115

Sensitivity threshold level (dBm)





Figure 3: Average probability for false negatives (for signals with receive power in the interval [‑140 dBm, ‑115 dBm]) compared with the probability for false positives as a function of the WSD sensitivity threshold level L

It is seen that the probability of false positives decreases much less that the average probability of false negatives increases.
Thus, we might expect that the probability of false positives will be reduced from 5% to 0% if the sensitivity threshold level is increased, but, at the same time, the average probability of a false negative for signals in the -140 dBm to -115 dBm range raises from 45.6% to 100% in the simulated case and from 68.4% to 100% in the measured case.

A2.11 Conclusions
DB calculations/determinations of ambient DTT signals may produce ‘false’ decisions, either ‘false positives’ or’ false negatives’. ‘False negatives’ are more serious than ‘false positives’ because they can lead to interference to DTT reception.

WSD sensors may also produce ‘false’ decisions.

Both DB and WSD decisions will have more likelihood of error in areas where the DTT field strength is low, and DTT reception is most likely to be degraded. Thus it is necessary to use both DB and WSD capabilities to their fullest extent.

Increasing the WSD sensing threshold can lead to an increased number of ‘false negatives’, which would lead to an increased number of interference situations for DTT reception. Conversely, the probability of ‘false positives’ would decrease with increasing WSD sensing threshold. 
Using the practical implementation described in Annex 1, the increase in the average probability for false negatives for receive signals in the lower power region (in our example, in the interval [-140 dBm, -115 dBm]) is greater than the decrease in probability for false positives.
It is most important that the ‘false negatives be kept to a minimum, in order to keep interference to DTT reception to a minimum. Therefore the WSD sensor threshold level should be kept as minimum as possible. 

Portable outdoor WSD sensing levels should be very stringent. Because their location is ‘variable’ they must be able to detect ambient DTT signals under the most severe receiving conditions. 

Field strength level





# of events detected





TH0





TH1





TH2





Probability of false negatives





TH0





TH1





TH2





Threshold level





Figure 1a: as the threshold is increased, ambient DTT fields at low levels may, erroneously, miss being detected





Figure 1b: Increased probability of ‘false negatives’ as the threshold increases





TH1





TH2





N





TH





% ‘false positives’





P1





P2





Figure 2: % ‘false positives’ when a DTT field is not present as a function of WSD sensor threshold





Channel Unoccupied





DS(T)=0





(DG =0





yes





no





Sensing Required ? 





yes





no





no





Channel Unoccupied





DS(T)=X





(DG =0





yes





Channel Occupied





DS(T)=1





(DG =0





Threshold ST?





Sensed signal above 





Threshold ST





Sense channel at required





no





x





Probability of False Negativesw (%)





0





100





-110





-120





-130





-140





� EMBED Equation.3  ���





� EMBED Equation.3  ���





Eqn. 1M





Eqn. 1S





Probability of Detection: signal present


x = received power 





L





x





Probability of Detection (%)





0





100





-110





-120





-130





-140





ST





Required Threshold





Can device Sense at 





ST





Sensing Threshold





Request Required 





yes





Channel Occupied





DS(T)=X





(DG =1





(DG =1 ?)





Channel Protected by DB?





DB Query





L





Probability of False Negatives


x = received power 





� EMBED Equation.3  ���





Eqn. 2S





Eqn. 2M





� EMBED Equation.3  ���








� The average is calculated as � EMBED Equation.3  ���.





[image: image31.wmf]117

127

115

.

12

095

.

117

1

127

134

05

.

-

£

£

-

+

=

-

>

=

-

<

<

-

=

x

if

x

PROB

dBm

x

if

PROB

dBm

x

if

PROB

measured

[image: image32.wmf]dBm

x

if

x

PROB

dBm

x

if

PROB

dBm

x

if

PROB

simulated

123

133

685

.

12

095

.

123

1

133

140

05

.

-

£

£

-

+

=

-

>

=

-

<

<

-

=

[image: image33.wmf]dBm

x

if

x

P

dBm

x

if

P

dBm

x

if

P

simulated

fn

fn

fn

123

133

685

.

11

095

.

123

0

133

140

95

.

-

£

£

-

-

-

=

-

>

=

-

<

<

-

=

[image: image34.wmf]117

127

115

.

11

095

.

117

0

127

140

95

.

-

£

£

-

-

-

=

-

>

=

-

<

<

-

=

x

if

x

P

dBm

x

if

P

dBm

x

if

P

measured

fn

fn

fn

[image: image35.wmf][image: image36.png]Efficiency (dB)
4 & o IS & o - °

~

25

3

35 4 a5
Frequency (GHz)

5

55




[image: image37.wmf]dBm

x

if

x

PROB

dBm

x

if

PROB

dBm

x

if

PROB

simulated

123

133

685

.

12

095

.

123

1

133

140

05

.

-

£

£

-

+

=

-

>

=

-

<

<

-

=

[image: image38.wmf]117

127

115

.

12

095

.

117

1

127

134

05

.

-

£

£

-

+

=

-

>

=

-

<

<

-

=

x

if

x

PROB

dBm

x

if

PROB

dBm

x

if

PROB

measured

[image: image39.wmf]dBm

x

if

x

P

dBm

x

if

P

dBm

x

if

P

simulated

fn

fn

fn

123

133

685

.

11

095

.

123

0

133

140

95

.

-

£

£

-

-

-

=

-

>

=

-

<

<

-

=

_1369001223.unknown

_1369004949.unknown

_1369040635.unknown

_1369004665.unknown

_1368998120.unknown

_1369001044.unknown

_1369000153.unknown

_1368993163.unknown

_1368993257.unknown

