| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 1B/TEMP/77 (edited) | Document 1/149-E |
|  | 31 May 2011 |
|  | English only |
| Working Party 1B |  |
| DRAFT REVISION OF REPORT ITU-R SM.2153-1* |  |
| Technical and operating parameters and spectrum use for short-range radiocommunication devices** |  |

(2009-2010)

TABLE OF CONTENTS

Page

1	Introduction		6

2	Definition of short-range radio devices		6

3	Applications		7

3.1	Telecommand		7

3.2	Telemetry		7

3.3	Voice and video		7

3.4	Equipment for detecting avalanche victims		7

3.5	Broadband radio local area networks		7

3.6	Railway applications		7

3.7	Road transport and traffic telematics		8

3.8	Equipment for detecting movement and equipment for alert		8

3.9	Alarms		8

3.10	Model control		9

3.11	Inductive applications		9

Page

3.12	Radio microphones		9

3.13	RF identification systems		9

3.14	Ultra low power active medical implant		10

3.15	Wireless audio applications		10

3.16	RF (radar) level gauges		10

4	Technical standards/regulations		11

5	Common frequency ranges		11

6	Radiated power or magnetic or electric field strength		12

6.1	European Conference of Postal and Telecommunications Administrations 
member countries		13

6.2	United States of America Federal Communications Commission (FCC), 
Brazil and Canadian general limits		16

6.3	Japan		17

6.4	The Republic of Korea		17

7	Antenna requirements		17

8	Administrative requirements		18

8.1	Certification and verification		18

8.2	Licensing requirements		20

8.3	Mutual agreements between countries/regions		20

9	Additional applications		21

Annex 1 – Additional applications		22

1	SRDs operating in the 57-64 GHz band		22

2	RF level gauges		23

2.1	Pulsed systems		23

2.2	FMCW systems		23

2.3	RF level gauge operating parameters and spectrum use		24

Annex 2		24

Appendix 1 to Annex 2  (Region 1; CEPT Countries) – Technical and operating 
parameters and spectrum use for SRDs		24

1	Recommendation CEPT/ERC/REC 70-03		24

Page

2	Applications and frequency bands		25

3	Technical requirements		30

3.1	ETSI standards		30

3.2	EMC and safety		31

3.3	National type approval specifications		31

4	Additional spectrum use		31

4.1	Radiated power or magnetic field strength		31

4.2	Transmitter antenna source		32

4.3	Channel spacing		32

4.4	Duty cycle categories		32

5	Administrative requirements		33

5.1	Licensing requirements		33

5.2	Conformity assessment, marking requirements and free circulation		33

6	Operating parameters		34

7	The R&TTE Directive		34

34

Appendix 2 to Annex 2  (United States of America) – Understanding the 
FCC rules for legal low-power, non-licensed transmitters		35

1	Introduction		35

2	Low-power, non-licensed transmitters − general approach		35

3	Definition list		36

4	Technical standards		36

4.1	Conducted emission limits		36

4.2	Radiated emission limits		37

5	Antenna requirement		45

6	Restricted bands		45

7	Equipment authorization		46

7.1	Certification		47

7.2	Verification		48

Page

8	Special cases		48

8.1	Cordless telephones		48

8.2	Tunnel radio systems		48

8.3	Home-built transmitters that are not for sale		49

9	Commonly asked questions		49

9.1	What happens if one sells, imports or uses non-compliant 
low-power transmitters?		49

9.2	What changes can be made to an FCC-authorized device without 
requiring a new FCC authorization?		49

9.3	What is the relationship between V/m and W?		50

Appendix 3 to Annex 2  (People’s Republic of China) – Provisions and 
technical parameters requirements for SRDs in China		51

1	Technical parameters requirements		51

1.1	Analogue cordless telephone		51

1.2	Wireless audio transmitters and measuring devices for civilian purposes		51

1.3	Model and toy remote-control devices		52

1.4	Citizen band private mobile radio equipment		52

1.5	General radio remote-control devices		53

1.6	Biomedical telemetry transmitters		53

1.7	Equipment for lifting		53

1.8	Equipment for weighing		53

1.9	Radio remote-control equipment used in industry		53

1.10	Equipment for transporting data		54

1.11	Radio control devices for civilian purposes		54

1.12	Other SRDs		54

1.13	Digital cordless telephone		55

1.14	Automotive radars (collision avoidance radars)		55

2	Operating parameters requirements		55

3	General requirements		57

3.1	Frequency ranges of measurement for radiated spurious emissions		57

3.2	Radiated spurious emission limits		58

Page

Appendix 4 to Annex 2  (Japan) – Japanese requirements for short-range 
radio devices		59

1	Radio stations emitting extremely low power		59

2	Low-power radio stations		60

Appendix 5 to Annex 2  (The Republic of Korea) – Technical parameters and 
spectrum use for SRDs in Korea		67

1	Introduction		67

2	Technical parameters and spectrum use for SRDs		68

2.1	Low-power devices and specific SRDs		68

2.2	Measurement instruments		73

2.3	Receiver		73

2.4	Radio equipment used for relaying public radiocommunication service 
or broadcasting service to shaded area		73

2.5	Measurement instruments		73

2.6	Receiver		73

2.7	Radio equipment used for relaying public radiocommunication service 
or broadcasting service to shadowed area		74

Appendix 6 to Annex 2  (Federative Republic of Brazil) – Regulation on 
restricted radiation radiocommunications  equipment in Brazil		74

1	Introduction		74

2	Definitions		75

3	General conditions		76

4	Restricted frequency bands		77

5	General emission limits		78

6	Exception or exclusions from the general limits		79

7	Certification and authorization procedures		85

7.1	Authorization validity and procedure		85

7.2	Authorization		86

Appendix 7 to Annex 2 – UAE Regulations for the use of SRDs and low power 
equipment permitted usage		87

Appendix 8 to Annex 2 – Technical parameters and spectrum use for SRDs in 
the Regional Commonwealth in the Field of Communications countries		90

Appendix 9 to Annex 2 – Technical parameters and spectrum use for SRDs in 
some APT member countries/territories (Brunei Darussalam, China (Hong Kong), 
Malaysia, Philippines, New Zealand, Singapore and Vietnam)		111

## 6.1	European Conference of Postal and Telecommunications Administrations member countries

## Radiated power and magnetic or electric field-strength limits for SRDs in CEPT countries can be found amongst frequency bands and other parameters in Table 9, Appendix 1 to Annex 2 of this report.

| - |  |
| --- | --- |

## 6.2	United States of America Federal Communications Commission (FCC), Brazil and Canadian general limits

TABLE 2

General limits for any intentional transmitter

## 6.3	Japan

TABLE 3

Tolerable value of electric field strength 3 m distant
from a radio station emitting extremely low power

## 6.4	The Republic of Korea

TABLE 4

The limit of electric field strength of the low power device

# 8	Administrative requirements

## 8.1	Certification and verification

### 8.1.1	CEPT countries

CEPT countries that are not EU/EFTA member states and have not implemented the radio and telecommunications terminal equipment (R&TTE) Directive, have national regulations and use specifications for radio equipment which are based on transposed ENs or still in some cases based on their predecessors as CEPT Recommendations or fully national standards.

Within European Union and European Free Trade Association (EFTA) member states, the radio and telecommunications terminal equipment (R&TTE) Directive now defines the rules for placing on the market and putting into service most products using the radio frequency spectrum. Each national authority is responsible for transposing the provisions of the R&TTE Directive into its legislation.

The easiest route for a manufacturer to demonstrate compliance with the R&TTE Directive is to comply with a relevant harmonized standards which, for spectrum aspects, are developed by ETSI. It is now possible to send notifications of the intention to place equipment on the market electronically using a one-stop procedure to a number of spectrum authorities simultaneously.

The purpose of marking equipment is to indicate its conformance to relevant European Union (EU) Directives.

### 8.1.2	United States of America FCC

TABLE 5

Authorization procedures for Part 15 transmitters

## 8.2	Licensing requirements

Licensing is an appropriate tool for administrations to regulate the efficient use of the frequency spectrum.

There is a general agreement that when the efficient use of the frequency spectrum is not at risk and as long as harmful interference is unlikely, the installation and use of the spectrum or radio equipment may be exempt from a general licence or an individual licence.

SRDs are generally exempt from individual licensing. However, exceptions may be made based on national regulations.

When radio equipment is subject to an exemption from individual licensing, generally speaking, anyone can buy, install, possess and use the radio equipment without any prior permission from the administration. Administrations will not register the individual equipment but the use of the

equipment can be subject to national provisions. Furthermore, the sale and possession of some short-range radiocommunication equipment such as ultra low power active medical implant devices may be controlled by either the manufacturer or the national administration.

Annex 1

Additional applications

# 2	RF level gauges

The operating parameters and spectrum uses of RF level gauges which are in operation today throughout the world are indicated in Tables 6 to 8.

## 2.1	Pulsed systems

Pulsed systems are low cost and have low power consumption. Today they operate at 5.8 GHz which is the centre frequency of the ISM allocation. However, manufacturers are expecting products in the 10 GHz, 25 GHz, and 76 GHz ranges. The exact frequency of operation will depend on a particular product. Typical characteristics are in Table 6.

TABLE 6

## 2.2	FMCW systems

This type of system is well developed. The FMCW is robust and uses advanced signal processing which provides good reliability. The characteristics of

FMCW systems are in Table 7.

TABLE 7

## 2.3	RF level gauge operating parameters and spectrum use

TABLE 8

Annex 2

This annex provides information on national/regional rules which contain technical and operational parameters and spectrum use. Those are given in Appendices 1 through 7 to this annex.

Appendix 1
to Annex 2

(Region 1; CEPT Countries)

Technical and operating parameters
and spectrum use for SRDs

# 2	Frequency bands and corresponding parameters

The following  frequency bands and parameters are covered by annexes of Recommendation CEPT/ERC/REC 70-03 which can be downloaded from the website of the European Communications Office: (http://www.cept.org/eco). It should be remembered that it represents the most widely accepted position within the CEPT member states but it should not be assumed that all frequency allocations are available in all countries.

|  |  | ≤ |
| --- | --- | --- |

|  |  | ≤ |
| --- | --- | --- |

## 9.1	Non-specific short-range devices

Table 9a covers frequency bands and regulatory as well as informative parameters recommended primarily for telemetry, telecommand, alarms and data in general and other similar applications. Video applications should be preferably used above 2.4 GHz.

This table also includes references to the generic UWB regulation which was primarily developed to allow communication applications using UWB technology in bands below 10.6 GHz, but also enables other types of radio applications.

## TABLE 9a

Regulatory parameters

| Frequency band |  | Power/magnetic field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 6 765-6 795 kHz | 42 dBµA/m at 10 m | No requirement | No spacing |  |  |
| b | 13.553-13.567 MHz | 42 dBµA/m at 10m | No requirement | No spacing |  |  |
| c | 26.957-27.283 MHz | 42 dBµA/m at 10 m 10 mW e.r.p | No requirement | No spacing | ERC/DEC/(01)02 |  |
| Frequency band |  | Power/magnetic field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| d | 40.660-40.700 MHz | 10 mW e.r.p. | No requirement | No spacing | ERC/DEC/(01)03 |  |
| e | 138.20-138.45 MHz | 10 mW e.r.p. | < 1.0% (Note 1) | No spacing |  |  |
| f | 433.050-434.790 MHz (Note 4) | 10 mW e.r.p. | < 10% (Note 1) | No spacing |  |  |
| f1 | 433.050-434.790 MHz (Note 4bis) | 1 mW e.r.p. −13 dBm/10 kHz | No requirement | No spacing |  | Power density limited to −13 dBm/10 kHz for wideband modulation with a bandwidth greater than 250 kHz |
| f2 | 434.040-434.790 MHz (Note 4bis) | 10 mW e.r.p. | No requirement | Up to 25 kHz |  |  |
| g | 863-870 MHz (Notes 3, 4 and 6) | ≤ 25 mW e.r.p. | ≤ 0.1% or LBT (Notes 1 and 5) | ≤ 100 kHz for 47 or more channels (Note 2) |  | FHSS modulation |
|  |  | ≤ 25 mW e.r.p. (Note 6 ) Power density: −4.5 dBm/100 kHz (Note 7) | ≤ 0.1% or LBT+AFA (Notes 1, 5 and 6) | No spacing |  | DSSS and other wideband modulation other than FHSS |
|  |  | ≤ 25 mW e.r.p. | ≤ 0.1% or LBT+AFA (Notes 1 and 5) | ≤ 100 kHz, for 1 or more channels modulation bandwidth ≤ 300 kHz (Note 2) |  | Narrow-/wideband modulation |
| g1 | 868.000-868.600 MHz (Note 4) | ≤ 25 mW e.r.p. | ≤ 1% or LBT+AFA (Note 1) | No spacing, for 1 or more channels (Note 2) |  | Narrow-/wideband modulation. No channel spacing, however the whole stated frequency band may be used |
| g2 | 868.700-869.200 MHz (Note 4) | ≤ 25 mW e.r.p. | ≤ 0.1% or LBT+AFA (Note 1) | No spacing, for 1 or more channels (Note 2) |  | Narrow-/wideband modulation. No channel spacing, however the whole stated frequency band may be used |

| Frequency band |  | Power/magnetic field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| g3 | 869.400-869.650 MHz | ≤ 500 mW e.r.p. | ≤ 10% or LBT+AFA (Note 1) | 25 kHz (for 1 or more channels) |  | Narrow-/wideband modulation. The whole stated frequency band may be used as 1 channel for high speed data transmission |
| g4 | 869.700-870.000 MHz (Note 4bis) | ≤ 5 mW e.r.p. ≤ 25 mW e.r.p. | No requirement up to 1% or LBT+AFA (Note 1) | No spacing (for 1 or more channels) |  | Narrow-/wideband modulation. No channel spacing, however the whole stated frequency band may be used |
| h | 2 400.0-2 483.5 MHz | 10 mW e.i.r.p. | No requirement | No spacing |  |  |
| i | 5 725-5 875 MHz | 25 mW e.i.r.p. | No requirement | No spacing |  |  |
| j | 24.00-24.25 GHz | 100 mW e.i.r.p. | No requirement | No spacing |  |  |
| k | 61.0-61.5 GHz | 100 mW e.i.r.p. | No requirement | No spacing |  |  |
| l | 122-123 GHz | 100 mW e.i.r.p. | No requirement | No spacing |  |  |
| m | 244-246 GHz | 100 mW e.i.r.p. | No requirement | No spacing |  |  |
| n | 3.1-4.8 GHz 6-9 GHz | * | * | * | ECC/DEC/(06)04 ECC/DEC/(06)12 | Generic UWB regulation * See detailed requirements in related ECC Decisions |
| NOTE 1 - When either a duty cycle, Listen Before Talk (LBT) or equivalent technique applies then it shall not be user dependent/adjustable and shall be guaranteed: by appropriate technical means; for LBT devices without Adaptive Frequency Agility (AFA), or equivalent techniques, the duty cycle limit applies; for any type of frequency agile device the duty cycle limit applies to the total transmission unless LBT or equivalent technique is used. NOTE 2 - The preferred channel spacing is 100 kHz allowing for a subdivision into 50 kHz or 25 kHz. NOTE 3 - Sub-bands for alarms are excluded (see Table 9 for alarms). NOTE 4 - Audio and video applications are allowed provided that a digital modulation method is used with a max. bandwidth of 300 kHz. Analogue and digital voice applications are allowed with a max. bandwidth ≤ 25 kHz. In sub-band 863-865 MHz voice and audio conditions of Annexes 10 and 13 of ERC/REC 70 – 03 apply respectively. NOTE 4bis - Audio and video applications are excluded. Voice applications (analogue or digital) are allowed with a max. bandwidth ≤ 25 kHz, spectrum access technique such as LBT or equivalent and a power output sensor controlling the transmitter to a maximum transmit period of 1 minute for each transmission. NOTE 5 - Duty cycle may be increased to 1% if the band is limited to 865-868 MHz. NOTE 6 - For other wideband modulation than FHSS and DSSS with a bandwidth of 200 kHz to 3 MHz, duty cycle can be increased to 1% if the band is limited to 865-868 MHz and power to ≤ 10 mW e.r.p. NOTE 7 - The power density can be increased to +6.2 dBm/100 kHz and −0.8 dBm/100 kHz, if the band of operation is limited to 865-868 MHz and 865-870 MHz respectively. |  |  |  |  |  |  |

## 9.2	Tracking, tracing and data acquisition

Table 9b covers frequency bands and regulatory as well as informative parameters recommended for a number of specific devices including:

•	detecting avalanche victims;

•	meter reading;

•	asset tracking and tracing.

TABLE 9b

Regulatory parameters

| Frequency band |  | Power/magnetic field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 456.9-457.1 kHz | 7 dBµA/m at 10 m | No requirement | Continuous wave (CW) – no modulation. | ECC/DEC/(04)01 | Detection of avalanche victims. Note: Centre frequency is 457 kHz |
| b | 169.4-169.475 MHz | 500 mW e.r.p. | < 10% duty cycle | Max 50 kHz | ECC/DEC/(05)02 | Meter reading |
| c | 169.4-169.475 MHz | 500 mW e.r.p. | < 1% duty cycle | Max 50 kHz | ECC/DEC/(05)02 | Asset tracking and tracing |

## 9.3	Wideband data transmission systems

Table 9c covers frequency bands and regulatory as well as informative parameters recommended for wideband data transmission systems and wireless access systems including radio local area networks (WAS/RLANs) (formerly known as Radio Local Area Networks (RLANs)) within the band 2 400-2 483.5 MHz, for wireless access systems including radio local area networks (WAS/RLANs) within the bands 5 150-5 350 MHz, 5 470-5 725 MHz and 17.1-17.3 GHz, and for multiple-gigabit WAS/RLAN systems within the band 57-66 GHz.

TABLE 9c

Regulatory parameters

| Frequency band |  | Power/magnetic field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 2 400.0-2 483.5 MHz | 100 mW e.i.r.p. | See Note 1 | No spacing | ERC/DEC/(01)07 | For wide band modulations other than FHSS , the maximum e.i.r.p. density is limited to 10 mW/MHz. |
| b | 5 150-5 350 MHz | 200 mW mean e.i.r.p. See Note 3 | See Notes 1 and 2 | No spacing | ECC/DEC/(04)08 | Restricted to indoor use. The maximum mean e.i.r.p. density shall be limited to 10 mW/MHz in any 1 MHz band. |
| c | 5 470-5 725 MHz | 1 W mean e.i.r.p. See Note 3 | See Notes 1 and 2 | No spacing | ECC/DEC/(04)08 | Indoor as well as outdoor use allowed. The maximum mean e.i.r.p. density shall be limited to 50 mW/MHz in any 1 MHz band. |
| d | 17.1-17.3 GHz | 100 mW e.i.r.p. | No requirement | No spacing |  |  |
| e | 57-66 GHz | 40 dBm mean e.i.r.p | See Note 1 | No spacing |  | Fixed outdoor installations are not allowed. The maximum mean e.i.r.p density is limited to 13 dBm/MHz. |
| NOTE 1 - The equipment shall implement an adequate spectrum sharing mechanism in order to facilitate sharing between the various technologies and applications. NOTE 2 - WAS/RLANs operating in the bands 5 250-5 350 MHz and 5 470-5 725 MHz shall use mitigation techniques that give at least the same protection as the detection, operational and response requirements described in ETSI EN 301 893 to ensure compatible operation with radiodetermination systems (radars). Such mitigation techniques shall equalise the probability of selecting a specific channel for all available channels so as to ensure, on average, a near-uniform spread of spectrum loading. Specific information about the applicability of ETSI EN 301 893 can be found at http://ec.europa.eu/comm/enterprise/rtte/harstand.htm. NOTE 3 - WAS/RLANs operating in the bands 5 250-5 350 MHz and 5 470-5 725 MHz shall employ transmitter power control (TPC), which provides, on average, a mitigation factor of at least 3 dB on the maximum permitted output power of the systems. If transmitter power control is not in use, the maximum permitted mean e.i.r.p. and the corresponding mean e.i.r.p. density limits shall be reduced by 3 dB. |  |  |  |  |  |  |

## 9.4	Railway applications

Table 9d covers frequency bands and regulatory as well as informative parameters recommended for applications specifically intended for use on railways.

The sub-bands below are intended for the following applications:

–	band a): Automatic vehicle identification systems for railways including automatic vehicle identification for railways (AVI);

–	band b): Balise tele-powering and downlink (train to ground) systems including Eurobalise and activation of the Loop/Euroloop;

–	band c): Balise uplink (ground to train) systems including Eurobalise;

–	bands d1) and d2): Loop uplink (ground to train) systems including Euroloop.

TABLE 9d

Regulatory parameters

| Frequency band |  | Power/magnetic field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 2 446-2 454 MHz | 500 mW e.i.r.p. | No requirement |  |  | Transmitting only in presence of trains. 5 channels, each 1.5 MHz wide within the band 2 446-2 454 MHz. |
| b | 27.090-27.100 MHz | 42 dBµA/m at 10 m | No requirement | No spacing |  | Tele-powering and downlink signal for Balise/Eurobalise. May also be optionally used for the activation of the Loop/Euroloop. NOTE - Centre frequency is 27.095 MHz. |
| c | 984-7 484 kHz | 9 dBµA/m at 10 m | < 1% duty cycle | No spacing |  | Transmitting only on receipt of a Balise/Eurobalise tele-powering signal from a train. NOTE - Centre frequency is 4 234 kHz. |
| d1 | 516-8 516 kHz | 7 dBµA/m at 10 m | No requirement | No spacing |  | Not intended for new applications, existing applications to be phased out by 2010. NOTE - Centre frequency is 4 516 kHz. |
| d2 | 7.3-23.0 MHz | −7 dBµA/m at 10 m | No requirement | No spacing |  | Maximum field strength specified in a bandwidth of 10 kHz, spatially averaged over any 200 m length of the loop. Transmitting only in presence of trains. Spread spectrum signal, code length: 472 chips. Note: Centre frequency is 13.547 MHz. |

## 9.5 	Road transport and traffic telematics (RTTT)

Table 9e covers frequency bands and regulatory as well as informative parameters recommended for road transport and traffic telematics (RTTT) including radar system installations to be used in vehicles.

TABLE 9e

Regulatory parameters

| Frequency band |  | Power/magnetic field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 5 795-5 805 MHz | 2 W e.i.r.p. 8 W e.i.r.p. | No requirement |  | ECC/DEC/(02)01 |  |
| b | 5 805-5 815 MHz | 2 W e.i.r.p. 8 W e.i.r.p. | No requirement |  | ECC/DEC/(02)01 | Individual license required. |
| c | 63-64 GHz |  |  | No spacing | ECC/DEC/(02)01 | Vehicle to vehicle and road to vehicle systems. Power level and spectrum access and mitigation requirement to be determined. |
| d | 76-77 GHz | 55 dBm peak e.i.r.p. | No requirement | No spacing | ECC/DEC/(02)01 | Power level 55 dBm peak power e.i.r.p. 50 dBm average power −23.5 dBm average power for pulse radar only. Vehicle and infrastructure radar systems. |
| e | 21.65-26.65 GHz | * | * | * | ECC/DEC/(04)10 | For automotive short-range radars (SRR) * See detailed requirements in related ECC Decision. New SRR equipment may only be placed on the market until 1 July 2013. |
| f | 77-81 GHz | * | * | * | ECC/DEC/(04)03 | For automotive short-range radars (SRR). * See detailed requirements in related ECC Decision. |
| g1 | 24.050-24.075 GHz | 100 mW e.i.r.p. | No requirement |  |  | For vehicle radars |
| g2 | 24.075-24.150 GHz | 0.1 mW e.i.r.p. | No requirement |  |  | For vehicle radars |
|  |  | 100 mW e.i.r.p. | 4 µs/40 kHz dwell time every 3 ms (Note 1) |  |  | For vehicle radars. The spectrum access and mitigation requirement is given for devices mounted behind a bumper. If mounted without a bumper, the requirement should be 3 µs/40 kHz maximum dwell time every 3 ms. |
|  |  |  | 1 ms/40 kHz dwell time every 40 ms (Note 1) |  |  | The spectrum access and mitigation requirement is given for devices mounted either behind a bumper or mounted without a bumper. |
| g3 | 24.150-24.250 GHz | 100 mW e.i.r.p. | No requirement |  |  | For vehicle radars |
| NOTE 1 - A requirement for minimum frequency modulation range (applicable to FMCW or step frequency signals) or minimum instantaneous bandwidth (applicable to pulsed signal) of 250 kHz applies in addition to the requirement on maximum dwell time. |  |  |  |  |  |  |

## 9.6 	Radiodetermination applications

Table 9f covers frequency bands and regulatory as well as informative parameters recommended for SRD radiodetermination applications including SRD radar systems, equipment for detecting movement and alert. Radiodetermination is defined as the determination of the position, velocity and/or other characteristics of an object, or the obtaining of information relating to these parameters, by means of the propagation properties of radio waves.

TABLE 9f

Regulatory parameters

| Frequency band |  | Power/magnetic field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 2 400.0-2 483.5 MHz | 25 mW e.i.r.p. | No requirement | No spacing | ERC/DEC/(01)08 |  |
| b | 9 200-9 500 MHz | 25 mW e.i.r.p. | No requirement | No spacing |  |  |
| c | 9 500-9 975 MHz | 25 mW e.i.r.p. | No requirement | No spacing |  |  |
| d | 10.5-10.6 GHz | 500 mW e.i.r.p. | No requirement | No spacing |  |  |
| e | 13.4-14.0 GHz | 25 mW e.i.r.p. | No requirement | No spacing |  |  |
| f | 24.05-24.25 GHz | 100 mW e.i.r.p. | No requirement | No spacing |  |  |
| g | 4.5-7.0 GHz | −41.3 dBm/MHz e.i.r.p. | No requirement | No spacing |  | Tank level probing radar (TLPR). |
| h | 8.5-10.6 GHz | −41.3 dBm/MHz e.i.r.p. | No requirement | No spacing |  | Tank level probing radar (TLPR). |
| i | 24.05-27.00 GHz | −41.3 dBm/MHz e.i.r.p. | No requirement | No spacing |  | Tank level probing radar (TLPR). |
| j | 57-64 GHz | −41.3 dBm/MHz e.i.r.p. | No requirement | No spacing |  | Tank level probing radar (TLPR). |
| k | 75-85 GHz | −41.3 dBm/MHz e.i.r.p. | No requirement | No spacing |  | Tank level probing radar (TLPR). |
| l | 17.1-17.3 GHz | +26 dBm e.i.r.p. | DAA | No spacing |  | Ground-based synthetic aperture radar (GBSAR) (Note 1). |
| m | 30 MHz – 12.4 GHz | * | * | * | ECC/DEC/(06)08 | For ground- and wall- probing radar (GPR/WPR) imaging systems, subject to an appropriate licensing regime. * See detailed requirements in related ECC Decision. |
| n | 2.2-8 GHz | * | * | * | ECC/DEC/(07)01 | For building material analysis (BMA) devices. * See detailed requirements in related ECC Decision.. |
| NOTE 1 - Specific requirements for the radar antenna pattern and for the implementation of detect and avoid (DAA) technique apply as described in ETSI EN 300 440 for ground-based synthetic aperture radar (GBSAR) systems. |  |  |  |  |  |  |

## 9.7	Alarms

Table 9g covers frequency bands and regulatory as well as informative parameters recommended exclusively for alarm systems including social alarms and alarms for security and safety.

The sub-bands below are intended for the following applications:

–	alarms in general: bands a), b) ,c) and e)

–	social alarms: bands d), f) and g).

TABLE 9g

Regulatory parameters

| Frequency band |  | Power/magnetic field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 868.6-868.7 MHz | 10 mW e.r.p. | < 1.0% duty cycle | 25 kHz |  | The whole frequency band may also be used as 1 channel for high speed data transmissions. |
| b | 869.250-869.300 MHz | 10 mW e.r.p. | < 0.1% duty cycle | 25 kHz |  |  |
| c | 869.650-869.700 MHz | 25 mW e.r.p. | < 10% duty cycle | 25 kHz |  |  |
| d | 869.200-869.250 MHz | 10 mW e.r.p. | < 0.1% duty cycle | 25 kHz |  | Social alarms |
| e | 869.300-869.400 MHz | 10 mW e.r.p. | < 1.0% duty cycle | 25 kHz |  |  |
| f | 169.4750-169.4875 MHz | 10 mW e.r.p. | < 0.1% duty cycle | 12.5 kHz | ECC/DEC/(05)02 | Social alarms (exclusive use) |
| g | 169.5875-169.6000 MHz | 10 mW e.r.p. | < 0.1% duty cycle | 12.5 kHz | ECC/DEC/(05)02 | Social alarms (exclusive use) |

## 9.8 	Model control

Table 9h covers frequency bands and regulatory as well as informative parameters recommended for the application of model control equipment, which is solely for the purpose of controlling the movement of the model, in the air, on land or over or under the water surface. Although the bands are not harmonized, the parameters given in the table are common in a majority of CEPT countries. It should be noted that the bands are not exclusive for this type of application.

TABLE 9h

Regulatory parameters

| Frequency band |  | Power/magnetic field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 26.995, 27.045, 27.095, 27.145, 27.195 MHz | 100 mW e.r.p | No requirement | 10 kHz | ERC/DEC/(01)10 |  |
| b | 34.995-35.225 MHz | 100 mW e.r.p | No requirement | 10 kHz | ERC/DEC/(01)11 | Only for flying models |
| c | 40.665, 40.675, 40.685, 40.695 MHz | 100 mW e.r.p | No requirement | 10 kHz | ERC/DEC/(01)12 |  |

## 9.9	Inductive applications

Table 9i covers frequency bands and regulatory as well as informative parameters recommended for inductive applications include for example car immobilizers, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, data transfer to handheld devices, automatic article identification, wireless control systems, automatic road tolling and anti-theft systems including RF anti-theft induction systems. It should be noted that other types of anti-theft systems can be operated in accordance with other relevant annexes.

TABLE 9i

Regulatory parameters

| Frequency band |  | Power/magnetic field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a1 | 9-90 kHz | 72 dBµA/m at 10 m (Note 1) | No requirement | No spacing |  | In case of external antennas only loop coil antennas may be employed. Field-strength level descending 3 dB/oct at 30 kHz. |
| a2 | 90-119 kHz | 42 dBµA/m at 10 m | No requirement | No spacing |  | In case of external antennas only loop coil antennas may be employed. |
| a3 | 119-135 kHz | 66 dBµA/m at 10 m (Note 1) | No requirement | No spacing |  | In case of external antennas only loop coil antennas may be employed. Field-strength level descending 3 dB/oct at 119 kHz. |
| b | 135-140 kHz | 42 dBµA/m at 10 m | No requirement | No spacing |  | In case of external antennas only loop coil antennas may be employed. |
| c | 140-148.5 kHz | 37.7 dBµA/m at 10 m | No requirement | No spacing |  | In case of external antennas only loop coil antennas may be employed. |
| d | 6 765-6 795 kHz | 42 dBµA/m at 10 m | No requirement | No spacing |  |  |
| e | 7 400-8 800 kHz | 9 dBµA/m at 10 m | No requirement | No spacing |  |  |
| f | 13.553-13.567 MHz | 42 dBµA/m at 10 m | No requirement | No spacing |  |  |
| f1 | 13.553-13.567 MHz | 60 dBµA/m at 10 m | No requirement | No spacing |  | For RFID and EAS only. |
| g | 26.957-27.283 MHz | 42 dBµA/m at 10 m | No requirement | No spacing | ERC/DEC/(01)16 |  |
| h | 10.200-11.000 MHz | 9 dBµA/m at 10 m | No requirement | No spacing |  |  |
| k | 3 155-3 400 kHz | 13.5 dBµA/m at 10 m | No requirement | No spacing |  | In case of external antennas only loop coil antennas may be employed. |
| l1 | 148.5 kHz - 5 MHz | −15 dBµA/m at 10 m | No requirement | No spacing |  | In case of external antennas only loop coil antennas may be employed. The maximum field strength is specified in a bandwidth of 10 kHz. The maximum allowed total field strength is −5 dBµA/m at 10 m for systems operating at bandwidths larger than 10 kHz whilst keeping the density limit (−15 dBµA/m in a bandwidth of 10 kHz). |
| l2 | 5-30 MHz | −20 dBµA/m at 10 m | No requirement | No spacing |  | In case of external antennas only loop coil antennas may be employed. The maximum specified in a bandwidth of 10 kHz. The maximum allowed total field strength is −5 dBµA/m at 10 m for systems operating at bandwidths larger than 10 kHz whilst keeping the density limit (−20 dBµA/m in a bandwidth of 10 kHz). |
| l3 | 400-600 kHz | −8 dBµA/m at 10 m | No requirement | No spacing |  | For RFID only. In case of external antennas only loop coil antennas may be employed. The maximum field strength is specified in a bandwidth of 10 kHz. The maximum allowed total field strength is −5 dBµA/m at 10 m for systems operating at bandwidths larger than 10 kHz measured at the centre frequency whilst keeping the density limit (−8 dBµA/m in a bandwidth of 10 kHz.) These systems should operate with a minimum operating bandwidth of 30 kHz |
| NOTE 1 - Limit is reduced to 42 dBµA/m at 10 m. Station; Frequency; Protection bandwidth; Maximum field strength at 10 m; Location. MSF; 60 kHz; ±250 Hz; 42 dBµA/m; United Kingdom. RBU; 66.6 kHz; ±750 Hz; 42 dBµA/m; Russian Federation. HBG; 75 kHz; ±250 Hz; 42 dBµA/m; Switzerland. DCF77; 77.5 kHz; ±250 Hz; 42 dBµA/m; Germany. DCF49; 129.1 kHz; ±500 Hz; 42 dBµA/m; Germany. Standard frequency and time signals to be protected within 9-90 kHz and 119-135 kHz. |

## 9.10 	Radio microphone applications including aids for the hearing impaired

Table 9j covers frequency bands and regulatory as well as informative parameters recommended for radio microphone applications (also referred to as wireless microphones or cordless microphones) including aids for the hearing impaired (also referred to as assistive listening devices). Radio microphones are small, low power (typically 50 mW or less) transmitters designed to be worn on the body, or hand-held, for the transmission of sound. The receivers are more tailored to specific uses and may range from small and portable to rack-mounted modules as part of a multichannel system. This annex covers professional and consumer radio microphones, both hand-held and body-worn, and aids for the hearing impaired.

Because of the difficulty in determining harmonised frequency bands for radio microphones, frequency band limits should be regarded as tuning ranges within which a device can be designated to operate. In most cases, parts of the  range that are not available in individual countries but this does not apply to the broadcasting bands at 174-216 MHz and 470-862 MHz where national geographical and licensing restrictions are likely to exist and the national administration should be contacted.

The sub-bands below are intended for the following applications:

–	aids for the hearing impaired: sub-bands b), c), d), h1), h2), i);

–	radio microphones: sub-bands a), c), d), e1), e2), e3), e4), f), g).

Aids for the hearing impaired are specific radio microphone applications which capture an acoustic signal that is transmitted by radio to the hearing aid receivers.

TABLE 9j

Regulatory parameters

| Frequency band |  | Power/magnetic field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 29.7-47.0 MHz | 10 mW e.r.p. | No requirement | 50 kHz |  | On a tuning range basis The frequency bands 30.3-30.5 MHz, 32.15-32.45 MHz and 41.015-47.00 MHz are harmonized military bands. |
| b | 173.965-174.015 MHz | 2 mW e.r.p. | No requirement | 50 kHz |  | Aids for the hearing impaired. |
| c | 863-865 MHz | 10 mW e.r.p. | No requirement | No spacing |  |  |
| d | 174-216 MHz | 50 mW e.r.p. | No requirement | No spacing |  | On a tuning range basis. Individual licence required. |
| e1 | 470-786 MHz | 50 mW e.r.p. | No requirement | No spacing |  | On a tuning range basis. Individual licence required. |
| e2 | 786-789 MHz | 12 mW e.r.p. | No requirement | No spacing |  | On a tuning range basis. Individual licence required. See technical conditions for PMSE (including radio microphones) in Annex 3 of Decision ECC/DEC/(09)03 section 3.1. |
| e3 | 823-826 MHz | 20 mW e.i.r.p. 100 mW e.i.r.p. | No requirement | 200 kHz |  | Individual licence required. 100 mW restricted to body worn microphones. See technical conditions for PMSE (including radio microphones) in Annex 3 of Decision ECC/DEC/(09)03 section 3.1. |
| e4 | 826-832 MHz | 100 mW e.i.r.p. | No requirement | 200 kHz |  | Individual licence required. See technical conditions for PMSE (including radio microphones) in Annex 3 of Decision ECC/DEC/(09)03 section 3.1. |
| f | 1 785-1 795 MHz | 20 mW e.i.r.p. 50 mW e.i.r.p. | No requirement | No spacing |  | Individual licence required. 50 mW restricted to body-worn microphones. |
| g | 1 795-1 800 MHz | 20 mW e.i.r.p. 50 mW e.i.r.p. | No requirement | No spacing |  | 50 mW restricted to body-worn equipment. |
| h1 | 169.4000-169.4750 MHz | 10 mW e.r.p. | No requirement | Max 50 kHz | ECC/DEC/(05)02 | Aids for the hearing impaired. (Personal hearing aid system). |
|  |  | 500 mW e.r.p. | No requirement | Max 50 kHz | ECC/DEC/(05)02 | Aids for the hearing impaired. (Public hearing aid system) Individual licence may be required. |
| h2 | 169.4875-169.5875 MHz | 10 mW e.r.p. | No requirement | Max 50 kHz | ECC/DEC/(05)02 | Aids for the hearing impaired. (Personal hearing aid system). |
|  |  | 500 mW e.r.p. | No requirement | Max 50 kHz | ECC/DEC/(05)02 | Aids for the hearing impaired. (Public Hearing aid system). Individual licence may be required. |
| i | 169.4-174.0 MHz | 10 mW e.r.p. | No requirement | Max 50 kHz |  | Aids for the hearing impaired. On a tuning range basis Administrations should consider channel plan for band 169.4-169.8125 MHz detailed in ECC/DEC/(05)02 and the risk of interference towards systems operated in the band 169.6-169.8125 MHz when developing their national frequency table. |

## 9.11	Radio-frequency identification applications

Table 9k covers frequency bands and regulatory as well as informative parameters recommended for radio-frequency identification (RFID) applications including for example automatic article identification, asset tracking, alarm systems, waste management, personal identification, access control, proximity sensors, anti-theft systems, location systems, data transfer to handheld devices and wireless control systems. It should be noted that other types of RFID systems can be operated in accordance with other relevant annexes.

TABLE 9k

Regulatory parameters

| Frequency band |  | Power/magnetic field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a1 | 2 446-2 454 MHz | ≤ 500 mW e.i.r.p. | No requirement | No spacing |  |  |
| a2 | 2 446-2 454 MHz | > 500 mW - 4 W e.i.r.p | ≤ 15% duty cycle FHSS techniques should be used | No spacing |  | Power levels above 500 mW are restricted to be used inside the boundaries of a building and the duty cycle of all transmissions shall in this case be ≤ 15% in any 200 ms period (30 ms on /170 ms off). |
| b1 | 865.0-865.6 MHz | 100 mW e.r.p. | No requirement | 200 kHz |  |  |
| b2 | 865.6-867.6 MHz | 2 W e.r.p. | No requirement | 200 kHz |  |  |
| b3 | 867.6-868.0 MHz | 500 mW e.r.p. | No requirement | 200 kHz |  |  |

## 9.12	Active medical implants and their associated peripherals

Table 9l covers frequency bands and regulatory as well as informative parameters recommended for active medical implants and their associated peripherals.

TABLE 9l

Regulatory parameters

| Frequency band |  | Power/magnetic field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 402-405 MHz | 25 µW e.r.p. | See Note 3 | 25 kHz | ERC/DEC/(01)17 | For ultra low power active medical implants covered by the applicable harmonized standard. Individual transmitters may combine adjacent channels for increased bandwidth up to 300 kHz. |
| a1 | 401-402 MHz | 25 µW e.r.p. | LBT or duty cycle ≤ 0.1% (see note 2) | 25 kHz |  | For ultra low power active medical implants and accessories covered by the applicable harmonized standard and not covered by band a. Individual transmitters may combine adjacent 25 kHz channels for increased bandwidth up to 100 kHz (see Note 1). |
| a2 | 405-406 MHz | 25 µW e.r.p. | LBT or duty cycle ≤ 0.1% (see note 2) | 25 kHz |  | For ultra low power active medical implants and accessories covered by the applicable harmonized standard and not covered by band a. Individual transmitters may combine adjacent 25 kHz channels for increased bandwidth up to 100 kHz (see Note 1). |
| b | 9-315 kHz | 30 dBµA/m at 10 m 9-315 k | < 10% | No spacing |  | The application is for ultra low power active medical implant systems using inductive loop techniques for telemetry purposes. |
| c | 315-600 kHz | −5 dBµA/m at 10 m | < 10% | No spacing |  | The application is for animal implantable devices. |
| d | 30.0-37.5 MHz | 1 mW e.r.p. | < 10% | No spacing |  | The application is for ultra low power medical membrane implants for blood pressure measurements. |
| e | 12.5-20.0 MHz | −7 dBµA/m at 10m | < 10% duty cycle | No spacing |  | The application is for ULP active animal implantable devices (ULP-AID), limited to indoor only applications. The maximum field strength is specified in a bandwidth of 10 kHz. The transmission mask of ULP-AID is defined as follows: 3 dB bandwidth 300 kHz 10 dB bandwidth 800 kHz 20 dB bandwidth 2 MHz. |
| f | 2 483.5-2 500 MHz | 10 dBm e.i.r.p | LBT+AFA and < 10% duty cycle. See Note 3 | 1 MHz |  | For Low power active medical implants and associated peripherals, covered by the applicable harmonized standard. Individual transmitters may combine adjacent channels on a dynamic basis for increased bandwidth higher than 1 MHz. Peripheral units are for indoor use only. |
| NOTE 1 - Due to the limited available spectrum of 1 MHz, a maximum bandwidth of 100 kHz is proposed for these bands to ensure that several users could access the band concurrently. NOTE 2 - Systems not providing frequency agility based on ambient RF field sensing, be limited to a maximum permitted e.r.p. of 250 nanowatts with a duty cycle of ≤ 0.1%. NOTE 3 - The equipment shall implement a spectrum access mechanism as described in the applicable harmonized standard or an equivalent spectrum access mechanism. |  |  |  |  |  |  |

## 9.13 	Wireless audio applications

Table 9m covers frequency bands and regulatory as well as informative parameters recommended for applications for wireless audio systems including the following, cordless loudspeakers; cordless headphones; cordless headphones for portable use, for example portable CD, cassette or radio devices carried on a person; cordless headphones for use in a vehicle, for example for use with a radio or mobile telephone etc.; in-ear monitoring, for use with concerts or other stage productions.

TABLE 9m

Regulatory parameters

| Frequency band |  | Power/magnetic field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 863-865 MHz | 10 mW e.r.p. | No requirement | No spacing |  |  |
| b | 864.8-865.0 MHz | 10 mW e.r.p. | No requirement | 50 kHz |  | Narrow-band analogue voice devices |
| c | 1 795-1 800 MHz | 20 mW e.i.r.p. | No requirement | No spacing |  |  |
| d | 87.5-108.0 MHz | 50 nW e.r.p. | No requirement | 200 kHz |  |  |

# 3	Technical requirements

## 3.1	ETSI standards

The ETSI is responsible for producing harmonized standards for telecommunications and radiocommunications equipment. These standards which are used for regulative purposes are known as European Norms (prefixed with EN).

Harmonized standards for radio equipment contain requirements relating to effective use of the spectrum and avoidance of harmful interference. These can be used by manufacturers as part of the conformity assessment process. The application of harmonized standards developed by ETSI is not mandatory, however where they are not applied, a notified body must be consulted. The national standardization organizations are obliged by EU law to transpose European Standards for Telecommunications (ETSs or ENs) into national standards, and to withdraw any conflicting national standards.

With regard to SRDs, ETSI developed four generic standards (EN 300 220; EN 300 330, EN 300 440, and EN 305 550) and a number of specific standards covering specific applications. All SRDs relevant standards are listed in Appendix 2 of Recommendation CEPT/ERC/REC 70-03.

## 3.2	EMC and safety

### 3.2.1	EMC

All CEPT countries have EMC requirements, mostly based on IEC and CISPR standards or in some cases on Cenelec and ETSI EMC standards. In the EU/EFTA, the European harmonized standards from ETSI and CENELEC are the reference documents for presumption of conformity with the essential requirements of EMC Directive 2004/108/EC (most of these European standards are referred to in Recommendation CEPT/ERC/REC 70-03). The manufacturer has to affix the CE marking to his electrical products and has to have available a CE declaration, signed by himself and a technical file. He can, base those documents on a conformity investigation carried out by himself. Most European harmonized standards in the EEA are based on IEC/CISPR standards.

The CEPT countries outside the EU/EFTA mostly accept a test report from an accredited EEA laboratory in the EU/EFTA as proof of conformity. However, some request a conformity test report from one of their national laboratories.

### 3.2.2	Electrical safety

In general, the European countries have (electrical) safety requirements, based on IEC standards. In most cases IEC 60950 + amendments apply to radiocommunication equipment.

In the EEA the European harmonized standards from CENELEC are the reference documents for presumption of conformity with the essential requirements of the low voltage Directive 2006/95/EC. The most relevant European harmonized standard for radiocommunication equipment is EN 60950 + amendments, which is based on IEC 60950.

The CEPT countries outside the EU/EFTA usually require, a CB scheme certificate (international certification scheme under IECEE), granted by one of the members of the CB scheme as proof of conformity to IEC 60950.

NOTE 1 − Most customs authorities of the EU, require that equipment coming from outside the EEA, should be CE-marked for EMC and (electrical) safety and that an EC declaration of conformity (of the manufacturer) should be presented, before they grant an import licence.

## 3.3	National type approval specifications

Members of CEPT, that are no EU/ EFTA member states and have not implemented the R&TTE Directive, have national regulations, sometimes based on this Directive, and use specifications for radio equipment which are based on transposed ENs or still in some cases based on their predecessors as CEPT Recommendations or fully national standards.

# 4	Additional spectrum use

## 4.1	Radiated power or magnetic field strength

The radiated power or H-field-strength limits mentioned in Recommendation CEPT/ERC/REC 70-03 are the maximum values allowed for SRDs. The levels were determined after careful analysis within ETSI and CEPT ECC (respectively ERC) and are dependent on the frequency range and the applications chosen. The average H-field strength/power level is 5 dB(A/m) at 10 m.

## 4.4	Duty cycle categories

ETSI EN 300 220-1 defines the duty cycle as follows:

For the purposes of this present text the duty cycle is defined as the ratio, expressed as a percentage, of the maximum transmitter “on” time monitored over one hour, relative to a one-hour period. The device may be triggered either automatically or manually and depending on how the device is triggered will also depend on whether the duty cycle is fixed or random.

For automatic operated devices, either software controlled or pre-programmed devices, the provider shall declare the duty cycle class or classes for the equipment under test, see Table 10.

TABLE 10

## 5.2	Conformity assessment, marking requirements and free circulation

The purpose of marking an equipment is to indicate its conformance to relevant EC Directives, ECC or ERC Decisions or Recommendations and national regulations.

In almost 100% of cases, requirements for marking and labelling of approved and licensed equipment is set in national law. Most administrations require at least that the logo or name of the approval authority is shown on the label, along with the approval number which may also indicate the year of approval.

Recommendation CEPT/ERC/REC 70-03 recommends three different possibilities of marking and free circulation for SRDs dependent on the conformity assessment used.

For EU/EFTA member states the placing on the market and free movement of SRDs is covered by the R&TTE Directive (see § 7).

Appendix 2
to Annex 2

(United States of America)

Understanding the FCC rules for legal low-power,
non-licensed transmitters

## 4.2	Radiated emission limits

TABLE 11

General limits for any intentional transmitter

Table 12 contains exceptions or exclusions (indicated) to the general limits, otherwise the general limits can still be used.

TABLE 12

Exception or exclusions from the general limits

# 6	Restricted bands

TABLE 13

Restricted bands − spurious emissions only with limited 
exceptions (not indicated)

# 7	Equipment authorization

TABLE 14

Authorization procedures for Part 15 transmitters

Appendix 3
to Annex 2

(People’s Republic of China)

Provisions and technical parameters requirements
for SRDs in China

# 3	General requirements

## 3.1	Frequency ranges of measurement for radiated spurious emissions

TABLE 15

## 3.2	Radiated spurious emission limits

### 3.2.1	Radiated spurious emission limits are shown in the following table when a transmitter is in the state of maximum emission power

TABLE 16

### 3.2.2	Radiated spurious emission limits are shown in the following table when a transmitter is in idle or standby state

TABLE 17

Appendix 4
to Annex 2

(Japan)

Japanese requirements for short-range radio devices

# 1	Radio stations emitting extremely low power

A radio station license is not required if the electric field strength meets the tolerable maximum value shown in Fig. 1 and Table 18 at a location 3 m distant from the radio equipment.

TABLE 18

Tolerable value of electric field strength 3 m distant
from a radio station emitting extremely low power

# 2	Low-power radio stations

TABLE 19

Technical regulations for representative low-power radio stations

Appendix 5
to Annex 2

(The Republic of Korea)

Technical parameters and spectrum use
for SRDs in Korea

# 2	Technical parameters and spectrum use for SRDs

## 2.1	Low-power devices and specific SRDs

TABLE 20

## 2.4	Radio equipment used for relaying public radiocommunication service or broadcasting service to shaded area

TABLE 21

## 2.7	Radio equipment used for relaying public radiocommunication service or broadcasting service to shadowed area

TABLE 22

Appendix 6
to Annex 2

(Federative Republic of Brazil)

Regulation on restricted radiation radiocommunications 
equipment1 in Brazil

# 4	Restricted frequency bands

The use of restricted radiation equipment is prohibited in the frequency bands listed in Table 23. In these frequency bands, only spurious emissions from the restricted radiation equipment operating in another band shall be allowed.

TABLE 23

Restricted frequency bands*

# 5	General emission limits

Except when explicitly stated otherwise in Resolution No. 506 Anatel, the emissions of restricted radiation equipment shall not be greater than the field-strength levels specified in Table 24.

TABLE 24

General emission limits

TABLE 25

Field strength limits for equipment operating within bands 902-907.5 MHz, 
915-928 MHz, 2 400-2 483.5 MHz, 5 725-5 875 MHz and 24.00-24.25 GHz

# 6	Exception or exclusions from the general limits

Table 26 contains other exceptions or exclusions to the general limits in Brazil. Additionally, under special conditions telecommand systems can operate in some specific frequencies of 26 MHz, 27 MHz, 50 MHz, 71 MHz and 75 MHz bands.

TABLE 26

Exception or exclusions from the general limits

Appendix 7
to Annex 2

UAE Regulations for the use of SRDs and low power
equipment permitted usage

1.4	The wireless equipment can be identify as short-range devices, low power wireless equipment or otherwise based on the following criteria:

1.4.1	Short-range device (SRD): if meet the technical condition in Table 27 of this Regulation.

TABLE 27

Technical conditions for short-range devices

The following technical conditions shall apply on the use of SRD

TABLE 28

Technical conditions for low-power wireless equipment

The following technical conditions shall apply on the use of LPWE

Appendix 8
to Annex 2

Technical parameters and spectrum use for SRDs in the Regional
Commonwealth in the Field of Communications countries

Information submitted in tables reflects state of affairs about the use SRDs in the Regional Commonwealth in the Field of Communications countries.

TABLE 29

Technical parameters and spectrum use for SRDs in Armenia (Republic of)

TABLE 30

Technical parameters and spectrum use for SRDs in Belarus (Republic of)

TABLE 31

Technical parameters and spectrum use for SRDs in Kazakhstan (Republic of)

TABLE 32

Technical parameters and spectrum use for SRDs in Kyrgyz Republic

TABLE 33

Technical parameters and spectrum use for SRDs in Moldova (Republic of)

TABLE 34

Technical parameters and spectrum use for SRDs in the Russian Federation

TABLE 35

Technical parameters and spectrum use for SRDs in Tajikistan (Republic of)

TABLE 36

Technical parameters and spectrum use for SRDs in Ukraine

TABLE 37

Technical parameters and spectrum use for SRDs in Uzbekistan (Republic of)

______________

____________________

* 	This Report replaces Recommendation ITU-R SM.1538.

**	Unless otherwise specified by mutual agreement between given administrations, status given to SRDs in individual country does not engage any other countries.

1 	In Brazil, the short range devices (SRDs) are referred to as the “restricted radiation radiocommunications equipments”.