## Annex 5		Road Transport and Traffic Telematics (RTTT)

Scope of Annex

This annex covers frequency bands and regulatory as well as informative parameters recommended for Road Transport and Traffic Telematics (RTTT) including radar system installations to be used in vehicles.

Regulatory parameters related to Annex 5

| Frequency Band |  |  | Power |  | Spectrum access and mitigation requirement |  | Channel spacing |  | ECC/ERC Decision | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a | 5795-5805 MHz | 2 W e.i.r.p. 8 W e.i.r.p. |  | No requirement |  |  |  | ECC/DEC/(02)01 |  |  |
| b | 5805-5815 MHz | 2 W e.i.r.p. 8 W e.i.r.p. |  | No requirement |  |  |  | ECC/DEC/(02)01 |  |  |
| c | 63-64 GHz | 40 dBm e.i.r.p. |  | No requirement |  | No spacing |  | ECC/DEC/(09)01 |  |  |
| d | 76-77 GHz | 55 dBm peak e.i.r.p. |  | No requirement |  | No spacing |  | ECC/DEC/(02)01 |  | 50 dBm average power - 23.5 dBm average power for pulse radar only For vehicle radars |
| e1 | 21.65-26.65 GHz | * |  | * |  | * |  | ECC/DEC/(04)10 |  | For automotive Short Range Radars (SRR) * See detailed requirements in related ECC Decision New SRR equipment may only be placed onto the market until 1 July 2013 |
| e2 | 24.25-26.65 GHz | * |  | * |  | * |  | ECC/DEC/(04)10 |  | For automotive Short Range Radars (SRR) * See detailed requirements in related ECC Decision New SRR equipment may only be placed onto the market until 1 January 2022 |
| f | 77-81 GHz | * |  | * |  | * |  | ECC/DEC/(04)03 |  | For automotive Short Range Radars (SRR) * See detailed requirements in related ECC Decision |
| g1 | 24.050-24.075 GHz | 100 mW e.i.r.p. |  | No requirement |  |  |  |  |  | For vehicle radars |
| g2 | 24.075-24.150 GHz | 0.1mW e.i.r.p. |  | No requirement |  |  |  |  |  | For vehicle radars |
|  |  | 100 mW e.i.r.p. |  | 4µs/40kHz dwell time every 3ms (note 1) |  |  |  |  |  | For vehicle radars. The spectrum access and mitigation requirement is given for devices mounted behind a bumper. If mounted without a bumper, the requirement should be 3µs/40kHz maximum dwell time every 3ms |
|  |  |  |  | 1ms/40kHz dwell time every 40ms (note 1) |  |  |  |  |  | The spectrum access and mitigation requirement is given for devices mounted either behind a bumper or mounted without a bumper |
| g3 | 24.150-24.250 GHz | 100mW e.i.r.p. |  | No requirement |  |  |  |  |  | For vehicle radars |
| g4 | 24.05-24.5 GHz | t.b.d |  | t.b.d. |  |  |  |  |  | Pending for definition in EN 303 057 (WLAM) |

Note 1: A requirement for minimum frequency modulation range (applicable to FMCW or step frequency signals) or minimum instantaneous bandwidth (applicable to pulsed signal) of 250 kHz applies in addition to the requirement on maximum dwell time.

Additional Information

Harmonised Standards

EN 300 674	sub-bands a) and b)

EN 302 686	sub-band c)

EN 301 091	sub-band d)

EN 302 288	sub-band e1, e2

EN 302 264	sub-band f)

EN 302 858  sub-bands g1), g2) and g3)

[EN 303 057       sub-band g4  (under development)