Draft ECC/DEC/(06)AA

Page 1
REVISED ECC/DEC/(06)04

Page 2
DRAFT amended ECC/DEC/(06)04

ELECTRONIC COMMUNICATIONS COMMITTEE

ECC Decision

of 24 March 2006  
amended 6 July 2007 at Constanta
amended dd mm yyyy at [location]

on the harmonised conditions for devices using 
Ultra-Wideband (UWB) technology 
in bands below 10.6 GHz
(ECC/DEC/(06)04)

amended 6 July 2007
amended dd mm yyyy

[image: image1.png]




EXPLANATORY MEMORANDUM

1
INTRODUCTION

This ECC Decision has been developed within the frame of EC mandates to CEPT to identify the conditions relating to the harmonised introduction in the European Union of radio applications based on ultra-wideband (UWB) technology. 

The underlying objective of these mandates was to provide ECC and the European Commission with the necessary information to adopt one or more technical implementing measures harmonising the use of the radio spectrum to enable the timely introduction of UWB technology within Europe.  To support the development and deployment of UWB technology it is essential that common spectrum, with the associated regulatory framework and harmonised standards, becomes available throughout Europe as soon as possible.  

UWB technology holds potential for a wide variety of new Short Range Devices (SRD) for communications, measurement, location tracking, imaging, surveillance and medical systems. This Decision defines conditions for the use of the radio spectrum by UWB devices. It also identifies some review mechanisms that will be required to ensure protection of radio services. 

It should be noted that this ECC Decision was initially supplemented by Decision ECC/DEC/(06)12 on specific mitigation techniques for UWB devices operating in bands below 10.6 GHz. Decisions ECC/DEC/(06)04 and ECC/DEC/(06)12 were merged as part of the review of the generic UWB regulation in 2010/2011.
2
BACKGROUND

Pursuant to the first mandate issued by the European Commission to CEPT on March 11th, 2004 to develop technical implementing measures for the harmonised use of radio spectrum for UWB applications in the European Union, ECC established a Task Group to develop the ECC responses and complete the technical studies already initiated within CEPT. This ECC Decision applies to generic UWB devices below 10.6 GHz that are exempted from individual licensing and operating on a non-interference, non-protected basis.

CEPT Report 27 (March 2009) provides an overview of the CEPT investigations on the generic UWB regulation completed with the amendment of Decision ECC/DEC/(06)12 in October 2008.

Some further amendments of Decisions ECC/DEC/(06)04 and ECC/DEC/(06)12 have been proposed within the frame of a new review of the generic UWB regulation in 2010/2011. They aim to provide additional clarification on the regulatory framework in Europe and to reflect the outcome of latest further studies on UWB. CEPT agreed to merge the 2 Decisions in order to facilitate possible future updates.
3
REQUIREMENT FOR AN ECC DECISION 

The allocation or designation of frequency bands under specified conditions in CEPT member countries is laid down by law, regulation or administrative action. ECC Decisions are required to deal with the carriage and use of equipment throughout Europe. The ECC also recognizes that for UWB devices to be introduced successfully throughout Europe, confidence must be given on the one hand to manufacturers to make the necessary investments and on the other hand to users of existing services that their protection will be ensured. 

The harmonisation on a European basis would support the Directive 1999/5/EC of the European Parliament and of the Council of 9 March 1999 on radio equipment and telecommunications terminal equipment and the mutual recognition of their conformity. 

A commitment by CEPT member countries to implement an ECC Decision will provide a clear indication that the required frequency range will be made available on time and on a Europe-wide basis and that the means to ensure protection of existing services will be applied. 

ECC Decision

of 24 March 2006
amended 6 July 2007 at Constanta
amended dd mm yyyy at [location]
on the harmonised conditions for devices using Ultra-Wideband (UWB) technology
 in bands below 10.6 GHz
(ECC/DEC/(06)04)
amended 6 July 2007
amended dd mm yyyy
Comparable technical specifications to those given in this ECC Decision are given in EC Decision 2007/131/EC amended by Commission Decision of 21 April 2009 (2009/343/EC).  EU/EEA-EFTA Member States are obliged to implement the EC Decision. (EEA-EFTA refers to those Member States of EFTA who participate in the EEA Agreement)

“The European Conference of Postal and Telecommunications Administrations,

considering

a) that UWB technology shall mean technology for short-range radiocommunication, involving the intentional generation and transmission of radio-frequency energy that spreads over a very large frequency range, which may overlap several frequency bands allocated to radiocommunication services;
b) that this Decision is applicable to technologies with bandwidth significantly wider than 50 MHz;
c) that Short Range Devices (SRD) using UWB technology can be used for communications, measurement, location, imaging, surveillance and medical systems;

d) that harmonised conditions across CEPT/EU help to establish an effective single market for these applications, with consequent economies of scale and benefits to the consumer, and avoid difficulties in enforcing divergent national regulations;

e) that a suitable CEPT/EU harmonized solution would encourage the global convergence of products by the UWB industry which would lead to greater economies of scale and the associated benefits;

f) that devices using UWB technology operate on a non-interference, non-protected basis ;
g) that the devices using UWB technology which are permitted to operate under this Decision present the potential to transmit in bands allocated to passive services that are covered in the RR footnote 5.340 which prohibits all emissions;

h) that this ECC Decision is primarily intended to respond to the market demand for UWB indoor and handheld devices providing communication applications;

i) that some categories of UWB devices characterized by predominantly outdoor usage are explicitly excluded from the scope of this Decision or subject to specific provisions as they can present a significant risk of interference to radio services deployed outdoor and operating in frequency bands where maximum UWB emission levels would be allowed;

j) that the protection requirements of radiocommunication systems below 10.6 GHz from Generic UWB Applications have been studied in ECC Report 64 assuming in particular for aggregate interference analyses 80% of UWB devices operating indoor, 20% outdoor and an average 5% activity factor;

k) that the maximum mean e.i.r.p. spectral densities limits of -85 dBm/MHz around 2 GHz and -80 dBm/MHz at 3.5 GHz are based on single interference analyses with IMT services and BWA services respectively assuming minimum separation distance of 36 cm and receiver sensitivity;
l) that the maximum mean e.i.r.p. spectral density limit of -70 dBm/MHz in the band 2.7 – 3.4 GHz is based on single interference analyses with aeronautical radars assuming in particular minimum separation distance of 170 m, a 7 dB main antenna beam attenuation and a 3 dB Multiple System/Technology Allowance factor;

m) that in addition a maximum peak e.i.r.p. limit of -36dBm defined in 50MHz in the band 2.7 – 3.4 GHz is based on test measurements with aeronautical radars and addresses the potential interference from UWB devices with low PRF and undithered pulses;
n) that a maximum mean e.i.r.p. spectral density of -65 dBm/MHz in the band 8.5 - 9 GHz is based on single interference analyses with radar systems assuming a minimum separation distance of 25 m;
o) that, for the Radio astronomy service, the protection levels given in ECC Report 64 are well below the maximum mean e.i.r.p. spectral densities given in the Annex 1, but when taking into account mitigation factors specific to the operation of particular UWB applications a  coexistence might be feasible; 
p) that ECC Report 64 has considered interference potential resulting from mean power and only limited consideration has been given to peak power interference, time gating and frequency hopping. ECC may review this Decision in the light of these possible implications;

q) that complementary technical studies presented in CEPT Report 9 (using different propagation models and assuming 100% of UWB devices operating indoor with an average 1% activity factor) provide some level of confidence regarding the protection of outdoor stations from the Fixed Service and the Fixed Satellite Service with a maximum mean e.i.r.p. spectral density level of -41.3 dBm/MHz;

r) that the issue of compatibility of UWB devices operated on-board either an aircraft or a vessel is the responsibility of the relevant aeronautical and maritime regulatory authorities;
s) that the compatibility studies performed within CEPT include, inter alia, the presumption that video signals will be transmitted using predominantly high-efficiency coding;

t) that if actual UWB deployment significantly exceeds assumptions used in the complementary technical studies, in particular if a significant amount of devices appear on the market without efficient video coding, then this regulation will have to be reviewed;

u) that limitations on the duty cycle of UWB devices and the implementation of Detect and Avoid (DAA) mechanism can improve the coexistence with other radiocommunication systems;

v) that technical requirements for LDC UWB devices to protect FWA terminals are presented in ECC Report 94;

w) that based on studies and measurement campaigns on the impact of LDC UWB devices on radars in the band 3.1 – 3.4 GHz, it was concluded in 2008 that the probability of a single LDC UWB device to radiate into the main beam of the radar was low and hence the risk of harmful interference was considered to be small;

x) that one study showed that aggregation effects from LDC UWB devices on radars could cause unacceptable probability of interference in the band 3.1–3.4 GHz. However, the various regulatory provisions aiming to minimise outdoor use could be sufficient to reduce the aggregate interference; 
y) that technical requirements for DAA UWB devices to ensure the protection of radiolocation services in the bands 3.1–3.4 GHz and 8.5–9 GHz and BWA terminals in the band 3.4–4.2 GHz are presented in ECC Report 120;

z) that DAA technical requirements in the band 8.5–9 GHz are based on characteristics of monostatic radiolocation systems and may be revised  subject to reported risk of interference to other classes of X-band radars considered to be deployed in the future, in particular passive radars;

aa) that DAA technical requirements given in Annex 3 of this ECC Decision need to be supplemented by adequate guidance on DAA measurement procedures and test patterns as defined in relevant standards (e.g. relevant versions of ETSI EN 302 065 including Harmonised European Standards adopted under Directive 1999/5/EC);

ab) that DAA technical requirements should safeguard the protection of BWA terminal stations for more than 99.75% of the time;

ac) that DAA technical requirements will need to be reviewed as existing systems are subject to technological change and other systems may be deployed or developed in the future; 

ad) that the band 3.4-3.6 GHz was identified for IMT applications at WRC-07;

ae) that the requirement of UWB DAA devices operating in the band 3.1–4.8 GHz to be capable of selecting an operating channel anywhere within the band 3.1-4.8 GHz band will provide additional mitigation to radio services operating in this band;

af) that, to reduce interference on outdoor radio stations, it is important to minimise the outdoor activity of UWB;
ag) that the exclusion of fixed outdoor transmitters from the scope of this Decision will also limit the operation of mobile outdoor devices;

ah) that special care may need to be given to deployment scenarios where a number of UWB transmitters with “mobile/nomadic” capabilities are operating outdoor under the provisions of this Decision in limited areas transmitting towards a “fixed” local UWB infrastructure (which is of “receive-only” nature) in case these UWB transmitters would remain permanently within the antenna main beam of a Fixed Service or Fixed Satellite Service station;

ai) that the operation of UWB devices installed in road and rail vehicles with their intended emissions directed towards the inside can be justified based on studies presented in CEPT Report 17;

aj) that CEPT Report 17 concluded that the activity factor of UWB devices installed inside vehicles is less than the indoor activity factor and that the average screening attenuation is comparable with typical indoor/outdoor attenuation. In addition there is a strong absorption loss due to human passengers providing an additional mitigation;
ak) that CEPT Report 17 recommends for UWB devices installed inside road and rail vehicles the implementation of Transmit Power Control (TPC) for devices which do not implement Low Duty Cycle (LDC) mitigation technique in view of reducing the aggregate interference on outdoor stations of radio services within the frequency bands 3.1–4.8 GHz and 6–8.5 GHz;
al) that ECC Report 170 provides compatibility studies in the bands 3.4–4.8 GHz and 6–8.5 GHz on the impact of LDC UWB devices installed inside road and rail vehicles assuming a penetration rate of 50%, 10 devices per vehicle (6 in 3.1–4.8 GHz and 4 devices per vehicle in 6–8.5 GHz) and their intended emissions directed towards the inside. ECC Report 170 concludes that an exterior limit of -53.3 dBm/MHz for emissions outside road and rail vehicles would provide a high level of confidence on the protection of most affected radio services;
am) that the definition of an exterior limit is primarily intended to improve the protection of radio services from UWB applications transmitting within the same vehicle towards an associated receiver but would also offer regulatory solutions for UWB installations with their intended emissions directed towards outside the vehicle;
an) that based on studies in ECC report 170 an additional mitigation of around 3dB would be required for applications with their intended emissions directed towards outside the vehicle, resulting in an emission limit emitting towards the outside of -56 dBm/MHz. However it is assumed that most of these outdoor automotive applications will operate in accordance with other spectrum regulations;  
ao) that if the actual number of UWB devices installed in vehicles significantly exceeds assumptions used in ECC Report 170, then this regulation will have to be reviewed;

ap) that for the band 4.2–4.8 GHz special care should be given to the aggregation of emissions from several UWB devices installed within the same vehicle and it was agreed that the duty cycle limit should be adjusted upon the vehicle speed;
aq) that the Methanol line at 6.7 GHz has only relatively recently been discovered and is becoming a focus of radio-astronomical research but an exterior limit of -53.3 dBm/MHz maximum mean e.i.r.p. outside a vehicle equipped with a single device would only offer a protection distance of about 700 m which may impact the observations of several RAS stations operating in Europe (Effelsberg, Jodrell Bank, Cambridge, Sardinia, Bleien…);
ar) that a harmonised transition measure (phased approach) was agreed in 2006 by the ECC in order to enable first generation (1G) of UWB devices operating in the 4.2-4.8 GHz frequency band with a maximum mean e.i.r.p. spectral density of –41.3 dBm/MHz without additional mitigation to be placed on the market until a fixed cut-off date (31 December 2010);

as) that the principle of a fixed cut-off date was justified by the need to ensure future development of radio services with indoor coverage operating in this band together with long-term protection of defence systems operating in this band;

at) that future World Radiocommunication Conferences may make revisions to the Radio Regulations that change the impact of UWB on Radiocommunication Services operating in accordance with the Table of Allocations;

au) that CEPT can develop other Decisions for specific classes of UWB device which do not meet the technical requirements of this Decision for generic UWB devices;

av) that CEPT overall approach and criteria for handling industry requests for specific UWB regulations is presented in CEPT Report 34;

aw) that, in order to support procedures of review of ECC Decisions, administrations are encouraged to collect market data on the numbers and types of UWB devices being placed on national markets;

ax) that administrations are encouraged to conduct measurements on the characteristics of these devices;

ay) that administrations are encouraged to monitor the impact of UWB devices on incumbent users, including the rise in noise due to the aggregate effect;

az) that administrations are encouraged to collect evidence of any interference caused to incumbent services by UWB devices;

ba) In EU/EFTA countries the radio equipment that is under the scope of this Decision shall comply with the R&TTE Directive. Conformity with the essential requirements of the R&TTE Directive may be demonstrated by compliance with the applicable harmonised European standard(s) or by using the other conformity assessment procedures set out in the R&TTE Directive.

DECIDES

1. that this ECC Decision defines general harmonised conditions for the use in Europe of devices using UWB technology in bands below 10.6 GHz;

2. that, for the purpose of the Decision, the following definitions apply:

a) Maximum mean e.i.r.p. spectral density: the highest signal strength measured in any direction at any frequency within the defined range. The mean e.i.r.p. spectral density is measured with a 1MHz resolution bandwidth, an RMS detector and an averaging time of 1ms or less.
b) Maximum peak e.i.r.p.: the highest signal strength measured in any direction at any frequency within the defined range. The peak e.i.r.p. is defined within a 50 MHz bandwidth.
c) LDC UWB devices are defined as devices using UWB technology that meet the technical requirements for Low Duty Cycle (LDC) mitigation technique given in Annex 2 of this Decision;

d) DAA UWB devices are defined as devices using UWB technology that meet the technical requirements for Detect And Avoid (DAA) mitigation technique given in Annex 3 of this Decision;

3. that the devices permitted under this ECC Decision are exempt from individual licensing and operate on a non-interference, non-protected basis;

4. that this ECC Decision is not applicable to:

a)
devices and infrastructure used at a fixed outdoor location or connected to a fixed outdoor antenna;
     b)    devices installed in flying models, aircraft and other aviation;

5. that the technical requirements detailed in Annex 1 apply to devices permitted under this ECC Decision;

6. to withdraw Decision ECC/DEC/(06)12 of 1 December 2006 on supplementary regulatory provisions to Decision ECC/DEC/(06)04 for UWB devices using mitigation techniques;
7. that this Decision enters into force on [date];

8. that the preferred date for implementation of this Decision shall be [date];

9. that CEPT administrations shall communicate the national measures implementing this Decision to the ECC Chairman and the Office when the Decision is nationally implemented.” 

Note: 

Please check the Office web site (http://www.cept.org/eco) for the up to date position on the implementation of this and other ECC Decisions. 

Annex 1

Technical requirements for devices using UWB technology in bands below 10.6 GHz

1. General case
The technical requirements below are not applicable to:

a) devices and infrastructure used at a fixed outdoor location or connected to a fixed outdoor antenna;

b) devices installed in flying models, aircraft and other aviation;

c) devices installed in road and rail vehicles.
Maximum e.i.r.p. limits

	Frequency range
	Maximum mean e.i.r.p. spectral density
	Maximum peak e.i.r.p. (defined in 50 MHz)



	Below 1.6 GHz
	-90 dBm/MHz
	-50 dBm

	1.6  to 2.7 GHz
	-85 dBm/MHz
	-45 dBm

	2.7  to 3.4 GHz (Notes 1 and 2)
	-70 dBm/MHz
	-36 dBm

	3.4  to 3.8 GHz (Notes 1 and 2)
	-80 dBm/MHz
	-40 dBm

	3.8 to 4.2 GHz (Notes -1 and 2)
	-70 dBm/MHz
	-30 dBm

	4.2 to 4.8 GHz (Notes 1 and 2)
	-70 dBm/MHz
	-30 dBm

	4.8 to 6 GHz
	-70 dBm/MHz
	-30 dBm

	6 to 8.5 GHz
	-41.3 dBm/MHz
	0 dBm

	8.5 to 10.6 GHz (Note 2)
	-65 dBm/MHz
	-25 dBm

	Above 10.6 GHz
	-85 dBm/MHz
	-45 dBm


Note 1: 
within the band 3.1 – 4.8 GHz, devices implementing Low Duty Cycle (LDC) mitigation technique (see Annex 2) are permitted to operate with a maximum mean e.i.r.p. spectral density of -41.3 dBm/MHz and a maximum peak e.i.r.p. of 0 dBm defined in 50 MHz.

Note 2: 
within the bands 3.1 – 4.8 GHz and 8.5 – 9 GHz, devices implementing Detect And Avoid (DAA) mitigation technique (see Annex 3) are permitted to operate with a maximum mean e.i.r.p. spectral density of -41.3 dBm/MHz and a maximum peak e.i.r.p. of 0 dBm defined in 50 MHz.
2. UWB devices installed in road and rail vehicles

The technical requirements below are applicable for the operation of UWB devices installed in road and rail vehicles: 

Maximum e.i.r.p. limits

	Frequency range
	Maximum mean e.i.r.p. spectral density
	Maximum peak e.i.r.p. (defined in 50 MHz)



	Below 1.6 GHz
	-90 dBm/MHz
	-50 dBm

	1.6 to 2.7 GHz
	-85 dBm/MHz
	-45 dBm

	2.7 to 3.4 GHz (Notes 1 and 2)
	-70 dBm/MHz
	-36 dBm

	3.4 to 3.8 GHz (Notes 1 and 2)
	-80 dBm/MHz
	-40 dBm

	3.8 to 4.2 GHz (Notes 1 and 2)
	-70 dBm/MHz
	-30 dBm

	4.2 to 4.8 GHz (Notes 1 and 2)
	-70 dBm/MHz
	-30 dBm

	4.8 to 6 GHz
	-70 dBm/MHz
	-30 dBm

	6 to 8.5 GHz (Notes 1 and 3)
	-53.3 dBm/MHz
	-13.3 dBm

	8.5 to 10.6 GHz (Note 2)
	-65 dBm/MHz
	-25 dBm

	Above 10.6 GHz
	-85 dBm/MHz
	-45 dBm


Note 1: 
within the band 3.1 – 4.8 GHz and 6 – 8.5 GHz, devices implementing Low Duty Cycle (LDC) mitigation technique (see Annex 2) are permitted to operate with a maximum mean e.i.r.p. spectral density of -41.3 dBm/MHz and a maximum peak e.i.r.p. of 0 dBm defined in 50MHz. Operation is in addition subject to the implementation of an exterior limit (see Annex 5) of -53.3 dBm/MHz. 
Note 2: 
within the bands 3.1 – 4.8 GHz and 8.5 – 9 GHz, devices implementing Detect And Avoid (DAA) mitigation technique (see Annex 3) are permitted to operate with a maximum mean e.i.r.p. spectral density of -41.3 dBm/MHz and a maximum peak e.i.r.p. of 0 dBm defined in 50MHz.
Operation is in addition subject to the implementation of Transmit Power Control (TPC) mitigation technique (see Annex 4) and an exterior limit (see Annex 5) of -53.3 dBm/MHz.
Note 3: 
within the band 6 – 8.5 GHz devices implementing Transmit Power Control (TPC) mitigation technique (see Annex 4) and an exterior limit (see Annex 5) of -53.3 dBm/MHz are permitted to operate with a maximum mean e.i.r.p. spectral density of -41.3 dBm/MHz and a maximum peak e.i.r.p. of 0 dBm defined in 50 MHz. 
Annex 2 
Technical requirements for Low Duty Cycle (LDC) mitigation technique

Ton max = 5 ms

Toff mean ≥ 38 ms (averaged over 1 sec)
Σ Toff > 950 ms per second

Σ Ton < 18 s per hour (see Note 1)
Note 1: 
in case of UWB devices installed in road and rail vehicles, within the band 3.4 – 4.8 GHz, this requirement does not apply for operation with vehicle speed above 40 km/h. For vehicle speeds between 20 km/h and 40 km/h a gradual implementation of the long term duty cycle limit from 18 seconds to 180 seconds per hour would be required as shown in figure 1.
[image: image2.emf]0

20

40

60

80

100

120

140

160

180

200

15 20 25 30 35 40 45

speed of vehicle in km/h

seconds per hour


Figure 1: Duty Cycle in seconds per hour in dependence of the vehicle speed

· Ton 

Ton is defined as the duration of a burst irrespective of the number of pulses contained.

· Toff 

Toff is defined as the time interval between two consecutive bursts when the UWB emission is kept idle.

· Limit definition 

The mean e.i.r.p. spectral density and peak e.i.r.p. power limits are defined during Ton. 

Annex 3 
Technical requirements for Detect And Avoid (DAA) mitigation technique

The flexible DAA proposal is based on the definition of different zones for which an appropriate UWB emission power level (maximum mean e.i.r.p. spectral density) is authorised. A zone is defined by a range of isolation between a device/system of a victim radio service and the UWB device. Theses zones and associated range of isolation correspond to the maximum mean e.i.r.p. spectral density levels specified in Table 1.

In the first zone, the UWB device shall operate at an emission level applied in the avoidance bandwidth as defined in table 1. In the last zone, the UWB device can operate without restriction up to the maximum permitted power level of -41.3 dBm/MHz or as defined in a future DAA regulation for the corresponding operational frequency range except in road and rail vehicles where additional restrictions apply. Between these extreme zones, a transition zone is defined for the band 3.4 – 4.8 GHz.

Before initiating UWB communications, the UWB device shall perform a monitoring of the RF environment during a minimum time to detect any actively operating victim signal (minimum initial channel availability check time value given in Table 1). Based on the result of this detection process, the UWB device has to determine the corresponding zone it occupies and react accordingly.

This function shall be able to detect victim systems signals and measure if this power level in a given bandwidth is above or below a detection threshold in any of the frequency bands denoted here after. This detection threshold is specified at the antenna connector assuming a 0dBi antenna gain for each detection operation and may be based on multiple levels. This detection threshold can alternatively be expressed as a field strength limit.

The DAA UWB devices shall be able to continuously detect any change of the RF configuration (e.g. modification of operating zone) and switch to corresponding emission level within a maximum detect and avoid time according to the victim service and procedural tests Harmonized European defined in relevant standards (e.g. relevant versions of ETSI EN 302 065, including Harmonised European Standards adopted under Directive 1999/5/EC).

	
	Operational frequency
	3.1 - 3.4 GHz
	3.4 - 3.8 GHz1
	3.8 - 4.8 GHz1
	8.5 - 9 GHz

	
	Minimum initial channel availability check time
	14s
	5.1s
	14s

	Zone 1

for Signal detection level

S >A
	Maximum mean e.i.r.p. spectral density
	-70 dBm/MHz
	-80 dBm/MHz
	-70 dBm/MHz
	-65 dBm/MHz

	
	Default Avoidance bandwidth 
	300 MHz
	200 MHz
	500 MHz

	Signal Detection threshold A
	-38 dBm
	-38 dBm
	-61 dBm

	Zone 2

for Signal detection level

A > S > B
	Maximum mean e.i.r.p. spectral density 
	-41.3 dBm/MHz
	-65 dBm/MHz
	-41.3 dBm/MHz

	
	Default Avoidance bandwidth
	-
	200 MHz
	-

	Signal Detection threshold B
	
	-61 dBm
	

	Zone 3

for Signal detection level

S < B
	Maximum mean e.i.r.p. spectral density 


	-
	-41.3 dBm/MHz


	-


Table 1: Technical parameters to be used by UWB DAA devices

0 Definitions of the parameters can be found in ECC Report 120 

1 Detection mechanism needs to be validated to protect existing operation of victim stations of radio services such as BWA terminals
Additional requirement for operation in the band 3.1 – 4.8 GHz

UWB DAA devices shall be capable of selecting an operating channel anywhere within the band 3.1 – 4.8 GHz.
Annex 4

Technical requirements for Transmit Power Control (TPC) mitigation technique

Devices implementing Transmit Power Control (TPC) should fulfil at least a dynamic range of 12 dB (mean e.i.r.p. range of -41.3 to -53.3 dBm/MHz).
Annex 5

Technical requirements for the exterior limit for road and rail vehicle applications

The exterior limit is defined, for each UWB device installed in a road or rail vehicle, as the maximum mean e.i.r.p spectral density for the emissions outside the vehicle at elevation angles higher than 0 degree.
Within the bands 3.1–4.8 GHz, 6–8.5 GHz and 8.5–9 GHz, an exterior limit of -53.3 dBm/MHz applies.

