| European Union | Radio Interface Specification | Land Mobile Earth Stations | Sub-class 11 old | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Land Mobile Satellite Service |  |
|  | 2 | Application | Land Mobile Earth Stations | providing voice and /or data communications |
|  | 3 | Frequency band | 1 525.0 MHz - 1 544.0 MHz 1 555.0 MHz - 1 559.0 MHz 1 631.5 MHz - 1 634.5 MHz 1 656.5 MHz - 1 660.5 MHz | receive 1 (space-to-Earth); receive 2 (space-to-Earth); transmit 1 (Earth-to-space); transmit 2 (Earth-to-space); |
|  | 4 | Channelling | to be defined by the satellite operator |  |
|  | 5 | Modulation / Occupied bandwidth | to be defined by the satellite operator |  |
|  | 6 | Direction / Separation |  | to be defined by the satellite operator |
|  | 7 | Transmit power / Power density | 148 dBpW 177 - 25 log f dBpW 130 dBpW | for φ < 40°; for 40° < f < 75°; for φ > 75°; (Ф is the angle, in degrees, between the main beam axis and the direction considered) |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 301 444 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Land Mobile Earth Stations | Sub-class 11 | Edition 2 xxxx |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Land Mobile Satellite Service |  |
|  | 2 | Application | Land Mobile Earth Stations | providing voice and /or data communications |
|  | 3 | Frequency band | 1 525.0 MHz - 1 544.0 MHz 1 555.0 MHz - 1 559.0 MHz 1 631.5 MHz - 1 634.5 MHz 1 656.5 MHz - 1 660.5 MHz | receive 1 (space-to-Earth); receive 2 (space-to-Earth); transmit 1 (Earth-to-space); transmit 2 (Earth-to-space); The use of the bands 1 544 – 1 545 MHz (space-to-Earth) and 1 645.5 – 1 646.5 MHz (Earth-to-space) is limited to distress and safety communications. |
|  | 4 | Channelling | to be defined by the satellite operator |  |
|  | 5 | Modulation / Occupied bandwidth | to be defined by the satellite operator |  |
|  | 6 | Direction / Separation |  | to be defined by the satellite operator |
|  | 7 | Transmit power / Power density | 148 dBpW 177 - 25 log f dBpW 130 dBpW | for φ < 40°; for 40° < f < 75°; for φ > 75°; (Ф is the angle, in degrees, between the main beam axis and the direction considered) |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | ECC/DEC/(04)09 ITU RR 5.356 and 5.375 Harmonised standard: EN 301 681 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Land Mobile Earth Stations operating in the Ku-band | Sub-class 12 | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Land Mobile-Satellite Service |  |
|  | 2 | Application | Land Mobile Earth Stations applications |  |
|  | 3 | Frequency band | 10.70 GHz - 11.70 GHz 12.50 GHz - 12.75 GHz 14.00 GHz - 14.25 GHz | (space-to-Earth) (space-to-Earth) (Earth to space) |
|  | 4 | Channelling | to be defined by the satellite operator |  |
|  | 5 | Modulation / Occupied bandwidth | to be defined by the satellite operator |  |
|  | 6 | Direction / Separation |  | to be defined by the satellite operator |
|  | 7 | Transmit power / Power density | 33-25 log(φ + δφ)-10 log(K) dBW/40kHz where 2,5° ≤φ+ δφ ≤7,0°; +12-10 log(K) dBW/40kHz where 7,0° <φ + δφ ≤9,2°; 36-25 log(φ + δφ )-10 log(K) dBW/40kHz where 9,2° <φ + δφ ≤48°; -6-10 log(K) dBW/40 kHz where 48° <φ + δφ ≤180° | φ is the angle, in degrees, between the main beam axis and the direction considered. K is the power density ratio between the fully loaded system and a single LMES measured in a 40 kHz bandwidth |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 301 427 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | TETRA end-user equipment | Sub-class 13 | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | TETRA | Emergency access end user terminals. Network stations without DMO. |
|  | 3 | Frequency band | 380 MHz - 385 MHz 390 MHz - 395 MHz |  |
|  | 4 | Channelling | 25 kHz |  |
|  | 5 | Modulation / Occupied bandwidth | π/4 shifted Differential Quaternary Phase Shift Keying“ (π/4 DQPSK) |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 45 dBm (30W) 40 dBm (10W) 35 dBm (3W) 30 dBm (1W) | power class 1 power class 2 power class 3 power class 4 |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 303 035 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Satellite Personal Communication Earth Stations (S-PCES) | Sub-class 14 | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Land Mobile-Satellite Service |  |
|  | 2 | Application | "Satellite-Personal Communications earth stations" applications. |  |
|  | 3 | Frequency band | 1 610 - 1 613.5 MHz 1 613.8 - 1 626.5 MHz 2 483.5 - 2 500 MHz | transmit (Earth-to-space) receive (space-to-Earth) receive (space-to-Earth) |
|  | 4 | Channelling | to be defined by the satellite operator |  |
|  | 5 | Modulation / Occupied bandwidth | to be defined by the satellite operator |  |
|  | 6 | Direction / Separation |  | to be defined by the satellite operator |
|  | 7 | Transmit power / Power density | -3 dB (W/4 kHz), (mean limit) -15 dB (W/4 kHz), (peak limit) |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 301 441 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Satellite Personal Communication Earth Stations (S-PCES) | Sub-class 15 | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Land Mobile-Satellite Service |  |
|  | 2 | Application | "Satellite-Personal Communications earth stations" applications. |  |
|  | 3 | Frequency band | 1 980 - 2 010 MHz 2 170 - 2 200 MHz | transmit (Earth-to-space); receive (space-to-Earth); |
|  | 4 | Channelling | to be defined by the satellite operator |  |
|  | 5 | Modulation / Occupied bandwidth | to be defined by the satellite operator |  |
|  | 6 | Direction / Separation |  | to be defined by the satellite operator |
|  | 7 | Transmit power / Power density | to be defined by the satellite operator |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 301 442 Commission Decision 2007/98/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Low data rate Land Mobile Earth Stations(LMES) | Sub-class 16 old | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Land Mobile-Satellite Service |  |
|  | 2 | Application | Low data rate LMES applications |  |
|  | 3 | Frequency band | 1 525.0 MHz - 1 544.0 MHz 1 555.0 MHz - 1 559.0 MHz 1 626.5 MHz - 1 645.5 MHz 1 656.5 MHz - 1 660.5 MHz | receive 1 (space-to-Earth); receive 2 (space-to-Earth); transmit 1 (Earth-to-space); transmit 2 (Earth-to-space); |
|  | 4 | Channelling | to be defined by the satellite operator |  |
|  | 5 | Modulation / Occupied bandwidth | to be defined by the satellite operator |  |
|  | 6 | Direction / Separation |  | to be defined by the satellite operator |
|  | 7 | Transmit power / Power density | to be defined by the satellite operator |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | ECC/DEC/(04)09 ITU RR 5.356 and 5.375 Harmonised Standard: EN 301 426 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Low data rate Land Mobile Earth Stations(LMES) | Sub-class 16 | Edition 2 xxxx |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Land Mobile-Satellite Service |  |
|  | 2 | Application | Low data rate LMES applications |  |
|  | 3 | Frequency band | 1 525.0 MHz - 1 544.0 MHz 1 555.0 MHz - 1 559.0 MHz 1 626.5 MHz - 1 645.5 MHz 1 656.5 MHz - 1 660.5 MHz The use of the bands 1 544 – 1 545 MHz (space-to-Earth) and 1 645.5 – 1 646.5 MHz (Earth-to-space) is limited to distress and safety communications. | receive 1 (space-to-Earth); receive 2 (space-to-Earth); transmit 1 (Earth-to-space); transmit 2 (Earth-to-space); |
|  | 4 | Channelling | to be defined by the satellite operator |  |
|  | 5 | Modulation / Occupied bandwidth | to be defined by the satellite operator |  |
|  | 6 | Direction / Separation |  | to be defined by the satellite operator |
|  | 7 | Transmit power / Power density | to be defined by the satellite operator |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 301 426 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Digital Enhanced Cordless Telephony | Sub-class 18 | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | DECT | user terminal equipment |
|  | 3 | Frequency band | 1880 MHz – 1900 MHz |  |
|  | 4 | Channelling | 1728 kHz |  |
|  | 5 | Modulation / Occupied bandwidth | See EN 301 406 |  |
|  | 6 | Direction / Separation | TDD | See EN 301 406 |
|  | 7 | Transmit power / Power density | 250 mWatt peak e.r.p. (peak power over time-slot) | Type of Antenna: integral or dedicated |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 301 406 Council Decision 91/287/EEC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Non-Specific Short Range Devices | Sub-class 19 old | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non-Specific Short Range Devices (Video applications are excluded) | This category is available for any type of application which fulfils the technical conditions (typical uses are telemetry, telecommand, alarms, data in general and other similar applications). |
|  | 3 | Frequency band | 40.660-40,700 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r,p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300-220-2 Commission Decision 2006/771/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 19 | Edition 2 01.06.2007 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | Video applications are excluded |
|  | 3 | Frequency band | 40.660 - 40.700 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2006/771/EC |  |

| European Union | Radio Interface Specification | Non Specific Short Range Devices | Sub-class 21 old | Edition 3 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non-Specific Short Range Device | This category is available for any type of application which fulfils the technical conditions (typical uses are telemetry, telecommand, alarms, data in general and other similar applications). |
|  | 3 | Frequency band | 2400 MHz – 2483.5 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r,p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300-220-2 Commission Decision 2006/771/EC or 2008/432 ? |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 21 | Edition 4 01.06.2007 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | This category is available for any type of application which fulfils the technical conditions (typical uses are telemetry, telecommand, alarms, data in general and other similar applications). |
|  | 3 | Frequency band | 2 400–2 483,5 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 440-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2006/771/EC |  |

| European Union | Radio Interface Specification | Wideband Data Transmission Systems | Sub-class 22 | Edition 2 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Wideband Data Transmission Systems | Wireless networking devices (including ad-hoc networking) for the transmission of Data (including digitized audio and video). Eg., technologies such as WiFi, BluetoothTM, HomeRFTM, ZigbeeTM, etc. |
|  | 3 | Frequency band | 2400 – 2454 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 100 mW eirp | For wide band modulations other than FHSS (e.g. DSSS, OFDM, ..), the maximum e.i.r.p. density is limited to 10 mW/1 MHz |
|  | 8 | Channel access and occupation rules |  | Devices use a medium access protocol designed to facilitate spectrum sharing |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 328 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Wideband data transmission systems | Sub-class 22 | Edition 3 01.11.2009 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Wideband data transmission systems | Wireless networking devices (including ad-hoc networking) for the transmission of Data (including digitized audio and video). Eg., technologies such as WiFi, BluetoothTM, HomeRFTM, ZigbeeTM, etc. |
|  | 3 | Frequency band | 2 400–2 483,5 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 100 mW e.i.r.p. and 100 mW/100 kHz e.i.r.p. density applies when frequency hopping modulation is used, 10 mW/MHz e.i.r.p. density applies when other types of modulation are used |  |
|  | 8 | Channel access and occupation rules | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | Commission Decision 2011/829/EC EN 300 328 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2009/381/EC |  |

| European Union | Radio Interface Specification | Inductive Applications | Sub-class 24 old | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 13.553 - 13.567 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42 dBµA/m at 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300-330-2 Commission Decision 2006/771/EC or 2008/432 ? |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 24 | Edition 2 01.06.2007 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 13 553–13 567 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2006/771/EC |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 25 old | Edition 3 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non-Specific Short Range Devices (Video applications are excluded) | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 26.957 - 27.283 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. which corresponds to 42 dBµA/m at 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300-220-2 Commission Decision 2006/771/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 25 | Edition 4 01.06.2007 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | Video applications are excluded |
|  | 3 | Frequency band | 26.957 - 27.283 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW effective radiated power (e.r.p.), which corresponds to 42 dBµA/m at 10 metres |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2006/771/EC |  |

| European Union | Radio Interface Specification | Movement Detection | Sub-class 26 | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Movement Detection | Equipment for Detection of movement and equipment for alert |
|  | 3 | Frequency band | 2446 - 2454 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 mW e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 440-2 - 3 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Movement Detection | Sub-class 27 | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Movement Detection | Equipment for Detection of movement and equipment for alert |
|  | 3 | Frequency band | 24.15 - 24.175 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 100 mW e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 440-2 - 3 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Non-Specific Short Range Devices | Sub-class 28 old | Edition 2 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non-Specific Short Range Device (video applications are excluded) | This category is available for any type of application which fulfils the technical conditions (typical uses are telemetry, telecommand, alarms, data in general and other similar applications). |
|  | 3 | Frequency band | 868.000 MHz - 868.600 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 mW e.r.p |  |
|  | 8 | Channel access and occupation rules | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle of 1 % may also be used |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300-220-2 Commission Decision 2006/771/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 28 | Edition 2 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | Analogue video applications are excluded |
|  | 3 | Frequency band | 868,000–868,600 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle limit of 1 % may also be us |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Non-Specific Short Range Devices | Sub-class 29 old | Edition 3 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non-Specific Short Range Device (video applications are excluded) | This category is available for any type of application which fulfils the technical conditions (typical uses are telemetry, telecommand, alarms, data in general and other similar applications). |
|  | 3 | Frequency band | 868.7 MHz – 869.2 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle of 0.1 % may also be used |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300-220-2 Commission Decision 2006/771/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 29 | Edition 4 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | Analogue video applications are excluded |
|  | 3 | Frequency band | 868,700–869,200 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle limit of 0,1 % may also be |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Non-Specific Short Range Devices | Sub-class 30 old | Edition 2 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non-Specific Short Range Device (video applications are excluded) | This category is available for any type of application which fulfils the technical conditions (typical uses are telemetry, telecommand, alarms, data in general and other similar applications). |
|  | 3 | Frequency band | 869.400 MHz – 869.650 MHz |  |
|  | 4 | Channelling | Spacing must be 25 kHz, except that the whole band may also be used as one single channel for high-speed data transmission |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 500 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle of 10 % may also be used. |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 - 2 Commission Decision 2008/432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 30 | Edition 3 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | This category is available for any type of application which fulfils the technical conditions (typical uses are telemetry, telecommand, alarms, data in general and other similar applications). Analogue video applications are excluded |
|  | 3 | Frequency band | 869,400–869,650 MHz |  |
|  | 4 | Channelling | Channel spacing must be 25 kHz, except that the whole band may also be used as a single channel for high-speed data transmission |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 500 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle limit of 10 % may also be u |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Non-Specific Short Range Devices | Sub-class 31 old | Edition 2 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non-Specific Short Range Device (audio and video applications are excluded) | This category is available for any type of application which fulfils the technical conditions (typical uses are telemetry, telecommand, alarms, data in general and other similar applications). |
|  | 3 | Frequency band | 869.700 MHz – 870.000 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 5 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Voice applications allowed with advanced mitigation techniques |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 Commission Decision 2008/432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 31 | Edition 3 01.06.2007 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | This category is available for any type of application which fulfils the technical conditions (typical uses are telemetry, telecommand, alarms, data in general and other similar applications). Audio and video applications are excluded |
|  | 3 | Frequency band | 869,700–870,000 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 5 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Voice applications allowed with advanced mitigation techniques |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2006/771/EC |  |

| European Union | Radio Interface Specification | Short Range Device alarm systems | Sub-class 32 old | Edition 2 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Alarm systems |  |
|  | 3 | Frequency band | 868.600 MHz – 868.700 MHz |  |
|  | 4 | Channelling | Channel spacing: 25 kHz The whole frequency band may also be used as one single channel for high-speed data transmission |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle up: 1 % |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 Commission Decision 2008/432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Alarm systems | Sub-class 32 | Edition 3 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Alarm systems |  |
|  | 3 | Frequency band | 868,600–868,700 MHz |  |
|  | 4 | Channelling | 25 kHz The whole frequency band may also be used as a single channel for high- speed data transmission |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle max 1,0 % |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Short Range Device alarm systems | Sub-class 33 old | Edition 2 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Alarm systems |  |
|  | 3 | Frequency band | 869.250 MHz – 869.300 MHz |  |
|  | 4 | Channelling | Channel spacing: 25 kHz |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle: 0.1 % |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 Commission Decision 2008/432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Alarm systems | Sub-class 33 | Edition 3 01.06.2007 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Alarm systems |  |
|  | 3 | Frequency band | 869,250–869,300 MHz |  |
|  | 4 | Channelling | 25 kHz |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle max 0,1 % |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2006/771/EC |  |

| European Union | Radio Interface Specification | Short Range Device alarm systems | Sub-class 34 old | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Alarm systems |  |
|  | 3 | Frequency band | 869.650 MHz – 869.700 MHz |  |
|  | 4 | Channelling | Channel spacing: 25 kHz |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle: 10 % |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 Commission Decision 2008/432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Alarm systems | Sub-class 34 | Edition 2 01.06.2007 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Alarm systems |  |
|  | 3 | Frequency band | 869,650–869,700 MHz |  |
|  | 4 | Channelling | 25 kHz |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle max 1,0 % |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2006/771/EC |  |

| European Union | Radio Interface Specification | Social Alarms | Sub-class 35 old | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Social alarms | Social alarm devices are used to assist elderly or disabled people living at home when they are in distress. |
|  | 3 | Frequency band | 869.200 MHz – 869.250 MHz |  |
|  | 4 | Channelling | Channel spacing: 25 kHz |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle: 0.1 % |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 Commission Decision 2008/432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Social alarms | Sub-class 35 | Edition 2 01.06.2007 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Social alarms | Social alarm devices are used to assist elderly or disabled people living at home when they are in distress. |
|  | 3 | Frequency band | 869,200–869,250 MHz |  |
|  | 4 | Channelling | 25 kHz |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle max 0,1 % |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2006/771/EC |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 36 old | Edition 2 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 20.050 – 59.750 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 72 dBµA/m at 10 metres |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 Commission Decision 2008/432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 36 | Edition 3 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 9,000–59,750 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 72 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 37 old | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 59.750 – 60.250 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42 dBµA/m at 10 metres |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 Commission Decision 2008/432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 37 | Edition 2 01.06.2007 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 59,750–60,250 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2006/771/EC |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 39 old | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 60,250 – 70,000 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 69 dBµA/m at 10 metres |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 - 2 Commission Decision 2008/432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 39 | Edition 2 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 60,250–74,750 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 72 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 40 old | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 70 – 119 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42 dBµA/m at 10 metres |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 Commission Decision 2008/432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 40a | Edition 2 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 74,750–75,250 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 40b | Edition 2 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 75,250–77,250 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 72 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 40c | Edition 2 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 77,250–77,750 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 40d | Edition 2 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 77,750–90 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 72 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 40e | Edition 2 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 90–119 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 41 old | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 119 – 127 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 66 dBµA/m at 10 metres |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 Commission Decision 2008/432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 41 | Edition 2 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 119–128,6 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 66 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 42 old | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 127 – 140 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42dBµA/m at 10 metres |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 Commission Decision 2008/432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 42a | Edition 2 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 128,6–129,6 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 42b | Edition 2 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 129,6–135 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 66 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 43 old | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | This category is available for any type of application which fulfils the technical conditions (typical uses are telemetry, telecommand, alarms, data in general and other similar applications). |
|  | 3 | Frequency band | 5725 – 5875 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 mW e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 440-2 Commission Decision 2008/432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 43 | Edition 2 01.06.2007 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | This category is available for any type of application which fulfils the technical conditions (typical uses are telemetry, telecommand, alarms, data in general and other similar applications). |
|  | 3 | Frequency band | 5 725–5 875 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 mW e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 440-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2006/771/EC |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 44 old | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 6765 - 6795 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42dBµA/m at 10 metres |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 Commission Decision 2008/432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 44 | Edition 2 01.06.2007 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | This category covers, for example, devices for car immobilisation, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF a |
|  | 3 | Frequency band | 6 765–6 795 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2006/771/EC |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 45 old | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | Includes for example car immobilisers, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF anti-theft induction systems, |
|  | 3 | Frequency band | 7400 - 8800 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 9 dBµA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 Commission Decision 2008/432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 45 old | Edition 2 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | Includes for example car immobilisers, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF anti-theft induction systems, |
|  | 3 | Frequency band | 7 400–8 800 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 9 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 45 | Edition 3 xxxx |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | Includes for example car immobilisers, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, anti-theft systems including RF anti-theft induction systems, |
|  | 3 | Frequency band | 7 400–8 800 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 9 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | Commission Decision 2010/368/EC EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification |  | Sub-class 46 ? | Edition |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service |  |  |
|  | 2 | Application |  |  |
|  | 3 | Frequency band |  |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density |  |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference |  |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Medical Implants | Sub-class 47 old | Edition 2 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Active medical implants | This category covers the radio part of active implantable medical devices, as defined in Council Directive 90/385/EEC of 20 June 1990 on the approximation of the laws of the Member States relating to active implantable medical devices and their peripheral |
|  | 3 | Frequency band | 402 - 405 MHz |  |
|  | 4 | Channelling | Channel spacing: 25kHz Other channelling restriction: individual transmitters may combine adjacent channels for increased bandwidth with advanced mitigation techniques that provide at least equivalent performance to the techniques described in harmonised |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 µW e.r.p |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 301 839 - 2 Commission Decision 2008432/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Active medical implants | Sub-class 47 | Edition 3 01.11.2009 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Active medical implants | This category covers the radio part of active implantable medical devices, as defined in Council Directive 90/385/EEC of 20 June 1990 on the approximation of the laws of the Member States relating to active implantable medical devices and their peripheral |
|  | 3 | Frequency band | 402–405 MHz |  |
|  | 4 | Channelling | Channel spacing: 25 kHz Individual transmitters may combine adjacent channels for increased bandwidth up to 300 kHz. |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 μW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Other techniques to access spectrum or mitigate interference, including bandwidths greater than 300 kHz, can be used provided they result at least in an equivalent performance to the techniques described in harmonised standards adopted under Directive 199 |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 301 839 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2009/381/EC |  |

| European Union | Radio Interface Specification | Wireless Audio Applications | Sub-class 48 old | Edition 1 03.09 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Wireless Audio applications | Applications for wireless audio systems, including: cordless loudspeakers; cordless headphones; cordless headphones for portable use, for example portable CD, cassette or radio devices carried on a person; cordless headphones for use in a vehicle, for exa |
|  | 3 | Frequency band | 863 - 865 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 301 357 - 2 Commission Decision 20086/432771/EC |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Wireless audio applications | Sub-class 48 | Edition 2 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Wireless audio applications | Applications for wireless audio systems, including: cordless loudspeakers; cordless headphones; cordless headphones for portable use, for example portable CD, cassette or radio devices carried on a person; cordless headphones for use in a vehicle, for exa |
|  | 3 | Frequency band | 863–865 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 301 357 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Avalanche Victims | Sub-class 49 | Edition 1 03.07 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Avalanche Victims | Equipment for detection of avalanche victims |
|  | 3 | Frequency band | 457 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth | Continuous wave (CW) – no modulation |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 7 dBµA/m at 10 m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 718 - 3 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Road Transport and Traffic Telematics (RTTT) | Sub-class 50 old | Edition 1 03.07 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Radiolocation | Equipment for Road Transport and Traffic Telematics, scope limited to vehicular or infrastructure radar systems. |
|  | 3 | Frequency band | 76 – 77 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 55 dBm e.i.r.p. | 50 dBm average power3.5 dBm average power for puls radar only |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 301 091 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Road transport and traffic telematics | Sub-class 50 | Edition 2 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Road transport and traffic telematics | Restricted to terrestrial vehicle and infrastructure systems |
|  | 3 | Frequency band | 76,0–77,0 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 55 dBm peak e.i.r.p. and 50 dBm mean e.i.r.p. and 23,5 dBm mean e.i.r.p. for pulse radars |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 301 091 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Private (Professional) Mobile Radio | Sub-class 51 | Edition 1 03.07 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | PMR446 | End user terminals for voice only |
|  | 3 | Frequency band | 446.0 – 446.1 MHz | Carrier frequencies: 446.00625, 446.01875, 446.03125, 446.04375, 446.05625, 446.06875, 446.08125, 446.09375 MHz |
|  | 4 | Channelling | 12.5 kHz |  |
|  | 5 | Modulation / Occupied bandwidth | Angle modulation |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 500 mW e.r.p. | Integral antenna only |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 296-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Road transport and traffic telematics | Sub-class 52 | Edition |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Road transport and traffic telematics | Automotive Short Range Radars |
|  | 3 | Frequency band | 21.65 - 26.65 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | Max. 20 dBm peak EIRP. Max. -41.3 dBm/MHz average PSD |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 288 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2005/50/EC as amended by Commission Decision 2011/485/EU |  |

| European Union | Radio Interface Specification | Road transport and traffic telematics | Sub-class 53 | Edition |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Road transport and traffic telematics | Automotive Short Range Radars |
|  | 3 | Frequency band | 77 GHz - 81 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | Max. 55 dBm peak EIRP. Max. 23.5 dBm mean EIRP. Max. -3 dBm/MHz mean EIRP power density |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | TR 102 263 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2004/545/EC |  |

| European Union | Radio Interface Specification | Wideband data transmission systems | Sub-class 54 | Edition 1 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Wideband data transmission systems |  |
|  | 3 | Frequency band | 5470 - 5725 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | Max. 1 W mean EIRP. Max. 50 mW/MHz mean EIRP density in any 1 MHz band. | shall either employ transmitter power control (TPC), which provides, on average, a mitigation factor of at least 3 dB on the maximum permitted output power of the systems; or if transmitter power control is not in use, the maximum permitted mean EIRP and |
|  | 8 | Channel access and occupation rules |  | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used Dwell time limits apply as specified in harmonised st |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 301 893 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification |  | Sub-class 55 | Edition |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service |  |  |
|  | 2 | Application |  |  |
|  | 3 | Frequency band |  |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density |  |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference |  |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification |  | Sub-class 56 | Edition |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service |  |  |
|  | 2 | Application |  |  |
|  | 3 | Frequency band |  |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density |  |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference |  |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification |  | Sub-class 57 | Edition |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service |  |  |
|  | 2 | Application |  |  |
|  | 3 | Frequency band |  |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density |  |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference |  |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification |  | Sub-class 58 | Edition |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service |  |  |
|  | 2 | Application |  |  |
|  | 3 | Frequency band |  |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density |  |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference |  |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification |  | Sub-class | Edition |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service |  |  |
|  | 2 | Application |  |  |
|  | 3 | Frequency band |  |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density |  |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference |  |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification |  | Sub-class | Edition |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service |  |  |
|  | 2 | Application |  |  |
|  | 3 | Frequency band |  |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density |  |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference |  |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 59 | Edition 1 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device |  |
|  | 3 | Frequency band | 6765 - 6795 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42 dBµA/m at 10 metres |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 60 | Edition 1 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device |  |
|  | 3 | Frequency band | 13.553 - 13.567 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42 dBµA/m at 10 metres |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 61 | Edition 1 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | Audio and video applications are excluded |
|  | 3 | Frequency band | 433,050–434,040 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 1 mW e.r.p. and - 13dBm/10 kHz power density for bandwidth modulation larger than 250 kHz |  |
|  | 8 | Channel access and occupation rules | Voice applications allowed with advanced mitigation techniques |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 62 | Edition 1 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | Analogue audio applications other than voice are excluded. Analogue video applications are excluded |
|  | 3 | Frequency band | 433,050–434,040 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle max : 10 % |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 63 | Edition 1 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | Audio and video applications are excluded |
|  | 3 | Frequency band | 434,040–434,790 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 1 mW e.r.p. and - 13dBm/10 kHz power density for bandwidth modulation larger than 250 kHz |  |
|  | 8 | Channel access and occupation rules | Voice applications allowed with advanced mitigation techniques |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 64 | Edition 1 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | Analogue audio applications other than voice are excluded. Analogue video applications are excluded |
|  | 3 | Frequency band | 434,040–434,790 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle max : 10 % |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 65 | Edition 1 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | Audio and video applications are excluded |
|  | 3 | Frequency band | 434,040–434,790 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle max: 100 % subject to channel spacing up to 25 kHz Voice applications allowed with advanced mitigation techniques |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 66 | Edition 1 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | Analogue audio applications other than voice are excluded. Analogue video applications are excluded |
|  | 3 | Frequency band | 863,000–865,000 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle limit of 0,1 % may also be |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 67 | Edition 1 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | Analogue audio applications other than voice are excluded. Analogue video applications are excluded |
|  | 3 | Frequency band | 865,000–868,000 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle limit of 1 % may also be us |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 68 | Edition 1 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | Analogue audio applications other than voice are excluded. Analogue video applications are excluded |
|  | 3 | Frequency band | 869,400–869,650 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle limit of 0,1 % may also be |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 69 | Edition 1 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device | Analogue audio applications other than voice are excluded. Analogue video applications are excluded |
|  | 3 | Frequency band | 869,700–870,000 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle limit of 1 % may also be us |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 70 | Edition 1 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device |  |
|  | 3 | Frequency band | 24,150–24,250 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 100 mW e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 440-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 71 | Edition 1 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device |  |
|  | 3 | Frequency band | 61,0–61,5 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 100 mW e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 305 550 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Alarm systems | Sub-class 72 | Edition 1 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Alarm systems |  |
|  | 3 | Frequency band | 869,300–869,400 MHz |  |
|  | 4 | Channelling | 25 kHz |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 10 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle max 1,0 % |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 73 | Edition 1 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications |  |
|  | 3 | Frequency band | 140–148,5 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 37.7 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 74 | Edition 1 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications |  |
|  | 3 | Frequency band | 148,5–5 000 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | – 15 dBμA/m at 10 metres in any bandwidth of 10 kHz Furthermore the total field strength is – 5 dΒμΑ/m at 10 m for systems operating at bandwidths larger than 10 kHz |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 75 | Edition 1 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications |  |
|  | 3 | Frequency band | 400–600 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | -8 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 76 | Edition 1 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications |  |
|  | 3 | Frequency band | 3 155–3 400 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 13.5 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 77 | Edition 1 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications |  |
|  | 3 | Frequency band | 5 000–30 000 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | – 20 dBμA/m at 10 metres in any bandwidth of 10 kHz Furthermore the total field strength is – 5 dΒμΑ/m at 10 m for systems operating at bandwidths larger than 10 kHz |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 78 | Edition 1 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications |  |
|  | 3 | Frequency band | 10 200–11 000 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 9 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 79 | Edition 1 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications | Only for RFID and EAS |
|  | 3 | Frequency band | 13 553–13 567 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 60 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 80 | Edition 1 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications |  |
|  | 3 | Frequency band | 26 957–27 283 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Active medical implants | Sub-class 81 | Edition 1 01.10.2008 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Active medical implants |  |
|  | 3 | Frequency band | 9–315 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 30 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules | Duty cycle max 10 % |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 195 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2008/432/EC |  |

| European Union | Radio Interface Specification | Active medical implants | Sub-class 82 | Edition 1 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Active medical implants | Only for ultra low power medical membrane implants for blood pressure measurements only |
|  | 3 | Frequency band | 30,0–37,5 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 1 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Duty cycle max 10 % |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 510 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Active medical implants and associated peripherals | Sub-class 83 | Edition 1 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Active medical implants and associated peripherals |  |
|  | 3 | Frequency band | 401–402 MHz |  |
|  | 4 | Channelling | Channel spacing: 25 kHz Individual transmitters may combine adjacent channels for increased bandwidth up to 100 kHz. |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 μW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle limit of 0,1 % may also be |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 537 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Active medical implants and associated peripherals | Sub-class 84 | Edition 1 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Active medical implants and associated peripherals |  |
|  | 3 | Frequency band | 405–406 MHz |  |
|  | 4 | Channelling | Channel spacing: 25 kHz Individual transmitters may combine adjacent channels for increased bandwidth up to 100 kHz. |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 μW e.r.p. |  |
|  | 8 | Channel access and occupation rules | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used. Alternatively a duty cycle limit of 0,1 % may also be |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 537 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Animal implantable devices | Sub-class 85 | Edition 1 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Animal implantable devices |  |
|  | 3 | Frequency band | 315–600 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | -5 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules | Duty cycle max 10 % |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 536 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Low power FM transmitters | Sub-class 86 | Edition 1 01.11.2010 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Low power FM transmitters |  |
|  | 3 | Frequency band | 87,5–108,0 MHz |  |
|  | 4 | Channelling | Channel spacing up to 200 kHz |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 50 nW e.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 301 357 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2010/368/EU |  |

| European Union | Radio Interface Specification | Radio determination applications | Sub-class 87 | Edition 1 01.11.2009 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Radio determination applications |  |
|  | 3 | Frequency band | 2 400–2 483,5 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 25 mW e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 440-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2009/381/EC |  |

| European Union | Radio Interface Specification | Radio determination applications | Sub-class 88 | Edition 1 01.11.2009 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Radio determination applications | Restricted to ground based systems |
|  | 3 | Frequency band | 17,1–17,3 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 26 dBm e.i.r.p. |  |
|  | 8 | Channel access and occupation rules | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 440-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2009/381/EC |  |

| European Union | Radio Interface Specification | Tank level probing radar | Sub-class 89 | Edition 1 01.11.2009 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Tank level probing radar |  |
|  | 3 | Frequency band | 4,5–7,0 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 24 dBm e.i.r.p. | The power limit applies inside a closed tank and corresponds to a spectral density of – 41,3 dBm/MHz e.i.r.p. outside a 500 litre test tank. |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 372 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2009/381/EC |  |

| European Union | Radio Interface Specification | Tank level probing radar | Sub-class 90 | Edition 1 01.11.2009 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Tank level probing radar |  |
|  | 3 | Frequency band | 8,5–10,6 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 30 dBm e.i.r.p. | The power limit applies inside a closed tank and corresponds to a spectral density of – 41,3 dBm/MHz e.i.r.p. outside a 500 litre test tank. |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 372 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2009/381/EC |  |

| European Union | Radio Interface Specification | Tank level probing radar | Sub-class 91 | Edition 1 01.11.2009 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Tank level probing radar |  |
|  | 3 | Frequency band | 24,05–27,0 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 43 dBm e.i.r.p. | The power limit applies inside a closed tank and corresponds to a spectral density of – 41,3 dBm/MHz e.i.r.p. outside a 500 litre test tank. |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 372 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2009/381/EC |  |

| European Union | Radio Interface Specification | Tank level probing radar | Sub-class 92 | Edition 1 01.11.2009 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Tank level probing radar |  |
|  | 3 | Frequency band | 57,0–64,0 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 43 dBm e.i.r.p. | The power limit applies inside a closed tank and corresponds to a spectral density of – 41,3 dBm/MHz e.i.r.p. outside a 500 litre test tank. |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 372 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2009/381/EC |  |

| European Union | Radio Interface Specification | Tank level probing radar | Sub-class 93 | Edition 1 01.11.2009 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Tank level probing radar |  |
|  | 3 | Frequency band | 75,0–85,0 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 43 dBm e.i.r.p. | The power limit applies inside a closed tank and corresponds to a spectral density of – 41,3 dBm/MHz e.i.r.p. outside a 500 litre test tank. |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 372 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2009/381/EC |  |

| European Union | Radio Interface Specification | Model control | Sub-class 94 | Edition 1 01.11.2009 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Model control |  |
|  | 3 | Frequency band | 26 990–27 000 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 100 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2009/381/EC |  |

| European Union | Radio Interface Specification | Model control | Sub-class 95 | Edition 1 01.11.2009 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Model control |  |
|  | 3 | Frequency band | 27 040–27 050 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 100 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2009/381/EC |  |

| European Union | Radio Interface Specification | Model control | Sub-class 96 | Edition 1 01.11.2009 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Model control |  |
|  | 3 | Frequency band | 27 090–27 100 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 100 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2009/381/EC |  |

| European Union | Radio Interface Specification | Model control | Sub-class 97 | Edition 1 01.11.2009 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Model control |  |
|  | 3 | Frequency band | 27 140–27 150 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 100 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2009/381/EC |  |

| European Union | Radio Interface Specification | Model control | Sub-class 98 | Edition 1 01.11.2009 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Model control |  |
|  | 3 | Frequency band | 27 190–27 200 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 100 mW e.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 220-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2009/381/EC |  |

| European Union | Radio Interface Specification | Private (Professional) Mobile Radio | Sub-class 99 | Edition 1 xxx |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | PMR446 Digital | End user terminals for voice only |
|  | 3 | Frequency band | 446.1 – 446.2 MHz | Carrier frequencies [MHz] for 12.5 kHz channeling: 446.106250; 446.118750; 446.131250; 446.143750; 446.156250; 446.168750; 446.181250; 446.193750 Carrier frequencies [MHz] for 6.25 kHz channeling: 446.103125; 446.109375; 446.115625; 446.121875; 446.128125; 446.134375; 446.140625; 446.146875; 446.153125; 446.159375; 446.165625; 446.171875; 446.178125; 446.184375; 446.190625; 446.196875. |
|  | 4 | Channelling | 6.25 kHz/ 12.5 kHz |  |
|  | 5 | Modulation / Occupied bandwidth | Digital modulation |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 500 mW e.r.p. | Integral antenna only |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 301 166 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification |  | Sub-class | Edition |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service |  |  |
|  | 2 | Application |  |  |
|  | 3 | Frequency band |  |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density |  |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference |  |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks |  |  |

| European Union | Radio Interface Specification | Radio frequency identification (RFID) | Sub-class 100 | Edition 1 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Radio frequency identification (RFID) |  |
|  | 3 | Frequency band | 2 446–2 454 MHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 500 mW e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 440-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Road transport and traffic telematics | Sub-class 101 | Edition 1 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Road transport and traffic telematics |  |
|  | 3 | Frequency band | 24,050–24,075 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 100 mW e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 858 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Road transport and traffic telematics | Sub-class 102 | Edition 1 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Road transport and traffic telematics |  |
|  | 3 | Frequency band | 24,075–24,150 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 0,1 mW e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 858 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Road transport and traffic telematics | Sub-class 103 | Edition 1 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Road transport and traffic telematics | Restricted to vehicle radars |
|  | 3 | Frequency band | 24,075–24,150 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth | Frequency modulation range apply as specified in harmonised standards |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 100 mW e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  | Techniques to access spectrum and mitigate interference that provide at least equivalent performance to the techniques described in harmonised standards adopted under Directive 1999/5/EC must be used Dwell time limits apply as specified in harmonised st |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 858 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Road transport and traffic telematics | Sub-class 104 | Edition 1 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Road transport and traffic telematics |  |
|  | 3 | Frequency band | 24,150–24,250 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 100 mW e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 858 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Road transport and traffic telematics | Sub-class 105 | Edition 1 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Road transport and traffic telematics | Restricted to vehicle-to-vehicle, vehicle-to-infrastructure and infrastructure-to- vehicle systems |
|  | 3 | Frequency band | 63–64 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 40 dBm e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 302 686 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Inductive applications | Sub-class 106 | Edition 1 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Inductive applications |  |
|  | 3 | Frequency band | 135–140 kHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 42 dBμA/m @ 10m |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 300 330-2 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 107 | Edition 1 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device |  |
|  | 3 | Frequency band | 122–123 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 100 mW e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 305 550 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |

| European Union | Radio Interface Specification | Non Specific Short Range Device | Sub-class 108 | Edition 1 01.06.2012 |
| --- | --- | --- | --- | --- |

|  | Nr | Parameter | Description | Comments |
| --- | --- | --- | --- | --- |
| Normative part | 1 | Radiocommunication Service | Mobile Service |  |
|  | 2 | Application | Non Specific Short Range Device |  |
|  | 3 | Frequency band | 244–246 GHz |  |
|  | 4 | Channelling |  |  |
|  | 5 | Modulation / Occupied bandwidth |  |  |
|  | 6 | Direction / Separation |  |  |
|  | 7 | Transmit power / Power density | 100 mW e.i.r.p. |  |
|  | 8 | Channel access and occupation rules |  |  |
|  | 9 | Authorisation regime |  |  |
|  | 10 | Additional essential requirements |  |  |
|  | 11 | Frequency planning assumptions |  |  |
| Informative part | 12 | Planned changes |  |  |
|  | 13 | Reference | EN 305 550 |  |
|  | 14 | Notification number |  |  |
|  | 15 | Remarks | Commission Decision 2011/829/EU |  |