

ERC/REC 70-03

Annex 3


Annex 3

Wideband Data Transmission systems

Scope of Annex
This annex covers frequency bands and regulatory as well as informative parameters recommended for Wideband Data Transmission Systems and Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) within the bands 2400-2483.5 MHz and 17.1-17.3 GHzand for Multiple-Gigabit WAS/RLAN Systems within the band 57-66 GHz.

Regulatory parameters related to Annex 3
	Frequency Band
	Power
	Spectrum access and mitigation requirement
	Channel spacing
	ECC/ERC Decision
	Notes

	a
	2400.0–2483.5 MHz
	100 mW e.i.r.p.
	Adequate spectrum sharing mechanism (e.g. Listen-before-Talk, Detect-And-Avoid) shall be implemented by the equipment.
	No spacing
	ERC/DEC/(01)07
	For wide band modulations other than FHSS, the maximum e.i.r.p. density is limited to 10 mW/MHz


	
	
	


	
	
	
	

	
	
	


	
	
	
	

	b
	17.1–17.3 GHz
	100 mW e.i.r.p.
	No requirement
	No spacing
	
	

	c
	57–66 GHz
	40 dBm 
mean e.i.r.p

This refers to the highest power level of the transmitter power control range during the transmission burst if transmitter power control is implemented.
	Adequate spectrum sharing mechanism (e.g. Listen-before-Talk, Detect-And-Avoid) shall be implemented by the equipment.

	No spacing
	
	Fixed outdoor installations are not allowed. 

The maximum mean e.i.r.p density is limited to 13 dBm/MHz
Point-to-point links of the Fixed Service are regulated by ECC/REC/(05)02 and ECC/REC/(09)01.






Additional Information

Harmonised Standards

EN 300 328
sub-band a)

sub-band b): t.b.d.
EN 302 567
sub-band c) .






Edition of November 2009
Edition of  2011

