ERC Recommendation (70-03)

Relating to the use of Short Range Devices (SRD)

Tromsø 1997

Subsequent amendments

22 August 2011

Please see the Document History at the end of this document for the revision status of individual annexes and appendices.

Please Note

Implementation status page 24

# Foreword

This Recommendation sets out the general position on common spectrum allocations for Short Range Devices (SRDs) for countries within the CEPT. It is also intended that it can be used as a reference document by the CEPT member countries when preparing their national regulations in order to keep in line with the provisions of the R&TTE Directive.

In using this Recommendation it should be remembered that it represents the most widely accepted position within the CEPT but it should not be assumed that all allocations are available in all countries.  An indication of where allocations are not available or where deviations from the CEPT position occur is to be found in Appendix 3.

It should also be remembered that the pattern of radio use is not static.  It is continuously evolving to reflect the many changes that are taking place in the radio environment; particularly in the field of technology.  Spectrum allocations must reflect these changes and the position set out in this Recommendation is therefore subject to continuous review.

Moreover, many administrations have designated additional frequencies or frequency bands for SRD applications on a national basis that do not conform to the CEPT position set out in this Recommendation.

For these reasons, those wishing to develop or market SRDs based on this Recommendation are advised to contact the relevant national administration to verify that the position set out herein still applies. Any inconsistencies between the national position stated in the implementation table in Appendix 1 of this Recommendation and those national positions stated elsewhere should be brought to the attention of the ECO (thomas.weber@eco.cept.org ) in order that these differences may be resolved.

When selecting parameters for new SRDs, which may have inherent safety of human life implications, manufacturers and users should pay particular attention to the potential for interference from other systems operating in the same or adjacent bands. Manufacturers should advice users on the risks of potential interference and its consequences.

TABLE OF CONTENTS

Foreword	2

introduction	4

ERC recommendation of 22 August 2011 on Relating to the use of short range devices (srd)	5

ANNEX 1: Non-specific Short Range Devices	6

ANNEX 2: Tracking, Tracing and Data Acquisition	9

ANNEX 3: Wideband Data Transmission systems	10

ANNEX 4: Railway applications	12

ANNEX 5: Road Transport and Traffic Telematics (RTTT)	14

ANNEX 6: Radiodetermination applications	16

ANNEX 7: Alarms	18

ANNEX 8: Model control	19

ANNEX 9: Inductive applications	20

ANNEX 10: Radio microphone applications including aids for the hearing impaired	23

ANNEX 11: Radio frequency identification applications	26

ANNEX 12: Active Medical Implants and their associated peripherals	28

ANNEX 13: Wireless Audio Applications	30

Appendix 1: Countries for Class 1 equipment	32

APPENDIX 2: List of relevant ECC/ERC Decisions, Reports, EC Decisions and ETSI Standards	40

Appendix 3 – National Restrictions	47

# introduction

CEPT has adopted this Recommendation to deal with Short Range Devices and the European Telecommunications Standards Institute (ETSI) has now developed harmonised European standards for the majority of these devices. Other standards or technical specifications will be applicable within the framework of the R&TTE Directive for placing on the market.

The term “Short Range Device” (SRD) is intended to cover the radio transmitters which provide either uni-directional or bi-directional communication and which have low capability of causing interference to other radio equipment.  SRDs use either integral, dedicated or external antennas and all modes of modulation can be permitted subject to relevant standards.  SRDs are not considered a “Radio Service” under the ITU Radio Regulations (Article 1).

This Recommendation describes the spectrum management requirements for SRDs relating to allocated frequency bands, maximum power levels, channel spacing and duty cycle.

For CEPT countries that have implemented the R&TTE Directive, Article 12 (CE-marking) and Article 7.2 on putting into service of radio equipment apply. Article 12 states that “any other marking may be affixed to the equipment provided that the visibility and legibility of the CE-marking is not hereby reduced” and Article. 7.2 states that “member states may restrict the putting into service of radio equipment only for reasons related to the effective and appropriate use of the radio spectrum, avoidance of harmful interference or matters relating to public health.”

"The CEPT has considered the use of SRD devices on board aircraft and it has concluded that, from the CEPT regulatory perspective, such use is allowed under the same conditions provided in the relevant Annex of Recommendation 70-03. For aviation safety aspects, the CEPT is not the right body to address this matter which remains the responsibility of aircraft manufacturers or aircraft owners who should consult with the relevant national or regional aviation bodies before the installation and use of such devices on board aircraft."

For Short Range Devices individual licenses are normally not required. Where licenses are required this is stated in the relevant Annex.

The following annexes define the regulatory parameters as well as additional information about harmonised standards, frequency issues and important technical parameters. Other technical parameters are indicated in the relevant standard.

Appendix 2 covers the relevant ECC/ERC Decisions and ETSI standards.

For countries having implemented the R&TTE Directive further details can be found on the relevant EC http://ec.europa.eu/enterprise/sectors/rtte/index_en.htm and the ECO web sites (www.cept.org/ecc).

Applications for certain short range devices within this recommendation are subject to EC Decisions including Decision 2006/771/EC and EU/EFTA Member States are obliged to implement the EC Decision in all these cases. These applications are identified by a footnote under “Additional Information” in the relevant Annex which also mentions any derogations that have been agreed. A list of relevant EC Decisions can be found in Appendix 2.

Member States of EU/EFTA may allow, at national level, equipment to operate under more permissive conditions than specified in the EC Decision if permitted by that EC Decision. However, in this case such equipment could not operate throughout the European Community without restrictions and would therefore be considered as ‘Class 2’ equipment under the classification in the 1999/5/EC (R&TTE) Directive.

# ERC recommendation of 22 August 2011 on Relating to the use of short range devices (srd)

“The European Conference of Postal and Telecommunications Administrations,

considering

that SRDs in general operate in shared bands and are not permitted to cause harmful interference to radio services;

that in general SRDs cannot claim protection from radio services;

that due to the increasing interest in the use of SRDs for a growing number of applications it is necessary to harmonise frequencies and regulations for these devices;

that there is a need to distinguish between different applications;

that additional applications and associated annexes will be added as necessary;

that for CEPT countries that have implemented the R&TTE Directive article 12 (CE marking) and article 7.2 on putting into service of radio equipment apply,

that equipment marketed before the adoption of this Recommendation marked with the abbreviation CEPT LPD Y according to the abrogated CEPT Recommendation T/R 01-04 should be allowed continuation of free circulation and use

that maintenance of Appendices 2 and 3 and also the related cross-references in the Annexes may be undertaken by the ECO based on information from Administrations,

that information about placing SRD equipment on the market and its use can be obtained by contacting individual administrations, especially with regard to equipment operating in frequencies or frequency bands that may be designated for SRDs by administrations in addition to those covered in this Recommendation;

that SRD equipment normally use either integral or dedicated antennas. In exceptional cases external antennas could be used which will be mentioned in the appropriate annex to this Recommendation;

that for those countries implementing the provisions of this Recommendation, national restrictions in respect of the annexes can be found in Appendix 3;

that EU/EFTA Member States are required to implement the EC Decisions listed in Appendix 2 of this recommendation and that for those countries a ”Y” indication in the implementation table means that the least restrictive regulatory parameters of any of  the respective EC Decisions listed in Appendix 2 applies. The parameters in the EC Decisions listed in Appendix 2 may be subject to a derogation for an individual country and this should be detailed in Appendix 3.

recommends

- that CEPT administrations implement the parameters in accordance with the indications mentioned in the annexes;

- that technical parameter limits should not be exceeded by any function of the equipment;

- that CEPT administrations should allow visitors from other countries to carry and use their equipment temporarily without any further formalities unless there are national restrictions as shown in Appendix 3.”

Note:

Please check the Office documentation database (http://www.ecodocdb.dk) for the up to date position on the implementation of this and other ECC/ERC deliverables

Non-specific Short Range Devices

Scope of Annex

This annex covers frequency bands and regulatory as well as informative parameters recommended primarily for Telemetry, Telecommand, Alarms and Data in general and other similar applications. Video applications should be preferably used above 2.4 GHz.

This annex also includes references to the generic UWB regulation which was primarily developed to allow communication applications using UWB technology in bands below 10.6 GHz; but enables also other types of radio applications.

Regulatory parameters

| Frequency Band |  | Power / Magnetic Field | Spectrum access and mitigation requirements | Channel spacing | ECC/ERC Deliverable | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 6765-6795 kHz | 42 dBµA/m at 10m | No requirement | No spacing | ECC Report 001 | The frequency band is also identified in Annex 9. |
| b | 13.553-13.567 MHz | 42 dBµA/m at 10m | No requirement | No spacing | ECC Report 001 | The frequency band is also identified in Annex 9. |
| c | 26.957-27.283 MHz | 42 dBµA/m at 10m 10 mW e.r.p | No requirement | No spacing |  | The frequency band is also identified in Annex 9. |
| d | 40.660-40.700 MHz | 10 mW e.r.p. | No requirement | No spacing |  |  |
| e | 138.20-138.45 MHz | 10 mW e.r.p. | < 1.0 % duty cycle (note 1) | No spacing |  |  |
| f | 433.050-434.790 MHz (note 4) | 10 mW e.r.p. | < 10 % duty cycle (note 1) | No spacing |  |  |
| f1 | 433.050-434.790 MHz (note 4bis) | 1 mW e.r.p. -13 dBm/10 kHz | (note 4bis) | No spacing |  | Power density limited to -13 dBm/10 kHz for wideband modulation with a bandwidth greater than 250 kHz |
| f2 | 434.040-434.790 MHz (note 4bis) | 10 mW e.r.p. | (note 4bis) | Up to 25 kHz |  |  |
| g | 863-870 MHz (note 3, and 4 ) | ≤ 25 mW e.r.p. | ≤ 0.1% duty cycle or LBT (note 1 and 5) | ≤ 100 kHz for 47 or more channels (note 2) | ECC Report 011 ECC Report 013 ECC Report 037 ECC Report 040 ERC Report 098 | FHSS modulation |
|  |  | ≤ 25 mW e.r.p. (note 6) Power density : - 4.5 dBm/100 kHz (note 7) | ≤ 0.1% duty cycle or LBT+AFA (note 1, 5 and 6) | No spacing |  | DSSS and other wideband modulation other than FHSS |
|  |  | ≤ 25 mW e.r.p. | ≤ 0.1% duty cycle or LBT+AFA (note 1 and 5) | ≤ 100 kHz, for 1 or more channelsmodulation bandwith ≤ 300 kHz (note 2) |  | Narrow /wide-band modulation |
| g1 | 868.000-868.600 MHz (note 4) | ≤ 25 mW e.r.p. | ≤ 1% duty cycle or LBT+AFA (note 1) | No spacing,for 1 or more channels (note 2) |  | Narrow / wide-band modulation. No channel spacing, however the whole stated frequency band may be used |
| g2 | 868.700-869.200 MHz (note 4) | ≤ 25 mW e.r.p. | ≤ 0.1% duty cycle or LBT+AFA (note 1) | No spacing,for 1 or more channels (note 2) |  | Narrow / wide-band modulation. No channel spacing, however the whole stated frequency band may be used |
| g3 | 869.400-869.650 MHz | ≤ 500 mW e.r.p. | ≤ 10% duty cycle or LBT+AFA (note 1) | 25 kHz (for 1 or more channels) |  | Narrow / wide-band modulation. The whole stated frequency band may be used as 1 channel for high speed data transmission |
| g4 | 869.700-870.000 MHz (note 4bis) | ≤ 5 mW e.r.p. ≤ 25 mW e.r.p. | No requirement up to 1% duty cycle or LBT+AFA (note 1) | No spacing (for 1 or more channels) |  | Narrow / wide-band modulation. No channel spacing, however the whole stated frequency band may be used |
| h | 2400.0-2483.5 MHz | 10 mW e.i.r.p. | No requirement | No spacing | ECC Report 011 ERC Report 109 | The frequency band is also identified in Annexes 3 and 6. |
| i | 5725-5875 MHz | 25 mW e.i.r.p. | No requirement | No spacing | ECC Report 002 ECC Report 068 |  |
| j | 24.00-24.25 GHz | 100 mW e.i.r.p. | No requirement | No spacing | ECC Report 002 | The frequency band is also identified in Annex 5. |
| k | 61.0-61.5 GHz | 100 mW e.i.r.p. | No requirement | No spacing |  |  |
| l | 122-123 GHz | 100 mW e.i.r.p. | No requirement | No spacing |  |  |
| m | 244-246 GHz | 100 mW e.i.r.p. | No requirement | No spacing |  |  |
| n | 3.1-4.8 GHz 6-9 GHz | * | * | * | ECC/DEC/(06)04 ECC Report 064 ECC Report 094 ECC Report 120 | Generic UWB regulation * See detailed requirements in related ECC Decision |

‘

Note 1:	When either a duty cycle, Listen Before Talk (LBT) or equivalent technique applies then it shall not be user dependent/adjustable and shall be guaranteed by appropriate technical means.

For LBT devices without Adaptive Frequency Agility (AFA), or equivalent techniques, the duty cycle limit applies.

For any type of frequency agile device the duty cycle limit applies to the total transmission unless LBT or equivalent technique is used.

Note 2:	The preferred channel spacing is 100 kHz allowing for a subdivision into 50 kHz or 25 kHz.

Note 3:	Sub-bands for alarms are excluded (see ERC/REC 70-03 Annex 7).

Note 4:	Audio and video applications are allowed provided that a digital modulation method is used with a max. bandwidth of 300 kHz.

Analogue and digital voice applications are allowed with a max. bandwidth≤ 25 kHz.

In sub-band 863-865 MHz voice and audio conditions of Annexes 10 and 13 of ERC/REC 70 – 03 apply respectively.

Note 4bis: Audio and video applications are excluded. Analogue or digital voice applications are allowed with a max. bandwidth≤ 25 kHz and with spectrum access technique such as LBT or equivalent. The transmitter shall include a power output sensor controlling the transmitter to a maximum transmit period of 1 minute for each transmission

Note 5:	Duty cycle may be increased to 1% if the band is limited to 865-868 MHz.

Note 6:	For other wide-band modulation than FHSS and DSSS with a bandwidth of 200 kHz to 3 MHz, duty cycle can be increased to 1% if the band is limited to 865-868 MHz and power to ≤10 mW e.r.p.

Note 7:	The power density can be increased to +6.2 dBm/100 kHz and -0.8 dBm/100 kHz, if the band of operation is limited to 865-868 MHz and 865-870 MHz respectively.

Additional Information

Harmonised Standards

EN 300 220	sub-bands c) to g4 )

EN 300 330	sub-bands a) to c)

EN 300 440	sub-bands h) i) and j)

EN 302 065	subband n)

EN 302 500	subband n).

Technical parameters also referred to in the harmonised standard

Listen before talk (LBT) with Adaptive Frequency Agility (AFA) technique feature may be used instead of duty cycle.

LBT is defined in EN 300 220.

Audio and voice are defined in EN 300 220.

Frequency issues

The bands in Annex 1 a - b - c - d   f - f1 - f2 - h - i - j - k - l and m are also designated for industrial, scientific and medical (ISM) applications as defined in ITU Radio Regulations.

Sub-band g)

Certain channels may be occupied by RFID operating at higher powers (See Annex 11 for further details). To minimise the risk of interference from RFID, SRDs should use LBT with AFA or observe suitable separation distances. (In the high power RFID channels typically these may vary from 918 m (indoor) to 3.6 km (rural outdoor). In the remaining 2.2 MHz, where tags at -20 dBm e.r.p. occupy the spectrum, this may vary from 24 m (indoor) to 58 m (rural outdoor)).

The adjacent frequency bands below 862 MHz and above 870 MHz may be used by high power systems. Manufacturers should take this into account in the design of equipment and choice of power levels

Tracking, Tracing and Data Acquisition

Scope of Annex

This annex covers frequency bands and regulatory as well as informative parameters recommended for a number of specific devices including:

Detecting avalanche victims;

Meter Reading;

Asset Tracking and Tracing.

Regulatory parameters

| Frequency Band |  | Power / Magnetic Field | Spectrum access and mitigation requirements | Channel spacing | ECC/ERC Deliverable | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 456.9-457.1 kHz | 7 dBµA/m at 10 m | No requirement | Continuous wave (CW) – no modulation. | ECC/DEC/(04)01 | Detection of avalanche victims. Note: Centre frequency is 457 kHz |
| b | 169.4-169.475 MHz | 500 mW e.r.p. | < 10% duty cycle | Max 50 kHz | ECC/DEC/(05)02 ECC Report 055 | Meter Reading |
| c | 169.4-169.475 MHz | 500 mW e.r.p. | < 1% duty cycle | Max 50 kHz | ECC/DEC/(05)02 ECC Report 055 | Asset Tracking and Tracing |

Additional Information

Harmonised Standards

EN 300 718	Sub-band a)

EN 300 220	Sub-bands b) and c)

Frequency issues

No information

Technical parameters also referred to in the harmonised standard

No information

Wideband Data Transmission systems

Scope of Annex

This annex covers frequency bands and regulatory as well as informative parameters recommended for Wideband Data Transmission Systems and Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) (formerly known as Radio Local Area Networks (RLANs)) within the band 2400-2483.5 MHz, for Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) within the bands 5150-5350 MHz, 5470-5725 MHz and 17.1-17.3 GHz and for Multiple-Gigabit WAS/RLAN Systems within the band 57-66 GHz.

Regulatory parameters

| Frequency Band |  | Power / Magnetic Field | Spectrum access and mitigation requirements | Channle spacing | ECC/ERC Deliverable | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 2400.0-2483.5 MHz | 100 mW e.i.r.p. | See note 1 | No spacing | ECC Report 011 ERC Report 001 ERC Report 109 | For wide band modulations other than FHSS , the maximum e.i.r.p. density is limited to 10 mW/MHz The frequency band is also identified in Annexes 1 and 6. |
| b | 5150-5350 MHz | 200 mW meane.i.r.p. See note 3 | See notes 1 and 2 | No spacing | ECC/DEC/(04)08 | Restricted to indoor use. The maximum mean e.i.r.p. density shall be limited to 10 mW/MHz in any 1 MHz band. |
| c | 5470-5725 MHz | 1 W meane.i.r.p. See note 3 | See notes 1 and 2 | No spacing | ECC/DEC/(04)08 | Indoor as well as outdoor use allowed. The maximum mean e.i.r.p. density shall be limited to 50 mW/MHz in any 1 MHz band. |
| d | 17.1-17.3 GHz | 100 mW e.i.r.p. | No requirement | No spacing |  | The frequency band is also identified in Annexes 6. |
| e | 57-66 GHz | 40 dBm meane.i.r.p | See note 1 | No spacing | ECC Report 113 ECC Report 114 | Fixed outdoor installations are not allowed. The maximum mean e.i.r.p density is limited to 13 dBm/MHz |

Note 1:	The equipment shall implement an adequate spectrum sharing mechanism in order to facilitate sharing between the various technologies and applications covered by this annex 3.

Note 2:	WAS/RLANs operating in the bands 5 250-5 350 MHz and 5 470-5 725 MHz shall use mitigation techniques that give at least the same protection as the detection, operation and response requirements described in EN 301 893 to ensure compatible operation with radiodetermination systems (radars).Such mitigation techniques shall equalise the probability of selectinga specific channel for all available channels so as to ensure, on average, a near-uniform spread of spectrum loading.Specific information about the applicability of EN 301 893 can be found at http://ec.europa.eu/comm/enterprise/rtte/harstand.htm.

Note 3:	WAS/RLANs operating in the bands 5 250-5 350 MHz and 5 470-5 725 MHz shall employ transmitter power control (TPC), which provides, on average, a mitigation factor of at least 3 dB on the maximum permitted output power of the systems. If transmitter power control is not in use, the maximum permitted mean e.i.r.p. and the corresponding mean e.i.r.p. density limits shall be reduced by 3 dB.

Additional Information

Harmonised Standards

EN 300 328	sub-band a)

EN 301 893	sub-bands b), and c) sub-band d): t.b.d.

EN 302 567	sub-bands e).

Frequency issues

Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) within the bands 5250-5350 MHz and 5470-5725 MHz shall only be allowed to operate when the mandatory features  required in the ECC Decision (04)08 are implemented.  See also note 1 above.

In the band 57-66 GHz, point-to-point links of the Fixed Service are regulated by ECC/REC/(05)02 and ECC/REC/(09)01.

Technical parameters also referred to in the harmonised standard

The power levels for band b), c), e) and f) refer to mean e.i.r.p. The mean e.i.r.p. refers to the highest power level of the transmitter power control range during the transmission burst if transmitter power control is implemented.

In bands a), b) and c), the adequate spectrum sharing mechanism referred to in Note 1 can be e.g. LBT (Listen Before Talk), DAA (Detect And Avoid) or any other mechanism providing a similar level of mitigation.

Railway applications

Scope of Annex

This annex covers frequency bands and regulatory as well as informative parameters recommended for applications specifically intended for use on railways.

The sub-bands below are intended for the following applications:

- band a) Automatic vehicle identification systems for railways including Automatic Vehicle Identification for Railways (AVI);

- band b) Balise tele-powering and down-link (train to ground) systems including Eurobalise and activation of the Loop / Euroloop;

- band c) Balise up-link (ground to train) systems including Eurobalise;

- band d) Loop up-link (ground to train) systems including Euroloop.

Regulatory parameters

| Frequency Band |  | Power / Magnetic Field | Spectrum access and mitigation requirements | Channel spacing | ECC/ERC Deliverable | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 2446-2454 MHz | 500 mW e.i.r.p. | No requirement |  | ECC Report 011 | Transmitting only in presence of trains. 5 channels, each 1.5 MHz wide within the band 2446-2454 MHz The frequency band is also identified in Annex 11. |
| b | 27.090-27.100 MHz | 42 dBµA/m at 10 m | No requirement | No spacing | ECC Report 098 | Tele-powering and Down-link signal for Balise / Eurobalise. May also be optionally used for the activation of the Loop / Euroloop. Note:Centre frequency is 27.095 MHz The frequency band is also identified in Annex 1. |
| c | 984-7484 kHz | 9 dBµA/m at 10m | <1% duty cycle | No spacing | ECC Report 098 | Transmitting only on receipt of a Balise / Eurobalise tele-powering signal from a train. Note:Centre frequency is 4234 kHz |
| d | 7.3-23.0 MHz | -7 dBµA/m at 10m | No requirement | No spacing | ECC Report 098 | Maximum field strength specified in a bandwidth of 10 kHz, spatially averaged over any 200m length of the loop. Transmitting only in presence of trains. Spread Spectrum Signal, Code Length: 472 Chips. Note: Centre frequency is 13.547 MHz |

Additional Information

Harmonised Standards

EN 300 761	sub-band a)

EN 302 608	sub-bands b) and c)

EN 300 330	sub-bands b), c),

EN 302 609	sub-band d)

Frequency issues

No information

Road Transport and Traffic Telematics (RTTT)

Scope of Annex

This annex covers frequency bands and regulatory as well as informative parameters recommended for Road Transport and Traffic Telematics (RTTT) including radar system installations to be used in vehicles.

Regulatory parameters

| Frequency Band |  | Power / Magnetic Field | Spectrum access and mitigation requirements | Channel spacing | ECC/ERC Deliverable | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 5795-5805 MHz | 2 We.i.r.p. 8 W e.i.r.p. | No requirement |  | ECC/DEC/(02)01 ECC Report 068 ERC Report 003 |  |
| b | 5805-5815 MHz | 2 We.i.r.p. 8 W e.i.r.p. | No requirement |  | ECC/DEC/(02)01 ECC Report 068 ERC Report 003 | Individual license required |
| c | 63-64 GHz |  |  | No spacing | ECC/DEC/(02)01 ECC Report 113 ECC Report 114 ERC Report 003 | Vehicle to vehicle and road to vehicle systems Power level and Spectrum access and mitigation requirement to be determined |
| d | 76-77 GHz | 55 dBm peak e.i.r.p. | No requirement | No spacing | ECC/DEC/(02)01 ERC Report 003 | Power level 55 dBm peak power e.i.r.p. 50 dBm average power - 23.5 dBm average power for pulse radar only Vehicle and infrastructure radar systems |
| e | 21.65-26.65 GHz | * | * | * | ECC/DEC/(04)10 ECC Report 023 | For automotive Short Range Radars (SRR) * See detailed requirements in related ECC DecisionNew SRR equipment may only be placed onto the market until 1 July 2013 |
| f | 77-81 GHz | * | * | * | ECC/DEC/(04)03 ECC Report 056 | For automotive Short Range Radars (SRR) * See detailed requirements in related ECC Decision |
| g1 | 24.050-24.075 GHz | 100 mW e.i.r.p. | No requirement |  | ECC Report 134 | For vehicle radars |
| g2 | 24.075-24.150 GHz | 0.1mW e.i.r.p. | No requirement |  |  | For vehicle radars |
|  |  | 100 mW e.i.r.p. | 4µs/40kHz dwell time every 3ms (note 1) |  |  | For vehicle radars. The spectrum access and mitigation requirementis given for devices mounted behind a bumper. If mounted without a bumper, the requirement should be 3µs/40kHz maximum dwell time every 3ms |
|  |  |  | 1ms/40kHz dwell time every 40ms (note 1) |  |  | The spectrum access and mitigation requirement is given for devices mounted either behind a bumper or mounted without a bumper |
| g3 | 24.150-24.250 GHz | 100mW e.i.r.p. | No requirement |  |  | For vehicle radars |
| The frequency band 24.00-24.50 GHz (g1 to g3) is also identified in Annex 1. Vehicular radars shall use the specific harmonized standard EN 300 858 or equivalent technical specifications. |  |  |  |  |  |  |

Note 1: A requirement for minimum frequency modulation range (applicable to FMCW or step frequency signals) or minimum instantaneous bandwidth (applicable to pulsed signal) of 250 kHz applies in addition to the requirement on maximum dwell time.

Additional Information

Harmonised Standards

EN 300 674	sub-bands a) and b)

ES 200 674	sub-bands a) and b)

EN 302 686	sub-band c)

EN 301 091	sub-band d)

EN 302 288	sub-band e)

EN 302 264	sub-band f)

EN 300 858	for sub-bands g1), g2) and g3).

Frequency issues

The frequency band a) is intended for road to vehicle systems, particularly (but not exclusively) road toll systems.

The frequency band a) and b) are recommended for 5 MHz channel spacing systems with the frequencies: 5797.5 MHz, 5802.5 MHz, 5807.5 MHz and 5812.5 MHz. For 10 MHz channel spacing systems 5800 MHz and 5810 MHz.

5805 - 5815 MHz on a national basis for multi-lane road junctions, particularly, but not exclusively road toll systems.

The use of 8 W e.i.r.p. allows for 1 Mbit/s  in accordance with ETSI standard ES 200 674-1.

2W e.i.r.p. allows for 500 kbit/s downlink and 250 kbit/s uplink in accordance with EN 300 674-1 and for low data rates (31 kbit/s) in accordance with EN 300 674-2.

Technical parameters also referred to in the harmonised standard

No information

Radiodetermination applications

Scope of Annex

This annex covers frequency bands and regulatory as well as informative parameters recommended for SRD radiodetermination applications including SRD radar systems, Equipment for Detecting Movement and Alert. Radiodetermination is defined as the determination of the position, velocity and/or other characteristics of an object, or the obtaining of information relating to these parameters, by means of the propagation properties of radio waves.

Regulatory parameters

| Frequency Band |  | Power / Magnetic Field | Spectrum access and mitigation requirements | Channel spacing | ECC/ERC Deliverable | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 2400.0-2483.5 MHz | 25 mW e.i.r.p. | No requirement | No spacing | ERC/DEC/(01)08 ECC Report 011 ERC Report 109 | The frequency band is also identified in Annexes 1 and 3. |
| b | 9200-9500 MHz | 25 mW e.i.r.p. | No requirement | No spacing |  |  |
| c | 9500-9975 MHz | 25 mWe.i.r.p. | No requirement | No spacing |  |  |
| d | 10.5-10.6 GHz | 500 mWe.i.r.p. | No requirement | No spacing | ERC Report 047 |  |
| e | 13.4-14.0 GHz | 25 mWe.i.r.p. | No requirement | No spacing |  |  |
| f | 24.05-24.25 GHz | 100 mWe.i.r.p. | No requirement | No spacing |  |  |
| g | 4.5-7.0 GHz | -41.3 dBm/MHz e.i.r.p. | No requirement | No spacing |  | Tank Level Probing Radar (TLPR) |
| h | 8.5-10.6 GHz | -41.3 dBm/MHz e.i.r.p. | No requirement | No spacing |  | Tank Level Probing Radar (TLPR) |
| i | 24.05-27.00 GHz | -41.3 dBm/MHz e.i.r.p. | No requirement | No spacing |  | Tank Level Probing Radar (TLPR) |
| j | 57-64 GHz | -41.3 dBm/MHz e.i.r.p. | No requirement | No spacing |  | Tank Level Probing Radar (TLPR) |
| k | 75-85 GHz | -41.3 dBm/MHz e.i.r.p. | No requirement | No spacing |  | Tank Level Probing Radar (TLPR) |
| l | 17.1-17.3 GHz | +26 dBm e.i.r.p. | DAA | No spacing | ECC Report 111 | Ground Based Synthetic Aperture Radar (GBSAR) (note 1) The frequency band is also identified in Annex 3. |
| m | 30 MHz – 12.4 GHz | * | * | * | ECC/DEC/(06)08 | For Ground- and Wall- Probing Radar (GPR/WPR) imaging systems, subject to an appropriate licensing regime * See detailed requirements in related ECC Decision |
| n | 2.2-8 GHz | * | * | * | ECC/DEC/(07)01 | For Building Material Analysis (BMA) devices. * See detailed requirements in related ECC Decision. |

Note 1:Specific requirements for the radar antenna pattern and for the implementation of Detect And Avoid (DAA) technique apply as described in EN 300 440 for Ground Based Synthetic Aperture Radar (GBSAR) systems.

Additional Information

Harmonised Standards

EN 300 440 	sub-bands a), b), c), d), e), f), l)

EN 302 372	(for TLPR) sub-bands g), h), i), j), k)

EN 302 066	sub-band m)

EN 302 435	sub-band n)

Frequency issues

Sub-bands a), b), c), d), e) and f)

Some countries may allow equipment with transmitter powers between 25 mW and 500 mW in which case an individual licence or a general licence may be required.

Technical parameters also referred to in the harmonised standard

Sub-bands g), h), i), j) and k) are to be used by TLPR equipment only.
The power limit is the radiated emission outside an enclosed tank structure.  
The maximum emission inside an enclosed tank structure is given in EN 302 372.

Sub-band h)

For the frequency range 10.6 GHz to 10.7 GHz, the radiated unwanted radiated emissions outside the tank enclosure shall be less than -60 dBm/MHz e.i.r.p.

Alarms

Scope of Annex

This annex covers frequency bands and regulatory as well as informative parameters recommended exclusively for alarm systems including social alarms and alarms for security and safety.

The sub-bands below are intended for the following applications:

- Alarms insub-bands a), b) ,c) and e);

- Social Alarms sub-bands d), f) and g).

Regulatory parameters

| Frequency Band |  | Power / Magnetic Field | Spectrum access and mitigation requirements | Channel spacing | ECC/ERC Deliverable | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 868.6-868.7 MHz | 10 mW e.r.p. | < 1.0 % duty cycle | 25 kHz | ECC Report 011 ECC Report 013 ECC Report 037 ECC Report 040 ERC Report 098 | The whole frequency band may also be used as1 channel for high speed data transmissions |
| b | 869.250-869.300 MHz | 10 mW e.r.p. | < 0.1 % duty cycle | 25 kHz |  |  |
| c | 869.650-869.700 MHz | 25 mW e.r.p. | < 10 % duty cycle | 25 kHz |  |  |
| d | 869.200-869.250 MHz | 10 mW e.r.p. | < 0.1 % duty cycle | 25 kHz |  | Social Alarms |
| e | 869.300-869.400 MHz | 10 mW e.r.p. | < 1.0 % duty cycle | 25 kHz |  |  |
| f | 169.4750-169.4875 MHz | 10 mW e.r.p. | < 0.1 % duty cycle | 12.5 kHz | ECC/DEC/(05)02 ECC Report 055 | Social Alarms (exclusive use) |
| g | 169.5875-169.6000 MHz | 10 mW e.r.p. | < 0.1 % duty cycle | 12.5 kHz | ECC/DEC/(05)02 ECC Report 055 | Social Alarms (exclusive use) |

Additional Information

Harmonised Standards

EN 300 220

Frequency issues

No information

Technical parameters also referred to in the harmonised standard

No information

Model control

Scope of Annex

This annex covers frequency bands and regulatory as well as informative parameters recommended for the application of model control equipment, which is solely for the purpose of controlling the movement of the model, in the air, on land or over or under the water surface. Although the bands are not harmonised, the parameters given in the table are common in a majority of CEPT countries. It should be noted that the bands are not exclusive for this type of application.

Regulatory parameters

| Frequency Band |  | Power / Magnetic Field | Spectrum access and mitigation requirements | Channel spacing | ECC/ERC Deliverable | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 26.995, 27.045, 27.095, 27.145, 27.195 MHz | 100 mW e.r.p | No requirement | 10 kHz |  |  |
| b | 34.995-35.225 MHz | 100 mW e.r.p | No requirement | 10 kHz | ERC/DEC/(01)11 | Only for flying models |
| c | 40.665, 40.675, 40.685, 40.695 MHz | 100 mW e.r.p | No requirement | 10 kHz | ERC/DEC/(01)12 |  |

Additional Information

Harmonised Standards

EN 300 220

Frequency issues

No information

Technical parameters also referred to in the harmonised standard

No information

Inductive applications

Scope of Annex

This annex covers frequency bands and regulatory as well as informative parameters recommended for inductive applications include for example car immobilisers, animal identification, alarm systems, cable detection, waste management, personal identification, wireless voice links, access control, proximity sensors, data transfer to handheld devices, automatic article identification, wireless control systems, automatic road tolling and anti-theft systems including RF anti-theft induction systems. It should be noted that other types of anti-theft systems can be operated in accordance with other relevant annexes.

Regulatory parameters

| Frequency Band |  | Power / Magnetic Field | Spectrum access and mitigation requirements | Channel spacing | ECC/ERC Deliverable | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a1 | 9 - 90 kHz | 72 dBµA/m at 10m Limits are reduced to 42 dBµA/m at 10 m as shown in table 10) | No requirement | No spacing | ECC Report 135 ERC Report 044 ERC Report 069 | In case of external antennas only loop coil antennas may be employed. Field strength level descending 3 dB/oct at 30 kHz |
| a2 | 90-119 kHz | 42 dBµA/m at 10m | No requirement | No spacing | ECC Report 135 ERC Report 044 ERC Report 069 | In case of external antennas only loop coil antennas may be employed |
| a3 | 119-135 kHz | 66 dBµA/m at 10m Limits are reduced to 42 dBµA/m at 10 m as shown in table 10 | No requirement | No spacing | ECC Report 135 ERC Report 044 ERC Report 069 | In case of external antennas only loop coil antennas may be employed.Field strength level descending 3 dB/oct at 119 kHz |
| b | 135-140 kHz | 42 dBµA/m at 10m | No requirement | No spacing | ECC Report 001 ECC Report 007 ECC Report 135 ERC Report 069 | In case of external antennas only loop coil antennas may be employed |
| c | 140-148.5 kHz | 37.7 dBµA/m at 10m | No requirement | No spacing | ECC Report 001 ECC Report 007 ECC Report 135 ERC Report 069 | In case of external antennas only loop coil antennas may be employed |
| d | 6765-6795 kHz | 42 dBµA/m at 10m | No requirement | No spacing | ECC Report 001 | The frequency band is also identified in Annex 1. |
| e | 7400-8800 kHz | 9 dBµA/m at 10m | No requirement | No spacing | ECC Report 001 ERC Report 069 |  |
| f | 13.553-13.567 MHz | 42 dBµA/m at 10m | No requirement | No spacing | ERC Report 069 | The frequency band is also identified in Annex 1. |
| f1 | 13.553-13.567 MHz | 60 dBµA/m at 10m | No requirement | No spacing | ECC Report 001 ERC Report 069 ERC Report 074 | For RFID and EAS only |
| g | 26.957-27.283 MHz | 42 dBµA/m at 10m | No requirement | No spacing | ERC Report 069 | The frequency band is also identified in Annex 1. |
| h | 10.200-11.000 MHz | 9 dBµA/m at 10m | No requirement | No spacing | ERC Report 069 ERC Report 092 |  |
| k | 3155-3400 kHz | 13.5 dBµA/m at 10m | No requirement | No spacing | ERC Report 069 ERC Report 095 | In case of external antennas only loop coil antennas may be employed |
| l1 | 148.5 kHz - 5 MHz | -15 dBµA/m at 10 m | No requirement | No spacing | ECC Report 067 ERC Report 069 ERC Report 096 | In case of external antennas only loop coil antennas may be employed. The maximum field strength is specified in a bandwidth of 10 kHz.The maximum allowed total field strength is -5 dBµA/m at 10 m for systems operating at bandwidths larger than 10 kHz whilst keeping the density limit (-15 dBµA/m in a bandwidth of 10 kHz) |
| l2 | 5 - 30 MHz | -20 dBµA/m at 10 m | No requirement | No spacing | ECC Report 067 ERC Report 069 | In case of external antennas only loop coil antennas may be employed. The maximum specified in a bandwidth of 10 kHz.The maximum allowed total field strength is -5 dBµA/m at 10 m for systems operating at bandwidths larger than 10 kHz whilst keeping the density limit (-20 dBµA/m in a bandwidth of 10 kHz) |
| l3 | 400 - 600 kHz | -8 dBµA/m at 10 m | No requirement | No spacing | ERC Report 069 ERC Report 096 | For RFID only. In case of external antennas only loop coil antennas may be employed. The maximum field strength is specified in a bandwidth of 10 kHz. The maximum allowed total field strength is -5dBµA/m at 10 m for systems operating at bandwidths larger than 10 kHz measured at the centre frequency whilst keeping the density limit (-8dBµA/m in a bandwidth of 10 kHz.) These systems should operate with a minimum operating bandwidth of 30 kHz |

Standard frequency and time signals to be protected within 9 - 90 kHz and 119 - 135 kHz

| Stations | Frequency | Protection bandwidth | Maximum field strength at 10 m | Location |
| --- | --- | --- | --- | --- |
| MSF | 60 kHz | +/-250Hz | 42 dBµA/m | United Kingdom |
| RBU | 66.6 kHz | +/-750Hz | 42 dBµA/m | Russian Federation |
| HBG | 75 kHz | +/-250Hz | 42 dBµA/m | Switzerland |
| DCF77 | 77.5 kHz | +/-250Hz | 42 dBµA/m | Germany |
| DCF49 | 129.1 kHz | +/-500Hz | 42 dBµA/m | Germany |

Additional Information

Harmonised Standards

EN 300 330 for all sub-bands

EN 302 291 sub-band f).

Frequency issues

Users should be aware that emissions from inductive applications could cause interference to nearby receivers of other radio services.

In case of loop antennas used within bands aa) and ac) integral or dedicated within an area between 0.05 m2 and 0.16 m2, the field strength is reduced by 10 * log (area/0.16 m2); for an antenna area less than 0.05 m2 the field strength is reduced by 10 dB.

Particular attention should also be paid to the more stringent protection requirements identified by the ITU for global distress and safety communications frequencies in the same or adjacent bands.

Technical parameters also referred to in the harmonised standard

Sub-band a3)

RFIDs operating in the frequency sub-band 119-135 kHz shall meet the spectrum mask given in EN 300 330. This will permit a simultaneous use of the various sub-bands within the range 90 – 148.5 kHz.

Radio microphone applications including aids for the hearing impaired

Scope of Annex

This annex covers frequency bands and regulatory as well as informative parameters recommended for radio microphone applications (also referred to as wireless microphones or cordless microphones) including aids for the hearing impaired (also referred to as assistive listening devices). Radio microphones are small, low power (typically 50 mW or less) transmitters designed to be worn on the body, or hand held, for the transmission of sound. The receivers are more tailored to specific uses and may range from small and portable to rack mounted modules as part of a multichannel system. This annex covers professional and consumer radio microphones, both hand-held and body-worn, and aids for the hearing impaired.

Because of the difficulty in determining harmonised frequency bands for radio microphones, frequency band limits should be regarded as tuning ranges within which a device can be designated to operate. In most cases, Appendix 3 indicates those parts of the  range that are not available in individual countries but this does not apply to the broadcasting bands at 174-216 MHz and 470-862 MHz where national geographical and licensing restrictions are likely to exist and the national administration should be contacted.

The sub-bands below are intended for the following applications:

- Aids for the hearing impaired: sub-bands b), c), d), h1), h2), i)

- Radio microphones: sub-bands a), c), d), e1), e2), e3), e4), f), g).

Aids for the hearing impaired are specific radio microphone applications which capture an acoustic signal that is transmitted by radio to the hearing aid receivers.

Regulatory parameters

| Frequency Band |  | Power / Magnetic Field | Spectrum access and mitigation requirements | Channel spacing | ECC/ERC Delieverable | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 29.7-47.0 MHz | 10 mW e.r.p. | No requirement | 50 kHz | ERC Report 042 | On a tuning range basis The frequency bands 30.3-30.5 MHz, 32.15-32.45 MHz and 41.015-47.00 MHz are harmonised military bands |
| b | 173.965-174.015 MHz | 2 mW e.r.p. | No requirement | 50 kHz | ERCReport 042 | Aids for the hearing impaired |
| c | 863-865 MHz | 10 mW e.r.p. | No requirement | No spacing | ECC Report 011 ECC Report 013 ECC Report 037 ECC Report 040 ERC Report 42 ERC Report 098 | The frequency band is also identified in Annexes 1 and 13. |
| d | 174-216 MHz | 50 mW e.r.p. | No requirement | No spacing | ERC Report 042 | On a tuning range basis. Individual licence required |
| e1 | 470-786 MHz | 50 mW e.r.p. | No requirement | No spacing | ECC Report 002 ERC Report 042 ERC Report 088 | On a tuning range basis. Individual licence required |
| e2 | 786-789 MHz | 12 mW e.r.p. | No requirement | No spacing |  | On a tuning range basis. Individual licence required. See technical conditions for PMSE (including radio microphones) in Annex 3 of Decision ECC/DEC/(09)03 section 3.1. |
| e3 | 823-826 MHz | 20 mW e.i.r.p. 100 mW e.i.r.p. | No requirement | 200 kHz |  | Individual licence required. 100 mW restricted to body worn microphones. See technical conditions for PMSE (including radio microphones) in Annex 3 of Decision ECC/DEC/(09)03 section 3.1. |
| e4 | 826-832 MHz | 100 mW e.i.r.p. | No requirement | 200 kHz |  | Individual licence required. See technical conditions for PMSE (including radio microphones) in Annex 3 of Decision ECC/DEC/(09)03 section 3.1. |
| f | 1785-1795 MHz | 20 mW e.i.r.p. 50 mW e.i.r.p. | No requirement | No spacing | ECC Report 002 ERC Report 042 ERC Report 063 | Individual licence required. 50 mW restricted to body worn microphones |
| g | 1795-1800 MHz | 20 mW e.i.r.p. 50 mW e.i.r.p. | No requirement | No spacing |  | 50 mW restricted to body worn equipment |
| h1 | 169.4000-169.4750 MHz | 10 mW e.r.p. | No requirement | Max 50 kHz | ECC/DEC/(05)02 ECC Report 055 | Aids for the hearing impaired (Personal Hearing Aid System) |
|  |  | 500 mW e.r.p. | No requirement | Max 50 kHz |  | Aids for the hearing impaired |
| h2 | 169.4875-169.5875 MHz | 10 mW e.r.p. | No requirement | Max 50 kHz |  | Aids for the hearing impaired (Personal Hearing Aid System) |
|  |  | 500 mW e.r.p. | No requirement | Max 50 kHz |  | Aids for the hearing impaired (Public Hearing Aid System) Individual licence may be required. |
| i | 169.4-174.0 MHz | 10 mW e.r.p. | No requirement | Max 50 kHz | ECC Report 055 | Aids for the hearing impaired On a tuning range basis Administrations should consider channel plan for band 169.4 - 169.8125 MHz detailed in ECC/DEC/(05)02 and the risk of interference towards systems operated in the band 169.6 - 169.8125 MHz when developing their national frequency table |

Additional Information

Harmonised Standards

EN 300 422	all sub-bands

EN 301 357	sub-band c)

Frequency Issues

Sub-band d)

Some countries may allow radio microphones and aids for the hearing impaired to operate in parts of this band with maximum transmitter power of 10 mW e.r.p. and without individual licence. Detailed information can be obtained from national administrations.

Sub-bands e2), e3), e4):

Some national administrations which have not introduced mobile/fixed communication networks (MFCN) in accordance with Decision ECC/DEC/(09)03 may authorise larger parts or the whole of the band 786 – 862 MHz to be used by radio microphones.

Technical parameters also referred to in the harmonised standard

No information

Radio frequency identification applications

Scope of Annex

This annex covers frequency bands and regulatory as well as informative parameters recommended for radio frequency identification (RFID) applications including for example automatic article identification, asset tracking, alarm systems, waste management, personal identification, access control, proximity sensors, anti-theft systems, location systems, data transfer to handheld devices and wireless control systems. It should be noted that other types of RFID systems can be operated in accordance with other relevant annexes.

Regulatory parameters

| Frequency Band |  | Power / Magnetic Field | Spectrum access and mitigation requirements | Channel spacing | ECC/ERC Deliverable | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a1 | 2446-2454 MHz | ≤500 mW e.i.r.p. | No requirement | No spacing | ERC Report 109 | The frequency band is also identified in Annexes 4. |
| a2 | 2446-2454 MHz | >500 mW-4 W e.i.r.p | ≤ 15% duty cycle FHSS techniques should be used | No spacing | ERC Report 109 | Power levels above 500 mW are restricted to be used inside the boundaries of a building and the duty cycle of all transmissions shall in this case be ≤15 % in any 200 ms period (30 ms on /170 ms off). The frequency band is also identified in Annexes 4. |
| b1 | 865.0-865.6 MHz | 100 mW e.r.p. | No requirement | 200 kHz | ECC Report 011 ECC Report 013 ECC Report 037 ECC Report 040 ERC Report 098 | The frequency band is also identified in Annex 1. |
| b2 | 865.6-867.6 MHz | 2 W e.r.p. | No requirement | 200 kHz |  | The frequency band is also identified in Annex 1. |
| b3 | 867.6-868.0 MHz | 500 mW e.r.p. | No requirement | 200 kHz |  | The frequency band is also identified in Annex 1. |

Additional Information

Harmonised Standards

EN 300 440 	Sub-band a)

EN 302 208 	Sub-bands b1), b2) and b3).

Frequency issues

Sub-band a)

To assist enforcement authorities any emissions due to the RFID device when measured outside of the building at a distance of 10 metres shall not exceed the equivalent field strength for a 500 mW RFID device mounted outside the building when measured at the same distance. Where a building consists of a number of premises, such as shops within a shopping arcade or Mall then the measurements shall be referenced to the boundary of the user's premises within the building.

Sub-bands b1), b2) and b3)

Channel centre frequencies are 864.9 MHz + (0.2 MHz * channel number).

The available channel numbers for each sub-band are:

b1: channel numbers 1 to 3;

b2: channel numbers 4 to 13;

b3: channel numbers 14 to 15.

Note: The same equipment is allowed to operate in several sub-bands.

Frequency hopping or other spread spectrum techniques shall not be used.

Technical parameters also referred to in the harmonised standard

Sub-band a)

In addition, antenna beamwidth limits shall be observed as described in the standard EN 300 440.

In addition, for an RFID device which can exceed 500 mW, the device should be fitted with an automatic power control to reduce the radiated power below 500 mW; this automatic power control shall guarantee the reduction of the power to a maximum of 500 mW in cases where the device is moved and used outside the boundary of the user's building or premises as described above.

Active Medical Implants and their associated peripherals

Scope of Annex

This annex covers frequency bands and regulatory as well as informative parameters recommended for Active Medical Implants and their associated peripherals.

Regulatory parameters

| Frequency Band |  | Power / Magnetic Field | Spectrum access and mitigation requirements | Channel spacing | ECC/ERC Deliverable | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 402-405 MHz | 25 µW e.r.p. | The equipment shall implement a spectrum access mechanism as described in the applicable harmonized standard or an equivalent spectrum access mechanism | 25 kHz | ERC/DEC/(01)17 ECC Report 012 | For Ultra Low Power Active Medical Implants covered by the applicable harmonised standard. Individual transmitters may combine adjacent channels for increased bandwidth up to 300 kHz. |
| a1 | 401-402 MHz | 25 µW e.r.p. Systems not providing LBT are limited to 250 nW e.r.p. | LBT or duty cycle ≤0.1% | 25 kHz | ECC Report 092 | For Ultra Low Power Active Medical Implants and accessories covered by the applicable harmonised standard and not covered by band a. Individual transmitters may combine adjacent 25 kHz channels for increased bandwidth up to 100 kHz. |
| a2 | 405-406 MHz | 25 µW e.r.p. Systems not providing LBT are limited to 250 nW e.r.p. | LBT or duty cycle ≤0.1% | 25 kHz | ECC Report 092 | For Ultra Low Power Active Medical Implants and accessories covered by the applicable harmonised standard and not covered by band a. Individual transmitters may combine adjacent 25 kHz channels for increased bandwidth up to 100 kHz. |
| b | 9-315 kHz | 30 dBµA/m at 10m | < 10% | No spacing | ECC Report 001 | The application is for Ultra Low Power Active Medical Implant systems using inductive loop techniques for telemetry purposes The frequency band is also identified in Annex 9. |
| c | 315-600 kHz | -5 dBµA/m at 10m | < 10% | No spacing |  | The application is for animal implantable devices. |
| d | 30.0-37.5 MHz | 1 mW e.r.p. | < 10% | No spacing |  | The application is for Ultra Low Power medical membrane implants for blood pressure measurements. The frequency band is also identified in Annex 9. |
| e | 12.5-20.0 MHz | -7 dBµA/m at 10m | < 10% duty cycle | No spacing | ECC Report 081 | The application is for ULP active animal implantable devices (ULP-AID), limited to indoor only applications. The maximum field strength is specified in a bandwidth of 10 kHz. The transmission mask of ULP-AID is defined as follows: 3dB bandwidth 300 kHz 10dB bandwidth 800 kHz 20dB bandwidth 2 MHz. The frequency band is also identified in Annex 9. |
| f | 2483.5-2500 MHz | 10 dBm e.i.r.p | LBT+AFA and < 10% duty cycle. The equipment shall implement a spectrum access mechanism as described in the applicable harmonized standard or an equivalent spectrum access mechanism | 1MHz | ECC Report 149 | For Low Power Active Medical Implants and associated peripherals, covered by the applicable harmonised standard. Individual transmitters may combine adjacent channels on a dynamic basis for increased bandwidth higher than 1 MHz. Peripheral units are for indoor use only. |

Additional Information

Harmonised Standards

EN 301 839	Sub-band a)

EN 302 537	Sub-bands a1) and a2)

EN 302 195	Sub-band b)

EN 302 536	Sub-band c)

EN 302 510	Sub-band d)

EN 300 330	Sub-band e)

Frequency issues

Technical parameters also referred to in the harmonised standard

No information.

Wireless Audio Applications

Scope of Annex

This annex covers frequency bands and regulatory as well as informative parameters recommended for applications for wireless audio systems including the following, cordless loudspeakers; cordless headphones; cordless headphones for portable use, for example portable CD, cassette or radio devices carried on a person; cordless headphones for use in a vehicle, for example for use with a radio or mobile telephone etc; in-ear monitoring, for use with concerts or other stage productions.

Regulatory parameters

| Frequency Band |  | Power / Magnetic Field | Spectrum access and mitigation requirements | Channel spacing | ECC/ERC Deliverable | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| a | 863-865 MHz | 10 mW e.r.p. | No requirement | No spacing | ECC Report 011 ECC Report 013 ECC Report 037 ECC Report 040 | The frequency band is also identified in Annex 1. |
| b | 864.8-865.0 MHz | 10 mW e.r.p. | No requirement | 50 kHz |  | Narrow band analogue voice devices The frequency band is also identified in Annex 1. |
| c | 1795-1800 MHz | 20 mW e.i.r.p. | No requirement | No spacing | ECC Report 002 |  |
| d | 87.5-108.0 MHz | 50 nW e.r.p. | No requirement | 200 kHz | ECC Report 073 |  |

Additional Information

Harmonised Standards

EN 301 357 	sub-band a) c) and d)

EN 300 220 	sub-band b)

Frequency issues

Sub-band b).

Narrow band analogue voice devices, such as baby voice monitors, door entry systems etc should only use the band b) 864.8-865 MHz.

Technical parameters also referred to in the harmonised standard

Systems should be designed so that when not in use there should be no transmission of an RF carrier.

Sub-band d)

The user interface of SRD shall permit as a minimum the selection of any and all possible frequencies within the 88.1 MHz to 107.9 MHz and as a maximum 87.6 MHz to 107.9 MHz.

When audio signals are not present, apparatus must employ a transmission time out facility. Pilot tones that ensure continuity of transmission are not permitted.

# Appendix 1: Countries for Class 1 equipment

# APPENDIX 2:List of relevant ECC/ERC Decisions, Reports, EC Decisions and ETSI Standards

ECC/ERC Decisions

| ECC/DEC/(11)02 | Industrial Level Probing Radars (LPR) operating in frequency bands 6 - 8.5 GHz, 24.05 - 26.5 GHz, 57 - 64 GHz and 75 - 85 GHz |
| --- | --- |
| ECC/DEC/(07)01 | Building Material Analysis (BMA) devices using UWB technology |
| ECC/DEC/(06)12 | Supplementary regulatory provisions to decision ECC/DEC/(06)04 for UWB devices using mitigation techniques |
| ECC/DEC/(06)08 | The conditions for use of the radio spectrum by Ground- and Wall- probing radar (GPR/WPR) imaging systems |
| ECC/DEC/(06)04 | The harmonised conditions for devices using Ultra-wideband (UWB) technology in bands below 10.6 GHz |
| ECC/DEC/(05)02 | The use of the frequency band 169.4-169.8125 MHz |
| ECC/DEC(04)10 | The frequency bands to be designatedfor the temporary introduction of Automotive Short Range Radars |
| ECC/DEC(04)08 | The harmonised use of the 5 GHz frequency bands for the implementation of Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) |
| ECC/DEC/(04)03 | The frequency band 77-81 GHz to be designated for the use of Automotive Short Range Radars |
| ECC/DEC/(04)01 | Short Range Devices for detection of Avalanche Victims |
| ECC/DEC/(02)01 | The frequency bands to be designated for the coordinated introduction of Road Transport and Traffic Telematic Systems. |
| ERC/DEC(01)08 | Short Range Devices for Movement Detection and Alert in 2400-2483.5 MHz |
| ERC/DEC(01)11 | Short Range Devices for Flying Model Control in 34.995-35.225 MHz |
| ERC/DEC(01)12 | Short Range Devices for Model Control in 40.665, 40.675, 40.685 and 40.695 MHz |
| ERC/DEC(01)17 | Short Range Devices for Medical Implants in 402-405 MHz |

ECC/ERC Reports

| ECC Report 001 | Compatibility between inductive LF and HF RFID transponder and other radio communications systems in the frequency ranges 135-148.5 kHz, 4.78-8.78 MHz and 11.56-15.56 MHz |
| --- | --- |
| ECC Report 002 | SAP/SAB (Incl. ENG/OB) spectrum use and future requirements |
| ECC Report 007 | Compatibility between inductive LF RFID systems and radio communications systems in the frequency range 135 - 148.5 kHz |
| ECC Report 011 | Strategic Plans for the future use of the frequency bands 862-870 MHz and 2400-2483.5 MHz for Short Range Devices |
| ECC Report 012 | Ultra Low Power Active Medical Implant systems (ULP-AMI) |
| ECC Report 013 | Adjacent band compatibility between Short Range Devices and TETRA TAPS mobile services at 870 MHz |
| ECCReport 023 | Compatibility of automotive collision warningshort range radar operating at 24 GHz with FS, EESS and Radio Astronomy |
| ECC Report 024 | PLT, DSL, CABLE communications (Including CABLE TV), LANS and their effect on radio services |
| ECC Report 037 | Compatibility of planned SRD applications in 863-870 MHz |
| ECC Report 040 | Adjacent band compatibility between CDMA-PAMR mobile services and Short Range Devices below 870 MHz |
| ECC Report 056 | Compatibility of automotive collision warningshort range radar operating at 79 GHz with radiocommunication services |
| ECC Report 064 | The protection requirements of radiocommunication systemsbelow 10.6 GHz from generic UWB applications |
| ECC Report 055 | Compatibility between existing and proposed SRDs and other radiocommunication applications in the 169.4-169.8 MHz frequency band. See supplementary excel spreadsheets in download |
| ECC Report 067 | Compatibility study for generic limits for the emission levels of inductive SRDs below 30 MHz |
| ECC Report 068 | Compatibility studies in the band 5725-5875 MHz between Fixed Wireless Access (FWA) systems and other systems |
| ECC Report 073 | Compatibility of SRD in the FM radio broadcasting band |
| ECC Report 081 | The coexistence between Ultra Low Power - Animal Implant Devices (ULP-AID) operating in the frequency band 12.5-20 MHz and existing radiocommunication systems |
| ECC Report 092 | Coexistence between Ultra Low Power Active Medical Implants devices (ULP-AMI) and existing radiocommunication systems and services in the frequency bands 401–402 MHz and 405–406 MHz |
| ECC Report 094 | Technical requirements for UWB LDC devices to ensure the protection of FWA systems |
| ECC Report 098 | Studying the compatibility issues of the UIC EUROLOOP system with other systems in the frequency band 9.5 to 17.5 MHz |
| ECC Report 111 | Compatibility studies between Ground Based Synthetic Aperture Radar (GBSAR) and existing services in the range 17.1 GHz to 17.3 GHz |
| ECC Report 113 | Compatibility studies around 63 GHz between Intelligent Transport Systems (ITS) and other systems |
| ECC Report 114 | Compatibility studies between multiple GIGABIT wireless systems in frequency range 57-66 GHz and other services and systems (except its in 63-64 GHz) |
| ECC Report 120 | Technical requirements for UWB DAA (Detect And Avoid) devices to ensure the protection of radiolocation in the bands 3.1-3.4 GHz and 8.5-9 GHz and BWA terminals in the band 3.4-4.2 GHz |
| ECC Report 134 | Analysis of potential impact of mobile Vehicle Radars (VR) on Radar Speed Meters (RSM) operating at 24 GHz |
| ECC Report 135 | Inductive limits in the frequency range 9 kHz to 148.5 kHz |
| ECC Report 149 | Compatibility of LP-AMI applications within 2360-3400 MHz, in particular for the band 2483.5-2500 MHz, with incumbent services |
| ECC Report 158 | The impact of SRR 26 GHz applications using Ultra-Wide-Band (UWB) Technology on Radio Services |
| ECC Report 164 | Compatibility between Wide Band Low Activity Mode (WLAM) automotive radars in the frequency range 24.25 GHz to 24.5 GHz, and other radiocommunication systems/services |
| ERC Report 001 | Harmonisation of frequency bands to be designated for Radio Local Area Networks (RLANs) |
| ERC Report 003 | Harmonisation of frequency bands to be designated for road transport information systems (RTTT) |
| ERC Report 005 | ERC Report on frequency bands for Low Power Devices |
| ERC Report 042 | Handbook on radio equipment and systems radio microphones and simple wide band audio links |
| ERC Report 044 | Sharing inductive systems and radiocommunication systems in the band 9-135 kHz |
| ERC Report 047 | Compatibility fixed services and motion sensors at 10.5 GHz |
| ERC Report 062 | Compatibility analysis regarding possible sharing between the UIC system and radio microphones in the frequency ranges 876 - 880 MHz and 921 - 925 MHz |
| ERC Report 063 | Radio microphone applications in the frequency range 1785-1800 MHz |
| ERC Report 069 | Propagation model and interference range calculation for inductive systems in 10 kHz – 30 MHz |
| ERC Report 072 | Compatibility studies related to the possible extension band for HIPERLANs at 5 GHz |
| ERC Report 074 | RFID and the radioastronomy services at 13 MHz |
| ERC Report 088 | Compatibility and sharing analysis between DVB-T and radio microphones in bands IV and V |
| ERC Report 092 | Sharing inductive Short Range Devices and radio communication systems in 10.2-11 MHz |
| ERC Report 095 | ERC Report on the use of 3155-3400 kHz for general inductive applications |
| ERC Report 096 | ERC Report on the use of 290-300 kHz and 500-510 kHz for general inductive applications |
| ERC Report 098 | ERC Report on compatibility of Short Range Devices at 900 MHz with adjacent services |
| ERC Report 109 | Compatibility of Bluetooth with other existing and proposed radiocommunication systems in the 2.45 GHz frequency band |

ETSI Standards including harmonised standards

Further information can be found at http://ec.europa.eu/enterprise/policies/european-standards/documents/harmonised-standards-legislation/list-references/rtte/index_en.htm

| Generic standards |  |
| --- | --- |
| EN 300 220 | Electromagnetic compatibility and Radio spectrum Matters (ERM); Short Range Devices (SRD); Radio equipment to be used in the 25 MHz to 1000 MHz frequency range with power levels ranging up to 500 mW |
| EN 300 330 | Electromagnetic compatibility and Radio spectrum Matters (ERM); Short Range Devices (SRD); Radio equipment in the frequency range 9 kHz to 25 MHz and inductive loop systems in the frequency range 9 kHz to 30 MHz |
| EN 300 440 | Electromagnetic compatibility and Radio spectrum Matters (ERM); Short range devices; Radio equipment to be used in the 1 GHz to 40 GHz frequency range |
| EN 302 065 | Electromagnetic compatibility and Radio spectrum Matters (ERM);Short Range Devices (SRD) using Ultra Wide Band technology (UWB) for communications purposes |
| Specific standards |  |
| EN 300 328 | Electromagnetic compatibility and Radio spectrum Matters (ERM); Wideband Transmission systems; Data transmission equipment operating in the 2.4 GHz ISM band and using spread spectrum modulation techniques |
| EN 300 422 | Electromagnetic compatibility and Radio spectrum Matters (ERM); Wireless microphones in the 25 MHz to 3 GHz frequency range |
| EN 300 674 | Electromagnetic Compatibility and Radio Spectrum Matters (ERM); Road Transport and Traffic Telematics (RTTT); Technical characteristics and test methods for Dedicated Short Range Communication (DSRC) transmission equipment (500 kbit/s / 250 kbit/s) operating in the 5.8 GHz Industrial, Scientific and Medical (ISM) band |
| EN 300 718 | Electromagnetic compatibility and Radio spectrum matters (ERM); Avalanche Beacons; Transmitter-receiver systems |
| EN 300 761 | Electromagnetic compatibility and Radio spectrum Matters (ERM); Short Range Devices (SRD); Automatic Vehicle Identification (AVI) for railways operating in the 2.45 GHz frequency range |
| EN 301 091 | Electromagnetic Compatibility and Radio Spectrum Matters (ERM); Road Transport and Traffic Telematics (RTTT); Technical characteristics and test methods for radar equipment operating in the 76 GHz to 77 GHz band |
| EN 301 357 | Electromagnetic compatibility and Radio spectrum Matters (ERM); Analogue cordless wideband audio devices using integral antennas operating in the CEPT recommended 863 MHz to 865 MHz frequency range |
| EN 301 839 | Electromagnetic compatibility and Radio spectrum Matters (ERM);Short Range Devices (SRD);Ultra Low Power Active Medical Implants (ULP-AMI) and Peripherals (ULP-AMI-P) operating in the frequency range 402 MHz to 405 MHz |
| EN 301 893 | Broadband Radio Access Networks (BRAN); 5 GHz high performance RLAN; Harmonised EN covering essential requirements of article 3.2 of the R&TTE Directive. |
| EN 302 066 | Electromagnetic compatibility and Radio spectrum Matters (ERM);Ground- and Wall- Probing Radar applications (GPR/WPR) imaging systems |
| EN 302 195 | Electromagnetic compatibility and Radio spectrum Matters (ERM); Radio equipment in the frequency range 9 kHz to 315 kHz for Ultra Low Power Active Medical Implants (ULP-AMI) and accessories |
| EN 302 208 | Electromagnetic compatibility and Radio spectrum Matters (ERM); Radio Frequency Identification Equipment operating in the band 865 MHz to 868 MHz with power levels up to 2 W |
| EN 302 291 | Close Range Inductive Data Communication equipment operating at 13.56 MHz |
| EN 302 372 | Electromagnetic compatibility and Radio spectrum Matters (ERM);Short Range Devices (SRD); Equipment for Detection and Movement; Tanks Level Probing Radar (TLPR) operating in the frequency bands 5.8 GHz, 10 GHz, 25 GHz, 61 GHz and 77 GHz |
| EN 302 264 | Electromagnetic compatibility and Radio spectrum Matters (ERM);Short Range Devices;Road Transport and Traffic Telematics (RTTT);Short Range Radar equipment operating in the 77 GHz to 81 GHz band |
| EN 302 288 | Electromagnetic compatibility and Radio spectrum Matters (ERM);Short Range Devices;Road Transport and Traffic Telematics (RTTT);Short range radar equipment operating in the 24 GHz range |
| EN 302 435 | Electromagnetic compatibility and Radio spectrum Matters (ERM);Short Range Devices (SRD);Technical characteristics for SRD equipment using Ultra WideBand technology (UWB);Building Material Analysis and Classification equipment applications operating in the frequency band from 2,2 GHz to 8,5 GHz |
| EN 302 500 | Electromagnetic compatibility and Radio spectrum Matters (ERM);Short Range Devices (SRD) using Ultra WideBand (UWB) technology;Location Tracking equipment operating in the frequency range from 6 GHz to 8.5 GHz |
| EN 302 510 | Electromagnetic compatibility and Radio spectrum Matters (ERM);Radio equipment in the frequency range 30 MHz to 37,5 MHz for Ultra Low Power Active Medical Membrane Implants and Accessories |
| EN 302 536 | Electromagnetic compatibility and Radio spectrum Matters (ERM);Short Range Devices (SRD);Radio equipment in the frequency range 315 kHz to 600 kHz |
| EN 302 537 | Electromagnetic compatibility and Radio spectrum Matters (ERM); Short Range Devices (SRD); Ultra Low Power Medical Data Service Systems operating in the frequency range 401 MHz to 402 MHz and 405 MHz to 406 MHz |
| EN 302 567 | Broadband Radio Access Networks (BRAN); 60 GHz Multiple-Gigabit WAS/RLAN Systems |
| EN 302 608 | Electromagnetic compatibility and Radio spectrum Matters (ERM);Short Range Devices (SRD);Radio equipment for Eurobalise railway systems |
| EN 302 609 | Electromagnetic compatibility and Radio spectrum Matters (ERM);Short Range Devices (SRD);Radio equipment for Euroloop railway systems |
| ES 200 674 | Electromagnetic compatibility and Radio spectrum Matters (ERM); Road Transport and Traffic Telematics (RTTT) |

| EC Decision | Title |
| --- | --- |
| 2011/485/EU | Harmonisation of the 24 GHz range radio spectrum band for the time-limited use by automotive SRR equipment in the Community |
| 2010/368/EU | Amending the Decision 2006/771/EC on harmonisation of the radio spectrum for use by SRDs |
| 2009/381/EC | Amending Decision 2006/771/EC on harmonisation of the radio spectrum for use by SRDs |
| 2009/343/EC | Amending the Decision 2007/131/EC on the harmonised use of the radio spectrum for equipment using UWB technology |
| 2008/673/EC | Amending Decision 2005/928/EC on the harmonisation of the 169.4-169.8125 MHz frequency band in the Community |
| 2008/432/EC | Amending Decision 2006/771/EC on harmonisation of the radio spectrum for use by short-range devices |
| 2007/346/EC | Granting a derogation requested by France pursuant to Decision 2006/804/EC on harmonisation of the radio spectrum for radio frequency identification (RFID) devices operating in the ultra high frequency (UHF) band |
| 2007/131/EC | Allowing the use of the radio spectrum for equipment using Ultra-wideband technology in aharmonised manner in the community |
| 2007/90/EC | Amending Decision 2005/513/EC on the harmonised use of radio spectrum in the 5 GHz frequency band for the implementation of Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) |
| 2006/804/EC | Harmonisation of the radio spectrum for radio frequency identification (RFID) devices operating in the ultra high frequency (UHF) band |
| 2006/771/EC | Harmonisation of the radio spectrum for use by short-range devices |
| 2005/928/EC | Harmonisation of the 169,4-169,8125 MHz frequency band in the Community |
| 2005/513/EC | Harmonised use of radio spectrum in the 5 GHz frequency band for the implementation of wireless access systems including radio local area networks (WAS/RLANs) |
| 2005/50/EC | The harmonisation of the 24 GHz range radio spectrum band for the time-limited use byAutomotive Short-Range Radar equipment in the community |
| 2004/545/EC | The harmonisation of radio spectrum in the 79 GHz range for the use of Automotive Short-Range Radar equipment in the community |

# Appendix 3 – National Restrictions

“Appendix 3 lists national restrictions. The first section contains general comments from administrations and these apply to all annexes in this Recommendation. The second section contains comments from administrations and these are on specific frequency bands contained within this Recommendation. These indicate where administrations are not able to implement frequency allocations or where implementation is incomplete. For consistency, one of the following four standard positions should be used:

- Implemented: If the Appendix entry is blank then Recommendation 70-03 has been fully implemented.

- Limited implementation: A short explanation can be provided. If under study or planned, then a date should be given.

- Not implemented: A short explanation can be provided. If under study or planned, then a date should be given.

No information: No information has yet been provided by the administration.”

| Frequency Band | Country | Implementation | Reason/remarks |
| --- | --- | --- | --- |
| All Annexes | France | France does not response the former marking CEPT SRD Aa Y and CEPT RLAN Y recommmended by T/R 01-04 and T/R 10-01 respectively. The free circulation and use of products bearing these old markings must then be confined to existing equipments and to countries which have already adopted these markings. The marking CEPT SRD Aa Y proposed by ERC/REC 70-03 will not be recognised in France. In any case in France marking issues are in line with the R&TTE Directive |  |
|  | Germany |  | Clarification of the terms contained in the table reference to the German Telecommunications Act of 22 June 2004: The use of frequencies or frequency bands for the operation of transmitting equipment requires "frequency assignment". There are two types of frequency assignments: individual frequency assignments are granted upon application and correspond to "individual license required" within the meaning of CEPT/ERC/REC 70-03; general frequency assignments are granted ex officio by administrative act, published in the Federal Network Agency's Official Gazette and correspond to "individual license not required" within the meaning of CEPT/ERC/REC 70-03 |
|  | Lithuania |  | The radio frequencies may be used without an individual authorisation in case the relevant radio frequency or radio frequencies band is included in the List of Radio Frequencies, which may be used without an Individual Authorisation, approved by Order No. 1V-893 of 9 September 2010 of the Director of the Communications Regulatory Authority (Official Gazette Valstybes zinios, Nr. 108-5577, 2010). Radio equipment must conform to the requirements of the List |
|  | Moldova | Telecommunication equipment and cables are imported commercialized only on basis of conformity certificates issued by the Telecommunication Products Certification Body of Moldova and must be marked in Moldova. It is not permitted to utilise non-certificated and non-marked telecommunication equipment and cables. Subject to the above all SRD frequency bands with technical parameters indicated in ERC REC 70-03 are permitted on secondary basis | In accordance with Law of Telecommunications of Republic of Moldova |
|  | Russian Federation | In accordance with the current National Frequency Allocation Table, different communication services, including special applications operate in frequency bands designated for SRD applications. All radiocommunication systems require individual license and authorisation for using certain radio frequencies, which is granted after conformity assessment procedures. All types of radio equipment require national approval based on the national standard system (GOST) and issue of conformity certificate. Only equipment with national mark can be placed on the market in Russia |  |
|  | Turkey |  | The short range and low powered devices under the scope of SRD Ordinance (enter into force 17 March 2007) can be used without any need to get the certificate, use permit and frequency registers on condition that they shall meet the determined conditions and be in accordance with the technical regulations specifications accepted by The Authority |
| Annex 1 Band A (Non- Specific SRDs) 6765-6795 kHz | Georgia | No info |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Limited implementation | The maximal strength of a magnetic field on distance of 10 m from the station is 42 dBmµA/m |
| Annex 1 Band E (Non- Specific SRDs) 138.20-138.45 MHz | Belgium | Not implemented |  |
|  | Croatia | Not implemented | Defence systems |
|  | France | Not implemented | Military use. The use of this band by SRDs is not planned in France |
|  | Georgia | Not implemented |  |
|  | Germany | Not implemented | Defence systems |
|  | Hungary | Not implemented | Aeronautical mobile applications operate in the band |
|  | Italy | Not implemented | Military application |
|  | Latvia | Not implemented | Exclusive defence systems |
|  | Liechtenstein | Not implemented |  |
|  | Luxembourg | Implemented | Notification Number: 2005/0347/L) |
|  | Poland | Not implemented | Military application |
|  | Russian Federation | Not implemented |  |
|  | Slovak Republic | Not implemented | Defence systems |
|  | Slovenia | Not implemented | Not available |
|  | Spain | Not implemented | Military application |
|  | Sweden | Not implemented |  |
|  | Switzerland | Not implemented | Exclusive defence systems |
|  | The Netherlands | Not implemented | Exclusive defence systems |
|  | Turkey | Not implemented | Defence systems |
|  | Ukraine | Not implemented |  |
|  | United Kingdom | Planned | Implementation planned for 2012 |
| Annex 1 Band F (Non- Specific SRDs) 433.050-434.790 MHz | Georgia | Limited implementation |  |
|  | Italy | Limited implementation | Audio applications are limited in the range 433.05-433.575 MHz with 12.5 or 25 kHz channel spacing |
|  | Luxembourg | Limited implementation | No audio and no voice |
|  | Russian Federation | Limited implementation | 433.075-434.790 MHz. Possible use of low power stations and devices for processing of bar-codes |
|  | Ukraine | Limited implementation | The maximal transmitter power 10 mW |
| Annex 1 Band F1 (Non- Specific SRDs) 433.050-434.790 MHz | Georgia | Limited implementation |  |
|  | Italy | Limited implementation | Audio applications are limited in the range433.05-433.575 MHz with 12.5 or 25 kHz channel spacing |
|  | Luxembourg | Limited implementation | No audio and no voice |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Limited implementation | The maximal transmitter power 10 mW |
| Annex 1 Band F2 (Non- Specific SRDs) 434.040-434.790 MHz | Georgia | Limited implementation |  |
|  | Luxembourg | Implemented | (Notification Number: 2009/375/L) |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Limited implementation | The maximal transmitter power 10 mW |
| Annex 1 Band G (Non- Specific SRDs) 863-870 MHz | Austria | Not implemented | Planned |
|  | Georgia | Not implemented |  |
|  | Greece | Limited implementation | to 863-865 MHz |
|  | Norway | Not implemented |  |
|  | Russian Federation | Limited implementation | 864-865 MHz with max e.r.p 25 mW, duty cycle 0.1% or LBT. Forbidden to use at the airports (aerodromes) |
|  | Spain | Limited implemented | to the band 863-868 MHz |
|  | Sweden | Not implemented |  |
|  | The Netherlands | Not implemented | Under study |
|  | Ukraine | Limited implementation | 863-865 / 868-868.6 / 868.6-868.7 / 869.2-869.25 MHz |
| Annex 1 Band G1 (Non- Specific SRDs) 868.000-868.600 MHz | Georgia | Not implemented |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Limited implementation | e.i.r.p. ≤25 mW |
| Annex 1 Band G3 (Non- Specific SRDs) 869.400-869.650 MHz | Georgia | Not implemented |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Not implemented |  |
| Annex 1 Band G4 (Non- Specific SRDs) 869.700-870.000 MHz | Georgia | Not implemented |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Not implemented |  |
| Annex 1 Band H (Non- Specific SRDs) 2400.0-2483.5 MHz | Norway | Implemented | This subsection does not apply for the geographical area within a radius of 20 km from the centre of Ny-Ålesund |
|  | Russian Federation |  | Bluetooth |
|  | Ukraine | Limited implementation | e.i.r.p. ≤100 mW |
| Annex 1 Band I (Non- Specific SRDs) 5725-5875 MHz | Russian Federation | Limited implementation | Duty cycle 0.1% or LBT. Antenna height should not exceed 5 m |
| Annex 1 Band J (Non- Specific SRDs) 24.00-24.25 GHz | France | Power limited to 0.1 mW e.i.r.p.in frequency band 24.10 - 24.15 GHz | Military Radiolocation use. Operation by police forces of Radar Speed Meters |
|  | Russian Federation | Not implemented |  |
|  | United Kingdom | Limited implementation | Only 24.150-24.250 GHz to protect police speedmeters |
| Annex 1 Band K (Non- Specific SRDs) 61.0-61.5 GHz | Croatia | Not implemented | Planned until 01.03.2012 |
|  | Georgia | No info |  |
|  | Russian Federation | Not implemented |  |
| Annex 1 Band L (Non- Specific SRDs) 122-123 GHz | Croatia | Not implemented | Planned until 01.03.2012 |
|  | France | Not implemented |  |
|  | Georgia | No info |  |
|  | Russian Federation | Not implemented |  |
|  | United Kingdom | Planned | Implementation planned 2012 |
| Annex 1 Band M (Non- Specific SRDs) 244-246 GHz | Croatia | Not implemented | Planned until 01.03.2012 |
|  | France | Not implemented |  |
|  | Georgia | No info |  |
|  | Russian Federation | Not implemented |  |
|  | United Kingdom | Planned | Implementation planned 2012 |
| Annex 1 Band N (Non- Specific SRDs) 3.1-4.8 GHz/6-9 GHz | Bosnia and Herzegovina | Not implemented | Committed |
|  | Macedonia (FYROM) | No info |  |
|  | Russian Federation | Limited | In accordance with National restrictions For Indoor applications: 1. Prohibited to use outside buildings 2. Prohibited to use onboard aircraft while arriving and departure 3. Prohibited to use in freight terminals in airports. Power spectral density limits: 2850-3375 MHz: -57 dBm/MHz 3375-3950 MHz: -61.5 dBm/MHz 3950-4425 MHz: -54.5 dB/MHz 4425-5470 MHz: -50 dB/MHz 5470-6000 MHz: -62.5 dBm/MHz 6000-8100 MHz: -47 dBm/MHz 8100-8625 MHz: -65 dBm/MHz 8625-9150 MHz: -47 dB/MHz 9150-10600 MHz: -45 dBm/MHz For Outdoor applications: Power spectral density limits: 2850-3375 MHz: -57 dBm/MHz 3375-4800 MHz: -76 dBm/MHz 4800-5475 MHz: -50 dBm/MHz 5475-6000 MHz: -62.5 dBm/MHz 6000-7250 MHz: -47 dBm/MHz 7250-7750 MHz: -73 dBm/MHz 7750-8625 MHz: -69 dBm/MHz 8625-9150 MHz: -47 dBm/MHz 9150-10600 MHz: -45 dBm/MHz |
|  | Serbia | No info |  |
|  | Turkey | No info |  |
|  | Ukraine | Not implemented | Under study for 3.1-4.8 GHz |
| Annex 2 Band A Tracking, Tracing and Data Acquisition 456.9-457.1 kHz | Bulgaria | Implemented | 457 kHz centre frequency is allocated 456.9-457.1 kHz band is not allocated |
|  | France | Implemented | National regulation specifies only the carrier frequency 457 kHz |
|  | Georgia | Not implemented |  |
|  | Ukraine | Limited implementation | The maximal strength of magnetic field is 7 dBmµA/m on distance of 10 m from a construction where the radiator is placed |
| Annex 2 Band B Tracking, Tracing and Data Acquisition 169.4-169.475 MHz | Austria | Not implemented | Planned |
|  | Belgium | No info |  |
|  | Bulgaria | Not implemented | The band is used for national security needs |
|  | Croatia | Limited implementation | Individual licence required |
|  | Cyprus | Implemented | Cyprus has implemented Decision 2005/928/EC |
|  | Georgia | Not implemented |  |
|  | Greece | Not implemented |  |
|  | Norway | Limited | Maximum radiated power = 10 mW |
|  | Russian Federation | Not implemented |  |
|  | The Netherlands | Implemented | Channel spacing 12.5 kH |
|  | Ukraine | Not implemented | Under study |
| Annex 2 Band C Tracking, Tracing and Data Acquisition 169.4-169.475 MHz | Austria | Not implemented | Planned |
|  | Belgium | No info |  |
|  | Bulgaria | Not implemented | The band is used for national security needs |
|  | Croatia | Limited implementation | Individual licence required |
|  | Cyprus | Implemented | Cyprus has implemented Decision 2005/928/EC |
|  | Denmark | Not implemented | PMR band |
|  | Georgia | Not implemented |  |
|  | Greece | Not implemented |  |
|  | Russian Federation | Not implemented |  |
|  | The Netherlands | Implemented | Channel spacing 12.5 kHz |
|  | Ukraine | Not implemented | Under study |
| Annex 3 Band A Wideband Data Transmission systems 2400.0-2483.5 MHz | France | Limited implementation | The outdoor use is limited to 10 mW e.i.r.p. within the band 2454-2483.5 MHz. Military Radiolocation use. Refarming of the 2.4 GHz has been ongoing in recent years to allow current relaxed regulation. Full implementation planned 2012 |
|  | Italy |  | For private use, a general authorisation is required if WAS/RLAN’s are used outside own premises. For public use, a general authorisation is required |
|  | Norway | Implemented | This subsection does not apply for the geographical area within a radius of 20 km from the centre of Ny-Ålesund |
|  | Russian Federation | Limited implementation | 1. SRD with FHSS modulation 1.1. Maximum 2.5 mW e.i.r.p. 1.2. Maximum 100 mW e.i.r.p. Permitted for use SRD for outdoor applications without restriction on installation height only for purposes of gathering telemetry information for automated monitoring and resources accounting systems. Permitted to use SRD for other purposes for outdoor applications only when the installation height is not exceeding 10 m above the ground surface. 1.3.Maximum 100 mW e.i.r.p. Indoor applications 2. SRD with DSSS and other than FHSS wideband modulation 2.1. Maximum mean e.i.r.p. density is 2 mW/MHz. Maximum 100 mW e.i.r.p. 2.2. Maximum mean e.i.r.p. density is 20 mW/MHz. Maximum 100 mW e.i.r.p. It is permitted to use SRD for outdoor applications only for purposes of gathering telemetry information for automated monitoring and resources accounting systems or security systems. 2.3. Maximum mean e.i.r.p. density is 10 mW/MHz. Maximum 100 mW e.i.r.p. Indoor applications |
|  | Ukraine | Limited implementation | e.i.r.p. ≤100 mW with built-in antenna with amplification factor up to 6 dBi |
| Annex 3 Band B Wideband Data Transmission systems 5150-5350 MHz | Georgia | Limited implementation |  |
|  | Italy |  | For private use, a general authorisation is required if WAS/RLAN’s are used outside own premises. For public use, a general authorisation is required |
|  | Russian Federation | Limited implementation | 5150-5250 MHz: SRD with DSSS and other than FHSS wideband modulation 1. Maximum mean e.i.r.p. density is 5 mW/MHz. Maximum 200 mW e.i.r.p. Indoor applications. 2. Maximum 100 mW. e.i.r.p. Permitted to use on board aircraft. 5250-5350 MHz: Maximum 100 mW e.i.r.p. 1. Permitted to use for local networks of aircraft crew service communications on board aircraft in area of the airport and at all stages of flight. 2. Permitted to use for public wireless access local networks on board aircraft during a flight at the altitude not less than 3000 m |
|  | Ukraine | Limited implementation | e.i.r.p. ≤200 mW at average power stream density 10 mW/MHz in any band in width of 1 MHz and use of the built-in antenna with amplification factor up to 6 dBi and presence of algorithm of radioation power control and a dynamic choice of frequency |
| Annex 3 Band C Wideband Data Transmission systems 5470-5725 MHz | Italy |  | For private use, a general authorisation is required if WAS/RLAN’s are used outside own premises. For public use, a general authorisation is required |
|  | Russian Federation | Limited implementation | 5650-5825 MHz with e.i.r.p. 100 mW. Permitted to use on board aircraft during a flight at the altitude not less than 3000 m |
|  | Turkey | Not implemented | Defence systems |
|  | Ukraine | Limited implementation | 5470-5670 MHz; e.i.r.p. ≤1 W at the maximal transmitter powerup to 250 mW and average power stream density 50 mW/MHz in any band in width of 1 MHz and use of the antenna with amplification factor 12 dBi |
| Annex 3 Band D Wideband Data Transmission systems 17.1-17.3 GHz | Belgium | Not implemented |  |
|  | Cyprus | Not implemented |  |
|  | France | Not implemented | Military Radiolocation use. Equipment/Standard not yet developed |
|  | Georgia | Limited implementation |  |
|  | Germany | Not implemented | Equipment/Standard not yet developed |
|  | Hungary | Planned | No equipment and standards are available |
|  | Italy |  | A general authorisation is required if WAS/RLAN’s are used outside own premises |
|  | Russian Federation | Not implemented |  |
|  | Slovak Republic | Not implemented | Planned service, currently not in use |
|  | Slovenia | Not implemented | Not available |
|  | Spain | Not implemented | Military application |
|  | Turkey | Not implemented |  |
|  | Ukraine | Not implemented |  |
|  | United Kingdom | Not implemented | No requirement |
| Annex 3 Band E Wideband Data Transmission systems 57-66 GHz | Georgia | No info |  |
|  | Russian Federation | Not implemented |  |
|  | Serbia | Available in the range: 61.0-61.5 GHz | According to the Frequency Plan, only this part of the spectrum is aimed for the SRD applications |
|  | Turkey | No info |  |
|  | Ukraine | No info |  |
| Annex 4 Band A Railway applications 2446-2454 MHz | Cyprus | Not applicable | No railways |
|  | Georgia | Limited implementation |  |
|  | Iceland | Not implemented | Service not applicable to Iceland |
|  | Italy | Not implemented |  |
|  | Malta | Not implemented | Service not applicable to Malta |
|  | Russian Federation | Not implemented |  |
|  | Slovak Republic | Not implemented | Under study |
|  | Sweden | Limited implementation | License required – Defence systems |
|  | Ukraine | Not implemented |  |
| Annex 4 Band B Railway applications 27.090-27.100 MHz (Centre frequency 27.095 MHz) | Bulgaria | Implemented | 27.095 MHz centre frequency is allocated. 27.090-27.100 MHz band is not allocated |
|  | Cyprus | Not implemented | Service not applicable to Cyprus |
|  | France | Implemented | National regulation specifies only the carrier frequency 27.095 MHz |
|  | Georgia | No info |  |
|  | Iceland | Not implemented | Service not applicable to Iceland |
|  | Ireland | Limited implementation | Max mean e.i.r.p. density is limited to 10mW/MHz in any 1 MHz band, as per Commission Decision 2007/90/EC |
|  | Malta | Not implemented | Service not applicable to Malta |
|  | Russian Federation | Not implemented |  |
|  | Sweden | Not implemented | 27.115 MHz used as provided in EU legislation |
|  | Ukraine | Not implemented |  |
| Annex 4 Band C Railway applications 984-7484 kHz (Centre frequency 4234 kHz) | Austria | Not implemented | Planned |
|  | Belgium | No info |  |
|  | Bulgaria | Implemented | 4234 kHz centre frequency is allocated 984-7484 kHz band is not allocated |
|  | Croatia | Not implemented | Planned until 01.03.2012 |
|  | Cyprus | Not implemented | Service not applicable to Cyprus |
|  | Czech Republic | Not implemented | Under study |
|  | France | Not implemented |  |
|  | Georgia | No info |  |
|  | Greece | Not implemented |  |
|  | Iceland | Not implemented | Service not applicable to Iceland |
|  | Italy | Not implemented |  |
|  | Latvia | Implemented | National regulation specifies only the carrier frequency 4234 kHz. The 984-7484 kHz band is not allocated |
|  | Luxembourg | Implemented | (New notification will be done) |
|  | Macedonia (FYROM) | Not implemented | Planned |
|  | Malta | Not implemented | Service not applicable to Malta |
|  | Poland | Not implemented | Planned in 2011 |
|  | Russian Federation | Not implemented |  |
|  | Serbia | Not implemented | According to the Frequency Plan, this part of the spectrum is aimed for the mobile maritime applications (4063-4438 kHz) |
|  | Slovak Republic | Not implemented | Under study |
|  | Slovenia | Not implemented |  |
|  | Spain | Not implemented | Not implemented due to lack of demand |
|  | Sweden | Not implemented |  |
|  | The Netherlands | Not implemented | Planned |
|  | Turkey | Under study | Planned 2009 |
|  | Ukraine | No info |  |
| Annex 4 Band D Railway applications 7.3-23.0 MHz (Centre frequency 13.547 MHz) | Belgium | No info |  |
|  | Bulgaria | Implemented | 11.1-16.0 MHz is allocated / 7.3-23.0 MHz band is not allocated |
|  | Croatia | Not implemented | Planned until 01.03.2012 |
|  | Cyprus | Not implemented | Service not applicable to Cyprus |
|  | Czech Republic | No info |  |
|  | France | Not implemented |  |
|  | Georgia | No info |  |
|  | Greece | Not implemented |  |
|  | Iceland | Not implemented | Service not applicable to Iceland |
|  | Ireland | Not implemented | Planned; Notification in progress |
|  | Italy | Not implemented |  |
|  | Latvia | Not implemented | Planned |
|  | Luxembourg | Implemented | (New notification will be done) |
|  | Macedonia (FYROM) | Not implemented | Planned |
|  | Malta | Not implemented | Service not applicable to Malta |
|  | Norway | Not implemented |  |
|  | Poland | Not implemented | Planned in 2011 |
|  | Russian Federation | Not implemented |  |
|  | Serbia | Available in the range: 13.553-13.567 MHz | According to the Frequency Plan, this part of the spectrum is available for the SRD applications |
|  | Slovak Republic | Not implemented | Under study |
|  | Slovenia | Not implemented |  |
|  | Spain | Not implemented | Not implemented due to lack of demands |
|  | Sweden | Not implemented |  |
|  | The Netherlands | Not implemented | Planned |
|  | Turkey | Under study | Planned 2009 |
|  | Ukraine | No info |  |
| Annex 5 Band A RTTT 5795-5805 MHz | France | Limited implementation | Limited to automatic toll collection.Power limited to 2 W e.i.r.p. Military Radiolocation and Meteorological use |
|  | Georgia | Limited implementation |  |
|  | Ireland | Limited implementation | 8W system not implemented |
|  | Liechtenstein | Limited implementation | Annex has two levels. Lower level with 2 W e.i.r.p.is implemented |
|  | Malta | Limited implementation | Power limited to 2 W e.i.r.p. as per the lower limit of the Annex |
|  | Norway | Limited implementation | Individual license required |
|  | Russian Federation | Limited implementation | 200 mW e.r.p. An authorisation for using radio frequencies or channels should too be obtained in established order |
|  | Switzerland | Limited implementation | Annex has two levels. Lower level with 2 W e.i.r.p. is implemented to protect defence systems |
|  | Ukraine | Not implemented |  |
|  | United Kingdom | Limited implementation | 2 Watts only permitted |
| Annex 5 Band B RTTT 5805-5815 MHz | Croatia | Not implemented | Planned until 01.03.2012 |
|  | France | Not implemented |  |
|  | Georgia | Limited implementation |  |
|  | Ireland | Limited implementation | 8W system not implemented |
|  | Liechtenstein | Limited implementation | Annex has two levels. Lower level with 2 W e.i.r.p. is implemented. For road toll systems only |
|  | Malta | Limited implementation | Power limited to 2 W e.i.r.p. as per the lower limit of the Annex |
|  | Norway | Limited implementation | Individual license required |
|  | Russian Federation | Limited implementation | 200 mW e.r.p. An authorisation for using radio frequencies or channels should too be obtained in established order |
|  | Switzerland | Limited implementation | Annex has two levels. Lower level with 2 W e.i.r.p. is implemented. For road toll systems only |
|  | Ukraine | Not implemented |  |
|  | United Kingdom | Limited implementation | 2 Watts only permitted |
| Annex 5 Band C RTTT 63-64 GHz | Croatia | Not implemented | Planned until 2015 |
|  | Estonia | Power limited to 2 W e.i.r.p |  |
|  | Georgia | No info |  |
|  | Germany | Not implemented | Equipment/standards not yet developed |
|  | France | Not implemented | Equipment/standards not yet developed |
|  | Liechtenstein | Not implemented | No standard available |
|  | Lithuania | Not implemented |  |
|  | Russian Federation | Not implemented |  |
|  | Sweden | Not implemented | Equipment/standard not available |
|  | Switzerland | Not implemented | No standard available |
|  | Turkey | Under study | Planned 2009 |
|  | Ukraine | Not implemented | Under study |
|  | United Kingdom | Planned | Implementation planned 2011 |
| Annex 5 Band D RTTT 76-77 GHz | Georgia | No info |  |
|  | Russian Federation | Not implemented |  |
| Annex 5 Band E RTTT 21.65-26.65 GHz | Georgia | No info |  |
|  | Ukraine | No info |  |
| Annex 5 Band F RTTT 77-81 GHz | Georgia | No info |  |
|  | Ukraine | No info |  |
| Annex 5 Band G1 RTTT 24.050-24.075 GHz | Austria | Not implemented | Due to implementation of Annex 1J |
|  | Belgium | No info |  |
|  | Bosnia and Herzegovina | Not implemented | Committed |
|  | Bulgaria | Not implemented |  |
|  | Croatia | Not implemented | Under study until 2015 |
|  | Cyprus | No info |  |
|  | Czech Republic | No info |  |
|  | Estonia | Not implemented | Under study |
|  | France | No info |  |
|  | Greece | No info |  |
|  | Hungary | Not implemented | Planned |
|  | Ireland | No info |  |
|  | Italy | No info |  |
|  | Lithuania | Not implemented |  |
|  | Luxembourg | No info |  |
|  | Malta | No info |  |
|  | Macedonia (FYROM) | No info |  |
|  | Norway | No info |  |
|  | Poland | Not implemented | Planned in 2011 |
|  | Portugal | No info |  |
|  | Romania | Not implemented | Under study |
|  | Russian Federation | No info |  |
|  | Serbia | No info |  |
|  | Slovak Republic | No info |  |
|  | Spain | Not implemented |  |
|  | Sweden | No info |  |
|  | Turkey | No info |  |
| Annex 5 Band G2 RTTT 24.075-24.150 GHz | Austria | Not implemented | Due to implementation of Annex 1J |
|  | Belgium | No info |  |
|  | Bosnia and Herzegovina | Not implemented | Committed |
|  | Bulgaria | Not implemented |  |
|  | Croatia | Not implemented | Under study until 2015 |
|  | Cyprus | No info |  |
|  | Czech Republic | No info |  |
|  | Estonia | Not implemented | Under study |
|  | France | No info |  |
|  | Greece | No info |  |
|  | Hungary | Not implemented | Planned |
|  | Ireland | No info |  |
|  | Italy | No info |  |
|  | Lithuania | Not implemented |  |
|  | Luxembourg | No info |  |
|  | Malta | No info |  |
|  | Macedonia (FYROM) | No info |  |
|  | Norway | No info |  |
|  | Poland | Not implemented | Planned in 2011 |
|  | Portugal | No info |  |
|  | Romania | Not implemented | Under study |
|  | Russian Federation | No info |  |
|  | Serbia | No info |  |
|  | Slovak Republic | No info |  |
|  | Spain | Not implemented |  |
|  | Sweden | No info |  |
|  | Turkey | No info |  |
| Annex 5 Band G3 RTTT 24.150-24.250 GHz | Austria | Not implemented | Due to implementation of Annex 1J |
|  | Belgium | No info |  |
|  | Bosnia and Herzegovina | Not implemented | Committed |
|  | Bulgaria | Not implemented |  |
|  | Croatia | Not implemented | Under study until 2015 |
|  | Cyprus | No info |  |
|  | Czech Republic | No info |  |
|  | Estonia | Not implemented | Under study |
|  | France | No info |  |
|  | Greece | No info |  |
|  | Hungary | Not implemented | Planned |
|  | Ireland | No info |  |
|  | Italy | No info |  |
|  | Lithuania | Not implemented |  |
|  | Luxembourg | No info |  |
|  | Malta | No info |  |
|  | Macedonia (FYROM) | No info |  |
|  | Norway | No info |  |
|  | Poland | Not implemented | Planned in 2011 |
|  | Portugal | No info |  |
|  | Romania | Not implemented | Under study |
|  | Russian Federation | No info |  |
|  | Serbia | No info |  |
|  | Slovak Republic | No info |  |
|  | Spain | Not implemented |  |
|  | Sweden | No info |  |
|  | Turkey | No info |  |
| Annex 6 Band A Radiodetermination applications 2400.0-2483.5 MHz | France | Limited implementation | Military Radiolocation use. Refarming of the 2.4 GHz band has been ongoing in recent years to allow current relaxed regulation. Full implementation planned 2012 |
|  | Georgia | Limited implementation |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Limited implementation | e.i.r.p. <100 mW |
| Annex 6 Band B Radiodetermination applications 9200-9500 MHz | Finland | Not implemented |  |
|  | France | Not implemented |  |
|  | Georgia | Limited implementation |  |
|  | Italy | Not implemented |  |
|  | Russian Federation | Not implemented |  |
|  | Spain | Not implemented | Military application |
|  | Sweden | Not implemented |  |
|  | Ukraine | Not implemented | Under study |
|  | United Kingdom | Limited implementation | May be used for Radar Level Gauges only |
| Annex 6 Band C Radiodetermination applications 9500-9975 MHz | France | Limited implementation | Limited to 9.88-9.92 with max e.i.r.p. 50 mW |
|  | Georgia | Limited implementation |  |
|  | Germany | Not implemented | Defence systems |
|  | Russian Federation | Not implemented |  |
|  | Slovak Republic | Not implemented | Defence systems |
|  | Spain | Not implemented | Military application |
|  | Sweden | Not implemented |  |
|  | Ukraine | Not implemented | Under study |
|  | United Kingdom | Limited implementation | May be used for Radar Level Gauges only |
| Annex 6 Band D Radiodetermination applications 10.5-10.6 GHz | Austria | Not implemented | Fixed Service |
|  | Czech Republic | Not implemented | Other service in the band |
|  | Estonia | Not implemented | FWA |
|  | Finland | Not implemented | 10.45-10.50 GHz available |
|  | France | Limited implementation | Limited to 10.57-10.61 with max e.i.r.p. 20 mW |
|  | Georgia | Limited implementation |  |
|  | Germany | Not implemented | ENG/OB video links equipment |
|  | Hungary | Limited implementation | e.i.r.p. 25 mW. ENG/OB systems are protected |
|  | Ireland | Limited implementation | Max power limitation of 25 mW to protect Fixed Wireless Access Local Area Service operating in the 10.5 GHz band |
|  | Luxembourg | Limited to 25 mW | Reason: To avoid interference with other services |
|  | Russian Federation | Not implemented | Under study |
|  | Slovak Republic | Not implemented | Fixed Service |
|  | Sweden | Limited implementation | Limited to 10.51-10.58 GHz |
|  | Turkey | Not implemented | Fixed Service and radiolocation |
|  | United Kingdom | Limited implementation | Limited to 10.577-10.597 GHz. May be used for Radar Level Gauges |
|  | Ukraine | Limited implementation | 10.51-10.54 GHz |
| Annex 6 Band E Radiodetermination applications 13.4-14.0 GHz | France | Not implemented |  |
|  | Georgia | Limited implementation |  |
|  | Russian Federation | Not implemented |  |
|  | Spain | Not implemented | Not implemented due to lack of demand |
|  | Sweden | Not implemented |  |
|  | Ukraine | Not implemented | Under study |
| Annex 6 Band F Radiodetermination applications 24.05-24.25 GHz | France | Limited implementation | No restriction for fixed applications. Power limited otherwise to 0.1 mW e.i.r.p. in frequency band 24.10 - 24.15 GHz. Alternatively for FMCW modulation, the following conditions are also allowed: power limited to 20 mW (+13 dBm) mean e.i.r.p. and 50 mW (+17 dBm) peak e.i.r.p. with a minimum frequency sweep speed of 5 MHz per millisecond. Military Radiolocation use. Operation by police forces of Radar Speed Meters |
|  | Georgia | Limited implementation |  |
|  | Russian Federation | Limited implementation | Vehicle radars: Maximum 100 mW e.i.r.p. No restrictions if emission bandwith is not less than 9 MHz. If emission bandwith is less than 9 MHz then the requirement should be 0.14 µs/60 kHz maximum dwell time every 3ms Fixed radars: Maximum 100 mW e.i.r.p. 1. The equipment for detecting movement should be installed along roads at 4 m distance from controlled part of road. 2. The installation of equipment for detecting movement should be performed perpendicularly to movement direction of one- or multilane road with permissible deviation 15 degrees. 3. The installation height of equipment for detecting movement should not exceed 5m above a road. 4. The tilt angle of the main beam to horizon should be minus 20 degrees or less |
|  | Ukraine | Limited implementation | e.i.r.p. ≤100 mW |
|  | United Kingdom | Limited implementation | To protect police speedmeters devices operating in 24.05-24.15 GHz must employ a minimum sweep rate |
| Annex 6 Band G Radiodetermination applications 4.5-7.0 GHz | Georgia | Not implemented | Under study |
|  | Macedonia (FYROM) | Not implemented | Planned |
|  | Russian Federation | Not implemented |  |
|  | Serbia | Available in the range: 5.725-5.875 GHz 5.15-5.25 GHz / 5.250-5.255 GHz and 5.255-5.350 GHz | According to the Frequency Plan, 5.725-5.875 GHz is available for the SRD applications. According to the Frequency Plan, 5.15-5.25 GHz, 5.250-5255 GHz and 5.255-5.350 GHz is available for the WAS and RLANS applications |
|  | Turkey | Under study | Planned 2009 |
|  | Ukraine | Not implemented | Under study |
| Annex 6 Band H Radiodetermination applications 8.5-10.6 GHz | Georgia | Not implemented |  |
|  | Macedonia (FYROM) | Not implemented | Planned |
|  | Russian Federation | Not implemented |  |
|  | Serbia | Available in the range: 10.50-10.55 GHz and 10.55-10.60 GHz | According to the Frequency Plan, this part of the spectrum is available for the SRD applications |
|  | Turkey | Under study | Planned 2009 |
|  | Ukraine | Not implemented | Under study |
| Annex 6 Band I Radiodetermination applications 24.05-27.0 GHz | Georgia | Not implemented | Under study |
|  | Macedonia (FYROM) | Not implemented | Planned |
|  | Russian Federation | Not implemented |  |
|  | Serbia | Available in the range: 24.05-24.25 GHz | According to the Frequency Plan, this part of the spectrum is available for the SRD applications |
|  | Turkey | Under study | Planned 2009 |
|  | Ukraine | Limited implementation | 24.05-24.25 GHz |
| Annex 6 Band J Radiodetermination applications 57-64 GHz | Georgia | No info |  |
|  | Macedonia (FYROM) | Not implemented | Planned |
|  | Russian Federation | Not implemented |  |
|  | Serbia | Available in the range: 61.0-61.5 GHz | According to the Frequency Plan, only this part of the spectrum is aimed for the SRD applications |
|  | Turkey | Not implemented | Planned 2009 |
|  | Ukraine | Not implemented | Under study |
| Annex 6 Band K Radiodetermination applications 75-85 GHz | Georgia | No info |  |
|  | Macedonia (FYROM) | Not implemented | Planned |
|  | Russian Federation | Not implemented |  |
|  | Serbia | Available in the range: 76.0-77.5 GHz | According to the Frequency Plan, only this part of the spectrum is aimed for the SRD applications (traffic radiolocation) |
|  | Turkey | Under study | Planned 2009 |
|  | Ukraine | Limited implementation | In the band 76-77 GHz average e.i.r.p. ≤23.5 dBm |
| Annex 6 Band L Radiodetermination applications 17.1-17.3 GHz | Belgium | No info |  |
|  | Croatia | Not implemented | Lack of demand |
|  | Georgia | Not implemented |  |
|  | Greece | Not implemented |  |
|  | Italy | Not implemented | Under study |
|  | Luxembourg | No info |  |
|  | Macedonia (FYROM) | Not implemented | Planned |
|  | Malta | Not implemented | Malta implemented the provision of 2006/771/EC, as amended |
|  | Poland | Not implemented | Planned in 2011 |
|  | Portugal | Not implemented | Under study |
|  | Russian Federation | Not implemented |  |
|  | Serbia | Not implemented | According to the Frequency Plan this part of the spectrum is aimed for WLL and RLANs |
|  | Spain | Not implemented |  |
|  | The Netherlands | Not implemented | Planned |
|  | Ukraine | Not implemented |  |
| Annex 6 Band M Radiodetermination applications 30 MHz-12.4 GHz | Austria | No info |  |
|  | Belgium | No info |  |
|  | Bosnia and Herzegovina | Not implemented | Committed |
|  | Cyprus | No info |  |
|  | France | Not implemented | Planned 2011 |
|  | Greece | No info |  |
|  | Hungary | Not implemented | Planned before end of 2011 |
|  | Ireland | Not implemented | Planned before end of 2011 |
|  | Italy | No info |  |
|  | Lithuania | Not implemented | Planned before end of 2011 |
|  | Luxembourg | No info |  |
|  | Malta | Not implemented | Planned before early 2012 |
|  | Macedonia (FYROM) | No info |  |
|  | Montenegro | Not implemented | Under study |
|  | Portugal | Not implemented | Planned |
|  | Romania | Not implemented | Under study |
|  | Russian Federation | Not implemented | Under study |
|  | Serbia | No info |  |
|  | Spain | Not implemented | Planned but not before third quarter of 2012 |
|  | Sweden | No |  |
|  | The Netherlands | Not implemented | Planned before end of 2011 |
|  | Turkey | No info |  |
|  | United Kingdom | Not implementation | Planned for 2012 |
| Annex 6 Band N Radiodetermination applications 2.2 - 8.0 GHz | Austria | Limited implementation | According to Commission Decision 2009/343/EC |
|  | Belgium | No info |  |
|  | Bulgaria | Implemented | Old version of ECC/DEC/(07)01 is implemented |
|  | Bosnia and Herzegovina | Not implemented | Committed |
|  | Cyprus | No info |  |
|  | Czech Republic | No info |  |
|  | France | Not implemented | Planned |
|  | Greece | No info |  |
|  | Hungary | Not implemented | Planned |
|  | Ireland | No info |  |
|  | Italy | No info |  |
|  | Lithuania | Limited implementation | only parameters set in 2009/343/EC are allowed |
|  | Luxembourg | No info |  |
|  | Malta | No info |  |
|  | Macedonia (FYROM) | No info |  |
|  | Norway | No info |  |
|  | Poland | Not implemented | Under study |
|  | Portugal | No info |  |
|  | Russian Federation | No info |  |
|  | Serbia | No info |  |
|  | Slovak Republic | No info |  |
|  | Spain | Not implemented |  |
|  | Sweden | No info |  |
|  | Turkey | No info |  |
|  | United Kingdom | Not implemented | Planned for 2012 |
| Annex 7 Band A Alarms 868.600-868.700 MHz | Georgia | No info |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Limited implementation | The maximal transmitter power 10 mW |
| Annex 7 Band B Alarms 869.250-869.300 MHz | Georgia | No info |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | No info |  |
| Annex 7 Band C Alarms 869.650-869.700 MHz | Georgia | No info |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Not implemented | Under study |
| Annex 7 Band D Alarms 869.200-869.250 MHz | Georgia | No info |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Limited implementation | The maximal transmitter power 10 mW |
| Annex 7 Band E Alarms 869.300-869.400 MHz (Technical parameters have been changed) | Georgia | No info |  |
|  | Greece | Not implemented |  |
|  | Macedonia (FYROM) | Not implemented | Planned |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | No info |  |
| Annex 7 Band F Alarms 169.4750-169.4875 MHz | Austria | Not implemented | Planned |
|  | Bulgaria | Not implemented | The band is used for national security needs |
|  | Cyprus | Implemented | Cyprus has implemented Decision 2005/928/EC |
|  | Denmark | Not implemented | PMR band |
|  | Georgia | Not implemented |  |
|  | Greece | Not implemented |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Not implemented |  |
| Annex 7 Band G Alarms 169.5875-169.6000 MHz | Austria | Not implemented | Planned |
|  | Bulgaria | Not implemented | The band is used for national security needs |
|  | Cyprus | Implemented | Cyprus has implemented Decision 2005/928/EC |
|  | Denmark | Not implemented | PMR band |
|  | Georgia | Not implemented |  |
|  | Greece | Not implemented |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Not implemented |  |
| Annex 8 Band A Model Control 26.995, 27.045, 27.095, 27.145, 27.195 MHz | Georgia | No info |  |
|  | Russian Federation | Limited implementation | Power limited to 10 mW. Maximum antenna gain is 3 dB, channel spacing 50 kHz |
|  | Ukraine | Limited implementation | The maximal transmitter power 10 mW |
| Annex 8 Band B Model Control 34.995-35.225 MHz | France | Limited implementation | Limited to 34.995-35.055 MHz. Dedicated networks for Ministry of transport. |
|  | Georgia | No info |  |
|  | Germany | Limited to 35.005-35.205 MHz | Emergency services |
|  | Russian Federation | Not implemented |  |
|  | Spain | Limited implementation | to 35.030-35.200 MHz |
|  | Ukraine | Limited implementation | The maximal transmitter power 10 mW |
| Annex 9 Band A1 Inductive applications 9 – 90 kHz | Austria | Not implemented |  |
|  | Belgium | No info |  |
|  | Bosnia and Herzegovina | Not implemented | Committed |
|  | Bulgaria | Partly implemented | Implemented in the band 9-59.75 kHz. The bands 59.75-60.25 kHz and 70-90 kHz are allowed with a maximum magnetic field strength of 42 dBµA/m at 10 m. The band 60.25-70.0 kHz is allowed with a maximum magnetic field strength of 69 dBµA/m at 10 m |
|  | Cyprus | No info |  |
|  | Denmark | Limited implementation | Implemented according to the EC SRD Decision 2006/771/EC |
|  | France | Limited implementation | Limited to +42 dBµA/m in the frequency band 70-90 kHz |
|  | Georgia | Not implemented |  |
|  | Greece | No info |  |
|  | Ireland | No info |  |
|  | Italy | No info |  |
|  | Latvia | Limited implementation | 9-59.750 kHz maximum field strength 72 dBµA/m at 10m; 59.750-60.250 kHz maximum field strength 42 dBµA/m at 10m; 60.250-70 kHz maximum field strength 69 dBµA/m at 10m; 70-119 kHz maximum field strength 42 dBµA/m at 10m |
|  | Lithuania | Limited implementation | Implemented according to the EC SRD Decision 2006/771/EC |
|  | Luxembourg | Implemented | New notification will be done |
|  | Malta | Not implemented | Malta implemented the provision of 2006/771/EC, as amended |
|  | Macedonia (FYROM) | No info |  |
|  | Norway | No info |  |
|  | Portugal | No info |  |
|  | Russian Federation | Limited implementation | 9-59.75 kHz: Maximum magnetic field strength is +72 dBµA/m at 10 m. In case of external antennas only loop coil antennas may be employed. Field strength level descending 3 dB/oct at 30 kHz. 59.75-60.25 kHz: Maximum magnetic field strength is +42 dBµA/m at 10 m. In case of external antennas only loop coil antennas may be employed. 60.25-70 kHz: Maximum magnetic field strength is +69 dBµA/m at 10 m. In case of external antennas only loop coil antennas may be employed. Field strength level descending 3dB/oct at 30 kHz. 70-90 kHz: Maximum magnetic field strength is +42 dBµA/m at 10 m. In case of external antennas only loop coil antennas may be employed |
|  | Serbia | No info |  |
|  | Slovak Republic | No info |  |
|  | Spain | Limited implementation | to 9-70 kHz |
|  | The Netherlands | Not implemented | Planned (Pending) |
|  | Turkey | No info |  |
|  | Ukraine | Limited implementation | The maximal strength of magnetic field on the distance of 10 m from a construction where the radiator is placed in the band 9-59.75 kHz is 72 dBmµA/m, in the band 59.75-60.25 kHz is 42 dBmµA/m, in the band 60.250-70 kHz is 69 dBmµA/m, in the band 70-119 kHz is 42 dBmµA/m |
| Annex 9 Band A2 Inductive applications 90-119 kHz | Austria | Limited to 42 dBµA/m at 10m | According to Subclass 40 and Commission Decision 2009/381/EC |
|  | Belgium | No info |  |
|  | Bosnia and Herzegovina | Not implemented | Committed |
|  | Cyprus | No info |  |
|  | Denmark | Limited implementation | Implemented according to the EC SRD Decision 2006/771/EC |
|  | Georgia | Limited implementation |  |
|  | Greece | No info |  |
|  | Ireland | No info |  |
|  | Italy | No info |  |
|  | Latvia | Partly implemented |  |
|  | Lithuania | Limited implementation | Implemented according to the EC SRD Decision 2006/771/EC |
|  | Luxembourg | Implemented | (New notification will be done) |
|  | Malta | Not implemented | Malta implemented the provision of 2006/771/EC, as amended |
|  | Macedonia (FYROM) | No info |  |
|  | Norway | No info |  |
|  | Portugal | No info |  |
|  | Serbia | No info |  |
|  | Slovak Republic | No info |  |
|  | The Netherlands | Not implemented | Planned (Pending) |
|  | Turkey | No info |  |
|  | Ukraine | Limited implementation | The maximal strength of magnetic field on the distance of 10 m from a construction where the radiator is placed in the band 70-119 kHz is 42 dBmµA/m |
| Annex 9 Band A3 Inductive applications 119-135 kHz | Georgia | Not implemented |  |
|  | Lithuania | Limited implementation | Implemented according to the EC SRD Decision 2006/771/EC |
|  | The Netherlands | Not implemented | Planned (Pending) |
|  | Ukraine | Limited implementation | The maximal strength of magnetic field on the distance of 10 m from a construction where the radiator is placed in the band 119-135 kHz is 66 dBmµA/m |
| Annex 9 Band B Inductive applications 135-140 kHz | Georgia | Not implemented |  |
|  | Greece | Not implemented |  |
|  | Hungary | Not implemented | Not allocated. Planned |
|  | Russian Federation | Not implemented |  |
|  | The Netherlands | Not implemented | Planned (Pending) |
|  | Ukraine | Limited implementation | The maximal strength of magnetic field on the distance of 10 m from a construction where the radiator is placed in the band 135-140 kHz is 42 dBmµA/m |
| Annex 9 Band C Inductive applications 140.0-148.5 kHz | Georgia | Not implemented |  |
|  | Greece | Not implemented |  |
|  | Russian Federation | Not implemented |  |
|  | The Netherlands | Not implemented | Planned (Pending) |
|  | Ukraine | Limited implementation | The maximalstrength of magnetic field on the distance of 10 m from a construction where the radiator is placed in the band 140-148.5 kHz is 37.7 dBmµA/m |
| Annex 9 Band D Inductive applications 6765-6795 kHz | Georgia | Not implemented |  |
|  | Ukraine | No info |  |
| Annex 9 Band E Inductive applications 7400-8800 kHz | Spain | No restriction | Frequency band 7350-8800 kHz |
| Annex 9 Band F1 Inductive applications 13.553-13.567 MHz | Georgia | Not implemented |  |
|  | Ukraine | Limited implementation | The maximalstrength of magnetic field on the distance of 10 m from a construction where the radiator is placed is 42 dBmµA/m |
| Annex 9 Band G Inductive applications 26.957-27.283 MHz | Georgia | Not implemented |  |
|  | Ukraine | Limited implementation | The maximalstrength of magnetic field on the distance of 10 m from a construction where the radiator is placed is 42 dBmµA/m |
| Annex 9 Band H Inductive applications 10.200-11.000 MHz | Georgia | Not implemented |  |
|  | Russian Federation | Limited implementation | Maximum magnetic field strength is -4 dBµA/m at 10 m |
|  | Ukraine | Limited implementation | The maximalstrength of magnetic field on the distance of 10 m from a construction where the radiator is placed is 9 dBmµA/m |
| Annex 9 Band K Inductive applications 3155-3400 kHz | Georgia | Not implemented |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Limited implementation | The maximalstrength of magnetic field on the distance of 10 m from a construction where the radiator is placed is 13.5 dBmµA/m |
| Annex 9 Band L1 Inductive applications 148.5 kHz-5 MHz | Georgia | Not implemented |  |
|  | Greece | Not implemented |  |
|  | Poland | Limited implementation | Implemented 148.5 kHz - 1.6. MHz |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Not implemented | Under study |
| Annex 9 Band L2 Inductive applications 5-30 MHz | Georgia | Not implemented |  |
|  | Greece | Not implemented |  |
|  | Poland | Not implemented | Planned in 2011 |
|  | Russian Federation | No info |  |
|  | Ukraine | No info |  |
| Annex 9 Band L3 Inductive applications 400-600 kHz | Georgia | Not implemented |  |
|  | Greece | Not implemented |  |
|  | Poland | Not implemented | Planned in 2011 |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Not implemented | Under study |
| Annex 10 Band A Radio Microphone applications including aids for the hearing impaired29.7-47.0 MHz | Austria | Limited implementation | only the frequencies 36.8, 36.85, 37.45, 37.50-37.55 MHz for narrow band and 36.7-37.1-44.55-45.0 MHz for broadband radio microphones are available |
|  | Croatia | Not implemented | Defence systems |
|  | Czech Republic | Limited implementation | Only four sub-bands allowed: 27.415-27.915 MHz 10 mW e.r.p. channel max 50 kHz 36.4-36.65 MHz 10 mW e.r.p. channel max 50 kHz 36.65-38.0 MHz 2 mW e.r.p. channel max 50 kHz 38.0-38.5 MHz 10 mW e.r.p. channel max 200 kHz |
|  | Estonia | Limited to 37.6-38.6 MHz | Land mobile |
|  | Finland | Limited implementation | only 31.1, 32.1, 32.9, 33.5, 36.7, 37.1 and 42.4-43.6 MHz with max 200 kHz channels |
|  | France | Limited implementation | to 32.8, 36.4, 39.2 MHz 1 mW e.r.p. and 200 kHz |
|  | Georgia | Not implemented |  |
|  | Germany | Limited implementation | to 32.4-38.2 MHz. Permitted channel spacing 10 kHz below 36 MHz and 40 kHz above 36 MHz |
|  | Greece | Limited implementation | to 30.00 MHz, 30.50 MHz, 31.00 MHz, 35.00 MHz, 36.50 MHz, 36.70 MHz, 37.00 MHz, 37.10 MHz, 37.50 MHz |
|  | Hungary | Limited implementation | 34.9-38.5 MHz band is allocated |
|  | Italy | Limited to 41.0-43.6 MHz | Military application |
|  | Liechtenstein | Limited implementation | Limited to 31.4-39.6 MHz |
|  | Lithuania | Limited implementation | only 30.01-30.3 MHz, 30.5-32.15 MHz, and 32.45-37.5 MHz are allowed |
|  | Luxembourg | Limited implementation | excluding the use of the band 34.995-35.225 MHz |
|  | Malta | Limited implementation | to 29.7-34.9 and 37.5-40.98 MHz |
|  | Norway | Limited implementation | to 41.0-43.6 MHz max channel spacing 10 kHz. Max 100 mW e.r.p. AM not allowed |
|  | Portugal | Not implemented | Defence systems |
|  | Russian Federation | Limited implementation | Hearing and speech training radio devices for persons with speech defects. Power limited to 10 mW Fixed frequencies in the bands 33.175-40MHz and 40.025-48.5 MHz: 33.2, 33.35, 33.45, 33.55, 33.575, 33.6, 33.75, 33.85, 33.875, 33.9, 34.05, 34.15, 34.175, 34.2, 34.3, 34.375, 34.4, 34.975, 35.025, 35.15, 35.225, 35.375, 35.55, 35.65, 35.95, 35.975, 36.025, 36.075, 36.125, 36.175, 36.225, 36.275, 36.325, 36.375, 36.425, 36.475, 36.525, 36.575, 36.625, 36.675, 36.725, 36.775, 36.825, 36.875, 36.925, 36.975, 37.025, 37.075, 37.125, 37.175, 37.225, 37.275, 37.325, 37.375, 37.425, 37.475, 37.525, 37.575, 37.625, 37.675, 37.725, 37.775, 37.825, 37.875, 37.925, 37.975, 38.025, 38.075, 38.125, 38.175, 38.225, 38.275, 38.325, 38.375, 38.425, 38.475, 38.525, 38.575, 38.625, 38.675, 38.725, 38.775, 39.025, 39.225, 39.400, 39.6, 39.75, 39.85, 39.925, 39.975, 40.05, 40.15, 40.25, 40.325, 40.425, 40.65, 40.825, 41.3, 41.325, 41.35, 41.375, 41.4, 41.5, 41.6, 41.625, 41.65, 41.675, 41.7, 41.75, 41.8, 41.9, 41.95, 42.1, 42.15, 42.2, 42.25, 42.35, 42.45, 42.475, 42.5, 42.525, 42.55, 42.575, 42.6, 42.625, 42.65, 42.675, 42.7, 42.725, 42.75, 42.8, 42.85, 42.95, 42.975, 43, 43.15, 43.175, 43.2, 43.225, 43.25, 43.4, 43.5, 43.7, 43.725, 43.75, 43.8, 44, 44.25, 44.4, 44.475, 44.5, 44.65, 44.75, 44.975, 45, 45.25, 45.45, 45.475, 45.5, 45.65, 45.75, 45.8, 45.95, 45.975, 46, 46.125. 46.175, 46.225, 46.425, 46.45, 46.475, 46.55, 46.575, 46.6, 46.65, 46.675, 46.7, 46.775, 46.8, 46.825, 46.85, 46.875, 46.925, 46.95, 46.975, 47, 47.075, 47.125, 47.25 MHz |
|  | Slovak Republic | Limited to 27.75-27.9 and 36.4-38.5 MHz | Defence systems in the rest of the band |
|  | Spain | Limited implementation | to 31.500, 31.750, 37.850, 38.300 and 38.550 MHz |
|  | Sweden | Limited implementation | Limited to 41.0-43.6 MHz - Land Mobile |
|  | Switzerland | Limited implementation | Limited to 31.4-39.6 MHz. Main use by defence systems |
|  | Ukraine | Limited implementation | In the band 30.01-47 MHz maximal transmitter power is 10 mW |
|  | United Kingdom | Not implemented |  |
| Annex 10 Band B Radio Microphone applications including aids for the hearing impaired173.965-174.015 MHz | Belgium | Not implemented |  |
|  | Bulgaria | Limited implementation | Limited to 174.000-174.015 MHz |
|  | Denmark | Not implemented | PMR band |
|  | France | Not implemented | Governmental band |
|  | Georgia | Not implemented |  |
|  | Greece | Not implemented |  |
|  | Liechtenstein | Not implemented | Occupied with mobile services |
|  | Poland | Not implemented | Planned in 2011 |
|  | Russian Federation | Not implemented |  |
|  | Spain | Not implemented | Not implemented due to lack of demand |
|  | Sweden | Not implemented | Land Mobile |
|  | Switzerland | Not implemented | Closely occupied with mobile services |
|  | Ukraine | Not implemented |  |
| Annex 10 Band C Radio Microphone applications including aids for the hearing impaired863-865 MHz | Croatia | Limited implementation | Individual license required |
|  | Georgia | Not implemented |  |
|  | Ireland | Implemented | Channel spacing of 200 kHz |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Limited implementation | The maximal transmitter power is 10 mW |
| Annex 10 Band D Radio Microphone applications including aids for the hearing impaired174-216 MHz | Denmark | Limited implementation | Tuning range |
|  | Finland | Limited implementation | Regional restrictions |
|  | France | Limited implementation | For professional users. 175.5-178.5 and 183.5-186.5 MHz also authorised for consumer products with 10 mW e.r.p. and 200 kHz channel spacing |
|  | Georgia | Not implemented |  |
|  | Ireland | Not implemented |  |
|  | Malta | Not implemented |  |
|  | Norway | Not implemented |  |
|  | Russian Federation | Limited implementation | 174-230 MHz. Power limited to 5 mW. Maximum antenna gain is 3 dB. Channel spacing is 200 kHz |
|  | Spain | Limited implementation | 174.100, 174.300, 175.500, 176.300, 179.300, 188.100, 188.500, 189.100, 191.900 and 194.500 MHz |
|  | Ukraine | Limited implementation | Under condition of not causing interference to other stations working in this band. In bands of 174.4-174.6 MHz and 174.9-175.1 MHz the maximal transmitter power is 10 mW |
| Annex 10 Band E1 Radio Microphone applications including aids for the hearing impaired470-786 MHz | Czech Republic | Partly implemented under conditions of former band E | This band will replace previous band 470–862 MHz arrangement before 2013 |
|  | Finland | Limited implementation | Regional restrictions. Radiomicrophones in the frequency band 694-786 MHz allowed until the end of year 2020 |
|  | France | Limited implementation | For professional users |
|  | Germany | Limited implementation |  |
|  | Greece | Limited implementation |  |
|  | Ireland | Not implemented |  |
|  | Lithuania | Limited implementation | In all 470–862 MHz band 50 mW e.r.p. Only for radio microphones. Individual registrations required |
|  | Malta | Limited implementation |  |
|  | Norway | Limited implementation |  |
|  | Poland | Limited implementation | Radio Microphones and Assistive Listening Devices are allowed in the whole band 470 - 862 MHz until introduction of MFCN networks in Poland. After that frequency band will be limited to the band 470-786 MHz. Individual licensing under study |
|  | Spain | Not implemented | Only broadcasting TV in this band |
| Annex 10 Band E2 Radio Microphone applications including aids for the hearing impaired786-789 MHz | Austria | No info |  |
|  | Belgium | No info |  |
|  | Croatia | Not implemented |  |
|  | Czech Republic | Partly implemented under conditions of former band E | This band will replace previous band 470–862 MHz arrangement before 2013 |
|  | Cyprus | No info |  |
|  | Finland | Limited implementation | Regional restrictions. Radiomicrophones in the frequency band 694-786 MHz allowed until the end of year 2020 |
|  | France | Limited implementation | For professional users |
|  | Greece | No info |  |
|  | Hungary | No info |  |
|  | Latvia | No info |  |
|  | Liechtenstein | Limited implementation | 50 mW e.r.p. until 31.12.2012 |
|  | Lithuania | Limited implementation | In all 470–862 MHz band 50 mW e.r.p. Only for radio microphones. Individual registrations required |
|  | Luxembourg | Limited implementation |  |
|  | Malta | Not implemented |  |
|  | The Netherlands | Not implemented |  |
|  | Norway | Not implemented |  |
|  | Poland | Limited implementation | With technical parameters for the “old” band E. Full implementation and individual licensing under study |
|  | Portugal | Not implemented |  |
|  | Romania | Not implemented |  |
|  | Slovak Republic | Not implemented |  |
|  | Spain | Not implemented | Only broadcasting TV in this band |
|  | Sweden | Not implemented |  |
|  | Switzerland | Limited implementation | 50 mW e.r.p. until 31.12.2012 |
| Annex 10 Band E3 Radio Microphone applications including aids for the hearing impaired823-826 MHz | Austria | No info |  |
|  | Belgium | No info |  |
|  | Croatia | Not implemented |  |
|  | Czech Republic | Partly implemented under conditions of former band E | This band will replace previous band 470–862 MHz arrangement before 2013 |
|  | Cyprus | No info |  |
|  | Estonia | Not implemented | Under study |
|  | France | Limited implementation | For professional users. Limited to 50 mW e.r.p. |
|  | Greece | No info |  |
|  | Hungary | No info |  |
|  | Latvia | No info |  |
|  | Liechtenstein | Limited implementation | 50 mW e.r.p. until 31.12.2012 |
|  | Lithuania | Limited implementation | In all 470–862 MHz band 50 mW e.r.p. Only for radio microphones. Individual registrations required |
|  | Luxembourg | Limited implementation |  |
|  | Malta | Not implemented |  |
|  | Norway | Not implemented |  |
|  | Poland | Limited implementation | With technical parameters for the “old” band E. Full implementation and individual licensing under study |
|  | Portugal | Not implemented |  |
|  | Romania | Not implemented |  |
|  | Slovak Republic | Not implemented |  |
|  | Spain | Not implemented | Only broadcasting TV in this band |
|  | Sweden | Not implemented |  |
|  | Switzerland | Limited implementation | 50 mW e.r.p. until 31.12.2012 |
|  | The Netherlands | Not implemented |  |
| Annex 10 Band E4 Radio Microphone applications including aids for the hearing impaired 826-832 MHz | Austria | No info | Austria |
|  | Belgium | No info | Belgium |
|  | Croatia | Not implemented | Croatia |
|  | Czech Republic | Partly implemented under conditions of former band E | Czech Republic |
|  | Cyprus | No info | Cyprus |
|  | Estonia | Not implemented | Estonia |
|  | France | Limited implementation | France |
|  | Greece | No info | Greece |
|  | Hungary | No info | Hungary |
|  | Latvia | No info | Latvia |
|  | Liechtenstein | Limited implementation | 50 mW e.r.p. until 31.12.2012 |
|  | Lithuania | Limited implementation | In all 470–862 MHz band 50 mW e.r.p. Only for radio microphones. Individual registrations required |
|  | Luxembourg | Limited implementation |  |
|  | Malta | Not implemented |  |
|  | Norway | Not implemented |  |
|  | Poland | Limited implementation | With technical parameters for the “old” band E. Full implementation and individual licensing under study |
|  | Portugal | Not implemented |  |
|  | Romania | Not implemented |  |
|  | Slovak Republic | Not implemented |  |
|  | Spain | Not implemented | Only broadcasting TV in this band |
|  | Sweden | Not implemented |  |
|  | Switzerland | Limited implementation | 50 mW e.r.p. until 31.12.2012 |
|  | The Netherlands | Not implemented |  |
| Annex 10 Band F Radio Microphone applications including aids for the hearing impaired1785-1795 MHz | Austria | Limited implementation | to 1785.7-1795 MHz |
|  | Georgia | Not implemented |  |
|  | Italy | Not implemented | Military application |
|  | Ireland | Not implemented | All island WAPECS licence in operation |
|  | Malta | Not implemented | Planned |
|  | Russian Federation | Not implemented |  |
|  | Slovak Republic | Not implemented | Fixed Service |
|  | Sweden | Not implemented |  |
|  | The Netherlands | Implemented | max 50 mW e.r.p. Channel spacing 600 kHz |
|  | Ukraine | Not implemented | Under study |
|  | United Kingdom | Implemented | Individual licence required |
| Annex 10 Band G Radio Microphone applications including aids for the hearing impaired1795-1800 MHz | Austria | Limited implementation | to the band 1795 - 1799.4 MHz |
|  | Croatia | Limited implementation | Individual licence required |
|  | Czech Republic | Limited implementation | Individual license required |
|  | Finland | Limited implementation | Individual license required |
|  | Georgia | Not implemented |  |
|  | Italy | Not implemented | Military application |
|  | Ireland | Not implemented | All island WAPECS licence in operation |
|  | Russian Federation | Not implemented |  |
|  | Slovak Republic | Not implemented | Fixed Service |
|  | Sweden | Not implemented |  |
|  | The Netherlands | Implemented | max 50 mW e.r.p. Channel spacing 600 kHz |
|  | Ukraine | Not implemented | Under study |
|  | United Kingdom | Limited implementation | Individual licence required |
| Annex 10 Band H1 Radio Microphone applications including aids for the hearing impaired169.4000-169.4750 MHz | Austria | Not implemented | Planned |
|  | Bulgaria | Not implemented | The band is used for national security needs |
|  | Cyprus | Implemented | Cyprus has implemented Decision 2005/928/EC |
|  | Denmark | Not implemented | PMR band |
|  | Georgia | Not implemented |  |
|  | Greece | Not implemented |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Not implemented |  |
| Annex 10 Band H2 Radio Microphone applications including aids for the hearing impaired169.4875-169.5875 MHz | Austria | Not implemented | Planned |
|  | Bulgaria | Not implemented | The band is used for national security needs |
|  | Cyprus | Implemented | Cyprus has implemented Decision 2005/928/EC |
|  | Denmark | Not implemented | PMR band |
|  | Georgia | Not implemented |  |
|  | Greece | Not implemented |  |
|  | Ireland | Not implemented | Planned; Notification in progress |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Not implemented |  |
| Annex 10 Band I Radio Microphone applications including aids for the hearing impaired 169.4-174.0 MHz | Austria | Not implemented | Implementation depends on market demand |
|  | Belgium | Not implemented |  |
|  | Bulgaria | Not implemented | The band is used for national security needs |
|  | Cyprus | Not implemented |  |
|  | Czech Republic | Limited implementation | Only two parts of the band allowed above 169.5875 MHz 173.3 MHz: 50 mW e.r.p. max 75 kHz 173.965-174.015 MHz: 2 mW e.r.p. channel spacing max 50 kHz. Other services in the rest of the band |
|  | Finland | Not implemented |  |
|  | France | Not implemented |  |
|  | Georgia | Not implemented |  |
|  | Greece | Not implemented |  |
|  | Hungary | Not planned | Governmental use in the band |
|  | Iceland | No info |  |
|  | Ireland | Not implemented |  |
|  | Italy | Limited to 169.815 MHz |  |
|  | Liechtenstein | Not implemented | Occupied with mobile services |
|  | Luxembourg | Implemented | (Notification Number: 2009/0375/L) |
|  | Malta | Not implemented |  |
|  | Poland | Not implemented |  |
|  | Portugal | Not implemented | Land Mobile |
|  | Russian Federation | Not implemented |  |
|  | Serbia | Not implemented | In the Frequency Plan in this part of the spectrum there are not available frequency slots for the radio microphones |
|  | Slovak Republic | Not implemented | Under study |
|  | Spain | Limited implementation | Channel plan for 169.4-169.8 MHz according ECC/DEC/(05)02 |
|  | Sweden | Not implemented |  |
|  | Switzerland | Not implemented | Occupied with mobile services |
|  | The Netherlands | Not implemented | Planned |
|  | Turkey | Not implemented | 169.8-174.0 MHz band is used by PMR/PAMR |
|  | Ukraine | Not implemented |  |
|  | United Kingdom | Limited implementation | Implemented in 173.325-174.000 MHz and at 2 mW only |
| Annex 11 Band A RFID 2446-2454 MHz | France | Limited implementation | Power limited to 500 mW e.i.r.p. Military Radiolocation and Fixed Service use |
|  | Russian Federation | Not implemented |  |
|  | Sweden | Limited implementation | Limited to 100 mW e.i.r.p. Defence systems |
|  | Ukraine | Not implemented | Under study |
| Annex 11 Band B1 RFID 865.0-865.6 MHz | Georgia | No info |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Not implemented | Under study |
| Annex 11 Band B2 RFID 865.6-867.6 MHz | France | Limited implementation | Power limited to 500 mW e.r.p. within defined zones around certain military camps in France (see list of military camps with geographical coordinates in national radio interface specification). Tactical Radio Relay |
|  | Georgia | No info |  |
|  | Russian Federation | Limited implementation | 866.6-867.4 MHz with e.r.p 100 mW. The assignment of radio frequencies or channels is not required in when: a) LBT is applied b) equipment is used at the airport 866.0-867.6 MHz with e.r.p 2 W. The assignment of radio frequencies or channels should too be performed in established order |
|  | Ukraine | Not implemented | Under study |
| Annex 11 Band B3 RFID 867.6-868.0 MHz | Georgia | No info |  |
|  | Russian Federation | Limited implementation | 866-868 MHz. The assignment of radio frequencies or channels should too be performed in established order |
|  | Ukraine | Not implemented | Under study |
| Annex 12 Band A Active Medical Implants and their associated peripherals 402-405 MHz | Georgia | No info |  |
|  | Romania | Limited implementation | The operation of these implants is possible in Romania only in the 402-403 MHz and 404-405 MHz frequency sub-bands. In the 403-404 MHz sub-band there are high power PMR applications. These applications cannot be migrated in other bands because of the current lack of available spectrum in the UHF bands designated for the narrow band PMR/PAMR systems |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Limited implementation | The maximal transmitter power is 25 mµW |
| Annex 12 Band A1 Active Medical Implants and their associated peripherals 401-402 MHz | Belgium | Not implemented | Planned |
|  | Georgia | No info |  |
|  | Greece | Not implemented | About to be implemented (info of June 2008) |
|  | Hungary | Not implemented | Planned 2011 |
|  | Italy | Not implemented | Military application |
|  | Russian Federation | Not implemented |  |
|  | Serbia | Not implemented | In the Frequency Plan in this part of the spectrum there are not available frequency slots for this applications |
|  | Spain | Not implemented | Planned |
|  | Turkey | Under study | Planned 2009 |
| Annex 12 Band A2 Active Medical Implants and their associated peripherals 405-406 MHz | Belgium | Not implemented | Planned |
|  | Georgia | No info |  |
|  | Greece | Not implemented | About to be implemented (info of June 2008) |
|  | Italy | Not implemented | Military application |
|  | Russian Federation | Not implemented |  |
|  | Serbia | Not implemented | In the Frequency Plan in this part of the spectrum there are not available frequency slots for this applications |
|  | Spain | Not implemented | Planned |
|  | Turkey | Under study | Planned 2009 |
| Annex 12 Band B Active Medical Implants and their associated peripherals 9-315 kHz | Georgia | No info |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Limited implementation | The maximal strength of magnetic field on the distance of 10 m from a construction where the radiator is placed is 30 dBmµA/m |
| Annex 12 Band C Active Medical Implants and their associated peripherals 315-600 kHz | Georgia | No info |  |
|  | Italy | Not implemented |  |
|  | Russian Federation | Not implemented |  |
|  | Ukraine | Limited implementation | The maximal strength of magnetic field on the distance of 10 m from a construction where the radiator is placed is 30 dBmµA/m |
| Annex 12 Band D Active Medical Implants and their associated peripherals 30.0-37.5 MHz | Georgia | No info |  |
|  | Italy | Not implemented | Military application |
|  | Russian Federation | Not implemented |  |
|  | Serbia | Not implemented | In the Frequency Plan in this part of the spectrum there are not available frequency slots for this applications |
|  | Slovak Republic | Limited to 33 – 37.5 MHz | Defence systems and other services in the rest of the band |
|  | Spain | Not implemented | Planned |
|  | Ukraine | Limited implementation | The maximal transmitter power is 1 mW |
| Annex 12 Band E Active Medical Implants and their associated peripherals 12.5-20.0 MHz | Belgium | Not implemented | Planned |
|  | Georgia | No info |  |
|  | Greece | Not implemented | Planned |
|  | Italy | Not implemented |  |
|  | Russian Federation | Not implemented |  |
|  | Serbia | Available in the range: 13.553-13.567 MHz | According to the Frequency Plan, this part of the spectrum is available for the SRD applications |
|  | Slovak Republic | Not implemented | Under study |
|  | Spain | Not implemented | Planned |
|  | Ukraine | Not implemented | Under study |
| Annex 12 Band F Active Medical Implants and their associated peripherals 2483.5-2500 MHz | Belgium | No info |  |
|  | Bulgaria | No info |  |
|  | Croatia | Not implemented |  |
|  | Czech Republic | Not implemented | Planned 2011 |
|  | Cyprus | No info |  |
|  | Denmark | No info |  |
|  | Estonia | Not implemented | Under study |
|  | Finland | Not implemented | Planned 2012 |
|  | France | Not implemented |  |
|  | Greece | No info |  |
|  | Hungary | No info |  |
|  | Iceland | No info |  |
|  | Italy | No info |  |
|  | Liechtenstein | Not implemented | Planned |
|  | Lithuania | Not implemented |  |
|  | Luxembourg | Not implemented |  |
|  | Malta | Not implemented |  |
|  | Norway | Not implemented |  |
|  | Poland | Not implemented | Under study |
|  | Portugal | Not implemented |  |
|  | Romania | Not implemented |  |
|  | Slovak Republic | Not implemented |  |
|  | Slovenia | Not implemented | Planned |
|  | Spain | Not implemented |  |
|  | Sweden | Not implemented |  |
|  | Switzerland | Not implemented | Planned |
|  | The Netherlands | Not implemented |  |
|  | United Kingdom | Not implemented |  |
| Annex 13 Band B Wireless Audio Applications 864.8-865 MHz | Russian Federation | Not implemented |  |
|  | Ukraine | Limited implementation | e.i.r.p. ≤10 MBT |
| Annex 13 Band C Wireless Audio Applications 1795-1800 MHz | Austria | Not implemented |  |
|  | Croatia | Not implemented | Lack of demand |
|  | Finland | Limited implementation | Individual license required |
|  | France | Not implemented |  |
|  | Georgia | Limited implementation |  |
|  | Ireland | Not implemented | All island WAPECS licence in operation |
|  | Italy | Not implemented | Military application |
|  | Russian Federation | Not implemented |  |
|  | Slovak Republic | Not implemented | Fixed service |
|  | Spain | Not implemented |  |
|  | The Netherlands | Not implemented |  |
|  | Turkey | Planned | Implemented after SRD Ordinance is revised |
|  | Ukraine | Not implemented | Under study |
|  | United Kingdom | Limited implementation | Individual licence required |
| Annex 13 Band D Wireless Audio Applications 87.5-108.0 MHz | Greece | Not implemented | About to be implemented (info of June 2008) |
|  | Russian Federation | Limited implementation | Maximum e.i.r.p. -43 dBm (50 nW). No spacing. Omnidirectional antenna. Permitted to use inside cars and other vehicles, and also inside of the closed premises |
|  | Turkey | Planned | Implemented after SRD Ordinance is revised |
|  | Ukraine | Limited implementation | 87.5-92 MHz; 100-108 MHz; (e.i.r.p. ≤50*10-9W); 89.9-90.2 MHz (the maximal transmitter power is 10 mW) |

# Appendix 4 – Illustrations

# 864-870 MHz frequency range

# 2400-2483.5 MHz frequency range

- Automatic Vehicle Identification for Railways (AVI) use 5 channels 1.5 MHz wide within the band 2446-2454 MHz  with 500 mW e.i.r.p. Transmission only in presence of train.

- Radio Frequency Identification (RFID) use 2446-2454 MHz with

- 500 mW e.i.r.p.indoor and outdoor using FHSS or  un-modulated carrier (CW)

- 4 W e.i.r.p. indoor only using FHSS only and with duty cycle limited to 15 %

- For direct sequence spread spectrum (DSS) the maximum spectrum power density is limited to

- -20 dBW/1 MHz. For frequency hopping spread spectrum (FHSS) the maximum spectrum density is limited to -10 dBW/100 kHz

General SRD  - 10 mW

AVI1

Movement and detection -  25 mW

RFID2

2400 MHz

2446

2454

2483.5 MHz

Wide Band Data transmission systems (RLANs) - 100 mW3

# 433 MHz frequency range (to be included)

List of abbreviations as used in this document

List of abbreviations as used in this document

| List of abbreviations as used in this document |  |
| --- | --- |
| AFA | Adaptive Frequency Agility |
| AVI | Automatic Vehicle Identification for Railways |
| BMA | Building Material Analysis |
| CEPT | European Conference of Postal and Telecommunications Administrations |
| CB | Citizens’ Band (27 MHz) |
| DAA | Detect and Avoid |
| DFS | Dynamic Frequency Selection |
| EAS | Electronic Article Surveillance |
| ECC | Electronic Communications Committee |
| ECO | European Communications Office |
| EFIS | ECO Frequency Information System |
| ENG/OB | Electronic News Gathering / Outside Broadcasting |
| ERC | European Radiocommunications Committee |
| ERM | Electromagnetic Compatibility and Radio Spectrum Matters |
| ETSI | European Telecommunications Standard Institute |
| FHSS | Frequency Hopping Spread Spectrum |
| FMCW | Frequency Modulated Continuous Wave |
| GBSAR | Ground Based Synthetic Aperture Radar |
| FHSS | Frequency Hopping Spread Spectrum |
| GPR/WPR | Ground- and Wall Probing Radars |
| ISM | Industrial, Scientific and Medical applications |
| LAN | Local Area Network |
| LBT | Listen Before Talk |
| PMR | Professional Mobile Radio / Private Mobile Radio |
| PMSE | Programme Making Special Events |
| R&TTE | Directive 1999/5/EC of the European Parliament and of the Council of 9 March 1999on radio equipment and telecommunications terminal equipment and the mutual recognition of their conformity |
| RFID | Radio Frequency Identification |
| RTTT | Road Transport & Traffic Telematics |
| SRD | Short Range Devices |
| SRR | Short Range Radar |
| TETRA | Terrestrial Trunked Radio |
| TLPR | Tank Level Probing Radar |
| ULP-AID | Ultra Low Power Animal Implant Devices |
| ULP-AIP | Ultra Low Power Animal Implantable |
| UWB | Ultra WideBand |
| WAS | Wireless Access Systems |
| WLL | Wireless Local Loop |

Duty cycle categories

For the purposes of this Recommendation the duty cycle is defined as the ratio, expressed as a percentage, of the maximum transmitter “on” time on one carrier frequency, relative to a one hour period unless otherwise mentioned in the relevant Annex.

For pre-programmed devices the maximum transmitter “on” time and minimum “off” time are given in the following table. These limits are advisory with a view to facilitating sharing between systems in the same frequency band.

|  | Name | Transmitting time / Full cycle | Maximum transmitter “on” time (seconds) | Minimum transmitter “off” time (seconds) | Explanation |
| --- | --- | --- | --- | --- | --- |
| 1 | Very Low | <0.1% | 0.72 | 0.72 | For example, 5 transmissions of 0.72 seconds within one hour |
| 2 | Low | <1.0% | 3.6 | 1.8 | For example, 10 transmissions of 3.6 seconds within one hour |
| 3 | High | <10% | 36 | 3.6 | For example, 10 transmissions of 36 seconds within one hour |
| 4 | Very High | Up to 100% | - | - | Typically continuous transmissions but also those with a duty cycle greater than 10% |

Document History

|  | Text | Page | Edition |
| --- | --- | --- | --- |
| Text of the ERC Recommendation changed to align with the R&TTE Directive |  | 4 | October 2010 |
| Rearranged text of Recommendation 18 October 2005 |  |  |  |
| Annex 1 | Non-specific Short Range Devices | 6 | January 2010 |
| Annex 2 | Tracking, Tracing and Data Acquisition | 8 | June 2009 |
| Annex 3 | Wideband Data Transmission systems | 9 | June 2010 |
| Annex 4 | Railway applications | 10 | August 2011 |
| Annex 5 | Road Transport & Traffic Telematics (RTTT) | 11 | January 2010 |
| Annex 6 | Radiodetermination applications | 13 | February 2009 |
| Annex 7 | Alarms | 14 | October 2006 |
| Annex 8 | Model Control | 15 | May 2003 |
| Annex 9 | Inductive applications | 16 | October 2009 |
| Annex 10 | Radio microphones and Assistive Listening Devices | 18 | February 2011 |
| Annex 11 | Radio frequency identification applications | 20 | January 2010 |
| Annex 12 | Active Medical Implants and their associated peripherals | 21 | February 2011 |
| Annex 13 | Wireless Audio applications | 23 | May 2008 |
| Appendix 1 | Implementation Status | 24 | September 2011 |
| Appendix 2 | List of relevant ECC/ERC Decisions, Reports, EC Decisions and ETSI Standards | 30 | October 2010 |
| Appendix 3 | National restrictions | 36 | September 2011 |