| #### SRD/MG |  |
| --- | --- |
| 53rd Meeting of SRD/MG Biel/Switzerland, 07 – 09 September 2011 | SRDMG(11)082 |
| Date issued: 25 August 2011 Source: Germany Subject: ERC Recommendation 70-03 (Annex 3); Removal of the bands 5150-5350 MHz and 5470-5725 MHz |  |

N

Password protection required? (Y/N)

### Summary WG FM, during its latest meeting in May, tasked SRD/MG to further discuss in detail the proposal for a removal of the 5 GHz WAS/RLAN bands from Annex 3 of ERC Recommendation 70-03 and to provide an in-depth analysis of the impact of this proposal to WG FM, so that WG FM can make a decision on that issue during its meeting in October 2011 (see section 5.8.2, paragraph “Annex 3 - Wideband Data Transmission Systems” in the WG FM meeting Report).

### Proposal Germany proposes to remove the frequency bands 5150-5350 MHz and 5470-5725 MHz from Annex 3 of ERC Recommendation 70-03. SRD/MG is kindly invited to consider the proposal as described in detail below.

### Background WRC-03 allocated the bands 5150-5350 MHz and 5470-5725 MHz to the mobile except aeronautical mobile service with a primary status for the use for WAS/RLANs (provision No. 5.446A in Article 5 of the ITU Radio Regulations refers to Resolution 229). ECC developed and adopted ECC Decision (04)08 (latest amendment of Oct. 2009) for the 5 GHz WAS/RLAN systems. The European Commission developed and adopted EC Decision 2005/513/EC (amended by Decision 2007/90/EC) for the 5 GHz WAS/RLAN systems.

Introduction

WG FM considered a proposal from SRD/MG during its meeting in Miesbach/Germany in May 2011 on the possible removal of the 5 GHz WAS/RLAN bands from Annex 3 of ERC Recommendation 70-03. After a short discussion, WG FM tasked SRD/MG to further discuss in detail the proposal for the removal of the 5 GHz WAS/RLAN bands and to provide an in-depth analysis of the impact of this proposal to WG FM, so that WG FM can make a decision on that issue during its meeting in Lille/France in October 2011 (see section 5.8.2, paragraph “Annex 3 - Wideband Data Transmission Systems” in the WG FM #72 meeting Report).

The German administration would like to support the discussion on SRD/MG level and therefore provides its view herewith.

Background

WRC-03

WRC-03 decided to allocate the bands 5150-5350 MHz and 5470-5725 MHz to the mobile except aeronautical mobile service with a primary status for the implementation of wireless access systems including radio local area networks. This result was very much based on the European Common proposal submitted to WRC-03. The CEPT position to the relevant agenda item (AI 1.5 of WRC-03) can be summarised as follows (text taken from the CEPT Brief on AI 1.5 of WRC-03):

- Proposal for new primary allocations to the mobile service in the bands 5 150-5 350 and 5 470-5 725 MHz for the implementation of wireless access systems including RLANs ensuring the implementation of mitigation techniques to protect the existing primary services and the radiolocation service, based on ERC Decision ERC/DEC/(99)23. Any proposal to remove the non-application of 5.43A in 5.YYY shall be strongly opposed.

- Ensure protection of the Earth exploration-satellite (active) and space research services in the band 5 250-5 350 MHz. The allocation to the fixed service in Region 3 should not prevent the worldwide primary allocation to the mobile service as proposed under point 1.

- Support of an additional primary allocation to EESS (active) in the frequency range 5 460-5 570 MHz subject to no additional constraints to the worldwide primary allocation to the mobile service as proposed under point 1; furthermore, CEPT will not oppose any proposal for an allocation to SRS (active) within the frequency range 5460 – 5570 MHz.

- Upgrade of the status of frequency allocations to the radiolocation service in the frequency range 5 350-5 650 MHz. In the band 5 470-5 650 MHz the upgrade is also seen as a consequence to a worldwide primary allocation to the mobile service as proposed under point 1. In the band 5460 – 5570 MHz the upgrade is also seen as a consequence to a worldwide primary allocation to the EESS as extended under point 3.

A world-wide primary allocation to the mobile service is proposed in the frequency ranges 5 150-5 350 MHz and 5 470-5 725 MHz. This will provide globally harmonised frequency allocations to the mobile service in support of wireless access systems including RLANs, while ensuring the protection and on-going operation of existing services due to associated footnotes providing the appropriate technical and operational constraints on the systems operating in the mobile service.

It is obvious that the aim of CEPT was to implement WAS/RLANs in the 5 GHz bands based on an allocation to the mobile service with a primary status. With that regard the outcome of WRC-03 is completely in line with the ECP.

CEPT/ECC

After WRC-03 ECC developed and adopted ECC Decision (04)08 for the 5 GHz WAS/RLAN systems. This Decision implements the results of WRC-03 (AI 1.5) within CEPT, the latest amendment was adopted in October 2009. The former ERC Decision (99)23 on HIPERLANs was replaced by the new ECC Decision. It was also decided to add the allocation to the mobile except aeronautical mobile service in the ECA Table (ERC Report 25, latest version of 2009) for CEPT, together with provision No. 5.446A which refers to Resolution 229.

ERC Recommendation 70-03 was updated and the WAS/RLAN systems in the 5 GHz bands were implemented in Annex 3 of this Recommendation instead of the former HIPERLAN bands.

European Union

Following the allocations to the mobile (except aeronautical mobile) service and the adoption of the ECC Decision, the European Commission developed a Decision within the Radio Spectrum Committee (RSC) which was finally adopted as EC Decision 2005/513/EC (amended by EC Decision 2007/90/EC). So it is important to note that on EU level the 5 GHz WAS/RLAN bands have not been implemented in the EC Decision on short-range devices (2006/771/EC and updates).

Proposal

By considering the WRC-03 results (AI 1.5) and the ECC as well as the EC Decisions it is obvious that the WAS/RLAN systems have to be considered as radio applications based on a radio service (in the legal sense of Article 1 of the ITU Radio Regulations). By also considering the approach as recently proposed by CEPT at WP 1B (ITU-R) level, which led to the removal of the 5 GHz WAS/RLAN bands from the draft new ITU-R Recommendation on “Frequency ranges for global or regional harmonization of short-range devices (SRDs)”, it emphasises the view that the WAS/RLAN systems at 5 GHz do not have a SRD status.

Therefore Germany proposes to remove the bands 5150-5350 MHz and 5470-5725 MHz from Annex 3 of ERC Recommendation 70-03. SRD/MG is kindly invited to introduce this proposal to WG FM.

There is no impact on the RLAN band at 2.4 GHz which have a different regulatory status, therefore and to avoid any ambiguity, the band 2400-2483.5 MHz should be kept in Annex 3 of the Recommendation.

ECC policy is that in general all documents should be publicly available unless the author of the document requires that it be restricted to ECC family participants only