FM(11)158 Annex 3

FM50(11)041rev1 Annex 5

Draft introduction and section 1 of the ECC Report

Introduction

The L-band (1452-1492 MHz) is currently allocated for use by terrestrial and satellite digital audio broadcasting (DAB) services in most European countries

- Terrestrial segment: 1452–1479.5MHz (27.5MHz)

- Satellite segment: 1479.5–1492MHz (12.5MHz)

The use of the terrestrial segment of the L-band is governed by the Maastricht Special Arrangement, 2002, as revised in Constanţa in 2007. The Constanţa revision of the Maastricht agreement opens the band (originally identified for T-DAB) for mobile multimedia systems and provides a significant flexibility to administrations in terms of technology use in this band at a national basis. The satellite segment of the L-Band has been designated for use by Satellite Digital Audio Broadcasting Systems by ECC Decision (03)02.

Although the revision of the Maastricht agreement in Constanţa in 2007 extended the utilization of the band to mobile multimedia systems and allowed a more flexible utilization, and there are some countries that have provided authorisations for terrestrial sound broadcast in the L band there are licenses given in some countries, the L-Band remains to this day, unused in most European countries as the appropriate usage had not emerged yet.

Despite the current situation, it should be noted that there is political interest within CEPT and a potential for having a harmonised solution for the use on this band across Europe. Consequently, the objective of the study presented in this ECC Report is to identify which future use(s) of the 1452-1492 MHz band would be the most appropriate for CEPT.

The following steps are considered in the study:

- Identification of objective and relevant criteria to assess the impact(s) of an implementation;

- Assessment of the relevant candidate applications according to the criteria;

- Identification whether further regulatory measures may be required to accommodate candidate applications, including the development of the least restrictive technical conditions for the application(s) retained, if appropriate.

[to be further developed as appropriate]

1. Description of the context of the study

1.1 Regulation within CEPT

The ITU Radio Regulations allocate the band 1452 – 1492 MHz to the Fixed, Mobile, Broadcasting and Broadcasting Satellite Service on a co-primary basis in all Regions with the exception of the mobile aeronautical service in Region 1. Several footnotes apply to the band. The extract from Article 5 of the Radio Regulations is given in Annex I.

The use of the band 1452 - 1492 MHz by the broadcasting satellite service, and by the broadcasting service, is limited to digital audio broadcasting. The band is subject to the provisions of Resolution 528 (WARC-92).

In some European countries, the use of the 1452 - 1492 MHz band is subject to footnote No. 5.342 related to the use by aeronautical mobile service on a primary basis exclusively for the purposes of aeronautical telemetry within the national territory.

In CEPT, the Maastricht Special Arrangement, 2002, as revised in Constanţa in 2007 MA02revCO07 Special Arrangement provides the technical and regulatory framework and the associated frequency plan for the introduction of terrestrial digital audio broadcasting (T-DAB) services in the frequency band 1452 - 1479.5 MHz. The MA02revCO07agreement as revised in Constanta in 2007 provides a framework for the introduction of mobile multimedia services. In particular, it allows the implementation of T-DAB networks for portable indoor reception, the implementation of mobile multimedia services through the envelop concept and the aggregation of blocks. These concepts were described in CEPT Report 18 in response to the Mandate on EU harmonisation of the band 1452-1479.5 MHz (lower part of L-band) to allow flexible use by mobile multimedia technologies.  Flexible spectrum usage may be subject to international coordination according to the provisions of the Special Arrangement [ref] (further details are provided in Annex II).

The ECC Decision ECC/DEC/(03)02 designates the frequency band 1479.5 - 1492 MHz for use by satellite DAB systems. This Decision shall be reviewed inter alia on the basis of market demand at least every 10 years according to Decides 2 of ECC Decision (03)02. However, according to the ECC Rules of Procedure (art. 12.4.1), each Decision shall be reviewed on a regular basis, but not later than every five years.

1.2 Description of the current use within CEPT

    - Broadcasting T-DAB and S-DAB

In most European countries, the 1452-1492 MHz band is currently identified for the use by terrestrial and satellite digital audio broadcasting (T-DAB and S-DAB) applications.

Relating to terrestrial T-DAB, most European countries have got one to three nation-wide T-DAB coverages in the MA02revCO07 Plan. One administration informed that it has granted a licence for T-DAB in January 2011 (see Annex III) and that a network is operated. One other administration informed that a licence was granted for a T-DAB network, however to date, it is still awaiting for network roll out (see Annex III).

Relating to S-DAB, a review undertaken in July 2011 of the ITU-R database of satellite filings covering Europe  showed that 7 CEPT administrations have 53 active requests for coordination which include the band 1479.5-1492 MHz over the arc 15°W to 55°E.Three of these have been notified. To date, none has resulted in the submission of Resolution 49 information.

None of the T-DAB and S-DAB usages have actually grown in the band to date as it has been shown by the Radio Spectrum Policy Group survey and Report on the future of digital audio broadcasting.

1.2.2	Other uses

There are some other systems in operation in few countries. In nine countries, the band is currently used by applications operated in services different from audio broadcasting: Fixed Service (6 countries), Mobile Service (3 countries), Ground and Wall probing Radar (1 country), Radio Astronomy (1 country), Aeronautical Mobile Service (aeronautical telemetry) (1 country). These uses are generally limited to sub-parts only of the 1452-1492 MHz band. Some of the countries indicated in their response to the ECC survey carried out in the late 2010 – early 2011 on the future use of the 1452-1492 MHz band that those mentioned different uses will continue until an already fixed date (1 country) or once digital radio will be implemented (2 countries) or until the band is used by multimedia services (1 country).  In one country, the whole 1452-1492 MHz band is licensed for terrestrial applications under technology neutral technical conditions of usage.

1.3 Current regulations and use outside CEPT

Many countries globally have aligned their plans on Europe by allocating the L-band to DAB services. There are currently limited commercial terrestrial DAB services. There are 2 satellites in operation on the L-band. A number of countries have been investigating the future use of the L-band.

Annex IV provides an overview of the use/regulations of the band 1452 – 1492 MHz in a number of countries outside from CEPT.

ANNEX I

Extract of the Radio Regulations Article 5 for 1452-1492 MHz

| Allocation to services |  |  |  |
| --- | --- | --- | --- |
| Region 1 | Region 2 |  | Region 3 |
| 1 452-1 492 FIXED MOBILE except aeronautical mobile BROADCASTING 5.345 BROADCASTING-SATELLITE 5.208B 5.345 5.341 5.342 |  | 1 452-1 492 FIXED MOBILE 5.343 BROADCASTING 5.345 BROADCASTING-SATELLITE 5.208B 5.345 5.341 5.344 |  |

5.208B*	In the bands:

137-138 MHz,
	387-390 MHz,
	400.15-401 MHz,
	1 452-1 492 MHz,
	1 525-1 610 MHz,
	1 613.8-1 626.5 MHz,
	2 655-2 690 MHz,
	21.4-22 GHz,

Resolution 739 (Rev.WRC-07) applies.     (WRC-07)

5.341	In the bands 1 400-1 727 MHz, 101-120 GHz and 197-220 GHz, passive research is being conducted by some countries in a programme for the search for intentional emissions of extraterrestrial origin.

5.342	Additional allocation:  in Armenia, Azerbaijan, Belarus, Bulgaria, the Russian Federation, Uzbekistan, Kyrgystan and Ukraine, the band 1 429-1 535 MHz is also allocated to the aeronautical mobile service on a primary basis exclusively for the purposes of aeronautical telemetry within the national territory. As of 1 April 2007, the use of the band 1 452-1 492 MHz is subject to agreement between the administrations concerned. (WRC-2000)

5.343	In Region 2, the use of the band 1 435-1 535 MHz by the aeronautical mobile service for telemetry has priority over other uses by the mobile service.

5.344	Alternative allocation:  in the United States, the band 1 452-1 525 MHz is allocated to the fixed and mobile services on a primary basis (see also No. 5.343).

5.345	Use of the band 1 452-1 492 MHz by the broadcasting-satellite service, and by the broadcasting service, is limited to digital audio broadcasting and is subject to the provisions of Resolution 528 (WARC-92) *.

Annex II: Flexibility in MA02revCO07 Special Arrangement (1452 - 1479.5 MHz)

Introduction

MA02 Special Arrangement was reviewed in 2007 in order to allow for more flexibility and to allow the deployment of terrestrial mobile multimedia in the framework of the revised Arrangement MA02revCO07.

Flexibility built-in in the MA02revCO07 Special Arrangement enables the national administrations to take the approach that best suits their national requirements. The range of possibilities includes:

- T-DAB or enhanced radio services in accordance with the Ma02revCO07 Plan

- introduction of other terrestrial mobile multimedia systems that are compatible with the Plan by application of the envelope concept.

- introduction of terrestrial mobile multimedia systems that require larger bandwidth than 1.7 MHz (e.g. 5 MHz) by aggregating T-DAB frequency blocks.

- any combination of the above

Envelop concept

The envelop concept and the possibility to use T-DAB Plan entries for terrestrial mobile multimedia was also introduced in the Arrangement through Article 2.4:

“

2.4	The T-DAB Plan entries may be used for terrestrial mobile multimedia services with characteristics that may be different from those appearing in the Plan but within the envelope of their T-DAB Plan entry or aggregate entries under the provisions of the Special Arrangement, and that their administrations agree that any such use will be afforded protection to the levels defined by the interfering field strengths as arising from their Plan entries, taking into account any relevant bilateral agreements.

”

Aggregation of T-DAB blocks

Beyond the envelop concept, the possibility to aggregate T-DAB blocks is reflected in a number of sections of the Arrangement which mentions the aggregated case and in particular in Annex 2 – section 5.3 which provides spectrum mask for two aggregated T-DAB frequency blocks.

Figure XX: Aggregation of 2 T-DAB blocks

The Arrangement also allows for the aggregation of any number of blocks.  In order to introduce terrestrial mobile multimedia services according to paragraph 2.4 of the Special Arrangement, the following provisions apply:

“

6.1.2 Services brought into use under the terms of paragraph 2.4 shall not cause more interference nor claim more protection than the relevant T-DAB allotments in the Plan. Systems with a bandwidth greater than one single T-DAB frequency block may be brought into operation by aggregating contiguous T-DAB frequency blocks which appear in the Plan under the conditions given in Annex 2.

“

Annex 2, Section 2.3.3:

“

If a system is to be brought into operation in accordance with paragraph 6.1.2 the radiated signal between the edges shall not exceed the limit as defined by the blue line in Figure 3 (example of two aggregated T-DAB frequency blocks). Case 1 or Case 2 masks may be applied independently at either band edge of aggregated T-DAB frequency blocks.

”

Channel Raster and Bandwidth

The associated frequency plan to the MA02revCO07 Arrangement uses 16 x 1.7 MHz T-DAB blocks. The Arrangement also includes regulatory provisions to add flexibility to the plan to allow a different use of a T-DAB plan entry and the possibility of operation of systems requiring a larger bandwidth. The following table provides the list of T-DAB blocks included in the Arrangement.

| T-DAB block number | Centre frequency (MHz) | Frequency range (MHz) |
| --- | --- | --- |
| LA | 1452.960 | 1452.192 - 1453.728 |
| LB | 1454.672 | 1453.904 - 1455.440 |
| LC | 1456.384 | 1455.616 - 1457.152 |
| LD | 1458.096 | 1457.328 - 1458.864 |
| LE | 1459.808 | 1459.040 - 1460.576 |
| LF | 1461.520 | 1460.752 - 1462.288 |
| LG | 1463.232 | 1462.464 - 1464.000 |
| LH | 1464.944 | 1464.176 - 1465.712 |
| LI | 1466.656 | 1465.888 - 1467.424 |
| LJ | 1468.368 | 1467.600 - 1469.136 |
| LK | 1470.080 | 1469.312 - 1470.848 |
| LL | 1471.792 | 1471.024 - 1472.560 |
| LM | 1473.504 | 1472.736 - 1474.272 |
| LN | 1475.216 | 1474.448 - 1475.984 |
| LO | 1476.928 | 1476.160 - 1477.696 |
| LP | 1478.640 | 1477.872 - 1479.408 |

Table AYY: List of T-DAB blocks in MA02revCO07 Special Arrangement

Figure XY below shows how 3 x 1.7 MHz blocks could be aggregated into a 5.1 MHz block allowing for 5 MHz based technology to be deployed.

5 MHz

Figure XY: Example of aggregation of 3 T-DAB blocks aggregated into a 5.1MHz block (from [CEPT18])

The following figure indicates an example on how the 1.7 MHz blocks could be aggregated into 5.1 MHz blocks allowing the deployment of 5 MHz based technology. The centre frequencies of the new 5 MHz blocks are located at the centre frequencies of the existing blocks LB, LE, LH, LK and LN.

Figure XZ: Frequency raster for block aggregation in the band 1452 – 1479.5 MHz

Systems with a bandwidth greater than one single T-DAB frequency block may be brought into operation by aggregating contiguous T-DAB frequency blocks which appear in the Plan under the conditions given in Annex 2 to the Special Arrangement. The Special Arrangement also allows for the aggregation of contiguous blocks operated toward the same area.

Existence of contiguous frequency blocks over a given area in the MA02 Plan

Administrations may choose to aggregate over a given area contiguous blocks in order to allow for the deployment of systems with a bandwidth greater than one single T-DAB frequency block in this area. The following table provides an overview of the number of Allotments per country (August 2011).

| Country |  | Allotment in Part 1 | Allotment in Part 2 | Allotment in Part 2sup | Total |
| --- | --- | --- | --- | --- | --- |
| Top of Form Albania Bottom of Form |  | 0 | 2 | 0 | 2 |
| Top of Form Austria Bottom of Form |  | 11 | 43 | 0 | 54 |
| Top of Form Belgium Bottom of Form |  | 6 | 11 | 0 | 17 |
| Top of Form Bosnia & Herzegovina Bottom of Form |  | 0 | 2 | 0 | 2 |
| Top of Form Bulgaria Bottom of Form |  | 8 | 14 | 0 | 22 |
| Top of Form Croatia Bottom of Form |  | 11 | 22 | 0 | 33 |
| Top of Form Czech Republic Bottom of Form |  | 1 | 26 | 1 | 28 |
| Top of Form Denmark Bottom of Form |  | 0 | 41 | 0 | 41 |
| Top of Form Estonia Bottom of Form |  | 4 | 4 | 0 | 8 |
| Top of Form Finland Bottom of Form |  | 0 | 21 | 0 | 21 |
| Top of Form France Bottom of Form | 134 | 67 | 0 | 201 |
| Top of Form Germany Bottom of Form | 108 | 112 | 0 | 220 |
| Top of Form Greece Bottom of Form | 4 | 60 | 0 | 64 |
| Top of Form Hungary Bottom of Form | 8 | 22 | 0 | 30 |
| Top of Form Ireland Bottom of Form | 0 | 31 | 0 | 31 |
| Top of Form Italy Bottom of Form | 21 | 193 | 0 | 214 |
| Top of Form Latvia Bottom of Form | 4 | 7 | 0 | 11 |
| Top of Form Liechtenstein Bottom of Form | 2 | 1 | 0 | 3 |
| Top of Form Lithuania Bottom of Form | 5 | 40 | 0 | 45 |
| Top of Form Luxemburg Bottom of Form | 1 | 1 | 0 | 2 |
| Top of Form FYR of Macedonia Bottom of Form | 0 | 2 | 0 | 2 |
| Top of Form Malta Bottom of Form | 0 | 1 | 0 | 1 |
| Top of Form Moldova Bottom of Form | 0 | 26 | 0 | 26 |
| Top of Form Monaco Bottom of Form | 2 | 0 | 0 | 2 |
| Top of Form The Netherlands Bottom of Form | 1 | 141 | 0 | 142 |
| Top of Form Norway Bottom of Form | 1 | 83 | 0 | 84 |
| Top of Form Poland Bottom of Form | 49 | 16 | 0 | 65 |
| Top of Form Portugal Bottom of Form | 14 | 60 | 0 | 74 |
| Top of Form Romania Bottom of Form | 11 | 48 | 0 | 59 |
| Top of Form San Marino Bottom of Form | 1 | 1 | 0 | 2 |
| Top of Form Slovak Republic Bottom of Form | 1 | 16 | 12 | 29 |
| Top of Form Slovenia Bottom of Form | 8 | 9 | 0 | 17 |
| Top of Form Spain Bottom of Form | 19 | 342 | 0 | 361 |
| Top of Form Sweden Bottom of Form | 0 | 93 | 0 | 93 |
| Top of Form Switzerland Bottom of Form | 33 | 17 | 0 | 50 |
| Top of Form Turkey Bottom of Form | 6 | 5 | 0 | 11 |
| Top of Form Ukraine Bottom of Form | 25 | 58 | 0 | 83 |
| Top of Form United Kingdom Bottom of Form | 1 | 276 | 0 | 277 |

Table AYY: Overview of the allotments in the Plan (August 2011)

Administrations that would choose this aggregation method would need to introduce new entries in the Plan as there are very few locations with three frequency adjacent blocks in a given area in the Plan as of August 2011.

Contiguous T-DAB blocks in the same area exist to a very limited extent in the MA02 Plan (July 2007). Analysis of the MA02 Plan shows that there are very few geographical areas in which two or three contiguous T-DAB blocks have been allotted in the Plan. In order to properly judge the result it should be noted that there are only a few countries which possess three layers of Plan entries.

The Plan established in 1995 at Wiesbaden (WI95) provided two T-DAB layers on the basis of national requirements either in the 1.5 GHz band (see figures XY.1 and XY.2) and/or VHF. At Maastricht 2002 one additional T-DAB layer was planned in the 1452 – 1479.5 MHz band (see figure XY.3). Furthermore, a new Special Arrangement was established, namely the MA02 Special Arrangement.

Following the two planning conferences WI95 and MA02 a few countries have three T-DAB layers in the 1.5 GHz band and no T-DAB in VHF, most countries have two T-DAB layers in the 1.5 GHz band and one VHF layer, while some only have one T-DAB layer in the 1.5 GHz band and two VHF layers.

Figure XY.1 : T-DAB layer 1 in the 1.5 GHz band originating from WI95

Figure XY.2 : T-DAB layer 2 in the 1.5 GHz band originating from WI95

Figure XY.3 : T-DAB layer 3 in the 1.5 GHz band originating from MA02

Potential addition of new contiguous blocks in a given area

The MA02 Special Arrangement contains procedures for the addition or modification of an allotment (Annex 4, section 3 of the MA02 Special Arrangement) which enable addition of new contiguous blocks in a given area. Co-ordination is necessary if a new allotment would:

- cause field strengths greater than or equal to 38 dB(μV/m) at the boundary of any other administration; or,

- with regard to any other services, cause field strengths greater than or equal to the maximum allowable interfering field strengths at the boundary of any other administration.

No coordination is required where the above limits are not exceeded. In practice, this corresponds to those parts of a country which are separated from the boundary of each neighbouring administration by a certain distance. This distance depends on:

- the reference network (see section 5 – Annex 2 - of the Special Arrangement) associated with a particular allotment;

- the type of service to be provided;

- the existence of other services in the neighbouring administration and

- the type of propagation path (Land, Cold Sea, Warm Sea or mixed) between the boundary of the new allotment and the boundary of each neighbouring administration.

|  |  | Reference network 1 (L-RN1) receiving interference | Reference network 2 (L-RN2) receiving interference | Reference network 3 (L-RN3) receiving interference |
| --- | --- | --- | --- | --- |
| Maximum permissible co-block interfering field strength (dBV/m) |  | 41 dBV/m | 43 dBV/m | 45 dBV/m |
| Reference network 1 (L-RN1) as interferer | Land | 50 km | 44 km | 38 km |
|  | Cold sea | 314 km | 290 km | 269 km |
|  | Warm sea | 434 km | 392 km | 350 km |
| Reference network 2 (L-RN2) as interferer | Land | 39 km | 34 km | 30 km |
|  | Cold sea | 341 km | 318 km | 297 km |
|  | Warm sea | 466 km | 426 km | 386 km |
| Reference network 3 (L-RN3) as interferer | Land | 23 km | 20 km | 17 km |
|  | Cold sea | 292 km | 272 km | 255 km |
|  | Warm sea | 388 km | 348 km | 313 km |

Table XX: Typical reference network separation distances used when developing the Plan (based on Recommendation ITU-R P.370-7) for compatibility between T-DAB allotments operating on the same block

Request for Coordination received by the ECO

The follow up of the request of coordination is available at http://cept.org/ecc/topics/broadcasting/t-dab/ma02revco07-special-arrangement/ma02revco07-t-dab-plan-circular-letters, where the Circular Letters published by the Office are available.

In total, seven Circular Letters have been published by the Office since May 2007. Among them, only two contained request for new allotments.

In Circular Letter 013, the Slovak Republic requested the coordination of several new allotments. Among them, it can be seen that 8 sets of them were for 3 contiguous channels operated over the same area. 5 sets of 3 contiguous channels operated over the same area are contained in Part2sup.

In Circular Letter 018, the Czech Republic requested the coordination of a new allotment. However, a brief analysis shows that this will not result in a set of 3 contiguous channels operated over the same area.

ANNEX III: L-band utilisation in some CEPT countries (updated information compared to the ECC survey conducted in December 2010 – January 2011)

CZE

Administration of the Czech Republic has issued, on public tender basis, a number of licences for terrestrial digital broadcasting services based on Eureka 147 technology in the L-Band. Nine licences for particular regions and thirteen metropolitan licences will expire in 2021.  Four T-DAB transmitters are already in regular operation.

NL

The Netherlands issued one license within the 1452-1479,5 MHz band. This license was awarded based on rights out of the MA02revCO07 Special Arrangement:

- 117 area’s, containing 141 allotments, forming 1 nationwide layer;

- The license is technology neutral, transmissions should fit within the T-DAB spectrum mask;

- The license is granted on 16-02-2009 and will end on 15-02-2024;

- There is a roll-out obligation for 16 area’s by 16-02-2012 and 94 area’s by 16-02-2015;

- Not fulfilling the roll-out obligation can be a reason to withdraw the license. No roll out was reported up to now.

According to the national frequency plan of The Netherlands, the band 1452-1479,5 MHz has to be used for digital broadcasting.

UK

The whole 1452-1492 MHz band is licensed until 2023 (renewable) for terrestrial applications under technology neutral technical conditions of usage.

ANNEX IV:

Overview of the use/regulations of the band 1452 – 1492 MHz in a number of countries outside from CEPT

| Country | Primary Services | Secondary Services | Note |
| --- | --- | --- | --- |
| Australia | Fixed, Mobile, Broadcasting, Broadcasting Satellite |  | http://www.acma.gov.au/WEB/STANDARD/pc=PC_2713 |
| Brazil | Fixed, Broadcasting, Broadcasting Satellite |  | http://www.cft.gob.mx/wb/Cofetel_2008/Cofe_cnaf_4 |
| Canada | Broadcast (T and S) Fixed | Mobile | http://www.ic.gc.ca/eic/site/smt-gst.nsf/vwapj/spectallocation-08.pdf/$FILE/spectallocation-08.pdf Industry Canada is consulting on the adoption of ‘a spectrum utilization policy allowing for flexible use of the spectrum to support a variety of services and technologies for subscription broadcasting, multimedia, fixed and mobile broadband applications.’ http://www.ic.gc.ca/eic/site/smt-gst.nsf/eng/sf09751.html#band1452a |
| China | Fixed, Broadcasting, Broadcasting Satellite |  | http://files.radioscanner.ru/bands/files/document76/castoti_kitaa_2005.pdf |
| Hong Kong | Fixed |  | http://www.ofta.gov.hk/en/freq-spec/freq-allocations.pdf |
| India | Fixed, Mobile, Broadcasting, Broadcasting Satellite |  | http://www.wpc.dot.gov.in/DocFiles/3-NATIONAL%20FREQUENCY%20TABLE.DOC |
| Japan | 1452 – 1453 MHz: Fixed and Mobile 1453 – 1475.9 MHz: Mobile 1475.9 – 1492 MHz: Fixed and Mobile |  | http://www.tele.soumu.go.jp/e/adm/freq/search/share/plan.htm |
| Kenya | Broadcasting, Broadcasting Satellite |  | http://www.cck.go.ke/licensing/downloads/Kenya_TOFA_2008_Edition.pdf |
| Pakistan | Fixed, Mobile, Broadcasting, Broadcasting Satellite |  | http://www.pta.gov.pk/media/Pakistan_Table_of_Frequency_Allocations.pdf |
| South Korea | Fixed and Mobile |  | http://rapa.or.kr/frequency/english/3_Korean_Allocations_Tables.pdf |
| United States | Mobile (aeronautical telemetry) - aviation |  | http://www.ntia.doc.gov/osmhome/allochrt.html |

Additional information

- In Canada, Industry Canada’s ‘Consultation on the Spectrum Allocations and Spectrum Utilization Policies for the Frequency Range 1435-1525 MHz (L-Band)’ (http://www.ic.gc.ca/eic/site/smt-gst.nsf/eng/sf09751.html) considered that “with the convergence of fixed, broadcasting and mobile services over digital wireless platforms, a regulatory approach promoting flexible use of spectrum is increasingly important […].  Several countries that initially considered the band 1452-1492 MHz for DAB services have recently begun to re-farm the spectrum for a range of broadcasting and multimedia applications within the international regulations and allocations […]. The prospect of a wide range of services in the band 1452-1492 MHz being allowed by industrialized countries in Europe and elsewhere further underlines the risk for Canada of pursuing only the conventional DAB service in this band. Consequently, the current allotment plan developed for the DAB implementation may not be appropriate given the new technologies that could be deployed in this band. Broadband wireless technologies, which support multimedia applications, are developed based on a 5 MHz channel width”. The department proposed to “adopt a spectrum utilization policy allowing for flexible use of the spectrum to support a variety of services and technologies for subscription broadcasting, multimedia, fixed and mobile broadband applications.” […] to streamline the Canadian frequency allocations in the band 1452-1492 MHz and to give full flexibility and priority to terrestrial services [… and] to elevate the status of mobile service to co-primary with broadcasting and fixed services in the band 1452-1492 MHz”.

- In Africa, CRASA, which groups together the 14 Southern African Development Community (SADC) countries, highlighted in its latest Frequency Allocation Plan (FAP) (www.crasa.org/download.php?doc=doc_pub_eng64.pdf), its framework for the harmonisation across SADC on the use of the radio frequency spectrum,  that: “T-DAB in the 1452-1492 MHz to be reconsidered: whereas this band was used for testing of T-DAB it was felt by the majority that this allocation is no longer required. The use of this band in the future should be further investigated and clarified.”

Note: this CEPT Report also contain considerations on Partial Revision of the MA02 Special Arrangement (section 4.2) and on Substantial Revision of the MA02 Special Arrangement (section 4.3) which may need to be considered by FM45.

http://rspg.groups.eu.int/consultations/consultation_futradio/rspg10_349_annex.pdf

See FM(11)038 Rev2.

* This provision was previously numbered as No. 5.347A. It was renumbered to preserve the sequential order.

* 	Note by the Secretariat:  This Resolution was revised by WRC-03.

Further information can be found in the CEPT Report 18.

Part 1 contains T-DAB allotments transferred from the original WI95 Plan in the 1.5 GHz band.

Part 2 contains the allotments originating at the MA02 planning conference.

Part2sup contains the allotments for which the coordination was completed after the publication of the revision of MA02 arrangement in 2007.

Some of those allotments are disputed by Hungary. The Slovak Republic also has 13 allotments still in the coordination process.