FM(11)202 Annex 16

Candidate bands for Broadband DA2GC

(Revised during the 73rd meeting of WG FM, October 2011)

Category 1: Frequency bands already submitted to WG SE for compatibility/sharing studies

| Frequency band | Existing allocation in the RR suitable for Broadband DA2GC (MS or AMS) ? | Current level of harmonisation ? | Would the bandwidth required (2 x 10 MHz FDD or 20 MHz TDD) fit in the band ? | Proposed or future alternative usage under consideration (within CEPT) ? | Actual usage of the band - national level - within CEPT (- worldwide) for future sharing studies ? | Actual usage of adjacent bands - national level - within CEPT for future compatibility studies ? |  | Likely time scales (target date 2012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2400 – 2483.5 MHz | yes | high | yes | --- | RLANs, SRDs/RFIDs, ISM | Aeronautical Telemetry, Amateur, Mobile applications, SAP/SAB (cordless cameras), MSS, ISM |  | possibly within |
| 3400 – 3600 MHz | yes | high | yes | band plan under consideration within ECC PT 1 | BWA, FSS, FS links, RLS | RLS, FSS, BWA, FS links |  | possibly within |
| 5855 – 5875 MHz | yes | medium | yes (air to ground) | AI 1.20 WRC-12 | BFWA, ITS, SRDs, FSS (uplink), military systems (on a national level), ISM | RLS, RTTT, BFWA, SRDs, ITS |  | within |

Category 2: Frequency bands to be considered next or in case the studies for the category 1 bands will not show positive results

| Frequency band | Existing allocation in the RR suitable for Broadband DA2GC (MS or AMS) ? | Current level of harmonisation ? | Would the bandwidth required (2 x 10 MHz FDD or 20 MHz TDD) fit in the band ? | Proposed or future alternative usage under consideration (within CEPT) ? | Actual usage of the band - national level - within CEPT (- worldwide) for future sharing studies ? | Actual usage of adjacent bands - national level - within CEPT for future compatibility studies ? |  | Likely time scales (target date 2012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2483.5 – 2500 MHz | yes | high | yes (ground to air) | AI 1.18 WRC-12 | MSS, ISM | RLANs, SRDs/RFIDs, ISM, ECS |  | possibly within |
| 5150 – 5170 MHz | yes (1) (see also 5.446C) | high | yes |  | FSS (feeder links), RLANs, BBDR, AMS (Flight Test Telemetry) | MLS, AM(R)S, RLANs, FSS (feeder links) (2) |  | within |

(1) The current AMS allocation would need to be modified to allow Broadband DA2GC

(2) Compatibility studies with MLS, AM(R)S, RLANs and FSS have been completed

Remark:
The band 5150-5170 MHz was proposed for deletion by FM PT 48, WG FM decided to keep this band for the time being and for further consideration by FM PT 48.

Category 3: Position on frequency bands still open/undecided

| Frequency band | Existing allocation in the RR suitable for Broadband DA2GC (MS or AMS) ? | Current level of harmonisation ? | Would the bandwidth required (2 x 10 MHz FDD or 20 MHz TDD) fit in the band ? | Proposed or future alternative usage under consideration (within CEPT) ? | Actual usage of the band - national level - within CEPT (- worldwide) for future sharing studies ? | Actual usage of adjacent bands - national level - within CEPT for future compatibility studies ? |  | Likely time scales (target date 2012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1452 – 1492 MHz (under consideration within FM PT 50) |  |  |  |  |  |  |  |  |
| 1900 – 1920 MHz / 2010 – 2025 MHz | yes, yes | high, high | yes | ECS | UMTS (TDD) | DECT, UMTS (FDD), EESS, SRS, FS, SAP/SAB |  | within |
| 3600 – 3800 MHz | yes | high | yes | band plan under consideration within ECC PT 1 | BWA, FSS, FS links | BWA, FSS, FS links |  | possibly within |
| 5905 – 5925 MHz | yes | high | yes (air to ground) | AI 1.20 WRC-12 | FSS (uplink), military systems (on a national level) | FS links |  | within |

Category 4: Frequency bands with high regulatory obstacles for an introduction of Broadband DA2GC systems

| Frequency band | Existing allocation in the RR suitable for Broadband DA2GC (MS or AMS) ? | Current level of harmonisation ? | Would the bandwidth required (2 x 10 MHz FDD or 20 MHz TDD) fit in the band ? | Proposed or future alternative usage under consideration (within CEPT) ? | Actual usage of the band - national level - within CEPT (- worldwide) for future sharing studies ? | Actual usage of adjacent bands - national level - within CEPT for future compatibility studies ? |  | Likely time scales (target date 2012) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2300 – 2400 MHz | yes | low | yes |  | Aeronautical Telemetry, Amateur, Mobile applications, SAP/SAB (cordless cameras) | FS, MS, SRS |  | within ??? |

List of abbreviations used in the tables

| Abbreviation | Explanation |
| --- | --- |
| AMS | Aeronautical Mobile Service |
| BFWA | Broadband Fixed Wireless Access |
| BWA | Broadband Wireless Access |
| DA2GC | Direct-Air-to-Ground Communications |
| DECT | Digital Enhanced Cordless Telecommunications |
| ECS | Electronic Communications Services |
| EESS | Earth Exploration-Satellite Service |
| FDD | Frequency Division Duplex |
| FS | Fixed Service |
| FSS | Fixed-Satellite Service |
| ISM | Industrial, scientific and medical applications |
| ITS | Intelligent Transport Systems |
| MS | Mobile Service |
| MSS | Mobile Satellite-Service |
| RFIDs | Radio frequency identification devices |
| RLANs | Radio Local Area Networks |
| RLS | Radiolocation Service |
| RTTT | Road Transport and Traffic Telematics |
| SAP/SAB | Services Ancillary to Programme making / Services Ancillary to Broadcasting |
| SRDs | Short Range Devices |
| SRS | Space Research Service |
| TDD | Time Division Duplex |
| TRA-ECS | Terrestrial radio applications capable of providing Electronic Communications Services |
| UMTS | Universal Mobile Telecommunications System |
| WAPECS | Wireless Access Policy for Electronic Communications Services |