FM(12)084 Annex 08

Elements on the draft ESOMP Decision:

FM44 should take into account the following elements:

- With regard to the notification issue (Decides 2 of the current draft ECC Decision):

  - A large majority of administrations that expressed their position in WGFM were not in favour of maintaining such mandatory notification;

  - Another solution should be considered:

    - Make this notification from the operator/service provider non-mandatory (voluntary basis);

    - And use EFIS to publish the information (ECO would receive the information and enter it in EFIS) so that it will be available for other administrations;

- With regard to the uplink band issue:

  - In October 2011, WGFM noted without comment the outline of the ECC Report on ESOMP which clearly mentions (6.1.1) that “ESOMPs are to be treated as typical uncoordinated FSS earth stations and are meant to operate only in bands available to uncoordinated FSS earth stations”;

  - There are no issues with the designation of the band 29.5-30 GHz for ESOMPs as this band is exclusively available to FSS;

  - The following applies with respect to the band 27.5-29.5 GHz: As the ECC/DEC/(05)01 recognises that the band 28.8365-28.9485 GHz is used by FS in some countries and as in international waters/airspace ESOMP may operate in bands not limited to the uncoordinated FSS Earth stations (as defined in ECC/DEC/(05)01), a solution should also be found for these cases (that can also be helpful for administrations that have not implemented the ECC/DEC/(05)01):

    - A first Decides should recognise clearly that the baseline for ESOMP is to transmit in the bands identified in the ECC/DEC/(05)01 for uncoordinated FSS Earth stations. In these bands, there would be no specific in-band restriction applicable to ESOMP (diverging from those of the usual uncoordinated FSS Earth stations);

    - Another Decides would deal with both the band 28.8365-28.9485 GHz and the case of ESOMP operated in international waters/airspace in other bands than those mentioned in 2.c.i above, e.g., by adopting a pfd threshold to protect in-band terrestrial services.