FM(12)084 Annex 19

ecC Decision of dd MONTH 2012 on The harmonised conditions for UWB applications inside aircraft (ECC/DEC/(12)0X)

“The European Conference of Postal and Telecommunications Administrations,

considering

- that every state has sovereignty over the airspace, including the radio spectrum, above its territory;

- that for the purposes of this Decision the aircraft is considered under the sovereignty of the country of aircraft registration;

- that accordingly responsibility for the authorisation of the frequency spectrum utilised onboard an aircraft will be that of the country of registration of the aircraft;

- that the use of the relevant frequencies will be authorised by one administration but those frequencies could also be used within the airspace of other countries which also implemented this ECC Decision;

- that the installation of UWB equipment onboard aircraft is subject to aviation regulation, including airworthiness certification, by the relevant aviation authorities and the equipment cannot be put into operation until it complies with these regulations;

- that UWB technology shall mean technology for short-range radiocommunication, involving the intentional generation and transmission of radio-frequency energy that spreads over a very large frequency range, which may overlap several frequency bands allocated to radiocommunication services;

- that this Decision is applicable to technologies with bandwidth significantly wider than 50 MHz;

- that Short Range Devices (SRD) using UWB technology may be used for wireless passenger communications and  entertainment non-safety wireless crew communications as well as non-safety wireless control and monitoring functions;

- that harmonised conditions across CEPT/EU help to establish an effective single market for these applications, with consequent economies of scale and benefits to the flying public, and avoid difficulties in enforcing divergent national regulations;

- that devices using UWB technology operate on a non-interference, non-protected basis;

- that the issue of compatibility of UWB devices with other radio equipment including aeronautical communication and navigation equipment operated on board an aircraft is the responsibility of the relevant aviation authorities;

- that a co-existence study considering UWB applications onboard aircraft and existing radio services in the frequency bands from 3.1 GHz to 4.8 GHz and from 6.0 GHz to 8.5 GHz has been conducted by the ECC and that the results of these studies are contained in ECC Report 175;

- that  in case the actual number of UWB devices installed or deployed onboard aircraft significantly exceeds assumptions used in ECC Report 175, then this regulation will have to be reviewed;

- that the Methanol line at 6.7 GHz has only recently been discovered and is becoming a focus of radio-astronomical research;

- In EU/EFTA countries the radio equipment that is under the scope of this Decision shall comply with the R&TTE Directive. Conformity with the essential requirements of the R&TTE Directive may be demonstrated by compliance with the applicable harmonised European standard(s) or by using the other conformity assessment procedures set forth in the R&TTE Directive.

DECIDES

- that this ECC Decision defines general harmonised conditions for the use of devices using UWB technology onboard aircraft operating in the frequency band from 6 GHz to 8.5 GHz;

- that, for the purpose of this Decision, the following definitions apply:

Maximum mean e.i.r.p. spectral density: the highest signal strength measured in any direction at any frequency within the defined range. The mean e.i.r.p. spectral density is measured with a 1 MHz resolution bandwidth, an RMS detector and an averaging time of 1 ms or less.

Maximum peak e.i.r.p.: the highest signal strength measured in any direction at any frequency within the defined range. The peak e.i.r.p. is defined within a 50 MHz bandwidth.

- that the devices permitted under this ECC Decision are exempt from individual licensing and operate on a non-interference, non-protected basis;

- that the following technical requirements apply to devices permitted under this ECC Decision:

Table 1: Maximum e.i.r.p. limits

| - Frequency range | - Maximum mean e.i.r.p. spectral density | - Maximum peak e.i.r.p. (defined in 50 MHz) | Requirements for mitigation techniques |
| --- | --- | --- | --- |
| Below 1.6 GHz | -90 dBm/MHz | -50 dBm |  |
| 1.6 to 2.7 GHz | -85 dBm/MHz | -45 dBm |  |
| 2.7 to 3.4 GHz | -70 dBm/MHz | -36 dBm |  |
| 3.4 to 3.8 GHz | -80 dBm/MHz | -40 dBm |  |
| 3.8 to 4.2 GHz | -70 dBm/MHz | -30 dBm |  |
| 4.2 to 4.8 GHz | -70 dBm/MHz | -30 dBm |  |
| 4.8 to 6 GHz | -70 dBm/MHz | -30 dBm |  |
| 6.0-6.650 GHz | -41.3 dBm/MHz | 0 dBm |  |
| 6.650 -6.6752 GHz | -62.3 dBm/MHz | -21 dBm | notch of 21 dB should be implemented to meet a level -62.3 dBm/MHz 1 |
| 6.6752-8.5 GHz | -41.3 dBm/MHz | 0 dBm | 7.25-7.75 GHz (FSS and MetSat (7.45-7.55 GHz) protection)1: -51.3 dBm/MHz (between altitudes of 10000 and 5000 m) -57.3 dBm/MHz (between altitudes of 5000 and 3000 m) 1 |
|  |  |  | 7,75-7,9 GHz (MetSat protection): -44.3 dBm/MHz (between altitudes of 10000 and 5000 m) -50.3 dBm/MHz (between altitudes of 5000 and 3000 m) |
| 8.5 to 10.6 GHz | -65 dBm/MHz | -25 dBm |  |
| Above 10.6 GHz | -85 dBm/MHz | -45 dBm |  |
| 1Alternative mitigation techniques offering equivalent protection such as the use of shielded portholes could be a solution. |  |  |  |

- that this Decision enters into force on [date];

- that the preferred date for implementation of this Decision shall be [date];

- that CEPT administrations shall communicate the national measures implementing this Decision to the ECC Chairman and the Office when the Decision is nationally implemented.”

Note:

Please check the Office documentation database http://www.ecodocdb.dk for the up to date position on the implementation of this and other ECC Decisions.