## FM(12)084 Annex 31

## Annex 11	Radio frequency identification applications

Scope of Annex

This annex covers frequency bands and regulatory as well as informative parameters recommended for radio frequency identification (RFID) applications including for example automatic article identification, asset tracking, alarm systems, waste management, personal identification, access control, proximity sensors, anti-theft systems, location systems, data transfer to handheld devices and wireless control systems. It should be noted that other types of RFID systems can be operated in accordance with other relevant annexes.

Regulatory parameters related to Annex 11

| Frequency Band | Power | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- |
| a1 2446-2454 MHz | ≤500 mW e.i.r.p. | No requirement | No spacing |  |  |
| a2 2446-2454 MHz | >500 mW-4 W e.i.r.p | ≤ 15% duty cycle FHSS techniques should be used | No spacing |  | Power levels above 500 mW are restricted to be used inside the boundaries of a building and the duty cycle of all transmissions shall in this case be ≤15 % in any 200 ms period (30 ms on /170 ms off). |
| b1 865.0-865.6 MHz | 100 mW e.r.p. | No requirement | 200 kHz |  |  |
| b2 865.6-867.6 MHz | 2 W e.r.p. | No requirement | 200 kHz |  |  |
| b3 867.6-868.0 MHz | 500 mW e.r.p. | No requirement | 200 kHz |  |  |

Additional Information

Harmonised Standards

EN 300 440 	Sub-band a1) and a2)

EN 300 761         Sub-band a1)

EN 302 208 	Sub-bands b1), b2) and b3)

Frequency issues

Sub-band a2)

To assist enforcement authorities any emissions from an RFID device when measured outside of the building at a distance of 10 metres shall not exceed the field strength from a 500 mW RFID device mounted outside the building when measured at the same distance. Where a building consists of a number of premises, such as shops within a shopping arcade or Mall then the measurements shall be referenced to the boundary of the user's premises within the building.

Sub-bands b1), b2) and b3)

Channel centre frequencies are 864.9 MHz + (0.2 MHz * channel number).

The available channel numbers for each sub-band are:

b1: channel numbers 1 to 3

b2: channel numbers 4 to 13

b3: channel numbers 14 to 15.

Note: The same equipment is allowed to operate in several sub-bands.

Frequency hopping or other spread spectrum techniques shall not be used.

Technical parameters also referred to in the harmonised standard

Sub-band a2)

In addition, antenna beamwidth limits shall be observed as described in the standard EN 300 440.

In addition, for an RFID device which can exceed 500 mW, the device should be fitted with an automatic power control to reduce the radiated power below 500 mW; this automatic power control shall guarantee the reduction of the power to a maximum of 500 mW in cases where the device is moved and used outside the boundary of the user's building or premises as described above.