Electronic Communications Committee (ECC)

within the European Conference of Postal and Telecommunications Administrations (CEPT)

COMPATIBILITY studies - MSS terminals transmitting to a satellite in the band 1980 – 2010 MHz and adjacent channel UMTS services

Location, Month, Year

# executive summary

Xxxxx.

The following conclusions can be drawn based on the analysis in this Report:

- Aaaa…..

- Bbbbb…..

- ...

Table of contents

0	executive summary	2

List of abbreviations	4

1	introduction	4

2	Frequency usage	4

2.1	2 GHz band	4

3	Sharing scenarios	4

4	methodology	4

4.1	UMTS system characteristics and parameters	4

4.1.1	FDD at 1980 MHz border	4

4.1.2	TDD at 2010 MHz border	4

4.2	MSS within the band 1980-2010 MHz, characteristics and system parameters	4

4.2.1	EN 302 574-2 (wideband)	4

4.2.2	EN 302 574-3 (narrowband)	4

4.2.3	Antenna and terminal types	4

4.2.4	Radiated Power assumptions for MSS type user terminals	4

4.2.5	Channel plans at the band edges	4

4.3	Propagation models	4

4.3.1	Deterministic case	4

4.3.2	Monte Carlo simulation	4

4.4	Deterministic analysis	4

4.4.1	Methodology	4

4.4.2	Assumptions	4

4.5	Monte-Carlo and SEAMCAT conditions	4

4.5.1	ECN relevant SEAMCAT receiver parameters	4

4.5.2	Methodology	4

5	calculation Analysis and results	4

5.1	Systems below 1980 MHz	4

5.1.1	Sub results from deterministic study	4

5.1.2	Sub results from Monte Carlo simulations	4

5.2	Systems above 2010 MHz	4

5.2.1	Sub results from deterministic study	4

5.2.2	Sub results from Monte Carlo simulations	4

5.3	Sensitivity analysis	4

5.3.1	Frequency planning between CGC and MSS uplink??	4

6	Conclusions	4

Annex 1: Specification for ETSI standards below 1980 MHz	4

Annex 2: Specification for ETSI standards above 2010 MHz	4

ANNEX 3: Specification for ETSI MSS ES standards between 1980 - 2010 MHz	4

ANNEX 4:SIMULATION PARAMETERS AND SCENARIOS	4

Annex 5: LIST OF REFERENCES	4

List of abbreviations

[Editors note: Update in the end]

| Abbreviation | Explanation |
| --- | --- |
| ACLR | Adjacent Channel Leakage Ratio |
| ACS | Adjacent Channel Selectivity |
| BS | Base Station |
| BTS | Base Transceiver Station |
| BW | Bandwidth |
| CEPT | European Conference of Postal and Telecommunications Administrations |
| C/I | Carrier to Interference ratio |
| DEC | Decision |
| DECT | Digital Enhanced Cordless Telecommunications |
| ECA | European Common Allocation |
| ECC | European Communications Committee |
| ERC | European Radiocommunications Committee |
| ERP | Effective Radiated Power |
| ETSI | European Telecommunications Standards Institute |
| MCL | Minimum Coupling Loss |
| MS | Mobile Station |
| SEAMCAT | Spectrum Engineering Advanced Monte-Carlo Analysis Tool |
| UMTS | Universal Mobile Telecommunications System |
| MSS UT |  |

# introduction

ECC Decision (06)09 designates the bands 1980 - 2010 MHz and 2170 - 2200 MHz to Mobile Satellite Services (MSS), which may incorporate Complementary Ground Component (CGC). It also states that the services will be compatible with adjacent mobile services. It is, therefore, important to determine the extent of any interference issues between MSS/CGC and adjacent band IMT services.

The frequency allocations in the band 1980 - 2010 MHz and adjacent bands are given in Table 1. The frequency allocations in the band 2170 - 2200 MHz and adjacent bands are given in Table 2. The relevant adjacent bands are 1900 - 1980 MHz, 2010 - 2025 MHz and 2110 - 2170 MHz.

ECC Decision (06)01 designates these bands to IMT2000/UMTS. The band 1900 - 1980 MHz is designated as an FDD uplink band (mobile to base). The band 2110 - 2170 MHz is designated as an FDD downlink band (base to mobile) and the band 2010 - 2025 MHz is designated as either TDD or FDD uplink.

ERC Report 65 (in 1999) contains comprehensive analyses of compatibility between UMTS and several other services in the 2GHz band. These other services include MSS in the bands 1980 – 2010 MHz and 2170 – 2200 MHz. However, the report did not cover the use of CGC base stations.

Although, not covered by Report 65, the CGC base station ETSI standards for out of band emissions are similar those of UMTS 3GPP base stations that already exist, using similar technology. Furthermore, we expect that they would use similar power levels and network deployment. Hence, the adjacency issues pertaining to these CGC base stations would essentially be identical to those that currently exist between different mobile network operators within the bands below 1980 MHz.

The analysis of adjacencies of 3GPP equipment and the use of FDD/TDD technology has been widely considered in CEPT Report 19 (for the band 2500-2690 MHz) and the issues are well explained and documented.

The difference between the adjacencies for MSS CGC base stations at 2170-2200 MHz and the adjacencies for IMT base stations (2500-2690 MHz) are small. They both use IMT equipment built to ETSI standards that have out of band emission masks based on the same 3GPP standards described in CEPT Report 19. CEPT Report 19 includes regulatory solutions based on the use of restricted blocks to mitigate against interference between base stations as may occur at the 2010 MHz boundary. Therefore, it is believed that these issues are already dealt with.

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 930-1 970 FIXED MOBILE 5.388A 5.388B | 1 930-1 970 FIXED MOBILE 5.388A 5.388B Mobile-satellite (Earth-to-space) | 1 930-1 970 FIXED MOBILE 5.388A 5.388B |
| 5.388 | 5.388 | 5.388 |
| 1 970-1 980 FIXED MOBILE 5.388A 5.388B 5.388 |  |  |
| 1 980-2 010 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.351A 5.388 5.389A 5.389B 5.389F |  |  |
| 2 010-2 025 FIXED MOBILE 5.388A 5.388B 5.388 | 2 010-2 025 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.388 5.389C 5.389E | 2 010-2 025 FIXED MOBILE 5.388A 5.388B 5.388 |

Table 1 Frequency allocations relevant to the band 1980 - 2010 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 2 120-2 160 FIXED MOBILE 5.388A 5.388B | 2 120-2 160 FIXED MOBILE 5.388A 5.388B Mobile-satellite (space-to-Earth) | 2 120-2 160 FIXED MOBILE 5.388A 5.388B |
| 5.388 | 5.388 | 5.388 |
| 2 160-2 170 FIXED MOBILE 5.388A 5.388B 5.388 | 2 160-2 170 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.388 5.389C 5.389E | 2 160-2 170 FIXED MOBILE 5.388A 5.388B 5.388 |
| 2 170-2 200 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.351A 5.388 5.389A 5.389F |  |  |
| 2 200-2 290 SPACE OPERATION (space-to-Earth) (space-to-space) EARTH EXPLORATION-SATELLITE (space-to-Earth) (space-to-space) FIXED MOBILE 5.391 SPACE RESEARCH (space-to-Earth) (space-to-space) 5.392 |  |  |

Table 2 Frequency allocations relevant to the band 2170 - 2200 MHz

In the case of MSS/CGC user terminals, two new ETSI standards have recently been developed. One of these relates to MSS/CGC terminals with a bandwidth in the range 1 - 5 MHz. The other relates to MSS only terminals with a bandwidth in the range 55 kHz - 1 MHz. The studies in Report 65, however, relate to narrow band satellite transmissions conforming to ETSI TBR 42. Furthermore, the 2 GHz MSS network previously considered was MEO whereas the network currently under consideration is GSO, and may use different types of terminals and higher power levels. The report, therefore, considers further compatibility studies using the new ETSI standards, to determine whether the principal findings of ERC Report 65 are still applicable.[Editorial comment: The paragraph needs some revision. The new ETSI standards should be named and identified.  The narrow band standard has no lower limit on the bandwidth.  Report 65 uses the unwanted emissions in TBR 42, which is applicable to GSO and non-GSO MSS systems].:

The SEAMCAT files used for the calculations are available in a zip-file at the www.ecodocdb.dk next to this Report.

# Frequency usage

## 2 GHz band

The different services in 2GHz and adjacent band s are illustrated in Figure 1. The more detail situation for the MSS uplink based on current ECC Decision 06(01) is illustrated in figure 2. [It should be noted that the ECC Decision is under review by ECC PT1]

1980

1920

1900

TDD

FDD UL

MSS/CGC

2010

2025

2110

MSS/CGC

2170

2200

FDD

TDD

MHz

FS, SS(E-S)

FS, SS(S-E)

DECT

Figure 1 the services/systems around the 2 GHz bands.

Figure 2 Information about the systems relevant for the studies in this Report.

# Sharing scenarios

The purpose of this Report is to analyse the consequences of the new ETSI standards for MSS UTs in 1980-2010 MHz as specified in 302 574-2 (Wide band) and 302 574-3 (Narrow band). Both these two standards specify new conditions for the MSS UT such as output power, ACLR etc. This report contains technical studies of adjacent channel compatibility between MSS terminals transmitting to a satellite in the band 1980 – 2010 MHz and adjacent channel UMTS services. [The studies involve higher power MSS terminals than those considered in ERC Report 65 which comply with the new ETSI Standards.][Editorial note: it appears that Report 65 did not consider in-band power] The satellite network can also operate in association with a Complementary Ground Component (CGC) providing additional terrestrial coverage, and the density of the terminals could exceed that considered in Report 65.

[Editors note: LS from PT1 requests information whether the high power in MSS terminals are for satellite only or also for CGC operation? (Who is to decide? MSS operator or regulator?) (ETSI EN 302 574-2 does not preclude the usage of high power terminals together with CGC. Terminal density will probably be different if high power terminals also is aloud to operate together with CGC.)]

There are two different frequency borders that are studied, around 1980 MHz and around 2010.

- Scenario 1: MSS UT (1980-2010 MHz)  ECN BS (1920-1980 MHz) – FDD operation

- Scenario 2 : MSS UT (1980-2010 MHz)  ECN MS and BS (2010-2025 MHz) – TDD operation

For the two scenarios the wide band and narrow band MSS UTs are studied both with deterministic analyze, Minimum Coupling Loss, and Monte Carlo calculations.

SEAMCAT® is used in this report to calculate the interference probability of MSS UTs into the terrestrial UMTS below 1980 MHz and above 2010 MHz.

The Minimum Coupling Loss method is used to analyse the interference between stations without taking dynamic aspects into account, and provides the necessary attenuation required between the systems to enable interference-free operation under specified conditions. Systems simulations provide more detailed analysis with realistic dynamic aspects of the involved systems.

# methodology

[Editors note: To included some intro text and a figure how the different compatibility cases look like.]

For the scenario 1 described in section 3, the BEM of table 11 of CEPT Report 39 is applicable to MSS UT (at the 1980 MHz boundary) and for the scenario 2 described section 3, the BEM of table 12 of CEPT Report 39 is applicable to MSS UT (at the 2010 MHz boundary).

Therefore, the most stringent BEM (table 12) has been applied. The consideration of those BEM would cover the impact to unwanted emissions of the MSS UT only.

Additionally to this topic, this issue of blocking of ECN BS (scenarios 1 and 2) and MS (scenario 2 only) by the in-band power of the MSS UT is also addressed, since this issue depends on the expected power of UT MSS.

## UMTS system characteristics and parameters

The receiver characteristics, such as ECN ACS, for the systems in the bands adjacent to the bands 1980-2010 MHz have been taken from the recently approved Report ITU-R M.2039-2 as a basis.

Proposed UMTS parameters for SEAMCAT simulations are listed in Annex xx. [Editors note: Annex xx to be developed.]

### FDD at 1980 MHz border

| UMTS FDD BS parameters at frequency border 1980 MHz |  |  |
| --- | --- | --- |
| Parameter | Value | Unit |
| Channel central frequency | 1977.2 | MHz |
| Channel bandwidth | 3.84 | MHz |
| Receiver sensitivity | -103 | dBm/3.84MHz |
| Allowed interference | -109 | dBm/3.84MHz |
| Receiver first adjacent ACS (Δf=5 MHz) | 46 | dB |
| Receiver second adjacent ACS (Δf=10 MHz) | 58 | dB |
| Receiver antenna gain | 18 | dBi |
| Receiver antenna feeder loss | 3 | dB |

Table 3 UMTS FDD base station parameters at 1980 MHz border.

### TDD at 2010 MHz border

| UMTS TDD BS parameters at frequency border 2010 MHz |  |  |
| --- | --- | --- |
| Parameter | Value | Unit |
| Channel central frequency | 2012.5 | MHz |
| Channel bandwidth | 3.84 | MHz |
| Receiver sensitivity | -103 | dBm/3.84MHz |
| Allowed interference | -109 | dBm/3.84MHz |
| Receiver first adjacent ACS (Δf=5 MHz) | 46 | dB |
| Receiver second adjacent ACS (Δf=10 MHz) | 58 | dB |
| Receiver antenna gain | 18 | dBi |
| Receiver antenna feeder loss | 3 | dB |

Table 4 UMTS TDD base station parameters at 2010 MHz border.

| UMTS TDD UE parameters at frequency border 2010 MHz |  |  |
| --- | --- | --- |
| Parameter | Value | Unit |
| Channel central frequency | 2012.5 | MHz |
| Channel bandwidth | 3.84 | MHz |
| Receiver sensitivity | -99 | dBm/3.84MHz |
| Allowed interference | -105 | dBm/3.84MHz |
| Receiver first adjacent ACS (Δf=5 MHz) | 33 | dB |
| Receiver second adjacent ACS (Δf=10 MHz) | 43 | dB |
| Receiver antenna gain | 0 | dBi |

Table 5 UMTS TDD user equipment parameters at 2010 MHz border.

## MSS within the band 1980-2010 MHz, characteristics and system parameters

Proposed MSS parameters for SEAMCAT simulations are listed in Annex xy. [Editors note: Annex xy to be developed.]

### EN 302 574-2 (wideband)

This standard applies to wideband UTs, with channel bandwidth 1 MHz or greater.  Different power classes are defined, as follows:

Table 6 Power classes in EN 302 574-2.

Table 4 in EN 302 574-2 shows the spectrum emission mask requirements. The parameter Δp is used to scale the relative requirements with increased transmit power. Due to the existence of the Δp, depending on antenna gain the standard may imply different out of band emissions for the same EIRP. Therefore, we have assessed the need for a guard band using paired EIRP antenna gain assumptions. Table 7 shows calculated ACLR values from the emission mask when assuming a 3.84 MHz wide carrier in a 5 MHz channel.

|  | 1st adjacent channel | 2nd adjacent channel |
| --- | --- | --- |
| power class | Calculated ACLR (dB) | Calculated ACLR (dB) |
| 1 | 37.4 | 50.4 |
| 1bis | 37.4 | 50.4 |
| 2 | 31.4 | 44.4 |
| 3 | 28.4 | 41.4 |

Table 7 Calculated ACLR values from spectrum emission mask.

Section 4.2.7 of EN 302 574-2 also contains a required ACLR value for first and second adjacent channels. Table 8 shows the specified values as for the calculated values in table 7.  A comparison show a difference between ACLR calculated from emission mask and specified ACLR of 4.6 dB for first adjacent and 1.6 dB for second adjacent channel.

|  | 1st adjacent channel |  | 2nd adjacent channel |  |
| --- | --- | --- | --- | --- |
| power class | ACLR (dB) | absolute (dBm/5 MHz) | ACLR (dB) | absolute (dBm/5 MHz) |
| 1 | 42 | -3 | 52 | -13 |
| 1bis2 | 42 | -9 | 52 | -19 |
| 2 | 36 | -9 | 46 | -19 |
| 3 | 33 | -9 | 43 | -19 |

Table 8 Specified MSS UT power in 1st and 2nd adjacent channels

Both the deterministic calculations and the Monte Carlo simulations will use the spectrum emission mask. But since the result will be a comparison between a terminal with 24 dBm output power and terminals using higher EIRP values will this ACLR difference have no impact.[Editorial comment: Since the ACLR values in Table 8 are the more stringent, these should  be used in the statistical analysis]

### EN 302 574-3 (narrowband)

This standard applies to narrowband UTs, defined as having a bandwidth less than 1 MHz.  The limits on the out-of-band emissions from UTs are defined by the limits in Tables 3a, 3b, 4a and 4b in the standard.  Tables 3a and 3b describes the limits outside 1980-2010 MHz band while tables 4a and 4b describes the limits inside the band 1980-2010 MHz. There are no limits for ACLR, however using the out-of-band emission limits it is possible to integrate the OOB limits over the first, second and third adjacent channels of 5 MHz to effectively determine the maximum power in each channel.  This is a conservative approach in that it effectively assumes that the UT OOB emissions conform exactly to the defined limits for each 5 MHz channel. Table 9 shows calculated ACLR values when using tables 3a, 3b and 4a in the standard 200 kHz bandwidth, and assuming that these are located directly adjacent to 1980 MHz border (centre frequencies 1980.0275 MHz and 1980.5 MHz). Adjacent channel is assumed to be same as for table 7.

|  | ACLR 200 kHz carrier |  |  |  |
| --- | --- | --- | --- | --- |
| EIRP (dBm) | 1:st adj (dB) | 2:nd adj (dB) |  |  |
| 39 | 36.6 | 47.9 |  |  |
| 33 | 30.6 | 41.9 |  |  |
| 27 | 24.6 | 35.9 |  |  |
| 24 | 21.6 | 32.9 |  |  |

Table 9 Calculated ACLR values from unwanted emission values.

[Editorial note: These calculated ACLR values need to be checked.  The calculations in doc SE40(11)16 give different results]

### Antenna and terminal types

- In order to conduct these studies it is necessary to make assumptions about terminal antenna gain. Various forms of user terminal are available for terrestrial or satellite communication and we have assumed that 2 GHz user terminals would develop similar characteristics of power level and likely antenna gains. 3G terminals used for terrestrial communication in adjacent bands have quite low antenna gains. However, the antenna gains for MSS type user terminals range from low to high. Overall, we have assumed the antenna gains as set out in the table 10 below as representative of current terminals. We used these in our analysis.

| Type | Hand held – terrestrial and/or satellite operation | Pocket – terrestrial and/or satellite operation | Notebook –terrestrial and/or satellite operation |
| --- | --- | --- | --- |
| Terminal power | 24 dBm | 39 dBm | 30 dBm for Narrowband MES 33 dBm for Wideband MES |
| Approximate Antenna gains | 0dBi | 0dBi | 15dBi |

Table 10 Antenna gain to be used in study.

[Editorial note: the use of the 24 dBm handheld terminal in the studies may be reviewed.  It is suggested to use this as a reference, but this does not represent real MSS operations.]

The terminal type and mapping power class and hence also the linked ACLR for the MSS UT is without antennas. It is therefore important to consider the adequate ACLR for the study cases of different antenna gain.

### Radiated Power assumptions for MSS type user terminals

- User terminals connecting with a CGC base station may use a power of typically 24 dBm. However, the 2 GHz user terminals considered in this document need to connect to the satellite component, which orbits at around 35,000 km. MSS user terminals having a power of 24 dBm (i.e. the same as terrestrial operation) would not be capable of operating with a satellite. Therefore, higher EIRP is required, which usually means there is an increase in the out of band emissions. The narrowband mode in the draft ETSI standard has stricter unwanted emissions limits than those for the wideband mode.

- The wideband CGC user terminal’s out of band emission levels increase with transmitter power and the standards indicate that additional limitations such as guard bands may apply to protect adjacent services.

- For user terminals that use the wideband signals, we have assessed whether or not there are guard band requirements to protect services in adjacent bands at 1980 and 2010MHz. These guard bands for the proposed wideband mode MSS type user terminal is dependent on the EIRP, which comes from the combination of transmit power and the antenna gain. However, the wideband ETSI standard has a factor (Δp) reducing the permitted out of band emissions for different transmit powers(Refer to wideband EN 302 574-2). Dependent on the transmit power; a terminal might need to apply a better ACLR. This factor is not present in similar 3GPP equipment standards.

- Due to the existence of the Δp factor, depending on antenna gain the standard may imply different out of band emissions for the same EIRP. Therefore, we have assessed the need for a guard band using paired EIRP antenna gain assumptions. It is assumed for these assessment of the out of band emissions that a terminal with a EIRP power up to 39dBm might use an omni antenna with 0dBi gain and that a user terminal with an EIRP of 48dBm might use an antenna gain of up to 15 dBi.

- For user terminals that use narrowband signals we have assessed the adjacent band edge compatibility by assuming that a MSS type user terminal might have a maximum EIRP of 39 dBm (with 0dBI antenna gain) or an EIRP 45dBm (with 15dBi antenna gain) to be capable of communication with the satellite component. We have assessed any requirement for guard bands on this basis.

### Channel plans at the band edges

- This section outlines the channel plans at the band edges for the MSS.

- There are two ETSI standards for the MSS/CGC user terminals, one using narrowband signals for satellite use and the other for wideband signals for satellite or CGC use. Both ETSI standards have a range of bandwidths within which manufacturers and operators can develop their networks. The narrow band terminal bandwidths are from 55 kHz to 1 MHz, the wideband terminals bandwidths from 1 MHz to 5 MHz.

- For 1980 MHz band edge the channelling of 2GHz terminals adjacent to mobile services could be: -

| 1980 MHz MSS wideband: 5MHz equipment centre frequencies (MHz) |  |  |
| --- | --- | --- |
| 1982.5 | 1987.5 | Etc. |

| 1980 MHz MSS wideband 1MHz: equipment centre frequencies (MHz) |  |  |
| --- | --- | --- |
| 1980.5 | 1981.5 | Etc. |

| 1980 MHz MSS Narrowband 1MHz : equipment centre frequencies (MHz) |  |  |
| --- | --- | --- |
| 1980.5 | 1981.5 | Etc. |

| 1980 MHz MSS Narrowband 55kHz : equipment centre frequencies (MHz) |  |  |
| --- | --- | --- |
| 1980.0275 | 1980.0825 | Etc. |

Table 11

- For 2010 MHz band edge the channelling of 2GHz terminals adjacent to mobile services could be: -

| 2010 MHz MSS wideband: 5MHz equipment centre frequencies (MHz) |  |  |
| --- | --- | --- |
| 2007.5 | 2002.5 | Etc. |

| 2010 MHz MSS wideband 1MHz: equipment centre frequencies (MHz) |  |  |
| --- | --- | --- |
| 2009.5 | 2008.5 | Etc. |

| 2010 MHz MSS Narrowband 1MHz : equipment centre frequencies (MHz) |  |  |
| --- | --- | --- |
| 2009.5 | 2008.5 | Etc. |

| 2010 MHz MSS Narrowband 55kHz : equipment centre frequencies (MHz) |  |  |
| --- | --- | --- |
| 2009.972 | 2009.917 | Etc. |

Table 12

[Editorial note: the above tables should be revised, as we are using MSS bandwidths of 200 kHz and 5 MHz]

## Propagation models

### Deterministic case

The propagation model will be the path loss model for outdoor to indoor and pedestrian test environment given in Section 1.1.2 of Appendix 1 to Annex 2 of Recommendation ITU-R M.1225.

This is expressed as:

L = 40 Log10 R + 30 Log10f + 49										(1)

Where:	R = path length (km)

f = frequency (MHz)

L= loss (dB)

### Monte Carlo simulation

[Editors note: add text for extended Hata model used in SEAMCAT]

## Deterministic analysis

### Methodology

The basic methodology for this exercise is presented in a report by the European Radio communications Committee (ERC) within the European Conference of Postal and Telecommunications Administrations (CEPT)124 and the same definitions apply.

Figure 3 below illustrates the basic concept of ACI. On the one hand, interference is caused to the victim receiver by out-of-band emissions from the adjacent system (blue shaded area on the right of the figure). The Adjacent Channel Leakage Ratio (ACLR) quantifies the degree to which this takes place.

A second source of interference is the victim receiver’s ability to reject signals in the adjacent channel otherwise known as the Adjacent Channel Selectivity (ACS). This is illustrated by the red shaded area (left hand shaded area) in Figure 3.

When considering the adjacent channel interference between two adjacent systems the ACLR of the interferer and the ACS of the victim receiver should be combined to give the overall Adjacent Channel Interference Ratio (ACIR) using the formula below:

ACIR = 1/(1/ACLR + 1/ACS)									(2)

ACIR is a measure of the combined interference due to the out-of-band emissions of a transmitter and the non-ideal selectivity of the receiver. It is a measure of the degree of isolation between adjacent systems and represents the degree of protection afforded to the receiver.

The formula shows that where one of the factors in the equation is much less than the other then it will tend to limit the overall ACIR performance of the system. This is also evident from Figure 3, where the total interfering power is the combination of the two shaded areas. If one is very much greater than the other then it will dominate the ACI performance between the two systems. If, for example, the ACLR is 10dB lower than the ACS, then the overall ACIR will only be 0.4dB worse than the ACLR.

Figure 3

Based on current 3G terminal and base station characteristics we have calculated the separation distance required between a transmitting 3G terminal with output power of 24 dBm and antenna gain of 0 dB and a base station receiving on an adjacent channel separated by 5 MHz. This distance is 223 metres. Consequently, in comparing CGC compatibility with adjacent band 3G services, the additional margin is compared to that at 223 metres.

To assess adjacent band compatibility it is necessary to determine the ACLR and ACS and combine them to derive ACIR as described in Section 4.1.1./4.1.2. as stated, we assume ACS values of 46 and 58 dB for separations of 5 MHz and 10 MHz. The ACLR of the terminals can be calculated by integrating the out of band emissions taken from the relevant ETSI standard EN 302 574-2 or EN 302 574-3.

For  a 24dBm EIRP wideband 5 MHz terminal (24 dBm transmit power with 0 dBi gain antenna), a numerical integration of the wideband out of band emission mask leads to an EIRP in the first 3.84 MHz channel of a 3GPP type system adjacent to 1980 MHz (centred on 1977.2 MHz):

Out of Band EIRP = -5.3 dBm

The ACLR is the ratio of wanted EIRP to the unwanted EIRP in the adjacent channel. Thus:

ACLR	= 24 – (-5.3) dB

= 29.3dB

Using this method, ACLR values have been derived for MSS MS EIRP values of 24, 39 and 48/45 dBm assuming antenna gains of 0, 0 and 15 dB respectively (as assumed in Section 4.2.3).

At both the 1980 MHz and 2010 MHz boundary, the gap between the edge of the MSS band and the first potential 3GPP channel is reduced [increased?] by 300 kHz [or 500 kHz] giving slightly higher ACLR values.

### Assumptions

[In the analysis we will assume that since existing UMTS networks can coordinate successfully with each other, we need to ensure that we achieve the same margin with the higher power MSS terminals. This will be achieved by introducing a frequency separation between the edge of the MSS channel and the 1980 or 2010 MHz band edge.][Editorial note: the idea here of using the coordination of two UMTS networks as a criterion for the assessment of interference from MSS is under discussion]

[We also assume worst case positioning of the MSS terminal and UMTS base station such that the base station is located in the bore sight of the MSS terminal antenna and the MSS terminal is located in the bore sight of the base station antenna. Consequently, the coupling between the antennas is subject to the maximum gain of both antennas.][Editorial comment: the worst case for the deterministic and monte-carlo analyses need to be further described)]

### Additional methodology, 2

- .TBD

## Monte-Carlo and SEAMCAT conditions

### ECN relevant SEAMCAT receiver parameters

[Editors note: The yellow marked text shoud be further developed.

The ECN design and receiver parameters are from the ITU-R M2039-2. The relevant SEAMCAT victim receiver parameters (including cell size, antenna gain, heights, carrier frequency assumptions etc.) could be extrapolated from this specification. Proposed parameters for SEAMCAT simulations are listed in Annex 4.]

### Methodology

- In section for deterministic methodology adjacent band compatibility between MSS Mobile Earth Stations operating within the band 1980 - 2010 MHz and 3GPP type base stations operating below 1980 MHz and above 2010 MHz has been analysed. These studies suggested the need for a frequency separation within the MSS band, in which terminals would not be able to connect to the satellite component in order to protect the base stations. This section uses a statistical Monte-Carlo analysis using the CEPT SEAMCAT analysis tool.

- For simplicity we have used a worst-case analysis using 3GPP centre frequencies of 1977.2 and 2012.8 and assumed that the results at both band edges will be equivalent.  [Editorial comment: the upper channel frequency is 2013 MHz]

- In performing the Monte-Carlo analysis we have assumed a density of active MSS terminals equivalent to either one or five MSS terminals per 900 km2 for the rural case and one MSS terminal per 9 km2 for the urban case. This is considered to represent a worst-case scenario as described in ECC Report 45.

- [Editors note: Agreement is needed for scenarios and parameters to use. The earlier Ofcom proposal was to only study the rural case (motivated by: in urban areas are CGC more likely). ECC PT1 LS requested some information/answers regarding high power terminal usage connected to CGC BS.  Requested information from ECC PT1 may require that also other scenarios are studied]

The studied scenarios are summarised in Table 13.

| - ETSI Standard | EN 302 574-2 (Wideband systems) |  |  | EN 302 574-3 (Narrowband systems) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| - MSS bandwidth | 5 MHz |  |  | 200 kHz |  |  |
| - EIRP (dBm) | 24 | 39 | 48 | 24 | 39 | 45 |
| - Frequency separation | 0 – 10 MHz (1 MHz steps) |  |  | 0 – 10 MHz (1 MHz steps) |  |  |

Table 13

- [Editorial comment: see comment above regarding use of 24 dBm eirp MESs]

# calculation Analysis and results

## Systems below 1980 MHz

[Editors note: text to be added.]

### Sub results from deterministic study

- The deterministic study is performed as described in 4.4.1.

- [Editors note: Added text here should be divided between sections 5.1.1. and 5.2.1. Given text is from Annex 4 of October 2010 SE40 meeting and is regarded as preliminary.]

- Wideband MES

- The calculated ACLR for the 1980 and 2010 MHz boundaries (3G centred on 1977.2 and 2012.8 MHz respectively) are shown in the Table 14 below

| - Boundary | - UMTS channel centre frequency (MHz) | - MSS EIRP (dBm) | - EIRP in receiver channel (dBm) | - ACLR (dB) |
| --- | --- | --- | --- | --- |
| - 1980 MHz | - 1977.2 - FDD | - 24 | - -5.3 | - 29.3 |
|  |  | - 39 | - 0.7 | - 38.3 |
|  |  | - 48 | - 9.7 | - 38.3 |
| - 2010 MHz | - 2012.8 - TDD | - 24 | - -5.3 | - 29.3 |
|  |  | - 39 | - 0.7 | - 38.3 |
|  |  | - 48 | - 9.7 | - 38.3 |

Table 14[Editorial comment: These ACLR values need to be checked, taking into account that the UMTS channel centre frequency is 2013 MHz]

- We can now calculate ACIR values based on the ACS value of 46 dB for 5 MHz separation of using the method described in Section 4.4.1. For the 1980 MHz boundary, the frequency separation is 5.3 MHz. In this case a value of ACS has been calculated by linear interpolation between the values of 46 and 58 dB for separations of 5 MHz and 10 MHz.

| - Boundary | - MSS EIRP (dBm) | - CGC ACLR (dB) | - UMTS ACS (dB) | - ACIR (dB) |
| --- | --- | --- | --- | --- |
| - 1980 MHz - FDD | - 24 | - 29.3 | - 46.7 | - 29.2 |
|  | - 39 | - 38.3 | - 46.7 | - 37.7 |
|  | - 48 | - 38.3 | - 46.7 | - 37.7 |
| - 2010 - MHz - TDD | - 24 | - 29.3 | - 46.7 | - 29.2 |
|  | - 39 | - 38.3 | - 46.7 | - 37.7 |
|  | - 48 | - 38.3 | - 46.7 | - 37.7 |

Table 15

The propagation model was used to calculate additional isolation required at 199 metre, the separation distance currently required between a transmitting 3G terminal with output power of 24 dBm and antenna gain of 0 dBi and a base station with 15 dBi antenna gain receiving on an adjacent channel separated by 5 MHz. This calculation was based on a maximum interference power of  -109 dBm as given in Sections 4.1.1/4.1.2.[Editorial comment: as noted above, the use of a 24 dBm as a reference is something which needs further consideration]

The table also shows the required frequency separation to achieve compatibility. This was calculated by changing the centre frequency of the MSS channel away from the band edge in small steps. At each step a modified value of ACLR was calculated by the integration procedure previously described and a revised value of ACS was calculated using the interpolation procedure. From these values the additional isolation was calculated as before. By repeating this procedure at each frequency step, the total shift in frequency at which no additional isolation was required was located.

| - UMTS Channel | - MSS EIRP (dBm) | - Additional isolation needed at 199 m (dB) | - Frequency Separation (MHz) |
| --- | --- | --- | --- |
| - 1977.2 MHz - FDD | - 24 | - -1 | - 0 |
|  | - 39 | - 5.6 | - 3 |
|  | - 48 | - 14.6 | - 7 |
| - 2012.8 - MHz - TDD | - 24 | - -1 | - 0 |
|  | - 39 | - 5.6 | - 3 |
|  | - 48 | - 14.6 | - 7 |

Table 16

Narrowband MES

Narrowband user terminals complying with EN 302 574-3 are primarily intended to operate towards a satellite. Terminals can operate at 55 kHz to 1MHz. The standard EN 302 574-3 has two applicable masks. One is an absolute mask for EIRP values up to +15 dBW (+45 dBm) independent of bandwidth or a bandwidth dependent mask. Manufacturers can opt for either mask.  However, as the narrow band standard depends mainly on spurious emissions, there shouldn’t be much difference between the two cases. With increased power, the standard requires an improved adjacent channel leakage ratio at the band edges 1980 and 2010MHz.

Based on a bandwidth dependent mask for 200 kHz bandwidth signal and using the numerical integration of the out of band and spurious emission mask against an UMTS channel in the adjacent band below 1980 MHz and above 2010 MHz we have calculated the ACLR values as follows:

|  |  | - MSS Bandwidth: 200 kHz |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Boundary | - MSS EIRP (dBm) | - EIRP in receiver channel (dBm) | - ACLR (dB) |  |  |
| - 1980 MHz | - 24 | - -2.6 | - 26.6 |  |  |
|  | - 39 | - -2.6 | - 41.6 |  |  |
|  | - 45 | - -2.6 | - 47.6 |  |  |
| - 2010 MHz | - 24 | - -2.6 | - 26.6 |  |  |
|  | - 39 | - -2.6 | - 41.6 |  |  |
|  | - 45 | - -2.6 | - 47.6 |  |  |

Table 17[Editorial comment: the ALCR values need to be checked]

To calculate the ACIR values we need suitable values for ACS. The assumptions from Section 4.1.1-2 are 46 and 58 dB for carrier separations of 5 MHz and 10 MHz. The same values are assumed to be valid for both a narrowband and a wide band interferer. ACIR is then calculated using the method described in Section 4.2.1.

|  |  | - MSS Bandwidth: 200 kHz |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Boundary | - MSS EIRP (dBm) | - ACS (dB) | - ACIR (dB) |  |  |
| - 1980 MHz | - 24 | - 46 | - 26.5 |  |  |
|  | - 39 | - 46 | - 40.3 |  |  |
|  | - 45 | - 46 | - 43.7 |  |  |
| - 2010 MHz | - 24 | - 46 | - 26.5 |  |  |
|  | - 39 | - 46 | - 40.3 |  |  |
|  | - 45 | - 46 | - 43.7 |  |  |

Table 18

The propagation model was used to calculate additional isolation required at 199 metre, the separation distance currently required between a transmitting 3G terminal with output power of 24 dBm and antenna gain of 0 dB and a base station receiving on an adjacent channel separated by 5 MHz. The table also shows the required frequency separation to achieve compatibility. This is based on a maximum interference power of  -109 dBm as given in Section 4.1.1/4.1.2.

|  |  | - MSS Bandwidth: 200 kHz |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Boundary | - MSS EIRP (dBm) | - Additional isolation needed at 199m - (dB) | - Frequency separation (MHz) |  |  |
| - 1980 MHz | - 24 | - 3.7 | - 1.25 |  |  |
|  | - 39 | - 6.8 | - 3.1 |  |  |
|  | - 45 | 11.0 | - 5.6 |  |  |
| - 2010 MHz | - 24 | 3.7 | - 1.25 |  |  |
|  | - 39 | 6.8 | - 3.1 |  |  |
|  | - 45 | 11.0 | - 5.6 |  |  |

Table 19

### Sub results from method 2 deterministic study

Results from a deterministic analysis are contained in Annex [5]

### Sub results from Monte Carlo simulations

- [Editorial note: Monte-Carlo simulations have been performed by Ericsson, Inmarsat and the ECO.  Further work is anticipated in a correspondence group to resolve the differences between the  results.  The preliminary results in this section may need to be revised accordingly.]

- The Monte Carlo simulations are performed as described in 4.5.

- Wideband MES

Urban Scenario with one interfering wideband MES per 9 km2

The below graph shows UMTS system degradation as a function of guard band for 3 different MES power levels (24 dBm, 39 dBm and 33 dBm + 15 dBi antenna). [As comparison, the degradation between 2 adjacent UMTS urban networks simulated with 35 users per cell is ~1% for the uplink.][Editorial comment: this idea of using the capacity loss between two UMTS networks as a reference is subject to discussion and review.  If it us used, the 1% value would need some justification]

The UMTS uplink capacity loss for single MSS terminal transmitting with 24 dBm as interferer is 6% without any guard separation. This value is substantially higher than between 2 UMTS networks. This is due to the used power control in UMTS networks. Both MSS terminals using 39 and 48 dBm EIRP levels have higher degradation and require a guard separation to reduce the capacity loss to same level as the MSS terminal using 24 dBm EIRP. The terminal using 39 dBm EIRP requires ~3.5 MHz frequency separation and the 48 dBm terminal requires ~0.5 MHz separation. The reason for this is that the 48 dBm terminal uses a directive antenna pointing somewhat upwards while the 39 dBm terminal uses an Omni antenna.

Rural Scenario (4.33 km cell radius) with one interfering wideband MES per 900 km2

The below graph shows UMTS system degradation as a function of guard band for 3 different MES power levels (24 dBm, 39 dBm and 33 dBm + 15 dBi antenna). [As comparison, the degradation between 2 adjacent UMTS rural networks simulated with 35 users per cell is ~2% for the uplink.  ] [Editorial comment: this idea of using the capacity loss between two UMTS networks as a reference is subject to discussion and review.  If it us used, the 2% value would need some justification]

The UMTS uplink capacity loss for single MSS terminal transmitting with 24 dBm as interferer is 6.6% without any guard separation. This value is substantially higher than between 2 UMTS networks. This is due to the used power control in UMTS networks. Both MSS terminals using 39 and 48 dBm EIRP levels have higher degradation and require a guard separation to reduce the capacity loss to same level as the MSS terminal using 24 dBm EIRP. The terminal using 39 dBm EIRP requires ~3.6 MHz frequency separation and the 48 dBm terminal requires ~1.5 MHz separation. The reason for this is that the 48 dBm terminal uses a directive antenna pointing somewhat upwards while the 39 dBm terminal uses an Omni antenna.

Rural Scenario (4.33 km cell radius) with five interfering wideband MES per 900 km2

The below graph shows UMTS system degradation as a function of guard band for 3 different MES power levels (24 dBm, 39 dBm and 33 dBm + 15 dBi antenna). [As comparison, the degradation between 2 adjacent UMTS rural networks simulated with 35 users per cell is ~2% for the uplink.] [Editorial comment: this idea of using the capacity loss between two UMTS networks as a reference is subject to discussion and review.  If it us used, the 2% value would need some justification]

The UMTS uplink capacity loss for five MSS terminal transmitting with 24 dBm as interferer is 10% without any guard separation. This value is substantially higher than between 2 UMTS networks. This is due to the used power control in UMTS networks. Both MSS terminals using 39 and 48 dBm EIRP levels have higher degradation and require a guard separation to reduce the capacity loss to same level as the MSS terminal using 24 dBm EIRP. The terminal using 39 dBm EIRP requires ~3.7 MHz frequency separation and the 48 dBm terminal requires ~2.1 MHz separation. The reason for this is that the 48 dBm terminal uses a directive antenna pointing somewhat upwards while the 39 dBm terminal uses an Omni antenna. One noticeable change when increasing the number of interferer is that the required guard for 39 dBm terminals is roughly the same while the required guard increases for the 48 dBm terminal. This is due to the increased probability with more interferer that the 48 dBm terminals directive antenna main lobe falls within the main lobe of the victim UMTS BS antenna.

Conclusions

- [TBD]

- Narrowband MES

## Systems above 2010 MHz

[Editors note: text to be added.]

### Sub results from deterministic study

For the ….

### Sub results from Monte Carlo simulations

## Sensitivity analysis

[Editors note: text to be added.]

This section gives an analysis on different assumptions, parameters and factors that have been considered in the study.

### Frequency planning between CGC and MSS uplink??

# Conclusions

[Editors note: conclusions to be added.]

For the band below 1980 MHz …

….

For the band above 2010 MHz …

Annex 1: Specification for ETSI standards below 1980 MHz

###### A1.1 Introduction

Annex 2: Specification for ETSI standards above 2010 MHz

###### Nmbnmbmbnm

ANNEX 3: Specification for ETSI MSS ES standards between 1980 - 2010 MHz

.

ANNEX 4:SIMULATION PARAMETERS AND SCENARIOS

In both scenarios, when studying the urban enviromnent, it is assumed that the CGC network will carry the major part of the load reducing the number of active MSS UTs transmitting to the satellite to only 1.

Scenario 1: MSS UT (1980-2010 MHz)  ECN BS (1920-1980 MHz) – FDD operation

Scenario 1.1 : Wideband MSS

Scenario 1.1.1 : Wideband MSS -> ECN BS in rural environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 30x30 km²

- 5 High gain terminals transmitting at full power to the satellite in 30x30 km² operating co-frequency (assuming CDMA) - TBC

- 1 Low gain terminal transmitting at full power to the satellite in 30x30 km²

- 5 Low gain terminals transmitting at full power to the satellite in 30x30 km² operating co-frequency (assuming CDMA) - TBC

- ECN terminals per ECN cell determined by SEAMCAT for the fully loaded system

Scenario 1.1.2 : Wideband MSS -> ECN BS in urban environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario 1.1.3 : Wideband MSS -> ECN BS in urban environment (Micro cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario 1.1.4 : Wideband MSS -> ECN BS in urban environment (Pico cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario 1.2 : Narrowband MSS

Scenario 1.2.1 : Narrowband MSS -> ECN BS in rural environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 30x30 km² (no CDMA used)

- 1 Low gain terminal transmitting at full power to the satellite in 30x30 km² (no CDMA used)

Scenario 1.2.2 : Narrowband MSS -> ECN BS in urban environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario 1.2.3 : Narrowband MSS -> ECN BS in urban environment (Micro cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario 1.2.4 : Narrowband MSS -> ECN BS in urban environment (Pico cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario 2 : MSS UT (1980-2010 MHz)  ECN MS and BS (2010-2025 MHz) – TDD operation

Assuming that there is no difference between the users load of FDD and TDD networks, the figures for the number of victim MS terminals should be derived from table 7A of ERC Report 65 reported below:

With regards to the number of MSS terminals transmitting to the satellite, the following scenarios should be considered:

Scenario 2.1 : Wideband MSS

Scenario 2.1.1 : Wideband MSS -> ECN BS in rural environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 30x30 km²

- 5 High gain terminals transmitting at full power to the satellite in 30x30 km² operating co-frequency (assuming CDMA) - TBC

- 1 Low gain terminal transmitting at full power to the satellite in 30x30 km²

- 5 Low gain terminals transmitting at full power to the satellite in 30x30 km² operating co-frequency (assuming CDMA) - TBC

Scenario 2.1.2 : Wideband MSS -> ECN BS in urban environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario 2.1.3 : Wideband MSS -> ECN BS in urban environment (Micro cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario 2.1.4 : Wideband MSS -> ECN BS in urban environment (Pico cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario 2.2 : Narrowband MSS

Scenario 2.2.1 : Narrowband MSS -> ECN BS in rural environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 30x30 km² (no CDMA used)

- 1 Low gain terminal transmitting at full power to the satellite in 30x30 km² (no CDMA used)

Scenario 2.2.2 : Narrowband MSS -> ECN BS in urban environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario 2.2.3 : Narrowband MSS -> ECN BS in urban environment (Micro cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario 2.2.4 : Narrowband MSS -> ECN BS in urban environment (Pico cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario 2.3 : Wideband MSS -> ECN BS

Scenario 2.3.1 : Wideband MSS -> ECN BS in rural environment (Macro cell)

Scenario 2.3.2 : Wideband MSS -> ECN BS in urban environment (Macro cell)

Scenario 2.3.3 : Wideband MSS -> ECN BS in urban environment (Micro cell)

Scenario 2.3.4 : Wideband MSS -> ECN BS in urban environment (Pico cell)

Scenario 2.4  : Narrowband MSS -> ECN BS

Scenario 2.4.1 : Narrowband MSS -> ECN BS in rural environment (Macro cell)

Scenario 2.4.2 : Narrowband MSS -> ECN BS in urban environment (Macro cell)

Scenario 2.4.3 : Narrowband MSS -> ECN BS in urban environment (Micro cell)

Scenario 2.4.4 : Narrowband MSS -> ECN BS in urban environment (Pico cell)

Propagation model

The extended hata model as implemented in SEAMCAT with the parameters defined in the Wiki for open environment will be used for the studies for rural environment. The extension to free space below 1 km can not be applied in urban environment because of smaller distances and presence of buildings. Therefore the extended hata model as implemented in SEAMCAT with the parameters defined in the Wiki for urban environment will be used for the studies for urban environment.

When assessing the UE-to-UE interference, PT1 noticed that in ECC Report 131 the free-space propagation model was used in the deterministic analysis and the IEEE 802.11 - C propagation model was used for statistical studies.

MSS terminals

- Wideband High gain  : 15 dBi, 33 dBm output power, 5° minimum elevation, 20° typical elevation, 1m above aground

- Narrowband High gain : 15 dBi, 30 dBm output power, 5° minimum elevation, 20° typical elevation, 1m above ground

(4)

where:

G() :	gain relative to an isotropic antenna (dBi)

G0 :	the main lobe antenna gain (dBi)

:	off-axis angle (degrees)

3 :	the 3 dB beamwidth of the low-gain antenna (degrees)

=  (degrees)

1	= 1.9 3 (degrees)

2	= 1 x 10(G0 – 6)/32 (degrees)

- Wideband and narrowband Low gain : 0 dBi, 39 dBm, omnidirectionnal, 1.5m

Unwanted emission mask and ACLR for wideband MSS terminals from the standard EN 302 574-2

|  | 1st adjacent channel |  | 2nd adjacent channel |  |
| --- | --- | --- | --- | --- |
| power class | ACLR (dB) | absolute (dBm/5 MHz) | ACLR (dB) | absolute (dBm/5 MHz) |
| 1 | 42 | -3 | 52 | -13 |
| 1bis2 | 42 | -9 | 52 | -19 |

Specified MSS UT power in 1st and 2nd adjacent channels

-	Unwanted emission mask and ACLR for narrowband MSS terminals from the standard EN 302 574-3

| Guard band (kHz) | 1st Adjacent 5 MHz Channel | 2nd Adjacent 5 MHz Channel | 3rd Adjacent 5 MHz Channel |
| --- | --- | --- | --- |
| 0 | 34.2 | -13.0 | -17.8 |
| 300 (FDD) | 15.5 | -13.0 | -17.8 |
| 500 (TDD) | 14.8 | -13.0 | -17.8 |

Calculation of ACLR using limits in Tables 3a, 3b and 4b
(values are the eirp in a 5 MHz channel in dBm)

The typical bandwidth for narrowband terminals is 200 kHz (the standard allows for 55 kHz).

The bandwidth for wideband terminals is 5 MHz.

ECN parameters

FDD/TDD Base station

- The following parameters are considered applicable to the ECN BS with regard to wideband MSS terminals. With regard to narrow band terminals, the ACS values for a given channel listed in the table below should be considered constant for the whole channel width.The value of ACS with regard to narrow band MSS terminals should be provided by ECC PT1.

|  | Macro BS | Micro BS | Pico BS |
| --- | --- | --- | --- |
| BS Antenna Gain (dBi) | 18 | 5 | 0 |
| Feeder loss (dB) | 3 | 0?1 | 0? |
| Reference sensitivity (dBm) | -121 | -111 | -107 |
| 1st channel ACS (dB) 1 | 46 | 46 | 46 |
| 2nd channel ACS (dB) | 58 | 53 | 54 |
| Maximum power interfering signal 1st ch. (dBm) | -62.74 | -52.74 | -48.74 |
| Maximum power interfering signal 2nd ch. (dBm) | -50.74 | -45.74 | -40.74 |

| Parameters | CDMA (FDD) 2GHz DL |
| --- | --- |
| Handover margin (dB) | 3 dB |
| Call drop threshold (dB) | 3 |
| Voice bit rate (kbps) | 12.2 |
| Reference bandwidth (MHz) | 3.84 |
| Voice activity factor | 0.5 |
| Minimum coupling loss (dB) | 70 |
| Link level data | W-CDMA/UMTS: Qualcomm Europe : 1900MHz: Uplink 1% FER |
| Link specific settings (for uplink) |  |
| Target Noise network rise, dB | 6 |
| Mobile station max transmit power, dBm | 24 |
| Mobile station power control range (dB) | 72 |
| Power control convergence precision, dB | 0.1 |
| Optimal capacity for CDMA system (users per cell) | Defined by SEAMCAT (35 users) |
| System layout (choice of the reference cell) | Center of “infinite” network |
| Positioning (number of BS in the system) | 57 (19 sites x 3 sectors/site) |
| BS antenna pattern | Recommendation ITU-R F.1336-2 |
| Mobile Terminal antenna height, m | 1.5 m |
| Terminal antenna gain, dB | 0 |
| Mobility (km/h) | see illustration of distribution on Fig. 3 |

CDMA FDD BS technical characteristics.

|  | Macro Cells | Micro Cells | Pico Cells |
| --- | --- | --- | --- |
| Typical cell radius (Km) | 4.33 | 1 | 0.315 |
| Antenna height (m) | 45 (Rural) 30 (Urban) | 5 | 2 |

The antenna pattern for BS : F.1336-2 peak sectoral, Azimuth aperture 65°, 18 dBi maximum gain, 3 dB feeder loss, 3° downtilt

TDD Base station

TBD

ANNEX 5:DETERMINISTIC STUDY RESULTS

[Editorial note: the propagation model used in these results needs to be confirmed]

Figures 3 - 18 show the interference in excess generated by an MSS UT at a given distance from an ECN BS, for the 3 types of cell, considering different offsets in the alignment between the two antennae.

Following the same approach, Figures 14 – 17 show the interference in excess generated by an MSS UT at a given distance from an ECN UT.

In the relevant figures, the different lines correspond to an increase of the off-axis offset in 5 degrees steps, starting from 0 up to 90.

CASE STUDY #1 – Wideband High Gain Terminal vs Macro Cell in rural environment

| Offset = 90 deg Offset = 0 deg | Offset = 0 deg Offset = 90 deg |
| --- | --- |

Figure 3a (left): Excess interference generated vs. distance in the 1st adjacent channel for different offsets between antennae

Figure 3b (right): Excess interference generated vs. distance in the 2nd adjacent channel for different offsets between antennae

CASE STUDY #2 – Wideband High Gain Terminal vs Macro Cell in urban environment

| Offset = 90 deg Offset = 0 deg | Offset = 0 deg Offset = 90 deg |
| --- | --- |

Figure 4a (left): Excess interference generated vs. distance in the 1st adjacent channel for different offsets between antennae

Figure 4b (right): Excess interference generated vs. distance in the 2nd adjacent channel for different offsets between antennae

CASE STUDY #3 – Wideband High Gain Terminal vs Micro Cell in urban environment

| Offset = 90 deg Offset = 0 deg | Offset = 90 deg Offset = 0 deg |
| --- | --- |

Figure 5a (left): Excess interference generated vs. distance in the 1st adjacent channel for different offsets between antennae

Figure 5b (right): Excess interference generated vs. distance in the 2nd adjacent channel for different offsets between antennae

CASE STUDY #4 – Wideband High Gain Terminal vs Pico Cell in urban environment

| Offset = 90 deg Offset = 0 deg | Offset = 0 deg Offset = 90 deg |
| --- | --- |

Figure 6a (left): Excess interference generated vs. distance in the 1st adjacent channel for different offsets between antennae

Figure 6b (right): Excess interference generated vs. distance in the 2nd adjacent channel for different offsets between antennae

CASE STUDY #5 – Wideband Low Gain Terminal vs Macro Cell in rural environment

Figure 7a (left): Excess interference generated vs. distance in the 1st adjacent channel [For Ericcson, they get crossover at 2.5 km for figure 7a.]

Figure 7b (right): Excess interference generated vs. distance in the 2nd adjacent channel

CASE STUDY #6 – Wideband Low Gain Terminal vs Macro Cell in urban environment

Figure 8a (left): Excess interference generated vs. distance in the 1st adjacent channel

Figure 8b (right): Excess interference generated vs. distance in the 2nd adjacent channel

CASE STUDY #7 – Wideband Low Gain Terminal vs Micro Cell in urban environment

Figure 9a (left): Excess interference generated vs. distance in the 1st adjacent channel

Figure 9b (right): Excess interference generated vs. distance in the 2nd adjacent channel

CASE STUDY #8 – Wideband Low Gain Terminal vs Pico Cell in urban environment

Figure 10a (left): Excess interference generated vs. distance in the 1st adjacent channel

Figure 10b (right): Excess interference generated vs. distance in the 2nd adjacent channel

CASE STUDY #9 – Narrowband High Gain Terminal vs Macro Cell in rural environment

| Offset = 90 deg Offset = 0 deg | Offset = 0 deg Offset = 90 deg |
| --- | --- |

Figure 11a (left): Excess interference generated vs. distance in the 1st adjacent channel for different offsets between antennae

Figure 11b (right): Excess interference generated vs. distance in the 2nd adjacent channel for different offsets between antennae

CASE STUDY #10 – Narrowband High Gain Terminal vs Macro Cell in urban environment

| Offset = 90 deg Offset = 0 deg | Offset = 90 deg Offset = 0 deg |
| --- | --- |

Figure 12a (left): Excess interference generated vs. distance in the 1st adjacent channel for different offsets between antennae

Figure 12b (right): Excess interference generated vs. distance in the 2nd adjacent channel for different offsets between antennae

CASE STUDY #11 – Narrowband High Gain Terminal vs Micro Cell in urban environment

| Offset = 90 deg Offset = 0 deg | Offset = 0 deg Offset = 90 deg |
| --- | --- |

Figure 13a (left): Excess interference generated vs. distance in the 1st adjacent channel for different offsets between antennae

Figure 13b (right): Excess interference generated vs. distance in the 2nd adjacent channel for different offsets between antennae

CASE STUDY #12 – Narrowband High Gain Terminal vs Pico Cell in urban environment

| Offset = 90 deg Offset = 0 deg | Offset = 90 deg Offset = 0 deg |
| --- | --- |

Figure 14a (left): Excess interference generated vs. distance in the 1st adjacent channel for different offsets between antennae

Figure 14b (right): Excess interference generated vs. distance in the 2nd adjacent channel for different offsets between antennae

CASE STUDY #13 – Narrowband Low Gain Terminal vs Macro Cell in rural environment

Figure 15a (left): Excess interference generated vs. distance in the 1st adjacent channel

Figure 15b (right): Excess interference generated vs. distance in the 2nd adjacent channel

CASE STUDY #14 – Narrowband Low Gain Terminal vs Macro Cell in urban environment

Figure 16a (left): Excess interference generated vs. distance in the 1st adjacent channel

Figure 16b (right): Excess interference generated vs. distance in the 2nd adjacent channel

CASE STUDY #15 – Narrowband Low Gain Terminal vs Micro Cell in urban environment

Figure 17a (left): Excess interference generated vs. distance in the 1st adjacent channel

Figure 17b (right): Excess interference generated vs. distance in the 2nd adjacent channel

CASE STUDY #16 – Narrowband Low Gain Terminal vs Pico Cell in urban environment

Figure 18a (left): Excess interference generated vs. distance in the 1st adjacent channel

Figure 18b (right): Excess interference generated vs. distance in the 2nd adjacent channel

CASE STUDY #17 – Wideband Terminal vs ECN UT in rural environment (using 802.11 – C model)

| Offset = 0 deg Offset = 90 deg |  |
| --- | --- |

Figure 14a (left): Excess interference generated by an MSS high gain UT vs. distance in the 1st adjacent channel for different offsets between antennae

Figure 14b (right): Excess interference generated by an MSS low gain UT vs. distance in the 1st adjacent channel for different offsets between antennae

CASE STUDY #18 – Wideband Terminal vs ECN UT in urban environment

| Offset = 90 deg Offset = 0 deg |  |
| --- | --- |

Figure 15a (left): Excess interference generated by an MSS high gain UT vs. distance in the 1st adjacent channel for different offsets between antennae

Figure 15b (right): Excess interference generated by an MSS low gain UT vs. distance in the 1st adjacent channel for different offsets between antennae

CASE STUDY #19 – Narrow Terminal vs ECN UT in rural environment (using 802.11 – C model)

| Offset = 0 deg Offset = 90 deg |  |
| --- | --- |

Figure 16a (left): Excess interference generated by an MSS high gain UT vs. distance in the 1st adjacent channel for different offsets between antennae

Figure 16b (right): Excess interference generated by an MSS low gain UT vs. distance in the 1st adjacent channel for different offsets between antennae

CASE STUDY #20 – Narrow Terminal vs ECN UT in urban environment

| Offset = 90 deg Offset = 0 deg |  |
| --- | --- |

Figure 17a (left): Excess interference generated by an MSS high gain UT vs. distance in the 1st adjacent channel for different offsets between antennae

Figure 17b (right): Excess interference generated by an MSS low gain UT vs. distance in the 1st adjacent channel for different offsets between antennae

Annex 5: LIST OF REFERENCES

- SEAMCAT Manual for Okumura-Hata model

- sfsfNet.

- Sdfsfsdf

Power class depends on the UT transmit power. Corresponding EIRP figure depends on the used antenna gain.

If necessary a guard band may be introduced. Any necessary guard band here should be added to guard band requirements in chapter 5.

Frequency separation is between the edge of the MSS channel and the edge of the band at 2010MHz or 1980 MHz.

If necessary a guard band may be introduced.

As specified in [4].

Cell radius are coherent with [4]