| To: Mr. Thomas Weilacher Chairman FM PT 48 Email: thomas.weilacher@bnetza.de |  |  |
| --- | --- | --- |
|  |  | Date: 29 February 2012 |
| Source: Project Team SE PT 44 |  |  |
| Subject: Results of the compatibility studies between Broadband Direct-Air-to-Ground Communications (BDA2GC) and BFWA in the frequency band 5855-5875 MHz |  |  |

Dear Thomas,

PT SE 44 is tasked to study compatibility and sharing issues concerning Broadband Direct-Air-to-Ground Communications (BDA2GC).

One of the candidate bands chosen by WG FM for compatibility and sharing studies is the band 5855-5875 MHz. The 5.8 GHz band is used for various radio applications; SE PT 44 studied firstly the compatibility scenario between BDA2GC and BFWA. Co-frequency scenario and TDD operation for BDA2GC operation has been considered.

For the results as summarised in this document, the BDA2GC system parameters which are based on ETSI TR 103 054 (System Reference Document on Broadband DA2GC) and, as far as available, which are based on inputs from Aero3G have been used. For BFWA, the system parameters as provided in ECC Report 68 and in ETSI TR 102 079 (System Reference Document for BFWA (HIPERMAN) in the 5.8 GHz band) have been used. Annex D of this ETSI Technical Report provides various radio parameters, especially table D.7 shows the following parameters for different modulation schemes:

Table D.7: FWA Mesh Radio parameters - II

| Modulation/Coding Rate |  | Rx Sensitivity (dBm @ 3 % PER) | C/I (dB @ 3 % PER) |
| --- | --- | --- | --- |
| BPSK | 1/2 | -82 | 6 |
|  | 3/4 | -81 | 11 |
| QPSK | 1/2 | -79 | 9 |
|  | 3/4 | -77 | 14 |
| 16QAM | 1/2 | -74 | 16 |
|  | 3/4 | -70 | 20 |
| 64QAM | 1/2 | -66 | 25 |
|  | 3/4 | -65 | 30 |

The relevant C/I value (20 dB)for a 16QAM BFWA system has been considered for the studies, although it was pointed out that a 64 QAM BFWA system with a C/I of 30 dB could be considered.

1. Interference between BFWA and BDA2GC AS (Aircraft Station)

Based on the calculations, the current conclusion is that the emissions from the DA2GC AS (aircraft station, according to the ETSI SRDoc. and to the Aero3G contributions) will interfere the BFWA system. Further studies are being undertaken. In the other direction (BFWA to DA2GC AS) compatibility may be achieved in case of a BFWA CS, when the altitude of the AS is above 3 000 m. In case of the BFWA TS as the interfering system and the DA2GC (ETSI) system as the victim station, interference can still be observed until an aircraft altitude of about 3 900 m is achieved. The calculation also show that the protection level of the aircraft receiver from the Aero3G system caused by emissions of a BFWA TS (Terminal Station) is exceeded significantly, which means that even at an aircraft altitude of 10 000 m interference may be observed. The situation will be worse by taking into account the aggregate effect in both interference directions.

2. Interference between BFWA CS and BDA2GC GS

Compatibility studies between BFWA CS (Central Station) and BDA2GC GS (Ground Stations according to ETSI TR 103 054 and according to Aero3G inputs to the meeting) have been carried out. The separation distances for all possible cases have been calculated. However, there is no need to consider the suitability of the separation distances more in detail because the co-channel case as described under No. 1 above has shown that compatibility between BDA2GC (according to ETSI TR 103 054 and according to the Aero3G contributions) and BFWA cannot be achieved. Further studies may be taken into account if appropriate.

Kind regards

Stefan Bach

Chairman SE 44

Email: stefan.bach@bnetza.de