|  |  | Doc. ECC/SE(12)022 |
| --- | --- | --- |
| #### Working Group SE |  |  |
| #### 61th Meeting of WG SE #### Åre, Sweden, 26 – 30 March 2012 |  |  |
| #### Date issued: | #### 19 March 2012 |  |
| #### Source: | #### SE44 Chairman |  |
| #### Subject: | #### SE44 Progress report |  |

| Password protection required? (Y/N) |  | N |
| --- | --- | --- |
| #### Summary: | #### This document contains the summary of the activities of SE44 since the 60th WG SE meeting in Tallinn. |  |
| #### Proposal: | #### WG SE is invited to consider this document and the related Annexes. |  |
| #### Background: | 1. Introduction SE44 has met twice since the previous WG SE meeting, from 15-16 December in Mainz, at the kind invitation of BNetzA, and from 28-29 February in Copenhagen, at the kind invitation of ECO. Good progress was made at both meetings. SE44 is tasked to perform sharing and compatibility studies related to Broadband Direct-Air-to-Ground Communications (DA2GC) in the frequency bands 3400-3600 MHz, 5855-5875 MHz, 1670-1675 MHz / 1800-1805 MHz and 2400-2483.5 MHz. WG FM during its meeting in Lille (October 2011), decided to remove the bands 1670-1675 MHz / 1800-1805 MHz from the list of candidate bands for Broadband DA2GC (ECC/SE(12)004). Therefore no further studies were carried out by SE44 regarding these bands. Three DA2GC system proposals are considered at this stage: - The Broadband DA2GC system described in the SRDoc ETSI TR 103 054. - An alternative system proposed by Lufthansa Systems planned to be operated in the unlicensed spectrum bands 2400–2483.5 MHz and 5855–5875 MHz. - A DA2GC system proposed by Aero3G based on 3G technology planned to be operated in the band 5850-5870 MHz. It is expected that, in addition to the ETSI System Reference Document TR 103 054 a second System Reference Document describing alternative Broadband DA2GC system characteristics will be provided by ETSI in the middle of 2012. For the compatibility and sharing studies, FM PT 48 recommends in a Liaison statement to SE44 using those parameters for Broadband DA2GC which are based on / derived from system descriptions as provided by ETSI. WG SE is invited: To note the discussions on the three different systems under discussion. To give guidance if any and to endorse the way forward. 2. Compatibility studies related to the band 2400 – 2483.5 MHz 2.1 Compatibility studies related to RLANs According to a Liaison statement from SE24 in response to a request on technical information on RLAN systems in the frequency band 2400-2483.5 MHz, the band is heavily used by RLAN, Bluetooth, Zigbee and many other applications. The document contains technical parameters of applications that could be used for the compatibility studies of SE44. An overview of SRD applications in the 2.4 GHz band with additional information (maximum output power, related ETSI standards, amount of chipsets shipped in 2011 …) is attached to the Liaison statement. From SE44 point of view the most critical compatibility scenario seems to be between Broadband DA2GC and outdoor RLANs, therefore this scenario has studied first. SE 44 considered a compatibility analysis, based on computer generated dynamic interference simulations, between the alternative DA2GC system employing beam forming antennas and outdoor RLANs operating co-frequency in the 2.4 GHz band. For this scenarios it was shown to be feasible for the DA2GC system to operate in both the air-to-ground and ground-to-air directions while ensuring that a C/I threshold of 10 dB is never exceeded at the RLAN device apart from an exclusion zone of around half a kilometre around the ground station. Several questions arrived with regard to the assumptions and the technical characteristics of the study, which need more explanation. A deeper consideration of these issues should be the subject of further studies, also taking into account the aggregate effect of multiple interfering sources. Moreover, it was stressed that a fuller set of technical characteristics of the alternative DA2GC system (e.g. receiver characteristics, spectrum emission mask, ..) are still required in order to allow the further studies to be carried out. Two further studies consider the co-frequency compatibility between the DA2GC Aircraft Station (AS) as described in ETSI TR 103 054 using TDD mode and RLANs in the band 2400-2483.5 MHz. The contributions come to the conclusion that with respect to the reverse link (air-to-ground) co-channel operation of DA2GC aircraft stations and outdoor RLAN systems in the band 2400-2483.5 MHz is not feasible. With respect to the forward link (ground-to-air) the main problem which arises is the aggregate effect of multiple transmitting RLANs on the same channel taking into account a density of 50 outdoor RLAN devices / km2. Even by assuming that these RLAN devices are partly operated on different channels and not during the same time, a sufficient high number of RLAN devices will remain to interfere the reception at the DA2GC AS. A study of the predicted throughput in a 20 MHz TDD BDA2GC system based on LTE in the frequency range 2400-2483.5 MHz was considered. Calculations based on the Shannon bound with LTE specific corrections for effective frequency usage, overhead and pilot tones show that RLANs deployed in this band produce high interference and as a result the Broadband DA2GC network cannot supply high capacity access to internet on the cell edge. As a result of the discussions regarding the 2.4 GHz band SE44 concluded on a Liaison statement to FM48 informing about the result of the studies. The Liaison statement is given in Annex 1. WG SE is invited: To note the results on the studies and the on-going discussions on the 2.4 GHz band. 3. Studies on DA2GC in the band 3400 - 3600 MHz 3.1 Compatibility studies related to FS and BWA systems in the 3.4 – 3.6 GHz band SE 44 received a study between a station from the Fixed Service in the frequency band 3400-3600 MHz and a Broadband DA2GC AS located on an aircraft. No coexistence issues were identified in the study, if the central frequencies of both systems are separated by at least 3 MHz. However, because it was questioned whether the technical characteristics of the FS P-P system, which were derived from ERC Report 040, are representative for systems in this frequency band, PT SE19 was asked for the technical characteristics of FS and BWA systems to be used in the studies. According to the response Liaison statement from SE19 received at the last SE44 meeting SE19 recommends using Recommendation ITU-R F.758-5 when frequency sharing with DA2GC is considered. Furthermore SE19 would recommend SE44 to emphasize more on the P-MP than on the P-P systems in that band. 3.2 Compatibility studies related to terrestrial LTE in the 3.4 – 3.6 GHz band SE 44 received several studies with regard to the compatibility between the DA2GC system as described in the ETSI TR 103 054. Preliminary result of the based on single entry studies was that for a terrestrial LTE station (BS and UE) and one DA2GC aircraft station (AS), both systems operating with a bandwidth of 10 MHz and the AS at an altitude of more than 3000m: - No coexistence issue was identified between one DA2GC AS and one LTE station in adjacent bands. - Co-existence issues were identified when the DA2GC AS and the LTE station are operating within the same frequency band. With regard to the compatibility between DA2GC ground stations (GS) and terrestrial LTE stations, the preliminary results showed that for co-location of a GS and a BS with unsynchronized TDD or TDD/FDD usage in adjacent frequency bands there is a risk of interference in both directions. Possible mitigation methods could be sufficient isolation, a frequency guard band or using different sites. In the latter case impacts could occur on LTE UE near the cell edge of the LTE BS coverage if located close to the DA2GC GS site. FM48 and ECC PT1 were informed about the preliminary results of the studies between DA2GC and LTE. The Liaison statement is given in Annex 2. A further contribution containing evaluations results with some modifications of base station antenna parameters, and additionally the results of Monte Carlo simulations with the SEAMCAT tool in order to evaluate dynamic interference scenarios between DA2GC and terrestrial LTE networks were considered at the 3rd SE44 meeting in Copenhagen. The evaluations results state that in principle a LTE-based DA2GC system can co-exist with a terrestrial mobile radio system with similar characteristics in the adjacent channel. This is primarily true if FDD mode operations are considered for both systems. No additional guard band between the channel block edges is required in that case. For TDD or mixed FDD/TDD mode operations the main problem to be solved is the decoupling of the antennas of the DA2GC GS and the LTE BS (eNodeB) which requires sufficient isolation in space and/or frequency or alternatively the usage of different sites for the systems. According to the current version of ECC Decision ECC/DEC/(11)06 both FDD and TDD arrangements for MFCN (including IMT/IMT-A systems) in the band 3400 – 3600 MHz are possible in European countries. Therefore, even with an implementation of DA2GC in FDD mode, frequency guard bands of about 5 MHz (in principle below as well as above the DA2GC spectrum) would be required for deployment at least in countries with TDD arrangement for terrestrial IMT/IMT-A. Under these circumstances, SE44 was of the view that the most spectrum efficient solution for DA2GC in the band 3400-3600 MHz would require a harmonized FDD frequency arrangement to include both DA2GC and MFCN. This would require a review of ECC Decision ECC/DEC(11)06 and ECC/DEC(07)02. Furthermore SE44 noted that coordination between countries might be an issue, which should be dealt in connection with DA2GC. 3.3 Compatibility studies related to the FSS in the band above 3400 MHz The band 3400-3600 MHz is allocated to the fixed satellite service (FSS) for space-to-Earth operations. The potential for interference from both aircraft and base-station Broadband DA2GC transmissions to FSS earth-station receivers was analysed. With respect to co-frequency operation of Broadband DA2GC base stations the exclusion areas for two example earth stations were determined by using terrain data and applying the ITU-R P.452 propagation model. The results show that the operation of DA2GC base stations in any parts of the band 3400-3600 MHz would require coordination with FSS earth stations on a case-by-case basis. Exclusion areas would also be required for DA2GC base stations operating on different frequencies from the FSS earth stations due to possible overload (or blocking) of the earth station LNB, and also due to reception of out-of-band emissions from the BDA2GC base station. Regarding the co-frequency transmissions of DA2GC aircraft stations the study showed the need for extensive physical separation (>450km) to avoid the potential for harmful interference into FSS earth station receivers. The operation of DA2GC aircraft stations on different frequencies from the FSS earth station also has the potential to cause harmful interference, due to unwanted emissions from the DA2GC aircraft station and due to overload of the earth station LNB. The possibility of overload could be avoided by the inclusion of filters at the earth station. SE44 noted that filtering will be greatly assisted if any operation of DA2GC aircraft transmission are identified to specific channels and having a maximum separation with existing FSS earth-stations operations (e.g., identified for operation at frequencies close to 3400 MHz instead of close to 3500 MHz). Considering the impact of DA2GC on MFCN and FSS Earth stations and vice versa and the limited number of FSS Earth station below 3520 MHz, the meeting was of the view that the following two options would be favourable for implementing DA2GC in the band 3400-3600 MHz, subject to analysis with regard to other services if considered as an appropriate basis for further studies: - 3410-3420 MHz (air-to-ground) paired with 3510-3520 MHz (ground-to-air) or - 3400-3410 MHz (air-to-ground) paired with 3500-3510 MHz (ground-to-air). The meeting agreed on a Liaison statement to ECC PT1 and PT FM48 informing about the results of the compatibility studies between the DA2GC system based on ETSI TR 103 054 and terrestrial LTE in the frequency band 3400-3600 MHz as well as about considerations on the co-existence between DA2GC and FSS Earth stations (s-E). The Liaison statement is given in Annex 3. A further Liaison Statement to PT FM48 regarding the compatibility with the FSS was developed in order to provide more detailed information relevant for the work on spectrum issues. The Liaison statement can be found in Annex 4. WG SE is invited: To note the results on the studies in the 3400 – 3600 MHz band. 4. Studies on DA2GC in the band 5855 -5875 MHz 4.1 Compatibility studies related to BFWA in the band 5855 -5875 MHz SE44 considered the compatibility between DA2GC systems and BFWA systems in the band 5855 – 5875 MHz. Co-frequency scenario and TDD operation for BDA2GC operation has been assumed. 4.1.1 Interference between BFWA and DA2GC AS (Aircraft Station) Based on the calculations, the current conclusion is that the emissions from the DA2GC AS (aircraft station, according to the ETSI SRDoc and to the Aero3G contributions) will interfere the BFWA system. Further studies are being undertaken. In the other direction (BFWA to DA2GC AS) compatibility may be achieved in case of a BFWA Central Station (CS), when the altitude of the AS is above 3 000 m. In case of the BFWA Terminal Station (TS) as the interfering system and the DA2GC (ETSI) system as the victim station, interference can still be observed until an aircraft altitude of about 3 900 m is achieved. The calculation also show that the protection level of the aircraft receiver from the Aero3G system caused by emissions of a BFWA TS is exceeded significantly, which means that even at an aircraft altitude of 10 000 m interference may be observed. The situation will be worse by taking into account the aggregate effect in both interference directions. 4.1.2. Interference between BFWA CS and BDA2GC GS Compatibility studies between BFWA CS and BDA2GC GS (Ground Stations according to ETSI TR 103 054 and according to Aero3G inputs) have been carried out. The separation distances for all possible cases have been calculated. However, there was no need to consider the suitability of the separation distances more in detail because the co-channel case as described under No. 4.1.1 above has shown that compatibility between DA2GC (according to ETSI TR 103 054 and according to the Aero3G contributions) and BFWA cannot be achieved. Further studies may be taken into account if appropriate. A Liaison statement was sent to PT FM48 informing about the current results of the compatibility studies between Broadband Direct-Air-to-Ground Communications (BDA2GC) and BFWA in the frequency band 5855-5875 MHz. The Liaison statement is given in Annex 5. Aero3G informed the meeting that additional studies are being undertaken and will be reported to the next meeting. These will include a mitigation mechanism to guard against any interference to other systems. WG SE is invited: To note the current results on the studies and the on-going discussions in the 5855 – 5875 MHz band. 5. Next meetings of SE44 The next meetings of SE PT44 will be held from 8th to 9th May 2012 by the kind invitation of Ofcom, UK, prior to the next FM48 meeting and from 21st to 22nd August 2012 (t.b.d) |  |