ECC Report <No>

COMPATIBILITY STUDIES – MSS TERMINALS TRANSMITTING TO A SATELLITE IN THE BAND 1980 – 2010 MHz AND ADJACENT CHANNEL UMTS SERVICES

Month YYYY (Arial 9pt bold)

(last updated: Month YYYY) (Arial 9pt)

# Executive summary

[Editor’s Note: Executive Summary to be drafted and agreed]

TABLE OF CONTENTS

0	Executive summary	2

1	Introduction	5

2	FREQUENCY USAGE	8

2.1	2 GHz band	8

3	SHARING SCENARIOS	9

4	ASSUMPTIONS AND METHODOLOGY	12

4.1	UMTS Base Stations characteristics and parameters	12

4.2	UMTS User Terminals characteristics and parameters	13

4.3	MSS User Terminals within the band 1980-2010 MHz, characteristics and system parameters	14

4.4	Frequency plans at the band edges	15

4.5	Propagation models	16

4.6	Methodology for deterministic analysis	17

4.7	Methodology for SEAMCAT analysis	18

5	analysis and results	20

5.1	ACIR for wideband MES	20

5.2	ACIR for narrowband MES	22

5.3	Calculation of the minimum required distance between an MSS UT and an ECN BS	24

5.4	Conclusion for the deterministic analysis	25

5.5	Statistical Study	26

5.6	conclusions for statistical analysis	29

5.6.1	Conclusions for the band below 1980 MHz	29

5.6.2	Conclusions for the band above 2010 MHz	30

6	conclusions	31

ANNEX 1: FIGURES RELATED TO THE DETERMINISTIC ANALYSIS	32

ANNEX 2: Calculated ACIR value variation with frequency offset	43

LIST OF ABBREVIATIONS

| Abbreviation | Explanation |
| --- | --- |
| ACLR | Adjacent Channel Leakage Ratio |
| ACS | Adjacent Channel Selectivity |
| BS | Base Station |
| BTS | Base Transceiver Station |
| BW | Bandwidth |
| CEPT | European Conference of Postal and Telecommunications Administrations |
| C/I | Carrier to Interference ratio |
| DEC | Decision |
| DECT | Digital Enhanced Cordless Telecommunications |
| ECA | European Common Allocation |
| ECC | European Communications Committee |
| ERC | European Radiocommunications Committee |
| ERP | Effective Radiated Power |
| ETSI | European Telecommunications Standards Institute |
| MCL | Minimum Coupling Loss |
| MS | Mobile Station |
| SEAMCAT | Spectrum Engineering Advanced Monte-Carlo Analysis Tool |
| UMTS | Universal Mobile Telecommunications System |
| MSS UT | Mobile Satellite System - User Terminal |

# Introduction

ECC Decision (06)09 designates the bands 1980 - 2010 MHz and 2170 - 2200 MHz to Mobile Satellite Services (MSS), which may incorporate Complementary Ground Component (CGC). It also states that the services will be compatible with adjacent mobile services. It is, therefore, important to determine the extent of any interference issues between MSS/CGC and adjacent band IMT services.

The frequency allocations in the bands 1980 - 2010 MHz and 2170 – 2200 MHz and their respective adjacent bands are given in Table 1. The relevant adjacent bands are 1900 - 1980 MHz, 2010 - 2025 MHz and 2110 - 2170 MHz.

ECC Decision (06)01 designates the adjacent bands mentioned above to IMT2000/UMTS. The band 1900 - 1980 MHz is designated for FDD uplink (mobile to base). The band 2110 - 2170 MHz is designated for FDD downlink (base to mobile) and the band 2010 - 2025 MHz is designated for either TDD or FDD uplink.

ERC Report 65 (completed in 1999) contains comprehensive analyses of compatibility between UMTS and several other services in the 2 GHz band. These other services include MSS in the bands 1980 – 2010 MHz and 2170 – 2200 MHz. However, the report did not cover the use of CGC base stations or user terminals accessing them.

Although not covered by Report 65, from the ETSI standards ETSI EN 302 574-1 and ETSI EN 301 908-3 it can be seen that the out of band emissions for CGC base stations are similar to those of UMTS 3GPP that already exist, since a similar technology is used. Furthermore, it is expected that the base stations of the two systems would use similar power levels and network deployment. Hence, the adjacency issues pertaining to these CGC base stations would essentially be identical to those that currently exist between different mobile network operators within the bands below 1980 MHz.

The analysis of adjacencies of 3GPP equipment and the use of new deployed FDD/TDD technology has been widely considered in Appendix 4 of CEPT Report 19 (for the band 2500-2690 MHz) and the issues are well explained and documented although no legacy equipment/system was available at the time the Report was published. The difference between the adjacencies for MSS CGC base stations at 2170-2200 MHz and the adjacencies for IMT base stations (2500-2690 MHz) are small. They both use IMT equipment built to ETSI standards that have out of band emission masks based on the same 3GPP standards described in CEPT Report 19.

CEPT Report 19 also includes regulatory solutions based on the use of restricted blocks to mitigate against interference between base stations as it may occur at the 2010 MHz boundary. Therefore, considering also that TDD networks are not largely deployed in the CEPT today, and that CGC BS parameters are quite similar to those of ECN FDD base stations, it is believed that there is no  compatibility issue between CGC base stations and ECN base stations.

Frequency allocations relevant to the bands 1980 - 2010 MHz and 2170 – 2200 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 930-1 970 FIXED MOBILE 5.388A 5.388B | 1 930-1 970 FIXED MOBILE 5.388A 5.388B Mobile-satellite (Earth-to-space) | 1 930-1 970 FIXED MOBILE 5.388A 5.388B |
| 5.388 | 5.388 | 5.388 |
| 1 970-1 980 FIXED MOBILE 5.388A 5.388B 5.388 |  |  |
| 1 980-2 010 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.351A 5.388 5.389A 5.389B 5.389F |  |  |
| 2 010-2 025 FIXED MOBILE 5.388A 5.388B 5.388 | 2 010-2 025 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.388 5.389C 5.389E | 2 010-2 025 FIXED MOBILE 5.388A 5.388B 5.388 |
| Allocation to services |  |  |
| Region 1 | Region 2 | Region 3 |
| 2 120-2 160 FIXED MOBILE 5.388A 5.388B | 2 120-2 160 FIXED MOBILE 5.388A 5.388B Mobile-satellite (space-to-Earth) | 2 120-2 160 FIXED MOBILE 5.388A 5.388B |
| 5.388 | 5.388 | 5.388 |
| 2 160-2 170 FIXED MOBILE 5.388A 5.388B 5.388 | 2 160-2 170 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.388 5.389C 5.389E | 2 160-2 170 FIXED MOBILE 5.388A 5.388B 5.388 |
| 2 170-2 200 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.351A 5.388 5.389A 5.389F |  |  |
| 2 200-2 290 SPACE OPERATION (space-to-Earth) (space-to-space) EARTH EXPLORATION-SATELLITE (space-to-Earth) (space-to-space) FIXED MOBILE 5.391 SPACE RESEARCH (space-to-Earth) (space-to-space) 5.392 |  |  |

In the case of MSS/CGC user terminals, two new ETSI standards have recently been developed. One of these relates to MSS/CGC terminals with a bandwidth in the range 1 - 5 MHz. The other one relates only to MSS user terminals with a bandwidth in the range 55 kHz - 1 MHz. The studies in the above mentioned Report 65, however, relate to narrow band satellite transmissions conforming to ETSI TBR 42. Furthermore, the 2 GHz MSS network previously considered was MEO whereas the network currently under consideration is GSO; therefore, different types of terminals and power levels may be used. This report then considers further compatibility studies using the new assumptions listed above, to determine whether the principal findings of ERC Report 65 are still applicable.

User Equipment (UE) terminals operating in ECN networks below 1980 MHz can have terminal powers operating up to 24 dBm. From a standard point of view, class 1 and 2 MSS terminals can connect to both satellite and CGC base stations; UE terminals operating within a satellite/CGC systems are assumed to have a maximum output power of +24dBm, when operating to CGC base station networks, in conformance with EN 302 574-2 power class 3 and CGC networks are assumed to be built in similar ways as terrestrial ECN networks and provide similar applications/services. Therefore, these are not studied within this report.

The SEAMCAT files used for the calculations are available in a zip-file at the www.ecodocdb.dk next to the publication of this Report.

The SEAMCAT release used was SEAMCAT 3.2.5 Beta7.

# FREQUENCY USAGE

## 2 GHz band

The different services in 2GHz and adjacent bands are illustrated in Figure 1. The more detailed situation for the MSS uplink based on current ECC Decision 06(01) is illustrated in Figure 2. [Editor’s note: It should be noted that the ECC Decision is under review by ECC PT1and usage of 2010-2025 MHz is under review by WGFM]

1980

1920

1900

TDD

FDD UL

MSS/CGC

2010

2025

2110

MSS/CGC

2170

2200

FDD

TDD

MHz

FS, SS(E-S)

FS, SS(S-E)

DECT

services/systems around the 2 GHz bands

Information about the systems relevant for the studies in this Report

# SHARING SCENARIOS

The purpose of this Report is to analyse the consequences of the new ETSI standards for MSS UTs in 1980-2010 MHz as specified in 302 574-2 (Wide band) and 302 574-3 (Narrow band). Both these two standards specify new conditions for the MSS UT such as output power, ACLR etc. This report contains technical studies of adjacent channel compatibility between MSS terminals transmitting to a satellite in the band 1980 – 2010 MHz and adjacent channel UMTS services. The studies involve higher power MSS terminals than those considered in ERC Report 65 which comply with the new ETSI standards. The satellite network can also operate in association with a Complementary Ground Component (CGC) providing additional terrestrial coverage and, therefore, the density of MSS terminals could exceed that considered in Report 65.

There are two different frequency borders that are studied, around 1980 MHz and around 2010.

Consequently, two different scenarios are defined, each one dealing with an edge of the 1980 – 2010 MHz band:

Scenario A: MSS UT transmitting in the band 1980-2010 MHz  ECN BS receiving in the band 1920-1980 MHz – FDD operation

Scenario A.1: Wideband MSS

Scenario A.1.1: Wideband MSS -> ECN BS in rural environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 30x30 km²

- 5 High gain terminals transmitting at full power to the satellite in 30x30 km² operating co-frequency (assuming CDMA)

- 1 Low gain terminal transmitting at full power to the satellite in 30x30 km²

- 5 Low gain terminals transmitting at full power to the satellite in 30x30 km² operating co-frequency (assuming CDMA) - TBC

- ECN terminals per ECN cell determined by SEAMCAT for the fully loaded system

Scenario A.1.2: Wideband MSS -> ECN BS in urban environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario A.1.3: Wideband MSS -> ECN BS in urban environment (Micro cell – studied with a deterministic approach only)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario A.1.4: Wideband MSS -> ECN BS in urban environment (Pico cell – studied with a deterministic approach only)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario A.2: Narrowband MSS

Scenario A.2.1: Narrowband MSS -> ECN BS in rural environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 30x30 km² (no CDMA used)

- 1 Low gain terminal transmitting at full power to the satellite in 30x30 km² (no CDMA used)

Scenario A.2.2: Narrowband MSS -> ECN BS in urban environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario A.2.3: Narrowband MSS -> ECN BS in urban environment (Micro cell – studied with a deterministic approach only)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario A.2.4: Narrowband MSS -> ECN BS in urban environment (Pico cell – studied with a deterministic approach only)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario B: MSS UT transmitting in the band 1980-2010 MHz  ECN MS receiving in the band 2010-2025 MHz – TDD operation

Scenario B.1 : Wideband MSS

Scenario B.1.1 : Wideband MSS -> ECN MS in rural environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 30x30 km²

- 5 High gain terminals transmitting at full power to the satellite in 30x30 km² operating co-frequency (assuming CDMA)

- 1 Low gain terminal transmitting at full power to the satellite in 30x30 km²

- 5 Low gain terminals transmitting at full power to the satellite in 30x30 km² operating co-frequency (assuming CDMA)

Scenario B.1.2 : Wideband MSS -> ECN MS in urban environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

Scenario B.2: Narrowband MSS

Scenario B.2.1: Narrowband MSS -> ECN MS in rural environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 30x30 km² (no CDMA used)

- 1 Low gain terminal transmitting at full power to the satellite in 30x30 km² (no CDMA used)

Scenario B.2.2: Narrowband MSS -> ECN MS in urban environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

It should be noticed that the assessment of the interference of MSS UT transmitting in the band 1980-2010 MHz into ECN BS receiving in the band 2010-2025 MHz and operating in TDD mode can be already studied in Scenario 1, since the assumptions to be made are very similar.

For the two scenarios listed above, the wide and narrow band MSS UTs are studied both with deterministic analysis, and Monte Carlo simulations; the latter approach is followed using the SEAMCAT software.

Moreover, a Minimum Coupling Loss method is used to analyse the interference between stations without taking dynamic aspects into account, providing the necessary attenuation required between MSS and ECN to enable interference-free operation under specified conditions.

# ASSUMPTIONS AND METHODOLOGY

Figure 3 below illustrates the interfering paths studied in this Report. In general, an MSS UT is supposed to transmit to a satellite in the 1980 – 2010 MHz band; therefore the following systems may be interfered due to unwanted emissions and the adjacent channel selectivity of the receiver:

- ECN FDD Base Stations receiving in the 1920 – 1980 MHz band;

- ECN TDD Base Stations receiving in the 2010 – 2025 MHz band;

- ECN TDD User Terminals receiving in the 2010 – 2025 MHz band.

Wanted signal paths

Interference paths

CGC BS

TDD MS

TDD BS

FDD BS

FDD MS

MSS UT

Interference scenarios

In the paragraphs below, the assumptions and the methodologies used for deriving the results in Section 5 are listed.

## UMTS Base Stations characteristics and parameters

The following parameters are applicable to the BS operating both in FDD mode (in the 1920 – 1980 MHz band) and in TDD mode (in the 2010 – 2025 MHz band).

- Antenna gain patterns:

The gain patterns for the ECN BS used in the calculations are listed in the following Table 2:

Parameters for BS antenna

| Type of analysis | Antenna gain pattern |
| --- | --- |
| Deterministic analysis | Macro and micro BS: Recommendation ITU-R F.1336-2, recommends 3.1 (“Peak side-lobe pattern”) Pico BS: Omni antenna |
| Statistical analysis with single MSS interferer | Macro BS: Recommendation ITU-R F.1336-2, recommends 3.1 (“Peak side-lobe pattern”) |
| Statistical analysis with multiple MSS interferers | Macro BS: Recommendation ITU-R F.1336-2, recommends 3.2 (“Average side-lobe pattern”) |

- Miscellaneous:

The parameters listed in the following Table 3 are applicable to the ECN BS and have been used in the calculations:

Parameters for ECN BS

|  | Macro BS | Micro BS | Pico BS |
| --- | --- | --- | --- |
| BS Antenna Gain (dBi) | 18 | 5 | 0 |
| Feeder loss (dB) | 3 | 1 | 0 |
| Reference sensitivity (dBm) | -121 | -111 | -107 |
| Channel bandwidth (MHz) | 5 |  |  |
| 1st channel ACS (dB) (± 5 MHz) | 46 | 46 | 46 |
| 2nd channel ACS (dB) (± 10 MHz) | 58 | 53 | 54 |
| Maximum power interfering signal 1st ch. (dBm) | -62.74 | -52.74 | -48.74 |
| Maximum power interfering signal 2nd ch. (dBm) | -50.74 | -45.74 | -40.74 |
| Noise Figure (dB) | 5.39 | 15.39 | 19.39 |
| Typical cell radius (km) | 6.0 (rural) 1.0 (urban) | 0.315 | 0.04 |
| Antenna height (m) | 45 (Rural) 30 (Urban) | 5 | 2 |
| Antenna downtilt (deg) | 3 | 0 | 0 |

## UMTS User Terminals characteristics and parameters

The parameters listed in the following Table 4 are applicable to the ECN User Terminals:

Parameters for ECN UT

| Parameter | Value |
| --- | --- |
| Antenna gain (dBi) | 0 (omni) |
| Feeder loss (dB) | 0 |
| Reference sensitivity (dBm) | -117 |
| Channel bandwidth (MHz) | 5 |
| 1st channel ACS (dB) (± 5 MHz) | 33 |
| Maximum power interfering signal 1st ch. (dBm) | -72.13 |
| Noise Figure (dB) | 9 |
| Antenna height (m) | 1.5 |

## MSS User Terminals within the band 1980-2010 MHz, characteristics and system parameters

This Section lists the parameters valid for MSS User Terminals.

In general, two types of UT are identified: wideband (operating with carrier bandwidths of 1 MHz or greater) and narrowband (operating with carrier bandwidths of less than 1 MHz). Each of them is referring to the ETSI standard EN 302 574-2 and EN 302 574-3, respectively.

For each of these two types, a given MSS UT can be a “High” or “Low” gain terminal. The following Tables 5 and 6 resumes the relevant parameters and assumptions applicable:

Parameters for MSS UT

| Parameter | Wideband High Gain | Narrowband High Gain | Wideband Low Gain | Narrowband Low Gain |
| --- | --- | --- | --- | --- |
| Maximum antenna gain (dBi) | 15 | 15 | 0 | 0 |
| Antenna gain pattern | As per rec. 4.1 of Rec. ITU-R F.1336-2 | As per rec. 4.1 of Rec. ITU-R F.1336-2 | Isotropic | Isotropic |
| Maximum output power at antenna connector (dBm) | 33 | 30 | 39 | 39 |
| Minimum antenna elevation (deg) | 5 | 5 | - | - |
| Typical antenna elevation (deg) | 20 | 20 | - | - |
| Carrier bandwidth | 5 MHz | 200 kHz | 5 MHz | 200 kHz |
| Antenna height (m) | 1 | 1 | 1.5 | 1.5 |

Power classes for wideband terminals (from ETSI EN 302 574-2)

The characteristics of the Adjacent Channel Leakage Ratio (ACLR) applicable to wideband terminals can be derived from Section 4.2.7 of the ETSI standard EN 302 574-2 mentioned above. The relevant parameters are resumed in the following Table 7.

Unwanted emission mask and ACLR for wideband MSS terminals (from ETSI EN 302 574-2)

|  | 1st adjacent channel |  | 2nd adjacent channel |  |
| --- | --- | --- | --- | --- |
| Type of terminal | ACLR (dB) | absolute (dBm/5 MHz) | ACLR (dB) | absolute (dBm/5 MHz) |
| High Gain | 42 | +6 | 52 | -4 |
| Low Gain | 42 | -3 | 52 | -13 |

Nevertheless, the same information for narrowband terminals is not contained in the ETSI standard EN 302 574-3. The best information that can be extrapolated from the document is contained in its Sections 4.2.2 and 4.2.3, dealing with the maximum EIRP spectral density of the unwanted emissions from the UE within and outside the 1980 – 2010 MHz band (shown in Table 8). Taking also into account the information contained in the previous Table 6, an ACLR can therefore be calculated. The relevant results are shown in Tables 9 and 10.

OOB emission mask for narrowband MSS terminals (calculated from ETSI EN 302 574-3) – values are absolute power in dBm in the Adjacent Channels 5 MHz wide dBm

| Guard band (kHz) | 1st Adjacent Channel | 2nd Adjacent Channel | 3rd Adjacent Channel |
| --- | --- | --- | --- |
| 0 | 34.0 | -13.0 | -17.8 |
| 100 | 14.7 | -13.0 | -17.8 |
| 200 | 14.7 | -13.0 | -17.8 |
| 300 | 14.6 | -13.0 | -17.8 |
| 400 | 14.6 | -13.0 | -17.8 |
| 500 | 14.5 | -13.0 | -17.8 |

Calculated ACLR for narrowband high gain MSS terminals (calculated from ETSI EN 302 574-3) – values are in dB and refer to Adjacent Channels 5 MHz wide

| Guard band (kHz) | 1st Adjacent Channel | 2nd Adjacent Channel | 3rd Adjacent Channel |
| --- | --- | --- | --- |
| 0 | 11.0 | 58.0 | 62.8 |
| 100 | 30.3 | 58.0 | 62.8 |
| 200 | 30.3 | 58.0 | 62.8 |
| 300 | 30.4 | 58.0 | 62.8 |
| 400 | 30.4 | 58.0 | 62.8 |
| 500 | 30.5 | 58.0 | 62.8 |

Calculated ACLR for narrowband low gain MSS terminals (calculated from ETSI EN 302 574-3) – values are in dB and refer to Adjacent Channels 5 MHz wide

| Guard band (kHz) | 1st Adjacent Channel | 2nd Adjacent Channel | 3rd Adjacent Channel |
| --- | --- | --- | --- |
| 0 | 5.0 | 52.0 | 56.8 |
| 100 | 24.3 | 52.0 | 56.8 |
| 200 | 24.3 | 52.0 | 56.8 |
| 300 | 24.4 | 52.0 | 56.8 |
| 400 | 24.4 | 52.0 | 56.8 |
| 500 | 24.5 | 52.0 | 56.8 |

## Frequency plans at the band edges

This section outlines the frequency plans at the band edges for both the MSS and ECN systems.

There are two ETSI standards for the MSS/CGC user terminals, one using narrowband signals for satellite use and the other for wideband signals for satellite or CGC use. Both ETSI standards have a range of carrier bandwidths within which manufacturers and operators can develop their networks. The narrow band terminal bandwidths are from 55 kHz to 1 MHz while the wideband terminals bandwidths are from 1 MHz to 5 MHz.

Taking into account the parameters listed in Table 5, for the1980 MHz band edge, the frequency plan applicable to the MSS user terminals could be:

Frequency plan for the 1980 MHz band edge – MSS

| 1980 MHz MSS wideband: 5 MHz UT centre frequencies (MHz) |  |  |
| --- | --- | --- |
| 1982.5 | 1987.5 | Etc. |
| 1980 MHz MSS Narrowband: 200 kHz UT centre frequencies (MHz) |  |  |
| 1980.1 | 1980.3 | Etc. |

Similarly, for the 2010 MHz band edge, the frequency plan applicable to the MSS user terminals could be:

Frequency plan for the 2010 MHz band edge – MSS

| 2010 MHz MSS wideband: 5MHz UT centre frequencies (MHz) |  |  |
| --- | --- | --- |
| 2007.5 | 2002.5 | Etc. |
| 2010 MHz MSS Narrowband: 200 kHz UT centre frequencies (MHz) |  |  |
| 2009.9 | 2009.7 | Etc. |

Decision ECC/DEC/(06)01 defines in Annex 1 the centre frequencies of the carriers nearest to the MSS band edges. Taking into account the channel bandwidth specified in Tables 3 and 4, two guard bands between the two services are identified: a guard band of 300 kHz on the 1980 MHz edge and a guard band of 500 kHz on the 2010 MHz edge. A frequency plan detailed on the following Tables 13 and 14 can then be outlined accordingly.

Frequency plan for the 1980 MHz band edge – ECN

| 1980 MHz ECN FDD: 5 MHz BS centre frequencies (MHz) – guard band of 300 kHz |  |  |
| --- | --- | --- |
| 1977.2 | 1972.2 | Etc. |

Frequency plan for the 2010 MHz band edge – ECN

| 2010 MHz ECN TDD: 5 MHz BS/UT centre frequencies (MHz) – guard band of 500 kHz |  |  |
| --- | --- | --- |
| 2013 | 2018 | Etc. |

## Propagation models

The propagation model to be used in the calculations varies accordingly to the particular scenario considered. The following Table 15 provides the right propagation model for each scenario:

Propagation models

| Scenario | Propagation Model | Notes |
| --- | --- | --- |
| MSS UT into ECN Macro BS in rural environment | Extended Hata – Rural | Outdoor and victim receiver “above roof”. |
| MSS UT into ECN Macro BS in urban environment | Extended Hata – Urban | Outdoor and victim receiver “above roof”. For deterministic study: Urban Hata. For statistical study use the Extended HATA model integrated in Seamcat. |
| MSS UT into ECN Micro/Pico BS in urban environment | Equation (5) in Annex 1 of Recommendation ITU-R P.1411-5 | Outdoor and victim receiver “below roof”. Median value of LoS propagation model within street canyons. |
| MSS UT into ECN UT | Free-space | To be used in deterministic analysis. |
| MSS UT into ECN UT | IEEE 802.11 - C | To be used in statistical analysis. |

## Methodology for deterministic analysis

This approach consists in determining the minimum distance at which an MSS User Terminal can generate an unacceptable interference into a particular ECN Base Station; the same approach can also be used for calculating the minimum required guard band to be put in place.

In order to determine the  interference due to both the in-band and out-of-band emissions of an MSS UT, a methodology aligned with that presented in the CEPT ECC Report 131 – “DERIVATION OF A BLOCK EDGE MASK (BEM) FOR TERMINAL STATIONS IN THE 2.6 GHz FREQUENCY BAND (2500-2690 MHz)” has been used.

Figure 4 below illustrates the basic concept of ACIR. On the one hand, interference is caused to the victim receiver by out-of-band emissions from the adjacent system (blue shaded area on the right of the figure). The Adjacent Channel Leakage Ratio (ACLR) quantifies the degree to which this takes place.

A second source of interference is the victim receiver’s ability to reject signals in the adjacent channel otherwise known as the Adjacent Channel Selectivity (ACS). This is illustrated by the red shaded area (left hand shaded area) in Figure 3.

When considering the adjacent channel interference between two adjacent systems the ACLR of the interferer and the ACS of the victim receiver should be combined to give the overall Adjacent Channel Interference Ratio (ACIR) using the formula below:

ACIR = 1/(1/ACLR + 1/ACS)					(2)

ACIR is a measure of the combined interference due to the out-of-band emissions of a transmitter and the non-ideal selectivity of the receiver. It is a measure of the degree of isolation between adjacent systems and represents the degree of protection afforded to the receiver.

The formula shows that where one of the factors in the equation is much less than the other then it will tend to limit the overall ACIR performance of the system. This is also evident from Figure 4, where the total interfering power is the combination of the two shaded areas. If one is very much greater than the other then it will dominate the ACI performance between the two systems. If, for example, the ACLR is 10dB lower than the ACS, then the overall ACIR will only be 0.4dB worse than the ACLR.

Description of ACIR, ACS and ACLR

To assess adjacent band compatibility it is necessary to determine the ACIR, taking into account the values of ACS and ACLR listed in the previous Tables 4, 7 and 8.

In the calculations it is assumed that the boresights of the MSS UT and ECN BS (or UT) antennae are always lying on the same plane, when these are directional. Yet in this case, results have been calculated for 4 different antenna elevation angles (5, 10, 15 and 20 deg); this is why they are shown as a range in the sections which follow.

## Methodology for SEAMCAT analysis

This section presents the methodology of simulations addressing those applicable scenarios established in this report to analyse potential interference to ECN networks from the operation of MSS UE.

Parameters of modelled land-based UMTS systems were established in accordance with the main parameters set out in Tables 3 and 4. In addition to those, several other secondary yet important parameters needed for SEAMCAT simulations were assumed as follows:

Additional parameters used to define victim UMTS systems

| Parameter | Value |
| --- | --- |
| Voice activity factor | 0.5 |
| Receiver noise figure: Macro BS | 5.39 dB |
| Voice bitrate | 12.2 kbps (Note 1) |
| Link Level Data sets | W-CDMA/UMTS: Qualcomm Europe: 1900MHz; 1 % FER |
| Target network noise rise for CDMA Uplink | 6 dB |
| UMTS BS’ adjacent channel selectivity/blocking rejection | - See Table 3 |
| UMTS cell radius | See Table 3 |
| Cell type | 3 - sector antenna |
| Initial UMTS capacity, MS per sector (Note 2) | ~35 – Uplink scenario |
| Propagation model for links inside victim CDMA system | See explanations in Table 15 |

Note 1: 	only voice communication channel is modelled in SEAMCAT CDMA module for certain simplification reasons (i.e. providing stable, non-bursty communication);

Note 2: 	values are derived by SEAMCAT tool per scenario to define non-interfered capacity.

It should be further noted that interference impacts CDMA system differently than a “traditional” system. In non-CDMA system, the victim is passive with regards to the external interference, and interference criterion (C/I) considered as a trigger for interference occurrence.

But when victim is a CDMA system, it may use its inherent power tuning mechanism trying to compensate for interference up to a point when relevant network resources reach their limits and the victim system starts to disconnect some of the earlier associated mobiles. The interference here is therefore measured not in terms of probability of exceeding the C/I criterion, but the probability of certain excess capacity loss in victim CDMA system. In order to model this power tuning process correctly, the SEAMCAT builds a cluster of 19 CDMA sites and further complements it for the effect of “endless network” by applying a certain “wrap-around” technique.

This scenario describes the possibility that MSS UT’s are interfering into victim CDMA BS receiver (uplink). The physical outline of this scenario was derived by random positioning 1 (5) UT within the area 3030 km (for rural environment) and 33 km (for urban environment) from the central (reference) cell of ECN network. An illustrating picture taken from the actual SEAMCAT simulation status window is shown below in Figure 5. Arrows on this figure show an example of a direction of an MSS UT.

Results are presented when considering MSS UT operating in the same location as the interfered UMTS network. At each snapshot SEAMCAT randomly positions 1 (or 5) interfering transmitter(s) depending on the considered scenario.

Outline of SEAMCAT simulations representing 1 interfering transmitter case
 (only 2 snapshots are shown)

Clarifications to SEAMCAT terms used in Figure 5 are given below:

SEAMCAT Term:

- Wanted Rx (WR);

- Interfering Tx (IT)

- Wanted Tx (WT) – numerous CDMA MSs are not plotted by SEAMCAT in order not to clog the screen;

- Victim Rx (VR) – CDMA BS in uplink

# analysis and results

The deterministic calculations are performed according to the scenarios defined in the previous Chapter 3, using parameters according to Chapter 4 and the methodology previously described. 
At the same time, without considering the 300/500 kHz guard bands that exist at the 1980 and 2010 MHz borders, the interference generated by an UMTS terminal equipped with an omnidirectional antenna and transmitting a maximum power of 24 dBm was selected as the criterion to trigger a deeper analysis through a statistical approach. Therefore, for a given scenario, if results obtained through a deterministic approach are considered acceptable, no statistical analysis is performed for that particular scenario.

Where required, statistical studies are performed using the SEAMCAT tool.  However in the case of Micro and Pico cells, the SEAMCAT tool (or any other freely available tool) is unable to model such use adequately, since it is not currently possible to model a realistic deployment of Micro and Pico cells as deployed within a typical real network.  Consequently, even where statistical analysis of interference to a Micro or Pico cell might be required, such analysis was not performed.

## ACIR for wideband MES

The calculated first and second adjacent channel ACIR values for wideband MES with respect to the ECN Base Stations for the 1980 and 2010 MHz boundaries are shown in the Table 17 below.

Table 18 contains, instead, the ACIR value with respect to the ECN User Terminals for the 2010 MHz boundary.

When assessing the ACIR with a guard band applied at one edge of the band, the values of ACLR and ACS have appropriately been re-calculated taking into account the values available for both the 1st and the 2nd adjacent channels. In Annex xx are figures showing how ACIR values vary with frequency offset (guard).

ACIR for wideband MSS User Terminals vs. ECN Base Stations

| - Boundary | UMTS channel centre frequency (MHz) | Scenario | ACLR (dB) first adjacent channel | ACS (dB) first adjacent channel | ACIR (dB) first adjacent channel | ACIR (dB) second adjacent channel |
| --- | --- | --- | --- | --- | --- | --- |
| 1980 MHz or 2010 MHz | 1977.5 (FDD) or 2012.5 (TDD) | A.1 | 42 | 46 | 40.5 | 51.0 |
| 1980 MHz | 1977.2 (300 kHz guard band) (FDD) | A.1.1 and A.1.2 | 42.2 | 46.3 | 40.8 | 51.3 |
|  |  | A.1.3 | 42.2 | 46.2 | 40.7 | 49.6 |
|  |  | A.1.4 | 42.2 | 46.2 | 40.8 | 50.0 |
| 2010 MHz | 2013 (500 kHz guard band) (TDD) | A.1.1 and A.1.2 | 42.4 | 46.4 | 41.0 | 51.3 |
|  |  | A.1.3 | 42.4 | 46.4 | 40.9 | 49.7 |
|  |  | A.1.4 | 42.4 | 46.4 | 40.9 | 50.1 |

ACIR for wideband MSS User Terminals vs. ECN User Terminals

| - Boundary | UMTS channel centre frequency (MHz) | Scenario | ACLR (dB) first adjacent channel | ACS (dB) first adjacent channel | ACIR (dB) first adjacent channel | ACIR (dB) second adjacent channel |
| --- | --- | --- | --- | --- | --- | --- |
| 2010 MHz | 2012.5 (TDD) | B.1 | 42 | 33 | 32.5 | 32.5 |
| 2010 MHz | 2013 (500 kHz guard band) (TDD) | B.1 | 42.4 | 33 | 32.5 | 32.5 |

## ACIR for narrowband MES

The calculated first and second adjacent channel ACIR values for narrowband MES with respect to the ECN Base Stations for the 1980 and 2010 MHz boundaries are shown in the Table 19 below.

Table 20 contains, instead, the ACIR value with respect to the ECN User Terminals for the 2010 MHz boundary.

When assessing the ACIR with a guard band applied at one edge of the band, the values ACS have appropriately been re-calculated taking into account the values available for both the 1st and the 2nd adjacent channels. In Annex xx are figures showing how ACIR values vary with frequency offset (guard).

ACIR for narrowband MSS User Terminals vs. ECN Base Stations

| - Boundary | UMTS channel centre frequency (MHz) | Scenario | ACLR (dB) first adjacent channel | ACS (dB) first adjacent channel | ACIR (dB) first adjacent channel | ACIR (dB) second adjacent channel |
| --- | --- | --- | --- | --- | --- | --- |
| 1980 MHz or 2010 MHz | 1977.5 (FDD) or 2012.5 (TDD) | High Gain UT | 11 | 46 | 11 | 52.5 |
|  |  | Low Gain UT | 5 | 46 | 5 | 49.9 |
| 1980 MHz | 1977.2 (300 kHz guard band) (FDD) | A.2.1.a and A.2.2.a | 30.4 | 46.3 | 30.3 | 55 |
|  |  | A.2.1.b and A.2.2.b | 24.4 | 46.3 | 24.4 | 51 |
|  |  | A.2.3.a | 30.4 | 46.2 | 30.3 | 51.8 |
|  |  | A.2.3.b | 24.4 | 46.2 | 24.4 | 49.5 |
|  |  | A.2.4.a | 30.4 | 46.2 | 30.3 | 52.5 |
|  |  | A.2.4.b | 24.4 | 46.2 | 24.4 | 49.9 |
| 2010 MHz | 2013 (500 kHz guard band) (TDD) | A.2.1.a and A.2.2.a | 30.5 | 46.4 | 30.4 | 55 |
|  |  | A.2.1.b and A.2.2.b | 24.5 | 46.4 | 24.5 | 51 |
|  |  | A.2.3.a | 30.5 | 46.4 | 30.4 | 52.5 |
|  |  | A.2.3.b | 24.5 | 46.4 | 24.5 | 49.5 |
|  |  | A.2.4.a | 30.5 | 46.4 | 30.4 | 52.5 |
|  |  | A.2.4.b | 24.5 | 46.4 | 24.5 | 49.9 |

ACIR for narrowband MSS User Terminals vs. ECN User Terminals

| - Boundary | UMTS channel centre frequency (MHz) | Scenario | ACLR (dB) first adjacent channel | ACS (dB) first adjacent channel | ACIR (dB) first adjacent channel | ACIR (dB) second adjacent channel |
| --- | --- | --- | --- | --- | --- | --- |
| 2010 MHz | 2012.5 (TDD) | B.1 – High Gain MSS UT | 11 | 33 | 11 | 33 |
|  |  | B.1 – Low Gain MSS UT | 5 | 33 | 5 | 32.9 |
| 2010 MHz | 2013 (500 kHz guard band) (TDD) | B.1 – High Gain MSS UT | 30.5 | 331 | 28.6 | 33 |
|  |  | B.1 – Low Gain MSS UT | 24.5 | 331 | 23.9 | 32.9 |

## Calculation of the minimum required distance between an MSS UT and an ECN BS

The following Table 21 shows the results obtained through a deterministic analysis of the adjacent channel interference scenarios listed in Section 3. The way the potential interference a given User Terminal could cause into the relevant ECN system is shown as a margin (in dB) of the excess interference generated into the receiver as a function of the separation distance and off-axis angle between the boresight of the UT antenna and that of the ECN base station.

Deterministic Study Results

|  | Minimum Required distance (km) |  |  |  |
| --- | --- | --- | --- | --- |
| Scenario | Elevation angle 5 deg | Elevation angle 10 deg | Elevation angle 15 deg | Elevation angle 20 deg |
| A.1.1.a | 1.5-5.7 | 1.5-5.4 | 1.5-4.8 | 1.5-4.2 |
| A.1.1.c | 3.6 | 3.6 | 3.6 | 3.6 |
| A.1.2.a | 0.1-0.6 | 0.1-0.6 | 0.1-0.5 | 0.1-0.5 |
| A.1.2.b | 0.4 | 0.4 | 0.4 | 0.4 |
| A.1.3.a | 0.2-0.7 | 0.2-0.7 | 0.2-0.6 | 0.2-0.5 |
| A.1.3.b | 0.5 | 0.5 | 0.5 | 0.5 |
| A.1.4.a | 0.1-0.3 | 0.1-0.3 | 0.1-0.2 | 0.1-0.2 |
| A.1.4.b | 0.2 | 0.2 | 0.2 | 0.2 |
| A.2.1.a | 2.3-9.2 | 2.3-8.6 | 2.3-7.7 | 2.3-6.6 |
| A.2.1.b3 | 10.0 | 10.0 | 10.0 | 10.0 |
| A.2.2.a3 | 0.2-1.0 | 0.2-0.9 | 0.2-0.8 | 0.2-0.7 |
| A.2.2.b3 | 1.0 | 1.0 | 1.0 | 1.0 |
| A.2.3.a3 | 0.3-1.1 | 0.3-1.0 | 0.3-0.9 | 0.3-0.8 |
| A.2.3.b3 | 1.4 | 1.4 | 1.4 | 1.4 |
| A.2.4.a3 | 0.1-0.4 | 0.1-0.4 | 0.1-0.4 | 0.1-0.3 |
| A.2.4.b3 | 0.6 | 0.6 | 0.6 | 0.6 |
| B.1.1.a | 1-13.2 | 1-11.7 | 1-9.5 | 1-7.2 |
| B.1.1.c | 4.9 | 4.9 | 4.9 | 4.9 |
| B.1.2.a | 1-13.2 | 1-11.7 | 1-9.5 | 1-7.2 |
| B.1.2.b | 4.9 | 4.9 | 4.9 | 4.9 |
| B.2.1.a | 1-14.6 | 1-12.9 | 1-10.6 | 1-8.0 |
| B.2.1.b | 13.1 | 13.1 | 13.1 | 13.1 |
| B.2.2.a | 1-14.6 | 1-12.9 | 1-10.6 | 1-8.0 |
| B.2.2.b | 13.1 | 13.1 | 13.1 | 13.1 |

Note: for a given base station, when results are indicated in range, the higher distance is the worst case for that particular base station.

## Conclusion for the deterministic analysis

The approach followed in Section 5.1 of this study shows the interference that could be generated by the MSS User Terminals’ In-Band emissions to ECN BSs and ECN UTs in the 2 GHz band.

Calculations of Section 5.1.3 related to the analysis of Scenario 1 determine the region within which an MSS UT transmitting to the satellite is generating interference greater than the maximum power shown in Table 3. The following Table 22 resumes the results obtained:

Deterministic Study Results

|  | Minimum distance from BS (km) | Worst Case |
| --- | --- | --- |
| Macro | 0.1-10.0 | Narrowband low gain UT in rural environment @ 5 deg el |
| Micro | 0.2-1.4 | Narrowband low gain UT in urban environment @ 5 deg el |
| Pico | 0.1-0.6 | Narrowband low gain UT in urban environment @ 5 deg el |
| ECN UT | 1-14.6 | Narrowband high gain UT in rural environment @ 5 deg el |

Table 22 shows that when the interfered system is a Macro BS, the minimum required distance between it and a transmitting narrowband MSS UT can be high if compared with the typical cell radius; for a given base station, when results are indicated in range, the higher distance is the worst case for that particular base station.

Given the results for the macro cell BS and for the ECN UT summarised in the table above, further analysis is conducted using SEAMCAT to more fully assess the potential for interference.  The SEAMCAT analysis is contained in the next section.

## Statistical Study

Frequency (Fc) of interferer 1982.5 MHz (wideband transmission); Frequency (Fc) of victim 1977.5 MHz (see also Table 9 for frequency plan);

Noise figure: Macro BS = 5.39 dB;

Scenario A.1 Wideband MSS transmission: MSS UT (1980-2010 MHz) into ECN BS (1920-1980 MHz) – FDD operation (BS is a victim, wideband transmission, rural/urban environment).

A.1.1.a) and b): the following results were derived for Scenario 1.1.1 (Macro cell) for high gain MSS terminals in rural environment:

1 or 5 High gain terminal transmitting at full power to the satellite in 30x30 km²

30 users per cell for the fully loaded system (SEAMCAT).

Scenario A.1.1.a) and b).: MSS UT (1980-2010 MHz) into ECN BS (1920-1980 MHz)

| MSS UE antenna tilt | Azimuth angle distribution of MSS UE antenna direction | N Int | Average capacity loss in reference cell, % | Average capacity loss in victim CDMA system, % | mean IRSSunw., dBm (reference cell) | mean IRSSblock., dBm (reference cell) |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 0 – 360 | 1 | 0.62 0.87 | 0.78 | -144.43 | -147.28 |
| 20 | 0 – 360 | 1 | 0.49 | 0.48 | -144.69 | -147.54 |
| 5 | 0 – 360 | 5 | 2.86 | 2.54 | -126.12 | -128.29 |

A.1.1.c) and d): the following results were derived for Scenario A.1.1 (Macro cell) for low gain terminals in rural environment:

- 1 (or 5) low gain terminal transmitting at full power to the satellite in 30x30 km²

- 30/33 users per cell for the fully loaded system (SEAMCAT).

Scenario A.1.1.c) and d).: MSS UT (1980-2010 MHz) into ECN BS (1920-1980 MHz)

| Azimuth angle distribution of MSS UE antenna direction | N Int | Average capacity loss in reference cell, % | Average capacity loss in victim CDMA system, % | mean IRSSunw., dBm (reference cell) | mean IRSSblock., dBm (reference cell) |
| --- | --- | --- | --- | --- | --- |
| 0 – 360 | 1 | 1.25 | 1.19 | -134.95 | -137.81 |
| 0 – 360 | 5 | 5.15 | 5.31 | -120.23 | -123.08 |

A.1.2: the following results were derived for Scenario A.1.2, low and high gain terminals:

Scenario A.1.2: Wideband MSS -> ECN BS in urban environment (outdoor (BS), outdoor (MS), above roof)(Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

- 30 user per cell for the fully loaded system (SEAMCAT).

Scenario A.1.2: MSS UT (1980-2010 MHz) into ECN BS (1920-1980 MHz)

| IT characteristics | Azimuth angle distribution of MSS UE antenna direction | MSS UE antenna tilt | Average capacity loss in reference cell, % | Average capacity loss in victim CDMA system, % | mean IRSSunw., dBm (reference cell) | mean IRSSblock., dBm (reference cell) |
| --- | --- | --- | --- | --- | --- | --- |
| Low gain | 0 – 360 | - | 1.27 | 1.27 | -144.99 | -147. 84 |
| High gain | 0 – 360 | 5 | 0.65 | 0.66 | -155.85 | -158.7 |
| High gain | 0 – 360 | 20 | 0.99 | 1.02 | -156.07 | -158.93 |

Scenario A.2: Narrowband MSS transmission

A.2.1: the following results were derived for Scenario A.2.1 for low and high gain terminals urban environment (Macro cell) (Fc=1980.1 MHz):

Scenario A.2.1: Narrowband MSS -> ECN BS in rural environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 30x30 km² (no CDMA used)

- 1 Low gain terminal transmitting at full power to the satellite in 30x30 km² (no CDMA used)

- Optimum number of users is 30

Scenario A.2.1: MSS UT (1980-2010 MHz) into ECN BS (1920-1980 MHz)

| IT characteristics | MSS UE antenna tilt | Average capacity loss in reference cell, % | Average capacity loss in victim CDMA system, % | mean IRSSunw., dBm (reference cell) | mean IRSSblock., dBm (reference cell) |
| --- | --- | --- | --- | --- | --- |
| Low gain | - | 44.63 | 45.63 | -104.85 | -138.7 |
| High gain | 5 | 27.65 | 27.53 | -125.35 | -159.2 |

A.2.1 (using guard bands): the following results were derived for Scenario 1.2.2_guard band for low and high gain terminals rural environment (Macro cell):

Scenario A.2.1_guard band 500 MHz: Narrowband MSS (Fc=1980.3 MHz) -> ECN BS in rural environment (Macro cell)

- 1 High gain terminal transmitting at full power to the satellite in 3x3 km²

- 1 Low gain terminal transmitting at full power to the satellite in 3x3 km²

- Optimum number of users is 30

Scenario A.2.1_guard band: High gain (500 kHz) /Low gain (300 kHz): MSS UT (1980-2010 MHz) into ECN BS (1920-1980 MHz)

| IT characteristics | MSS UE antenna tilt | Average capacity loss in reference cell, % | Average capacity loss in victim CDMA system, % | mean IRSSunw., dBm (reference cell) | mean IRSSblock., dBm (reference cell) |
| --- | --- | --- | --- | --- | --- |
| Low gain | - | 6.06 | 5.46 | -123.21 | -137.76 |
| High gain | 5 | 2.13 | 1.96 | -136.24 | -150.8 |

Scenario B.1: Wideband MSS UT transmitting in the band 1980-2010 MHz  ECN MS receiving in the band 2010-2025 MHz (TDD operation)

The following results were derived for Scenario B.1.1 (Macro cell) for high gain wideband MSS terminals in rural environment:

- 1 or 5 High gain terminal transmitting at full power to the satellite in 30x30 km²

- 36 users per cell for the fully loaded system (SEAMCAT).

Scenario B.1.a) and b): MSS UT (1980-2010 MHz) into ECN MS (2010-2025 MHz)

| MSS UE antenna tilt | Azimuth angle distribution of MSS UE antenna direction | N Int | Average capacity loss in reference cell, % | Average capacity loss in victim CDMA system, % | mean IRSSunw., dBm (reference cell) | mean IRSSblock., dBm (reference cell) |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 0 – 360 | 1 | 0.0 | 1.51 | -185.05 | -174.5 |
| 5 | 0 – 360 | 5 | 0.0 | 1.98 | -166.42 | -156.27 |

Following results were derived for Scenario B.1.1 (Macro cell) for low gain wideband MSS terminals in rural environment:

- 1 or 5 Low gain terminal transmitting at full power to the satellite in 30x30 km²

- 36 users per cell for the fully loaded system (SEAMCAT).

Scenario B.1.c) and d): MSS UT (1980-2010 MHz) into ECN MS (2010-2025 MHz)

| MSS UE antenna tilt | Azimuth angle distribution of MSS UE antenna direction | N Int | Average capacity loss in reference cell, % | Average capacity loss in victim CDMA system, % | mean IRSSunw., dBm (reference cell) | mean IRSSblock., dBm (reference cell) |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 0 – 360 | 1 | 0.0 | 1.85 | -185.05 | -174.5 |
| 5 | 0 – 360 | 5 | 0.0 | 1.85 | -160.38 | -150.24 |

Scenario B.2: Narrowband MSS

Scenario B.2.2: Narrowband MSS -> ECN MS in rural environment (Macro cell) - with guard band (Fit = 2,009.917 MHz)

- 1 High gain terminal transmitting at full power to the satellite in 30x30 km² (no CDMA used)

- 1 Low gain terminal transmitting at full power to the satellite in 30x30 km² (no CDMA used)

- 38 users per cell for the fully loaded system (SEAMCAT).

| MSS UE antenna tilt | Azimuth angle distribution of MSS UE antenna direction | Terminal type | Average capacity loss in reference cell, % | Average capacity loss in victim CDMA system, % | mean IRSSunw., dBm (reference cell) | mean IRSSblock., dBm (reference cell) |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 0 – 360 | H | 0.0 | 1.98 | -156.86 | -177.72 |
|  | 0 – 360 | L | 0.0 | 4.69 | -147.34 | -168.2 |

Scenario B.2.1: Narrowband MSS -> ECN MS in rural environment (Macro cell) (Fit = 2,009.972 MHz)

- 1 High gain terminal transmitting at full power to the satellite in 30x30 km²

- 1 Low gain terminal transmitting at full power to the satellite in 30x30 km²

- 36 users per cell for the fully loaded system (SEAMCAT).

Scenario B.2.1.a) and b): MSS UT (1980-2010 MHz) into ECN MS (2010-2025 MHz)

| MSS UE antenna tilt | Azimuth angle distribution of MSS UE antenna direction | Terminal type | Average capacity loss in reference cell, % | Average capacity loss in victim CDMA system, % | mean IRSSunw., dBm (reference cell) | mean IRSSblock., dBm (reference cell) |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 0 – 360 | H | 0.0 | 4.63 | -156.51 | -177.36 |
|  | 0 – 360 | L | 0.0 | 8.79 | -138.54 | -166.38 |

## conclusions for statistical analysis

Statistical analysis was performed using SEAMCAT tool. Analysis was limited to the ECN Macro cell case. Results presented in Tables 22 – 34 above. Following conclusions can be drawn from the study:

### Conclusions for the band below 1980 MHz

Wideband transmission (high/low gain terminals):

- Rural environment (no guard band? – If any guard band considered, which one? ECN and/or MSS?):

average capacity loss in victim CDMA system in the rural area case varying from 0.48% (Table 22 - high gain MSS) to 5.31% (Table 23 - low gain MSS);

- Urban environment (no guard band? – If any guard band considered, which one? ECN and/or MSS):

average capacity loss in victim CDMA system in the urban area case varying from 0.66% (Table 24 - high gain MSS) to 1.27% (Table 24 - low gain MSS);

Narrowband transmission (high/low gain terminals):

- Rural environment (no guard band):

average capacity loss in victim CDMA system in the rural area case varying up to 27.53% (Table 28 - high gain MSS) to 45.63% (Table 28 - low gain MSS);

- Rural environment (with guard band):

average capacity loss in victim CDMA system in the rural area case varying up to 1.96% (Table 29 - high gain MSS) to 5.46% (Table 29 - low gain MSS);

### Conclusions for the band above 2010 MHz

- Wideband transmission (high/low gain terminals):

- Rural environment (no guard band? – If any guard band considered, which one? ECN and/or MSS):

average capacity loss in victim CDMA system in the rural area case varying from 1.51% (Table 30 - high gain MSS) to 1.85% (Table 31 - low gain MSS);

- Narrowband transmission (high/low gain terminals):

- Rural environment (no guard band? – If any guard band considered, which one? ECN and/or MSS):

average capacity loss in victim CDMA system in the rural area case varying up to 27.53 % (high gain MSS) to 45.63% (low gain MSS);

- Rural environment (with guard band):

average capacity loss in victim CDMA system in the rural area case varying up to 1.98% (Table 32 - high gain MSS) to 4.69% (Table 32 - low gain MSS).

# conclusions

Results shown in the previous sections highlight potential issues when an MSS transmitting to the satellite is in the vicinity of a base station in an ECN network.

Deterministic results show that when an MSS UT is near to a victim ECN BS, in the absence of any mitigation technique, the interference caused is above the recommended criteria.

It should also be taken into account that ECN networks are built with more than one base station. Table 21 shows, in some cases, a range of distances in which interference would be caused to the victim ECN BS; in those cases, the lower distance may mean that the interferer is pointing towards another base station. Therefore, the statistical analysis was also performed as complementary analysis.

Statistical analysis performed with the SEAMCAT tool show that, in the absence of any mitigation technique, the network average capacity loss of the ECN network is lower when using MSS high gain terminals compared to that obtained with MSS low gain terminals. In the majority of the scenarios, the capacity loss is less than 5.5%, while in few cases it raises up to 45% when considering an MSS low gain terminal operating in rural environment.

Better results would be obtained if the ACIR for the various scenarios would be improved through appropriate interference mitigation solutions. Interference from MSS terminals to ECN BS comes from two contributions: out-of-band emissions of MSS terminals (derived through the ACLR) and selectivity/blocking capability of the ECN BS receiver (derived through the ACS).

- For improving interference from MSS UT out-of-band emissions to ECN BS, for the narrowband terminals, the relevant ETSI standard has no ACLR specified: in this Report, such parameter has been calculated from the specified limits of the EIRP radiated in the UE’s adjacent channels. Since the limits are expressed in absolute terms (EIRP in dBW), the same numbers applying irrespective of the in-band EIRP of the UE, the ACLR for low EIRP terminals is very low and almost certainly not realistic. Real hardware is supposed to have better performance and is therefore suggested that measures aimed to solve this issue would foresee a minimum ACLR or block-edge mask the terminals should comply with;

- Another measure aimed to solve the same issue would be an increase of the typical ACS of the ECN systems operating at the edges of the 1980 – 2010 MHz band and/or a reduction of the MSS UT in-band transmitting power operating near to the band edges to get the needed ACIR improvement;

Another potential concern comes from the ECN UT’s which could operate in the band above 2010 MHz, if that band is used for TDD systems.  Given their low capacity of filtering signals in the adjacent bands, these UTs require considerably big distances between users.
Nevertheless, an element to keep into account for well assessing the problem is that most of the ECN users will be concentrated in urban areas, where MSS users are less likely transmitting to the satellite, but eventually to the Complementary Ground Component of their network (when CGC is implemented), using powers and other parameters similar to an ECN network.
It should be noted that the band 1980-2010 MHz has been available for transmitting narrowband MESs for many years with those characteristics contained in standard EN 301-442. The introduction of regulations to permit CGC systems to be deployed has not changed the potential for in-band interference to ECN systems in the adjacent frequency bands.

FIGURES RELATED TO THE DETERMINISTIC ANALYSIS

| Scenario | Graph of the excess interference generated by the MSS UT into the ECN system |
| --- | --- |
| A.1.1.a | Offset = 90 deg Offset = 0 deg |
| A.1.1.c |  |
| A.1.2.a | Offset = 0 deg Offset = 90 deg |
| A.1.2.b |  |
| A.1.3.a | Offset = 90 deg Offset = 0 deg |
| A.1.3.b |  |
| A.1.4.a | Offset = 90 deg Offset = 0 deg |
| A.1.4.b |  |
| A.2.1.a (no guard-band) | Offset = 90 deg Offset = 0 deg |
| A.2.1.a (300 kHz guard-band) | Offset = 0 deg Offset = 90 deg |
| A.2.1.b (300 kHz guard-band) |  |
| A.2.2.a (300 kHz guard-band) | Offset = 90 deg Offset = 0 deg |
| A.2.2.b (300 kHz guard-band) |  |
| A.2.3.a (300 kHz guard-band) | Offset = 90 deg Offset = 0 deg |
| A.2.3.b (300 kHz guard-band) |  |
| A.2.4.a (300 kHz guard-band) | Offset = 90 deg Offset = 0 deg |
| A.2.4.b (300 kHz guard-band) |  |
| B.1.1.a B.1.2.a | Offset = 90 deg Offset = 0 deg |
| B.1.1.c B.1.2.b |  |
| B.2.1.a B.2.2.a (500 kHz guard-band) | Offset = 90 deg Offset = 0 deg |
| B.2.1.b B.2.2.b (500 kHz guard-band) |  |

Calculated ACIR value variation with frequency offset

ACIR variation due to frequency separation (guard band) for Macro BS and wideband/narrowband MES

ACIR variation due to frequency separation (guard band) for Micro BS and wideband/narrowband MES

______________________

If necessary a guard band may be introduced. Any necessary guard band here should be added to guard band requirements in chapter 5

It should be noted that there is no information available as far as the ACS of ECN UT on the 2nd adjacent channel; therefore, as a conservative approach, the same ACS as the 1st adjacent is here used.

Results are obtained applying a 300 kHz guard band