| To: Mr. Stefan Bach Chairman SE PT 44 Email: stefan.bach@bnetza.de |  |  |
| --- | --- | --- |
|  |  | Date: 01 December 2011 |
| Source: Project Team SE PT 24 |  |  |
| Subject: Broadband Direct-Air-to-Ground Communications (BDA2GC); Request for technical information on RLAN systems in the frequency band 2400-2483.5 MHz |  |  |

Dear Stefan,

SE24 discussed your Liaison statement at its meeting 27-28 October 2011 regarding protection criteria and characteristics for RLANs in the 2.4GHz band.

The band 2.4-2.4835 GHz is heavily used by RLAN, Bluetooth, Zigbee and many other applications; an overview of those SRD applications is attached to this document.

Examples of characteristics and protection criteria for compatibility studies with SRDs in the band 2.4-2.4835 GHz can be found in ERC Report 109.

The following Table provides some parameters of existing applications that could be used for the compatibility studies of SE44.

|  | RLAN/ Wifi – Outdoor (including p2p, p2mp, mesh,..) | RLAN/ Wifi – indoor | Bluetooth | Bluetooth (class 1) | Zigbee – 802.15.4 | RFID |
| --- | --- | --- | --- | --- | --- | --- |
| Tx-Power (conducted) | 10 to 17 dBm | 13 to 17 dBm | 0 to 5 dBm | 13 dBm | 5 to 15 dBm | 14 dBm |
| Bandwidth | 20 / 40 MHz | 20 / 40 MHz | 1 MHz | 1 MHz | 5 MHz | 3 MHz |
| Rx Sensitivity | -82 / -79 dBm | -82 / -79 dBm | -90 dBm | -90 dBm | -83 dBm | -90 dBm |
| Antenna Gain | 3 to 10 dBi | 3 to 7 dBi | 0 to 3 dBi | 0 to 3 dBi | 0 to 3 dBi | 6 dBi |
| Duty Cycle | Up to 100% | Up to 100% | Up to 100% | Up to 100% | 100% | Up to 100 % |
| Hopper/non-hopper | Non-hopper | Non-hopper | Hopper | Hopper | Hopper | Non-Hopper |
| Uniform Density per km2 | 50 | 5000 (e.g. Monaco) | 10.000 (e.g. Monaco) | 1000 | 100 | 100 |
| Rural/Urban | Rural + urban | Urban | Urban | Urban | Urban | Rural+Urban |
| Protection Criterion | C/I ≥ 10 dB | C/I ≥ 10 dB | C/I ≥ 10 dB | C/I ≥ 10 dB | C/I ≥ 10 dB | C/I ≥ 10 dB |
| Percentage indoor | 0 % | 99 % | 75 % | 98 % | 99 % | 75 % |
| Wall attenuation | 0 dB | 15 dB | 15 dB | 15 dB | 15 dB | 15 dB |

The next meeting of SE PT 24 is planned 13-15 December 2011 in Mainz.

Kind regards

Marc Le Devendec

Chairman SE 24

Overview of 2.4 GHz applications:

- WIFI or IEEE 802.11 based consumer and enterprise applications, including smart phones etc.. These products comply with EN 300 328 and Annex 3 of ERC Rec 70-03. Maximum output power is 100 mW e.i.r.p.

  - In 2011 only, in total 1 billion (1.000.000.000) chipsets were shipped!!!

- Bluetooth for short distance links used in Mobile (smart) phones, notebooks, game consoles, ..) These products comply with EN 300 328 and Annex 3 of ERC Rec 70-03. Maximum output power ranges from 1 mW e.i.r.p. (Class 3) to 100 mW (Class 1)

  - In 2011 only, in total 2 Billion (2.000.000.000) chipsets were shipped !!!

- ZigBee or 802.15.4 used in applications for Home Automation, Smart Grid, Building Automation, etc... These products comply with EN 300 328 and Annex 3 of ERC Rec 70-03. Maximum output power is 100 mW e.i.r.p.

  - In 2010, in total 60 million chipsets were shipped

- Other applications include

  - non specific SRDs (ERC Rec 70-03 Annex 1 / EN 300 440)

  - Railway applications in 2446-2454 MHz (ERC Rec 70-03 Annex 4 / EN 300 761)

  - Radiodetermination applications (ERC Rec 70-03 Annex 6 / EN 300 440

  - RFID (ERC Rec 70-03 Annex 11 / EN 300 440).