SE(12)034 rev1

Extract of FM(12)019rev2

(does not contain the Annexes)

| Working Group FM |  |  |
| --- | --- | --- |
| 74th Meeting |  |  |  |  |
| Bern, 23 – 27 April 2012 |  |  |  |  |
| Date issued: | 24 March 2012 |  |  |  |
| Source: | ECO |  |  |  |
| Subject: | WGFM QUESTIONNAIRE ON THE CURRENT STATUS OF DFS (DYNAMIC FREQUENCY SELECTION) IN THE 5 GHZ FREQUENCY RANGE |  |  |  |
| N Group membership required to read? (Y/N) |  |  |  |  |
| ### Summary This questionnaire was sent out from the Office on 24 October 2011. The official deadline was on the 1 March 2012. By 21 March 2012, a total of 32 countries provided an answer to the questionnaire. All the detailed responses are contained in annex 1 attached to this summary. |  |  |  |  |
| ### Proposal WG FM is invited to consider the responses and the ECO summary for necessary actions to be taken on the subject. The SRD/MG and WGSE are invited to consider the responses and the ECO summary and to provide guidance to WG FM. |  |  |  |  |

- Responses

Replies were received from 32 administrations by 21 March 2012.

| Albania Austria (11) up to 23 km Belarus Belgium (1) 10 km Bosnia Herzegovina Croatia Cyprus Czech Republic (about 50 per annum) critical cases up to 50 km, some up to 100 km Estonia (1) 6 km Finland (a few) 1 to 6 km France (11) 1 to 28 km Georgia Germany (10) No data Hungary (45) 4 to 45 km Ireland Italy (3) 2 to 30 km Latvia (23) 4 to 20 km Lithuania | Macedonia Malta Montenegro Polen (some) Up to 20 km Portugal Romania (20) 1.7 to 31 km Serbia Slovak Republic (1) No data, interference was gone before it could fully assessed Slovenia Spain (3) 1 to 8 km Sweden The Netherlands (15) 4 to 30 km The United Kingdom (24, many thousands of possible inter-ference events were noted. Of these, 24 were sufficiently long lasting and detectable by the Ofcom monitoring team. Up to 100 km Turkey |
| --- | --- |

The 16 countries which are in bold reported interference cases.

The number of cases within the timeframe 2010/2011 is added in brackets behind the respective country. All the reported cases were with meteorological radars; with some only minor exception (Germany reported one case with military radar, Austria one in 2008). This can either mean no information or no complaints from the military users.

The interference distance between the WAS/RLAN/BFWA interfering device and the victim radar is also given behind the respective country (14 countries provided interference distances). Most of the reported distances are between 1 km and 50 km while the UK and the Czech Republic reported distances up to 100 km. The interfering device has often been an outdoor device that operated within the frequency range 5600-5650 MHz. The configuration settings of the interfering device have often been accessible to the user (such as country setting, DFS on/off, output power, channel selection).

Several countries (Spain, Belgium, France, Latvia, Romania) included snapshots in their responses.

The UK indicated that many plots were produced by the “Met Office” in the UK. A representative selection of these plots has been made available to the ECO and can be provided for more detailed investigations, if needed. The Czech Republic provided a more detailed report including information about the 5 GHz Wifi market estimates in the Czech Republic and related interference problems (see addendum to answer 19 from the Czech Republic annexed to this document). The Czech Republic also offered additional information, i.e. technical protocols and statistical data, if needed, in support of the work in the ECC and ECO has already requested to send in this information.

Based on information of the Czech telecommunication cluster, there are more than 275 000 P-P or PMP connections in 5 GHz band provided by a thousand of operators in the Czech Republic. There are almost three times more other users of WiFi technology in 5 GHz band for different purposes. With regard to the number of users of RLANs technology in 5 GHz band, the Czech Republic stated it has already reached a “point of no-return”.

Spain:

Real time snapshot of the nature of the interference experienced on the radar screen or console

Belgium provided 3 snapshots

Radar with RLAN ON

Radar with RLAN OFF

Distance between radar and RLAN : ~10km

France

Straight lines observed on each illustration are the interferences caused by WAS/RLAN systems.

Latvia

Some examples of intereference to meteorological radar

sITUATION AFTER ENFORCEMENT CAMPAIGN carried out during FEBRUARY/mARCH 2012

Spectrum of the meteorological radar (provided by Latvia)

Typical description of a Meteorological Radar (from Romania):

Radar system manufacturer is EEC (Electronic Enterprise Corporation, USA, http://www.eecradar.com) and system type is DWSR-2500C. Most important features are:

- Frequency range: 5500-5700MHz
- Peak power: 250kW

- Pulse Duration: 0.8 μsec & 2.0 μsec
- Antenna gain: 45 dB (4.3m satellite dish, HPBW = 1° vertical and horizontal secondary lobes <25dB up to ± 12° and <34 dB over ± 12°)
- Receiver noise factor: ≤ 2 dB
- Dynamic range: 100dB
- Receiver sensitivity (MDS - Minimal Detectable Signal) versus band at the reception:

-112 dBm@0.75 MHz – in mode "Intensity"
-109 dBm@1.5 MHz – in mode "Velocity1"
-105dBm @ 3 MHz – in mode "Velocity2"

Question 2 (in cases of several different interference cases, the box below could be used several times)

| Was the WAS/RLAN/BFWA device operating co-channel with the radar? Please specify the operating frequencies of both interferer (WAS/RLAN/BFWA) and victim (radar) | The majority of the reported interference cases seem to be co-channel operation of the meteorological radar and the interfering device (WAS, RLAN, BFWA, or other): Spain: 2 cases YES and 1 case NO France: 4 different cases Yes and 3 different cases NO UK: YES Belgium: 1 case YES Poland: YES (all cases) Germany: Unknown. Due to the DFS mechanism the data base contains only the frequency band. Italy: YES (2 cases), 1 case nearly co-frequent Czech Republic: YES NL: YES, except one case where there was out of band emissions by BFWA equipment, but meteo-radar has no input filtering. Slovak Republic: YES Estonia: Yes Hungary: Yes Austria: YES Latvia: YES , co-channel or overlapping Romania: YES |
| --- | --- |
| Radar operating frequency? | Spain: 5620 MHz (co-frequent interference case) UK: 5600 – 5650 MHz Belgium 5620 MHz Poland: 5620-5680 MHz France: 5630 MHz, 5627.5 MHz, 5635 MHz (2x), 5640 MHz (3x) Italy: 5613 MHz, 5667,200 MHz – Bw 30 MHz Czech Republic: 5630 MHz, 5645 MHz Slovak Republic: 5600 MHz Estonia: 5640 MHz Hungary: 5610 MHz, 5625 MHz Austria: 5625 MHz, 5600 MHz, 5640 MHz Latvia: 5660 MHz Romania: 5531 MHz, 5637 MHz The problem exists mostly for the frequency range 5600-5650 MHz |
| WAS/RLAN/BFWA operating frequency? | (includes only the frequencies for the cases which were not co-frequent operations of the radar and the interfering device) Spain: Frequency: 5500 MHz (victim on different frequency, The frequency 5500 MHz is the image frequency of the radar system (FRadar= 5620 MHz, FLO=5560 MHz and IF= 60 MHz) so the interference was produced by poor image frequency rejection. UK: a bandwidth exceeding the 5600-5650 MHz Belgium: 5200-5825 MHz Italy: 5680 MHz Finland: close adjacent (overlapping) channel (only 10 MHz offset) at 5605/5615 MHz or radar image frequency |

Question 3: What was the type of device/equipment that caused the interference?

The vast majority of proven interference cases are caused by outdoor operation of RLAN, wireless networks, links such as for wireless cameras, or BFWA.

|  | Remarks |
| --- | --- |
| Individual Indoor RLAN device (private user) |  |
| Indoor RLAN network (office/enterprise/factory type of deployment) |  |
| Individual Outdoor WAS/RLAN/BFWA device | Czech Republic Hungary (10% of all cases) Austria Latvia Romania |
| Outdoor RLAN network, including Mesh networks, public networks, hotspot applications, … | Belgium: 1 case Poland France: Video surveillance equipment (2 out of 11 interference cases) Italy Czech Republic Hungary (20% of all cases) Austria Finland: See best documented ref: 2006.09.11 105SE(06) The case we had inyear 2010 is almost similar but not documented in the same way. Romania |
| Outdoor Point-to-Point link or BFWA | Spain: 3 cases UK: 2 cases wireless cameras, 8 cases point to point links France: 9 out of 11 interference cases Czech Republic NL: Wireless camera connections for police and security Estonia Hungary (70% of all cases) Austria Latvia |
| Other: …… | UK (14 cases): 1X cell phone enhancer, 1X mast head amplifier, 1 X Military radar, 1X military (unknown), 10 X not resolved and unknown transmitters. Slovak Republic: Unknown, because interference had disappeared before it has been identified |

Question 4: How was the interference case solved?

The responses received suggest that part of the problem is unlawful operation (DFS disabled, transmitter output power too high, etc.). In addition, the avoidance of co-channel operation by selecting a different operating channel has often been used.

|  | Remarks |
| --- | --- |
| (Re-)Configuration of the WAS/RLAN/BFWA device (e.g. different operating channel) | Spain: 3 cases UK: 5 of the 8 point to point links were retuned 2 of the 2 wireless cameras were retuned. Poland (for all cases) France: For 9 out of 11interference cases, the operating frequency has been changed, the frequency of the radar and two adjacent channels have been put on the blacklist Italy: 1 case Czech Republic YES Netherlands: BFWA replaced by reseller. Estonia YES Hungary: indicated this as the preferred method Finland: manual selection Latvia: skipping channel Romania: change channel of the interferer, the TCP/ DFS/ RID methods implemented in EN 301893 standard weren’t effective enough. |
| (Re-)Enabling DFS (in cases where DFS was disabled) | UK: 3 of the 8 point to point links had DFS re-enabled France: when the DFS was disabled (3 out of 9 cases), it has been enabled. For 2 cases, this was not sufficient since the interference came back after few months of safe radar operation. Czech Republic: Enabling DFS, however, sometimes the device interferes though DFS is enabled NL (DFS enabled) Austria: YES in all cases |
| Upgrading equipment (software/hardware) by vendor (to comply with later version of the standard) | France: For 1 equipment NL (software upgrade or factory correction) Latvia |
| Other: …… | UK: The 1X cell phone enhancer, 1X mast head amplifier, 1 X Military radar, 1X military (unknown) were all turned off or removed by the enforcement team. 10 X interference have not yet been solved France: For one interference case, the power of the WLAN equipment has been decreased in order to solve the problem. The DFS (v1.4.1) was operational but it had no effect. Belgium: Adjustment of the transmitter power Italy: Change of radar operating frequency to 5577,200 MHz NL (no further information) Slovak Republic: Interference disappeared |

Question 5: What is the version of EN 301 893/EN 302 502 to which compliance was declared?

Please check the standard version(s) that apply

| EN 301 893 v 1.2.3 EN 301 893 v 1.3.1 EN 301 893 v 1.4.1 EN 301 893 v 1.5.1 RLANs EN 302 502 v 1.1.1 EN 302 502 v 1.2.1 Fixed broadband data transmitting “non-standard/illegal” | Spain, Belgium, Czech Republic: EN 301 893 V1.5.1 Spain, Czech Republic: EN 302 502 V1.2.1 UK: Remarks: This data was not collected for Wireless Access Systems. The non-standard / illegal apparatus were; 2X military apparatus (non-standard), 1X mast head amplifier (unlawful) and 1X cell phone enhancer (unlawful) No data: Poland, Germany, Finland France: EN 301 893 V1.3.1 and V1.4.1 These answers are not exhaustive; the DFS version for some equipment wasn’t identified. For 1 case, the Standard version was not given in the documentation and neither the vendor nor the manufacturer was able/willing to provide the information. NL: EN 301 893 V1.3.1 Estonia: EN 301 893 V1.4.1 Hungary: EN 301 893 V1.3.1, V1.4.1, and V1.5.1 Latvia: EN 301 893 V1.2.3, V1.3.1, 1.41 , and 1.5.1 as well as non-standard / illegal apparatus Romania: EN 301 893 V1.3.1 and V1.5.1 |
| --- | --- |

Question 6: Was a Notified Body involved in the conformity assessment? (Can be checked by verifying the CE marking)

| Was a Notified Body involved in the conformity assessment? | YES: Hungary, Spain, Belgium, Poland, France: only for some equipment (2 out of 11), Czech Republic, Netherlands (software update caused DFS to fail), Romania No data: UK, Germany, Italy Estonia: No (for one specific equipment named in the response) Austria, Finland: No Latvia: in most cases only manufacturer’s declaration |
| --- | --- |

Optional Questions:

The below are optional questions for which the answers may not always be known. However, if known, they may help to assess whether DFS, as defined in the harmonised standard, is adequate, or if additional measures are required to further improve the sharing of WAS/RLAN/BFWA and radars in the 5 GHz bands.

Questions 7 to 10 request details about the equipment that caused the interference.

Question 7: What is/are the operating frequency range(s) of the equipment?

Summary: Most cases concern the 5600-5650 MHz frequency range

| - 5150-5250 MHz - 5250-5350 MHz - 5470-5725 MHz, INCLUDING 5600 – 5650 MHz - 5470-5725 MHz, EXCLUDING 5600-5650 MHz - 5725-5850 MHz | Austria, Hungary, Estonia Belgium, UK, Spain, Poland, France, Italy, Finland, Latvia: 5470-5725 MHz, INCLUDING 5600 – 5650 MHz (all cases), Romania Germany, Czech Republic: All these frequency ranges are involved Netherlands: 5725-5850 MHz |
| --- | --- |

Question 8: What type of antenna was used by the WAS/RLAN/BFWA device?

Summary: In almost all reported cases, directional or high gain directional antennas are used. Only Latvia and Romania reported also cases with omnidirectional antenna.

| i. Omni directional antenna ii. Directional antenna (patch, sector, …) iii. High gain directional antenna (in case of P2P links) | Directional (Spain: 2 cases, Belgium 1 case, Poland) High Gain (Spain 1 case, Poland) UK: Either directional or high gain directional. The gain of the directional antenna was not recorded. France: Directional and High Gain Italy: directional Czech Republic: direction and High Gain NL: directional Estonia: 14 dBi Hungary: directional and high gain Austria: directional or high gain directional Finland: directional Latvia: all cases Romania: all three types |
| --- | --- |

Question 9: Has the user the possibility to (re-)configure the equipment? (Please check all that apply):

Summary: User can often re-configure the equipment (country settings, DFS on/off, frequency selection)

| Select a (different) country of operation (where other DFS rules might apply, or where no DFS is required. E.g. countries outside the EU) | YES: Austria, Hungary, Estonia, Belgium, Poland, Czech Republic, Finland, Latvia, Romania No: Spain, Netherlands No data: UK, Germany, France, Italy |
| --- | --- |
| Alter or disable the DFS functionality (directly or indirectly), e.g. to avoid false DFS triggering. | YES Austria, Hungary, Estonia, Belgium, Poland, France, Finland, Czech Republic, NL : (after software upgrade), Latvia, Romania UK: 3 cases: point to point links had DFS re-enabled. Whether other devices had this facility was not recorded. No Spain No data: Germany, Italy |
| Select operating frequencies which are not allowed in your country | Yes Hungary, Estonia, Belgium, Czech Republic, Finland Netherlands NL (after software upgrade), Latvia, Romania No Spain, Austria No data: UK, Poland, Germany, France, Italy |

Question 10: Verification/Testing performed?

| Did you check/verify by testing the compliance of the product against the ETSI standard? | YES Spain, Czech Republic, NL Hungary (only some) Netherlands: field tested UK: None as yet. However, the UK may take the opportunity to use the Met Office operational weather radar testing facility at Wardon Hill to undertake this testing in the near future. No data or not checked: all others |
| --- | --- |
| For EN 301 893 v 1.5.1 compliant equipment, is the radar operational mode covered by any of the radar test signals in EN 301 893 v 1.5.1? | YES Spain, Belgium, Czech Republic, Hungary, Romania No data or not checked: all others |
| For EN 302 502 v 1.2.1 compliant equipment, is the radar operational mode covered by any of the radar test signals in EN 302 502 v 1.2.1? | YES Spain, NL, Hungary, Romania No data or not checked: all others |