RESOLUTION 251 (REV.WRC-23)

Considerations on the removal of the limitation regarding aeronautical mobile in the frequency range 694-960 MHz [and possible other frequency bands] for the use of International Mobile Telecommunications user equipment by non-safety applications

The World Radiocommunication Conference (Dubai, 2023),

considering

a)	that there is a demand for greater connectivity for passenger and aeronautical communication in different classes of aeronautical vehicles at lower and higher altitude;

b)	that current and future International Mobile Telecommunications (IMT) networks can already provide connectivity  to helicopters, small aircraft and unmanned aircraft systems (UAS) at lower altitudes;

c)	that future IMT networks may also provide connectivity for passenger and aeronautical communication at higher altitudes;

d)	that future IMT networks may provide communication functions beyond line-of-sight;

e)	that IMT networks already support direct air-ground connectivity services to commercial airplanes with specific equipment on board airplanes in other frequency bands;

that earlier ITU-R sharing and compatibility studies supporting the IMT identification of specific frequency bands in the range under consideration did not consider aeronautical use cases,

noting

a)	that ITU Radiocommunication Sector sharing and compatibility studies supporting the identification of specific frequency bands for IMT did not consider the use cases described in considering b) to e);

a)	that the frequency band 694-960 MHz is allocated on a primary basis to the mobile, except aeronautical mobile, service in Region 1;

b)	that the frequency bands 890-902 MHz and 928-942 MHz are allocated on a primary basis to the mobile, except aeronautical mobile, service in Region 2 and that the frequency band 902-928 MHz is allocated on a secondary basis to the mobile, except aeronautical mobile, service in Region 2;

c)	that Nos. 5.312 and 5.323 allocate the frequency band 645-960 MHz or parts thereof to the aeronautical radionavigation service on a primary basis in several countries of Region 1;

d)	that the frequency band 694-960 MHz is allocated on a primary basis to the broadcasting service in Region 1;

e)	that the frequency bands 1 400-1 427 MHz, 1 610.6-1 613.8 MHz, and 1 660-1 670 MHz are allocated to the radio astronomy service (RAS) on a primary basis;

f)	that the frequency bands under consideration are identified for use by IMT user equipment in accordance with Nos. 5.286AA, 5.295, 5.308A and 5.317A, and Recommendation ITU-R M.1036;

g)	that Resolution 224 (Rev.WRC-19) addresses frequency bands for the terrestrial component of IMT below 1 GHz;

h)	that Resolution 749 (Rev.WRC-19) addresses the use of the frequency band 790-862 MHz in countries of Region 1 and the Islamic Republic of Iran by mobile applications and by other services;

i)	that Resolution 760 (Rev.WRC-19) addresses provisions relating to the use of the frequency band 694-790 MHz in Region 1 by the mobile, except aeronautical mobile, service and by other services,

recognizing

a)	that the removal of the limitation regarding aeronautical mobile in the proposed frequency bands would enable the unified use of the IMT identifications by aeronautical user equipment throughout the Regions;

b)	that IMT networks can already support a significant number of airborne user equipment and therefore the expected deployment density of airborne user equipment is high;

c)	that RAS stations are extremely sensitive to air- and spaceborne transmissions of active services (see No. 5.149) owing to the increased likelihood of line-of-sight conditions and main-beam to main-beam coupling and may suffer from 2nd harmonics of airborne user equipment in the IMT networks,

resolves to invite the ITU Radiocommunication Sector to complete in time for WRC-27

1	assessment of relevant aeronautical mobile service scenarios for air-ground and ground-air connectivity for airborne user equipment in IMT networks to be addressed in compatibility and sharing studies;

2	identification of relevant technical parameters associated with the aeronautical mobile systems;

3	sharing and compatibility studies with existing services, including in adjacent frequency bands, in order to ensure protection of, and not impose undue constraints on, the existing services, and their future development;

4	studies on spurious emissions, including 2nd harmonics of airborne user equipment in IMT networks into RAS stations in the frequency bands listed in noting e) in order to ensure protection of, and not impose undue constraints on, existing and future RAS stations in those frequency bands,

invites administrations

to participate actively in the studies and provide the technical and operational characteristics of the systems involved by submitting contributions to ITU-R,

invites the 2027 World Radiocommunication Conference

to consider, based on the results of studies, the possibility of removing the aeronautical mobile service exception or other suitable regulatory measures in the frequency [ranges 694-960 MHz in Region 1, XXX-XXX MHz in Region 1], and 890-942 MHz in Region 2.

invites the 2027 World Radiocommunication Conference

_______________