- MFCN TECHNICAL PARAMETERS AND ASSUMPTIONS FOR 2.3-2.4 GHz band Xborder SIMULATIONS

  - Simulation assumptions

Table 9: Generic parameters

| BS/UE mode | Parameter | Value | Notes |
| --- | --- | --- | --- |
|  | Centre Frequency (MHz) | 2350 | Centre frequency of the 2300-2400 MHz band |
|  | Propagation model | ITU-R P.1546 [8] | Intra-network : 50% location, 50% time. Interfering link : 50% location, 10% time. No clutter layer |
| AAS | BS Tx Power | 200 W (53 dBm) for AAS 68 dBm/5 MHz EIRP for non-AAS |  |
|  | SCS (kHz) | 30 kHz | Subcarrier spacing |
|  | BS SSB antenna gain (dBi) | Multi- beams: 24 dBi. Fixed beam: 17 dBi |  |
|  | BS ACLR (dB) | 45dB | 3GPP TS 38.104 [14] |
|  | BS noise figure (dB) | 3 | 5 dB from 3GPP withour- MIMO processing gain, (real BS NF is better than 3 dB ) |
|  | Antenna pattern | ITU-R M.2101 [15] | See also table below for suburban/rural detailed parameters |
|  | Conducted power (before Ohmic loss) per antenna element (dBm) | 35 | The conducted power per element assumes 8×8×2 elements (i.e. power per H/V polarized element). Conducted Pwr per ant. Elem.= BS Conducted Pwr - 10*log 10(8x8) |
|  | Antenna array configuration (Row × Column) | 8 × 8 elements | 8×8 means there are 8 vertical and 8 horizontal radiating elements. In the sub-array case, one implementation is 2 vertical radiating elements combined in a 2×1 sub-array |
|  | Base station maximum coverage angle in the horizontal plane (degrees) | 120 |  |
|  | Antenna polarisation | Linear ±45º |  |
|  | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V |
| Non-AAS | BW | 40 MHz | In this Report, it is assumed that Non-AAS will rely on LTE 15 kHz SCS, although 5G-NR 30 kHz SCS could theoretically also exist with this configuration |
|  | SCS (kHz) | 30 kHz |  |
|  | Antenna pattern | Recommendation ITU-R F.1336 [16] (recommends 3.1), ka = 0.7, kp = 0.7, kh = 0.7, kv = 0.3, Horizontal beamwidth = 65° | Vertical 3 dB beamwidth: determined from the horizontal beamwidth by equations in Recommendation ITU-R F.1336 . Vertical beamwidths of actual antennas may also be used when available. |
|  | Antenna polarisation | Linear/±45 degrees |  |
|  | Feeder loss | 3 dB |  |
|  | Maximum base station output power (per cell) | 59 dBm | The value for the BS output maximum power is given in case of MIMO 2 (with two antenna per cell) |
|  | Maximum base station antenna gain | 18 dBi |  |
|  | Maximum base station output power/sector (EIRP) | 77 dBm | 68 dBm/5 MHz per antenna, 2 antennas/sector |
| UE | UE Tx Power (dBm) | 23 | 3GPP 38.101 [9] |
|  | UE antenna gain (dBi) | -4 | ITU-R M.2292 [10] |
|  | Data user body loss (dB) | 0 | Provided by a MNO |
|  | HO margin (dB) | 0 | Provided by a MNO |
|  | UE height (m) | 1,5 | ITU-R M.2292 |
|  | Wall Loss for indoor UE (dB) standard deviation (dB) | 5 |  |
|  | UE Power control parameter | CLx 95th percentile | Derived for the frequency band 2300 MHz for SEAMCAT |
|  | BS-UE MCL | 70 | ITU-R M.2292 |
|  | UE noise figure (dB) | 9 | ITU-R M.2292 |
|  | UE ACLR (dB) | 30 dB | 3GPP TS 38.101 |
|  | UE ACS (dB) | 33 dB | 3GPP TS 38.101 |

Table 10: co-channel / adjacent-channel specific parameters

| Parameter | Co-channel | Adjacent-channel | Notes |
| --- | --- | --- | --- |
| Channel bandwidth (MHz) | 40 MHz | 20 MHz |  |
| n_RB | 106 | 51 | maximum RB in the transmitted BW for an SCS of 30kHz, with 12 subcarriers per RB |
| BS SSB Tx Power | 22 dBm/30 kHz for AAS BS 28 dBm/30 kHz for non-AA BS | 25 dBm/30 kHz for AAS BS 31 dBm/30 kHz for non-AAS BS | 200 W / n_RB * 360 kHz) * 30 kHz for AAS BS 59 dBm+10*log10(30 kHz/(n_RB*360 kHz) |
| BS ACS | 38.2 dB for 40 MHz channel | 41.2 dB for 20 MHz channel | 3GPP TS 38.104 [14]. (Calculated with -52 dBm ACS level and 3 dB noise figure) |

Table 11: Suburban / rural specific parameters

|  | Parameter | Suburban | Rural | Notes |
| --- | --- | --- | --- | --- |
|  | Cell Range (m) | 1000 | 5000 | Typical cell range with the existing radio sites in France |
|  | UE Indoor/Outdoor percentage(%) | 70/30 | 50/50 | ITU-R M.2292 [10] |
|  | Wall Loss for indoor UE (dB) | 20 | 10 | 20 dB for urban in ITU-R M.2292 |
|  | BS antenna height (m) | 25 | 35 |  |
|  | Network loading | 70% | 50% |  |
|  | Mechanical downtilt (degrees) | 6 | 3 | Mechanical downtilt is typically an optimization parameter as it can reduce the inter-cell interference within a network and also decrease the interference in cross-border situations |
| AAS | Element gain (dBi) | 6.4 | 6.4 | Includes the 2dB array ohmic loss |
|  | Maximum antenna Gain (dBi) | 24.5 | 24.5 | the maximum antenna gain has been calculated has follow : element antenna gain + 10xLog10 ( 8x8 elements) |
|  | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H, 65º for V | 90º for H, 54º for V |  |
|  | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.7 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V |  |
|  | Base station vertical coverage range (degrees) | 90-120 | 90-100 | given for the elevation angle θ, defined between 0° and 180° as in ITU-R M.2101 [15] |

Simulation scenarios and methodology

The simulation scenario is illustrated in Figure 21. Two network clusters A and B are separated of distance D. Interference from network cluster A to network cluster B is simulated.

Figure 21: Simulation scenario between two TDD networks (Network A (Left) and Network B (Right))

As shown in Figure 21, two networks A and B are separated of distance D. In each network, UEs are randomly generated in each cell (1 UE per cell occupying the whole channel bandwidth), interference from network A to network B is simulated. First the Network B reference throughput is simulated as the throughput without interference from Network A, then the Network B throughput loss caused by the presence of interference from network A is simulated.

Synchronised operation

In this case, Network A and B are in operation in co-channel, the co-channel interference from network A downlink to network B downlink is simulated.

| 40 MHz Hz 40 MHz 0 MHz |  |  |
| --- | --- | --- |
| a) AAS to AAS b) AAS to non-AAS c) non-AAS to AAS |  |  |

Figure 22: Co-channel DL to DL

Unsynchronised operation with non-preferential frequency blocks

In this case, Network A and B are in operation in co-channel, the co-channel interference from network A downlink to network B uplink is simulated.

| 40 MHz Hz 40 MHz 0 MHz |  |  |
| --- | --- | --- |
| a) AAS to AAS b) AAS to non-AAS c) non-AAS to AAS | b |  |

Figure 23: Co-channel DL to UL

Unsynchronised operation with preferential frequency blocks

In this case, Network A and B are in adjacent band operation, the adjacent-channel interference from network A downlink to network B uplink is simulated.

| 20 MHz Hz 20 MHz 0 MHz |  |  |
| --- | --- | --- |
| a) AAS to AAS b) AAS to non-AAS c) non-AAS to AAS |  |  |

Figure 24: Adjacent channel DL to UL