| #### SEAMCAT Technical Group (STG) Send to : seamcat@eco.cept.org |  |  |
| --- | --- | --- |
| #### Date issued: | #### 25 October 2021 |  |
| #### Source: | #### Ivica Stevanovic (OFCOM CH) |  |
| #### Subject: | #### PMP ITU-R P.452-17 |  |

Type of report (Bug, Enhancement or New Feature)

New Feature

Issue ID

SEAMCAT version

5.4.3 Alpha1

SEAMCAT component involved

PMP

Background information, reference documents and related tickets

In July 2021, ITU-R SG 3 approved draft revision of Recommendation ITU-R P.452 “Prediction procedure for the evaluation of interference between stations on the surface of the Earth at frequencies above about 0.1 GHz” which is now published by ITU-R (https://www.itu.int/rec/R-REC-P.452/en).

Description of the issue

This revision aligns the free-space propagation model with Recommendation ITU-R P.525-4 “Calculation of free-space attenuation” by introducing slant-path distance between transmitter and receiver antennas instead of great-circle (horizontal) path distance (this was already implemented in SEAMCAT in version P.452-16) and by aligning the constant in the free space equation (92.4 dB instead of 92.5 dB).

Proposal

To consider implementing the new PMP that includes the most recent version of Recommendation ITU-R P.452-17. Annex 1 provides the JAVA source code of the new PMP and includes updated and new unit tests used to validate the implementation against the ITU-R SG 3 MATLAB reference results as well as the corresponding documentation for SEAMCAT Handbook.

Suggested Priority (High, Medium or Low)

High

PMP code

PMP UI

PMP Unit Tests

PMP Documentation